ESM File 1: Sequential Extraction Procedure

The method as modified by Horowitz et al (1991) was used for the sequential extraction of the trace metals
under investigation. The procedure is as follows:

I. Exchangeable fraction: 1.0 g of the dried sediment sample was weighed into a beaker. 50 cm3 of 1.0 M MgCl2
solution was measured and added. The extraction was done at room temperature for 1hr with continuous
agitation on an electric shaker. The supernatant was allowed to settle and filtration was done into a 100 cm3
standard volumetric flask. Excess deionized water was used to wash the residue on the filter paper. This
was later made up to the mark and stored in a sample bottle, labelled and kept until needed for the analysis.

II. Bound to carbonate fraction: The residue from above was transferred into a clean and dry beaker. It was
leached at room temperature with 50 cm3 solution of 1.0 M CH3COONa prepared and adjusted to pH 5.0 with
ethanoic acid. Continuous agitation was maintained for 1hr on an electric shaker and the resulting solution
was filtered through Whatman No1 filter paper into 100 cm3 standard volumetric flask. The residue on the
filter paper was further washed to remove the remaining reagent that might retain in the residue. The filtrate
was made up to the mark with deionized water. The extract was stored in a sample bottle labelled and kept
ready for analysis.

III. Bound to Fe-Mn oxides: The residue from (ii) above was transferred into a beaker. It was extracted with
50 cm3 mixture of 1.0 M each, ammonium oxalate and oxalic acid for 1hr with continuous agitation over an electric
shaker. The resulting solution was filtered into a 100 cm3 standard volumetric flask. The residue on the filter
paper was further washed with deionized water. The leachate was made up to the mark. The extract was
stored in a sample bottle and finally labelled ready for AAS analysis.

IV. Bound to organic matter: The residue from the previous leachate was transferred into a clean and dry
beaker. It was extracted with 50 cm3 solution of 0.1 M NaOH with continuous agitation over an electric
shaker for 1 hr. this was then filtered through Whatman No1 filter paper into a 100 cm3 standard volumetric
flask. The residue on the filter paper was further washed with deionized water. The content was later made
up to the mark, stored in a sample bottle and labelled properly prior to analysis.

V. Residual: The residue from (iv) above was digested with a mixture of hydrochloric and hydrofluoric acids
for 5hrs. This was later washed and filtered with deionized water into a 100 cm3 standard volumetric flask
and made up to the mark. The filtrate was allowed to cool and stored in a sample bottle and labelled
accordingly ready for AAS analysis.

ESM file 2 Results of validation

Cd (0.01), Mn (0.5), Cr (0.02), Cu (0.02), Fe (0.08), Pb (0.01), and Zn (0.10).

Table ESM1: Results of concentration levels (mg kg-1) of trace metals in the floodplain suspension collected from Enugu, Nigeria.

Fraction	Fe	Mn	Cu	Zn	Pb	Ni	Cd	Cr

Exchangeable	42.47 ± 3.37	<LoD	2.58	47.70	3.55	0.58	0.18	1.11

Carbonate	827 ± 16.35	8.89	6.01	12.50	34.31	6.25	0.57	0.33

Reducible	226 ± 6.88	3.63	1.57	5.00	1.58	1.39	3.03	0.73

Oxidizable	383 ± 5.66	7.57	2.53	9.37	2.62	2.54	3.87	1.21

Residual	207 ± 9.64	10.66	6.54	2.15	66.44	1.83	5.19	4.24

Total Fraction	1685.47	30.75	19.23	76.72	108.5	12.59	19.46	6.62

Total Content	1784.52	40.81	19.30	74.97	110.01	13.34	19.20	6.94

% Bioavailability	51.59	28.91	44.67	78.47	34.89	54.25	39.83	6.65

Table ESM2: Results of concentration levels (mg kg-1) of trace metals in the floodplain suspension collected from Aba, Nigeria.

Fraction	Fe	Mn	Cu	Zn	Pb	Ni	Cd	Cr

Exchangeable	76.87 ± 3.37	9.14	2.80	1.48	64.32	2.46	0.51	0.06

Carbonate	788.10±10.22	10.71	4.24	4.21	42.70	1.46	3.36	0.04

Reducible	395 ± 6.42	16.45	2.33	4.49	8.47	0.49	1.58	0.19

Oxidizable	98.33 ± 3.31	<LoD	1.00	0.23	33.41	0.53	1.56	1.22

Residual	622.00 ± 9.35	0.28	1.02	3.98	11.42	0.52	5.22	2.30

Total Fraction   1986.30	36.58	11.39	14.39	160.32	5.46	12.23	3.81

Total Content	     1989.06	46.53	11.45	14.87	160.48	4.95	12.22	4.05

% Bioavailability	43.55	54.26	61.81	56.13	66.75	71.79	70.33	2.62

Table ESM3: Results of concentration levels (mg kg-1) of trace metals in the floodplain suspension collected from Onitsha, Anambra State, Nigeria.

Fraction	Fe	Mn	Cu	Zn	Pb	Ni	Cd	Cr

Exchangeable	76 ± 4.47	1.42	2.01	0.10	6.58	3.22	4.16	0.10

Carbonate	139 ± 7.27	<LoD	1.54	2.91	18.55	2.00	0.57	0.09

Reducible	320 ± 15.62	8.82	1.55	8.76	4.34	0.57	1.81	0.72

Oxidizable	960 ± 10.83	<LoD	1.08	11.69	0.71	1.09	<LoD	1.37

Residual	958 ± 9.65	4.91	1.10	9.98	1.56	1.10	1.14	3.60

Total Fraction	2453	15.15	7.28	33.44	31.74	7.98	7.68	5.88

Total Content	        2466.69	25.16	7.33	24.47	31.48	8.03	7.70	9.16

% Bioavailability	87.65	9.37	48.76	9.00	79.27	65.41	61.59	3.23

Table ESM4: Concentration levels (mg kg-1) of heavy metals partitioned in the floodplain suspension collected from Afikpo, Nigeria

Fraction	Fe	Mn	Cu	Zn	Pb	Ni	Cd	Cr

Exchangeable	550± 4.11 <LoD	3.71	6.23	32.54	3.56	6.04	1.40

Carbonate	252±10.02	6.23	4.09	8.90	3.45	0.50	4.01	0.80

Reducible	557 ± 6.37	6.55	3.56	11.83	0.54	0.54	3.07	2.30

Oxidizable	1150± 27.27 	0.27	1.20	2.55	66.60	5.48	5.40	2.80

Residual	653 ± 8.84	9.91	3.24	11.62	1.62	2.30	1.49	5.10

Total Fraction	3562	22.96	15.80	41.13	104.75	12.38	20.01	12.4

Total Content	     3569.01	33.02	15.88	32.60	109.49	8.12	20.52	12.77

% Bioavailability	25.74	27.13	49.37	36.79	34.36	32.79	50.22	11.69

Table ESM5: Results of concentration levels (mg kg-1) of trace metals in the floodplain suspension collected from Owerri, Nigeria

Fraction	Fe	Mn	Cu	Zn	Pb	Ni	Cd	Cr

Exchangeable 6.52 	0.81	2.82	0.32	8.35	0.39	0.90	0.20

Carbonate	321.57 22.19	72.33	14.53	39.44	1.46	3.95	1.33

Reducible	298.78  8.65	13.91	15.65	20.35	1.36	1.84	0.64

Oxidizable	42.01  	0.38	0.67	0.35	2.30	0.06	< LoD	2.71

Residual	2407.00 10.33	16.14	22.52	7.39	3.28	11.39	11.85

Total Fraction  3075.61 42.36 105.87 53.37	77.83	6.55	18.08	16.73

Total Content	    3106.25 42.96 105.92 53.50 77.91	6.64	18.02	17.04

% Bioavailability 10.57 54.30 70.98	 27.82	61.40	28.24	26.83	9.15

Principal component and correlation analyses

The PCA was used to illustrate the overview and deviations among the industrialized areas in Southeast Nigeria. Fig. 4(a–d) shows the core plots and biplots for the CF of immediate and future HM pollution in the FSS of the study areas. The distribution of HM responsible for immediate pollution (Fig. 4a and b) indicated that Cu, Mn, Cr, and Pb were more prevalent in Owerri; Zn and Fe in Enugu; Cd in Afikpo, while Ni was more prevalent in Onitsha and Enugu. In the future, there may be an accumulation of HMs in Onitsha. Generally, the accumulation of Fe, Cu, and Cr was highest in Afikpo and Owerri. Meanwhile, Zn, Cd, Mn, and Ni were the most HMs found in Enugu, while Pb was the most prevalent heavy metal in Aba.

- Factor plot                                    	(b) Biplot

- Factor plot                                    	(d) Biplot

Fig. 4 PCA for the CF of immediate (a and b) and future (c and d) HM pollution of FSS in industrialized areas of Southeast Nigeria

The results in Figures 5(a–d) indicate that various biochemical reactions occur at industrial areas, leading to changes in CF for both immediate and future pollution levels. This may have led to the generation of new bio-contaminants, resulting in a shift in the concentration of HM pollution among the sites. Again, the ERF results indicated that Owerri had the highest concentration of Cr for both immediate and future pollution levels, while Enugu had the highest concentration of Zn and Ni for the same reason.

- Factor plot                                     	(b) Biplot

(c) Factor plot                                               	(d) Biplot

Fig. 5 PCA for ERF of immediate (a and b) and future (c and d) HM pollution of FSS in major cities in Southeast Nigeria

In Figure 6(a–d), the mPI, mCd, and ERI are presented. Owerri had the highest mPI for both immediate and future pollution. In general, for mCd and ERI, Cr levels were higher in Owerri for both immediate and future pollution effects, whereas Enugu recorded the highest levels of Zn for the same reason given above. The results of this study could serve as a guide for determining the appropriate measures needed to control HM pollution in industrialized areas of Southeast Nigeria.

- Factor plot                                     		(b) Biplot

(c) Factor plot                                   		(d) Biplot

Fig. 6 PCA for mPI, mCd, and ERI of immediate (a and b) and future (c and d) HM pollution of FSS in major cities in Southeast, Nigeria