# Selective agents, microbial ecology, and horizontal gene transfer underly microdiversity-level heterogeneity in the fate of resistance genes during biological wastewater treatment

Connor L. Brown1, Ayella Maile-Moskowitz2, Delaney Snead3, Allison J. Lopatkin4, Kang Xia5, Latania Logan6, Benjamin Davis2, Liqing Zhang7, Peter Vikesland2, Amy Pruden2

Dept. of Genetics, Bioinformatics, and Computational Biology, Virginia Tech1

Dept. of Civil and Environmental Engineering, Virginia Tech2

Dept. of Civil and Environmental, University of Michigan3

Dept. of Chemical Engineering, University of Rochester4

Dept. of Plant and Environmental Sciences, Virginia Tech5

Dept. of Pediatrics, Emory University6

Dept. of Computer Science, Virginia Tech7

# Supplementary Methods

# Section S1. Scaffolding phage genomes

Draft phage genomes were constructed by aligning scaffolds produced by hybrid assembly to a 45,000 basepair putative prophage extracted from CP064980.1 (pos 10,000-55,000 bp). Contigs with alignments to the reference genome were visualized and the appropriate orientation and order inferred from the clinker1 plot. Sequences were merged and/or reversed as appropriate using seqkit.2

# Section S2. Untargeted PPCP analysis

### Extraction and cleanup of water samples using solid phase extraction (SPE)

An aliquot of 200 mL water sample  pre-filtered with a 0.7 µm 55 mm in diameter glass fiber filter (Whatman, Maidstone, UK) was loaded onto a SPE cartridge (hydrophilic-lipophilic balance cartridge, Waters, Milford, MA) that was pre-conditioned with 3 mL HPLC-grade methanol and 3 mL ultra-pure water. The sample flow through the cartridge was maintained at 5 mL min-1. Once the sample passed through the cartridge, the cartridge was then dried for 10 mins by gently pulling air through it to remove any remaining water. The PPCPs were then eluted off the cartridge with 3 mL HPLC-grade methanol. The eluted sample was then completely dried under N2 gas on a vacuum evaporation system (RapidVap, Labconco Kansas City, MO), reconstituted with LC-grade acetonitrile-water solution (1:1, v:v) to 1 mL, filtered through a 0.2 µm polytetrafluoroethylene (Thermo Scientific, Waltham, MA) syringe filter into a 2 mL glass amber vial (Agilent, Santa Clara, CA), and then screened for 138 PPCPs on a ultra-performance liquid chromatography-tandem mass spectrometry (UPLC/MS/MS) using method described below.

### UPLC/MS/MS screening for PPCPs

An ultra-performance liquid chromatography (UPLC) (1290, Agilent Technology, Santa Clara, CA) coupled with a tandem mass spectrometry (6490, Agilent Technology, Santa Clara, CA) was used for the multi-compounds screening of 138 PPCPs. The PPCP screening method was developed based on a compounds database and chromatographic method that was custom-developed by the Agilent Technology upon analysis of 140 analytical standards of compounds which were reported to be the most detected in environmental water. This database was developed using the same model of analytical column (Agilent Zorbax Extend C18 analytical column, 5 µm × 4.6 mm × 50 mm) and UPLC/MS/MS (Agilent 1290-6490) as that used in our study. This database contains specific compound identification criteria (i.e., retention time, precursor ions, product ions, collision energies) for the identification of each of the 138 PPCPs.

For the chromatographic section of the method, compounds were separated on the Agilent Zorbax Extend C18 analytical column (4.6 mm × 50 mm, 5 µm). The temperature of the analytical column oven was kept at 40°C. The mobile phase consisted of water with 0.1% formic acid (component A) and water with 95% acetonitrile (component B). The mobile phase flow rate was 0.3mL min-1 under a gradient elution of 0 - 15 min, 95% - 10% A, 15 - 17.5 min, 10%A, 17.5-18 min, 10% - 95%A, and 18 – 20 min, 95%A. The total sample injection volume was 20 µL.

For the mass spectrometry section of the method, electrospray ionization (ESI) and multiple reaction monitoring (MRM) mode was used. Each sample was analyzed separately in both positive and negative modes to detect oppositely charged ions. A compound in a sample was identified based on matching the masses of its precursor ion, product ion I, and product ion II with those listed in the database and further confirmed based on matching chromatographic retention time and peak shape between product ion I and II. In addition, if the signal to noise ratio of the chromatographic peak of one of the product ions for a suspected compound was less than 3, the identity of this compound was rejected.

For quality assurance and quality control (QA/QC), one laboratory blank (ultrapure Milli-Q water) and one spiked recovery sample (ultrapure Milli-Q water spiked with known concentrations of four analytical standards: Carbamazepine, Erythromycin,	Sulfamethazine, Chlorotetracycline, Tylosin, Sulfamethoxazole, and Triclosan) were extracted, cleaned up, concentrated, and analyzed along with the water samples for every 20 samples to monitor possible cross contamination of the compounds screened for and recovery of the spiked compounds. In addition, an instrument blank sample (UPLC mobile phase) was injected for every 20 samples to monitor possible instrument cross contamination. All analytes screened for were not detectable in any of the instrumental blanks or laboratory blanks, indicating free of cross contamination of those compounds during sample processing and analysis.

References

1.	Gilchrist, C. L. M. & Chooi, Y.-H. clinker & clustermap.js: automatic generation of gene cluster comparison figures. Bioinformatics 37, 2473–2475 (2021).

2.	Shen, W., Le, S., Li, Y. & Hu, F. SeqKit: A Cross-Platform and Ultrafast Toolkit for FASTA/Q File Manipulation. PLoS One 11, e0163962 (2016).