Supplementary information

# Co-assessment of costs and environmental impacts for off-grid direct air carbon capture and storage (DACS) systems

Moritz Gutscha,b,* , Jens Lekera,b

a Institute of Business Administration at the Department of Chemistry and Pharmacy, University of Münster, Leonardo Campus 1, Münster 48148, Germany

b Helmholtz Institute Münster, IEK-12, Forschungszentrum Jülich GmbH, Corrensstraße 46, Münster 48149, Germany

* Corresponding author. E-mail address: moritz.gutsch@uni-muenster.de

# Content

1. Supplementary Note 1: Literature overview

2. Supplementary Note 2: Life cycle inventory data

# 1. Literature overview

## 1.1. Life cycle assessments (Global warming potential)

Supplementary Figure 1: Reported results for global warming potential associated with capture/removal of 1 ton CO2 from the atmosphere with low-temperature, solid sorbent technology.

With Deutz & Bardow1, Madhu et al.2 and Terlouw et al.3, three comprehensive LCAs of low-temperature DACS have recently been conducted. All share that adapted life cycle inventory (LCI) data from Climeworks´ DACS plant in Iceland was used. Since our interest lies in PV powered autonomous systems, presented results from reviewed studies focus on situations which come closest to this setting (see Supplementary Table 1). Results from the IEAGHG4 report are also included, although authors of the report caution that no complete LCA was conducted.

Technology learning plays a crucial role for fast developing technologies such as DACSs. Moving from a reference 4 kt/year DAC to a future 100 kt/year DAC reduces specific heat requirements per ton CO2 by 55% from 3,300 kWhheat to 1,500 kWhheat in Deutz & Bardow1. Specific electricity consumption comes down from 700 kWh to 500 kWh. These improvements, alongside lower adsorbent material consumption increase the CRE from 80% to 89% for a generic PV system in Germany as source of energy (see Supplementary Figure 1). Energy requirements obtained by Deutz & Bardow1 have, with some adjustments, served as input for subsequent work of Terlouw et al.3 and IEAGHG4.

The IEAGHG compares a first of a kind (FOAK) DAC with a nth of a kind (NOAK), incorporating learning effects4. Energy requirement for the first 1 Mt/year plant was derived from Deutz & Bardow´s reference technology with a reduction of 10% due to a tenfold increase in capacity. Whether capacity increase by itself is associated with lower energy requirements is subject of debate. In an upscaling scenario from 50kt/year to 1Mt/year, Madhu et al.2 reported reduced energy requirements only for HT liquid solvent systems but not LT solid sorbent ones due to the modular design of the latter. Reduction in energy related GHG emissions is more substantial in IEAGHG4 than Deutz & Bardow1. This is explained by updates in the carbon intensity of PV electricity in IEAGHG4 (from 50.9g / kWh to 24.8 / kWh from FOAK to NOAK) while Deutz & Bardow1 use the same carbon footprint for PV in both scenarios.

Both Deutz & Bardow1 and IEAGHG4 did not include any impacts of intermittency in their PV setup. Consequently, presented results might overestimate the achievable CRE. Terlouw et al.3 who included impacts of a LIB storage, report that the battery accounts for around 20% of GWP for carbon removal. However, only one battery configuration (221MWh) is looked at for PV-powered systems with a heat pump.

Recently, Qui et al.5 evaluated effects of technology learning and updates of background inventory data on the environmental impacts of DAC (using a 100kt/year scenario from Deutz & Bardow1 as a start).  Results are interesting but not included in Figure 1 since only grid-connected systems were assessed.

## 1.2 Cost assessments:

Supplementary Figure 2: Reported results for costs associated with capture/removal of 1 ton CO2 from the atmosphere with low-temperature, solid sorbent technology.

In 2019, Fasihi et al.6 provided a cost model for solid sorbent LT and HT solvent DAC based on an extensive literature review. A hypothetical LT DAC plant with a capacity 360,000 t/year was assessed. Costs were calculated for 2020, 2030, 2040 and 2050, using learning-rates to account for decreases in energy consumption and capital expenditure of the DAC plant. Heat demand for the reference year of 2020 was estimated as 1,750 kWhheat/ton CO2, which is comparable to Terlouw et al.3 and the future scenario of Deutz & Bardow1. The reference for specific electricity consumption is, with 250 kWh in 2020, substantially lower than reported in LCAs. Also, Fasihi et al.6 paid early interest to stand-alone designs of DACs, modelling an energy system using wind power and PV alongside a battery storage system. Their work acknowledged that intermitted RESs reduce the full load hours of the DAC plant. Two designs with 4,000 and 8,000 full load hours per year, corresponding to a used DAC capacity of ca. 165 and 330 kt/year, were assessed. Lower utilization of the DAC with the 4,000 Flh layout increased the contribution of plant annuity costs compared to the 8,000 Flh design, see Supplementary Figure 2. On the other hand, specific energy costs were lower as less overcapacity must be provided to achieve 4,000 full load hours.

Reports from the IEA7 and IEAGHG4 for PV-powered systems have not accounted for reduced full load hours or additional costs associated with an energy storage medium. Learning effects between FOAK and NOAK were included by both IEA7 and IEAGHG4. The IEAGHG4 reports reference cost for a FOAK, powered by PV with a heat pump of $607 (100kt/year DAC) and $560 (1Mt/year DAC). Through learning, costs come down by 60% to $243 and $232 for a NOAK plant. Combined with GWP estimates, IEAGHG4 allows to estimate costs for net CO2 removal (although only carbon capture efficiency, not carbon removal efficiency was calculated by IEAGHG in Supplementary Figure 1). IEA puts average costs for a 1Mt/year LT solid-sorbent FOAK and NOAK system at $310 and $160. Using average CRE calculated by LCAs, but not necessarily for the same system boundary, increases costs.

Cost estimates from the National Academy of Science (NAS)8 use low energy requirements of ca. 1140 kWh in their middle scenario. Surprisingly, specific annuities associated with the capital expenditure of the DAC plant is less than $20. Fasihi et al.6, IEA7 and IEAGHG4 share that adsorbent costs are part of operation costs and cannot easily be divided into contribution of adsorbent material degradation and other operating costs, which would increase transparency and was done by McQueen et al.9 The setup in McQueen et al.9 for a 100kt/year system is only in part comparable to a stand-alone system because electricity is used from the U.S. grid and heat as waste heat from a geothermal power plant.

Publication shown in Supplementary Table 1 and Supplementary Table 2 have served as benchmark for life cycle assessments and cost evaluations of solid-sorbent direct air carbon capture (and storage).

| Study | Technology | Functional unit | Capacity | Locations | Heat demand (capture): kWhth/t | Heat source | Electricity demand (capture) : kWhel/t | Electricity source | Time | LCIA model | Impact categories | Capture or removal | Scaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Deutz & Bardow1 | LT solid sorbent (amine on silica) – Climeworks | FU1: 1 kg CO2 captured, (FU2: 1 kg CO2 removed) | S1: 4 kt/year, S2: 100 kt/year | Iceland, Germany for generic pv-system | S1: 3,300 S2: 1,500 | Heat pump* | S1: 700 S2: 500 | PV* | S1: today S2: future | EF 2.0 | 16 EI from EF 2.0 | Capture is focus, removal estimated | Technological learning from FOAK to NOAK (Changes of energy system only for grid in 2030 or 2050 |
| Terlouw et al.3 | LT solid sorbent - Climeworks | FU: “Gross removal of 1 ton CO2 from the atmosphere via the use of a DAC plant combined with geological storage” | S1: 100kt/year | Jordan (L1), Greece (L2)* | S1: 1,500 | Heat pump* | S1: 500 | PV* (battery storage) | S1: near-future | EF 2.0, ReCiPe16 | 15 EI from EF 2.0, 1 EI from ReCiPe | Removal | Changes of energy system for grid-connected. Also update of background data but no results for stand-alone system. Upscaling from 4kt to 100kt with effect on less specific material for plant |
| Madhu et al.2 | LT solid sorbent – Climeworks, (HT liquid – Carbon Engineering) | FU: “1 ton of CO2 captured and stored” | S1: 50t/year S2: 1Mt/year | Switzerland | S1: 720 S2: 720 (75% heat recovery rate, 2,890 without heat recovery) | Heat pump* | S1: 180 S2: 180 | Low carbon* (ca 60% wind, 35% pv) | S1: today S2: future | ReCiPe16 | 18 EI from ReCiPe | Removal | Same results for S1 and S2. Scaling has effect on liquid system but not solid sorbent. Results are same for S1,S2. |
| Qui et al.5 | LT solid sorbent - Climeworks, (HT liquid - Carbon Engineering) | FU: „Capturing and sequestering one metric ton (1t) of atmospheric CO2 by DACCS technologies“ | S1: 100 kt/year | USA* | 1,500 | Heat pump* | 500 | Grid electricity | 2020 – 2100 (T1: 2020, T2: 2030, T3: 2040, T4: 2050) | ReCiPe16 | 8 EI from ReCiPe | Removal | Technology learning, changes in background energy system |
| IEAGHG4 | LT solid sorbent, (HT liquid) |  | 1 Mt/year |  | S1: 3,000 S2: 1,360 (both scaled from 100kt/year literature system with scaling factor of 1.1) | Heat pump* | S1: 640 S2: 440 (both scaled from 100kt/year literature system with scaling factor of 1.1) | PV* | S1: today S2: 2050 (but could be earlier) | not specified | GHG | Capture | Technology learning FOAK, NOAK. Update of CO2 intensity of pv electricity (from 50.9kg CO2 / MWh in 2020 to 24.8 kg CO2/MWh in 2050 NOAK equivalent to 5-7 doublings of large-scale production capacity |

Supplementary Table 1: Publications of life cycle assessments included in benchmark review.

*Other combinations have also been evaluated within the study.

Supplementary Table 2: Publications of cost calculation included in benchmark review.

| Study | Technology | Reference unit | Capacity | Locations | Heat demand (capture): kWhth/t | Heat source | Electricity demand (capture) : kWhel/t | Electricity source | Time | Cost method | Capture or removal | Scaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fasihi et al.6 | LT solid sorbent, (HT liquid sorbent) | 1 ton CO2 (gross) captured | 360,000 C1: 4,000 Flh C2: 8,000 Flh | Morocco | S1: 1750 S2: 1500 S3: 1286 S4: 1102 | Heat pump* | S1: 250 S2: 225 S3: 203 S4: 182 | Wind, PV* | S1: 2020, S2: 2030, S3: 2040, S4: 2050 | Levelized cost of CO2 DAC (LCOD) | Capture | Learning applied to CAPEX, deployment of DAC based on literature, with DAC capacity of 15Gt/year in 2050. Learning applied to energy demand. Cost decline in RES modelled. |
| NAS8 | LT solid sorbent, (HT liquid sorbent) | 1 Mt CO2 captured and 1 Mt CO2 net-captured (CCE calculated based on emission factors for energy) | 1,000,000 | USA | S2: 940 S4: 1333 | Solar* | S2: 155 S4: 315 | Solar* |  | “LCOD” (annualized capex + opex) | Capture | No scaling? |
| McQueen et al.9 | LT solid sorbent | 1 ton CO2 captured and 1 ton CO2 net-captured | 100,000 | USA | 1666 | Waste heat geothermal* | 416 | U.S. Grid |  |  | Capture | Learning by doing (Capex scaled up from 10kt/year with Ci = C0 * (Pi/P0)^(2/3) |
| IEAGHG4 | LT solid sorbent, (HT liquid) |  | C1: 100 kt/year C2: 1 Mt/year |  | S1: 3,000 S2: 1,360 (both scaled from 100kt/year literature system with scaling factor of 1.1) | Heat pump* | S1: 640 S2: 440 (both scaled from 100kt/year literature system with scaling factor of 1.1) | PV* | S1: today S2: 2050 (but could be earlier) | LCOD | Removal (emissions only for capture) | Technology learning FOAK, NOAK. Update of CO2 intensity of pv electricity (from 50.9kg CO2 / MWh in 2020 to 24.8 kg CO2/MWh in 2050 NOAK equivalent to 5-7 doublings of large-scale production capacity |
| IEA7 | LT solid sorbent, (HT liquid solvent) | 1 ton CO2 gross captured | 1 Mt/year |  | 1500 - 2110 | Heat pump* | 400 - 660 | PV* | S1: today S2: NOAK | LCOD | Removal | Technology learning from FOAK to NOAK. R&D, learning by doing and economies of scale as key components |

*Other combinations have also been evaluated within the study, **conversion rate USD:€ of 1.1:1.

# 2. Life cycle inventory data

Supplementary Figure 1 shows the model graph of the DACS system as implemented in OpenLCA, in alignment with the system boundary shown in Supplementary Figure 1 of the manuscript. Life cycle inventory data for the DAC plant was taken from ref.3, which have provided a public proxy for a low-temperature, solid sorbent direct air capture system based on proprietary data from Climeworks which are not available publicly. While caution should be taken with the simplified infrastructure modelling for the DAC system, internal comparison by ref.3 showed alignment between the simplified LCI data and the original data from Climeworks. The PEI adsorbent material was modelled as monomethylamine, following the approach of ref.2.

Supplementary Figure 3: Model graph LCA and data sources (Terlouw et al.3, Ecoinvent 3.810, Nordin et al.11, Madhu et al.2 and Crenna et al.12). Energy input data for reference scenario.

Supplementary Figure 4: Model graph transportation and storage, implemented in OpenLCA13.

Supplementary Figure 5: Model graph adsorbent material, implemented in OpenLCA13.

Supplementary Figure 6: Model graph direct air capture plant and compressor, implemented in OpenLCA13.

Supplementary Figure 7: Model graph photovoltaic system, battery storage and heat pump, implemented in OpenLCA13

# Supplementary References

1	Deutz, S. & Bardow, A. Life-cycle assessment of an industrial direct air capture process based on temperature–vacuum swing adsorption. Nat. Energy 6, 203-213, doi:10.1038/s41560-020-00771-9 (2021).

2	Madhu, K., Pauliuk, S., Dhathri, S. & Creutzig, F. Understanding environmental trade-offs and resource demand of direct air capture technologies through comparative life-cycle assessment. Nat. Energy, doi:10.1038/s41560-021-00922-6 (2021).

3	Terlouw, T., Treyer, K., Bauer, C. & Mazzotti, M. Life Cycle Assessment of Direct Air Carbon Capture and Storage with Low-Carbon Energy Sources. Environ. Sci. Technol. 55, 11397-11411, doi:10.1021/acs.est.1c03263 (2021).

4	IEAGHG. Global Assessment of Direct Air Capture Costs. (2021).

5	Qiu, Y. et al. Environmental trade-offs of direct air capture technologies in climate change mitigation toward 2100. Nature Communications 13, 3635, doi:10.1038/s41467-022-31146-1 (2022).

6	Fasihi, M., Efimova, O. & Breyer, C. Techno-economic assessment of CO 2 direct air capture plants. J. Clean. Prod. 224, 957-980, doi:10.1016/j.jclepro.2019.03.086 (2019).

7	IEA. Direct Air Capture 2022. (IEA, Paris, 2022).

8	National Academies of Sciences Engineering & Medicine. Negative Emissions Technologies and Reliable Sequestration: A Research Agenda.  (The National Academies Press, 2019).

9	McQueen, N. et al. Cost Analysis of Direct Air Capture and Sequestration Coupled to Low-Carbon Thermal Energy in the United States. Environmental Science & Technology 54, 7542-7551, doi:10.1021/acs.est.0c00476 (2020).

10	Wernet, G. et al. The ecoinvent database version 3 (part I): overview and methodology. Int. J. Life Cycle Assess. 21, 1218-1230, doi:10.1007/s11367-016-1087-8 (2016).

11	Mohd Nordin, A. H., Sulaiman, S. I., Shaari, S. & Mustapa, R. F. Energy and environmental impacts of a 37.57 MW dc ground-mounted large-scale photovoltaic system in Malaysia: A life-cycle approach. J. Clean. Prod. 335, 130326, doi:https://doi.org/10.1016/j.jclepro.2021.130326 (2022).

12	Crenna, E., Gauch, M., Widmer, R., Wäger, P. & Hischier, R. Towards more flexibility and transparency in life cycle inventories for Lithium-ion batteries. Resources, Conservation and Recycling 170, 105619, doi:10.1016/j.resconrec.2021.105619 (2021).

13	Ciroth, A. ICT for environment in life cycle applications openLCA—A new open source software for life cycle assessment. The international journal of life cycle assessment 12, 209-210 (2007).

14	Sala, S. C., E.; Secchi, M.; Pant R. Global normalisation factors for the Environmental Footprint and Life Cycle Assessment.  (Publication Office of the European Union, 2017).