Embodied carbon emissions of aluminum-containing commodities in international trade: China’s perspective

Qiangfeng Lia, Huabo Duana*, Tianjiao Li b, Yanjing Zhou b, Ying Chen c*, Ruoyu Zhongd

a College of Civil Engineering, Shenzhen University, Shenzhen, 518060, China

b Research Center for Strategy of Global Mineral Resources, Chinese Academy of Geological Sciences, Beijing, 100037, China

c Center of Solid Waste and Chemical Management Technology, Ministry of Ecology and Environmental of the People's Republic of China, Beijing 100029, China

d China Center for Special Economic Zone Research, Shenzhen University, Shenzhen, 518060, China

*Co-correspondence to:

Huabo Duan: College of Civil Engineering, Shenzhen University, Shenzhen, 518060, China. Tel: 0755-86674644, 18682368069 (Mobile), Email address: huabo@szu.edu.cn.

Ying Chen: Solid Waste and Chemicals Management Technology Center, Ministry of Environmental Protection, Beijing, 100029, China. Email address: chenying@meescc.cn.

Supplementary materials

Calculation method and data sources

(1). Ore mining

The carbon emission coefficient of bauxite can be calculated by Eq. (4).

E1=(E1A*θ1+E1B*θ2)*Ece,#(4)

where E1 is the carbon emission coefficient of bauxite, in the unit of kgCO2/t; E1A and E1B are the energy intensity of bauxite mined by opencast mining and underground mining, respectively, in the unit of kgce/t; Ece is the carbon emission coefficient of standard coal, which is 2.7725kgCO2/kgce, calculated according to the IPCC National Greenhouse Gas Emissions Inventory 2006 (Lubetsky et al. 2006); θ1 and θ2 are the proportion of bauxite mined in opencast mining and underground mining, respectively. Bauxite mining in China includes opencast mining and underground mining, and the opencast mining bauxite account for 93.02% of the domestic total bauxite. China’s imported bauxite mainly comes from Australia, Guinea, Malaysia, India (Pan et al. 2019), and bauxite mining in these countries are all opencast mining. Combined with the energy intensity and related technical indicators for bauxite in Schedule 3, the carbon emission coefficient of the bauxite produced in China is 38.36kgCO2/t, and the carbon emission coefficient of the imported bauxite ore is 36.04kgCO2/t.

(2). Alumina production

At present, the production of alumina are mainly Bayer process (major), Sintering process and Bayer-sintering combination process. The quality of bauxite produced in China is relatively poor (the bauxite has a low aluminum to silicon ratio, and there are a lot of silicon, sulfur, iron, organics, etc. that are harmful elements or compounds for alumina production in bauxite). If harmful elements or compounds are effectively removed before smelting, it can effectively reduce and save production costs in the alumina production process (Feng et al. 2016). Therefore, before smelting, the gangue minerals, harmful elements and compounds contained in the bauxite need to be removed through the beneficiation process.

The carbon emission coefficient of aluminum concentrate can be calculated by Eq. (5).

E2=E2A*Ece+1*γ2γ1*ρ1*E1,#(5)

where E2 is the carbon emission coefficient of aluminum concentrate, in the unit of kgCO2/t; E2A is the energy intensity of aluminum concentrate, in the unit of kgce/t; Ece is the carbon emission coefficient of standard coal; γ1 is the percentage of Al2O3 content in bauxite; γ2 refers to the percentage of Al2O3 content in aluminum concentrate; ρ1 is the recovery ratio of metal in beneficiation process. Combined with the energy intensity and related technical indicators for aluminum concentrate in Schedule 3, the carbon emission coefficient of the aluminum concentrate produced in China is 58.78kgCO2/t. China’s imported bauxite does not need to be beneficiated.

The carbon emission coefficient of alumina can be calculated by Eq. (6).

E3=E3A*Ece+1*γ3γ2*ρ2*E2,#(6)

where E3 is the carbon emission coefficient of alumina, in the unit of kgCO2/t; E3A is the energy intensity of alumina, in the unit of kgce/t; Ece is the carbon emission coefficient of standard coal; γ2 is the percentage of Al2O3 content in aluminum concentrate; γ3 is the percentage of Al2O3 content in alumina; ρ2 is the recovery ratio of metal in alumina production process. According to the data of "China Nonferrous Metals Industry Yearbook" (CNMIA 2009, 2012, 2015, 2018a), the ratio of China's domestic bauxite to imported bauxite is about 13:12 (Schedule 4). Combined with the energy intensity  and related technical indicators for alumina in Schedule 3, the carbon emission coefficient of the exported alumina is 1731.91～1838.48 kgCO2/t, while the carbon emission coefficient of the imported alumina is 1181.09kgCO2/t.

(3). Unwrought aluminum production

The basic principle of unwrought aluminum (primary aluminum or electrolytic aluminum) production is that solid alumina is dissolved in the electrolyte melt, and under the action of direct current, an electrochemical reaction takes place on the cathode and anode. As the reaction continues, the alumina and carbon anodes in the electrolyte melt are continuously consumed and replenished. The aluminum liquid is continuously precipitated in the cathode area, and the anode gas is continuously generated in the anode area (CEPA 2006). The reaction formula is as follows:

2Al2O3 (liquid) +3C (solid) =4Al (liquid) +3CO2 (gas) ,#(7)

The carbon emission coefficient of UA can be calculated by Eq. (8).

E4=E4A*Eee*Ece+ω1*E3+ω2*E4B+ω3*E4C+ω4*E4D+ω4γ,#(8)

where E4 is the carbon emission coefficient of UA, in the unit of kgCO2/t; E4A is the electricity consumption intensity of UA, in the unit of kwh/t; Eee is the carbon emission coefficient of electricity, which is 0.1229kgce/kwh according to the “Aluminum industry standard conditions” (MIAITC 2013); Ece is the carbon emission coefficient of standard coal; ω1，ω2，ω3 and ω4 are the material consumption of alumina, aluminum fluoride, cryolite, and carbon anode for the production of 1 ton of UA, respectively; E3 is the carbon emission coefficient of alumina, in the unit of kgCO2/t; E4B、E4C and E4D are the carbon emission coefficient of aluminum fluoride, cryolite, and carbon anode (The values of E4B、E4C and E4D are 5.701kgCO2/kg, 0.5947kgCO2/ kg and 4.226kgCO2/ kg, respectively, shown in Schedule 5). In addition, during the production of UA, carbon anodes continuously undergo chemical reactions and produce carbon dioxide. Therefore, the calculation of the carbon emission coefficient of UA must not only consider the carbon emissions from the energy intensity of the process and the energy consumed by the production of raw materials, but also the carbon dioxide generated by the chemical reaction of the carbon anode. γ is the molecular mass ratio of C and CO2 (12/44). Combined with the energy intensity and related technical indicators for UA in Schedule 3, the carbon emission coefficient of the exported UA is 10293.50～11107.12kgCO2/t, while the carbon emission coefficient of the imported UA is 9109.71kgCO2/t.

(4). Aluminum old scraps production

The end of the aluminum life cycle does not mean that aluminum is completely trashed and discarded. Most ACC still have good value after the end of the life cycle (Shinzato and Hypolito 2005). In order to make full use of waste aluminum resources and improve resource supply capacity, China is vigorously developing the recycled aluminum industry (Yang et al. 2011). The carbon emission coefficient of AOS can be calculated by Eq. (9).

E5=E5A*Ece,#(9)

where E5 is the carbon emission coefficient of AOS, in the unit of kgCO2/t; E5A is the energy intensity of AOS production, in the unit of kgce/t; Ece is the carbon emission coefficient of standard coal. Combined with the energy intensity and related technical indicators for AOS in Schedule 3, the carbon emission coefficient of the exported UA is 415.88kgCO2/t, while the carbon emission coefficient of the imported UA is 360.43kgCO2/t.

(5). Semi-products production

Semi-products production process mainly includes casting, rolling, extrusion and other processes. Casting is the first process of semi-products production process, providing aluminum ingots for the processing workshop through melting and casting (Chen et al. 2009). The carbon emission coefficient of aluminum ingots can be calculated by Eq. (10).

E6=E6A*Ece+E4*λ+E5*1-λ1-η1,#(10)

where E6 is the carbon emission coefficient of aluminum ingots, in the unit of kgCO2/t; E6A is the energy intensity of aluminum ingots. Ece is the carbon emission coefficient of standard coal.  E4 and E5 are the carbon emission coefficient of UA and AOS; η1 is the loss rate of aluminum during the processing of aluminum ingots; λ is the proportion of UA output in total aluminum output (UA + AOS) (Schedule 6). Combined with the energy intensity and related technical indicators in Schedule 3 in Supplementary materials, the carbon emission coefficient of the exported aluminum ingots is 9587.54～10306.95kgCO2/t, while the carbon emission coefficient of the imported aluminum ingots is 8015.54kgCO2/t.

Due to different processing techniques, aluminum ingots are further processed into various types of SP, including cold-rolled sheets, hot-rolled sheets, cold-rolled strips, hot-rolled strips, foils, profiles, and tubes. The carbon emission coefficient of SP can be calculated by Eq. (11).

E7i=E7Ai*Ece+E61-η2,#(11)

where E7i is the carbon emission coefficient of SP, in the unit of kgCO2/t; E7Ai is the energy intensity of various types of SP. Ece the carbon emission coefficient of standard coal.  E6 is the carbon emission coefficient of aluminum ingots, in the unit of kgCO2/t; η2 is the loss rate of aluminum during the processing of SP. Combined with the energy intensity  and related technical indicators of various types of SP in Schedule 3, the carbon emission coefficients of the exported and imported SP are shown in Schedule 9.

(6). Aluminum-containing end products production

There are various types of ACEP, including vehicles, machinery, ships, etc. Thousands of ACEP made of SP must have different comprehensive. The workload of calculating the energy intensity of all ACEP is too large, and it is impossible to collect all data. Therefore, it is assumed that the carbon emission coefficient of pure aluminum in ACEP is equal to the carbon emission coefficient of pure aluminum in SP.

(7). Summary

The carbon emission coefficients of all kinds of ACC were calculated (Table 2). However, the aluminum material flow of China’s international trades was calculated based on pure aluminum. The carbon emission coefficient of pure aluminum materials was determined by the aluminum content of ACC, as shown in Table 2.

Schedules

Schedule 1. The aluminum content coefficients of different types of imported and exported ACC.

| Categories | Al content (%) | Coefficient description |
| --- | --- | --- |
| Bauxite | 31.76 | CNMIA 2018b |
| Alumina |  |  |
| Aluminum oxide | 52.09 | MOEPC 2009 |
| Aluminum hydroxide | 34.62 | According to the molecular mass of the aluminum hydroxide |
| Aluminum fluoride | 32.14 | According to the molecular mass of the aluminum fluoride |
| Aluminum chloride | 20.45 | According to the molecular mass of the aluminum chloride |
| Aluminum sulfate | 36.00 | According to the molecular mass of the aluminum sulfate |
| UA | 99.30 | SEROCSP 2004 |
| AOS |  |  |
| Aluminum waste and scrap | 45.00 | Xiong et al. 2005 |
| Aluminum powders | 45.00 |  |
| Aluminum slag and ash | 45.00 |  |
| SP |  |  |
| Aluminum profiles | 99.30 | SEROCSP 2004 |
| Aluminum strip | 99.30 |  |
| Aluminum plates | 99.30 |  |
| Aluminum foils | 99.30 |  |
| Aluminum tubes and pipes | 99.30 |  |
| ACEP |  |  |
| Transportation equipment |  |  |
| Light Motor vehicles | 98 kg/unit | Recalde et al. 2008 |
| Medium/ large motor truck/trailers | 150 kg/unit |  |
| Buses and coaches | 200 kg/unit |  |
| Motorcycles/snowmobiles/ATVs | 10 kg/unit |  |
| Bicycles | 20% |  |
| Aircraft | 80% |  |
| Marine vessels | 5% |  |
| Railroad vehicles | 1% |  |
| Automotive tools | 5% |  |
| Electronic and electrical equipment |  | Recalde et al. 2008 |
| Computers | 5% |  |
| Televisions | 1% |  |
| Telephones | 1% |  |
| Stereos/radios | 2% |  |
| Copiers | 5 kg/unit |  |
| Fax machines | 1% |  |
| Industrial equipment |  | Recalde et al. 2008 |
| Agriculture equipment | 1% |  |
| Forestry equipment | 1% |  |
| Forklifts equipment | 30 kg/unit |  |
| Manufacturing equipment | 0.5% |  |
| Boilers | 0.5% |  |
| Industrial vehicles | 0.5% |  |
| Cranes | 0.5% |  |
| Tools | 0.2% |  |
| Household appliances |  |  |
| Refrigerators/freezers | 1.7 kg/unit | Recalde et al. 2008 |
| Washing machines | 1.9 kg/unit |  |
| Clothes dryers | 1.4 kg/unit |  |
| Dishwashers | 1.4 kg/unit |  |
| Stoves/ovens/steamers | 1.8 kg/unit |  |
| Microwave ovens | 0.5 kg/unit |  |
| Portable air conditioners | 2.5 kg/unit |  |
| Ice machines | 2% |  |
| Sewing machines | 25% |  |
| Lawn mowers | 1% |  |
| Snow blowers | 1% |  |
| Building and Infrastructure |  |  |
| Aluminum structures/doors | 3 kg/m2 | Recalde et al. 2008 |
| Aluminum cables | 4.05 kg/km | Calculated according to the data from MIAITC 2016 |
| Packaging materials & other parts |  |  |
| Aluminum containers | 6% | SEROCSP 2004 |
| Aluminum nails/tacks/staples | 6% |  |
| Aluminum netting/fencing | 6% |  |
| Other aluminum parts | 6% |  |

Schedule 2. Trade codes of Aluminum-containing commodities.

Bauxite 260600

Alumina 281810, 281820, 281830, 282612, 282732, 283322

Unwrought aluminum 760110, 760120

Aluminum old scrap 760200, 760310, 760320, 262040

Semi-products 760410, 760429, 760511, 760519, 760521, 760529, 760611, 760612, 760691, 760692, 760711, 760719, 760720, 760810, 760820, 760900

Aluminum-containing end products 761010, 761090, 761100, 761210, 761290, 761300, 761410, 761490, 761511, 761519, 761520, 761610, 761690, 761691, 761699, 840211, 840212, 840219, 840220, 840290, 840310, 840390, 840410, 840420, 840490, 840510, 840590, 840610, 840611, 840619, 840681, 840682, 840690, 840710, 840721, 840729, 840731, 840732, 840733, 840734, 840790, 840810, 840820, 840890, 840910, 840991, 840999, 841011, 841012, 841013, 841090, 841111, 841112, 841121, 841122, 841181, 841182, 841191, 841199, 841210, 841221, 841229, 841231, 841239, 841280, 841290, 841311, 841319, 841320, 841330, 841340, 841350, 841360, 841370, 841381, 841382, 841391, 841392, 841410, 841420, 841430, 841440, 841510, 841520, 841581, 841582, 841583, 841590, 841610, 841620, 841630, 841690, 841710, 841720, 841780, 841790, 841810, 841821, 841822, 841829, 841830, 841840, 841850, 841861, 841869, 841891, 841899, 841931, 841932, 841939, 841940, 841950, 841960, 841981, 841989, 841990, 842010, 842091, 842099, 842111, 842112, 842119, 842121, 842122, 842123, 842129, 842131, 842139, 842191, 842199, 842211, 842219, 842230, 842240, 842290, 842430, 842441, 842449, 842481, 842482, 842489, 842490, 842511, 842519, 842520, 842531, 842539, 842541, 842542, 842549, 842611, 842612, 842619, 842620, 842630, 842641, 842649, 842691, 842699, 842710, 842720, 842790, 842810, 842820, 842831, 842832, 842833, 842839, 842850, 842860, 842890, 842911, 842919, 842920, 842930, 842940, 842951, 842952, 842959, 843010, 843020, 843031, 843039, 843041, 843049, 843050, 843061, 843062, 843069, 843131, 843139, 843141, 843149, 843230, 843231, 843239, 843240, 843241, 843242, 843280, 843290, 843311, 843319, 843320, 843330, 843340, 843351, 843352, 843353, 843359, 843360, 843390, 843410, 843420, 843490, 843510, 843590, 843610, 843621, 843629, 843680, 843691, 843699, 843710, 843780, 843790, 843810, 843820, 843830, 843840, 843850, 843860, 843880, 843890, 843910, 843920, 843991, 843999, 844010, 844090, 844110, 844120, 844130, 844140, 844180, 844190, 844210, 844220, 844230, 844240, 844250, 844311, 844312, 844313, 844314, 844315, 844316, 844317, 844319, 844321, 844329, 844330, 844331, 844332, 844339, 844340, 844350, 844351, 844359, 844360, 844390, 844391, 844399, 844400, 844511, 844512, 844513, 844519, 844520, 844530, 844540, 844590, 844610, 844621, 844629, 844630, 844711, 844712, 844720, 844790, 844811, 844819, 844820, 844831, 844832, 844833, 844839, 844841, 844842, 844849, 844851, 844859, 844900, 845011, 845012, 845019, 845020, 845090, 845110, 845121, 845129, 845130, 845140, 845150, 845180, 845190, 845210, 845221, 845229, 845230, 845240, 845290, 845310, 845320, 845380, 845390, 845410, 845420, 845430, 845490, 845510, 845521, 845522, 845530, 845590, 845610, 845611, 845612, 845620, 845630, 845640, 845650, 845690, 845691, 845699, 845710, 845720, 845730, 845811, 845819, 845891, 845899, 845910, 845921, 845929, 845931, 845939, 845940, 845941, 845949, 845951, 845959, 845961, 845969, 845970, 846011, 846012, 846019, 846021, 846022, 846023, 846024, 846029, 846031, 846039, 846040, 846090, 846110, 846120, 846130, 846140, 846150, 846190, 846210, 846221, 846229, 846231, 846239, 846241, 846249, 846291, 846299, 846310, 846320, 846330, 846390, 846410, 846420, 846490, 846510, 846520, 846591, 846592, 846593, 846594, 846595, 846596, 846599, 846610, 846620, 846630, 846691, 846692, 846693, 846694, 846711, 846719, 846721, 846722, 846729, 846781, 846789, 846791, 846792, 846799, 846810, 846820, 846880, 846890, 846900, 846910, 846911, 846912, 846920, 846921, 846929, 846930, 846931, 846939, 847010, 847021, 847029, 847030, 847040, 847050, 847090, 847110, 847120, 847130, 847141, 847149, 847150, 847160, 847170, 847180, 847190, 847191, 847192, 847193, 847199, 847210, 847220, 847230, 847290, 847310, 847321, 847329, 847330, 847340, 847350, 847410, 847420, 847431, 847432, 847439, 847490, 847510, 847520, 847521, 847529, 847590, 847611, 847619, 847621, 847629, 847681, 847689, 847690, 847710, 847720, 847730, 847740, 847751, 847759, 847780, 847790, 847810, 847890, 847910, 847920, 847930, 847940, 847950, 847960, 847971, 847979, 847981, 847982, 847989, 847990, 848010, 848020, 848030, 848041, 848049, 848050, 848060, 848071, 848079, 848110, 848120, 848130, 848140, 848180, 848190, 848210, 848220, 848230, 848240, 848250, 848280, 848291, 848299, 848310, 848320, 848330, 848340, 848350, 848360, 848390, 848410, 848420, 848490, 848510, 848610, 848620, 848630, 848640, 848690, 848710, 850110, 850120, 850131, 850132, 850133, 850134, 850140, 850151, 850152, 850153, 850161, 850162, 850163, 850164, 850211, 850212, 850213, 850220, 850230, 850231, 850239, 850240, 850300, 851410, 851420, 851430, 851440, 851490, 851511, 851519, 851521, 851529, 851531, 851539, 851580, 851590, 851650, 851660, 851710, 851711, 851712, 851718, 851719, 851720, 851721, 851722, 851730, 851740, 851750, 851790, 851810, 851821, 851822, 851829, 851830, 851840, 851850, 851890, 851910, 851920, 851921, 851929, 851930, 851931, 851939, 851940, 851950, 851981, 851989, 851991, 851992, 851993, 851999, 852010, 852020, 852031, 852032, 852033, 852039, 852090, 852110, 852190, 852210, 852290, 852530, 852711, 852712, 852713, 852719, 852721, 852729, 852731, 852732, 852739, 852790, 852791, 852792, 852799, 852810, 852812, 852813, 852820, 852821, 852822, 852830, 852861, 852862, 852869, 852871, 852872, 852873, 852910, 852990, 853710, 853720, 853810, 853890, 860110, 860120, 860210, 860290, 860310, 860390, 860400, 860500, 860610, 860620, 860630, 860691, 860692, 860699, 860711, 860712, 860719, 860721, 860729, 860730, 860791, 860799, 860800, 870110, 870120, 870130, 870190, 870191, 870192, 870193, 870194, 870195, 870210, 870220, 870230, 870240, 870290, 870310, 870321, 870322, 870323, 870324, 870331, 870332, 870333, 870340, 870350, 870360, 870370, 870380, 870390, 870410, 870421, 870422, 870423, 870431, 870432, 870490, 870510, 870520, 870530, 870540, 870590, 870710, 870790, 870810, 870821, 870829, 870830, 870831, 870839, 870840, 870850, 870860, 870870, 870880, 870891, 870892, 870893, 870894, 870895, 870899, 870911, 870919, 870990, 871110, 871120, 871130, 871140, 871150, 871160, 871190, 871200, 871491, 871492, 871493, 871494, 871495, 871496, 871499, 871610, 871620, 871631, 871640, 871680, 871690, 880100, 880110, 880190, 880211, 880212, 880220, 880230, 880240, 880250, 880260, 880310, 880320, 880330, 880390, 880400, 880510, 880520, 880521, 880529, 890110, 890120, 890130, 890190, 890200, 890310, 890391, 890392, 890399, 890400, 890510, 890520, 890590, 890600, 890610, 890690, 890710, 890790, 890800

Schedule 3. The process energy intensity, technical and economic indicators, and data sources of various ACC

| Process |  | Energy | Units | Export | Import | Data sources |
| --- | --- | --- | --- | --- | --- | --- |
| Ore mining | Opencast mining | Standard coal | kgce/t | 13 | 13 | MIAITC 2013; IAI 2007 |
|  | Underground mining | Standard coal | kgce/t | 25 | — | MIAITC 2013 |
|  | Opencast mining bauxite account for 93.02% of the domestic total bauxite |  |  |  | — | Feng et al. 2016 |
|  | Percentage of Al2O3 content in bauxite |  |  | 60% | 60% | CNMIA 2018b |
|  | Comprehensive energy intensity | Standard coal | kgce/t | 13.84 | 13 | IAI 2007 |
| Beneficiation | Comprehensive energy intensity | Standard coal | kgce/t | 4.92 | — | CQM 2013 |
|  | Percentage of Al2O3 content in aluminum concentrate |  |  | 60% | 60% | CNMIA 2018b |
|  | Recovery ratio of metal |  |  | 0.85 | — | Feng et al. 2016 |
| Alumina production | Bayer process | Standard coal | kgce/t | 500-520 | 400 | MOEPC 2009 |
|  |  | Recovery ratio of metal |  | 81% | 82% | MOEPC 2009 |
|  | Sintering process | Standard coal | kgce/t | 1900 | — | MIAITC 2013 |
|  |  | Recovery ratio of metal |  | 85% | — | MIAITC 2013 |
|  | Bayer-sintering combination process | Standard coal | kgce/t | 900 | — | MOEPC 2009 |
|  |  | Recovery ratio of metal |  | 90% | — | MOEPC 2009 |
|  | Ratio of alumina production by Bayer process, Sintering process and Combination process. |  |  | 18:1:1 | 1:0:0 | Li and Gu 2011 |
|  | Comprehensive energy intensity | Standard coal | kgce/t | 590-608 | 400 |  |
|  | Recovery ratio of metal |  |  | 81.65% | 82% |  |
|  | Percentage of Al2O3 content in alumina |  |  | 98.4% | 98.4% | CNSMC 2009 |
| Unwrought aluminum production | Electricity consumption |  | kwh/t* | 14700-15400 | 14500 | CEPA 2006 |
|  | Alumina consumption |  | kg/t | 1930-1940 | 1930 | CEPA 2006 |
|  | Aluminum fluoride consumption |  | kg/t | 23-28 | 22 | CEPA 2006 |
|  | Cryolite consumption |  | kg/t | 5 | 4 | CEPA 2006 |
|  | Carbon anode consumption |  | kg/t | 420-500 | 410 | CEPA 2006 |
| Aluminum old scraps production | Comprehensive energy intensity | Standard coal | kgce/t | 150 | 130 | MIAITC 2013 |
| Semi-products production | Casting process | Standard coal | kgce/t | 185-205 | 95-105 | MIAITC 2017a |
|  | Cold rolled sheet process | Standard coal | kgce/t | 155-170 | 140-155 | MIAITC 2017b |
|  | Hot rolled sheet process | Standard coal | kgce/t | 125-135 | 100-110 | MIAITC 2017b |
|  | Cold rolled strip process | Standard coal | kgce/t | 90-105 | 75-90 | MIAITC 2017b |
|  | Hot rolled strip process | Standard coal | kgce/t | 120 | 95 | MIAITC 2017b |
|  | Foil rolling process | Standard coal | kgce/t | 170-230 | 135-195 | MIAITC 2017c |
|  | Profile rolling process | Standard coal | kgce/t | 210-510 | 180-460 | MIAITC 2017d |
|  | Tube rolling process | Standard coal | kgce/t | 280-580 | 190-530 | MIAITC 2017d |
|  | The loss rate of aluminum in the casting process is 5%, and the loss rate of aluminum in the processing of other aluminum materials is 2.4% |  |  |  |  | Chen et al. 2009 |

Note: 1 kWh of electricity equals 0.1229 kg of standard coal (MIAITC 2013)

Schedule 4. China's domestic and imported bauxite from 2008-2016 (unit: Tg)

| Year | Domestic bauxite | Imported bauxite | Date source |
| --- | --- | --- | --- |
| 2008 | 25.18 | 25.93 | CNMIA 2009 |
| 2009 | 29.21 | 19.80 | CNMIA 2009 |
| 2010 | 36.83 | 30.36 | CNMIA 2012 |
| 2011 | 37.17 | 45.24 | CNMIA 2012 |
| 2012 | 44.05 | 40.07 | CNMIA 2012 |
| 2013 | 50.34 | 71.61 | CNMIA 2015 |
| 2014 | 59.21 | 36.53 | CNMIA 2015 |
| 2015 | 60.79 | 56.10 | CNMIA 2015 |
| 2016 | 66.16 | 52.05 | CNMIA 2018a |
| Total | 408.95 | 377.69 |  |
| Share | 52.0% | 48.0% |  |

Schedule 5. Carbon emission coefficient of aluminum fluoride, cryolite, and carbon anode.

| Category | Calculation boundary | Country | Updated | Database | Carbon emission coefficient |
| --- | --- | --- | --- | --- | --- |
| Aluminum fluoride | All materials from resource mining to delivery | China | 2013/2/9 | CLCD-China-ECER 0.8.1 2012 | 5.701kgCO2eq/kg |
| Cryolite |  | China | 2013/2/9 | CLCD-China-ECER 0.8.1 2012 | 0.5947kgCO3eq/kg |
| Carbon anode |  | China | 2013/2/9 | CLCD-China-ECER 0.8.1 2012 | 4.226kgCO4eq/kg |

Schedule 6. Production of UA and AOS in China and the world from 2008–2016 (unit: Tg)

| Year | China |  | Globe |  |  |
| --- | --- | --- | --- | --- | --- |
|  | UA | AOS | UA | AOS | Data soure |
| 2008 | 13.16 | 2.75 | 39.49 | 10.83 | CNMIA 2009 |
| 2009 | 12.99 | 3.11 | 36.99 | 9.90 | CNMIA 2009 |
| 2010 | 15.52 | 4.00 | 41.15 | 10.71 | CNMIA 2012 |
| 2011 | 17.45 | 4.40 | 43.99 | 12.02 | CNMIA 2012 |
| 2012 | 19.72 | 4.83 | 46.21 | 12.69 | CNMIA 2012 |
| 2013 | 21.94 | 5.27 | 47.97 | 13.22 | CNMIA 2015 |
| 2014 | 23.94 | 5.65 | 50.78 | 13.88 | CNMIA 2015 |
| 2015 | 31.30 | 5.77 | 56.43 | 13.71 | CNMIA 2015 |
| 2016 | 31.65 | 6.30 | 58.17 | 14.40 | CNMIA 2018a |
| Total | 187.67 | 36.30 | 421.17 | 111.37 |  |
| Share | 84% | 16% | 79% | 21% |  |

Schedule 7. ECE of ACEP in international trade (import)

| No. | Country/region | ECE of ACEP (Tg) | % of ACC's ECE | Type of ACEP | ECE (Tg) |
| --- | --- | --- | --- | --- | --- |
| 1 | USA | 45.57 | 99.4% |  |  |
|  |  |  |  | Light Motor vehicles | 41.1 |
|  |  |  |  | Medium and large Motor truck/trailers | 2.0 |
|  |  |  |  | Aircraft | 0.9 |
| 2 | Japan | 32.04 | 96.5% |  |  |
|  |  |  |  | Light Motor vehicles | 26.2 |
|  |  |  |  | Medium and large Motor truck/trailers | 2.3 |
|  |  |  |  | Manufacturing equipment | 1.8 |
| 3 | Germany | 13.10 | 98.2% |  |  |
|  |  |  |  | Light Motor vehicles | 6.9 |
|  |  |  |  | Medium and large Motor truck/trailers | 2.7 |
|  |  |  |  | Manufacturing equipment | 1.9 |
| 4 | United Kingdom | 4.61 | 98.8% |  |  |
|  |  |  |  | Light Motor vehicles | 3.4 |
|  |  |  |  | Medium and large Motor truck/trailers | 0.9 |
|  |  |  |  | Tools | 0.2 |
| 5 | Rep. of Korea | 3.48 | 80.0% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 1.0 |
|  |  |  |  | Manufacturing equipment | 0.9 |
|  |  |  |  | Light Motor vehicles | 0.7 |
| 6 | France | 2.45 | 97.8% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 1.3 |
|  |  |  |  | Aircraft | 0.8 |
|  |  |  |  | Manufacturing equipment | 0.1 |
| 7 | Italy | 1.77 | 98.1% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 0.7 |
|  |  |  |  | Manufacturing equipment | 0.4 |
|  |  |  |  | Aircraft | 0.2 |
| 8 | Finland | 1.69 | 99.9% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 1.6 |
|  |  |  |  | Manufacturing equipment | 0.1 |
|  |  |  |  | Tools | 0.0 |
| 9 | Other Asia, nes | 1.18 | 68.3% |  |  |
|  |  |  |  | Manufacturing equipment | 0.7 |
|  |  |  |  | Tools | 0.1 |
|  |  |  |  | Stoves/ovens/steamers | 0.1 |
| 10 | Sweden | 1.04 | 99.6% |  |  |
|  |  |  |  | Light Motor vehicles | 0.5 |
|  |  |  |  | Medium and large Motor truck/trailers | 0.3 |
|  |  |  |  | Manufacturing equipment | 0.1 |

Schedule 8. ECE of ACEP in international trade (export)

| No. | Country/region | ECE of ACEP (Tg) | % of ACC's ECE | Type of ACEP | ECE (Tg) |
| --- | --- | --- | --- | --- | --- |
| 1 | USA | 50.50 | 87.8% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 22.5 |
|  |  |  |  | Forklifts equipment | 8.1 |
|  |  |  |  | Aircraft | 6.1 |
| 2 | China, Hong Kong SAR | 14.18 | 92.2% |  |  |
|  |  |  |  | Aircraft | 7.4 |
|  |  |  |  | Marine vessels | 3.0 |
|  |  |  |  | Medium and large Motor truck/trailers | 1.6 |
| 3 | Myanmar | 13.08 | 97.8% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 7.7 |
|  |  |  |  | Buses and coaches | 4.7 |
|  |  |  |  | Manufacturing equipment | 0.3 |
| 4 | Japan | 10.46 | 69.8% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 5.5 |
|  |  |  |  | Manufacturing equipment | 1.1 |
|  |  |  |  | Forklifts equipment | 1.0 |
| 5 | India | 8.68 | 80.5% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 4.4 |
|  |  |  |  | Manufacturing equipment | 1.0 |
|  |  |  |  | Stoves/ovens/steamers | 0.9 |
| 6 | Russian Federation | 8.56 | 96.1% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 4.1 |
|  |  |  |  | Forklifts equipment | 1.3 |
|  |  |  |  | Manufacturing equipment | 1.3 |
| 7 | Rep. of Korea | 8.02 | 72.7% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 5.4 |
|  |  |  |  | Forklifts equipment | 0.7 |
|  |  |  |  | Manufacturing equipment | 0.6 |
| 8 | Iran | 7.19 | 92.2% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 4.8 |
|  |  |  |  | Buses and coaches | 0.8 |
|  |  |  |  | Automotive tools | 0.4 |
| 9 | Indonesia | 6.59 | 77.9% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 3.2 |
|  |  |  |  | Manufacturing equipment | 0.8 |
|  |  |  |  | Cranes | 0.4 |
| 10 | Australia | 6.20 | 82.7% |  |  |
|  |  |  |  | Medium and large Motor truck/trailers | 2.7 |
|  |  |  |  | Forklifts equipment | 1.5 |
|  |  |  |  | Manufacturing equipment | 0.4 |

Schedule 9. The carbon emission coefficients of the exported and imported SP (unit: kgCO2/t)

| Types of SP | Exported SP | Imported SP |
| --- | --- | --- |
| Cold rolled sheet | 10,253.0–11,031.7 | 8,600.8–8,642.4 |
| Hot rolled sheet | 10,169.9–10,934.7 | 8,489.9–8,517.6 |
| Cold rolled strip | 10,072.8–10,851.5 | 8,420.6–8,462.2 |
| Hot rolled strip | 10,156.0–10,893.1 | 8,476.0–8,476.0 |
| Foil | 10,294.6–11,198.1 | 8,586.9–8,753.3 |
| Profile | 10,405.5–11,974.4 | 8,711.7–9,488.0 |
| Tube | 10,599.6–12,168.5 | 8,739.4–9,682.1 |

Schedule 10. Data quality indicators and Uncertainty

| Data indicators | Indicator explanation and description |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | level 1 | level 2 | level 3 | level 4 | level 5 |
| Source reliability | Raw data from field survey or measurement (0) | Data from authoritative and regularly updated (0.025) | Irregularly updated data from general literature or monographs (0.050) | Data from estimates, assumptions, and conjectures from the literature (0.100) | Data from unfounded estimates and assumptions (0.200) |
| Sample completeness | Data contains sufficient samples and the period is appropriate (0) | Representative data from a smaller range but the period is appropriate (0.010) | Representative data from a suitable range but the period is slightly shorter (0.025) | Representative data from a smaller range and period (0.050) | Unknown and insufficient data from small areas and short periods (0.100) |
| Technical representativeness | Data from the researched company (0) | Data from different companies but with the same technology, process and raw materials (0.025) | Data from the same technology, but the process and raw materials are different (0.100) | Data from the same products with different technology (0.200) | Data from similar products (0.350) |
| Time representativeness | Data irrelevant to time or within the past 3 years (0) | Data within the past 6 years (0.015) | Data within the past 10 years (0.050) | Data within the past 15 years (0.100) | Data from more than 15 years or unknown years (0.200) |
| Geographical representativeness | Data from the study area (0) | Average data from a larger area including the research area (0.005) | Data from areas with highly similar production conditions and productivity levels (0.010) | Data from areas with moderately similar production conditions and productivity levels (0.025) | Data from unknown regions or regions with completely different production conditions and productivity levels (0.050) |

Note: The value in brackets is the uncertain value of the indicator

Schedule 11. GHG emissions intensity of a wind farm and of other types of electricity generation in China, 2019

| Electricity type | GHG emissions intensity (g CO2 eq./kWh) | Data sources |
| --- | --- | --- |
| Wind power | 16.4–28.2 | Li et al., 2021 |
| Nuclear power | 10.9–13.9 | Wang et al., 2019 |
| Hydropower | 3.1–3.9 | Wang et al., 2019 |
| Thermal power | 810.0–820.0 | Zhao et al., 2017; Jiang et al., 2019 |
| Biomass power | 206.0 | Gao et al., 2019 |
| Photovoltaic power | 16.0-40.0 | Dong et al., 2019 |

References

Chen WQ, Shi L, Qian Y (2009) Substance Flow Analysis of Aluminium in China for 1991-2007 (Ⅱ): Quantity loss of Aluminum from a Perspective of Life Cycle and Its Policy Implications. Res Sci 31(12):2120–2129 (In Chinese)

Chen WQ, Shi L, Qian Y (2009) Substance Flow Analysis of Aluminium in China for 1991-2007 (Ⅱ): Quantity loss of Aluminum from a Perspective of Life Cycle and Its Policy Implications. Res Sci 31(12):2120–2129 (In Chinese)

China Environmental Protection Administration (2006) Cleaner Production Standard-Aluminum electrolytic industry, HJ/T187. China Standard Press, pp 1–10 (In Chinese)

China Environmental Protection Administration (2006) Cleaner Production Standard-Aluminum electrolytic industry, HJ/T187. China Standard Press, pp:1–10 (In Chinese)

China National Standardization Management Committeem (2009) National Standard of the People’s Republic of China-Alumina, GB/T 24487-2009: 1–4 (In Chinese)

China Nonferrous Metals Industry Association (2009) The yearbook of nonferrous metals industry of China 2009, in: Association, C.N.M.I. (Ed.), The Yearbook of Nonferrous Metals Industry of China. China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2009) The yearbook of nonferrous metals industry of China 2009, in: Association, C.N.M.I. (Ed.), The Yearbook of Nonferrous Metals Industry of China. China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2012) The yearbook of nonferrous metals industry of china 2012. In: Association C.N.M.I. (ed) The yearbook of nonferrous metals industry of china. China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2012) The yearbook of nonferrous metals industry of china 2012. In: Association, C.N.M.I. (Ed.) The yearbook of nonferrous metals industry of china, China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2015) The yearbook of nonferrous metals industry of china 2015. In: Association C.N.M.I. (ed) The yearbook of nonferrous metals industry of china. China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2015) The yearbook of nonferrous metals industry of china 2015. In: Association, C.N.M.I. (Ed.), The yearbook of nonferrous metals industry of china, China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2018a) The yearbook of nonferrous metals industry of china 2018. In: Association, C.N.M.I. (Ed.), The yearbook of nonferrous metals industry of china, China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2018a) The yearbook of nonferrous metals industry of china 2018. In: Association, C.N.M.I. (Ed.), The yearbook of nonferrous metals industry of china, China Nonferrous Metals Industry Yearbook Press, Beijing (In Chinese)

China Nonferrous Metals Industry Association (2018b) Dynamic tracking and analysis of aluminum material flow data-2017 annual plan research report-aluminum, pp: 1–93 (In Chinese)

China Nonferrous Metals Industry Association (2018b) Dynamic tracking and analysis of aluminum material flow data-2017 annual plan research report-aluminum, pp: 1–93 (In Chinese)

China Quality Mark Certification Group Co., Ltd. (2013) Implementation Rules for Certification of Energy Management System in Nonferrous Metal Mining Industry and Mineral Processing Industry, 1–52 (In Chinese)

China Quality Mark Certification Group Co., Ltd. (2013) Implementation Rules for Certification of Energy Management System in Nonferrous Metal Mining Industry and Mineral Processing Industry, 1–52 (In Chinese)

Dong X, Liu GJ, Dong YK, Huang KL, Liu X (2019) Effects of Copper, Indium, Gallium and Selenium Photovoltaic Power Generation on Ecological Environment. Construction Science and Technology 25-28.

Feng, A., Wu, B., Lü, Z., Guo, M. (2016) "Three rates" investigation and evaluation of my country's bauxite resources development and utilization. Mineral protection and utilization, 16–18 (In Chinese)

Feng, A., Wu, B., Lü, Z., Guo, M. (2016) "Three rates" investigation and evaluation of my country's bauxite resources development and utilization. Mineral protection and utilization, 16–18 (In Chinese)

Gao CK, Na HM, Song KH, Dyer N, Tian F, Xu QJ, Xing YH (2019) Environmental impact analysis of power generation from biomass and wind farms in different locations. Renew Sust Energy Rev 102:307–17.

International Aluminum Institute (2007) Life cycle assessment of aluminum: Inventory data for the worldwide primary aluminum industry year 2005 update [EB/OL]. London

International Aluminum Institute (2007) Life cycle assessment of aluminum: Inventory data for the worldwide primary aluminum industry year 2005 update [EB/OL]. London

Jiang YY, Peng L, Zhao S, Liu L (2019) Carbon emission efficiency of China provincial thermal power industry based on spatial measurement. J Shandong University Finance Econ 31:31–42+83.

Li HL, Gu SQ (2011) Aluminum Smelting Production Technology Manual, Metallurgical Industry Press, Beijing, pp: 1–990 (In Chinese)

Li HL, Gu SQ (2011) Aluminum Smelting Production Technology Manual, Metallurgical Industry Press, Beijing, pp: 1–990 (In Chinese)

Li Q, Duan H, et al. (2021) Life cycle assessment and life cycle cost analysis of a 40 MW wind farm with consideration of the infrastructure. Renewable and Sustainable Energy Reviews 138.

Lubetsky J, Steiner BA, Lanza R (2006) 2006 IPCC Guidelines for National Greenhouse Gas Inventories. https://www.ipcc-nggip.iges.or.jp/public/2006gl/. Accessed 26 June 2020

Ministry of Environmental Protection of China (2009) Cleaner production standard-Alumina industry, in: Ministry of Environmental Protection of China (Ed.), HJ 473-2009, China Environmental Science Press, Beijing, pp: 1–16 (In Chinese)

Ministry of Environmental Protection of China (2009) Cleaner production standard-Alumina industry, in: Ministry of Environmental Protection of China (Ed.), HJ 473-2009, China Environmental Science Press, Beijing, pp: 1–16 (In Chinese)

Ministry of Environmental Protection of China (2009) Cleaner production standard-Alumina industry, in: Ministry of Environmental Protection of China (Ed.), HJ 473-2009, China Environmental Science Press, Beijing, pp: 1–16 (In Chinese)

Ministry of Industry and Information Technology of China (2013) Aluminum industry standard conditions, in: Ministry of Industry and Information Technology of China (Ed.), China Standard Press, pp: 1–10 (In Chinese)

Ministry of Industry and Information Technology of China (2013) Aluminum industry standard conditions, in: Ministry of Industry and Information Technology of China (Ed.), China Standard Press, pp: 1–10 (In Chinese)

Ministry of Industry and Information Technology of China (2016) People‘s Republic of China Machinery Industry Standard-Polyvinyl chloride insulated cables of rated voltages up to and including 450/750 V, in: Ministry of Industry and Information Technology of China (Ed.), GB∕T 8734.2, China Standard Press, Beijing (In Chinese)

Ministry of Industry and Information Technology of China (2016) People‘s Republic of China Machinery Industry Standard-Polyvinyl chloride insulated cables of rated voltages up to and including 450/750 V, in: Ministry of Industry and Information Technology of China (Ed.), GB∕T 8734.2, China Standard Press, Beijing (In Chinese)

Ministry of Industry and Information Technology of China (2017a) China Nonferrous Metal Industry Standard: The norm of  per unit product of wrought aluminum and aluminum alloys-Part 1: Ingots of Cast, YS/T 694.1: 1–14 (In Chinese)

Ministry of Industry and Information Technology of China (2017a) China Nonferrous Metal Industry Standard: The norm of energy consumption per unit product of wrought aluminum and aluminum alloys-Part 1: Ingots of Cast, YS/T 694.1: 1–14 (In Chinese)

Ministry of Industry and Information Technology of China (2017b) China Nonferrous Metal Industry Standard: The norm of  per unit product of wrought aluminum and aluminum alloys-Part 2: Plates, sheets and strips, YS/T 694.2: 1–16 (In Chinese)

Ministry of Industry and Information Technology of China (2017b) China Nonferrous Metal Industry Standard: The norm of energy consumption per unit product of wrought aluminum and aluminum alloys-Part 2: Plates, sheets and strips, YS/T 694.2: 1–16 (In Chinese)

Ministry of Industry and Information Technology of China (2017c) China Nonferrous Metal Industry Standard: The norm of  per unit product of wrought aluminum and aluminum alloys-Part 3: Foils, YS/T 694.3: 1–11 (In Chinese)

Ministry of Industry and Information Technology of China (2017c) China Nonferrous Metal Industry Standard: The norm of energy consumption per unit product of wrought aluminum and aluminum alloys-Part 3: Foils, YS/T 694.3: 1–11 (In Chinese)

Ministry of Industry and Information Technology of China (2017d) China Nonferrous Metal Industry Standard: The norm of  per unit product of wrought aluminum and aluminum alloys-Part 4: Extruded profiles and tubes, YS/T 694.4: 1–12 (In Chinese)

Ministry of Industry and Information Technology of China (2017d) China Nonferrous Metal Industry Standard: The norm of energy consumption per unit product of wrought aluminum and aluminum alloys-Part 4: Extruded profiles and tubes, YS/T 694.4: 1–12 (In Chinese)

Pan ZS, Zhang ZZ, Zhang ZN, Feng GQ, Cao XS (2019) Analysis of the import source country of the bauxite in China. China Mining Magazine 28(02): 13-17+24 (In Chinese)

Recalde K, Wang J, Graedel TE (2008) Aluminium in-use stocks in the state of Connecticut. Resources, Conservation & Recycling, 52: 1271-1282.

Recalde K, Wang J, Graedel TE (2008) Aluminium in-use stocks in the state of Connecticut. Resources, Conservation & Recycling, 52: 1271-1282.

Second Editing Room of China Standard Press (2004) Compilation of aluminum and aluminum alloy standards, China Standard Press (In Chinese).

Second Editing Room of China Standard Press (2004) Compilation of aluminum and aluminum alloy standards, China Standard Press (In Chinese).

Shinzato MC, Hypolito R (2005) Solid waste from aluminum recycling process: characterization and reuse of its economically vatuable constituents. Waste Management 25(1):37–46

Wang L, Wang Y, Du HB, Zuo J, Li RYM, Zhou ZH, Bi FF, Garvlehn MP(2019) A comparative life-cycle assessment of hydro-, nuclear and wind power, A China study. Appl Energy 249:37–45

Xiong H, Guan H, Chen Q (2005) Analysis on China's aluminium scrap import (Ⅱ). Non-Ferrous Metals Recycling and Utilization: 23–25 (In Chinese).

Xiong H, Guan H, Chen Q (2005) Analysis on China's aluminium scrap import (Ⅱ). Non-Ferrous Metals Recycling and Utilization: 23–25 (In Chinese).

Yang ZS, Lv SX, Cao PC, Abdullah MI, Liu W (2011) Key technology development trend for recycled aluminum industry based on technology roadmap. Advanced Materials Research. https://doi.org/10.4028/www.scientific.net/AMR.335-336.845

Zhao XL, Cai Q, Zhang SF, Luo KY (2017) The substitution of wind power for coal-fired power to realize China's CO2 emissions reduction targets in 2020 and 2030. Energy 120:164–78.

Bauxite is divided into many different grades according to different aluminum content, The formula for calculating the weighted average of the percentage of aluminum in bauxite produced in China is: β=q=1nθq* λq*(1/54102), where q refers to different grades of bauxite; θqrefers to the content of Al2O3 in q-grade bauxite, and; λq refers to the proportion of q-grade bauxite in the bauxite reserves. Values were calculated according to the data from CNMIA (2018b), the weighted average of the percentage of aluminum in bauxite produced in China is 31.76%.