Table 1: Different biodiesel production processes used in this study

| Process No. | Process description | Catalyst | Co-solvent |
| --- | --- | --- | --- |
| Process I | Homogeneous alkali-catalyzed | KOH | THF |
| Process II | Heterogeneous alkali-catalyzed | CaO | THF |
| Process III | Supercritical | N/A | THF |

Table 2: Process conditions for the three different processes

| Parameter | Process I | Process II | Process III |
| --- | --- | --- | --- |
| Methanol:Oil mole ratio | 4.5:1 | 12:1 | 42:1 |
| reaction time (min) | 10 | 360 | 30 |
| Catalyst type | KOH | CaO |  |
| Catalyst wt% | 0.5 | 2.5 |  |
| conversion % | 91 | 97 | 98.5 |
| Temp (°C) | 25 | 60 | 350 |
| Press (kpa) | 100 | 100 | 19000 |
| Co-solvent conc. wt% | 25 | 30 | 1.63 |
| Reference | [15] | [16] | [17] |

Table 3: Costs of raw materials, catalysts and products used in the process.

| Item | Specification | Price ($/ton) | Source |
| --- | --- | --- | --- |
| Raw materials |  |  |  |
| Refined UCO | 5% max FFA | 600 | [19] |
| Methanol | 99.85% | 420 | [20] |
| KOH | 95% | 750 | [21] |
| H3PO4 | 75% tech. | 675 | [22] |
| CaO | 96% Calcined | 450 | [23] |
| Products |  |  |  |
| Biodiesel | < 96.5% | 990 | [24] |
| Glycerol | 95% | 855 | [25] |
| K3PO4 | 98% | 1690 | [26] |
| Utilities |  |  |  |
| Low press. Steam (LPS) | 5 barg & 160 °C | 33.10 | [27] |
| Mid press. Steam (MPS) | 10 barg & 184 °C | 33.46 | [27] |
| High press. Steam (HPS) | 41 barg & 254 °C | 33.73 | [27] |
| Superheated HPS | 41 barg & 500 °C | 46.44 | [27] |
| Cooling water | 30 °C to 45 °C | 0.016 $/m3 | [27] |
| Refrigeration water | 5 °C to 15 °C | 0.201 $/m3 | [27] |
| Low refrigeration water | -20 °C to -10 °C | 0.359 $/m3 | [27] |
| Electricity | N/A | 0.07 $/kw.hr | [27] |
| Waste disposal (solid & liquid) |  |  |  |
| Hazardous | N/A | 217 | [27] |
| Non-hazardous | N/A | 39 | [27] |

Table 4: Multiplication factors for estimating direct manufacturing costs

| Item | Factor | Multiply by |
| --- | --- | --- |
| Direct supervisory and clerical labor | 18% | Operating labors (COL) |
| Maintenance and repairs | 6% | Fixed capital costs (CFC) |
| Operating supplies | 15% | Maintenance and repairs (CM&R) |
| Laboratory charges | 15% | Operating labors (COL) |
| Patents and royalties | 3% | Cost of manufacturing (COM) |

.

Table 5: Multiplication factors for estimating fixed manufacturing costs

| Item | Factor | Multiply by |
| --- | --- | --- |
| Depreciation | 10% | Fixed capital costs (CFC) |
| Plant overhead | 60% | Operating labors (COL) + Direct supervisory (CSUP) + Maintenance and repairs (CM&R) |
| Taxes and insurance | 3.2% | Fixed capital costs (CFC) |

Table 6: Multiplication factors for estimating general expenses

| Item | Factor | Multiply by |
| --- | --- | --- |
| Administrative | 15% | Operating labors (COL) + Direct supervisory (CSUP) + Maintenance and repairs (CM&R) |
| Distribution and selling | 11% | Cost of manufacturing (COM) |
| Research and development | 5% | Cost of manufacturing (COM) |

Table 7: Identification of control variables for each process

| Control variable | Process |  |  |
| --- | --- | --- | --- |
|  | I | II | III |
| Methanol recovery % | √ | √ | √ |
| Methanol recovery tower vacuum | √ | √ | √ |
| Biodiesel purification tower vacuum | √ | √ | √ |
| Oil price | √ | √ | √ |
| Methanol price | √ | √ | √ |
| Catalyst price | √ | √ |  |
| Co-solvent price | √ | √ | √ |
| Neutralization agent price | √ |  |  |
| Biodiesel price | √ | √ | √ |
| Glycerol price | √ | √ | √ |
| By-product salt price | √ |  |  |

Table 8: Description of control variables in Process I.

| Control variables | Description | Units | Level |  |
| --- | --- | --- | --- | --- |
|  |  |  | -1 | 1 |
| A | Methanol recovery | % | 91 | 99 |
| B | Methanol recovery tower vacuum | kpa | 20 | 40 |
| C | Biodiesel purification tower vacuum | kpa | 15 | 35 |
| D | Oil price | $/ton | 200 | 600 |
| E | Methanol price | $/ton | 220 | 620 |
| F | Catalyst price | $/ton | 550 | 950 |
| G | Co-solvent price | $/ton | 2525 | 2925 |
| H | Neutralization agent price | $/ton | 475 | 875 |
| J | Biodiesel price | $/ton | 790 | 1190 |
| K | Glycerol price | $/ton | 655 | 1055 |
| L | By-product salt price | $/ton | 1490 | 1890 |

Table 9: Description of sensitivity analysis’s system for each process

| Item | Process |  |  |
| --- | --- | --- | --- |
|  | I | II | III |
| System level | 2IV11-6 | 2IV9-4 | 2IV8-3 |
| No. of factors | 11 | 9 | 8 |
| No. of responses | 1 | 1 | 1 |
| No. of runs | 32 | 32 | 32 |

Table 10: Unit operating conditions foreach process

| Parameter | Process I | Process II | Process III |
| --- | --- | --- | --- |
| Transesterification |  |  |  |
| Catalyst | KOH | CaO | N/A |
| Temp (°C) | 25 | 60 | 350 |
| Press (kpa) | 100 | 100 | 19000 |
| MeOH:Oil mole ratio | 4.5:1 | 12:1 | 42:1 |
| Reaction time (min) | 10 | 360 | 30 |
| Co-solvent (wt%) | 25 | 30 | 1.63 |
| Conversion % | 91 | 97 | 98.5 |
| Size (D × H, m) | 1.8 x 3.6 | 6.4 x 12.8 | 5.2 x 10.4 |
| Methanol recovery |  |  |  |
| Temp (°C) | 21.1/148.0 | 22.6/71.7 | 28.0/50.0 |
| Press (kpa) | 20/30 | 20/30 | 20/30 |
| Condenser duty (MJ) | 7.32 | 19.29 | 60.75 |
| Reboiler duty (MJ) | 11.15 | 18.78 | 34.53 |
| Vapor load (kg/hr) | 11431 | 19985 | 51453 |
| Size (D × H, m) | 2.0 x 3.7 | 2.7 x 3.7 | 4.3 x 3.7 |
| Catalyst neutralization |  |  |  |
| Catalyst | H3PO4 | N/A | N/A |
| Temp (°C) | 25 |  |  |
| Press (kpa) | 100 |  |  |
| Reaction time (min) | 10 |  |  |
| Conversion % | 100 |  |  |
| Size (D × H, m) | 1.7 x 3.4 |  |  |
| Biodiesel purification |  |  |  |
| Temp (°C) | 190.5/504.8 | 183.7/395.8 | 168.9/360.3 |
| Press (kpa) | 15/25 | 15/25 | 15/25 |
| Condenser duty (MJ) | 17.71 | 18.02 | 19.33 |
| Reboiler duty (MJ) | 22.49 | 19.08 | 19.92 |
| Vapor load (kg/hr) | 56940 | 57412 | 60684 |
| Final purity (wt%) | 99.78 | 99.84 | 99.47 |
| Size (D × H, m) | 3.8 x 2.9 | 3.5 x 2.9 | 3.7 x 2.9 |
| Glycerol separation |  |  |  |
| Final purity (wt%) | 95.79 | 98.74 | 99.22 |
| Size (D × H, m) | 0.6 x 1.2 | 0.6 x 1.2 | 0.6 x 1.2 |

Table 11: Number of main processing units required for each process

| Equipment type | Process I | Process II | Process II |
| --- | --- | --- | --- |
| Reactors | 2 | 1 | 1 |
| Columns | 2 | 2 | 2 |
| Exchangers | 0 | 1 | 3 |
| Mixers | 1 | 1 | 2 |
| Pumps | 2 | 2 | 4 |
| Gravity separators | 2 | 2 | 1 |
| Total | 9 | 9 | 13 |

Table 12: Total capital investment for each process

| Item | Process I | Process II | Process III |
| --- | --- | --- | --- |
| Reactors |  |  |  |
| Transesterification |  |  |  |
| Size (D × H, m) | 1.8 x 3.6 | 6.4 x 12.8 | 5.2 x 10.4 |
| Cost ($) | 0.07 | 1.53 | 1.98 |
| Neutralization |  |  |  |
| Size (D × H, m) | 1.7 x 3.4 | N/A | N/A |
| Cost ($) | 0.07 | 0.00 | 0.00 |
| Columns |  |  |  |
| Methanol recovery |  |  |  |
| Size (D × H, m) | 2.0 x 3.7 | 2.7 x 3.7 | 4.3 x 3.7 |
| Cost ($) | 0.54 | 0.67 | 1.64 |
| Biodiesel purification |  |  |  |
| Size (D × H, m) | 3.8 x 2.9 | 3.5 x 2.9 | 3.7 x 2.9 |
| Cost ($) | 0.62 | 0.53 | 0.57 |
| Other |  |  |  |
| Pumps | 0.03 | 0.03 | 0.32 |
| Heat Exchangers | 0.00 | 0.09 | 0.38 |
| Mixers | 0.01 | 0.01 | 0.02 |
| Gravity separators | 0.02 | 0.02 | 0.01 |
| Total bare module cost, CBM | 1.36 | 2.87 | 4.92 |
| Contingency fee, CCF=0.18CBM | 0.24 | 0.52 | 0.89 |
| Total module cost, CTM=CBM+CCF | 1.61 | 3.39 | 5.81 |
| Auxiliary facility cost, CAC=0.3CBM | 0.41 | 0.86 | 1.48 |
| Fixed capital cost, CFC=CTM+CAC | 2.01 | 4.25 | 7.29 |
| Working capital cost, CWC=0.15CFC | 0.30 | 0.64 | 1.09 |
| Total capital investment, CTCI=CFC+CWC | 2.32 | 4.89 | 8.38 |
| * Costs are reported as $ millions. |  |  |  |

Table 13: Total cost of manufacturing for each process

| Item | Process I | Process II | Process III |
| --- | --- | --- | --- |
| Direct manufacturing cost |  |  |  |
| Raw materials, CRM |  |  |  |
| Oil | 43.96 | 41.24 | 40.61 |
| Methanol | 4.71 | 5.27 | 7.57 |
| Catalyst | 0.41 | 1.16 | 0.00 |
| THF | 3.74 | 4.21 | 0.23 |
| H3PO4 | 0.22 | 0.00 | 0.00 |
| Utilities, CUT |  |  |  |
| Electricity | 0.00 | 0.00 | 0.14 |
| L.P.S | 1.42 | 2.71 | 4.93 |
| Superheated H.P.S | 18.57 | 7.46 | 11.28 |
| Refrigeration water | 0.50 | 1.32 | 4.17 |
| Cooling water | 0.05 | 0.04 | 0.05 |
| Waste disposal CWD |  |  |  |
| Non-hazardous | 0.39 | 0.13 | 0.06 |
| Hazardous | 0.23 | 0.03 | 0.01 |
| Operating labors, COL | 0.95 | 0.89 | 0.95 |
| Direct supervisory and clerical labors, 18% of COL | 0.17 | 0.16 | 0.17 |
| Maintenance and repairs, 6% of CFC | 0.12 | 0.26 | 0.44 |
| Operating supplies, 15% of maintenance and repairs | 0.02 | 0.04 | 0.07 |
| Laboratory charges, 15% of COL | 0.14 | 0.13 | 0.14 |
| Patents and royalties | 2.84 | 2.47 | 2.70 |
| Subtotal | 78.44 | 67.52 | 73.53 |
| Fixed manufacturing costs |  |  |  |
| Depreciation, ADEP | 0.20 | 0.43 | 0.73 |
| Plant overhead costs, 60% of the sum of operating labor, supervision and maintenance | 0.75 | 0.78 | 0.94 |
| Local taxes and insurance, 3.2% of CFC | 0.06 | 0.14 | 0.23 |
| Subtotal | 1.01 | 1.34 | 1.90 |
| General manufacturing expenses |  |  |  |
| Administrative costs, 15% of the sum of operating labor, supervision and maintenance | 0.19 | 0.19 | 0.23 |
| Distribution and selling cost | 10.43 | 9.04 | 9.91 |
| Research and development | 4.74 | 4.11 | 4.50 |
| Subtotal | 15.36 | 13.35 | 14.65 |
| Total cost of manufacturing (COM) | 94.81 | 82.20 | 90.07 |
| *Costs are reported as $ millions. |  |  |  |

Table 14 : After tax net profit and return on investment for each process

| Item | Process I | Process II | Process III |
| --- | --- | --- | --- |
| Products |  |  |  |
| Biodiesel | 99.45 | 99.41 | 99.25 |
| Glycerol | 9.24 | 8.79 | 8.53 |
| K3PO4 | 1.17 | 0.00 | 0.00 |
| Total Revenue, AR | 109.86 | 108.20 | 107.78 |
| Annual net profit, ANP= AR-COM | 15.05 | 26.00 | 17.71 |
| Income taxes, AIT = 30% of ANP | 4.52 | 7.80 | 5.31 |
| After tax net profit, ANNP=ANP-AIT | 10.54 | 18.20 | 12.40 |
| After tax rate of return on investment, ATROR = (ANNP+ADEP)/CFCI*100 | 513.21 | 417.86 | 160.12 |
| Payback period (years), PBP = CFCI/ANNP | 0.19 | 0.23 | 0.59 |
| *Costs are reported as $ millions. |  |  |  |

Table 15: Analysis of variance (ANOVA)

| Parameter | Process I |  | Process II |  | Process III |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Prob>F | Significance | Prob>F | Significance | Prob>F | Significance |
| MeOH recovery | 0.0067 | Major | < 0.0001 | Major | 0.0049 | Major |
| MeOH rec tower vac | 0.3253 | - | 0.1325 | - | 0.026 | Major |
| Biodiesel pur tower vac | < 0.0001 | Major | 0.0009 | Major | 0.4201 | - |
| Oil price | < 0.0001 | Major | < 0.0001 | Major | < 0.0001 | Major |
| MeOH price | 0.0002 | Major | < 0.0001 | Major | < 0.0001 | Major |
| Catalyst price | 0.5372 | - | 0.1296 | - | - | - |
| Co-solvent price | 0.6476 | - | 0.3253 | - | 0.9997 | - |
| Neut. agent price | 0.6975 | - | - | - | - | - |
| Biodiesel price | < 0.0001 | Major | < 0.0001 | Major | < 0.0001 | Major |
| Glycerol price | 0.0017 | Major | < 0.0001 | Major | 0.0276 | Major |
| By-product salt price | 0.7319 | - | - | - | - | - |

Table 16: Summary of sensitivity analyses’ results

| Process | Major effects |
| --- | --- |
| Process I | Methanol recovery % Biodiesel purification tower vacuum Oil price Methanol price Biodiesel price Glycerol price |
| Process II | Methanol recovery % Biodiesel purification tower vacuum Oil price Methanol price Biodiesel price Glycerol price |
| Process III | Methanol recovery % Methanol recovery tower vacuum Oil price Methanol price Biodiesel price Glycerol price |