| Factors | Recommended values with units | References |
| --- | --- | --- |
| Concentration of heavy metal/loids (C) | Present value, mg/kg | Present study |
| Ingestion rate (IngR) | 200 mg/day | USEPA (2001) |
| Inhalation rate (InhR) | 10 m3/day | Liu et al. (2013) |
| Exposure frequency (EF) | 350 day/year | USEPA (2011) |
| Exposure duration (ED) | 6 years for children, 30 years for adults | USEDOA (2011) |
| Exposed skin area (SA) | 2800 cm2 | USEPA (2001) |
| Dermal absorption factor (ABS) | 0.03 for As, 0.001 for other elements | USEPA (2011) |
| PEF | 1.36x109m3/kg | USEPA (2002) |
| Average body weight (BW) | 15 kg for children, 56.8 kg for adults | USEPA (2011) |
| Averaging time (AT) for non-carcinogens | EDx365 day | USEPA (2011) |
| Averaging time (AT) for carcinogens | EDx70 | USEPA (2002) |

SI Table 1 Standard recommended values for the calculation of average daily intake of soil through three varied pathways (ingestion, inhalation, dermal)

SI Table 2 References dose for some toxic elements (USEPA, 2002)

| Elements | RfD (mg.(Kg.d)-1) |  |  |
| --- | --- | --- | --- |
|  | Intake | Inhalation | Skin Contact |
| Cu | 4.00x10-2 | 4.02x10-3 | 1.20x10-2 |
| Pb | 3.50x10-3 | 3.52x10-3 | 3.25x10-3 |
| Zn | 3.00x10-1 | 3.00x10-1 | 6.00x10-2 |
| Ni | 2.00x10-2 | 2.06x10-2 | 5.40x10-3 |
| As | 3.00x10-4 | 1.23x10-4 | 1.23x10-4 |
| Cd | 1.00x10-3 | 1.00x10-5 | 1.00x10-5 |
| Hg | 3.00x10-4 | 8.57x10-5 | 2.10x10-4 |