Degradation of Rhodamine dye using a fluctuating flow type photocatalytic reactor by

external oxidants

Appendix

I) Calculations of energy and treatment cost

| Degradation of Rhodamine B obtained | = 91.6% |
| --- | --- |
| Treatment time | = 180 min (10800 sec) |
| Volume | = 10 L |
| Power rating of pump | = 0.5 HP (370 W) |
| Concentration removed after treatment (mg/L) | = 91.6100×20(mg/L) = 18.32 mg/L |
| Total energy required | = (Energy required for pump + Energy required for UV lamp = 10800 (370 W + (6 ×16W)) = 5032800 J =5032800/1000 =5032.8 kJ |
| Total energy required per unit volume | =5032.8 (kJ)/10 (L) =503.28 kJ/L |
| Total energy required for process | = (18.32 mg/L)/419.4 (kJ/L) = 0.0364 mg/kJ |
| Total Energy required for removal of Rh B | = 20 (mg/L)/ 0.0364 (mg/kJ) =549.43 kJ/L |
| Energy required (kWh) | = 0.152 kWh/L |
| In Pune (India), considering 1 kWh | = US $ 0.073 (Maharashtra State Electricity Distribution Co. Ltd., 2022-23) |
| Cost of treatment | = 0.1274 × 0.073 = US $ 0.011/L |
| Cost of additive (1 Kg of H2O2) | = US $ 0.48 |
| Cost of 8 mg/L of H2O2 (Total treatment cost for 91.6% Rhodamine B removal /L) | = US $ 0.0002 = Cost of Treatment + Cost of 8 mg/L of H2O2 =0.011+ 0.0002 =0.0113 |
| Hence, the estimated total treatment cost for 94.02% Rhodamine B removal /L is US$ 0.0113/L |  |