Supplementary Materials:

- MR imaging parameters are as follows:

section thickness, 5–6.0 mm; intersection gap, 0.5–1.6 mm; repetition time (TR) for coronal T1-w images, 500.0, ranging from: 400.0 to 800.0 msec; echo time (TE) for T1-w images, 9.2, ranging from 7.1 to 15.2 msec; TR for T2-wI images, 3140.0, ranging from 2259.0 to 8002.0 msec; TE for T2-w images, 84.5, ranging from 80.0 to 121.1 msec; TR for CET1-w images, 520.0, ranging from 300.0 to 700.0 msec; TE for CET1-w images, 9.8, ranging from 7.4 to 16.00 msec; matrix, 256 × 256 to 624 × 640; field of views (FOV),200 × 200 to 280 × 280 mm; and in-plane resolution: 0.35 × 0.35 to 0.78 × 0.78 mm.

2.Radiomics feature extraction methodology

In this study, a total of 2074 candidate radiomics features were generated, which were divided into four types: first-order statistics features, shape- and size-based features, statistics-based textural features, and features after wavelet transform.

2.1 First-order statistics features:

Shape features describe the shapes of segmented ROI areas, including 54 features extracted from T2-w images, T1-w images and subtraction images respectively.

First-order statistics describe the distribution of voxel intensities within the MRI image through commonly used and basic metrics, including 54 features extracted from T2-w images and CET1-w images respectively. To analyze the spatial distribution of the pixels’ hue matrix and extract static features of images, a fuzzy similitude matrix is defined. The matrix describes the image’s feature space. The first-order statistics features mainly include 10 percentile, 90 percentile, energy, entropy, interquartile range, robust mean absolute deviation, root mean squared, total energy, uniformity, variance, skewness, kurtosis, mean, median, maximum, range and minimum.

2.2 Shape based features:

Shape features describe the shapes of segmented ROI areas, including 14 features extracted from T2-w images and CET1-w images respectively. In this group of features, we included the descriptors of the three-dimensional size and shape of the tumor region. They included voxel volume, sphericity, minor axis length, mesh volume, maximum-3D-diameter, maximum-2D-diameter slice, maximum-2D-diameter row, maximum-2D-diameter column, major axis length, least axis length, flatness, elongation, surface area, volume, surface-to-volume ratio.

2.3 Statistics-based textural features:

Textural features are visual characteristics that reflect the homogeneity phenomenon of images and the arrangement of properties that change slowly or periodically on the body surface. Our textural features mainly included 5 typical matrices: the gray-level co-occurrence matrix (GLCM), the gray-level different matrix (GLDM), the gray-level run-length texture matrix (GLRLM), the gray-level size zone length matrix (GLSZM), the neighborhood gray-tone difference matrix (NGTDM). We extracted 72 radiomics features from the GLCM, 42 radiomics features from the GLDM,48 features from the GLRLM, 48 features from the GLSZM and 15 features from the NGTDM. The radiomics features in the GLCM mainly consisted of cluster prominence, cluster shade, cluster tendency, energy, entropy, correlation, contrast, homogeneity, auto correlation, mean and variance. The radiomics features in the GLDM mainly consisted of dependence entropy, Gray Level variance, large dependence emphasis, low Gray Level emphasis, small dependence emphasis. The radiomics features in the GLRLM mainly consisted of features such as run length non-uniformity, short/long run emphasis, and Gray level non-uniformity. The radiomics features in the GLSZM mainly consisted of features such as Gray level non-uniformity, Gray Level variance, high Gray Level zone emphasis, large area emphasis and size zone nonuniformity. The radiomics features in the NGTDM mainly consisted of features such as busyness, coarseness, complexity, contrast, strength, energy, entropy and kurtosis.

2.4 Wavelet features

The undecimated three-dimensional (3D) wavelet transform was used to decompose the original image, which can be regarded as preprocessing prior to feature extraction. By changing the ratio of high-frequency to low-frequency signal in images, wavelet transform increases the information of low-frequency signal. Consider L and H to be a low-pass and high-pass functions respectively, X to be the decomposing image, and the wavelet decompositions of X to be labeled as XLLL,XLLH XLHL ,XLHH, XHLL,  XHLH,XHHL,XHHH Then, we can obtain eight new images which are decomposed in three directions (x, y, z). The size of each decomposition is equal to the original image and each decomposition is shift invariant. For each decomposition, we computed the first-order statistics and textural features described above. This resulted in 744 features.

3.Radiomics score (Rad-score) calculation formula

Rad-score = 0.235 + (T2-w -GLCM-Idn *0.032)

− (T2-w -GLCM-inverse variance *0.016)

+( T2-w -first-order-kurtosis * 0.043)

−( T2-w -first-order -contrast * 0.019)

+( T2-w -GLCM- maximum probability*0.00008)

+( T2-w -GLCM- zone variance*0.023)

+ (T2-w -wavelet-LLH-GLSZM-Gray Level nonuniformity * 0.010)

+( T2-w -wavelet-LLH-GLSZM- zone variance * 0.046)

+( T2-w -wavelet-HLH-GLCM- Idn * 0.017)

+( T2-w -wavelet-HLH-GLCM- maximum probability * 0.032)

-( T2-w -wavelet-LLL-GLCM- difference entropy*0.011)

+( CET1-w -NGTDM- busyness*0.022)

+( CET1-w -GLSZM- large area low gray level emphasis*0.009)

-( CET1-w -GLRLM- short run low gray level emphasis*0.025)

+( CET1-w- wavelet-LLH- GLSZM- zone variance*0.043)

+( CET1-w - wavelet-LHL-GLDM- dependence nonuniformity*0.018)

+( CET1-w - wavelet-HHL-NGTDM- busyness*0.011)

-( CET1-w - wavelet-LLL-first-order- skewness*0.064)

+( CET1-w - wavelet-LLL-GLDM-Gray Level nonuniformity*0.054)

+( CET1-w - wavelet-LLL-GLSZM- Gray Level nonuniformity*0.008)