| Input | Source | Numerical parameter |
| --- | --- | --- |
| Topographic data | DEM in 1:10,000 scale | Grid of 8 meters |
| Release area | Landslide scars from aerial photos (1973) | ----------- |
| Release height | Back analysis (Fúlfaro et al. 1976; Massad et al. 1997; Massad 2002) and fieldwork observations | 1.0 meter |
|  |  | 1.3 meters 1.5 meters |
| Calculation domain | Santo Antônio and Guaxinduba catchments with 600 meters of buffer | ------------ |
| Erosion information | Back analysis (IPT 2006) | 5 meters |
| Debris flow duration | Back analysis (Gramani 2001) | 45 minutes (2.700 seconds) |
| Material density | Back analysis (Fúlfaro et al. 1976; Listo and Vieira 2015) | Fúlfaro et al. (1976)–1.8 ton/m3 (1,800 kg/m3) |
|  |  | Listo and Vieira (2015)–1,700 kg/m3, 1,800 kg/m3, 1,900 kg/m3 and 2,000 kg/m3 |
| µ (dry Coulomb type friction coefficient) | Back analysis/tan(α) (α is the slope angle in the deposition zone) | 0.05 |
| Ɛ (viscous–turbulent friction coefficient) | Back analysis (flow characteristics described by Cruz (1974), Fúlfaro et al. (1976), and Gramani (2001)) and empirical tests | 190 and 200 m/s2 |