Simultaneous time-resolved aqueous haloamine measurement empowers robust kinetic model analysis

Samuel H. Brodfuehrer (1) ‡, Daniel C. Blomdahl (1) ‡, David G. Wahman (2), Gerald E. Speitel Jr. (1), Pawel K. Misztal (1) and Lynn E. Katz (1)

- University of Texas at Austin, Department of Civil, Architectural, and Environmental Engineering, 301 E. Dean Keaton St. Stop C1786, Austin TX 78712-0284

- United States Environmental Protection Agency, Office of Research and Development, Cincinnati, OH 45268

# 2. Supplementary Materials

2.1: Discussion of mass spectra for NH2Cl, NHCl2, and NHBrCl

The molecular ions NH235ClH+ (m/z 51.995) and NH237ClH+ (m/z 53.992) were observed in the mass spectrum for NH2Cl (Fig. 1A). The mass spectrum for NHCl2 (Fig. 1B) has the molecular ions NH35Cl2H+ (m/z 85.956), and NH35Cl37ClH+ (m/z 87.952); however, the molecular ion NH37Cl2H+ was not seen or present in the mass list. The molecular ions NH35ClH+ (m/z 50.987) and NH37ClH+ (m/z 52.995) in the NHCl2 spectrum are likely due to fragmentation of NHCl2 via abstraction of Cl⁻.

The standard used to collect the mass spectrum for NHBrCl also contained NH2Cl to ensure the NHBrCl was stable (5); so, the mass spectrum contained the parent ions NH235ClH+ (m/z 51.995), NH237ClH+ (m/z 53.992), NH79Br35ClH+ (m/z 129.904), NH35Cl81BrH+ (m/z 131.903), NH37Cl79BrH+ (m/z 131.903), and NH37Cl81BrH+ (m/z 133.900). The m/z of 131.903 was used for quantification of NHBrCl and is the sum of both NH37Cl79BrH+ and NH35Cl81BrH+. NHBrCl creates a wide range of fragments that can interfere with haloamine quantification because, when the N-Cl bond breaks, it results in the observable fragments of NH279BrH+ (m/z 95.944) and NH281BrH+ (m/z 97.942) while, when the N-Br bond breaks, NH235ClH+ (m/z 51.995) and NH237ClH+ (m/z 53.992) fragments form which are identical to those produced from degradation pathways for the NH2Cl already in the standard solution.

2.2: Standard curves for haloamines

Calibration curves were made using the standards ranging in concentration from 2 μM to 200 μM which covers dilute to considerably greater concentrations than would be seen in drinking water (lower limit 2.8 and 1.4 μM and upper limit of 56 and 28 μM for monohaloamines and dihaloamines respectively). The calibration curves for all five haloamines follow linear relationships with R2 > 0.98 (Fig. S2). The standards for the NH2Br and NHBr2 standard only went up to 50 and 116 μM, respectively, because NH2Br standards had high ammonia concentrations that used up available H3O+ in the ionization process and NHBr2 rapidly decayed.

Fig. S1: Mass spectrum for ambient air during sampling period.

Fig. S2: PTR-ToF-MS calibration curves for haloamines: a) NH2Cl (m/z =51.995), b) NHCl2 (m/z = 85.956), c) NH2Br (m/z = 95.944), d) NHBrCl (m/z = 131.903), and e) NHBr2 (m/z 175.852). Units Counts per Second (CPS).

Fig. S3: Comparison of Pope (2006) model (lines) to measured haloamine concentrations during the kinetic experiment using PTR-ToF-MS. pH 7.2, 10 mM phosphate buffer, and [Br⁻] = 2 mg/L as Br⁻ (25 µM) dosed with [HOCl] = 4 mg/L as Cl2 (56 µM) for 1 minute followed by addition of [NH3] = 1.3 mg/L as N (93 µM). a) 0-30 μM concentration range b) 0-15 μM concentration range.

Fig. S4: Example of kinetic data collected by Luh and Mariñas (2014) to develop their haloamine model. pH 7.22, 10 mM phosphate buffer, [NH2Cl] = 0.20 mM, [Br⁻] = 1.00, [NH3] = 0.21 mM.

Fig. S5: Measured haloamine concentrations during the kinetic experiment using PTR-ToF-MS. pH 7.2, 10 mM phosphate buffer, and [Br⁻] = 2 mg/L as Br⁻ (25 µM) dosed with [HOCl] = 4 mg/L as Cl2 (56 µM) for 1 minute followed by addition of [NH3] = 1.3 mg/L as N (93 µM). [NOM] = 2 mg/L as C. a) 0-30 μM concentration range b) 0-15 μM concentration range.

Table S1: Level of Detection (LOD) and Level of Quantification (LOQ) for monochloramine (NH2Cl), dichloramine (NHCl2), monobromamine (NH2Br), bromochloramine (NHBrCl), and dibromamine (NHBr2). Unit μM.

|  | NH2Cl | NHCl2 | NH2Br | NHBrCl | NHBr2 |
| --- | --- | --- | --- | --- | --- |
| LOD | 0.90 | 0.0023 | 0.059 | 0.0086 | 0.00072 |
| LOQ | 1.1 | 0.0045 | 0.092 | 0.016 | 0.0016 |

Table S2: Bromochloramine formation and decomposition model developed by Luh and Mariñas (2014)

|  | Reaction | Rate Expression | Rate Constant |
| --- | --- | --- | --- |
| 1 | HOCl + NH3 → NH2Cl + H2O | k1[HOCl][NH3] | k1 = 4.2 × 106 M–1 s–1 |
| 2 | NH2Cl + H2O → NH3 + HOCl | k2[NH2Cl] | k2 = 2.1 × 10–5 M–1 s–1 |
| 3 | NH2Cl + HOCl → NHCl2 + H2O | k3[HOCl][NH2Cl] | k3 = 278 M–1 s–1 |
| 4 | NHCl2 + H2O → NH2Cl + HOCl | k4[NHCl2] | k4 = 6.4 × 10–7 s–1 |
| 5 | 2NH2Cl → NHCl2 + NH3 | k5[NH2Cl]2 | k5 = (6.9 × 103 M–2 s–1)[H+] + (0.36 M–2 s–1 )[H2PO4–] + 889 M–2 s–1 [H3PO4] |
| 6 | NHCl2 + NH3 → NH2Cl + NH2Cl | k6[NHCl2][NH3][H+] | k6 = 6.0 × 104 M–2 s–1 |
| 7 | NH2Cl + NHCl2 → products | k7[NH2Cl][NHCl2] | k7 = 1.5 × 10–2 M–1 s–1 |
| 8 | NHCl2 + H2O → I + 2H+ + 2Cl– | k8[NHCl2][OH–] | k8 = 111 M–1 s–1 |
| 9 | I + NHCl2 → HOCl + products | k9[I][NHCl2] | k9 = 2.8 × 104 M–1 s–1 |
| 10 | I + NH2Cl → products | k10[I][NH2Cl] | k10 = 8.3 × 103 M–1 s–1 |
| 11 | HOCl + NHCl2 → NCl3 + H2O | k11[HOCl][NHCl2] | k11 = (1.6 × 104 M–2 s–1)[HPO42-] + (9 × 104 M–2 s–1)[OCl–] + (3.3 × 109 M–2 s–1)[OH–] |
| 12 | NHCl2 + NCl3 + H2O → 2HOCl + products | k12[NHCl2][NCl3][OH–] | k12 = 5.6 × 1010 M–1 s–1 |
| 13 | NH2Cl + NCl3 + H2O → HOCl + products | k12[NH2Cl][NCl3][OH–] | k13 = 1.4 × 109 M–1 s–1 |
| 14 | NHCl2 + 2HOCl +H2O → NO3– + 5H+ + 4Cl– | k14 [NHCl2][OCl−] | k14 = 231 M–1 s–1 |
| 15 | NH2Cl + Br– + HA → NH3 + BrCl + A– | k15[NH2Cl][Br−][H+] | k15 = 3.8 × 105 M–2 s–1 |
| 16 | BrCl + Br– → Br2 + Cl– | k16[BrCl][Br−] | k16 = 108 M–1 s–1 |
| 17 | BrCl + H2O → HOBr + H+ + Cl– | k17[BrCl] | k17 = (3 × 106 s−1) + (109.8 M−1 s−1)[OH−] |
| 18 | HOBr + H+ + Cl– → BrCl + H2O | k18[HOBr][H+][Cl−] | k18 = 2.3 × 1010 M−2 s−1 |
| 19 | Br2 + H2O → HOBr + H+ + Br– | k19[Br2] | k19 = 97 s−1 + (109.8 M−1 s −1)[OH−] |
| 20 | HOBr + H+ + Br– → Br2 + H2O | k20[HOBr][H+][Br−] | k20 = 1.6 × 1010 M−2 s−1 |
| 21 | HOCl + H+ + Br– → BrCl + H2O | k21[HOCl][H+][Br−] | k21 = 1.32 × 106 M−2 s−1 |
| 22 | OCl– + H+ + Br– → BrCl + OH– | k22[OCl−][H+][Br−] | k22 = 3.65 × 1010 M−2 s−1 |
| 23 | HOBr + NH3 → NH2Br + H2O | k23[HOBr][NH3] | k23 = 7.5 × 107 M−1 s−1 |
| 24 | NH2Br + NH2Br → NHBr2 + NH3 | k24[NH2Br][NH2Br] | k24 = 0.5 M−1 s−1 + (5.0 × 108 M−2 s−1)[H+] + (290 M−2 s−1)[NH4+] + (3.4 × 105 M−2 s−1)[H2PO4−] |
| 25 | NHBr2 + NH3 → NH2Br + NH2Br | k25[NHBr2][NH3] | k25 = 1.0 M−1 s−1 + (1.0 × 109 M−2 s−1)[H+] + (190 M−2 s−1)[NH4+] + (6.5 × 104 M−2 s−1)[H2PO4−] |
| 26 | NH2Br + NHBr2 → products + 3Br– | k26[NH2Br][NHBr2] | k26 = 6.2 M−1 s−1 + (8.3 × 104 M−2 s−1)[OH−] |
| 27 | NHBr2 + NHBr2 → HOBr + products + 3Br– | k27[NHBr2][NHBr2] | k27 = 8.9 M−1 s−1 + (1.4 × 103 M−2 s−1)[HPO42-] |
| 28 | NH2Br + Br– + H+ → NH3 + Br2 | k28[NH2Br][Br−][H+] | k28 = 3.6 × 109 M−2 s−1 |
| 29 | NH2Br + Br2 → NHBr2 + Br– + H+ | k29[NH2Br][Br2] | k29 = 4.6 × 106 M−1 s−1 |
| 30 | NH2Cl + Br2 → NHBrCl + Br– + H+ | k30[NH2Cl][Br2] | k30 = 4.18 × 108 M−1 s−1 |
| 31 | NH2Cl + Br3– → NHBrCl + 2Br– + H+ | k31[NH2Cl][Br3−] | k31 = 3.07 × 107 M−1 s−1 |
| 32 | NH2Cl + HOBr → NHBrCl + H2O | k32[NH2Cl][Br3−] | k32 = 2.86 × 105 M−1 s−1 |
| 33 | NH2Cl + OBr– → NHBrCl + OH– | k33[NH2Cl][OBr−] | k33 = 2.20 × 104 M−1 s−1 |
| 34 | NH2Cl + NH2Br + H+ → NHBrCl + NH3 | k34[NH2Cl][NH2Br][H+] | k34 = 3.1 × 107 M−2 s−1 |
| 35 | NH2Cl + NHBr2 → NHBrCl + products | k35[NH2Cl][NHBr2] | k35 = 0.19 M−1 s−1 |
| 36 | NHBrCl + NHBrCl + OH– →HOBr + products +Br– | k36[NHBrCl][NHBrCl] | k36 = 0.52 M−1 s−1 + (0.0015 M−2 s−1)[OH−] + (139 M−2 s−1)[HPO42-] |
| 37 | NHBrCl + NH2Cl → products | k37[NHBrCl][NH2Cl] | k37 = 5.6 × 10−6 M−1 s−1 |
| 38 | NHBrCl + NH2Br → products | k38[NHBrCl][NH2Br] | k38 = 0.071 M−1 s−1 |
| 39 | NHBrCl + NHBr2 → HOBr + products + 2Br– | k39[NHBrCl][NHBr2] | k39 = (1.4 × 103 M−1 s−1) + (41.7 M−2 s−1)[OH−] + (1389 M−2 s−1)[HPO42-] |
| 40 | NHBrCl + NH3 + H+ → NH2Cl + NH2Br | k40[NHBrCl][NH3][H+] | k40 = 3.9 × 109 M−2 s−1 |
| 41 | NHBrCl + Br– + H+ → NH2Cl + `Br2 | k41[NHBrCl][Br−][H+] | k41 = 3.3 × 109 M−2 s−1 |
| E1 | H2O ⇌ H+ + OH– |  | pKa,E1 = 13.8 |
| E2 | H3PO4 ⇌ H+ + H2PO4– |  | pKa,E2 = 1.9 |
| E3 | H2PO4– ⇌ H+ + HPO42– |  | pKa,E3 = 6.79 |
| E4 | HPO42– ⇌ H+ + PO43– |  | pKa,E4 = 11.62 |
| E5 | NH4+ ⇌ H+ + NH3 |  | pKa,E5 = 9.34 |
| E6 | HOCl ⇌ H+ + OCl– |  | pKa,E6 = 7.5 |
| E7 | HOBr ⇌ H+ + OBr– |  | pKa,E7 = 8.42 |
| E8 | Br2 + Br– ⇌ Br3– |  | pKa,E8 = 16.1 |

Table S3: Unified Haloamine Model developed by Pope (2006)

|  | Reaction | Rate Expression | Rate Constant |
| --- | --- | --- | --- |
| 1 | HOCl + NH3 → NH2Cl + H2O | k1[HOCl][NH3] | k1 = 4.2 × 106 M–1 s–1 |
| 2 | NH2Cl + H2O → NH3 + HOCl | k2[NH2Cl] | k2 = 2.1 × 10–5 M–1 s–1 |
| 3 | NH2Cl + HOCl → NHCl2 + H2O | k3[HOCl][NH2Cl] | k3 = 280 M–1 s–1 |
| 4 | NHCl2 + H2O → NH2Cl + HOCl | k4[NHCl2] | k4 = 6.4 × 10–7 s–1 |
| 5* | 2NH2Cl → NHCl2 + NH3 | k5[NH2Cl]2 | k5 = (6.9 × 103 M–2 s–1)[H+] + (0.36 M–2 s–1 )[H2PO4–] + 889 M–2 s–1 [H3PcO4] |
| 6 | NHCl2 + NH3 → NH2Cl + NH2Cl | k6[NHCl2][NH3][H+] | k6 = 6.1 × 104 M–2 s–1 |
| 7 | NH2Cl + NHCl2 → products | k7[NH2Cl][NHCl2] | k7 = 1.5 × 10–2 M–1 s–1 |
| 8 | NHCl2 + H2O → I + 2H+ + 2Cl– | k8[NHCl2][OH–] | k8 = 110 M–1 s–1 |
| 9 | I + NHCl2 → HOCl + products | k9[I][NHCl2] | k9 = 2.8 × 104 M–1 s–1 |
| 10 | I + NH2Cl → products | k10[I][NH2Cl] | k10 = 8.3 × 103 M–1 s–1 |
| 11 | HOCl + Br– → HOBr + Cl– | k11[HOCl][Br−] | k11 = 1.55 × 103 M−1 s−1 |
| 12 | HOBr + NH3 → NH2Br + H2O | k12[HOBr][NH3] | k12 = 7.5 × 107 M−1 s−1 |
| 13 | OBr– + NH3 → NH2Br + OH– | k13[OBr–][NH3] | k13 = 7.6 × 104 M−1 s−1 |
| 14 | NH2Br + H2O → HOBr + NH3 | k14[NH2Br] | k14 = 1.5 × 10-3 s−1 |
| 15* | NH2Br + NH2Br → NHBr2 + NH3 | k15[NH2Br][NH2Br] | k15 = 0.5 M−1 s−1 + (5.0 × 108 M−2 s−1)[H+] + (290 M−2 s−1)[NH4+] + (3.4 × 105 M−2 s−1)[H2PO4−] |
| 16* | NHBr2 + NH3 → NH2Br + NH2Br | k16[NHBr2][NH3] | k16 = 1.0 M−1 s−1 + (1.0 × 109 M−2 s−1)[H+] + (190 M−2 s−1)[NH4+] + (6.5 × 104 M−2 s−1)[H2PO4−] |
| 17* | NH2Br + NHBr2 → products + 3Br– | k17[NH2Br][NHBr2] | k17 = 6.2 M−1 s−1 + (8.3 × 104 M−2 s−1)[OH−] |
| 18* | NHBr2 + NHBr2 → HOBr + products + 3Br– | k18[NHBr2][NHBr2] | k18 = 8.9 M−1 s−1 + (1.4 × 103 M−2 s−1)[HPO42-] |
| 19 | NH2Cl + HOBr → NHBrCl + H2O | k19[NH2Cl][Br3−] | k19 = 2.86 × 105 M−1 s−1 |
| 20 | NH2Cl + OBr– → NHBrCl + OH– | k20[NH2Cl][OBr−] | k20 = 2.20 × 104 M−1 s−1 |
| 21 | NH2Cl + NH2Cl + Br–→ NHBrCl + NH3 + Cl– | k21[NH2Cl][Br−][H+] | k21 = 8.3 × 105 M–2 s–1 |
| 22 | NHBrCl + NHBrCl + H2O → HOBr + N2 + HBr + 2HCl | k22[NHBrCl][NHBrCl] | k22 = 17 M−1 s−1 |
| E1 | H2O ⇌ H+ + OH– |  | pKa,E1 = 14 |
| E2 | H3PO4 ⇌ H+ + H2PO4– |  | pKa,E2 = 2.12 |
| E3 | H2PO4– ⇌ H+ + HPO42– |  | pKa,E3 = 7.2 |
| E4 | HPO42– ⇌ H+ + PO43– |  | pKa,E4 = 11.62 |
| E5 | NH4+ ⇌ H+ + NH3 |  | pKa,E5 = 9.24 |
| E6 | HOCl ⇌ H+ + OCl– |  | pKa,E6 = 7.54 |
| E7 | HOBr ⇌ H+ + OBr– |  | pKa,E7 = 8.8 |

* Phosphate buffer catalytic rate constants from literature were added to reactions because the original Unified Haloamine Model was intended for carbonate buffered system.