Supplementary Information

Characteristics, removal and risk of antibiotics in on-site hospital wastewater treatment plants of three types of hospital

Guoao Li a,b,c, Chengyi Sun c, Bao Jiang c, Yaling Wang c, Fei Liu a,b *

a MOE Key Laboratory of Groundwater Circulation and Environmental Evolution, China University of Geosciences (Beijing), Beijing, 100083, PR China

b Beijing Key Laboratory of Water Resources and Environmental Engineering, China University of Geosciences (Beijing), Beijing, 100083, PR China

c Beijing Municipal Research Institute of Eco-Environmental Protection, Beijing 100037, China

*Address correspondence to Dr. Liu Fei, MOE Key Laboratory of Groundwater Circulation and Environmental Evolution, China University of Geosciences (Beijing), Beijing, 100083, PR China.

E-mail:  feiliu@cugb.edu.cn (F. Liu).

Table S1. Information of antibiotics

| No. | Antibiotic | Abbreviation | Manufacturer | Purity | Molecular weight | CAS number |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Sulfapyridine | SPD | Solarbio life sciences | 98.0% | 249.29 | 144-83-2 |
| 2 | Sulfacetamide | SA | Go Trace | 99.3% | 214.22 | 144-80-9 |
| 3 | Sulfamethizole | SMTZ | LGC Labor Gmbh | 99.3% | 270.33 | 144-82-1 |
| 4 | Sulfadoxine | SDO | Stanford Analytical Chemicals Inc | 99.0% | 310.34 | 2447-57-6 |
| 5 | Sulfisomidine | SM2 | LGC Labor Gmbh | 93.4% | 300.3 | 515-64-0 |
| 6 | Sulfameter | SMT | National Institute for Food and Drug Control | 98.0% | 280.3 | 651-06-9 |
| 7 | Sulfachloropyridazine | SCPD | Solarbio life sciences | 98.0% | 284.74 | 80-32-0 |
| 8 | Sulfadimethoxine | SDM | Stanford Analytical Chemicals Inc | 99.0% | 310.3 | 122-11-2 |
| 9 | Sulfadiazine | SDI | Solarbio life sciences | 98.0% | 250.28 | 68-35-9 |
| 10 | Doxycycline hydrochloride | DOX | Solarbio life sciences | 97.0% | 512.9 | 24390-14-5 |
| 11 | Chlorotetracycline hydrochloride | CTC | Solarbio life sciences | 95.0% | 515.3 | 64-72-2 |
| 12 | Tetracycline | TC | Solarbio life sciences | 98.0% | 480.9 | 64-75-5 |
| 13 | Oxytetracycline | OTC | Solarbio life sciences | 93.0% | 496.9 | 2058-46-0 |
| 14 | Spiramycin | SPI | Solarbio life sciences | 90.0％ | 843.06 | 8025-81-8 |
| 15 | Erythromycin | ERY | Solarbio life sciences | 94.1％ | 733.93 | 114-07-8 |
| 16 | Lincomycin | LIN | Solarbio life sciences | 85.0％ | 406.54 | 7179-49-9 |
| 17 | Roxithromycin | ROX | Solarbio life sciences | 94.0％ | 837.05 | 80214-83-1 |
| 18 | Chloramphenicol | CHL | Solarbio life sciences | 99.0% | 323.13 | 56-75-7 |
| 19 | Clarithromycin | CLA | Solarbio life sciences | 95.0% | 747.95 | 81103-11-9 |
| 20 | Azithromycin | AZM | Solarbio life sciences | 98.0% | 748.98 | 83905-01-5 |
| 21 | Ofloxacin | OFL | Solarbio life sciences | 99.0% | 361.37 | 82419-36-1 |
| 22 | Ciprofloxacin hydrochloride | CIP | Solarbio life sciences | 88.5% | 367.8 | 93107-08-5 |
| 23 | Gatifloxacin | GAT | Solarbio life sciences | 98.0% | 375.3941 | 112811-59-3 |
| 24 | Piperacillin | PIP | Solarbio life sciences | 99.0% | 517.56 | 61477-96-1 |
| 25 | Ampicillin | AMP | Solarbio life sciences | 85.0% | 349.4 | 69-53-4 |
| 26 | Benzylpenicillin sodium | PG | Cato Research Chemicals Inc | 98.4% | 356.372 | 69-57-8 |
| 27 | Penicillin V potassium salt | PV | Solarbio life sciences | 95.0% | 388.48 | 132-98-9 |
| 28 | Cefradine | CEF | Solarbio life sciences | 99.0% | 349.4 | 38821-53-3 |
| 29 | Cephalexin monohydrate | CEP | Solarbio life sciences | 98.0% | 365.4 | 23325-78-2 |
| 30 | Proctaphlin Sodium | OXC | Stanford Analytical Chemicals Inc | 99.0％ | 423.419 | 1173-88-2 |
| 31 | Erythromycin-13C·D3 | ERY-13C·D3 | MACKLIN | 99.2% | 737.94 | 959119-26-7 |
| 32 | Sulfachloropyridazine-13C6 | SCPD-13C6 | WiTEGA | 98.4% | 290.74 |  |
| 33 | Benzylpenicillin sodium-D6 | PG-D6 | WiTEGA | 98.4% | 362.42 |  |
| 34 | Difloxacin-D3 | DIF-D3 | WiTEGA | 99.6% | 402.409 | 1173147-93-7 |
| 35 | Sulfadimethoxine-D6 | SDM-D6 | ANPEL Laboratory Technologies | 98.4% | 316.378 | 73068-02-7 |

LC-MSMS analysis was performed using a Waters ACQUITY UPLC ultra-high performance liquid chromatography (UPLC) system (Waters, Milford, MA, USA), using an orthogonal Z-spray electrospray ionization (ESI) interface operating in both positive and negative ionisation modes.

The UPLC separation was performed using a BEH C18 (2.1 mm i.d × 50 mm length，1.7 μm particle size, Waters), at a flow rate of 200 µL/min. The mobile phases were (A) H2O with 0.1 % HCOOH and (B) MeOH/ acetonitrile (1:1) contained 0.1 % formic acid. The mobile phase gradient was list in table S2. The injection volume was 10 µL.

Table S2 High-performance liquid chromatography gradient conditions

| Time/min | mobile phase A/% | mobile phase B/% |
| --- | --- | --- |
| 0 | 90 | 10 |
| 7.0 | 60 | 40 |
| 7.5 | 0 | 100 |
| 8.5 | 90 | 10 |
| 10 | 90 | 10 |

MS data were acquired over a m/z range of 50-1000. Nitrogen was used as drying and nebulizing gas. The gas flow was set at 550 L/h and temperature was set at 500℃. Capillary voltages and 3.2 V were used in positive ionisation modes. Collision gas was argon 99.999 % (AIR PRODUCTS, Beijing, China).

A Xevo TQ-DTM triple quadrupole (QQQ) mass spectrometer, used for quantitative analyses, was interfaced to a Waters ACQUITY ultra performance liquid chromatography (UPLCTM) system (Waters Corp., Milford, MA, USA), equipped with a binary pump system. Data were automatically processed by ChromaLynx XS software (MassLynx v 4.1, Waters). TargetLynx application manager was used to quantify the concentration levels of the target analytes.

Table S3. Mass spectrum parameters

| No. | Antibiotics | Parent MS （m/z） | Cone voltage （V） | Daughter MS 1（m/z）/Collision energy 1（eV） | Daughter MS 2（m/z）/ Collision energy 2（eV） |
| --- | --- | --- | --- | --- | --- |
| 1 | SPD | 350.4 | 20 | 147/30 | 281/22 |
| 2 | SA | 350.1 | 26 | 279.1/15 | 201.3/30 |
| 3 | SMTZ | 335.07 | 48 | 217.22/14 | 202.2/24 |
| 4 | SDO | 351.09 | 50 | 125.2/15 | 192.32/15 |
| 5 | SM2 | 402.09 | 53 | 144.1/23 | 186.2/15 |
| 6 | SMT | 518.04 | 20 | 143.2/20 | 160.14/14 |
| 7 | SCPD | 749.61 | 36 | 116.18/46 | 158.26/42 |
| 8 | SDM | 348.2 | 23 | 158/8 | 106/20 |
| 9 | SDI | 215 | 18 | 155.95/10 | 107.94/14 |
| 10 | DOX | 289.22 | 36 | 124.05/26 | 185.9/15 |
| 11 | CTC | 251.03 | 32 | 156.2/15 | 92.16/26 |
| 12 | TC | 250.19 | 32 | 156.1/15 | 92.15/27 |
| 13 | OTC | 281.1 | 32 | 92.05/30 | 155.94/16 |
| 14 | SPI | 270.98 | 24 | 155.95/14 | 91.06/28 |
| 15 | ERY | 284.99 | 25 | 155.97/14 | 91.9/28 |
| 16 | LIN | 310.95 | 33 | 155.95/18 | 92.06/28 |
| 17 | ROX | 310.95 | 33 | 155.98/18 | 91.9/32 |
| 18 | CHL | 332.2 | 54 | 288.28/17 | 314.34/20 |
| 19 | CLA | 362.3 | 44 | 318.4/18 | 261.3/26 |
| 20 | AZM | 376.13 | 46 | 261.19/26 | 332/17 |
| 21 | OFL | 445.04 | 42 | 153.93/30 | 409.89/24 |
| 22 | CIP | 461.02 | 28 | 441.8/12 | 200.93/40 |
| 23 | GAT | 479.03 | 34 | 443.82/20 | 153.93/26 |
| 24 | PIP | 445.04 | 34 | 97.9/40 | 153.94/30 |
| 25 | AMP | 748.63 | 35 | 158.26/35 | 590.55/18 |
| 26 | PG | 843.4 | 40 | 174/38 | 101/46 |
| 27 | PV | 837.61 | 37 | 158.26/30 | 679.7/22 |
| 28 | CEF | 407.13 | 40 | 126.01/26 | 358.98/20 |
| 29 | CEP | 325 | 18 | 276.85/14 | 164.94/24 |
| 30 | OXC | 734.7 | 30 | 576.5/15 | 158.39/35 |
| 31 | ERY-13C·D3 | 317.15 | 40 | 162.25/22 | 156.2/20 |
| 32 | SCPD-13C6 | 291.05 | 30 | 98/30 | 162.15/15 |
| 33 | PG-D6 | 342.2 | 50 | 218.25/15 | 98.25/40 |
| 34 | DIF-D3 | 403.2 | 43 | 359.3/20 | 299.3/30 |
| 35 | SDM-D6 | 317.27 | 10 | 162.20/22 | 158.08/20 |

Table S4. Recoveries at low concentration level

| Antibiotics | 0.1 μg/L |  |  |  |  |  | Min | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SA | 103% | 105% | 99% | 90% | 102% | 107% | 90% | 105% |
| SM2 | 104% | 117% | 116% | 121% | 121% | 121% | 104% | 121% |
| SDI | 94% | 92% | 89% | 100% | 94% | 90% | 89% | 100% |
| SPD | 96% | 87% | 84% | 85% | 90% | 82% | 82% | 96% |
| SMT | 89% | 87% | 93% | 88% | 86% | 106% | 86% | 93% |
| SMTZ | 97% | 115% | 104% | 104% | 107% | 102% | 97% | 115% |
| LIN | 89% | 80% | 80% | 73% | 71% | 79% | 71% | 89% |
| TC | 54% | 54% | 57% | 54% | 54% | 61% | 54% | 57% |
| OTC | 56% | 51% | 50% | 50% | 50% | 50% | 50% | 56% |
| CIP | 102% | 105% | 102% | 92% | 91% | 90% | 90% | 105% |
| OFL | 102% | 101% | 98% | 99% | 99% | 101% | 98% | 102% |
| AMP | 95% | 109% | 108% | 105% | 108% | 96% | 95% | 109% |
| CEF | 101% | 115% | 100% | 107% | 91% | 94% | 91% | 115% |
| PG | 103% | 104% | 104% | 101% | 99% | 108% | 99% | 104% |
| PV | 99% | 101% | 99% | 94% | 95% | 96% | 94% | 101% |
| OXC | 103% | 104% | 103% | 98% | 104% | 104% | 98% | 104% |
| PIP | 88% | 82% | 82% | 80% | 78% | 76% | 76% | 88% |
| GAT | 101% | 99% | 100% | 104% | 103% | 97% | 97% | 104% |
| SCPD | 101% | 100% | 99% | 102% | 100% | 97% | 97% | 102% |
| SDO/SDM | 99% | 100% | 99% | 100% | 99% | 96% | 96% | 100% |
| CTC | 54% | 56% | 60% | 66% | 71% | 68% | 54% | 71% |
| SPI | 77% | 72% | 64% | 65% | 75% | 60% | 60% | 77% |
| DOX | 56% | 58% | 59% | 59% | 60% | 50% | 50% | 60% |
| ERY | 52% | 57% | 51% | 51% | 47% | 42% | 42% | 57% |
| ROX | 75% | 75% | 71% | 70% | 70% | 75% | 70% | 75% |
| CAP | 90% | 90% | 77% | 80% | 76% | 76% | 76% | 90% |
| AZM | 66% | 66% | 72% | 70% | 79% | 63% | 63% | 79% |
| CLA | 78% | 73% | 73% | 73% | 73% | 68% | 68% | 78% |
| CEP | 78% | 74% | 69% | 68% | 65% | 62% | 62% | 78% |

Table S5. Recoveries at high concentration level

| Antibiotics | 0.5 μg/L |  |  |  |  |  | Min | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SA | 87% | 87% | 94% | 88% | 91% | 93% | 87% | 94% |
| SM2 | 101% | 97% | 96% | 95% | 91% | 91% | 91% | 101% |
| SDI | 95% | 90% | 93% | 92% | 90% | 95% | 90% | 95% |
| SPD | 101% | 94% | 109% | 86% | 82% | 75% | 75% | 109% |
| SMT | 78% | 77% | 75% | 74% | 72% | 71% | 71% | 78% |
| SMTZ | 104% | 97% | 105% | 108% | 106% | 96% | 96% | 108% |
| LIN | 95% | 90% | 90% | 94% | 89% | 82% | 82% | 95% |
| TC | 89% | 92% | 89% | 88% | 84% | 85% | 84% | 92% |
| OTC | 67% | 66% | 66% | 61% | 71% | 60% | 60% | 71% |
| CIP | 101% | 105% | 95% | 92% | 103% | 98% | 92% | 105% |
| OFL | 98% | 99% | 100% | 102% | 104% | 105% | 98% | 104% |
| AMP | 97% | 100% | 103% | 97% | 96% | 99% | 96% | 103% |
| CEF | 101% | 100% | 99% | 100% | 96% | 99% | 96% | 101% |
| PG | 102% | 103% | 102% | 105% | 103% | 107% | 102% | 105% |
| PV | 98% | 93% | 107% | 107% | 117% | 116% | 93% | 117% |
| OXC | 104% | 96% | 107% | 108% | 91% | 109% | 91% | 108% |
| PIP | 69% | 70% | 70% | 69% | 68% | 67% | 67% | 70% |
| GAT | 101% | 100% | 103% | 103% | 102% | 95% | 95% | 103% |
| SCPD | 103% | 96% | 104% | 103% | 97% | 97% | 96% | 104% |
| SDO/SDM | 88% | 88% | 85% | 86% | 86% | 85% | 85% | 88% |
| CTC | 92% | 91% | 86% | 86% | 88% | 85% | 85% | 92% |
| SPI | 101% | 98% | 96% | 107% | 107% | 111% | 96% | 107% |
| DOX | 90% | 87% | 84% | 82% | 79% | 76% | 76% | 90% |
| ERY | 73% | 70% | 71% | 69% | 70% | 70% | 69% | 73% |
| ROX | 75% | 75% | 76% | 79% | 70% | 69% | 69% | 79% |
| CAP | 96% | 89% | 89% | 88% | 85% | 85% | 85% | 96% |
| AZM | 88% | 78% | 76% | 74% | 74% | 74% | 74% | 88% |
| CLA | 85% | 81% | 86% | 78% | 81% | 77% | 77% | 86% |
| CEP | 87% | 84% | 83% | 88% | 87% | 81% | 81% | 88% |

In table S4, The recovery of erythromycin was below 50% for the last 3 samples. Erythromycin dehydrates under acidic conditions to produce erythromycin dehydrates. This phenomenon is due to the fact that there are many hydroxyl groups and one carbonyl group in the 14-membered ring of erythromycin structure, which is prone to intramolecular dehydration ring under acidic conditions. Therefore it is necessary to detect dehydrated erythromycin to realize the monitoring of erythromycin in the future research.

Tetracyclines are easy to form complexes with polyvalent heavy metals dissolved in water [1]. Adding complexing agents such as Na2EDTA or EDTA into the samples can complex metal ions in water, release antibiotics complexing with metal ions, and avoid the catalytic reaction between metal ions and target antibiotics, thus improving the recovery rate. But complexing agent added too much, the recovery rate was too high[2].

Table S6. Literature review of antibiotics

| Antibiotic | Nation | Hospital type | Influent (ng/L) | Effluent (ng/L) | Treatment | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Sulfisomidine | China | General | 1.6 ± 0.4 | nd | MBR | This study |
|  | China | TCM | nd | 1.9 ± 0.4 | MBR | This study |
|  | China | Cardiac | nd | nd | MBR | This study |
| Sulfadiazine | China | General | 37.0 ± 1.2 | nd | MBR | This study |
|  | China | TCM | 23.0 ± 2.0 | nd | MBR | This study |
|  | China | Cardiac | nd | nd | MBR | This study |
|  | China | General | -- | 48 | -- | [41] |
|  | China | General | -- | 253 | -- | [41] |
|  | China | -- | 368 | 205 | -- | [29] |
|  | China | -- | 521 | 278.5 | -- | [29] |
|  | China | -- | 177 | 86.2 | -- | [29] |
|  | China | -- | 477 | 251 | -- | [29] |
|  | China | -- | 573 | 373 | -- | [29] |
|  | China | -- | -- | 50 | -- | [62] |
|  | Greece | General | 347.6 | 251.8 | -- | [65] |
|  | Greece | General | 262.1 | 116.9 | -- | [65] |
|  | Italy | General | -- | 0.029-0.033 | -- | [15] |
|  | Italy | General | -- | 0.077-0.12 | -- | [15] |
|  | Italy | General | -- | 0.27-0.38 | -- | [15] |
|  | Kenya | -- | -- | 3440 | -- | [11] |
|  | Netherlands | -- | 24 | 16 | MBR+O3+GAC+UV | [49] |
|  | Malaysia | Clinic | -- | 7 | -- | [66] |
|  | Malaysia | Clinic | -- | 33.13 | -- | [66] |
|  | Spain | -- | -- | 9-137 |  | [50] |
|  | Turkey | -- | -- | 160 | -- | [32] |
| Sulfapyridine | China | General | 37.0 ± 1.2 | nd | MBR | This study |
|  | China | TCM | 20.3 ± 2.0 | nd | MBR | This study |
|  | China | Cardiac | 35.2 ± 15.3 | nd | MBR | This study |
|  | Greece | General | 262.1 | 116.9 | -- | [65] |
|  | Malaysia | Clinic | -- | 33.13 | -- | [66] |
|  | Turkey | -- | -- | 47 | -- | [32] |
| Sulfametoxydiazine | China | General | 7.1 ± 0.5 | nd | MBR | This study |
|  | China | TCM | nd | nd | MBR | This study |
|  | China | Cardiac | nd | 27.9 ± 2.8 | MBR | This study |
| Sulfamethizole | China | General | 2.6 ± 0.2 | nd | MBR | This study |
|  | China | TCM | nd | nd | MBR | This study |
|  | China | Cardiac | 2.0 ± 1.4 | nd | MBR | This study |
|  | Denmark | General | 2100 | 1400 | AS+MBR | [67] |
|  | Denmark | -- | 1200000 | 0 | MBBR | [68] |
| Lincomycin | China | General | 9.2 ± 0.4 | nd | MBR | This study |
|  | China | TCM | nd | nd | MBR | This study |
|  | China | Cardiac | 35.7 ± 9.4 | 7.4 ± 2.1 | MBR | This study |
|  | China | -- | -- | 24 | -- | [62] |
|  | China | General | -- | 174±18 | -- | [41] |
|  | China | Tumor | -- | 63 | -- | [41] |
|  | Greece | -- | -- | 60 | -- | [64] |
|  | Greece | -- | -- | 10 | -- | [64] |
|  | Italy | -- | -- | 11.9 | -- | [48] |
|  | Korea | General | 315 | 103 | BR | [69] |
|  | Korea | General | 4690 | 2980 | -- | [52] |
|  | Spain | -- | -- | 119 | -- | [42] |
|  | Thailand | -- | -- | 20 | -- | [4] |
|  | Thailand | -- | -- | 10 | -- | [4] |
|  | American | -- | -- | 80 | -- | [39] |
|  | American | -- | -- | 20 | -- | [39] |
| Tetracycline | China | General | 84.2 ± 6.2 | nd | MBR | This study |
|  | China | TCM | nd | 392.0 ± 91.8 | MBR | This study |
|  | China | Cardiac | 904.5 ± 181.6 | nd | MBR | This study |
|  | China | -- | 174 | 48.5 | -- | [29] |
|  | China | -- | 374 | 73.8 | -- | [29] |
|  | China | -- | 124 | 45.8 | -- | [29] |
|  | China | -- | 255 | 65.1 | -- | [29] |
|  | China | -- | 294 | 81.6 | -- | [62] |
|  | China | -- | -- | 59 | -- | [62] |
|  | Finland | -- | 1.6 | 0.23 | AOP | [70] |
|  | Finland | -- | 1.6 | <0.1 | UF+AOP | [70] |
|  | Ghana | -- | -- | 58–116 | -- | [58] |
|  | Italy | -- | -- | 1.5 | -- | [48] |
|  | Iran | -- | 786.5 | 49.9125 | VF | [17] |
|  | Iran | -- | 786.5 | 318.62 | AS | [17] |
|  | Qatar | -- | -- | 210 | -- | [7] |
|  | Romania | General | -- | 1340 | -- | [28] |
|  | Spain | -- | -- | 1100 | -- | [61] |
|  | Thailand | -- | 0.31 | Nd | AC | [4] |
|  | Thailand | -- | 0.78 | 0.15 | AC | [4] |
|  | Thailand | -- | 0.45 | 0.15 | RBC | [4] |
|  | Vietnam | -- | -- | 100 | -- | [31] |
|  | Vietnam | -- | -- | 2700 | AOP | [60] |
|  | Vietnam | -- | 0.76 ± 0.73 | 0.00 ± 0.00 | S-MBR+O3 | [59] |
|  | Vietnam | General | -- | 36 | S-MBR | [71] |
| Oxytetracycline | China | General | 38.8 ± 16.2 | nd | MBR | This study |
|  | China | TCM | 9.1 ± 0.8 | nd | MBR | This study |
|  | China | Cardiac | 143.4 ± 50.0 | nd | MBR | This study |
|  | China | -- | 237 | 66.3 | -- | [29] |
|  | China | -- | 341 | 77.2 | -- | [29] |
|  | China | -- | 187 | 42 | -- | [29] |
|  | China | -- | 448 | 71.6 | -- | [29] |
|  | China | -- | 376 | 78.6 | -- | [29] |
|  | Ghana | -- | -- | 75-252 | -- | [58] |
|  | Italy | -- | -- | 0.30-1.3 | -- | [15] |
|  | Italy | -- | -- | 0.074-0.10 | -- | [15] |
|  | Norway | General | -- | 750 | -- | [72] |
|  | Turkey | -- | -- | 1.94 | -- | [32] |
|  | Turkey | -- | -- | 17.4 | -- | [32] |
|  | Turkey | -- | -- | 0.1 | -- | [32] |
| Ciprofloxacin | China | General | 507.2 ± 163.5 | 231.4 ± 33.2 | MBR | This study |
|  | China | TCM | 128.2 ± 10.4 | 259.1 ± 29.6 | MBR | This study |
|  | China | Cardiac | 502.7 ± 78.5 | nd | MBR | This study |
|  | China | General | -- | 136 | -- | [41] |
|  | China | General | -- | 217 | -- | [41] |
|  | China | Tumor | -- | 11 | -- | [41] |
|  | China | -- | 197 | 92.5 | -- | [29] |
|  | China | -- | 163 | 90.2 | -- | [29] |
|  | China | -- | 345 | 87.2 | -- | [29] |
|  | China | -- | 227 | 92.6 | -- | [29] |
|  | China | -- | 246 | 78.6 | -- | [29] |
|  | China | General | 8480 | 3784.4 | -- | [26] |
|  | Colombia |  |  | 5560 |  | [44] |
|  | Colombia |  |  | 1490 |  | [44] |
|  | Colombia |  |  | 2020 |  | [44] |
|  | Colombia | -- | -- | 10700 | MBR | [73] |
|  | Cameroon | -- | -- | 24000 | -- | [74] |
|  | Finland | -- | 13 | 5.4 | AOP | [70] |
|  | Finland | -- | 13 | <0.5 | UF+AOP | [70] |
|  | France | General | 22 | 363.5 | AS | [75] |
|  | France | Medical center | -- | 5800 | -- | [46] |
|  | France | General | 46200 | 1500 | MBR+AS | [76] |
|  | Ghana | -- | -- | 11352–15733 | -- | [58] |
|  | Italy | General | -- | 10-15 | -- | [15] |
|  | Italy | General | -- | 1.4-1.9 | -- | [15] |
|  | Italy | General | -- | 15-26 | -- | [15] |
|  | Italy | -- | -- | 38-237000 | -- | [48] |
|  | Iran | -- | 7573 | 203 | VF | [17] |
|  | Iran | -- | 7505 | 140 | VF | [17] |
|  | Iran | -- | 12980 | 1421 | VF | [17] |
|  | Iran | -- | 14732 | 2127 | VF | [17] |
|  | Iran | -- | 7573 | 659 | AS | [17] |
|  | Iran | -- | 7505 | 899 | AS | [17] |
|  | Iran | -- | 12980 | 2439 | AS | [17] |
|  | Iran | -- | 14732 | 3764 | AS | [17] |
|  | Kenya | -- | -- | 70 | -- | [77] |
|  | Korea | -- | 1980 | 3080 | -- | [52] |
|  | Mexico | -- | -- | 190 | -- | [13] |
|  | Netherlands | -- | 2706 | 62 | MBR+O3+GAC+UV | [49] |
|  | Netherlands | -- | 3752 | 0 | MBR+O3+GAC+UV | [49] |
|  | Norway | General | -- | 23 336 | -- | [72] |
|  | Norway | General | -- | 15 531 | -- | [72] |
|  | Pakistan | -- | -- | 0-18000 | -- | [56] |
|  | Portugal | General | -- | 2259-38689 | -- | [51] |
|  | Portugal | General | -- | 457-13344 | -- | [51] |
|  | Portugal | Children | -- | 120-1334 | -- | [51] |
|  | Portugal | Clinic | -- | 101-2000 | -- | [51] |
|  | Qatar | -- | -- | 1990 | -- | [7] |
|  | Saudi Arabia | General | 5600 | nd | -- | [78] |
|  | Saudi Arabia | General | 2180 | nd | -- | [78] |
|  | Spain | -- | -- | 3014 | -- | [61] |
|  | Spain | -- | 12100 | 3751 | MBR (Sterile) | [38] |
|  | Spain | -- | 13000 | 130 | MBR (non-sterile) | [38] |
|  | Spain | -- | -- | 8,372.88±67.80 | -- | [30] |
|  | Spain | -- | -- | 8,305.08±47.94 | -- | [30] |
|  | Spain | -- | -- | 13,779.66±23.97 | -- | [30] |
|  | Spain | -- | >4000 | 3721 | CAWO | [35] |
|  | Spain | -- | >4000 | bdl | Fenton | [35] |
|  | Spain | -- | >4000 | 294 | RBC | [35] |
|  | Sweden | -- | -- | 3600-101000 | -- | [53] |
|  | Switzerland | General | -- | 30000-90000 | -- | [79] |
|  | Switzerland | -- | -- | 15700 | -- | [34] |
|  | Swiss | -- | 31980 | 15670 | MBR | [45] |
|  | Thailand | -- | 2.98 | NA | AS | [4] |
|  | Thailand | -- | 3.44 | 0.71 | AS | [4] |
|  | Thailand | -- | 4.35 | 1.7 | MBR | [4] |
|  | Vietnam | General | -- | 1926-23841 | S-MBR | [71] |
|  | Vietnam | Emergency | -- | 45400 | MBR | [60] |
|  | Vietnam | General | 42800 | 21500 | MBR | [54] |
|  | Vietnam | General | 19100 | 7900 | MBR | [54] |
|  | Vietnam | Polyclinic | -- | 7000 | -- | [55] |
|  | Vietnam | Surgery | -- | 10900 | -- | [55] |
|  | Vietnam | Cancer | -- | 1200 | -- | [55] |
|  | Vietnam | Obstetric | -- | 2100 | -- | [55] |
|  | Vietnam | Obstetric | -- | 1100 | -- | [55] |
|  | Vietnam | Polyclinic | 25800 | 3700 | ASc | [55] |
|  | Vietnam | -- | 12880 | 21300 | Sponge-MBR+O3 | [59] |
| Ofloxacin | China | General | 564.9 ± 86.9 | 2245.2 ± 65.9 | MBR | This study |
|  | China | TCM | 1617.7 ± 211.7 | 2195.9 ± 309.9 | MBR | This study |
|  | China | Cardiac | 576.3 ± 27.2 | 1586.2 ± 175.1 | MBR | This study |
|  | China | General | -- | 4240 | -- | [41] |
|  | China | General | -- | 3440 | -- | [41] |
|  | China | Tumor | -- | 2340 | -- | [41] |
|  | China | General | -- | 1600 | -- | [41] |
|  | China | -- | 678 | 377 | -- | [29] |
|  | China | -- | 845 | 455 | -- | [29] |
|  | China | -- | 894 | 496 | -- | [29] |
|  | China | -- | 783 | 423 | -- | [29] |
|  | China | -- | 942 | 507 | -- | [29] |
|  | China | -- | -- | 1088 | -- | [62] |
|  | China | General | 16800 | 10430 | -- | [26] |
|  | China | General | -- | 2380-9230 | -- | [80] |
|  | China | General | -- | 1390-25650 | -- | [80] |
|  | China | General | -- | 6950-49470 | -- | [80] |
|  | France | -- | -- | 2000-5000 | -- | [47] |
|  | France | -- | -- | 17900 | -- | [46] |
|  | Italy | General | -- | 13-22 | -- | [15] |
|  | Italy | General | -- | 3.3-4.1 | -- | [15] |
|  | Italy | General | -- | 25-37 | -- | [15] |
|  | Italy | -- | -- | 5-73000 | -- | [48] |
|  | Iran | -- | 13441 | 2395 | VF | [17] |
|  | Iran | -- | 4662 | 274 | VF | [17] |
|  | Iran | -- | 3750 | 718 | VF | [17] |
|  | Iran | -- | 3381 | 542 | VF | [17] |
|  | Iran | -- | 13441 | 2002 | AS | [17] |
|  | Iran | -- | 4662 | 852 | AS | [17] |
|  | Iran | -- | 3750 | 1020 | AS | [17] |
|  | Iran | -- | 3381 | 1860 | AS | [17] |
|  | Netherlands | -- | 260 | <LOD | MBR+O3+GAC+UV | [49] |
|  | Pakistan | -- | -- | 6800-66000 | -- | [56] |
|  | Portugal | General | -- | 3135-24811 | -- | [51] |
|  | Portugal | General | -- | 1986-12865 | -- | [51] |
|  | Portugal | Children | -- | nd-662 | -- | [51] |
|  | Spain | -- | -- | 10368 | -- | [42] |
|  | Spain | -- | -- | 2978 | -- | [42] |
|  | Spain | -- | 32000 | 320 | MBR(Sterile) | [38] |
|  | Spain | -- | 3340 | 66.8 | MBR(Non-Sterile) | [38] |
|  | Spain | -- | -- | 14,377.78±50.92 | -- | [30] |
|  | Spain | -- | -- | 5,700.00±33.33 | -- | [30] |
|  | Spain | -- | -- | 4,750.00±23.75 | -- | [30] |
|  | Spain | -- | >4000 | 1720 | CWAO | [35] |
|  | Spain | -- | >4000 | bdl | Fenton | [35] |
|  | Spain | -- | >4000 | 142 | Photo-Fenton | [35] |
|  | Spain | General | -- | 1547-4778 | -- | [50] |
|  | Spain | -- | -- | 2537.1 | Fungus | [43] |
|  | Spain | -- | -- | 459.4 | Fungus | [43] |
|  | Sweden | -- | -- | 200-7600 | -- | [53] |
|  | Turkey | -- | -- | 82 | -- | [32] |
|  | Turkey | -- | -- | 14000 | -- | [32] |
|  | Turkey | -- | -- | 200000 | -- | [32] |
|  | Vietnam | Health care | -- | 10900 | -- | [31] |
|  | Vietnam | General | 4600 | 2100 | MBR | [54] |
|  | Vietnam | General | 23600 | 5000 | MBR | [54] |
|  | Vietnam | -- | 15120 | 1830 | S-MBR+O3 | [59] |
|  | Vietnam | Emergency |  | 55100±47100 | MBR | [60] |
|  | Vietnam | General | 7634 | 40261 | S-MBR | [71] |
| Cefalexin | China | General | 564.9 ± 86.9 | 2245.2 ± 65.9 | MBR | This study |
|  | China | TCM | 1617.7 ± 211.7 | 2195.9 ± 309.9 | MBR | This study |
|  | China | Cardiac | 576.3 ± 27.2 | 1586.2 ± 175.1 | MBR | This study |
|  | China | General (1) | -- | 90-560 | -- | [80] |
|  | China | General (2) | -- | 3-100 | -- | [80] |
|  | China | General (3) | -- | 3-88 | -- | [80] |
| Piperacillin | China | General | 307.6 ± 32.6 | nd | MBR | This study |
|  | China | TCM | 2447.6 ± 168.6 | 49.6 ± 38.1 | MBR | This study |
|  | China | Cardiac | 696.6 ± 52.3 | 185.0 ± 23.3 | MBR | This study |
|  | France | Medical center | -- | 20 | -- | [46] |
|  | Romania | Tumor | -- | 7810 | -- | [28] |
| Doxycycline | China | General | 216.1 ± 17.4 | nd | MBR | This study |
|  | China | TCM | 103.2 ± 12.6 | 7.3 ± 10.4 | MBR | This study |
|  | China | Cardiac | 42.9 ± 2.2 | nd | MBR | This study |
|  | Finland | public hospital | 2000 | 320 | AOP | [70] |
|  | Finland | public hospital | 2000 | <200 | UF+AOP | [70] |
|  | Ghana | -- | -- | 24–120 | -- | [58] |
|  | Italy | -- | -- | 0.5-7000 | -- | [48] |
|  | Sweden | General | -- | 600-6700 | -- | [53] |
|  | Turkey | -- | -- | 7.7 | -- | [81] |
|  | Turkey | -- | -- | 11.6 | -- | [81] |
| Erythromycin | Australia | -- | -- | 39760 | -- | [40] |
|  | China | General | nd | 285.1 ± 201.6 | MBR | This study |
|  | China | TCM | nd | 432.4 ± 4.3 | MBR | This study |
|  | China | Cardiac | 427.7 ± 0.4 | nd | MBR | This study |
|  | China | General | -- | 261 |  | [41] |
|  | China | General | -- | 13 |  | [41] |
|  | China | General | 240.8 | 190.1 |  | [26] |
|  | Colombia | -- | -- | 1850 | None | [44] |
|  | Colombia | -- | -- | 310 | None | [44] |
|  | Colombia | Local hospital | -- | 360 | -- | [73] |
|  | Cameroon | -- | -- | 24000 | -- | [74] |
|  | Denmark |  | 5000 | 3000 | AC+MBR | [67] |
|  | France | Medical center | -- | 800 | -- | [46] |
|  | Greece | General | -- | 10 | -- | [35] |
|  | Greece | General | -- | 57 | -- | [35] |
|  | Ghana | Referral | -- | 7944–10613 | -- | [58] |
|  | Italy | -- | -- | 19-83000 | -- | [48] |
|  | Italy | General | -- | 0.06-0.32 | -- | [15] |
|  | Italy | General | -- | 0.080-0.086 | -- | [15] |
|  | Italy | General | -- | 0.091-0.23 | -- | [15] |
|  | Korea | General | 0.33 | 0.47 | -- | [52] |
|  | Portugal | General | -- | <MQL–1075 | -- | [51] |
|  | Portugal | General | -- | nd-22.2 | -- | [51] |
|  | Portugal | Children | -- | nd-913 | -- | [51] |
|  | Portugal | Clinic | -- | 47.8-7545 | -- | [51] |
|  | Qatar | -- | -- | 5170 ± 1560 | -- | [7] |
|  | Romania | Tumor | -- | 7520 ± 390 | -- | [28] |
|  | Spain | -- | -- | 29.46 ± 0.17 | -- | [30] |
| Erythromycin | Spain | -- | -- | 71.39 ± 2.42 | -- | [30] |
|  | Spain | -- | 2.73 | bdl | CWAO | [35] |
|  | Spain | -- | 1.84 | bdl | Fenton | [35] |
|  | Spain | -- | 0.94 | bdl | Photo-Fenton | [35] |
|  | Swiss | -- | -- | 188 | -- | [45] |
|  | Switzerland | -- | <LOQ-140 | <LOQ-16.8 | PAC | [34] |
|  | Switzerland | -- | <LOQ-140 | <LOQ-9.8 | O3 | [34] |
|  | Switzerland | -- | <LOQ-140 | <LOQ-126 | UV | [34] |
|  | Turkey | -- | -- | 8760 | -- | [81] |
|  | Turkey | -- | -- | 10800 | -- | [81] |
|  | USA | General | -- | 1.35±2.30 | -- | [39] |
|  | USA | General | -- | 0.06±0.04 | -- | [39] |
|  | Vietnam | Emergency | -- | 6300 ± 5900 | MBR | [60] |
|  | Vietnam | -- | 2.15 ± 0.26 | 0.21 ± 0.03 | Sponge-MBR+O3 | [59] |
|  | Vietnam | General | -- | 135-2407 | Sponge-MBR | [71] |
| Roxithromycin | China | General | 146.0 ± 16.9 | 70.4 ± 4.8 | MBR | This study |
|  | China | TCM | 118.7 ± 8.3 | 232.6 ± 20.4 | MBR | This study |
|  | China | Cardiac | 374.8 ± 16.5 | 268.6 ± 19.5 | MBR | This study |
|  | China | -- | -- | 676 | -- | [41] |
|  | China | -- | -- | 1180±69 | -- | [41] |
|  | China | -- | -- | 2189±362 | -- | [26] |
| Azithromycin | China | General | 1511.6 ± 111.6 | 306.2 ± 6.8 | MBR | This study |
|  | China | TCM | 1416.1 ± 42.0 | 660.9 ± 85.2 | MBR | This study |
|  | China | Cardiac | 355.1 ± 15.3 | 266.4 ± 21.4 | MBR | This study |
|  | China | -- | -- | 227 | -- | [62] |
|  | Colombia | Small size institution | -- | 2790 | -- | [73] |
|  | Colombia | -- | -- | 6930 | -- | [44] |
|  | Colombia | -- | -- | 2610 | -- | [44] |
|  | Italy | General | -- | <LOD - 0.11 | -- | [15] |
|  | Italy | General | -- | 0.045-0.050 | -- | [15] |
|  | Italy | General | -- | 0.58-1.04 | -- | [15] |
|  | Netherlands | -- | <LOD | 9 | MBR+O3+GAC+UV | [49] |
|  | Portugal | General | -- | <MQL–1075 | -- | [51] |
|  | Portugal | General | -- | 89.2-4492 | -- | [51] |
|  | Portugal | Children | -- | <mdl-376 | -- | [51] |
|  | Portugal | Clinic | -- | <mql-2665 | -- | [51] |
|  | Spain | -- | -- | 113 | -- | [42] |
|  | Spain | -- | -- | 85 | -- | [42] |
|  | Spain | -- | 1370 | 10138 | MBR (non-sterile) | [38] |
|  | Spain | -- | -- | 45.4 | C-F+F | [43] |
|  | Spain | -- | 17141 | bdl | CWAO | [35] |
|  | Spain | -- | 3731 | bdl | Fenton | [35] |
|  | Spain | -- | 3647 | 259 | Photo-Fenton | [35] |
|  | Switzerland | -- | 110±180 | 0 | PAC | [34] |
|  | Switzerland | -- | 110±180 | 9.9±16.2 | O3 | [34] |
|  | Switzerland | -- | 110±180 | 84.7±138.6 | UV | [34] |
| Clarithromycin | China | General | 4555.2 ± 139.0 | 315.8 ± 12.7 | MBR | This study |
|  | China | TCM | 1521.8 ± 27.8 | 733.6 ± 93.0 | MBR | This study |
|  | China | Cardiac | 285.6 ± 22.9 | 388.0 ± 18.4 | MBR | This study |
|  | China | -- | -- | 721 | -- | [62] |
|  | Colombia | -- | -- | 11800 | None | [44] |
|  | Colombia | -- | -- | 110 | None | [44] |
|  | Colombia | -- | -- | 26800 | None | [44] |
|  | Canada | -- | -- | 60±50 | EO | [82] |
|  | Germany | -- | >10000 | 100 | MBR | [36] |
|  | Germany | Clinics toilet | -- | 5100-1200000 | -- | [83] |
|  | Germany | Clinics sink | -- | 70-20000 | -- | [83] |
|  | Germany | Clinics shower | -- | 60-73000 | -- | [83] |
|  | Greece | General | -- | 51 | -- | [64] |
|  | Greece | General | -- | 40 | -- | [64] |
|  | Italy | General | -- | 0.02-0.14 | -- | [15] |
|  | Italy | General | -- | 0.05-0.064 | -- | [15] |
|  | Italy | General | -- | 9.3-14 | -- | [15] |
|  | Netherlands | -- | 6 | <LOD | MBR+O3+GAC+UV | [49] |
|  | Saudi Arabia | General | 83 ± 72 | 22 ± 9 | -- | [37] |
|  | Saudi Arabia | General | 38 | nd | -- | [37] |
|  | Spain | -- | 2200 | 440 | MBR (non-sterile) | [38] |
|  | Spain | -- | -- | 113 | -- | [42] |
|  | Spain | -- | -- | 973 | -- | [42] |
|  | Spain | -- | -- | 167.26±10.73 | -- | [30] |
|  | Spain | -- | -- | 941.10±1.94 | -- | [30] |
|  | Swiss | General | -- | 2555 ± 1558 | MBR | [45] |
|  | Switzerland | -- | 1280±840 | 0 | PAC | [34] |
|  | Switzerland | -- | 1280±840 | 0 | O3 | [34] |
|  | Switzerland | -- | 1280±840 | 1100.8±739.6 | UV | [34] |
|  | Turkey | -- | -- | 228 | -- | [81] |
|  | Turkey | -- | -- | 19330 | -- | [81] |
|  | USA | -- | -- | 1420±1450 | -- | [39] |
|  | USA | -- | -- | 250±230 | -- | [39] |
|  | USA | -- | -- | 630±800 | -- | [39] |

Notes: not mentioned (--), MBR (Membrane), S-MBR (Sponge-MBR), O3(Ozone), AS (activated sludge), AC (Activated Carbon), BR (Bio reactor), UV (ultraviolet), RBC (rotating biological contactor), CAWO (catalytic wet air oxidation), Fungal (Fungal reactor), PAC (Powdered Activated Carbon), VF (earthworm-based vermifilteration), EO (electro-oxidative), AOP (advanced oxidation process), UF (ultrafiltration), nd (not detected), bdl (below determination limit), LOD (limit of detection), mql (method quantification limit), LOQ  (limit of quantification)

Reference

[1] Kim C , Ryu H D , Chung E G , et al. A review of analytical procedures for the simultaneous determination of medically important veterinary antibiotics in environmental water: Sample preparation, liquid chromatography, and mass spectrometry[J]. Journal of Environmental Management, 2018, 217(JUL.1):629-645.

[2] Grujić S , Vasiljević T , Laušević M . Determination of multiple pharmaceutical classes in surface and ground waters by liquid chromatography-ion trap-tandem mass spectrometry.[J]. Journal of Chromatography A, 2009, 1216(25):4989-5000.