MATLAB Programming for carbonation batch trial study

MATLAB Programming for the carbonation batch trial study was done as per above mentioned chemical equations. Complete programming details are as follows;

MATLAB Programming for determination of reaction rate ‘k1’ and ‘k3’

clear all

time(1) = 0;

A(1) = 41.11;

B(1)=0;

C(1)= 16.29;

dt = 0.01;

k1=0.0216;

k2=0.0040;

k3=k2*55*1.45/(2.36)^2;;

Cs=1.45;

for i = 1:6000

A(i+1) = A(i) + dt * (-k2*Cs*A(i)+k3*B(i)^2+k1*B(i)*C(i));

B(i+1)  =B(i) +dt*(2*k2*Cs*A(i)-k3*B(i)^2-k1*B(i)*C(i)) ;

C(i+1) =C(i) +dt*( -k1* B(i) *C(i));

time(i+1)=time(i) +dt;

end

figure(1)

plot(time, A)

hold on

plot(time, B)

plot(time, C)

t_expt = [0 30 60];

A_expt=[41.11 44.24 55.66];

B_expt=[0 0 2.36];

C_expt=[16.29 8.23 0];

plot(t_expt,A_expt, 'o')

plot(t_expt,B_expt, 'sq')

plot(t_expt,C_expt, '^')

legend('Na_2CO_3', 'NaHCO_3', 'NaOH', 'Na_2CO_3', 'NaHCO_3', 'NaOH')

clear all

time(1) = 0;

A(1) = 33.22;

B(1)=0;

C(1)= 13.21;

dt = 0.01;

k1=0.0216;

k2=0.0040;

k3=k2*55*1.45/(2.36)^2;;

Cs=1.45;

for i = 1:9000

A(i+1) = A(i) + dt * (-k2*Cs*A(i)+k3*B(i)^2+k1*B(i)*C(i));

B(i+1)  =B(i) +dt*(2*k2*Cs*A(i)-k3*B(i)^2-k1*B(i)*C(i)) ;

C(i+1) =C(i) +dt*( -k1* B(i) *C(i));

time(i+1)=time(i) +dt;

end

figure(1)

plot(time, A)

hold on

plot(time, B)

plot(time, C)

t_expt = [0 30 60 90];

A_expt=[33.22 39.68 46.78 54.91];

B_expt=[0 0 0 0];

C_expt=[13.21 10.54 4.46 2.01];

plot(t_expt,A_expt, 'o')

plot(t_expt,B_expt, 'sq')

plot(t_expt,C_expt, '^')

legend('Na_2CO_3', 'NaHCO_3', 'NaOH', 'Na_2CO_3', 'NaHCO_3', 'NaOH')

MATLAB Programming for determination of reaction rate ‘k2’

%cabonation 3

clear all

t = [0 30 60 90];

A=[41.11 44.24 55.66 48.32];

B=[0 0 2.36 10.01];

C=[16.29 8.23 0 0];

AB=A+B;

ABC=A+B+C;

Cs=1.45;%gpL

k2avg=0.004;

for i = 1:2

%k2(i)=((AB(i+1)-AB(i))/30)/(0.5*(A(i)+A(i+1))*Cs)

k1=-(((ABC(i+1)-ABC(i))/30) - k2avg*Cs*0.5*(A(i)+A(i+1))) / (0.5*(B(i+1)+B(i))*0.5*(C(i+1)+C(i)))

end

The time vs. concentration graph was drawn using MATLAB programming as shown in Figure 5. In Figure 5, the blue, red, and yellow color line represents the simulation concentration values of Na2CO3, NaHCO3, and NaOH respectively whereas, symbols (O, □ and Δ) represents the experimental trials concentration values respectively. After 60 minutes, NaOH is consumed completely in the chemical reaction. It denotes the equilibrium position due to an increase in Na2CO3 and NaHCO3.

Figure 5. Validation graphical analysis using MATLAB Programming for carbonation of process liquor in the mining and mineral processing industry.