Appendix A: The model of Taitel and Dukler (1976) in the case of horizontal pipe

According to Taitel and Dukler (1976), each flow regime transition can be detected using a specific coordinate system. The stratified to non-stratified flow transition is detected using the coordinate systems F vs X, given by Eq. 1 and Eq. 2 respectively in the case of horizontal pipes. The parameter X is called the Lockhart-Martinelli parameter and used as the abscissa for all transition lines.

| F=ρGρG-ρGVSGgD | (A.1) |
| --- | --- |
| X=dPdLLdPdLG | (A.2) |

The pressure drop gradients dPdL for the liquid (L) and gas (G) phase are calculated using Eqs. (A.3) and (A.4), respectively.

| dPdLL=2fLρLVSL2D | (A.3) |
| --- | --- |
| dPdLG=2fGρGVSG2D | (A.4) |

which fL and fG refer to the fanning friction factor for liquid and gas phase, respectively, which are calculated as:

| fL=CReSLn | (A.5) |
| --- | --- |
| fG=CReSGn | (A.6) |

For each phase, C = 16 and n = -1 in the case of laminar flow, and C = 0.046 and n = -0.2 for the turbulent flow, as recommended by Taitel and Dukler (1976).

The Reynolds number of liquid (ReSL) and gas (ReSG), based respectively on the liquid (VSG) and gas (VSG) superficial velocities, are obtained as follows:

| ReSL=ρLVSLDµL | (A.7) |
| --- | --- |
| ReSG=ρGVSGDµG | (A.8) |

The ordinates K and T are used to detect the stratified smooth-to-stratified wavy flows and intermittent-to-bubbly flows transitions, respectively. These two parameters are calculated using Eq. 9 and Eq.10, respectively.

| K=FReSL | (A.9) |
| --- | --- |
| T=dPdLLρL-ρGg | (A.10) |

Appendix B: The formulas of the existing parameters used as coordinate for horizontal gas-liquid two-phase flow map

These parameters are input gas fraction (λG), mixture velocity (VM), input liquid fraction (λL), liquid Reynolds number (ReLS), gas Reynolds number (ReGS), liquid Froude number (FrSL), gas Froude number (FrSG) and mixture Froude number (FrM).

| λG=VSGVM | (B.1) |
| --- | --- |
| VM=VSL+VSG | (B.2) |
| λL=VSLVM | (B.3) |
| ReSL=ρLVSLDµL | (B.4) |
| ReSG=ρGVSGDµG | (B.5) |
| FrSL=VSLgD | (B.6) |
| FrSG=VSGgD | (B.7) |
| FrM=VMgD | (B.8) |

Note that there is another definition of the Froude number which takes into account the densities of the two phases. Thus, the liquid, gas and mixture Froude number can be expressed respectively as:

| FrSL=ρLρG-ρGVSLgD | (B.9) |
| --- | --- |
| FrSG=ρGρG-ρGVSGgD | (B.10) |
| FrM=ρLρG-ρGVMgD | (B.11) |