Supplementary Material for

Target screening of plant secondary metabolites in river waters by liquid chromatography coupled to high resolution mass spectrometry (LC-HRMS)

Mulatu Yohannes Nanusha1, 2, Martin Krauss1, Carina D. Schönsee3,4, Barbara F. Günthardt3, Thomas D. Bucheli3 and Werner Brack1, 2

1Helmholtz Centre for Environmental Research - UFZ, Department of Effect-Directed Analysis, Permoserstraße 15, 04318 Leipzig, Germany

2 Goethe University Frankfurt,Max-von-Laue Str. 13, 60438 Frankfurt (Main), Germany

3Agroscope, Environmental Analytics, Reckenholzstrasse 191, 8046 Zürich, Switzerland

4ETH Zürich, Department of Environmental Systems Science, Universitätstrasse 16, 8092 Zürich, Switzerland

Supplementary Tables and Figure

List of Tables

Table S1: Information on river samples and sampling locations………………………………..……….In excel format

Table S2: Plant metabolites selected for target screening of river samples…………..….…...… In excel format

Table S3: Setting for MZmine data processing.	5

Table S4: Internal standards used for the chemical analysis (ESIpos)………………………………… In excel format

Table S5: Mass spectrometry setting for the chemical analysis of water samples.	5

Table S6:  Quantifier and confirming ions used in TraceFinder for quantification of detected target compounds.	6

Table S7: The concentration of detected metabolites in river water from Leipzig and Bode catchment.	7

List of Figures

Figure S1: Workflow for screening toxic plant metabolites in river waters.	4

Figure S2: MS/MS spectra (HCD fragmentation at 45 a.u.) of coumarin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	8

Figure S3: MS/MS spectra (HCD fragmentation at 45 a.u.) of esculetin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	9

Figure S4: MS/MS spectra (HCD fragmentation at 45 a.u.) of fraxidin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	10

Figure S5: MS/MS spectra (HCD fragmentation at 45 a.u.) of scopoletin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	11

Figure S6: MS/MS spectra (HCD fragmentation at 45 a.u.) of bergapten in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	12

Figure S7: MS/MS spectra (HCD fragmentation at 45 a.u.) of psoralen in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	13

Figure S8: MS/MS spectra (HCD fragmentation at 45 a.u.) of lycorine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	14

Figure S9: MS/MS spectra (HCD fragmentation at 45 a.u.) of narciclasine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	15

Figure S10: MS/MS spectra (HCD fragmentation at 45 a.u.) of nicotine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	16

Figure S11: MS/MS spectra (HCD fragmentation at 45 a.u.) of piperine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	17

Figure S12: MS/MS spectra (HCD fragmentation at 45 a.u.) of formononetin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	18

Figure S13: MS/MS spectra (HCD fragmentation at 45 a.u.) of coniferyl aldehyde in a reference standard and water sample (NL: signal intensity at 100% relative abundance).	19

Figure S1: Workflow for screening toxic plant metabolites in river waters.

Table S3: Setting for MZmine data processing.

|  | Parameters | Mass detection | Chromatogram building | Chromatogram deconvolution | Join aligner | Gap filling | Target annotation (identification) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Peak detection | Mass detector | Centroid |  |  |  |  |  |
|  | Noise level | 5.00E+3 |  |  |  |  |  |
|  | MS level | 1.0 |  |  |  |  |  |
|  | Group intensity threshold |  | 1.00E+4 |  |  |  |  |
|  | Min height intensity |  | 5.00E+03 |  |  |  |  |
|  | m/z tolerance |  | 0.001 |  |  |  |  |
|  | Algorithm |  |  | Local minimum search |  |  |  |
|  | Chromatographic threshold (%) |  |  | 60.0 |  |  |  |
|  | Search minimum in retention time range |  |  | 0.1 |  |  |  |
|  | Minimum relative height (%) |  |  | 30 |  |  |  |
|  | Minimum absolute height |  |  | 5.0E+4 |  |  |  |
|  | Min ratio of peak top/edge |  |  | 2.3 |  |  |  |
|  | Peak duration range (min) |  |  | 0.1-0.5 |  |  |  |
| Peak alignment and identification | m/z tolerance |  |  |  | 0.001 | 0.001 | 0.001 |
|  | Weight for m/z tolerance |  |  |  | 70 |  |  |
|  | Retention time tolerance (absolute, min) |  |  |  | 0.3 | 0.15 | 0.5 |
|  | Weight for RT |  |  |  | 30 |  |  |
|  | Intensity tolerance (%) |  |  |  |  | 30.00 |  |
|  | RT range |  |  |  |  |  |  |
|  | Adducts |  |  |  |  |  | M+H+, M+Na+, M+NH4+, |

Table S5: Mass spectrometry setting for the chemical analysis of water samples.

| Parameters | Full MS | MS/MS |
| --- | --- | --- |
| Polarity | positive | positive |
| Resolving power (m/z 200) | 70,000 | 35,000 |
| Automated gain control target value | 3e6 | 5e5 |
| Maximum injection time | 200 ms | 110 ms |
| HCD collision energy |  | 45 a.u |

Table S6:  Quantifier and confirming ions used in TraceFinder for quantification of detected target compounds.

| Compound Name | Chemical formula | CAS | Quantifier MS1 (Precursor) m/z (M+H+) | Adduct | Polarity | Retention time (min) | Confirming ion – 1 (MS2, M+H+) | Confirming ion – 2 (MS2, M+H+) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coumarin | C9H6O2 | 91-64-5 | 147.0441 | M+H | + | 7.3 | 103.0542 | 91.0542 |
| Esculetin | C9H6O4 | 305-01-1 | 179.0336 | M+H | + | 4.2 | 133.0283 | 123.044 |
| Fraxidin | C11H10O5 | 525-21-3 | 223.06 | M+H | + | 7.8 | 190.026 | 162.0311 |
| Scopoletin | C10H8O4 | 92-61-5 | 193.0496 | M+H | + | 7.1 | 133.0283 | 165.0545 |
| Bergapten | C12H8O4 | 484-20-8 | 217.0495 | M+H | + | 10.1 | 202.0259 | 174.031 |
| Psoralen | C11H6O3 | 66-97-7 | 187.0388 | M+H | + | 9.1 | 143.049 | 131.049 |
| Lycorine | C16H17NO4 | 476-28-8 | 288.1225 | M+H | + | 1 | 177.0546 | 119.0491 |
| Narciclasine | C14H13NO7 | 29477-83-6 | 308.0765 | M+H | + | 5.7 | 248.0553 | 214.0499 |
| Nicotine | C10H14N2 | 54-11-5 | 163.1228 | M+H | + | 0.9 | 132.0807 | 106.0651 |
| Piperine | C17H19NO3 | 94-62-2 | 286.1434 | M+H | + | 11.9 | 201.0544 | 135.0439 |
| Formononetin | C16H12O4 | 485-72-3 | 269.0804 | M+H | + | 10.8 | 237.0544 | 213.0908 |
| Coniferyl aldehyde | C10H10O3 | 458-36-6 | 179.0701 | M+H | + | 7.6 | 147.044 | 119.0492 |

| Sample ID | Sampling season | Sampling location | Concentration (ng/l) |  |  |  |  |  |  |  |  |  |  |  | Number of SPMs detected per sample | Grand total (all SPMs detected) concentration (ng/l) per sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bergapten | Coniferyl aldehyde | Coumarin | Esculetin | Formononetin | Fraxidin | Lycorine | Narciclasine | Nicotine | Piperine | Psoralen | Scopoletin |  |  |
| ELP19 | Summer | Leipzig | ND | ND | ND | ND | 8 | 546 | ND | ND | ND | 271 | ND | 47 | 4 | 872 |
| ELP23 | Spring | Leipzig | 510 | 46 | ND | ND | ND | 56 | ND | ND | ND | 338 | ND | 31 | 5 | 981 |
| ELP17 | Spring | Leipzig | ND | 13 | ND | ND | ND | 294 | 2331 | 506 | ND | ND | ND | 13 | 5 | 3157 |
| ELP18 | Summer | Leipzig | ND | ND | ND | 1658 | ND | 1144 | 1015 | 3352 | ND | ND | ND | 41 | 5 | 7211 |
| ELP2 | Spring | Leipzig | ND | ND | ND | ND | 12 | 84 | ND | ND | ND | 1 | ND | 18 | 4 | 115 |
| ELP3 | Spring | Leipzig | ND | ND | ND | ND | 23 | 173 | ND | ND | 2 | ND | ND | 9 | 4 | 207 |
| ELP9 | Summer | Leipzig | ND | ND | ND | ND | 33 | 132 | ND | ND | ND | ND | ND | 21 | 3 | 186 |
| ELP10 | Spring | Leipzig | ND | ND | 12 | ND | 35 | 196 | ND | ND | 6 | 1 | ND | 34 | 6 | 284 |
| ELP21 | Spring | Leipzig | ND | ND | ND | 116 | ND | 228 | ND | ND | ND | ND | ND | 20 | 3 | 363 |
| BD2 | Summer | Bode | ND | ND | ND | ND | ND | 52 | ND | ND | 35 | ND | ND | 47 | 3 | 134 |
| BD8 | Summer | Bode | ND | ND | ND | ND | ND | 130 | ND | ND | ND | 294 | ND | 13 | 3 | 436 |
| BD9 | Summer | Bode | ND | ND | ND | ND | ND | 85 | ND | ND | ND | ND | 224 | 13 | 3 | 321 |
| BD10 | Summer | Bode | 541 | ND | 43 | 104 | ND | 106 | ND | ND | ND | ND | 141 | 49 | 6 | 984 |
| BD1 | Summer | Bode | ND | ND | ND | ND | ND | 36 | ND | ND | ND | ND | ND | 8 | 2 | 44 |
| BD3 | Summer | Bode | ND | ND | ND | 113 | 123 | 59 | 11 | ND | ND | ND | ND | 38 | 5 | 343 |
| BD4 | Summer | Bode | ND | ND | ND | ND | ND | 155 | ND | ND | 4 | 4 | ND | 27 | 4 | 190 |
| BD5 | Summer | Bode | ND | ND | ND | ND | ND | 19 | ND | ND | ND | ND | ND | 7 | 2 | 25 |
| BD12 | Spring | Bode | ND | ND | ND | 157 | ND | ND | ND | ND | ND | ND | ND | 23 | 2 | 179 |

Table S7: The concentration of detected metabolites in river water from Leipzig and Bode catchment.

Figure S2: MS/MS spectra (HCD fragmentation at 45 a.u.) of coumarin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S3: MS/MS spectra (HCD fragmentation at 45 a.u.) of esculetin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S4: MS/MS spectra (HCD fragmentation at 45 a.u.) of fraxidin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S5: MS/MS spectra (HCD fragmentation at 45 a.u.) of scopoletin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S6: MS/MS spectra (HCD fragmentation at 45 a.u.) of bergapten in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S7: MS/MS spectra (HCD fragmentation at 45 a.u.) of psoralen in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S8: MS/MS spectra (HCD fragmentation at 45 a.u.) of lycorine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S9: MS/MS spectra (HCD fragmentation at 45 a.u.) of narciclasine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S10: MS/MS spectra (HCD fragmentation at 45 a.u.) of nicotine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S11: MS/MS spectra (HCD fragmentation at 45 a.u.) of piperine in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S12: MS/MS spectra (HCD fragmentation at 45 a.u.) of formononetin in a reference standard and water sample (NL: signal intensity at 100% relative abundance).

Figure S13: MS/MS spectra (HCD fragmentation at 45 a.u.) of coniferyl aldehyde in a reference standard and water sample (NL: signal intensity at 100% relative abundance).