Table S1 Effect of different incubation temperature on PHB production by ES021

| Temp | PHB g/l | DCW g/l | PHB yieled |
| --- | --- | --- | --- |
| 25 30 | 1.54 3.46 | 5.99 5.96 | 25.71 58.05 |
| 35 LSD 0.05 | 2.41 0.1358 | 2.82 0.6425 | 85.46 |

Incubation at 30°C for 48 h (110 rpm), Inoculum size: 1% fermentation medium: M4 with 6.2+ 0.2 P value <0.0001

Table S2. Effect of different pH values on PHB production by ES021

| pH | PHB g/l | DCW g/l | PHB yieled |
| --- | --- | --- | --- |
| 6.2 | 3.46 | 5.96 | 58.05 |
| 7 | 5.16 | 7.78 | 66.32 |
| 7.6 | 4.32 | 6.78 | 63.72 |
| LSD 0.05 | 0.0461 | 0.3792 |  |

Incubation at 30°C for 48 h (110rpm), Inoculum size: 1% fermentation medium: M4

P value 0.05: <0.0001

Table S3. Effect of different inoculum size on PHB production by ES021

| inoculum size | PHB g/l | DCW g/l | PHB yieled |
| --- | --- | --- | --- |
| 1% | 5.16 | 7.78 | 66.32 |
| 2% | 3.92 | 15.29 | 25.64 |
| 3% | 3.38 | 16.71 | 20.23 |
| LSD 0.05 | 0.0873 | 0.2365 |  |

Incubation was at 30°C for 48 h (110 rpm). Fermentation medium M4 with pH value of 7

P value: <0.0001

Table S4. Effect of different aeration on PHB production by ES021

| Aeration | PHB g/l | DCW g/l | PHB yieled |
| --- | --- | --- | --- |
| Static 110 rpm | 3.9 5.16 | 5.12 7.78 | 76.17 66.32 |
| 150 rpm | 5.39 | 6.86 | 78.57 |
| LSD 0.05 | 0.0524 | 0.2429 |  |

Incubation temperature 30°C for 48 h, Fermentation medium M4 with pH value of 7.

P value <0.0001

Table.S5. Effect of different incubation period on PHB production by ES021

| incubation period | PHB g/l | DCW g/l | PHB yield |
| --- | --- | --- | --- |
| 24 h | 3.53 | 3.82 | 92.24 |
| 48 h | 5.39 | 6.89 | 78.34 |
| 72 h | 2.96 | 3.26 | 90.71 |
| 96 h | 1.24 | 4.26 | 29.01 |
| LSD 0.05 | 0.1274 | 0.0282 |  |

Incubation temperature 30°C at 150 rpm, pH: 7, fermentation medium: M4, inoculum size 1%. P value <0.0001

Table.S6.GCMS analysis of PHB standard showing chemical composition of biodegradable polymer

| S/N | Retention time (minutes) | % Area | Compound Name | Molecular Weight | Molecular Formula |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.95 | 1.71 | Diallyl disulphide | 146 | C6H10S2 |
| 2 | 4.31 | 0.91 | Diallyl disulphide | 146 | C6H10S2 |
| 3 | 7.49 | 0.75 | Tetradecane, 2,6,10-trimethyl- | 240 | C17H36 |
| 4 | 7.85 | 0.69 | Trisulfide, di-2-propenyl | 178 | C6H10S3 |
| 5 | 8.01 | 4.67 | Trisulfide, di-2-propenyl | 178 | C6H10S3 |
| 6 | 8.18 | 0.87 | Phenol, 2-methyl-5-(1methyl ethyl | 150 | C10H14O |
| 7 | 9.28 | 1.03 | 5-Methyl-1,2,3,4-tetrathiane | 170 | C3H6S4 |
| 8 | 11.76 | 1.09 | Decane, 2,3,5,8-tetramethyl | 198 | C14H30 |
| 9 | 11.96 | 20.26 | 1-Dodecanamine, N,N-di methyl | 213 | C14H31N |
| 10 | 12.22 | 3.08 | 3,4-Di hydro-2H-1,5-(3"-T-butyl) benzo dioxepine | 206 | C13H18O2 |
| 11 | 12.65 | 0.68 | Pentacosane | 352 | C25H52 |
| 12 | 13.65 | 0.58 | 1-Nonadecene | 266 | C19H38 |
| 13 | 15.75 | 8.72 | Nizatidine | 331 | C12H21N5O2S2 |
| 14 | 16.62 | 1.24 | Tetradecane, 2,6,10-trimethyl | 240 | C17H36 |
| 15 | 17.33 | 0.50 | 1-Nonadecene | 266 | C19H38 |
| 16 | 19.53 | 1.72 | 7,9-Di-tert-butyl-1-oxaspiro(4,5)dec a-6,9-diene-2,8-dione | 276 | C17H24O3 |
| 17 | 19.61 | 23.13 | Hexadecanoic acid, methyl ester | 270 | C17H34O2 |
| 18 | 20.23 | 0.76 | Tetradecane, 2,6,10-trimethyl- | 240 | C17H36 |
| 19 | 22.38 | 22.30 | 9-Octadecenoic acid (Z)-, methyl ester | 296 | C19H36O2 |
| 20 | 22.55 | 3.38 | 2-Methyl enebrexane | 134 | C10H14 |
| 21 | 22.90 | 0.68 | Docosane | 310 | C22H46 |
| 22 | 23.53 | 0.51 | Dotriacntane | 450 | C32H66 |
| 23 | 25.60 | 0.75 | N-Methyl-N-benzyl tetra decanamine | 317 | C22H39N |