Waste reintroduced in the kitchen: life cycles inventories of representative waste-to-nutrition pathways

Supporting information document

U. Javourez1*, L. Tiruta-Barna1, L. Hamelin1

1TBI, Université de Toulouse, CNRS, INRAE, INSA, Toulouse, France

*: Corresponding author (javourez@insa-toulouse.fr)

Summary

This supporting information document (SI) specifies the harmonized modeling approach proposed to derive the life cycle inventories (LCI) of selected waste-to-nutrition pathways. First, the residual biomass resource potential was established to uncover the quantities, compositions and current uses of the waste, residues and co-products available in France. The section SI.1 provides the basis to select the representative current use of bioresources to include in the model, as well as the key individual streams for France. Then, section SI.2 details the selection rational to screen the scaling plausibility of waste-to-nutrition pathways (in the short-, mid-term) in order to refine the modeling efforts where it matters. The section SI.3 specifies the LCI modeling strategy implemented. Then, each of the 6+1 parametric LCI models and the definition of nutritional services are transparently and extensively detailed in the section SI. 4, complementing the 4 models proposed in the companion paper Javourez et al., (2024)1. All the LCI models are shared in open-access (available in data repository). Background calculation supporting Figure 6 of the main manuscript is also available in the repository.

Supporting data

Data generated and used for the analysis is available at the link: https://doi.org/10.48531/JBRU.CALMIP/UJVBTR and is composed by the following documents:

- A.Resource_pot: Description, composition, amount and current use of French residual biomass streams

- B.BSF_module, B.CF_module, B.GA_module, B.GB_module, B.MP_module, B.Myco_module, B.SSF_module: Parametric LCI models of all waste-to-nutrition pathways assessed (including unit operation definitions, calculation details, background processes selected, etc.)

- C.Data_Fig6: Background data to derive Figure 6 of the main manuscript

Table of content

1.	Feedstock selection and modeling	6

2.	Waste-to-nutrition pathways selection	9

2.1.	EU Legislative background	9

2.2.	Process intensity to mitigate risk transmission	12

3.	Design and harmonization of the LCI models	18

4.	Life cycle inventories modeling documentation	20

4.1.	Generic LCI unit operations	21

4.1.1.	Constants	21

4.1.2.	Gases compression	21

4.1.3.	Liquid pumping	22

4.1.4.	Heating and cooling	22

4.1.5.	Combustion enthalpy potential estimation	22

4.1.6.	Heat recovery modeling	23

4.1.7.	Handling and transportation operations	26

4.1.8.	Wastewater treatment requirements	27

4.1.9.	Estimation of the nutritional services of edible products and co-products	27

4.1.10.	Energy and fertilizer services	31

4.1.11.	General principles for reuse of the co-products	31

4.2.	Compound feed LCI module	32

4.3.	Nutritional enhancement through solid fermentation LCI module	32

4.4.	Insects farming on residual biomass streams LCI module	34

4.4.1.	Set-up selection	34

4.4.2.	Unit operation design and assumptions	34

4.4.3.	Technology parameters and background processes	43

4.5.	Mycoproteins production from residual-based sugars LCI module	45

4.5.1.	Set-up selection	45

4.5.2.	Unit operation design and assumptions – the lignocellulosic pretreatment	47

4.5.3.	Technology parameters and background processes – Lignocellulosic pretreatment	51

4.5.4.	Unit operation design and assumptions – the starch pretreatment	52

4.5.5.	Unit operation design and assumptions – Mycoproteins production	53

4.5.6.	Technology parameters and background processes – Mycoproteins production stage	56

4.6.	Microbial proteins production from residual-based gases LCI module	58

4.6.1.	Set-up selection	59

4.6.2.	Unit operation design and assumptions – gas post-treatments	61

4.6.3.	Technology parameters and background processes – gas post-treatments	67

4.6.4.	Unit operation design and assumptions – MP production stage	69

4.6.5.	Technology parameters and background processes – MP production stage	77

4.7.	Rubisco proteins extraction from green residues LCI module	79

4.7.1.	Set-up selection	79

4.7.2.	Unit operation design and assumptions	81

4.7.3.	Technology parameters and background processes	88

4.8.	Gasification LCI module	90

4.8.1.	Set-up selection	90

4.8.2.	Unit operation design and assumptions	91

4.8.3.	Technology parameters and background processes	96

4.8.4.	Process flow diagram of the adapted anaerobic digestion module	98

4.9.	Summary of the technology parameters and pairing with the LCI models	99

5.	References	113

List of figures

Figure SI. 1 – Composition distribution of the 132 identified residual biomass streams for France	7

Figure SI. 2 – MP production stage water balance to produce 1kgww, illustrated for baseline parameter values	75

Figure SI. 3 – Scope, system boundaries and process flow diagram of GA	91

Figure SI. 4 – Scope, system boundaries and process flow diagram of AD	98

List of tables

Table SI. 1 – Scope and classification of residual biomass streams	6

Table SI. 2 – Selection of eleven representative residual biomass streams for France	8

Table SI. 3 – Reuse of residual biomass within food system: EU legislative background	10

Table SI. 4 – Residual biomass legislative status: an EU overview	11

Table SI. 5 – Estimation of process operation’s risk mitigation potential for waste-to-nutrition pathways	14

Table SI. 6 – Selection principles for waste-to-nutrition pathways	15

Table SI. 7 – Data selection hierarchy for technology parameters	19

Table SI. 8 – Modular life cycle inventories of 16 waste-to-nutrition options	20

Table SI. 9 – Calculation constants	21

Table SI. 10 – Parameters specific to gas compression	21

Table SI. 11 – Parameters specific to HT WHR	24

Table SI. 12 – Parameters specific to LT WHR	25

Table SI. 13 – Parameters specific to gaseous co-products energy valorization	25

Table SI. 14 – Background processes used specifically for gaseous co-products energy valorization	26

Table SI. 15 – Supply chain distances considered for waste-to-nutrition pathways	26

Table SI. 16 – Parameters specific to transportation distances	27

Table SI. 17 – Background processes used specifically for transportation inventory	27

Table SI. 18 – Estimation of the chemical oxygen demand	27

Table SI. 19 – Marginal concentrated feed suppliers mix identification, per nutritional service	29

Table SI. 20 – Marginal roughage feed suppliers mix	30

Table SI. 21 – Nutritional value of marginal feed ingredients	30

Table SI. 22 – Parameters specific to food services	31

Table SI. 23 – Estimation of process operation’s risk mitigation potential for co-products.	31

Table SI. 24 – Parameters specific to CF pathways	32

Table SI. 25 – Background processes used specifically for CF pathways	32

Table SI. 26 – Parameters specific to SSF pathways	33

Table SI. 27 – Background processes used specifically for SSF pathways	33

Table SI. 28 – BSF pretreatment modeling parameters	35

Table SI. 29 – BSF seed larvae modeling parameters	36

Table SI. 30 – Main parameters used to model BSF growth	37

Table SI. 31 – BSF growth performance simulation for seven agrifood co-products	38

Table SI. 32 – BSF farming direct emissions modeling	39

Table SI. 33 – BSF farming stage utilities LCI modeling	40

Table SI. 34 – BSF killing and drying modeling parameters	40

Table SI. 35 – BSF frass composting modeling parameters	42

Table SI. 36 – BSF infrastructure proxy used	42

Table SI. 37 – Parameters specific to BSF pathways	43

Table SI. 38 – Background processes used specifically for BSF pathways	44

Table SI. 39 – Mycoproteins production assessment review, to date	46

Table SI. 40 – Illustration of the panel of mycoproteins production set-ups	46

Table SI. 41 – Review of lignocellulosic pretreatment performances, with a focus on IL techniques	48

Table SI. 42 – Heat balance modeling involved in IL pretreatment	49

Table SI. 43 – IL pretreatment modeling parameters	50

Table SI. 44 – Heat balance modeling involved in enzymatic hydrolysis	50

Table SI. 45 – Lignocellulosic enzymatic hydrolysis modeling parameters	51

Table SI. 46 – Parameters specific to ionic liquid and enzymatic hydrolysis pretreatment operations	51

Table SI. 47 – Background processes used specifically for lignocellulosic pretreatment operations	52

Table SI. 48 – Starch enzymatic hydrolysis modeling parameters	53

Table SI. 49 – Review and calculation of mycoproteins growth performances	54

Table SI. 50 – Medium formulation modeling parameters	55

Table SI. 51 – Mycoproteins composition	56

Table SI. 52 – Heat recovery modeling involved in mycoproteins production and post-treatments	56

Table SI. 53 – Parameters specific to mycoproteins production stage and downstream operations	57

Table SI. 54 – Background processes used specifically for mycoproteins production	57

Table SI. 55 – MP production assessment review, to date	58

Table SI. 56 – Main gas origin for HOB and MOB production, review	59

Table SI. 57 – Selected microbial proteins pathways	60

Table SI. 58 – WCS building and catalysis modeling parameters	62

Table SI. 59 – Methanation building and catalysis modeling parameters	62

Table SI. 60 – Electrolysis modeling parameters	63

Table SI. 61 – Heat recovery modeling involved in MP_meths pathway	63

Table SI. 62 – MP_meths pathway gas flows and energy performances illustrated for hardwood residues	64

Table SI. 63 – Heat recovery modeling involved in MP_biometh pathway gas pretreatment	64

Table SI. 64 – MP_biometh pathway gas flows and energy performances illustrated for bovine manure	65

Table SI. 65 – SR building and catalysis modeling parameters	65

Table SI. 66 – Heat recovery modeling involved in MP_shifts pathway pretreatment	66

Table SI. 67 – MP_shifts pathway gas flows and energy performances illustrated for hardwood residues	66

Table SI. 68 – Heat recovery modeling involved in MP_shiftb pathway pretreatment	66

Table SI. 69 – MP_shiftb pathway gas flows and energy performances illustrated for bovine manure	67

Table SI. 70 – Parameters specific to MP pathways pretreatments	67

Table SI. 71 – Background processes used specifically for MP pathways pretreatments	68

Table SI. 72 – Review and calculation of MOB and HOB growth performances	69

Table SI. 73 – Bioreactor cleaning modeling parameters	72

Table SI. 74 – Medium formulation modeling parameters	72

Table SI. 75 – MP post-treatments modeling parameters	74

Table SI. 76 – MP composition	75

Table SI. 77 – Heat recovery modeling involved in MP production and post-treatments	76

Table SI. 78 – Energy performance of MP production and post-treatments	76

Table SI. 79 – Productivity of the full second-generation MP production routes	76

Table SI. 80 – Calculated biomethane production compared to experimental benchmark	76

Table SI. 81 – Parameters specific to MP production stage and downstream operations	77

Table SI. 82 – Background processes used specifically for MP production and downstream operations	78

Table SI. 83 – Published LCA of green biorefinery pathways and assimilated	80

Table SI. 84 – GB pretreatment modeling parameters	82

Table SI. 85 – Benchmark of green juice extraction performances	83

Table SI. 86 – Selected parameters and variation ranges for green juice extraction	84

Table SI. 87 – Benchmark of green protein recovery performances	85

Table SI. 88 – Selected parameters and variation ranges for green proteins recovery	85

Table SI. 89 – Calculated green pellet composition and GB efficiency against benchmark	86

Table SI. 90 – Benchmarked valorization options for green biorefinery co-products*	87

Table SI. 91 – GB mass balances and distribution in final co-products	88

Table SI. 92 – Parameters specific to green biorefinery pathways	88

Table SI. 93 – Background processes used specifically for GB pathways	89

Table SI. 94 – GA pretreatment requirements per residual biomass category	91

Table SI. 95 – GA storage modeling	92

Table SI. 96 – GA standardization modeling	92

Table SI. 97 – Reviewed performances of air-based GA performances	93

Table SI. 98 – GA performances calculated on selected case studies	94

Table SI. 99 – Gaschar management modeling	95

Table SI. 100 – Syngas cleaning modeling parameters	96

Table SI. 101 – Parameters specific to GA pathways	96

Table SI. 102 – Background processes used specifically for GA pathways	97

Table SI. 103 – Technology parameters retained for the LCA model	99

Table SI. 104 – Influence of parameters on the different pathways	106

List of abbreviation

AD, Anaerobic digestion; BSF, Black soldier fly; C1,2,3, Category 1,2,3 (Animal by-product EU legislation); CCU, Carbon capture and use; CIP, Clean in place; CF, Compound feed; COD, Chemical oxygen demand; COP, Coefficient of performance; DM, Dry matter; DOL, Day of life; EF, Emission factor; EFSA, European Food Safety Agency; ER, Equivalence ratio; EU, European Union, FCR, Feed conversion ratio; FU, Functional unit; GA, Gasification; GB, Green biorefinery; HHV, Higher heating value; HOB, Hydrogen-oxidizing bacteria; HT, High temperature; IL, Ionic liquid; K, Potassium; KPI, key performance indicator; LCA, Life cycle assessment; LCI, Life cycle analysis; LHV, Lower heating value; LT, Low temperature; MOB, Methane-oxidizing bacteria; MP, Microbial proteins; Myco, Mycoproteins; N, Nitrogen; OFMSW, Organic fraction of municipal solid waste; OM, Organic matter; P, Phosphorus; PAT, Processed animal proteins; PCR, Protein conversion ratio; PFD, Process flow diagram; RME, Rapeseed methyl ether; RNA, Ribonucleic acid; SFU, Scandinavian feed unit; SNG, Synthetic natural gas; SOC, Soil organic carbon; SR, Steam reforming; SSF, Solid state fermentation; TBMP, Theoretical biomethane potential; TEA, Techno-economic analysis; TRL, Technology readiness level; UF, Ultrafiltration; WCS, Water conversion shift; WHR, Waste heat recovery; WWTP, Wastewater treatment plant.

# Feedstock selection and modeling

The Table SI. 1 lays down the residual biomass scope, classification and terminology further used in the manuscript, based on Javourez et al., (2021)2.

Table SI. 1 – Scope and classification of residual biomass streams

| Category* | Sub-categories* |
| --- | --- |
| Wood-related residual biomass | Primary forestry residues |
|  | Pruning residues |
|  | Wood-processing waste |
|  | Paper & packaging waste |
| Primary crop residues | Cereals residues |
|  | Oilseed residues |
|  | Protein crop residues |
|  | Maize residues |
|  | Non-food crop residues |
| Manure & slurry | Classified per livestock |
| Sludge** | Sewage sludge |
|  | Agrifood sludge |
| Green residual biomass | Garden and park waste |
|  | Unused grasslands |
|  | Vegetable tops |
| Slaughterhouse by-products | C1/C2*** |
|  | Meat production C3*** |
|  | Fishery and aquaculture C3*** |
| Agrifood co-products | Classified per agrifood sector |
| Food waste | Classified per generating sector |

*: Waste, residues and co-products terminologies as defined in Javourez et al., (2021)2. **: Wastewater is not part of the scope of the study, focusing rather on the final waste (here: sludge). ***: as defined in the Animal by-products regulation3. Subcategories in orange tinted cells were not retained for further analysis: paper & packaging waste are already managed by centralized supply chains to enhance recycling rates, and unused grasslands are consequences of land allocation decisions (not entering residual biomass definition). Organic/Inorganic composite streams (e.g. solid recovered fuel) were also discarded.

The identification of individual streams as well as theoretical resource potential estimation were established by inventorying, comparing and complementing data published by studies of similar scope including (but not limited to Javourez et al., 2022, 2024; Lodato et al., 2022; FranceAgrimer, 2021)1,4–6. The granularity of biomass composition modeling was selected to capture the main biochemical families (lignin, hemicelluloses, cellulose, ashes, proteins, lipids and carbohydrates) and elements (C, H, O, N, P, K, S), to allow differentiation between streams, as performed in Javourez et al., (2024)1. Further refinements such as the content of specific amino acids, fatty acids, tannins, or other compounds are not always documented for residual biomass streams, and are too dispersed to derive a representative average. This is because these are tightly linked with specific management practices, varieties and pedoclimatic conditions, among others. Biomass compositions were mostly derived from specialized databases7–9, ensuring completed mass balance (i.e. biochemical fractions summing 100%). When not available, elemental composition (C, H, N, O) were estimated through stoichiometric proxies per fraction as developed in Hamelin et al., 201410. At all time, we ensured to track back primary data source used by each reference in order to only account for different primary data sources. Single streams were further clustered per similar composition-structure to feed the case studies’ selection criteria (Figure SI. 1).

Figure SI. 1 – Composition distribution of the 132 identified residual biomass streams for France

Ternary diagram representing the diversity of biomass composition and highlighting common patterns between streams of the same category (Table SI. 1), as in Javourez et al., 20212. Structural content gathers cellulose and hemicelluloses fractions: these are dietary fibers with nutritional value for ruminant. Macronutrient content gathers proteins, lipids and carbohydrates usable by monogastric livestock (mostly free sugars and starch). Anti-nutritional content gathers ashes and lignin. The “other” fraction was divided equally between the three axes. The position of the eleven streams selected in section 2 are highlighted. Background data in A.Resource_pot spreadsheet.

Primary crop residues and wood-related biomass streams appear as clusters, which means that these categories are relatively homogeneous in terms of composition (mostly lignocellulose; Figure SI. 1). The main discriminant factor between these two categories is the lignin content (accounted as anti-nutritional in Figure SI. 1). On the other hand, lignocellulose-free biomasses are grouped close to the lower axis, notably gathering all the slaughterhouse by-products and sludge streams (except for sludge from the paper industry). In turn, the continuous space between lignocellulosic and non-lignocellulosic streams is fully covered. Manure & slurry streams are clustered in the middle, meaning that these still host non neglectable content of macronutrients (e.g. undigested fractions). This is also the case for green residual biomasses, for hosting free sugars and proteins (Rubisco) within their cellulosic matrix. Finally, agrifood co-products and food waste are heterogeneous families in terms of composition, mostly for being defined by generating sectors (Table SI. 1). From the 132 identified streams, 11 were selected as individual case studies (rationales in Table SI. 2), nevertheless representing around half of the total French residual biomass resource potential mostly due to wheat straw and bovine manure.

Table SI. 2 – Selection of eleven representative residual biomass streams for France

| Category | Subcategory | Representative stream | Comment / Selection rationale |
| --- | --- | --- | --- |
| Wood-related residual biomass | Primary forestry residues | Hardwood forestry residues | Highest theoretical potential of wood-related residual biomass in France11. Forestry and pruning residues host similar composition and valorization potentials so only one stream for both sub-categories was selected |
|  | Pruning residues |  |  |
|  | Wood-processing | Sawmill waste | Highest potential of wood-processing waste for France12. |
| Primary crop residues (PCR) |  | Wheat straw | Highest theoretical potential of PCR for for France11. |
| Manure & Slurry |  | Bovine manure | Highest theoretical potential of manure for France 11. |
| Sludge |  | Sewage sludge | No detail at the individual stream, so considered as generic urban wastewater treatment plant sludge. |
| Green residual biomass | Garden & park waste | Countryside management grass | Highest theoretical potential of garden & park waste identified in France13. |
|  | Vegetable tops | Sugarbeet tops | Highest theoretical potential of green crops tops identified in France5. |
| Food waste |  | Household food waste | Aim to be representative of future segregated organic fraction of municipal solid waste (OFMSW). Consumption stage (household) is where the highest amount of food waste is generated in France13–15. |
| Slaughterhouse by-products | C1/C2 | “PAT” | “Processed animal proteins”, typical output of slaughterhouse rendering processes (here C3 according to the EU legislation)3. See Box 2. |
|  | C3 |  |  |
| Agrifood co-products |  | Corn spent grains | Lignocellulose- and proteins-rich streams are already represented in other categories e.g. crop residues and PAT. Here, selected stream aim to represent feed-quality and nutritional-balanced co-products from the cereals sector4. |
|  |  | Potato screenings | Representative of starch-rich co-products4. |

Agrifood co-products category was relatively less represented by choice. This is because this category is made of highly disaggregated streams (around 85) compared to the other categories and because most of its abundant streams (e.g. sugarbeet pulp or wheat bran) are close to crop residues in terms of composition, hence not yielding any additional insights. Moreover, detailed analysis focusing on alternative management practices for agrifood co-products streams were already detailed in Javourez et al., (2022)4. Therefore, only two streams of this category were retained, representing feed-grade streams: distiller’s spent grains and potato screenings co-products (Table SI. 2). While capturing the heterogeneity of residual biomass streams in terms of composition, the eleven selected streams also cover different generation sites, pretreatment requirements and legislative backgrounds, hence providing a comprehensive overview of bioresource management conditions and constraints.

The residual biomass resource potential drew in this section aimed to picture the French context and specify the scope of this study, but many uncertainties remain when it comes to establish net values for streams’ composition, quantity and share of current use. This is due to the lack of data and the difference in the data quality between streams, leading to highly dispersed levels of aggregation (e.g. specific maize germ cake stream versus generic orchard pruning stream). Moreover, while the composition of feed-grade streams (e.g. agrifood co-products and primary crop residues) are already fully characterized by many specialized databases, scarce-to-no data is available for residual streams of secondary importance (e.g. carrot tops, small ruminants’ manure, etc.) or heterogeneous and site-specific streams (e.g. retail food waste, agrifood industries sludge, etc.). The same considerations apply for data on theoretical quantities, with scarce-to-no information on livestock manure other than swine, cattle and poultry sectors, on vegetable tops other than sugarbeet and potato, on bristle and feather generation and current use, among others.

# Waste-to-nutrition pathways selection

The objective of this section is to specify a qualitative selection rationale to focus the modeling efforts on relevant and plausible waste-to-nutrition pathways. This means, to refine the scope of the LCI models only to emerging supply chains likely to effectively enter food and feed markets within the time and geographical scope of this study (see main manuscript). Several structured tools exist to develop such screenings, like the PESTEL analysis16 to identify the external marketing factors that can impact the development of a project. Due to the explorative scope of the present study, we did not include political, economic nor social limitations. Instead, the potential acceptability of emerging technologies was estimated by positioning them in the landscape of current legislation.

## EU Legislative background

At the frontier of waste/biomass management and food production, the legislative body framing the development of waste-to-nutrition pathways is rapidly evolving, challenged by the conundrum of desired acceleration towards innovation for sustainable food systems while maintaining a precautionary principle. This permanent evolution is traduced by the moving terminologies frontiers of “waste”, “by-products”, “co-products” and “residues”, whose harmonization is still ongoing in the EU17. For example, local legislative bodies ought to identify and label secondary production streams as “co-products” whenever possible to unlock industrial symbiosis (e.g. by granting “end-of-waste” status) which would otherwise be prevented by the Waste framework regulation (Table SI. 3). Another harmonization attempt by the EU is, for example, the will to label as “product” any material considered as a feed ingredient, instead of by- or co-product (see the Catalogue of feed materials; Table SI. 3). The dynamic aspect of related legislation is also witnessed by the many modifications faced by the Animal by-products regulations body during the last decade; the latest change reintroducing authorizations for specific inter-species by-products recycling18.

Table SI. 3 – Reuse of residual biomass within food system: EU legislative background

| Name* | Official EU acts | Comment |
| --- | --- | --- |
| Waste/bioresource |  |  |
| Waste framework directive | Directive 2008/98/CE, modified by Directive EU 2018/851 | Terminology and categorization harmonization. Sets the rules for “end-of-waste” status as well as waste management hierarchy and responsibility in the EU |
| List of waste | Decision 200/532/EC, amended by Decision 2014/955/EU | Harmonized and classified list of materials labelled as waste in the EU |
| Circular economy action plan | Communication COM(2020) 98 | Part of the EU Green Deal, present next priorities for legislation including the enhancement of waste policies, circularities and markets for secondary raw materials |
| Human food |  |  |
| General food law | Regulation EU 2002/178 | Framework and requirements to control food chain’s products (including supplements) and safety, creates the EFSA and mentions need of “farm-to-fork” assessments |
| Novel foods regulation | Regulation EU 2011/1169 amended by Regulation EU 2015/2283 | Ensure food safety of novel products intended for human consumption, based on case-by-case EFSA risk assessments. Set a single list of novel food authorized in EU markets, the “Novel food catalogue” |
| Qualified presumption of safety (QPS) list | EFSA’ scientific opinion | List of microorganisms which does not raise safety concerns when integrated in food or feed formulations |
| Animal feed |  |  |
| Undesirable substance legislation | Directive 2002/32/EC | Sets maximum levels of undesirable substances in animal feed |
| Feed additives legislation | Regulation EC 2003/1831 | Sets procedures for animal feed additives EU markets authorization |
| Feed hygiene regulation | Regulation EC 2005/183 | Basis for animal feed hygiene compliance and traceability |
| Feed market legislation | Regulation EC 2009/767 | Harmonized framework and requirements for animal feed markets and use (safety, labelling, etc.). |
| Catalogue of feed materials | Regulation EU 2013/68 | Exhaustive list classifying names and definitions of authorized feed ingredients in the EU |
| Fertilizer** |  |  |
| EU Fertilizing product regulation | Regulation EU 2019/1009 repealing Regulation EC 2003/2003 | Sets categories and criteria for safe inclusion of residual biomass in fertilizing products based on “component material categories” |
| Sewage sludge directive | Directive 86/278/EEC | Sets the framework for reuse of sewage sludge in agriculture |
| Urban wastewater treatment directive | Directive 91/271/EEC | Sets the framework for urban wastewater management |
| Specific |  |  |
| Animal by-products regulations | Regulation EC 2009/1069 repealing Regulation EC 2002/1774. Also considered all amendments to Regulation EC 2001/999 (such as 2017/893 and 2021/1372) and Regulation EU 2011/142 | Sets of regulations laying down categorization and management practices for animal by-products, particularly when it comes to reuse as food, feed or fertilizer |
| Genetically modified (GM) food and feed regulation | Regulations EC 1829/2003 and 1830/2003 | Sets authorization, labelling and monitoring procedures for GMOs used as food and feed |

Non-exhaustive list of applicable EU legislation when it comes to reintroduce waste, residue or co-products directly or indirectly (as agricultural inputs) in the food chain, building on previous works by Delsignore & Siddiqui., 202217 and James et al., 202218. Subjected to changes (last update the 12/04/2023). *: as referred to along this study, but does not necessarily correspond to any official name of the juridical acts. **: more specific legislations exist at the country level.

The EU legislative background framing the reuse of residual biomass and/or the production of food and feed is strongly oriented towards safety/sanitary risk prevention and transmission mitigation. For example, regarding food production, the General food law enforces constant and comprehensive risk monitoring of the full food system (i.e. the “farm-to-fork” approach) by scientific authorities (here, the European Food Safety Authority; EFSA). Similarly, the Novel food regulation enforces the implementation of exhaustive safety/sanitary assessment of every novel food aiming to enter EU markets, and the body of animal feed legislations (e.g. Feed markets legislation) fixes the conditions under which the animals intended for human consumption can be fed (e.g. maximal amount of undesirable substances, feed management practices, etc.). Finally, risks are also mitigated at the agronomic level, with regulations (e.g. the EU Fertilizing product regulation) framing the reuse of residual biomass streams as agricultural inputs. In this sense, albeit formally prohibited to be directly used as food and feed, some streams (e.g. manure, sewage sludge, etc.) can nevertheless “indirectly” enter the food chain, under specific conditions. The legislative status of the residual biomass streams assessed in this study are further summarized in Table SI. 4.

Table SI. 4 – Residual biomass legislative status: an EU overview

| Residual biomass sub-categories |  | Legislative status |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | Waste a | Fertilizer b | Feed c |  |
| Primary forestry residues |  | x | No | x | both waste & feed-grade |
| Pruning residues | General/Hedges | x | No | No | Direct reuse prohibited |
|  | Vineyards/Orchards |  |  |  | Nowhere mentioned |
| Wood-processing waste |  | x | No | No | Direct reuse prohibited (exception if untreated) |
| Primary crop residues |  |  | x | x | Considered feed-grade |
| Manure/Slurry |  | x | x | No | Explicitly prohibited except in agronomy under controlled management plan |
| Sludge |  | x | Sewage sludge directive | No |  |
| Garden & park waste |  | x | No | No | Direct reuse prohibited |
| Vegetable tops |  |  | x | Only sugarbeet tops mentioned | Considered feed-grade under conditions |
| Municipal food waste |  | x | No | No | Direct reuse prohibited |
| Industrial & services food waste |  | x | No | Only catering and retail waste mentioned | Both waste & feed-grade. Animal by-products regulation also applies if animal products present |
| C1/C2 by-products |  |  | x | No | Specific rules of the Animal by-products regulation (Box 2) |
| C3 by-products |  |  | x | x |  |
| Agrifood co-products |  |  | x | x | Considered feed-grade |

Generic trend at the sub-category level, the reader is referred to A.Resource_pot spreadsheet for stream-level details. a: Mentioned in the EU List of waste. Empty if not explicitly mentioned. b: Mentioned as a “component material category” in the EU Fertilizing product regulation. “No” if explicitly discarded, and empty if not explicitly mentioned. c: Mentioned in the EU Catalogue of feed materials. “No” when explicitly prohibited by the Animal by-products regulation or the EU Feed market legislation. Empty if not explicitly mentioned in corresponding texts.

Overall, there is a relative consistency between the different EU legislative texts applicable to residual biomass streams (Table SI. 4). As a generic trend, streams considered as feed-grade do not host the waste status and also meet requirements to be reused as agricultural inputs. On the other hand, waste streams which are not granted with a “end-of-waste” status to be reused as agricultural inputs seems to be limited to streams currently incompatible with traceability standards (e.g. from municipal origins19). While the full detail at the individual stream level is available in the A.Resource_pot spreadsheet, there are two notable exceptions of categories which, despite being present in the EU List of waste, are also present in the Catalogue of feed materials: primary forestry residues and catering/retail food waste.

Regardless of the residual biomass categories, transversal rules also apply, such as the prohibition of fecally-contaminated material in feed formulations (Catalogue of feed materials). Similarly, many exceptions also exist as each novel (bioresource; valorization pathway) combination is individually assessed. Moreover, these principles and rules are far from being homogeneously considered worldwide, indicating the blurry lines and constantly evolving interactions between legislation, science, practice and technology development (e.g. Box 1). In a nutshell, while the origin and initial composition of residual biomass streams strongly conditions their legal acceptability to enter the food chain, their management practices (e.g. implementation of risks clearance barriers20, their traceability, etc.) and final target markets17 (as agricultural input, feed or food ingredients) are also decisive factors.

## Process intensity to mitigate risk transmission

While none of the residual biomass streams included in the scope were labelled as hazardous waste, the whole legislation body framing their reuse builds on their potential chemical and/or biological contamination risks (section SI. 2.1). The list of undesirable substances monitored in safety assessments of edible ingredients can be found elsewhere (e.g. in applicable regulations listed in Table SI. 3), and include substances from atomic (e.g. heavy metals) to macromolecular or biological structures (e.g. pathogens) scales, spanning the whole panel of micropollutants, allergens, carcinogens, mycotoxins and so one18,21,22. For example, the Animal by-products regulation builds on the challenge of mitigating the risks of prions (degenerative proteins) diseases transmissions and dioxins contaminations (Box 2).

On the other hand, risk mitigation measures enabling reuse of residual biomasses in food systems are already existing and implemented. These build on valorization chains including processes which act as risk clearance barriers20. For example, biological activity risks mitigation by sterilization/hygienization heat treatments, lime treatment (e.g. of activated sludge), composting (e.g. of municipal waste) are part of the considerations enabling the reuse of corresponding treated waste as agricultural inputs in the EU (while prohibited if untreated, see Table SI. 4). Similarly, proteolysis of animal by-products enables to mitigate prions transmission risks and participated to the slackening of eponym EU legislation in the past years. As noted by Verstraete et al., 201620, on top of technically mitigating risks, the implementation of various process steps also establishes a relative (and subjective) “distance” between the initial input and the final product which can favor public acceptance by lowering the GIGO effect (standing for “garbage in, garbage out”)23. As illustrated in Box 1 for the case of fecal-related inputs, many risk clearance barriers (i.e. conversion chains) for safe and accepted reintroduction of residual biomass into the food chain are already existing. These aim to modify the composition-structure properties of the input streams (i.e. generate the input-output “distance”), but also to mitigate risk transmission (e.g. removal of anti-nutritional compounds). Albeit culturally differentiated, the acceptability by legislative bodies of novel/emerging supply chains (in particular when it comes to safety risk mitigation) will likely depend on the type, the duration and the intensity of the conversion chains involved24.

Box 1: Examples of reuse of fecally-contaminated inputs in food systems

Manure and sludge are commonly reused as a fertilizing agent. Mushroom culture on wood-straw-manure substrates also illustrates the conventional reuse of manure to formulate food ingredients (according to the EU legislation, fertilizing plants or mushrooms is the same). The integration of fish pond with poultry production, commonly performed in some regions, is an example where fecal matter is reused directly as feed ingredient. Under specific conditions, initially fecally-contaminated materials can even reach European markets, such as the “Kopi Luwak” coffee beans. Examples retrieved from the waste-to-nutrition database established in Javourez et al., 20212.

Box 2: The Animal by-products regulation

The EU Animal by-products regulations defines three categories framing their reuse. Categories 1 and 2 (C1/C2) by-products are stemming from animals not intended for human consumption and/or hosting prohibited substances (e.g. veterinary), but also digestive tract and manure, among others. These are considered hosting intrinsic sanitary risks and need proper disposal (mostly eliminated through incineration or landfill). Albeit some exceptions exist (e.g. reuse of some streams as feed for circus or zoo animals under specific conditions), C1/C2 by-products are prohibited as feed. Category 3 (C3) by-products are only stemming from animal intended for human consumption and complying with applicable food standards. If not introduced within food markets, these can be reused as animal feed either for animals not intended for human consumption (e.g. pets), either for livestock. For the latter, reuse framework is differentiated per animal species group (e.g. swine, poultry, aquaculture, etc.) and mostly enforces proteolysis as part of rendering operations to inactivate risks, leading to transformed animal proteins (PAT) and tallow as final products.

To screen the plausible waste-to-nutrition pathways for further assessments, a selection rationale was established. It builds on the identification of the minimum level of processing intensity likely required to mitigate the specific risks associated with each residual biomass streams. The following considerations remain qualitative and only serve as a generic screening guideline.

First, the multidimensional panel of safety risks associated with residual biomass streams were simplified into three umbrella categories based on their legislative status (see Table SI. 4).

  - Streams under the EU waste status:

These potentially host a cocktail of safety risk concerns stemming from their intrinsic characteristics (e.g. sewage sludge and manure hosting pathogens, toxins, antibiotics residues, etc., but also treated wood) and/or the contamination risk associated with their incompliance with traceability standards (e.g. municipal waste). As defined, this category includes C1/C2 animal by-products categories. These are explicitly prohibited for direct reuse as feed ingredients in the EU.

  - Streams mentioned in the EU Catalogue of feed materials:

Providing their compliance with enforced management practices, these streams are considered risk-free to directly enter the food chain as feed ingredients. In some cases, these even meet food markets standards.

  - C3 animal by-products as a standalone category:

Indeed, despite being mostly food-grade, it nevertheless hosts specific transmission risks conditioning their reuse as animal feed (see Box 2).

Some agricultural residues are not explicitly mentioned in the reviewed legislation bodies (Table SI. 4). We considered here that their reuse risk level depends on field management practices, for example on the frequency and intensity of chemical inputs application, such as pesticides. Therefore, these are further assumed as feed-grade, considering that practices can evolve to emerging reuse constraints.

Second, the waste-to-nutrition description framework established in Javourez et al., (2021)2 was reused to estimate a process intensity level for each pathway. Indeed, these pathways were systematically expressed as a sequence of unit operations varying in nature and intensity, generically classified into four building blocks umbrella categories. These process blocks were adapted and ranked according to their risk mitigation potential. This was done acknowledging that some substances can be inactivated through generalized deconstruction (breaking them up to inoffensive compounds, such as prions into amino acids), while other cannot be inactivated through deconstruction (e.g. presence of heavy metals). Table SI. 5 summarizes the main rationale pairing process intensity and input risk level.

Table SI. 5 – Estimation of process operation’s risk mitigation potential for waste-to-nutrition pathways

| Waste-to-nutrition pathways a |  | Risk mitigation potential per input risk level |  |  |
| --- | --- | --- | --- | --- |
| Core unit operation | Pathways included | Waste | C3 by-products b | Feed-grade |
| Enhancement | Nutritional enhancement pathways | None | None, not enough to inactivate prions transmission risks when reused as feed. | No need: already considered safe enough for direct reuse as feed ingredients (when not prevented by other barriers such as digestibility) |
| Extraction | Proteins recovery from agrifood co-products, green residual biomasses and activated sludge | None c: current technical difficulties of refining and purifying complex solid structures or liquid matrices to reach food standards. |  |  |
| Bioconversion | Only direct bioconversion, rest of (indirect) bioconversion pathways act after cracking / Wastewater-to-nutrition pathways / Insects bioconversion | None d: risk of bioaccumulation and neoformed compounds. To add to the limitations of the extraction block. | None: precisely what aims to avoid current legislation, because prions are transmissible through direct bioconversion of animal by-products by other animals (difference between intra and/or inter-species recycling). Unknown risks for novel livestock (e.g. insects)18 |  |
| Cracking (to soluble nutrients) | Amino acids recovery from slaughterhouse by-products / Lignocellulosic feedstocks to sugars and dietary fibers / Liquid substrate alternative for microbial bioconversion | None: Risks remain latent in the lysed medium (small molecular scale toxicity remaining from the input and potentially neoformed), and not confidently mitigated by extraction or bioconversion as pre/post treatments (see corresponding lines) | Acceptable: proteolysis already considered enough to inactivate risks linked to this category |  |
| Cracking (to gases) | Gas-to-nutrition pathways | Acceptable: combined deconstruction e and phase separation. Volatile (light) compounds can be purified up to microbial production requirements with already high TRL techniques. Sanitary risks of the remaining solid/liquid phase as for the cracking (to solubles) case. | No need |  |

Summary of the potential of waste-to-nutrition conversion chains in mitigating safety risks associated with residual biomass streams. Those are qualitative considerations to provide a global picture. The risks linked with the processing activities themselves are not included here, these will be discussed case by case with the retained supply chains (section SI. 4). For this first screening, the technical and upscaling challenges of developing food-grade processing (e.g. for lignocellulosic fractionation) are considered not limiting25. TRL: Technology Readiness Level. a: Pathways core unit operation and families as defined in Javourez et al., (2021)2. The cracking building block was separated in two sub-blocks. b: Interpretation of current EU Animal by-products regulations, here considering feed markets. c: Mostly linked to the intrinsic a-priori unknown nature of the potential substances present within the input. d: Only reported exception: the use of streams compliant with the EU fertilizing product regulation for fungiculture, considered as an agronomy practice by EU legislation (i.e. fungiculture assimilated as crop culture). e: Anaerobic digestion consists in a complex chain of bioconversion operations, but here is considered as a black box converting organic inputs into biogas and digestate (hence from the gas standpoint, a cracking operation has been developed).

Third, the following principles were used to refine the selection of waste-to-nutrition pathways for further assessments. The pathways should preferably (in order):

- be based on unavoidable residual biomass streams (under current production patterns).

- upgrade current use of the target streams regarding its contribution to food system (according to the waste management hierarchy framework).

- ensure acceptable safety risk mitigation potential (as previously defined, see Table SI. 5)

- be designed around minimal processing principles (same as for green chemistry) to minimize risks of burden-shifting26.

- offer substitution of commodities associated with relatively important environmental impacts.

- present reasonably enough published data enabling the identification of a representative process workflow set-up and the elaboration of parametric LCI.

A detailed version of the outcome of the selection criteria is available in Table SI. 6, complementing the Table 1 of the main manuscript.

Table SI. 6 – Selection principles for waste-to-nutrition pathways

| (Pathways; feedstocks) combinations at the sub-category level |  | Selection principles |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Unavoidable | Upgrade current use | Risk mitigation compliance a | Target markets | Data availability |
| Nutritional enhancement (includes direct feeding) | Municipal food waste | Arguable | Agronomy to feed | No | Feed | Yes |
|  | Industrial & services food waste | Arguable |  | No need if no C3 |  |  |
|  | Primary crop residues | Yes |  | No need | Ruminant (roughage) feed |  |
|  | Garden & park waste |  |  | No |  |  |
|  | Vegetable tops |  |  | No need |  |  |
|  | C3 by-products (for food reuse) |  | Already performed when possible (acceptance) | No need | Meat-equivalent (animal proteins) |  |
|  | Agrifood co-products | Most streams | Only if feed to food |  | Stream- and pathway-specific |  |
| Proteins extraction | Sludge | Yes | Agronomy to feed | No | Protein feed | Low |
|  | Primary crop residues | Yes |  | No need | Protein feed (ruminant to monogastric) |  |
|  | Garden & park waste | Yes |  | No |  | Yes |
|  | Vegetable tops | Yes |  | No need |  |  |
|  | Municipal food waste | Arguable |  | No | Protein feed | Limited |
|  | Industrial & services food waste | Arguable |  | No need if no C3 |  |  |
|  | C3 by-products (for food reuse) | Yes | Only if feed to food | No need | Meat-equivalent (animal proteins) | Yes |
|  | Agrifood co-products | Most streams |  |  | Meat-equivalent (plant-based proteins isolates) | Stream-specific |
| Wastewater-to-nutrition | Wastewater | Out of scope (solid streams only) |  |  |  |  |
| Insects bioconversion | Manure/Slurry | Yes | Agronomy/Energy to feed | No | Protein and lipid feed | Yes, but on few species |
|  | Sludge | Yes |  | No |  |  |
|  | Garden & park waste | Yes |  | No |  |  |
|  | Vegetable tops | Yes |  | No need |  |  |
|  | Municipal food waste | Arguable |  | No |  |  |
|  | Industrial & services food waste | Arguable |  | No need if no C3 |  |  |
|  | C1/C2 by-products | Yes |  | No |  |  |
|  | C3 by-products | Yes | Only if feed to food | ? | Meat-equivalent (insects) / Snacks |  |
|  | Agrifood co-products | Most streams |  | No need |  |  |
| Amino acids recovery | C1/C2 by-products | Yes | Agronomy/Energy to feed | No | Protein feed | Limited for keratin |
|  | C3 by-products | Yes | Already performed when required | Yes | Protein feed |  |
| Lignocellulosic feedstocks to sugars and dietary fibers (includes further bioconversion) | Primary forestry residues | Yes | Agronomy/Energy to feed | No need | Either sugars, fibers or protein feed (microorganisms) | Yes (but limited for the full route towards proteins) |
|  | General/Hedges | Yes |  | No |  |  |
|  | Vineyards/Orchards | Yes |  | No need |  |  |
|  | Wood-processing waste | Yes |  | No |  |  |
|  | Primary crop residues | Yes |  | No need |  |  |
|  | Garden & park waste | Yes |  | No |  | Limited |
|  | Vegetable tops | Yes |  | No need |  |  |
|  | Municipal food waste | Arguable |  | No |  |  |
|  | Industrial & services food waste | Arguable |  | No need if no C3 |  |  |
|  | Agrifood co-products | Most streams | Only if feed to food | No need | Either sugars, fibers or meat-equivalent (microorganisms) |  |
| Gas-to-nutrition | Primary forestry residues | Yes | Agronomy/Energy to feed | No need | Protein feed (microorganisms) | Limited (yes for bioreactor stage, no for the integrated route) |
|  | General/Hedges | Yes |  | Yes |  |  |
|  | Vineyards/Orchards | Yes |  | No need |  |  |
|  | Wood-processing waste | Yes |  | Yes |  |  |
|  | Primary crop residues | Yes |  | No need |  |  |
|  | Manure/Slurry | Yes |  | Yes |  |  |
|  | Sludge | Yes |  | Yes |  |  |
|  | Garden & park waste | Yes |  | Yes |  |  |
|  | Vegetable tops | Yes |  | No need |  |  |
|  | Municipal food waste | Arguable |  | Yes |  |  |
|  | Industrial & services food waste | Arguable |  | No need |  |  |
|  | C1/C2 co-products | Yes |  | Yes |  |  |
|  | C3 co-products | Yes | Only if feed to food | No need | Meat-equivalent (Microorganisms) |  |
|  | Agrifood co-products | Most streams |  | No need |  |  |

Only (pathways; feedstocks) pairs with compatible composition-structure were included. The table was filled based on information reviewed in Javourez et al., (2021)2 and the pathway-specific literature review (detailed along section SI. 4). a: Risk mitigation and minimal processing requirements are merged (“no need” stands for: “from a risk transmission mitigation standpoint, the conversion chain is not necessary”). However, processing might be necessary to reach target markets in terms of functionality, organoleptic properties, etc.

First & second criteria (i.e. unavoidability and upgrade current use) were already mostly met by prescreened waste-to-nutrition pathways. Most of waste-to-nutrition pathways already target protein markets, either for food or feed purposes, which can reasonably be considered as high environmental footprints nutritional commodities. The direct reintroduction of C3 by-products as food ingredients (offal, etc.) was not carried further in the analysis for being already implemented whenever possible (limits are not technical but social and cultural). Proteins extraction from traceable food waste streams was not retained neither. This is because, in a context of increasing food waste reduction and prevention plans, the composition of these streams shifts towards unavoidable fractions, of lower nutritional quality and likely lower protein content, reducing incentives of developing such extraction. Similarly, no established proteins extraction pathways based on low-protein content primary crop residues were found to enable LCI modeling. The same applies for agrifood co-products and C3 by-products proteins extractions because these build on feedstock-specific process workflows. Direct livestock bioconversion was already captured through the CF pathway. Fungiculture (e.g. based on forestry residues or straws) was not modeled for already being a common practice with established performances. While there is still room for marketing innovations, the development of fungiculture was assumed limited by current mushroom demand. Cracking pathways culminating at the sugars production stage were discarded because corresponding markets are not the priority of this study (replacement of high environmental burden commodities). Therefore, only the pathways contemplating subsequent bioconversion of sugars into mycoproteins (Myco) were included. The production of edible microbial lipids was discarded for the lack of modeling data available. Cracking down safety-compliant food waste streams was not modeled due to the complexity of food waste structures and induced lack of detailed data on required post/pre-treatments, meaning that only lignocellulosic residues and agrifood co-products were considered as suitable feedstocks. Cracking down C3 by-products was not specifically modeled neither, as this operation is already performed as part of established rendering operations which yield hydrolyzed proteins considered as the starting point of present analysis (the PAT case). Moreover, data regarding novel techniques for keratin-rich products proteolysis are still limited.

Due to the exploratory endeavor of this work, and considering the blurry and moving lines of societal acceptance, some currently safety-incompliant (feedstock; pathway) combinations were nevertheless included (see * in Table 1 of the main manuscript). For example, untreated wood-processing waste (e.g. sawmill waste) could be considered as a suitable candidate for mycoproteins production, provided enforced traceability standards. The same applies for garden & park waste, as some of these streams could be seen as nearer to crop residues than to municipal waste (in terms of associated risks, such as countryside management grass). Finally, with the fast-evolving EU Animal by-products regulations, it was assumed here that BSF can fed on C3 by-products-containing feedstocks.

# Design and harmonization of the LCI models

The objective of the LCI models is to capture the inventory of elementary flows associated with novel residual biomass management practices as a function of context-dependencies and technological improvements, to support prospective environmental/economic assessments, which is the same objective as performed in Javourez et al., (2024)1, for conventional biomass management practices. Accordingly, the same goal, scope and modeling strategies were performed as described in the SI of the aforementioned study, further summarized. This strategy ensure consistency between all the pathways modeled in both studies, and unlock the reuse of already modeled pathways as building blocks for the others.

- The study is forward-looking (ca. 2030 horizon) and target the French situation (but is seen valid at the European scale providing some adjustments (e.g. modifying average temperature in the models).

- A consequential modelling was performed* (management of constrained resources and forward-looking analysis).

- The functional unit (FU) was set as: “The management of one wet tonne of a given residual biomass stream per year, delivered at gate”.

- A gate-to-grave system boundaries was selected, because (i) the study is comparative and (ii) compared systems differ from the waste’s generation gate.

- Multifunctionality was dealt with system expansion (where functions were expressed in terms of “services supplied”).

- Dynamic effects of emissions timing were neglected due to the preponderance of short shelf-life products in the studied systems (mostly edible ingredients and energy carriers). Some exceptions were dealt with separately and detailed in corresponding LCI description (e.g. biochar).

*: As present study stops at the LCI stage, the consequential modeling mentioned here only influenced the definition of feed services (as defined in section SI. 4.1.9) and the ecoinvent background database selection. All the elementary flows and processes mentioned afterwards remain valid regardless of the attributional/consequential approach of future studies.

An additional challenge was posed by the different TRL of selected waste-to-nutrition pathways, preventing their fair comparison16. This was dealt with by implementing a common inventory upscaling strategy, already presented in the main manuscript. Reviewed by the Planet Bio report16, upscaling is commonly performed by combining process calculation, extrapolation and proxies. The consistency of all upscaled LCI models was ensured by systematically implementing the following specifications.

- LCI calculation models are connectable.

This is because several studied systems are composed of common sub-systems. For example, anaerobic digestion is a standalone bioresource management option producing energy, but is also part of waste-to-nutrition pathways as pretreatment (e.g. biogas used for microbial proteins production) or co-product management (e.g. to treat the brown juice of green biorefineries). On top of ensuring harmonization of LCI models, the modularity allows to save modeling time when complexifying the systems, and enables future reuses of the present LCI in other studies.

- LCI calculation models are harmonized.

This means that unified modeling choices (e.g. granularity) and assumptions were consistently applied across all systems. For example, common unit operations within different pathways (e.g. centrifuge) were modeled the same way and using the same parameters across models.

- LCI models are flexible to input’s characteristics.

This means that inventories consider the composition-structure (including biochemical composition) and gate of the specific residual stream assessed. Thus, they can be applied to streams with different composition and are not bound to be used for one fixed composition only.

- LCI models are flexible to operating parameters as well as the many routes future technological performances can take.

This implies that for each technology, unit operation, process involved in the systems, the key parameters influencing the inventories and the outputs’ quality were identified, documented and integrated to the models.

A data selection rationale to document technology parameters was defined (Table SI. 7). Albeit alternative approaches to estimate values distribution with few data points might exist (e.g. pedigree matrices or the use of triangular distributions), these choice remains highly subjective when applied to prospective assessments. Acknowledging the exploratory endeavor of this study, the less restrictions possible to the achievable future technology performances were considered. These future technology performances were therefore only defined with min-max value ranges, and indicative baseline values were used for illustration purposes. The number of data sources used to derive min-max and baseline values for a specific parameter depends on the quality and quantity of data available for each specific pathway and/or unit operation, but was always based on at least two different sources (often: lab-scale data and assumed forecasted performance by stakeholders). This comes to implicitly consider that future performances will range between current performances and future expectations. No general trend for technological improvements were considered, and all parameters defined as independent. The underlying assumption is that technological improvement is context and technology-specific, and that it is as imprudent to develop a technology-optimistic approach than a technology-pessimistic one when conducting prospective assessments.

Table SI. 7 – Data selection hierarchy for technology parameters

|  |  | Data source |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | meta-analysis | review | experiment | assumption | no mention |
| Scale | Industrial | +++ | ++ | + | - | -- |
|  | Pilot | ++ | ++ | + | - | -- |
|  | Lab | + | + | - | - | -- |

Meta-analysis and dedicated reviews are primarily used, before deriving self-made averaged proxies, in line the Planet Bio report guidelines16. The parameters linked with emerging technologies which were only documented at the lab scaled or through practitioners’ assumptions were found to generally display higher variation ranges, as further described.

# Life cycle inventories modeling documentation

This section details the six (+ one) LCI models developed to provide the parametric life cycle inventories (LCI) of 16 selected waste-to-nutrition pathways. It lays down the main assumptions, rationales, calculations and references which were used. Each model is individually presented (sections SI. 4.2-4.8), after summarizing the common and transversal modeling items (section SI. 4.1). Importantly, for sake of conciseness, the models elaborated in Javourez et al., (2024)1 (that is: anaerobic digestion, composting, decay on land and direct combustion), albeit reused for some pathways (Table SI. 8) are not detailed again here. This also applies for the two models already established in Javourez et al., (2022)4, that is compound feed and solid fermentation models: only differences to original models are mentioned. A summary of the LCI models involved in each pathway’s LCI modeling is provided in Table SI. 8.

Table SI. 8 – Modular life cycle inventories of 16 waste-to-nutrition options

| Pathways acronyms | LCI models | Pathways description |
| --- | --- | --- |
| CF | CF | Direct inclusion in compound feed formulations for livestock |
| GB_AD | GB + AD | All the GB co-products are valorized through the AD_grid pathway |
| GB_rough | GB + AD + CF | GB’s brown juice is valorized through the AD_grid pathway while the press-cake is used as ruminant roughage |
| GB_mycofood | GB + AD + Myco | GB’s brown juice is valorized through the AD_grid pathway while the press-cake is sent to Myco_food pathway |
| BSF_feed | BSF | Farmed insects are directed towards feed markets |
| BSF_food | BSF | Farmed insects are directed towards food markets |
| SSF_feed | SSF | Fermented streams are directed towards feed markets |
| SSF_food | SSF | Fermented streams are directed towards food markets |
| Myco_feed | Myco | Produced mycoproteins are directed towards feed markets |
| Myco_food | Myco | Produced mycoproteins are directed towards food markets |
| MP_biogas | MP + AD | Microbial proteins are cultured on cleaned biogas |
| MP_biometh | MP + AD | Microbial proteins are cultured on biomethane |
| MP_shiftb | MP + AD | Microbial proteins are cultured on shifted biogas |
| MP_syngas | MP + GA | Microbial proteins are cultured on cleaned syngas |
| MP_meths | MP + GA | Microbial proteins are cultured on methanized syngas |
| MP_shifts | MP + GA | Microbial proteins are cultured on shifted syngas |

Terminology and variants selection as defined in the main manuscript. To model pathways involving several value chains, corresponding LCI models were paired and inventories are added up. CF, Compound Feed; GB, Green Biorefinery; AD, Anaerobic Digestion; GA, Gasification; BSF, Black Soldier Fly; Myco, Mycoproteins; MP, Microbial proteins.

The description of each models is organized as follows: (i) background/set-up selection, (ii) unit operation design and assumptions and (iii) summary of technology parameters and background processes involved. Mass, fractions, elements and energy flows are available directly in the excel spreadsheets, and were not mentioned in this document because these are parameter- and feedstock-dependent.

## Generic LCI unit operations

### Constants

Unless stated, the following constants were used for all the calculations (Table SI. 9).

Table SI. 9 – Calculation constants

| Constant | Value | Constant | Unit | Value |
| --- | --- | --- | --- | --- |
| Molar mass (g.mol-1) |  | g | m.s-2 | 9.81 |
| C | 12.011 | R | J.mol-1.K-1 | 8.314 |
| N | 14.007 | Pair | Pa | 101325 |
| H | 1.008 | γair |  | 1.41 |
| O | 15.999 | γsteam |  | 1.33 |
| S | 32.065 | Vmol | L.mol-1 | 24 |
| P | 30.974 | Hvap | MJ.kg-1 | 2.26 |
| K | 39.098 | ρwater | kg.m-3 | 993 |
| Air composition (%vol) |  | Tb, water | °C | 100 |
| N2 | 78.8% |  |  |  |
| O2 | 21.0% |  |  |  |
| Higher heating value (HHV; MJ.kg-1) |  | Specific heat (cp; kJ.kg-1.°C-1) |  |  |
| CH4 | 55.5 | Water |  | 4.2 |
| H2 | 141.8 | Steam |  | 1.9 |
| CO | 12 | Dry matter |  | 0.9 |
| NH3 | 22.5 | H2 |  | 0.01 |
| Lignin | 26.5 | CH4 |  | 0.19 |
| (Hemi-) Cellulose | 17.5 | CO |  | 1.04 |
| Lipids | 39 | N2 |  | 1.04 |
| Carbohydrates | 17.5 | O2 |  | 0.92 |
| Proteins & RNA | 24 | CO2 |  | 0.04 |
| Other organic matter | 17.5* |  |  |  |

Standard conditions were considered. *: assumed as carbohydrates as dominated by non-starch polysaccharides.

### Gases compression

Gas compression energy requirements were estimated, in the whole study, through the Reech equation for isentropic compression (Equation 1).

Equation 1

Ei=R×T×γMmol×(1-γ)×PoutPinγ-1γ-1

with Ei: isentropic compression energy (kJ.kggas-1); R: ideal gas constant (J.mol-1.K-1); Mmol: gas molar mass (g.mol-1); T: temperature (K);  Pout & Pin outlet and inlet pressures (Pa); γ: heat capacity ratio.

When N2 is present in significant levels in compressed gases, values of γ and Mmol were assumed as air. Compression (or blowing) energy were then calculated with the compressor yield (i.e. distance to isentropic conversion; Table SI. 10).

Table SI. 10 – Parameters specific to gas compression

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 29 | Gas compression yield | %kJ | 80% | 70-90% | Range based on review27, baseline in-between |

Numbering of parameters corresponds to the one of Table SI. 103.

### Liquid pumping

For all operations involving pumping of liquid streams, centrifuge pumps were considered. The volumetric energy was estimated through Bernoulli's equation, corrected with an assumed 10% load losses and 55% energy efficiency. To pump liquids towards a pressure enough to remain liquid after their boiling point; the corresponding pressure Pout was estimated through Clausius-Clapeyron equation (Equation 2).

Equation 2

lnPinPout=-Hvap1000×R×1Tvap,atm-1Ttarget

With Pout & Pin outlet and inlet pressures (Pa); Hvap: latent heat of evaporation (kJ.mol-1); R: ideal gas constant (J.mol-1.K-1); Tvap,atm & Tworking: boiling and working temperatures (K).

### Heating and cooling

Through the study, heating or cooling requirements to meet target temperature for a given unit operations (including steam generation) were established with Equation 3 and Equation 4.

Equation 3

If both Tin and Tout < Tb or > Tb (100°C for water streams)

E=Tin-Tout ×(m×cp)

Equation 4

If Tin < Tb and Tout > Tb (or vice versa)

E=Tout-Tb×cp,vap+Tb-Tin×cp,liq+Hvap×m

With E: sensible heat (kJ); m: mass of a component of the stream (kg); cp: specific heat at constant pressure of the component (kJ.kg-1.°C-1); Hvap: latent heat of evaporation (kJ.kg-1), Tb: boiling temperature (°C).

This calculation was adapted and applied for either gas, liquid or solids, using the constants presented in Table SI. 9. Average exterior temperature was considered in the whole study to be 13°C (France). This was used to calculate e.g. input air or water heating requirements, but also solid-liquid mixtures such as anaerobic digestion substrate. Heating and cooling requirements to manage exo- or endo-thermic reactions were calculated independently (section SI. 4.1.6.).

### Combustion enthalpy potential estimation

The combustion enthalpy of all the substances involved in biorefinery operations were estimated based on their higher heating value (HHV), with the Equation 5.

Equation 5

For a stream composed of i components (i.e. fractions)

EHHV=imi×EHHV,  i

With EHHV: higher heating value of the studied substance (MJ); mi: mass of fraction i (kg); EHHV,i: higher heating value of fraction i (MJ.kg-1), see Table SI. 9.

### Heat recovery modeling

Retained pathways involve processes both demanding and generating heat, under various form. Albeit direct heat synergies were already integrated in the models, excess heat to manage remains for some unit operations. It includes (i) sensible gas and liquid heat to evacuate, (ii) heat regulation during exothermic processes and (iii) co-products energy valorization (through combustion).

#### Waste heat recovery modeling

Sensible gas and liquid heat evacuation (i) and heat regulation of exothermic processes (ii) are usually achieved through cooling (passive or active) combined or not with waste heat recovery (WHR) systems. When reported as technically possible in the literature, WHR systems were considered in this study. However, an exhaustive review of WHR techniques application potential and performances was out of the present scope. The objective of this paragraph is only to provide the basis on which the WHR modeling assumptions were derived.

WHR potential and practical implementation depends mainly on the quality, volume, temperature and form of the heat source28. For example, for mid-high temperatures (ca. >600°C), WHR systems are usually based on passive techniques (e.g. heat exchanger surfaces, waste heat boilers to generate steam) but the harsh conditions imply specific materials and dispositions generating significant extra-CAPEX and maintenance requirements (pros and cons discussed in e.g. Table 2 of Vance et al., 201929 and Table 1 of Jouhara et al., 201828). On the other hand, for low temperature heat (ca. <60°C), WHR system are less efficient (due to lower temperature gradient) and are rather based on active technologies (e.g. heat pumps requiring pumping and compression of an intermediary agent28,30).

Therefore, similarly considered in Marami et al., 202231, two heat ranges were defined for WHR: LT (low-temperature; <100°C) and HT (high-temperature; >100°C). 100°C was selected as generally considered as the temperature threshold to use common (i.e. mechanically-driven) heat pumps32,33. HT is usually subdivided in groups corresponding to specific technologies29, but to simplify HT was here considered as one single group. Separation of LT and HT was also done because LT and HT heat markets are different (see SI of Javourez et al., 20241).

Additionally, albeit WHR-to-electricity systems are reported (e.g. Rankine cycles34), only WHR supplying a heating service was considered here. This is because, regardless of France deployment context, marginal electricity mix is less environmentally-intensive than marginal HT heat mix (SI of Javourez et al., 20241) on an energy basis. No infrastructure nor spare parts specific to WHR systems were considered in this study, likely a limitation to be considered in future works.

##### High-temperature waste heat recovery (HT WHR) modeling

To model HT WHR, only passive systems were considered, neglecting any extra energy needed. This choice was made because, even for the lower range of HT (<200°C), techniques such as absorption heat pumps already achieve a coefficient of performance (COP) higher than 4033. It means that the useful heat generated is 40 times higher than the extra-energy required by the WHR system. The share of HT heat streams effectively recoverable with WHR systems highly differs among the assessments. Seck et al ., 201335 estimated that a 70% WHR rate is achievable for the food industry, while McKenna et Norman, 201036 and Zuberi et al., 201837 considered that the technically achievable WHR rate for the industry sector in respectively United Kingdoms and Switzerland was 50% and 60% (based on expert judgments). Jouhara et al., 201828 reviewed WHR rates ranging (40-70%) depending on context and technologies, which could be theoretically increased towards 80-85% (Table 2 of the reference). Passive WHR systems yielding ca. 80% are indeed already reported in the literature (e.g. heat pipe heat exchangers38). Yet, Vance et al., 201929 mentioned that WHR systems achieving WHR rates higher than 50% were not available so far.

Therefore, in this study, HT WHR was modeled with a single parameter: the HT WHR rate, sizing the share of HT waste heat effectively recovered (Table SI. 11). HT heat consumers were assumed as not limiting, and integrated within the same industrial facility (that is, in an industrial symbiosis framework).

Table SI. 11 – Parameters specific to HT WHR

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 32 | Sensible HT heat reuse | %MJ | 80% | 50-80% | Lower range corresponds to currently available technologies29 while higher range correspond to theoretical limit28. Only technical consideration, optimistic baseline value. |

Numbering of parameters corresponds to the one of Table SI. 103.

##### Low-temperature waste heat recovery (LT WHR) modeling

LT WHR was modeled as active systems for the two-unit operations generating excess LT heat: (i) electrolysis (ca. 80°C) and microbial proteins bioreactor (30-40°C; sections SI. 4.5.5 and 4.6.4). While 80°C corresponds to common temperature used in district heating systems39, 30-40°C heat sources are usually considered as unrecoverable33. Indeed, temperature gradients between the source and sink drives heat pump’s COP. Recovering usable heat from such LT heat sources requires injection of extra-energy30. For example, as showed in e.g. Kreyenschulte et al., 201627, cooling of aerobic bioreactors represents important energy costs mainly composed by (i) water (or cooling agent) pumping (ca. 20% of cooling energy expenses) and (ii) the regeneration of cooling temperature (including the potential reuse of produced heat; ca. 80% of energy expenses).

Accordingly, to simplify LT WHR modeling, it was assumed that all the cooling-related energy expenses were gathered within the COP indicator. Similar studies modeling the LT WHR of mesophilic bioreactor already reviewed and used COP values ranging 3-740,41. These are in line with common COP values usually reported for mechanical heat pumps (ranging 2-532,33,42). However, LT WHR COP can be as low as 1.5-2 to recover LT heat lower than 50°C towards district heating requirements (values reported by Keil et al., 200843 for a composting plant integration project). Such LT heat sources (i.e. lower than 50°C) can nevertheless still help maintaining operational conditions of aquaculture ponds or greenhouses30. The same authors argue that the theoretical (exergy-based) WHR rate for a 35°C heat source is 65%. Moreover, they report that the distance to end-users is a critical aspect for LT WHR, considering ca. 9km as the radius threshold for an economically-feasible valorization of such heat (extra expenses and losses during transports via pipeline grids, ducts and associated auxiliary equipment44). Finally, the lack of suitable end-users and associated performance risks are often reported as the main barriers to the implementation of LT WHR37,45.

Therefore, in this study, LT WHR was modeled with two parameters sizing the energy required by the active WHR system and the overall share of waste heat effectively recovered (Table SI. 12). The cooling agent was assumed reused at a high rate (i.e. neglecting make-up and losses).

Table SI. 12 – Parameters specific to LT WHR

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 30 | Cooling COP | kW.kWe-1 | 5 | 2-7 | Compression heat pump, based on published reviews40–42. Baseline value as forecasted by the EU reference scenario46 |
| 31 | LT heat reuse | %MJ | 50% | 20-65% | Higher range as theoretical limit30, lower range and baseline assumed considering seasonality of LT heat demand (lower in summer) and required spatial integration. |

Numbering of parameters corresponds to the one of Table SI. 103.

#### Co-products enthalpy valorization modeling

The chemical energy useful for combustion embedded in solid and gaseous co-products was estimated based on a calculated proxy of their higher heating value (Equation 5).

Solid co-products direct energy valorization was modeled by adapting the “domestic heating pathway” modeled by Javourez et al., 20241 as follows. In-situ valorization was considered (no transportation), without standardization. It implies that the uncertainties related to emission factors (EF) were also neglected. Similarly, the industrial furnace efficiency was set as a technology parameter, and produced heat fully reused, as solid biomass can be stored and used only when required. The ash mixture was assumed either land-farmed or landfilled depending on the waste status or the input (as for gaschar, see Table SI. 99), and modeled with corresponding ecoinvent processes (Table SI. 102).

Gaseous co-products with significant LHV (here, only off-gases stemming from bioreactors, like CH4, CO, H2) were considered continuously valorized in-situ through an industrial gas boiler. Therefore, no additional pipeline network nor gas storage facilities were modeled. The in-situ valorization was assumed to supply the own industrial heat demand of the integrated facility, therefore avoiding HT heat supplies from corresponding markets. However, the continuous energy valorization potentially leads to HT heat generation when not demanded. Accordingly, a share of HT heat effectively reused was considered, assumed the same as for sensible HT WHR (Table SI. 13).

Table SI. 13 – Parameters specific to gaseous co-products energy valorization

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 32 | Sensible HT heat reuse | %MJ | 80% | 50-80% | Assumed as for HT WHR (linked to the level of integration of the industrial facility), see Table SI. 11 |
| 35 | Gas boiler efficiency | %LHV | 98% | 92-102% | Based on future forecasted range by the EU reference scenario46. Higher than 100% because based on LHV. |

Numbering of parameters corresponds to the one of Table SI. 103.

Associated inventory was directly estimated with a background inventory proxy on an LHV basis, as detailed in Table SI. 14. This simplification leads to a constant combustion emission pattern (here corresponding to methane combustion) regardless of the composition of input off-gases. The latter depend on many parameters, but mostly consist in a mix of methane, carbon monoxide and hydrogen, among other inert gases (e.g. partly exhaust syngas).

Table SI. 14 – Background processes used specifically for gaseous co-products energy valorization

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Heat production, natural gas, at boiler condensing modulating >100kW, Europe w/ Switzerland | Off-gases energy recovery | Process considered as a proxy for the whole infrastructure, utilities and emissions associated with the unit operation. It was adapted as follow: “Market for natural gas…” process was removed and direct emissions were changed from “fossil” to “biogenic”. Emitted CO2 was also adapted to close the carbon balance. |

All processes are from the ecoinvent 3.8 consequential database.

### Handling and transportation operations

Handling operations and transportation requirements were estimated based on the benchmark elaborated by Javourez et al., 20241 covering current bioresource management practices. These were assumed to remain similar (i.e. regional scale valorization) for emerging waste-to-nutrition and gasification platforms (Table SI. 15).

Table SI. 15 – Supply chain distances considered for waste-to-nutrition pathways

| Transportation | Pathway | Distance (km) | Range (km) | Comment |
| --- | --- | --- | --- | --- |
| Diffuse origin to collection platform | Solid fermentation | 40 | 20-60 | As in Javourez et al., 20241 |
|  | Mycoproteins production |  |  |  |
|  | Insects farming |  |  |  |
| SSF factory to compound feed factory | Solid fermentation | 50 | 10-100 | As pathway “use as feed ingredients” (considering SSF factory as a collection platform) in Javourez et al., (2022)4 |
| Compound feed factory to client |  | 50 | 10-100 |  |
| Green biomass to biorefinery | Green biorefinery | 40 | 3-75 | Assumed as for anaerobic digestion (Javourez et al., 20241), because fed on high moisture inputs |
| Collection platform to waste-to-nutrition pathways | Solid fermentation | 50 | 10-100 | Considered as current common distances for co-products valorization in France (63% travels 10-100km, 35% > 100km)12. |
|  | Mycoproteins production |  |  |  |
|  | Insects farming |  |  |  |
| From waste-to-nutrition facilities to clienta | Mycoproteins production | 50 | 10-100 |  |
|  | Microbial proteins production |  |  |  |
|  | Insects farming |  |  |  |
|  | Green biorefinery |  |  |  |
| Composted frass to field | Insect farming | 40 | 20-100 | As compost return to field in Javourez et al., 20241 |
| Collection platform to gasification | Gasification | 40 | 15-100 | Assumed as for incineration in Javourez et al., 20241 |
| Gaschar to valorisation |  | 75 | 50-150 | Current incineration slag valorization distance in France is 42km but as high as 150km in some regions47. |

a: the waste-to-nutrition pathways modeled are not considered to be sent towards regular compound feed / agrifood industry for inclusion in products, but rather sent directly to a final client (except for the SSF case).

Therefore, the following technology parameters of transportation distances were considered (Table SI. 16), and the corresponding background inventory was estimated as in Table SI. 17.

Table SI. 16 – Parameters specific to transportation distances

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 1 | Transport to CF facility | km | 50 | 10-100 | See Table SI. 15 and Javourez et al., 20241 |
| 2 | Transport to final client | km | 50 | 10-100 |  |
| 3 | Diffuse streams towards collection platforms | km | 40 | 20-60 |  |
| 4 | Diffuse streams towards AD/GB facilities | km | 40 | 3-75 |  |
| 5 | Collection platform to direct CHP/GA facilites | km | 40 | 15-100 |  |
| 6 | Collection platform to composting facilities | km | 33 | 12-54 |  |
| 7 | Transport of compost to land | km | 40 | 20-100 |  |
| 8 | Diffuse streams towards domestic furnace | km | 10 | 2-20 |  |
| 9 | Generation site towards industrial furnace | km | 50 | 30-150 |  |
| 10 | Digestate/slurry spreading distance | km | 10 | 3-50 |  |
| 11 | Collection platforms to waste-to-nutrition facilities | km | 50 | 10-100 |  |
| 263 | Sludge spreading distance | km | 21.5 | 5-40 |  |

Numbering of parameters corresponds to the one of Table SI. 103.

Table SI. 17 – Background processes used specifically for transportation inventory

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Market for transport, freight, lorry >32 metric ton, EURO6, RER | n.a. | Except when stated (e.g. cold chain management), proxy for all transportation requirements inventories. |

All processes are from the ecoinvent 3.8 consequential database.

### Wastewater treatment requirements

When data or proxy to model the wastewater treatment plant (WWTP) required for a specific stream was not available, the following estimation was implemented. First, based on the stream composition, the chemical oxygen demand (COD) was estimated (see Table SI. 18). Second, the closest WWTP already modeled in ecoinvent48 based on the treated COD value was selected as the background inventory proxy.

Table SI. 18 – Estimation of the chemical oxygen demand

| Fraction | calories.g-1 49 | gCOD.calories-1 50 | gCOD.g-1 (calculated) |
| --- | --- | --- | --- |
| Proteins | 4 | 0.26 | 1.04 |
| Lipids | 9 | 0.32 | 2.88 |
| Carbohydrates | 4 | 0.29 | 1.16 |
| Sugars | 4 | 0.29 | 1.16 |
| Fibers | 4 | 0.29 | 1.16 |
| Ashes | 0 | 0 | 0 |

The specific WWTP selected is therefore case-dependent, and is displayed where corresponding in the LCI models description.

### Estimation of the nutritional services of edible products and co-products

The novel food and feed ingredients supplied by waste-to-nutrition pathways can provide one or several nutritional services, hence interacting with different food and feed markets. This section summarizes how these nutritional services were identified and modeled.

#### General assumptions

Waste-to-nutrition directly targets food and feed markets, which need to be characterized. However, the panel of roadmaps towards sustainable food systems is broader than for energy systems, due to the multidimension of nutritional services, embedded within global markets51. For example, prospective scenarios of future food systems developments existing at the country (e.g. Afterres for France15) or regional (e.g. Europe52,53) scales screen highly differentiated future development options, mostly framed by the levels and orientations of political and societal wills (such as the deployment of agroecology practices). Therefore, relying on existing prospective food system assessment scenarios to estimate future market trends, as performed for energy (section SI. 4.1.10), was not developed here. Instead, a baseline food and feed markets context for each identified service was developed as current market trends (see below).

First, end-of-life effects of novel ingredients (consumption and post-consumption) were neglected. Accounting for these effects would require extended data on growth and metabolism patterns as well as defining inclusion rate in diets and drawing a baseline situation to measure the relative changes. Such consolidated data, particularly for novel food and feed ingredients, are missing to our knowledge. This means that the “gate-to-grave” scope of the waste-to-nutrition pathways LCA modeling (defined in section SI. 3) is actually a “gate-to-consumption”16. The sole post-consumption modeling was done to balance carbon flows, by considering that all carbon embedded in edible ingredients was released as CO2. Yet, all the edible ingredients produced were assumed formulated in a way these are seen interchangeable with the other suppliers providing the same services within the identified markets. This was done differently for food and feed markets.

#### Feed markets and services modeling

Feed markets were subdivided in concentrated feed and roughage feed. The latter corresponds to ingredients mostly directed towards non-intensive ruminant livestock production. Concentrated feed markets were further separated into protein, energy and lipid feed markets, corresponding to the main feed nutritional functions as performed in Javourez et al., (2022)4. Therefore, in total, the feed markets were simplified in this study as (i) protein feed, (ii) energy feed, (iii) lipid feed and (iv) roughage feed markets. While markets corresponding to concentrated ingredients can be considered as global51, this is not the case for roughage feed markets (low value and nutritional density) assumed specific to France here.

##### Equivalence in terms of feed services

The estimation of the concentrated feed services provided by edible ingredients was based on their macronutrient functions, as detailed in Javourez et al., (2022)4 and expressed in terms of Scandinavian Feed Unit (SFU) equivalent. The share of protein, lipids and energy services provided were estimated based on the contributions of their respective weight to the total SFU. The equivalent in terms of roughage feed service was directly derived based on the whole SFU of the fibrous stream. That is, the SFU of the fibrous stream was first calculated, and this stream was assumed to be equivalent to the weight of marginal roughage feed corresponding to the same SFU (Table SI. 21). This means that this modeling depends on the SFU of the representative ingredient on corresponding market, which needs to be identified. Here, we implement a consequential approach, therefore the marginal feed suppliers were identified (but the method can be adapted to an average approach as well).

##### Identification of marginal concentrated feed suppliers

The marginal feed suppliers identified in Javourez et al., (2022)4 based on last 10 years trend reported by FAOSTAT54 were kept. In summary, the suppliers providing (i) a protein feed service gathers oilseeds meal, pulse and animal by-products commodity groups, (ii) an energy feed service mostly includes cereals, roots and tubers, and (iii) a lipids feed service only contemplates vegetable and animal-based oil. Summary of the global trends and the selected marginal concentrated feed suppliers are summarized in Table SI. 19.

Table SI. 19 – Marginal concentrated feed suppliers mix identification, per nutritional service

| Protein feed services |  |  | Energy feed services |  |  | Lipids feed services |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Commodity (group) | Last 10 years trend, global (FAOSTAT54) | Corrected share | Commodity (group) | Last 10 years trend, global (FAOSTAT54) | Corrected share | Commodity (group) | Last 10 years trend, global (FAOSTAT54) | Corrected share |
| Soybean | 0% | 0% | Wheat | 24% | 24% | Palm oil | 50% | 50% |
| Soybean meal | 52% | 52% | Barley | 8% | 9% | Rapeseed oil | 2% | 2% |
| Other oilseeds | 17% | 0% | Durum | 0% | 0% | Soybean oil | 34% | 34% |
| Rapeseed meal | 7% | 24% | Maize | 67% | 67% | Sunflower oil | 13% | 13% |
| Sunflower meal | 13% | 24% | Rye | 0% | 0% | Fish oil | 0% | 0% |
| Pulses | 10% | 0% | Oats | 1% | 0% | Others | 0% | 0% |
| Fishmeal | 0% | 0% | Others | 0% | 0% | In FAOSTAT54, food and feed are aggregated, but OECD-FAO Agricultural outlook55 states that food use represents 65% of the vegetable oil demand. According to the same study, future trends on vegetable oil productions depend on many factors; for example, how major palm oil actors (e.g. Malaysia) will deal with oldling trees and limited room for expansion, among others. Current global marginal trends from FAO were kept. |  |  |
| Pulses and “other oilseeds” commodity groups gather many small contributions. These were discarded and rapeseed meal and sunflower meal were set to the same contribution share. Soybean meal remains the main marginal suppliers of protein feed globally, in line with projected trends by OECD-FAO Agricultural outlook55. |  |  | Maize is the major contributor to energy feed services, followed by wheat. This trend is similar to the OECD-FAO Agricultural outlook55 who projects that 50% of the global increase in cereals production will come from maize, followed by wheat (20%). |  |  |  |  |  |

See background data of Javourez et al., (2022)4 for calculation details.

Due to lack of data, emerging unconstrained feed sources such as microalgae or first-generation microbial production (defined in section SI. 4.6.4) were not included.

##### Identification of marginal roughage feed suppliers

Roughage feed markets were considered local; therefore, future marginal suppliers depend on the French food system roadmap. Roughage feed only contemplates grass and fodder crops (but not accounting for fodder maize which were already included in the energy feed category). Past trends based on Agreste data56 indicate a slight increase in French fodder crops production, while grasslands production oscillates. Looking forward in time, the different production scenarios modeled by Afterres15 all indicate an increase in legume grass and low-productive grasslands production, and a reduction of the other roughage feed suppliers (e.g. mixed and graminate grass, productive grasslands). This is mainly because in these scenario, French ruminant meat production is decreasing, following current trends54.

Accordingly, in front of the few data available and diversity of roughage feed production, all the ecoinvent processes representing France-like legume grass or low-productive grasslands production were retained, and marginal suppliers assumed as an average of the background processes categories identified (Table SI. 20).

Table SI. 20 – Marginal roughage feed suppliers mix

| Roughage feed |  |
| --- | --- |
| Commodity group | Share |
| Grass silage | 25% |
| Ryegrass silage | 25% |
| Alfalfa-grass silage | 25% |
| Ryegrass mixture silage (Egyptian & Persian clover) | 12.5% |
| Ryegrass mixture silage (Egyptian & Red clover) | 12.5% |

Proposed modeling enables to express any ingredient with its equivalent in terms of feed service this would supply if entering feed markets (Table SI. 21). Yet, this equivalent is a function of the suppliers which would be displaced (Table SI. 19 and Table SI. 20), requiring an additional layer of hypothesis.

Table SI. 21 – Nutritional value of marginal feed ingredients

| Marginal feed service | SFU.kgDM-1 |
| --- | --- |
| Protein feed | 1.10 |
| Lipid feed | 1.16 |
| Energy feed | 2.73 |
| Roughage feed | 0.60 |

SFU stands for Scandinavian Feed Unit. Rounded to two digits

#### Food markets and services modeling

The studied pathways provided novel foods targeting two different markets: the flour-equivalent (SSF_food)4 and the meat-equivalent (Myco_food, BSF_food). As described in corresponding model’s description, meat-equivalent is a category of product aiming at mimicking conventional meat products (also called meat substitutes). These can be formulated from basically all protein-rich (and food-grade) novel ingredients57, and are considered key to facilitate the transitions of meat eaters towards lower conventional meat consumption58. It is out of the scope of this study to draw a complete panorama of the dynamics driving the adoption of novel food in human diets. To simplify while still acknowledging that food provides many services beyond their strict nutritional values, a portion-based approach was adopted. This means that the equivalence was considered by direct weight, not by strict nutritional equivalency. This assumption is seen in line with the fact that, in the French context, there is already an overconsumption of proteins and calories which does not need full strict one-by-one nutritional equivalence59. Moreover, the novel foods providing a meat-equivalent services (insects and mycoproteins) already host a macronutrients composition not so far from conventional meat (that is, proteins- and lipids-rich). Accordingly, we considered that such ingredients were formulated in a way that they provide the same sensory properties and satiety than their equivalent in weight of meat portion. This means that the novel food is part of a standardized ready-to-eat formulation, so the equivalence ratio can be assumed on a dry matter basis (to avoid distortions due to different moisture content). This is the same than providing a standardized burger patty in weight from different primary ingredients60. The rest of the diet was assumed unchanged61. The ready-to-eat formulation was assumed to have the exact same consumption impacts (e.g. cooking operations) than the product they mimic, meaning that these operations were not modeled. All these simplifications require further refinements, particularly when it comes to the quantification of the effects of missing micronutrients in the meat-equivalent service (e.g. B12, iron)62. These same considerations applied for flour-equivalent services. To generically account for the uncertainty around the real adoption potential of novel meat-equivalent formulations, an equivalence ratio was defined as a parameter (Table SI. 22), as performed in Javourez et al., (2022)4 for the case of SSF_food.

Table SI. 22 – Parameters specific to food services

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 33 | Meat-equivalent ratio | %DM | 80% | 50-100% | Seen to depend on marketing strategies, organoleptic properties, socio-economic factors, among others. 100% represents the best case: the full equivalence of meat-equivalent services between novel food and conventional meat. On the other hand, the displacement paradox63 is documented enough for other cases (e.g. for meat64) to challenge the fact that the introduction of alternative products on markets will suppress the demand for the initial product. Here, a 50% equivalence rate was arbitrarily selected as a threshold where there is no utility for the consumer to buy the novel product. (0% would mean that the novel ingredient just adds to the diet without replacing anything). |
| 61 | Flour-equivalent ratio | %DM | 80% | 50-100% |  |

Numbering of parameters corresponds to the one of Table SI. 103.

### Energy and fertilizer services

These services were considered as defined in Javourez et al., 20241.

### General principles for reuse of the co-products

The rationale sustaining the selection of plausible reuse of co-products generated by the studied waste-to-nutrition pathways is summarized in Table SI. 23 and based on the section SI. 2.2. Specific technical choices are further detailed in each corresponding section.

Table SI. 23 – Estimation of process operation’s risk mitigation potential for co-products.

| Pathways | Co-product | Input risk level a | Reuse framework b |
| --- | --- | --- | --- |
| Insects farming | Frass | Waste | ? c |
|  |  | Feed-grade | Fertilizer: if complying with Animal by-products regulations and Waste framework directive (CMC 10/11) |
| Microbial proteins production | Gaschar (gasification-based pathways) | Waste | Not in food system: considered as “pyrolysis waste containing hazardous substances” (19 01 17 in the EU List of waste) |
|  |  | Feed-grade | Fertilizer: mention importance of biochar’s return to soil in EU fertilizing products regulation |
|  | Digestate (anaerobic digestion-based pathways) | Waste | Fertilizer (i) following Sewage sludge directive for sludge streams, (ii) following Animal by-products for corresponding streams (incl. manure), (iii) rest entering EU fertilizing products regulation (CMC 5) |
|  |  | Feed-grade |  |
| Green biorefinery | Brown juice | Waste | Fertilizer, entering CMC 2/11 (no presence in the EU Catalogue of feed materials) |
|  |  | Feed-grade |  |
|  | Press-cake | Waste | Fertilizer, entering CMC 2/11 |
|  |  | Feed-grade | Feed ingredient (6.6.2 in the EU Catalogue of feed materials), but also fertilizer (CMC 2/11) |
| Mycoproteins production | Lignin-rich cake | Waste or Feed-grade | Energy (lack of nutrients) |
|  | Press-cake (starch set-up) | Feed-grade | Feed ingredient: enzymatic hydrolysis is considered feed-grade operation by the EU catalogue of feed materials. |

a: As defined in section SI. 2.1
b: Terminology of Table SI. 3
c: It is still unknown if emerging supply chains based on waste can enter food systems even as agricultural inputs18

## Compound feed LCI model

The present Compound Feed (CF) LCI model directly builds on the CF model used in Javourez et al., (2022)4, but is amenable to a wider scope of residual biomasses (including crop residues). Therefore, corresponding documentation can be found in the aforementioned reference. The option to direct feed residues to livestock on farm was not specifically modeled, but would differ in terms of drying and transportation requirements4. This is a rather conservative assumptions for e.g. vegetable tops streams; as these would likely be used as roughage directly on farm.

The technology parameters retained for CF pathways modeling are displayed in the Table SI. 24.

Table SI. 24 – Parameters specific to CF pathways

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 12 | Humidity for grinding | %ww | 9% | 9-13% | Already detailed in Javourez et al., 20224, based on review of feed formulation processes. |
| 13 | Cabinet dryer energy | MJ.kgwater-1 | 5.6 | 5.2-6 |  |
| 14 | Cabinet dryer capacity | tww.h-1 | 12 | 5-12 |  |
| 17 | Grinding energy | kWh.tww-1 | 1 | 0.2-2 |  |
| 18 | Water loss at pelletizing | %water | 16% | 7-16% |  |
| 19 | Feed pelletizing energy | kWh.tww-1 | 20 | 15-28.2 |  |
| 20 | Humidity for pelletizing | %ww | 10% | 8-10% |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to CF model but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 16.

The background processes used specifically for this LCI model are gathered in Table SI. 25.

Table SI. 25 – Background processes used specifically for CF pathways

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Market for tap water, CH | Pelletizing | Compensate water loss and meet pelletizing requirements4 |
| Market for wood pellet factory, GLO | n.a. | Proxy of the CF facility, adapted per kgDM (0.00005 unit.kgDM-1) |

All processes are from the ecoinvent 3.8 consequential database. Direct flows to biosphere are displayed in B.CF_module spreadsheet.

Overall, only two studies were found (Albizzati et al., 202165 and Salemdeeb et al., 201766) with a similar FU and scope (i.e. the comparison of competing bioresource management options including direct feeding) albeit limited to food waste streams. Both highlighted the predominant contribution of the avoided feed services basket on the environmental impacts, albeit in their case they modeled an additional operation to hygienize food waste before feeding (considered as not safety-compliant here).

## Nutritional enhancement through solid fermentation LCI model

As for CF, the present Solid State Fermentation (SSF) LCI model directly builds on the SSF model proposed in Javourez et al., (2022)4, but is amenable to a wider scope of residual biomasses (including crop residues). Therefore, corresponding documentation can be found in the aforementioned reference. In a nutshell, streams are fist dried in-situ or at the collection platform if originally diffuse, with cabinet driers. Dried streams are then transported towards the SSF facility where they are successively standardized (size, moisture, N content, etc.) and sterilized (steam-based). Then, the SSF stage is carried out in an aerobic packed column bioreactor (30 days). Fungal growth was modeled with fraction-specific heterotrophic cell growth equations (here corresponding to the Pleurotus sp. strain). After fermentation, the substrate is dried towards the requirements of either flour mill or compound feed processing, depending on target market (SSF_food and SSF_feed pathways). If targeting food markets (food-grade streams only), SSF-based alternative flour supplies what is referred to as a flour-equivalent service4. Even after SSF, fiber-rich streams (e.g. crop residues) were still assumed to supply roughage feed services.

The technology parameters retained for SSF pathways modeling are displayed in the Table SI. 26.

Table SI. 26 – Parameters specific to SSF pathways

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 21 | Humidity for crushing | %ww | 9% | 9-13% | Already detailed in Javourez et al., 20224, based on literature review of each unit process involved. |
| 22 | Crushing dust generation | %ww | 0.5% | 0-0.5% |  |
| 60 | Water loss at crushing | %water | 0.15% | 0.1-0.2% |  |
| 48 | SSF moisture | %ww | 65% | 50-80% |  |
| 49 | SSF sterilization energy | MJ.kgww-1 | 0.84 | 0.4-1.3 |  |
| 50 | Fungal energy coefficient | % | 60% | 50-70% |  |
| 51 | Fungal protein content | %DM | 40% | 24.4-53.3% |  |
| 52 | SSF cellulose degradation | %cellulose | 40% | 2.4%-49.9% |  |
| 53 | SSF hemicellulose degradation | %hemicellulose | 40% | 16%-44.9% |  |
| 54 | SSF lignin degradation | %lignin | 40% | 11.9%-50.9% |  |
| 55 | SSF humidity degradation | %moisture | 10% | 3.2%-15.9% |  |
| 56 | SSF crude fiber degradation | %fibers | 20% | (-2.3)-28.7% |  |
| 57 | SSF free sugars degradation | %sugars | 70% | 16.1%-92.8% |  |
| 58 | Roller mill energy | kWh.tww-1 | 40 | 35-45 |  |
| 59 | Humidity for milling | %ww | 14% | 12-16% |  |
| 61 | Wheat flour replacement | % | 80% | 50-100% |  |
| All the CF model parameters (Table SI. 24) are also included in the SSF model |  |  |  |  |  |
| 62 | SSF digestibility enhancement: agrifood co-products | %OMdigestibility | 15% | (-10)-20% |  |
| 63 | SSF digestibility enhancement: crop residues | %OMdigestibility | 20% | 0-35% |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to SSF model but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15.

The background processes used specifically for this LCI model are gathered in Table SI. 27.

Table SI. 27 – Background processes used specifically for SSF pathways

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Market for tap water, CH | where required | Meet SSF moisture, sterilization and spawn production requirements |
| Market for compressed air production, 600 kPa gauge, >30kW, optimized generation, RER | SSF | Proxy of air blowing, background electricity replaced by the marginal mix of this study |
| Market for lime, RER | Spawn production | Part of the spawn substrate |
| Ethanol fermentation plant construction, CH | n.a. | Proxy of the SSF facility, considering same plant production and lifetime, output weight equivalent. |

All processes are from the ecoinvent 3.8 consequential database. Direct flows to biosphere are displayed in B.SSF_module spreadsheet.

Background processes and technology parameters associated with transversal unit operations between many models (e.g. pretreatments, transportation, etc.) were displayed in the common section SI. 4.1. For consistency with the rest of the models, N supply for substrate standardization was modeled as the marginal N fertilizer service. Similarly, cereals required for spawn production were modeled as the marginal energy feed service. Rest of modeling details (e.g. all biosphere flows) are available directly within corresponding excel model. Last but not least, no alternative comparable studies were found to confront the outcomes of the present SSF LCI.

## Insects farming on residual biomass streams LCI model

This section presents the details (including assumptions, choice rationales, calculations and data sources) of the insects farming (BSF) life cycle inventory (LCI) model, with the objective to perform the LCA of transforming bioresources into edible insects. The insects farming LCI model supporting both variants BSF_feed and BSF_food pathways is available in B.SSF_module spreadsheet.

### Set-up selection

Albeit insect species include a wide panel of physiologically-distinct species (e.g. crickets and termites), the unit operations workflow developed to achieve mass production is similar67, and comparable to conventional livestock farming. Indeed, insects are “mini-livestocks”68. Therefore, the main factors shaping the differences in the performance of insects farming between species are related to (i) different feed conversion ratios (FCR), (ii) growth time period and (ii) necessity of post-treatments (including difference in terms of final nutritional profile). Naturally, additional considerations on equipment and infrastructure are required if rearing flying species, but otherwise utilities are mostly considered similar, such as cleaning requirements or climate systems as insects are mostly poikilothermic69, among others. The legislative status in the EU are differentiated among species and target final markets, but rapidly evolving; BSF (fractions or as a whole) can already supply most of feed markets and its Novel food application is still ongoing (time of writing), while Tenebrio Molitor mealworms and Acheta Domesticus crickets were already granted with the Novel food status70. It is often considered that, albeit hosting different functional and nutritional properties, these insects belong to the same food product group for consumers71. Therefore, while still unauthorized to date in Europe, we considered here the scenario of food markets entry for dried BSF larvae as representative of generic insects’ inclusions into food markets (BSF_food pathway). Finally, it is worth mentioning that the wide literature body documenting BSF development and performances allow to build more robust LCI than for other insect species (e.g. lack of direct emissions data for Tenebrio Molitor mealworm and the Acheta Domesticus cricket).

Present BSF farming LCI model builds on the framework provided by Spykman et al., 202167, based on scaled installations set-up. In front of the high heterogeneity and complex interactions between insects and their rearing substrates, coupled with a lack of harmonized specific data, this analysis remains at the macronutrient level, and does not include details about fatty or amino acids, nor other elements than N, P and K.

### Unit operation design and assumptions

#### Pretreatments

Transportation requirements were modeled in Table SI. 15. As insects require a wet rearing substrate, and to avoid energy-intensive sequences of drying and moistening operations, input feedstocks were assumed transported “as generated”, without previous drying operations. A 20%ww input’s initial moisture threshold value was set to ensure stability during transportation and storage. If higher, a cold chain management was considered necessary to avoid spoiling and ensure compliance with food and feed standard. Storage was assumed to last 1-2 weeks. Losses and emissions were estimated based on the bioresource’s piling-up modeling used for the GA model (section SI. 4.8.2), proportionally adapting to the duration (Table SI. 28). Importantly, input streams able to enter the BSF pathway were generally assumed feed-grade (as defined in section SI. 2.1), that is source-segregated and contaminants-, packaging-free. No specific detoxification or de-packaging operation were further modeled. However, as a borderline case, streams currently under waste regulation but with potential near future plausible end-of-waste status were assumed obliged to undergo a decontamination stage, as in Spykman et al., 202167. Then, the rearing substrate was standardized in terms of size (towards 0.5-2mm) through successive comminution and crushing/grinding operations and moisture content (Table SI. 28). To do so, water was either added if moisture content was initially too low, or passive (gravitational) water draining towards target moisture content was assumed if the content was initially too high. The rest of utilities and operations involved in this stage were assumed as proposed by Spykman et al., 202167. Emerging pretreatment operations to increase BSF FCR or uptake of residual biomass feedstocks (e.g. fermentation)72,73 were not directly considered. These effects (and any techniques to include residues in BSF diets even at low rates74) were rather indirectly considered through the insects performance parameters.

Table SI. 28 – BSF pretreatment modeling parameters

| Parameter | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| Storage losses | %VS | 1% |  | As piling-up lignocellulosic biomass for 1 week (Table SI. 95), range ±15% |
| CH4-C storage | gCH4.kgVS-1 | 1 | 0.85-1.15 |  |
| Refrigeration | kWh.tww-1 | 3 |  | One week, only for perishable streams67 |
| Decontamination energy | kWh.tww-1 | 26.5 | 3-50 | Review of available technologies67, based as average. |
| Target feed DM | %DM | 30% | 20-38% | 30% often mentioned67,75, but possible range76,77 |
| Comminution energy | kWh.tDM-1 | 7.5 |  | 10 |
| Crushing energy | kWh.tww-1 | 17 |  | Average of crushing/grinding adapted from tDM |
| Inner transportation | kWh.tww-1 | 5 |  | As proposed by Spykman et al., 202167 |
| Mixing energy | kWh.tww-1 | 18 |  |  |
| Cleaning water | L.tww-1 | 670 |  |  |
| Cleaning detergent | kg.tww-1 | 0.838 |  |  |

Feedstock-specific handling in section SI. 4.1.7.

#### Seed larvae production

Insects reach their nutritional optimum before their reproduction period (e.g. for BSF, the prepupa stage is targeted). Therefore, a breeding and nursery cycle is required to maintain the delivery of starter larvae, usually called X-DOL (i.e. X days of life-old larvae). For BSF, 5-DOL larvae are usually considered mature enough to enter the rearing process. To ensure survival rates of hatching, during the initial days of life and pupation stage, a premium starter substrate is fed to the breeding and nursery operations. A standardized representative diets for such operation is the Gainesville fly diets which consists in 30% ground alfalfa, 50% wheat bran and 20% corn meal, by weight78. These ingredients were traduced in terms energy and protein feed service demand based on their nutritional composition, for consistency with the rest of the study (section SI. 4.1.9). The breeding and nursery cycles were designed based on the BSF-specific performance indicators (Table SI. 29) provided by the detailed operational guide of Dortmans et al., 201776. Residues (including frass, uneaten meal, dead flies, etc.) were estimated by mass balance, neglecting emissions, and considered mixed with the frass generated at the BFS farming stage (see below).

Table SI. 29 – BSF seed larvae modeling parameters

| Parameter | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| Starter feed mix | mgww.5-DOL-1 | 12 | 10-15 | At 30%DM, 65% protein feed and 35% energy feed |
| 5-DOL weight | mgww.5-DOL-1 | 6 | 2-10 | Range reported, depend on diet67,75,79 |
| 5-DOL moisture | %ww | 80% |  | 67,80,81 |
| 5-DOL cycled | %prod | 2% |  | See Dortmans et al., 201776 |
| Eggs weight | mg | 0.025 |  |  |
| Hatching yield | 5-DOL.gegg-1 | 40 |  |  |
| BSF eggs yield | eggs.female-1 | 600 |  |  |
| Hatching survival rate | %eggs | 70% |  |  |
| Pupae survival rate | % | 56% |  |  |
| FCR nursery | kgfeed.kglarvae-1 | 4 |  |  |
| Adult weight | mgww.adult-1 | 200 |  |  |
| Reproduction rate | %females | 70% |  |  |
| Electricity | Wh.5-DOL-1 | 0.12 |  | Aggregated from Spykman et al., 202167 |
| LT Heating | kJ.5-DOL-1 | 0.13 |  |  |
| Cleaning water | mL.5-DOL-1 | 1.9 |  |  |
| Cleaning detergent | mg. 5-DOL-1 | 2 |  |  |

#### Insect farming

##### Insect growth

This stage corresponds to the conversion of the standardized substrate into larvae biomass until reaching the pre-pupae stage, in a controlled environment. In an industrial context, when targeting food and feed markets, substrates supplied to insects are carefully tailored in terms of composition to meet their nutritional requirements, favoring FCR, survival rates and growth period length75. For example, high fibers content usually limits final larvae weight, increases growth period, and damages FCR82. Existing benchmarks of insect growth performances when fed specific residual biomasses streams also demonstrate that survival rates are shorten (e.g. down to 70-80% for sugar beet pulp or winery co-products83) and growth period before pre-pupae extended (up to 40-50 days for vegetable residues and fruit pomaces only83) compared to conventional diets. The same authors also showed the influence of the individual substrate composition on the final composition of the BSF larvae, which can therefore be indirectly monitored (e.g. yielding specific fatty acids84). This model considers that single residual streams will be part of a tailored substrate for insect rearing, for example resulting from a multi-objective optimization as implemented for common livestock feed formulation85. With these simplifying assumptions, the protein conversion ratio (PCR) and the number of 5-DOL required per weight unit of substrate remains the sole variables which adjust to the specific composition of input feedstock. In other words, the model aimed to provide a proxy of the effects to include a specific residual biomass streams within a tailored insect mass rearing diet. A recent study showed that manure-based ammonia (NH3) can be assimilated in the BSF larvae body into proteins86. Yet, it remains unclear if this assimilation resulted from the insect’s metabolism itself (transamination) or came from the action of external microorganisms acting as intermediates (synthesizing microbial proteins, further ingested as proteins by the insects). A review by Bar-Shmuel et al., 202087 arrived to a similar conclusion (synergy between diazotroph microbes and insects), acknowledging that the ability to directly fixed mineral N as proteins is restricted to a small group of insects which does not include BSF. Considering that (i) studied feedstocks to target food and feed markets mostly include streams hosting protein-bound N (hence low presence of ammoniacal nitrogen; manure being explicitly prohibited in feed) and (ii) rearing was assumed in a controlled and hygienized environment, limited in time (growth period of ca. 20 days), we considered that the proteins initially present in the feedstock were the only nitrogen source for insects’ development.

Additionally, insect’s PCR was found inversely correlated to the amount of protein in the rearing substrate82. While a minimum amount of proteins needs to be supplied to ensure BSF larvae’s development and competitive FCR (estimated around 5-10%DM of the diet), the authors showed that providing (relatively) more proteins was unnecessary as most of these will remain untouched or unconverted into insect proteins, somehow similarly to the common “barrel principle” of animal nutrition. Albeit depending on many factors and assumptions (e.g. nitrogen-to-protein ratio for insects, among others), common PCR values for tailored diets range between 25-60% for BSF. Therefore, this PCR value was set as a technology parameter and most of remaining performance indicators deduced from it. Table SI. 30 further summarizes the main assumptions used.

Table SI. 30 – Main parameters used to model BSF growth

| Composition (%DM) | 5-DOL79,81 | Finishing larvae |  |  |
| --- | --- | --- | --- | --- |
|  | Value | Value | Range | Comment |
| Moisture (%ww) | 80% | 64% | 60-70% | Technology parameter: reviewed range75 |
| Proteins | 45% | 40% |  | Fixed: average value83,88 |
| Fats | 9.7% | 34% | 25-43% | Technology parameter: own range review |
| Carbs | 0% | 1.4% |  | Fixed: Reported by Parodi et al., 2020 79 |
| Ashes | 15.9% | 18% |  | Fixed: average value77,89 |
| Others | 29.4% | 6.6% |  | Close mass balance (chitin, etc.) |
| C | 53.8% | 59.5% |  | Fixed: based on Parodi et al., 202079, nitrogen-to-protein ratio used was 5.76. |
| N | 9.7% | 6.9% |  |  |
| P | 1.8% | 0.9% |  |  |
| K | 1.8% | 1.3% |  |  |
| O, H, etc. | 32.9% | 31.4% |  |  |
| BSF growth parameter |  |  |  |  |
| Parameter | Unit | Value | Range | Comment |
| Protein conversion ratio (PCR) | %proteins | 38% | 25-60% | Own review |
| Survival rate | %5-DOL | 90% | 78-98% | Own review and 83 |
| Finishing larvae weight | mg.larvae-1 | 150 | 100-200 | Own review |
| Growth period length | days | 20 | 11-29 | Own review |
| Albeit diets were assumed tailored to BSF needs, these were considered mostly based on a mix of residual biomasses (framework of circular economy). Accordingly, selected parameters are slightly less performants than those usually reported for premium mass rearing diets (e.g. growth period of 10-16 days77; final weight close to 200mg90; etc.). Yet, selected parameters remain optimistic regarding usual reported performances of insect reared on individual residual or fibrous streams (see Table SI. 31). |  |  |  |  |

BSF protein content was mostly reviewed within the range (34-46%DM), while BSF fat content displayed a wider variation range (25-43%DM). This is why the former was considered as fixed, while the latter considered as a parameter. Full details on references and parameters are available in B.BSF_module spreadsheet.

To validate the model, a harmonized benchmark of studies (or reviews of studies) reporting BSF reared on agrifood co-products and food waste was elaborated (B.BSF_module). It yielded a dataset used not only to derive generic performance parameters, but also to compare experimental data obtained for a specific stream with the proxy calculated with this model. The results and comparison for seven representative agrifood co-products streams (the ones studied in Javourez et al., 20224) are presented in Table SI. 31.

Table SI. 31 – BSF growth performance simulation for seven agrifood co-products

| Parameter/Indicator |  | CSG | CPC | SBP | AP | FM | OP | PS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Benchmark | dataset | n=10 | n=1 | n=2 | n=2 | n=3 | n=3 | n=1 |
|  | FCR, ww | 4.1 | n.a | 20.5 | 4.6 | 7 | 4.6 | n.a |
|  | FCR, DM | 2.8 | n.a | 13.7 | 3.6 | 6 | 17.6 | 6.28 |
|  | Residues/substrate, ww | 43% | n.a | 39% | 39% | n.a | 95% | 11% |
|  | Residues/substrate, DM | 37% | n.a | 40% | 30% | 86% | 80% | n.a |
|  | Growth period length, d | 12.5 | 19 | 15 | 36 | 9 | 19 | 30 |
|  | Final weight, mg | 160 | 83 | 35 | 120 | 145 | 130 | n.a |
|  | Converted DM (1/FCR) | 4-28% 75,79; 6-28% (own review) |  |  |  |  |  |  |
|  | Degraded DM | 18-46% 75; 26-54% (own review) |  |  |  |  |  |  |
|  | Water retention in BSF | Only one measurement found: 10% 79 |  |  |  |  |  |  |
|  | Converted C | 8-20% 79,91,92, 66% by 93 |  |  |  |  |  |  |
|  | Converted P | Only two values found: 28% 79 and 53% 93 |  |  |  |  |  |  |
|  | Converted K | Only two values found: 14% 79 and 9% 92 |  |  |  |  |  |  |
|  | Total C emitted as CO2 | 9-13% 94; 24% 79; 37-62% 77 |  |  |  |  |  |  |
|  | Total water evaporated | Only two values: 67% 77 and 73% 95 |  |  |  |  |  |  |
| Estimated with the model | FCR, ww | 4.4 | 3.4 | 15 | 16.3 | 7.6 | 11.9 | 16.5 |
|  | FCR, DM | 3.6 | 2.8 | 12.3 | 13.4 | 6.2 | 9.7 | 13.5 |
|  | Residues/substrate, ww | 24% | 17% | 40% | 40% | 33% | 38% | 40% |
|  | Residues/substrate, DM | 51% | 37% | 86% | 87% | 71% | 82% | 87% |
|  | Growth period length, d | Fixed to 20 days, see Table SI. 30 |  |  |  |  |  |  |
|  | Final weight, mg | Fixed to 150mg, see Table SI. 30 |  |  |  |  |  |  |
|  | Converted DM (1/FCR) | 27% | 35% | 8% | 7% | 16% | 10% | 7% |
|  | Degraded DM | 51% | 37% | 86% | 87% | 71% | 82% | 87% |
|  | Water retention in BSF | 20% | 26% | 6% | 6% | 12% | 8% | 5% |
|  | Converted C | 32% | 39% | 10% | 9% | 21% | 12% | 10% |
|  | Converted P | 30% | 24% | 70% | 59% | 13% | 10% | 26% |
|  | Converted K | 34% | 32% | 22% | 14% | 15% | 13% | 8% |
|  | Total C emitted as CO2 | 42% | 51% | 13% | 11% | 27% | 15% | 13% |
|  | Total water evaporated | 68% | 65% | 74% | 74% | 72% | 73% | 74% |
| Comparison on a wet basis is difficult as substrate’s moisture is not always reported during experiments and are highly variable between studies. Moreover, for most case studies, relatively few data are available and these were not established in the same conditions, widening variation ranges (not displayed here). For example, intrinsic bias is induced if comparing studies whose objective was to assess a specific parameter, with studies rather aiming to simulate real rearing conditions. For example, the wide difference in terms of growth days can be explained with the fact that some studies fixed this value, while others considered it ended when 10-20-30% of the larvae entered the prepupae stage. Such different objectives also directly influence larval weight at the end of the experiment. Here, the 150mg value retained corresponds to the higher range of values reviewed for finishing BSF larvae reared on residual substrates (i.e. quality residual biomass such as CSG), which can be seen as slightly optimistic. FCR, DM is mostly coherent with or lower (hence more optimistic) than benchmarked data, except for AP and PS. In the former case, the two experiments reviewed involved AP included within a diet, and not alone, hence explaining why parameters achieved quality substrate performances. For the PS case, the only reported data was based on untraceable proxies (not own experiment). For the other streams, FCR, DM is of the same magnitude than experimentally observed, or lower (OP). The amount of remaining residue (DM basis) tends to be higher than reviewed benchmarked for some biomasses (e.g. SBP, AP), for the same reasons. Another reason lies in the ways CO2 emissions were modeled, proportional to insect biomass hence assuming a linear heterotrophic growth equation while emissions dynamics might vary depending on the specific substrate composition. This is further highlighted with the wide range of reported CO2-C emissions reviewed ranging 9-62%C. In the model these seems limited to <50%, but directly linked with initial proteins content. Water balance is coherent with benchmarked value, with 6-26% of initial water being retained in insect biomass and 65-74% evaporated. |  |  |  |  |  |  |  |  |

CSG, Corn Spent Grains; CPC, Canola press-cake; SBP, Sugar beet pulp; AP, Apple pomace; FM, Flour mill co-products; OP, Olive press-cake; PS: potato screenings. Converted N not displayed as fixed in this model (see Table SI. 30). Background data in B.BSF_module spreadsheet. Model calculation established with baseline values.

The large variation ranges of reviewed PCR and FCR (feed conversion ratio) are a consequence of diverging approaches regarding the role of BSF rearing facilities in the literature. Some consider insects farming as a waste treatment operation, while others as optimized food or feed production processes, and both approaches are hardly reconcilable in practice. In reviewed studies, high PCR were mostly achieved when BSF were reared on feed quality substrates, while lower PCR were achieved when including a higher share of fibrous residues in BSF diets. From a feed production standpoint, Roffeis et al., (2020) already noted that the induced environmental benefits of increased PCR/FCR of BSF reared on quality raw substrates offset their higher environmental burden, which call for strategies to optimize insect rearing on residual streams 97. For example, one leverage is to use insect’s short lifecycle to enhance substrate-specific BSF conversion efficiencies over many generations 95,98. The harmonization of insect rearing experiments, their systematic methodological transparency (e.g. providing unaggregated data), and the consolidation of existing databases will likely altogether allow to draw robust correlation between insect growth parameters, growth conditions and substrate’s composition19,78. Existing correlations e.g. lipids positively and fibers negatively influencing waste reduction rates99; lignin and hemicellulose negatively correlated with larvae growth rate75 should be complemented to draw enhanced prediction models, but this was out of the scope of this study. Similarly, pretreatment strategies could further enhance bioconversion of residual substrates through e.g. fungal fermentation, enzymatic treatment, ammonia treatment, etc.100,101, but consolidated data are required. Micronutrients, albeit key to ensure larvae development (e.g. calcium102) were not accounted in this study as assumed to stem from the rest of the diet.

##### Insect direct emissions

Insects rearing generates direct emissions stemming from residues, frass and insect’s own metabolism. Yet, studies documenting these emissions are limited (seven found) and displayed wide variation ranges. This is in line with the recent BSF greenhouse gas emissions review of Boakye-Yiadom et al., 2022103. Following the suggestion of Spykman et al., 202167, emissions were all modeled per dry weight of finishing larvae. Table SI. 32 displays the main assumptions regarding direct emissions modeling.

Table SI. 32 – BSF farming direct emissions modeling

| Emissions | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| CO2 | kgCO2.kgDM-1 | 2.75 | 1.5-4 | benchmarked range of 3.6 (SD 2.7). However, applied to the model, this range lead to CO2-C emissions in the higher range of benchmark (50-70%; see Table SI. 31). Accordingly, the range of 1.5-479,91,101 was selected as representative of cumulative CO2 emissions by BSF during rearing. The share of CO2 emitted by microbial activities on the substrate was assumed included within these values (only representing ca. 3% of total CO2 emissions in similar context)94. |
| CH4 | mgCH4.kgDM-1 | 28 | 9-95 | Base as average of reviewed values. NH3 selected to fit established correlation with NH3 emissions79. |
| N2O | mgN2O.kgDM-1 | 106 | 7-215 |  |
| NH3 | gNH3.kgDM-1 | 3.8 | 0.1-8.0 |  |
| N emission (rest) | %N | 4.4% | 0.7-8.0% | Estimations of volatilized N during insects rearing on protein-bound N substrates (excluding manure) were only reported by three studies: 0.7% 79, 8% 95 and 50.9% 77. It was assumed that near-future rearing strategies will strive to limit N losses: hence the lower range 0.7-8% was kept as representative of insects farming. These were fully accounted as N2 (by difference with N2O and NH3). |
| PPM10 | kg.tww-1 | 0.15 |  | Adapted from Roffeis et al., 202096 |

No information on other C-emissions were found in the literature benchmarked (e.g. CO). Background data in B.BSF module spreadsheet. It should be noted that albeit defined per dry weight of larvae, estimated N emissions remains proportional to the N initially present in the substrate by construction, as larvae yield is linearly linked to the substrate’s protein content in this model.

##### Residues and frass generation

In practice, it is hardly possible to differentiate insect’s frass (i.e. manure) from unused substrate. Here remaining substrate and frass was deduced by mass balance only (that is, per weight, residues = substrate + 5-DOL – larvae – emissions, for each fraction including C, N, P, water, etc.). The only fixed parameter was the final residue moisture, ranging 25-50%ww in the literature reviewed. Of similar content (i.e. organic residues, frass, etc.), residues from the breeding and nursery stages were added. Separation of the larvae from the substrate was assumed by sieving, and larvae washed with water.

##### Climate control, cleaning and other utilities

Inventories were mainly derived from guidelines of Spykman et al., 202167 based on industrial data of Smetana et al., 201978. Indeed, authors mentioned that their assessment were based on a Protix plant based in the Netherland, which was assumed to have similar pedoclimatic conditions than France (no precise data found for the French context). Dynamic modeling of seasonal climate cycles combined with insect’s metabolism could yield tailored estimation of ventilation and heat needs for insect rearing, but this was not part of the scope of the present study. The Table SI. 33 displays the rest of the assumptions used to model LCI of the BSF farming stage.

Table SI. 33 – BSF farming stage utilities LCI modeling

| Emissions | Unit | Value | Comment |
| --- | --- | --- | --- |
| Cleaning water | L.tww-1 | 340 | Aggregated values from Spykman et al., 202167 |
| Cleaning detergent | kg.tww-1 | 0.42 |  |
| Inner transportation | kWh.tww-1 | 54 |  |
| Climate system electricity | kWh.kgww-1.day-1 | 0.032 |  |
| Climate system LT heat | kWh.kgww-1.MJ-1 | 0.109 |  |

#### Post-farming operations

Post-farming operations include killing, drying and feed/food ingredients formulation. Larvae were assumed fasted to clear their digestive tracks104 albeit not specifically modeled herein. Importantly, no fractionation of BSF larvae into its different functional compounds (i.e. protein meal and oil) was modeled. Indeed, many processes exist104, but all induce additional environmental impacts while not modifying the aggregated final mix of avoided food and feed services as modeled in this study. Consideration to fractionate/refine or not obviously depend on final target markets, but also on local legislation (e.g. in EU). Here, such additional processes were not modeled (optimistic regarding environmental impacts), and food and feed formulators were assumed supplied with whole larvae.

##### Killing and drying

Spykman et al., 202167 reviewed existing technologies for larvae killing and drying, used here to establish ranges of energy consumption (Table SI. 34). For consistency with the rest of the LCI models, BSF larvae was considered dried towards 91%DM.

Table SI. 34 – BSF killing and drying modeling parameters

| Emissions | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| Sieving and washing energy | Wh.kgww-1 | 16 |  | Aggregated values from Spykman et al., 202167 |
| Washing water | L.kgww-1 | 0.98 |  |  |
| Cleaning water | L.kgww-1 | 0.84 |  |  |
| Cleaning detergent | g.kgww-1 | 1.2 |  |  |
| Killing energy | kWh.kgww-1 | 0.14 | 0.09-0.25 | Review by Spykman et al., 202167, freeze-drying selected as intermediary. |
| Drying electricity | kWh.kgwater-1 | 0.11 | 0.03-0.28 | Reviewed drying requirements by Spykman et al., 202167 yield 5-6 MJ.kgwater-1, higher than spray-drying techniques. This is likely because here the larvae are dried as a whole, and not crushed into powder/meal before, hence limiting heat and water evaporation transfers. Here, parameters selected as for MP production model (spray-drying). |
| Drying heat | MJ.kgwater-1 | 3 | 2.7-4.1 |  |

##### Inclusion in diets

###### Targeting animal feed markets

BSF stakeholders target value added animal feed markets, such as aquaculture feed105 or petfood84. Here, we explain why only generic modeling of feed was considered as in the rest of the study. First, the rationale of entering aquaculture markets is to provide an alternative to fish meal and oil, which are already stagnating resources106. Accordingly, BSF would rather compete with the other rising (unconstrained) alternatives to fish meal and oil, such as algae or soy extracts106. In turn, this lead to avoid marginal suppliers of generic feed services such as the ones described in section SI. 4.1.9. Second, there is no clear trends on which petfood ingredients are avoided by BSF larvae from the literature reviewed. Petfood market is mainly based on ready-to-eat formulations, either wet or dry. The main European petfood formulator syndicate mentions that they usually formulate petfood preparations with slaughterhouse by-products (as these were historically banned from livestock feed in the EU), hunted animals, fishes and seafood, vegetable proteins (e.g. soya) and finally cereals and tubers107,108. Many different formulations exist, from whole vegan mixes up to premium petfood including raw pieces of meat. Indeed, it is acknowledged that petfood mainly follows the same trend than human food109. Therefore, considering global petfood markets, ingredients likely substituted by BSF larvae would be vegetable proteins, cereals and tubers which are the only non-constrained raw feedstocks, and therefore correspond to feed services as defined in this study. On the other hand, as a best case, BSF larvae inclusion in the premium petfood market segment could substitute meat-equivalent services, which yields the same results than considering BSF as an alternative to meat for human consumption (therefore entering BSF_food variant, see next section). Accordingly, only generic BSF inclusion in animal diet was contemplated, modeled as the CF pathway (section SI. 4.2). Already performed here, the drying stage of CF pathway was not included. Final larvae composition was provided in Table SI. 30.

###### Targeting human food markets

To enter food markets despite the absence of insects in the European culinary traditions, stakeholders developed a wide panel of strategies mostly targeting the inclusion of BSF in common food products (such as snacks, meat or milk analogues, to fortify bakery products, etc.), (Borges et al., 2022; Gravel and Doyen, 2020; Poortvliet et al., 2019; Tello et al., 2021). The formulation of meat-analogue products appears in prime position (Tan, 2017). Indeed, insects not only host meat-like nutritional profile (such as proteins and fatty acids profiles and contents), but also are considered as “animal proteins”, which position them as key ingredients to formulate meat-analogues. Insects were also found to present favorable organoleptic properties over plant-based alternatives (for example to formulate meat paddies analogues; Smetana et al., 2021). See section SI. 4.1.9 for further details on the modeling.

#### Frass management

Frass residues stemming from BSF rearing and breeding stages were considered mixed prior composted. These residues are acknowledged for their potential as organic fertilized materials 110, but usually require stabilization beforehand for presenting features comparable to immature compost 77.

##### Frass composting

Guo et al., 202177 proposes a 2 months composting stage and provided the corresponding utilities and emissions inventory, considering N2O and NH3 emissions proportional to the amount of input N. This inventory was refined with data of existing industrial composting facilities shared by Albizzati et al., 202165. This was done because frass composting emissions were mentioned as an important source of environmental impacts in previously published assessments77,90. While being mostly based on the composting pathway proposed in Javourez et al., 20241, the parameters were adapted to reflect the specificity of frass (shorter period required, etc.; see Table SI. 35).

Table SI. 35 – BSF frass composting modeling parameters

| Parameters | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| Frass composting electricity | kWh.tww-1 | 26 | 0.9-50 | In between reported for frass composting77 and real composting plant65,111 |
| Frass composting C-emissions | %C | 20% | 10-35% |  |
| Frass composting CH4 emissions | %C_emit | 1% | 0.2-1.7% |  |
| Frass composting N2O emissions | %N_emit | 1.2% | 0.5-1.9% |  |
| Frass composting N emissions | %N | 22% |  | Plant studied by Guo et al., 202177 |
| Frass composting water emissions | %water | 69% |  |  |
| NH3 reduction | %NH3 | 95% |  | As composting model in Javourez et al., 20241 |
| Composted frass TAN | %N | 19% | 11-37% | Reviewed range as wide as 1%77 to 88%112. Selected range without outliers113,114 (here, stream-independent) |
| Frass composting diesel | L.tww-1 | 0.65 | 0.3-2.8 | Range representing industrial installations65,111 |

##### Land spreading

Matured frass was further considered as compost, hence the inventory linked to its land spreading was modeled as in the composting pathway (Javourez et al., 20241). Albeit selected with the same characteristics than for regular compost, frass MFE-P and -K were assumed as an independent technology parameter. Frass-specific N mineralization rate was assumed also independently defined   (Table SI. 37).

#### Equipment and infrastructure

Both infrastructure and equipment aim to be representative of a medium- small-scale facility semi-automatized (e.g. presence of some conveyor belts). The infrastructure required was modeled as building halls using the area proxy proposed by Dortmans et al., 201776 for each unit operations, as no other sources were found (Table SI. 36). The equipment list was derived from the detailed inventory provided by Roffeis et al., 202096. Only the processes with non-neglectable impacts were accounted (e.g. discarding plastic glasses). Disposable equipment was assumed incinerated, but infrastructure’s and other equipment’s end of life management were neglected.

Table SI. 36 – BSF infrastructure proxy used

| Unit operation (m2.ktww-1 input) | Value | Comment |
| --- | --- | --- |
| Storage | 3.6 | Adapted from layout proposed by Dortmans et al., 201776. Depend on parameter (see Table SI. 37), here calculation for a 25 years lifetime and 420 tBSF.y-1 production capacity. |
| Breeding/Nursery | 9 |  |
| BSF farming | 26.5 |  |
| Post-treatments | 5 |  |
| Other | 13 |  |
| Composting | 7.2 |  |

Infrastructure, equipment and area required are tightly related to the development strategy, as low-tech facilities e.g. in Zurbrugg et al., 2018115 likely require more building area but less equipment compared to full automatized and optimized facilities e.g. Kröncke et al., 2020116. Identified as important contributors to environmental impacts in assessment by Roffeis et al., 2017117, a variation range for both infrastructure and equipment requirements was developed by assuming a longer lifespan and treatment capacity potential than reported. Therefore, it was assumed that the maximum gain on infrastructure and equipment use efficiency was of 50% compared to the plant proxies of Dortmans et al., 201776 for infrastructure and Roffeis et al., 202096 for equipment. The baseline value was set in-between, representing semi-automatized facilities (Table SI. 37). The full equipment list and ecoinvent background inventory is detailed in corresponding study and not repeated here.

### Technology parameters and background processes

The technology parameters retained for BSF pathways modeling are displayed in the Table SI. 37. Background processes and technology parameters associated with transversal unit operations between many models (e.g. pretreatments, transportation, etc.) are displayed in the common section SI. 4.1. Rest of modeling details (e.g. all biosphere flows) are available directly within corresponding excel model.

Table SI. 37 – Parameters specific to BSF pathways

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 65 | BSF substrate CH4-C storage emissions | gCH4.kgVS-1 | 1 | 0.85-1.15 | See Table SI. 28 |
| 66 | Decontamination energy | kWh.tww-1 | 26.5 | 3-50 |  |
| 67 | Target BSF substrate DM | %DM | 30% | 20-38% |  |
| 68 | Starter feed mix | mgww.5-DOL-1 | 12 | 10-15 | See Table SI. 29 |
| 69 | 5-DOL weight | mgww.5-DOL-1 | 6 | 2-10 |  |
| 70 | Moisture content BSF larvae | %ww | 64% | 60-70% | See Table SI. 30 |
| 71 | Fat content BSF larvae | %DM | 34% | 25-43% |  |
| 72 | Protein conversion ratio (PCR) | %proteins | 38% | 25-60% |  |
| 73 | BSF larvae survival rate | %5-DOL | 90% | 78-98% |  |
| 74 | Finishing BSF larvae weight | mg.larvae-1 | 150 | 100-200 |  |
| 75 | BSF growth period length | days | 20 | 11-29 |  |
| 76 | BSF larvae CO2 emissions | kgCO2.kgDM-1 | 2.75 | 1.5-4 | See Table SI. 32 |
| 77 | BSF larvae CH4 emissions | mgCH4.kgDM-1 | 28 | 9-95 |  |
| 78 | BSF larvae N emissions | %N | 4.4% | 0.7-8.0% |  |
| 79 | BSF larvae N2O emissions | mgN2O.kgDM-1 | 106 | 7-215 |  |
| 80 | BSF larvae NH3 emissions | gNH3.kgDM-1 | 3.8 | 0.1-8.0 |  |
| 81 | BSF frass moisture content | %ww | 65% | 50-75% | Reviewed range67,75,79 |
| 82 | Facilities use efficiency enhancement | %current | 28% | 0-50% | 0% for low-tech set-ups76,117, considering up to 50% enhancement (i.e. more production per facility unit) |
| 83 | BSF killing energy | kWh.kgww-1 | 0.14 | 0.09-0.25 | See Table SI. 34 |
| 15 | Drying electricity | kWh.kgwater-1 | 0.11 | 0.03-0.28 | As for MP production (section SI. 4.6.4) |
| 16 | Drying heat | MJ.kgwater-1 | 3 | 2.7-4.1 |  |
| 84 | Frass composting electricity | kWh.tww-1 | 26 | 0.9-50 | See Table SI. 35 |
| 85 | Frass composting C-emissions | %C | 20% | 10-35% |  |
| 86 | Frass composting CH4 emissions | %C_emit | 1% | 0.2-1.7% |  |
| 87 | Frass composting N2O emissions | %N_emit | 1.2% | 0.5-1.9% |  |
| 88 | Composted frass TAN | %N | 19% | 11-37% |  |
| 89 | Frass composting diesel | L.tww-1 | 0.65 | 0.3-2.8 |  |
| 64 | Frass kmin | %N | 55% | 35-75% | Between compost and biosolids118 |
| 90 | Frass MFE-P | %P | 90% | 80-100% | As for compost, paired with MFE-K |
| 91 | BSF larvae digestibility | %OM | 88% | 85-90% | Reviewed range monogastrics119,120 |
| 92 | BSF larvae crude fibers | %DM | 7% | 6-8% | Feedipedia8 ± 10% |
| Other parameters linked with compost land spreading are detailed in Javourez et al., 20241. |  |  |  |  |  |
| Other common parameters in section SI. 4.1. |  |  |  |  |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to mycoproteins production stage but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15.

The background processes used specifically for this LCI model are gathered in Table SI. 38.

Table SI. 38 – Background processes used specifically for BSF pathways

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Market for water, deionized, RoW | All where cleaning is involved | Proxy of cleaning utilities inventory. Wastewater was calculated as sum of cleaning water and detergent, assumed with a 2000 ppm of COD96. |
| Market for detergent, GLO |  |  |
| Market for wastewater, from residence, CH |  |  |
| Building construction, hall, CH | Infrastructure | Amount were calculated with Table SI. 36. |
| Forwarding, forwarder, RER | Composting | Linked with diesel requirements (0.83 kgdiesel.L-1 and 9.24 kgdiesel.h-1)48 |
| List of equipment as in Roffeis et al., 202096 (also available in B.BSF_module spreadsheet) |  |  |
| Background processes for frass composting available in Javourez et al., 20241 |  |  |
| Background processes for compost spreading available in Javourez et al., 20241 |  |  |

All processes are from the ecoinvent 3.8 consequential database. Direct flows to biosphere are displayed in B.BSF_module.

## Mycoproteins production from residual-based sugars LCI model

This section presents the details (including assumptions, choice rationales, calculations and data sources) of the mycoproteins (myco) production life cycle inventory (LCI) model, with the objective to perform the LCA of transforming fibrous/starchy biomass streams into edible microorganisms (e.g. the “wood-to-proteins” route). The mycoprotein production LCI model supporting both variants myco_feed and myco_food pathways is available in B.Myco_module spreadsheet.

### Set-up selection

The objective of this model is to build on already established worked, summarizing the recent advances and expanding possibilities offered by the conversion of secondary sugars into mycoproteins. These pathways initially emerged in the context of wartime, to provide edible ingredients in a context of food insecurity (either biomass-based molasses directly or protein-rich edible microorganisms cultured on these121).  ). Yet, current approaches rather build on the potential environmental benefits of sourcing alternative carbohydrates sources (compared to the common land-dependent ones such as sugar beet, maize, potato)122. Referred to as mycoproteins, the inclusion of fungal biomass in human diets was reported to yield overall health benefits when substituting meat products (e.g. reduced cholesterol levels, increased bioavailability of amino acids, etc.; Derbyshire and Delange, 2021; Souza Filho et al., 2019). Yet, fungi are mostly aerobic heterotrophs, which in the case of submerged fermentation, require a growth media supplied with organic soluble (C1-C6 compounds). Indeed, current mycoproteins production set-ups based on raw or starch-based glucose do not provide clear enhanced environmental benefits compared to chicken and pork meat124.

When these residual streams are compliant with safety standards and host nutrients under readily-available forms, mycoproteins production has already been successfully reported, for example using molasses, potato processing wastewater, bred leftovers or seaweeds 125–127. Yet, when these nutrients remain untangled within strong polymer networks (for example, the lignocelluloses matrix of primary crop residues), a cracking pretreatment should be performed 128,129. Importantly, there is a TRL difference between the myco production stage (already commercialized), and the food-grade lignocellulosic sugar production stage (TRL < 4-5, albeit production plants existed during WWII, likely with different safety and sanitary standards). Accordingly, the precise technical design and forecasted environmental performances remain mostly unknown as most of the scaling initiatives (e.g. Arbiom; CometBio; BALI sugar process; etc.) are under patent with low to no data publicly available. Therefore, only two LCA targeting the full wood-to-proteins route were identified in the literature (see Table SI. 39). On top of hosting a different LCA goal and scope (gate-to-mycoproteins accounting for carbon sequestration, attributional LCA), they display their results for one residual stream only, and one of them does not transparently display its modeling and performance assumptions. Therefore, while the present mycoproteins production model serves the objective of this study, it also provides an additional tool to assess the environmental consequences of implementing the “wood-to-proteins” route.

Table SI. 39 – Mycoproteins production assessment review, to date

| Study and scope | Second-generation sugars | Mycoproteins production | Type | Comment |
| --- | --- | --- | --- | --- |
| Pihlajaniemi et al., 2020130 | x | x | TEA | Focus pretreatment |
| Upcraft et al., 2021131 | x | x | TEA & aLCA | Adaptation of Quorn process (food applications) |
| Voutilainen et al., 2021132 | x | x | TEA | For food applications |
| Møller et al., 2022133 | x | x | aLCA | For feed applications |
| Throup et al., 2022134 | x |  | TEA | Based on David et al., 2018135 |
| Blanco et al., 2020136 | x |  | aLCA |  |
| Bello et al., 2021137 | x |  | aLCA |  |
| Fabbri et al., 2022138 | x |  | cLCA |  |
| Smetana et al., 2015139 |  | x | aLCA | Based on Quorn secondary data |
| Nonhebel and Raats, 2007140 |  | x | aLCA |  |
| Potter et al., 2020141 |  | x | aLCA |  |
| Risner et al., 2023142 |  | x | TEA |  |

cLCA, consequential LCA; aLCA, attributional LCA; TEA, techno-economic analysis. Conference proceedings not included.

The panel of set-ups for second-generation mycoproteins production was reviewed in Javourez et al., (2021)2, some examples extracted from the database are summarized in Table SI. 40.

Table SI. 40 – Illustration of the panel of mycoproteins production set-ups

| Input stream | Conversion | Strain | Study |
| --- | --- | --- | --- |
| Wheat straw | Integration to bioethanol plant | Neurospora Intermedia | Lab scale143 |
| Paper waste | Milling and acid hydrolysis | Scytalidium Acidophilus | Lab scale144 |
| Rice straw | Ionic liquid pretreatment and enzymatic hydrolysis | Fusarium Venenatum | Lab scale pretreatment131. Quorn fermentation |
| Diverse on raw sugar | Batch mode, biomats | Fusarium Yellowstonensis | Industrial scaling: Nature’s Fynd145 |
| Rice straws and hulls | Hydrolysis | Mortierella isabelline / Yarrowia lipolytica | Lab scale146 |
| Grass cake | Steam explosion, ammonia soaking and hydrolysis | Paecilomyces variotii (Pekilo process) | Lab scale130 |
| Spruce wood | BALI sugar process or Glommen technology | Candida Utilis (Arbiom) | Commercial development: Arbiom and others133,147 |

The aim of the process modeled herein is the production of mycoproteins only, hence not considering potential co-production of metabolites or platform chemicals along with fungal biomass148,149.

Many techniques are currently developed to produce industrial sugars from lignocellulose (Table SI. 39 for references). Rationales to select a set-up depend on target specifications, input composition and overall costs, among others. Yet, almost all of them, to achieve delignification, use high severity (energy- and or chemical-intensive) processes which strongly influence the environmental performance of cellulosic sugars production137. Such harsh pretreatments (e.g. dilute acid, organosolv, steam explosion, etc.), apart from their operational challenges135, also induce the production of neoformed molecules which can hosts both anti-nutritional (e.g. toxic) or inhibitory (for further fermentation) properties131,150,151. It is unlikely that resulting hydrolysates output meet food standards without implementing intensive detoxification stages. From the reviewed lignocellulosic pretreatments, only ionic liquid (IL) was found as compliant with all the specifications required to generate food-grade cellulosic sugars. Albeit biocompatible and food-grade ionic liquid pretreatments  are at early development stages, the process workflow has already been described 131,152 which is not the case for alternative wood sugar production such as patented BALI or “wood molasses” processes 133. IL pretreatments build on the high solubility of lignin in IL solvents (association of a cation and an anion) due to hydrogen bonds and electrostatic interactions153. IL are mostly tunable to desired chemo selectivity, are quite stable and recyclable154 and can be formulated with food-compliant and biocompatible compounds such as choline, lysine and hydrogen sulfate131,152. Yet, albeit high recyclability is mandatory in front of IL’s important production costs154, the IL recycling stage currently requires (in the water-wash configuration) energy-intensive evaporation and concentration operations, even if there is room for improvements155,156. Moreover, biocompatible IL treatments are at early development stage. However, even if current performances do not reach the ones of optimized conventional IL treatments (see Table SI. 41), authors mainly agree that the gap can be bridged156,157. Solubilized lignin is further recovered and valorized energetically to compensate for the significant heat requirements by IL process, while more advanced functional valorization of lignin-derived compounds (e.g. furfurals) are also investigated131,158. Finally, albeit highly tunable, increasing delignification objectives of IL treatments generally lead to the dissolution of part of the hemicellulose, which seems to be an intrinsic trade-off in IL treatment developments so far157. Notably, as this study includes starch-rich co-products, a simplified mycoproteins production variant based on glucose recovery from starch was also modeled.

### Unit operation design and assumptions – the lignocellulosic pretreatment

As a common assumption, the difficulties related to the intrinsic heterogeneity and the far from ideal composition of residual biomass streams (e.g. high lignin-to-cellulose ratio and recalcitrance)151 were neglected. This implies that the performance indicators of the present supply chains only depend on the input’s composition as defined through all the models. As detailed in section SI. 2.2, only streams already considered as feed-grade were assumed suitable to supply mycoproteins production pathways. Lignocellulosic feedstocks handling and storage were modeled exactly as for gasification (GA), see section SI. 4.8.2.

#### Standardization

To enhance IL pretreatment efficiency, stored inputs were standardized in terms of particles size towards 200-900 μm131. A first size of 10-30 mm was achieved through grinding operations, and final size was obtained through a disc refiner and Szego mill (defibrillation; 0.2 kWh.kgDM-1)135. Dust generation and recovery was modeled as for GA pretreatment (see section SI. 4.8.2). Moisture standardization was neglected as most pretreatments involve contact with water.

#### Ionic liquid pretreatment

##### Design

The set-up considered a simplified version of the one proposed in Upcraft et al., 2021131. Standardized substrate was first put in contact with an IL mixture during 1-2h at 150°C temperature131,156. After a heat exchanger, the cellulose-rich pulp was further separated through membrane filtration158 and washed to recover most of IL in the liquid stream. By considering that the pretreatment is optimized for both cellulose and hemicellulose recovery, the valorization of furfural at this stage was not considered. Instead, the whole IL-rich stream hosting dissolved lignin and some sugars (see Table SI. 41) was sent to the IL recovery stage. There, antisolvent was used to separate lignin, precipitating with most of present soluble compounds (e.g. glucans). Recovered lignin-rich cakes through a thickener and filter were further washed before being valorized in-situ in an industrial furnace (same modeling than dust valorization). While ethanol addition was mentioned as an antisolvent to precipitate lignin, only water was assumed here131. Finally, the IL-rich stream (along with water and some remaining dissolved compounds) was sent to a multiple-effects evaporation equipment allowing to recover most of IL at target moisture for direct re-looping. The rest of the stream was sent to a wastewater treatment plant (WWTP) after sensible heat recovery. In current scaled assessments, IL make-up is therefore limited due to the assumed high IL recycling rates (90-99%159 see Table SI. 43) which conditions this pathway’s viability. Albeit the main goal of pretreatment is to removed lignin, this is often accompanied by hemicellulose removal. Here, as we assume future C5 sugars utilization for mycoproteins production131, the selected IL pretreatment aims to limit hemicelluloses losses (optimistic as the removal rate of both fractions are correlated). The different IL treatment sub-sections are further detailed.

Table SI. 41 – Review of lignocellulosic pretreatment performances, with a focus on IL techniques

| Pretreatment conditions | Biomass-to-pulp ratio, for each fraction (%) |  |  |  |  |  |  | Other indicators |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cellulose | Hemi- cellulose | Solubles, carbs | Lignin | Proteins | Ashes | Others | Mass loss | Glucose yield |  |
| [Ch][HSO4], 150°C, 1h, food-grade | 71% | 31%* |  | 58% |  | 100% |  | ca. 30% | 43% | 131 |
| [Ch][Lys], 120°C, 8h, 20%DM, food-grade | >90% | 15% | 50% | 15% |  | 60% |  | 43% | 77% | 152 |
| [TEA][HSO4], 120°C, 1-24h, 9%DM | 86-97% | 0-60% |  | 12-47% |  |  |  | 27-54% |  | 157 |
| [TEA][HSO4], 120°C, 4h, 15%DM | 41% | 44% |  | 74% |  |  |  | 45% |  |  |
| [HBim][HSO4] | 80% | 39% |  | 66% |  |  |  | 36% |  |  |
| [HMim][HSO4] | 79% | 33% |  | 94% |  |  |  | 17% |  |  |
| [TEA][HSO4], 150°C, 17%DM | >95% | 11-43% |  | 29-50% |  |  |  | 27-40% |  |  |
| [HBim][HSO4], 120°C, 9%DM | 85-93% | 15-49% |  | 39-69% |  |  |  | 28-48% |  |  |
| [DBMA][HSO4], 170°C, 30mn | 83-93% | 0-20% |  | 20-60% |  |  |  |  | 60-80% | 155 |
| [Ch][OAc], 120°C, 6h | 90% | 34% |  | 90% |  |  |  | 18% | 70-80% | 159 |
| Other pretreatment technics (non-IL), for comparison |  |  |  |  |  |  |  |  |  |  |
| Steam explosion (15mn, 1%H2SO4, 190°C, 5.57bars) | 83% | 31% |  |  |  |  |  |  | 57% | 132 |
| SPORL treatment | 59% | 10%* |  |  |  |  |  | 35% |  | 160 |
| Alkaline extraction and deacetylation (NaOH) | 98% | 86%* | 50% | 53% | 100% | 34% | 70% | 32% |  | 135 |
| 210°C, 19bars | 94% | 20-50% | 0-10% | 85% |  |  |  |  |  | 161 |
| Dilute acid pretreatment | 97% | 1% |  |  |  | 100% |  |  |  | 162 |
| Selected for this study as technology parameters** |  |  |  |  |  |  |  |  |  |  |
| Food-grade IL, future performances | 90% | 40% | 10% | 20% | 70% | 50% | 70% | 50-70% | 60-75% |  |
| Variation range considered | 70-95% | 20-50% | 0-20% | 10-60% | 50-100% | 30-70% | 50-100% |  |  |  |
| Comments | 90% Currently achievable152, state-of-art achieves 95%157 | Average reviewed, uncertain as compound specific. | Low data, assumed mostly removed through washing161 | State-of-the-art centered152, rather ranging 20-60% | Low data, assuming that most proteins complex and remain within pulp, see text | Highly variable, intermediate value | Assumed as proteins | Calculated (baseline values), depend on initial composition |  |  |

Here, only the best case of author’s experiments is displayed. *: average of galactans, mannans, xylans, arabinans, as this study considers hemicellulose as a single compound. **: assumed fixed regardless of entry feedstock specific composition, which is likely already an optimistic assumption.

##### Biomass-to-pulp modeling

The Table SI. 41 displays values for pretreatment processes of different TRL, targeting both food and non-food applications, supporting the selection of biomass-to-pulp ratios to capture food-grade IL pretreatment’s forecasted performances.

Dissolved glucans fraction was assumed lost either within wastewater of during the lignin precipitation. Proteins are highly denatured in IL pretreatment temperature and conditions (>120°C), likely forming complex interactions with other fractions such as lignin130,163. Accordingly, current assessments usually consider that most of proteins are lost during lignocellulosic treatments135. Here the protein fraction was assumed entirely lost at the enzymatic hydrolysis stage (remaining bounded with lignin). Nevertheless, suitable streams for mycoproteins production are usually proteins-poor, else these would be recovered beforehand in e.g. a green biorefinery set-up 130.

##### IL treatment energy requirements

IL treatment demands heat mainly for the IL recovery and concentration stages156; heat recovery systems are mandatory for the viability of this pathways (e.g. flash steam131). To estimate a lower range of heat needed for IL treatments, a design considering sensible heat recovery was considered. Indeed, pretreated mixture (after IL treatment) at 150°C can prewarm inlet flows (consisting in recirculated IL and biomass feedstocks) before going to the washing stage (cold water). Heat demand was further estimated with simple heat balances (ideal conditions, no losses), following guidelines of section SI. 4.1.6. The evaporator’s energy consumption was calculated considering a running temperature of 105°C and a multi-effects efficiency164. Liquid waste streams go out from the evaporator at ca. 80-90°C 135, hence sensible heat was also assumed recovered before discharge to WWTP systems. The heat balance key shaping parameters (IL stage) are summarized in Table SI. 42.

Table SI. 42 – Heat balance modeling involved in IL pretreatment

| Unit operation | Heat balance | Temperatures | Modeling |
| --- | --- | --- | --- |
| IL pretreatment | Heating demand | 13°C to 150°C | Mixed with warm IL recovered, heat exchanger, and finally complemented with HT heat. |
| Standardization | Cooling demand | 150°C to 30°C | Heat exchanger, mixed with washing water |
| Evaporation | Heating demand | 50°C to 105°C | Multiple effects evaporator efficiency parameter (Table SI. 43) |
| Evaporation | Wastewater cooling | 90°C to 30°C | LT WHR |
| Cake washing | Lignin-rich cake | n.a. | Solid co-products energy valorization |
|  | Only processes linked with IL pretreatment |  | See section SI. 4.1 |

The rest of the parameters used to model IL pretreatment are displayed in Table SI. 43.

Table SI. 43 – IL pretreatment modeling parameters

| Parameter | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| IL make-up moisture content | %ww | 20% | 10-30% | As Upcraft et al., 2021131 |
| IL lost in pulp | %in | 0.01% |  | One simulation found131 |
| Relooped IL | %rec | 99.5% | 99-99.9% | Currently reported values ranging 85-99% within less than 10-20 cycles157. Here, selecting optimistic range. |
| IL density | kg.m-3 | 1143 |  | Choline-based165 |
| IL Cp | kJ.kg-1.K-1 | 1.5 |  |  |
| Mixing electricity | kWh.m-3 | 0.3 |  | Impeller liquid mixing |
| IL requirements | kgIL.kgDM-1 | 5.6 | 4-7 | Base as Upcraft et al, 2021131. Range reviewed159 |
| Washed pulp moisture | %ww | 80% |  | Enough to pump towards next step131 |
| Washing water lignin cake | kg.kgDM-1 | 1.85 |  | 131 |
| Evaporator efficiency | %MJ_th | 75% | 70-80% | Review164, defines heating requirements for IL recovery |
| Evaporator electricity | %MJ | 10% | 5-15% | No data, common range for drying technologies |

#### Enzymatic hydrolysis

This stage was modeled as in Davis et al., 2018135, a study commonly referred to in similar assessments131,134. The enzymatic hydrolysis takes place in a continuous stirred tank reactor, including both cellulases and hemicellulases. The hydrolysates are separated with ultrafiltration, and the lignin-rich cake (assumed to contained all remaining lignin at this stage) is further pressed to recover the most of soluble before its energy valorization (as a solid co-product). The main assumptions were summarized in Table SI. 45, and considered as representative of food-grade cellulosic sugars production. Then, hydrolysates (at ca. 5%DM) are concentrated towards target moisture content to enter the bioreactor stage. The concentration stage was modeled with an evaporator following the same assumptions as for IL recovery. The heat balance key shaping parameters (enzymatic hydrolysis stage) are summarized in Table SI. 44.

Table SI. 44 – Heat balance modeling involved in enzymatic hydrolysis

| Unit operation | Heat balance | Temperatures | Modeling |
| --- | --- | --- | --- |
| Enzymatic hydrolysis | Heating demand | 13°C/30°C to 48°C | Pulp mixed with make-up water at ambient T°C. Assumed supplied through LT heat. |
|  | Lignin-rich cake | n.a. | Solid co-products |
| Evaporation | Heating demand | 48°C to 105°C | Multiple effects evaporator efficiency parameter (Table SI. 43) |
| Enzyme production | Heating demand | 28°C | Small, assumed met by heat exchanger |
|  | Only processes linked with IL pretreatment |  | See section SI. 4.1 |

Base on the enzyme requirements, a share of output hydrolysate is assumed reused to feed the in-situ enzyme production (calculated to 5-10% in the baseline context). This set-up choice influences the interpretation of the results, as the impacts of the enzyme production stage becomes indirect. The less efficient the enzyme production, the more hydrolysate is required per enzyme unit and consequently the less hydrolysate enters the bioreactor (leading to less mycoproteins produced). In other words, here, the performance of the enzyme production stage influences the whole downstream reference flow, which would not have been the case if biological enzyme production were assumed based on marginal carbohydrates suppliers. Cellulases and hemicellulases were both assumed produced with the same efficiency (Table SI. 45), and the only mass losses were through metabolic CO2 venting. The rest of the nutrients such as ammonia and sulfur dioxide were fed stoichiometrically. No recycling of remaining cellulases and hemicellulases within the hydrolysate stream was considered. The soluble carbohydrates remaining in the input pulp were considered as metabolizable glucose (optimistic).

Table SI. 45 – Lignocellulosic enzymatic hydrolysis modeling parameters

| Parameter | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| Pulp loading | %DM | 7.6% | 5-10% | Lower value as currently achieved, value as short-term achievable135. Range also reviewed130 |
| Glucose yield | %cell | 80% | 50-96% | Lower range currently achieved on food-grade IL treated pulp131,132, base value as the best of reported food-grade performance152. Forecasted values over 85% (higher range)134,135 |
| C5 sugars yield | %hemi | 80% | 50-99% | Upcraft et al., 2021 considered same as glucose131, while others135,156 consider specific fractions (e.g. xylans) can achieve over 90-99%. |
| Cellulases loading | mg.gcell-1 | 15 | 10-20 | Food-grade range 10-30131 while conventional rather range 10-20132 |
| Hemicellulases loading | mg.ghemi-1 | 15 | 10-20 |  |
| Sugars recovery | %sugars | 99% |  | To account for losses at the ultrafiltration135 |
| Enzymatic treat. electricity | kWh.tpulp-1 | 0.25 |  | UF electricity accounted apart. |
| Hydrolysate final concentration | %DM | 10% | 5-16% | Define the sugar content in mycoproteins bioreactor. Base as reported optimum166, range reviewed132,133 |
| Enzyme production | kg.kgsugar-1 | 0.24 | 0.2-0.28 | Established process48,135. Assumed range ±15% |
| Rest of enzyme production utilities exactly as David et al., 2018135 |  |  |  |  |
| Evaporator modeled as for IL pretreatment (Table SI. 43). |  |  |  |  |

At this stage, proteins, lipids and ashes are assumed lost within lignin-rich cake, so that hydrolysates are considered food-grade and suitable for direct fermentation135. More details provided in B.Myco_module spreadsheet.

### Technology parameters and background processes – Lignocellulosic pretreatment

The technology parameters retained for the lignocellulosic pretreatment of mycoproteins production pathways (input-to-hydrolysates), are displayed in the Table SI. 46.

Table SI. 46 – Parameters specific to ionic liquid and enzymatic hydrolysis pretreatment operations

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 93 | IL make-up moisture content | %ww | 20% | 10-30% | See Table SI. 43 |
| 94 | Relooped IL | %rec | 99.5% | 99-99.9% |  |
| 95 | IL requirements | kgIL.kgDM-1 | 5.6 | 4-7 |  |
| 96 | Cellulose-to-pulp | %cell_in | 90% | 70-95% | See Table SI. 41 |
| 97 | Hemicellulose-to-pulp | %hemi_in | 40% | 20-50% |  |
| 98 | Solubles-to-pulp | %sol_in | 10% | 0-20% |  |
| 99 | Lignin-to-pulp | %lignin_in | 20% | 10-60% |  |
| 100 | Evaporator efficiency | %MJ_th | 75% | 70-80% | See Table SI. 43 |
| 101 | Evaporator electricity | %MJ | 10% | 5-15% |  |
| 102 | Pulp loading | %DM | 7.6% | 5-10% | See Table SI. 45 |
| 103 | Enzymes loading | mg.gcell-1 | 15 | 10-20 |  |
| 104 | Glucose yield | %cell | 80% | 50-96% |  |
| 105 | C5 sugars yield | %hemi | 80% | 50-99% |  |
| 106 | Hydrolysate final concentration | %DM | 10% | 5-16% |  |
| 107 | Enzyme production | kg.kgsugar-1 | 0.24 | 0.2-0.28 |  |
| Other common parameters in section SI. 4.1. |  |  |  |  |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to lignocellulosic pretreatments but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15.

The background processes used specifically for this LCI model are gathered in Table SI. 47.

Table SI. 47 – Background processes used specifically for lignocellulosic pretreatment operations

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Forwarding, forwarder, RER | Standardization | Feeding to the biorefinery (0.315 PMH.tww-1)167 |
| Market for water, deionized, RoW | IL treatment | Make up and cooling water |
| Market for sulfuric acid, RER |  | Proxy of IL composition131,156 (1:1 proportions), assumed electricity as background context. |
| Market for trimethylamine, RER |  |  |
| Treatment of wastewater from anaerobic digestion of whey, CH | IL recovery | In baseline configuration, WWTP calculated at ca. 20 gCOD.L-1. The closest ecoinvent process was selected. Background electricity adapted. |
| Ultrafiltration module production, hollow fiber, GLO | Enzymatic hydrolysis | Same modeling as in MP model (see section SI. 4.6.4) |
| treatment of wastewater, unpolluted, capacity 5E9l/year, CH |  | Proxy of wastewater discharge by evaporator, assumed unpolluted. |
| Market for sulfur dioxide, liquid, RER | Enzymes production | See Davis et al., 2018 135. Ammonium modeled as adapted N fertilizer service. |
| Paper mill construction, integrated, RER | n.a. | Proxy of the whole lignocellulosic pretreatment plant, considered equivalent per kgsugar output (1.9e-10 unit.kgsugar-1 based on production capacities). |
| Other common background processes in section SI. 4.1. |  |  |

All processes are from the ecoinvent 3.8 consequential database. Direct flows to biosphere are displayed in B.Myco_module spreadsheet.

Background processes and technology parameters associated with transversal unit operations between many models (e.g. pretreatments, transportation, etc.) are displayed in the common section SI. 4.1. Rest of modeling details (e.g. all biosphere flows) are available directly within corresponding excel model.

### Unit operation design and assumptions – the starch pretreatment

In this section, the starch variant set-up is detailed.  The agrifood co-products resource potential estimation highlighted the availability some starch-rich streams, in particular from the potato processing industries (e.g. peals, screenings, etc.). Starch-based glucose being already commonly developed (e.g. corn syrup), an alternative modeling of mycoproteins based on starch-rich streams was developed. Such streams have in common to host a low content (adding up few percent) of cellulose, hemicelluloses and lignin, so that no intensive pretreatments before hydrolysis are required (only size standardization through grinding). To avoid unnecessary energy expenses, input feedstocks suitable with starch hydrolysis and displaying a moisture content higher than 20%ww were assumed to follow a cold chain management before the hydrolysis. Cold chain was assumed through a refrigerated storage of one week (3 kWh.tww-1) and by taking refrigerated lorry transportation background processes.

Gelatinization, liquefaction or chemical addition (e.g. NaOH, CaCl2, etc.)168 before the hydrolysis were neglected. The starch hydrolysis stage was modeled as for cellulose hydrolysis (see section SI. 4.5.2), but adapting the parameters to reflect the established performances of maize starch hydrolysis48, see Table SI. 48. Moreover, albeit some hemicelluloses fractions could also be dissolved and saccharified at this stage, we assumed that only starch and other soluble carbohydrates were recovered. Proteins were also assumed precipitated and not remaining within the hydrolysate fractions (due to the temperature of 60°C). The solid remaining from the hydrolysis was assumed to supply feed markets for its almost lignin-free composition and for the food-grade aspect of starch-hydrolysis. Based on mass balance, the biocomposition’s fractions were estimated and this stream was directed towards the CF model (assumed digestibility: 90%OM as cooked product with no fibers).

Table SI. 48 – Starch enzymatic hydrolysis modeling parameters

| n° | Parameter | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- | --- |
| Direct utilities are exactly as “glucose production, GLO” ecoinvent process (maize starch hydrolysis), except: |  |  |  |  |  |
| 108 | Starch product loading | %DM | 10% | 5-20% | Reviewed common range136,169,170, higher range challenged by viscosity and separation. |
| 109 | Amylases loading | mg.gstarch-1 | 0.44 | 0.4-1 | Current industrial range171 |
| 110 | Starch-glucose yield | % | 95% | 90-99% | Max considering high grade raw starch48, range for difficulties linked to heterogeneous substrates and different proportions of amylose / amylopectins172 |
| Evaporator modeled as for IL pretreatment, see Table SI. 43. |  |  |  |  |  |

More details provided in B.myco_module spreadsheet.

Notably, the infrastructure of this starch variant was proxied exactly as performed in the glucose production inventory of ecoinvent with the “chemical factory construction, RER” process (0.013 kgfacto.kgsugars-1).

### Unit operation design and assumptions – Mycoproteins production

The unit operations described below are common to both the lignocellulosic and starch variants. For its modular design, this model can also directly be supplied with sugar-rich streams (e.g. molasses), without any pretreatment requirements, but this option was not contemplated in this study.

#### Bioreactor

This section and every related unit operation were mostly derived from Upcraft et al., 2021131 and Risner et al., 2023142 who build on the already existing mycoproteins production (Quorn; industrial scale), but was adapted to be homogeneous with MP production model. Indeed, both fungi and bacteria selected in this study are aerobics cultured in similar bioreactors (airlifts and U-loops). Quorn development was forecasted to reach a 250 m3 continuous airlift-bioreactor (50m height), including a 25% extra-volume173. The same plant availability than for the lignocellulosic facility was assumed (8000 h.y-1), and both plants were considered synergistically integrated within the same industrial complex. Seed production was neglected in front of the large production cycles, and no formation of extra-cellular compounds were considered. Effects of the hydrolysate’s impurities on culture were also neglected, and the sterilization was assumed to already have been performed during the hydrolysate’s concentration stage so no specific process was modeled. Accordingly, the present design relies on the optimistic assumption that mycoproteins production performances on low-refined C5-C6 mix hydrolysates are as good as on current tailored substrates.

#### Microbial strains

As argued in the main manuscript, the strain Fusarium Venenatum was selected to represent mycoproteins, the microbial biomass (X; ash-free) was modeled as C5.8H11.5O3.7N131. Table SI. 49 summarizes the main assumptions and indicators considered regarding microbial biomass production.

Table SI. 49 – Review and calculation of mycoproteins growth performances

| Performance indicators | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| C6 yield (Yx/s) | gX.gS-1 | 0.29 | 0.26-0.36 | Parameters: Baseline values as Upcraft et al., 2021131. C6 higher range mentioned for Torula132, while C5 lower range if not possible. |
| C5 yield (Yx/s) | gX.gS-1 | 0.29 | 0-0.30 |  |
| Biomass concentration | gX.L-1 | 30 | 20-40 | Parameter: commonly assumed for all submerged microbial production131,173 |
| Dilution rate | h-1 | 0.2 | 0.17-0.25 | Parameter: range lower than µmax (0.28)132,173 |
| Productivity | kg.m-3.h-1 | 6 | 4-8 | Calculated |
| Production | kg.h-1 | 1125 |  |  |
| Temperature | °C | 30 | n.a. | Set value: Reported Quorn value |
| Metabolic heat | kJ.molCX-1 | 250 | 160-332 | Parameter: thermodynamic balance range review (various strains)131,174 |

Calculation developed with baseline values for technology parameters.

From the growth parameters, the stoichiometric biomass growth equations were derived. Equation 6 and Equation 7 display their baseline version (using values from Table SI. 49), which are in line with the one used by Upcraft et al., 2021131.

Equation 6

Mycoproteins on glucose, baseline value

C6H12O6+3.95O2+0.34NH3=0.34X+4.03CO2+4.56H2O

Equation 7

Mycoproteins on xylose, baseline value

C5H10O5+3.29O2+0.28NH3=0.28X+3.36CO2+3.80H2O

With X: C5.8H11.5O3.7N being the fixed mycoprotein elemental composition (ash-free). Xylose was considered as the proxy for all C5 sugars. Two-digits rounding.

It is important to note that no evidences were found during the literature review to support the suitability of mycoprotein production on C5 sugars, so this possibility rather represents an optimistic future development. Additionally, xylose is the C5 sugars the most affected by pretreatment. Here no details on the C5 sugars specific composition were assessed (input-dependent), so xylose was considered as a proxy for all C5 sugars. Accordingly, this model estimates the ideal higher range achievable by mycoprotein technologies, but the performances are likely strongly sensitive to xylose’s use rate by fungi131 (here defined as a parameter).

#### Gases supply and off-gases reuse

The exact same modeling as for MP gas injection, oxygen and nitrogen supplies were considered here, see section SI. 4.6.4. Mycoproteins production does not face the flammable gas mixture challenge, and off-gases are mostly inert (unused O2 and NH3, metabolic CO2, and N2). These were assumed directly vented to the atmosphere after passing through biofilters to oxidize ammonia65.

#### Metabolic heat

Cooling is required to maintain mesophilic conditions (here, 30°C) in the bioreactor, due to the exothermic mycoproteins growth (Table SI. 49). For homogeneity, the same assumptions that for MP were adopted, see section SI. 4.6.4.

#### Cleaning requirements and infrastructure

Having in common most of unit operations and production capacity, the cleaning requirements of mycoproteins production were modeled as for MP (see section SI. 4.6.4), only adapting the frequency (see Table SI. 53). Related infrastructure was also modeled through the same proxy than for MP (see Table SI. 54).

#### Medium preparation

Other nutrients and micronutrients necessary for fungal growth were included, based on values specifically reported for the target strain (Table SI. 50). Make-up water was limited as hydrolysates were already supplied at the target moisture content. Similarly modeled in Van Peteghem et al., 202241, missing nutrients were mixed with make-up water and further sterilized at 120°C, before being pumped towards the bioreactor, following the main heat balance and modeling than for MP medium (section SI. 4.6.4).

Table SI. 50 – Medium formulation modeling parameters

| Compounds | Unit | Value | Comment |
| --- | --- | --- | --- |
| P | mg.gX-1 | 17.5 | 131 |
| K |  | 19 | 131 |
| MgSO4 |  | 17.4 | 131 “Market for magnesium sulfate, GLO” |
| CuSO4 |  | 0.15 | 131 “Market for copper sulfate, GLO” |
| MnSO4 |  | 0.35 | 131 “Market or manganese sulfate, GLO” |
| FeSO4 |  | 0.26 | 131 “Market for iron(III) sulfate, without water, RoW” |
| ZnSO4 |  | 1.17 | 131 “Market for zinc monosulfate, RER” |
| CaCl2 |  | 2.8 | 131 “Market for calcium carbonate, precipitated, RER” |
| NaOH |  | 8.2 | 133“Market for sodium hydroxide (50%), GLO |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants. X: microbial biomass. Antifoam was neglected. P & K requirements LCI were modeled as fertilizers services.

#### Post-treatments

The post-treatment workflow slightly differs compared to MP production due to the filamentous aspect of the selected fungal strain, and the end market. It was selected to correspond to proxy Quorn design. Fusarium Venenatum also presents ribonucleic acid (RNA) content (Table SI. 51), which is removed as the final ingredient targets food markets. Many technics exist41,175, but the filamentous aspect of fungi to preserve (organoleptic and texturization properties) led to the implementation of a heat treatment (70°C, 5 bars)131,176. Pasteurization was then performed until the centrifugation stage, where resulting supernatant (mycoproteins at 30%DM) was further cooled, texturized, and enters the cold supply chain (4°C) as a fresh meat-analogue product (myco_food variant). Accordingly, the modeling of mycoproteins post-treatments was based on MP post-treatments, removing UF and final drying. To simplify, given the temperatures involved, RNA heat shocks were assumed to not require any additional energy compared to the subsequent pasteurization stage (as the broth needs to be heated anyway). This is a rather optimistic assumption as heat shocks are reported to be performed with a 268°C steam injection131. Here, pasteurized wastewater (after centrifuge) was used to prewarm the broth, and was further cooled to 30°C through LT WHR (see section SI. 4.1.6). Due to the amount of water brought by the hydrolysates, no water looping system was implemented, it was assumed treated in a WWTP (see Table SI. 54). Accordingly, the case where mycoproteins target feed markets (myco_feed variant) was modeled almost identically than when targeting food. The only difference is the additionally induced mass loss when RNA removal is performed (myco_food; see main manuscript).

#### Final ingredient

Mycoproteins were not dried. The meat-analogue texturization of Quorn currently relies on the addition of egg white, among others. Yet, even if recent advances could partly remove this necessity177,  the organoleptic properties of mycoproteins would likely still be linked with texturization and additives incorporation60,178. As an additional optimistic assumption, it was considered that mycoproteins could directly be texturized in a meat-analogue product without any additives. Texturization was simply modeled with an additional electricity consumption (assumed as pelletizing in the CF pathway). The composition of mycoproteins are displayed in Table SI. 51.

Table SI. 51 – Mycoproteins composition

| Composition (%DM) | Without RNA removal | With RNA removal |
| --- | --- | --- |
| Proteins | 41.4% | 45.0% |
| Lipids | 10.7% | 11.6% |
| Carbohydrates | 9.2% | 10.0% |
| Sugars | 1.8% | 2.0% |
| Fibers | 22.1% | 24.0% |
| Ashes | 3.1% | 3.4% |
| RNA/DNA | 9.0% (ranging 7-11%) | 1.0% |
| Others | 2.8% | 3.0% |
| Digestibility (%OM) | 88% (ranging 85%-93%) | n.a. |
| Crude fibers | 5.0% |  |
| SFU.kgDM-1 | 1.47 |  |
| Comments | Base as Quorn176, but adaptable to variable RNA content covering range for fungi/yeast. Reviewed feed value range for yeast179 | 180,181 |

Values displayed in the baseline configuration. SFU: Scandinavian feed unit. Fibers mostly composed by chitin and beta-glucans.

When targeting feed markets, mycoproteins was assumed to directly supply the CF supply chain, but without any drying. Accordingly, fresh or wet feed markets (e.g. as petfood) are targeted. Accordingly, either when targeting food or feed markets, cooling to 4°C was performed through refrigeration (see Table SI. 53), and all downstream transportation developed with refrigerated lorry (see Table SI. 54).

#### Mycoproteins production heat integration

While the main outcomes of the mycoproteins production LCI model was already discussed and compared with benchmarked literature in the main manuscript, the main hypothesis regarding heat integration are summarized in the Table SI. 52.

Table SI. 52 – Heat recovery modeling involved in mycoproteins production and post-treatments

| Unit operation | Heat recovery | Temperatures | Modeling |
| --- | --- | --- | --- |
| Mycoproteins production | Metabolic heat | 30°C | LT WHR |
| Sterilization | Medium cooling | 66°C to 30°C | LT WHR (after heat exchange) |
| Centrifuge (starch variant) | Wastewater cooling | 72°C to 30°C | LT WHR (after heat exchange) |
|  | Only processes linked with mycoproteins production |  | See section SI. 4.1 |

### Technology parameters and background processes – Mycoproteins production stage

The technology parameters retained for mycoproteins production pathways modeling (excluding pretreatment operations, already detailed earlier), are displayed in the Table SI. 53.

Table SI. 53 – Parameters specific to mycoproteins production stage and downstream operations

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| Parameters linked to lignocellulosic pretreatment are available in Table SI. 46 and for starch pretreatment in Table SI. 48. |  |  |  |  |  |
| 111 | Sugars use efficiency | %in | 95% | 90-99% | In between ideal value131 and currently achieved132 |
| 112 | Myco C6 yield | gX.gS-1 | 0.29 | 0.26-0.36 | See Table SI. 49 |
| 113 | Myco C5 yield | gX.gS-1 | 0.29 | 0-0.30 |  |
| 114 | Myco biomass concentration | gX.L-1 | 30 | 20-40 |  |
| 115 | Myco dilution rate | h-1 | 0.2 | 0.17-0.25 |  |
| 117 | Metabolic heat | kJ.molCX-1 | 250 | 160-332 |  |
| 116 | Myco gas use efficiency | %in | 90% | 80-92% | As for MP (similar bioreactor), but independent: other strain (Table SI. 81) |
| 118 | Myco CIP frequence | year-1 | 8 | 7-9 | As Quorn176, assumed ±1 |
| 119 | Myco mineral medium use | %supplied | 95% | 80-95% | As for MP (similar bioreactor), but independent: other strain (Table SI. 81) |
| 120 | Myco RNA removal losses | %DMin | 28% | 23-30% | Reviewed range, base value current Quorn41,131,132 |
| 121 | Myco RNA content | %DM | 9% | 7-11% | See Table SI. 51 |
| 122 | Myco digestibility | %OM | 88% | 85-93% |  |
| 123 | Myco crude fibers | %DM | 5% | 1-10% |  |
| 23 | COP frigo | kW.kWe-1 | 3.5 | 3-4 | Common refrigeration systems. |
| Other common parameters in section SI. 4.1. |  |  |  |  |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to mycoproteins production stage but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15.

The background processes used specifically for this LCI model are gathered in Table SI. 54.

Table SI. 54 – Background processes used specifically for mycoproteins production

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Market for sodium hydroxide (50%), GLO | Cleaning | As for MP production, see Table SI. 82 |
| Market for tap water, CH |  |  |
| Market for nitric acid, without water (50%), RoW |  |  |
| Treatment of wastewater from maize starch production, CH |  |  |
| Background inventory for mineral medium already detailed in Table SI. 50. |  |  |
| Treatment of wastewater from anaerobic digestion of whey, CH | Centrifuge (myco_food) | Closest proxy of reject wastewater treatment (ca. 18 gCOD.L-1 based on Table SI. 18). Background electricity was replaced by the one of this study as treatment assumed to take place in France. |
| Treatment of wastewater from maize starch production, CH | Centrifuge (myco_feed) | Closest proxy of reject wastewater treatment (ca. 9 gCOD.L-1 based on Table SI. 18). (different than myco_food because no RNA induced losses) |
| Transport, freight, lorry with refrigeration machine, 7.5-16 ton, EURO6, GLO | n.a. | Proxy of cold chain transportation, replaced conventional transport data where required. |
| Chemical factory construction, RER | n.a. | Proxy of the mycoproteins production facility. Adapted to this plant’s production capacity and lifetime: 3.6 g.kgDM-1. |
| Rest of background processes used for mycoproteins pathways were displayed in Table SI. 47. |  |  |

All processes are from the ecoinvent 3.8 consequential database. Direct flows to biosphere are displayed in B.Myco_module spreadsheet.

Background processes and technology parameters associated with transversal unit operations between many models (e.g. pretreatments, transportation, etc.) are displayed in the common section SI. 4.1. Rest of modeling details (e.g. all biosphere flows) are available directly within corresponding excel model.

## Microbial proteins production from residual-based gases LCI model

This section presents the details (including assumptions, choice rationales, calculations and data sources) of the microbial proteins (MP) production life cycle inventory (LCI) model, with the objective to perform the LCA of transforming residual biomass streams into edible microorganisms. The objective of this model is to build on already established worked, summarizing the recent advances and expanding possibilities offered by methane-oxidizing bacteria (MOB) and hydrogen-oxidizing bacteria (HOB). Albeit scaling, with already planned industrial projects, open-access primary data on HOB and MOB production largely remain at the experimental scale182. A review of the modeling assumptions used in already published LCA and TEA (techno-economic analysis) was developed (list in Table SI. 55) and showed that these mostly rely in the same primary data. LCA results are shown to be primarily dependent on the background context in which such technologies are implemented (e.g. electricity mix183); comprehensive parametric assessments are missing184. In a nutshell, while most of published assessments aim to answer how to best produce HOB and MOB, the present study rather aims to answer if it a good idea (environmentally speaking) to produce HOB and MOB. To do so, the aforementioned benchmark review (Table SI. 55) allowed to screen the wide panel of set-ups, data available and calculation strategies already implemented.

Table SI. 55 – MP production assessment review, to date

| Study | Type | Microbe | Comment |  |
| --- | --- | --- | --- | --- |
| de Boer et al., 2014173 | aLCA | MOB |  | Novel feed comparison, including NG-based MOB production (Unibio) |
| Cumberlege et al., 2016185 | aLCA | MOB |  | LCA of Calysta proteins |
| Levett et al., 2016186 | TEA | MOB |  | Modeling of a MP route to produce PHB (non-feed) |
| Pikaar et al., 2018187 | CO2 and land use only | HOB & MOB |  | Production potential assessment and implications of various MP pathways |
| Tallentire et al., 2018188 | aLCA | MOB |  | Includes estimation of Unibio process LCA |
| Couture et al., 2019189 | aLCA | MOB |  | Comparison of alternative salmons feed, including MP |
| Sillman et al., 2019, 2020190,191 | aLCA | HOB |  | Drivers for electrolysis-based HOB production (similar to Solar Food set-up) |
| Spiller et al., 2020192 | cLCA | PB & microalgae |  | Comparison of feed-oriented valorization of potato wastewater |
| Khoshnevisan et al., 2020193 | cLCA | MOB |  | Screening the best options to manage biowaste |
| Matassa et al., 2020194 | Mass, energy balances | HOB & MOB |  | Concept study about scaling and quantifying potential for bio-based MP production from various residues |
| Verbeeck et al., 2020195 | TEA | HOB & MOB |  | Integration of MP production within the anaerobic digestion of agricultural residues |
| Nappa et al., 2020196 | TEA | HOB |  | Comparison of solar-to-nutrition conversion efficiency applied to bacteria and microalgae |
| Järviö et al., 2021183 | aLCA | HOB |  | Case study assessment of Solar Food concept and eco-design considerations |
| Leger et al., 2021175 | Efficiency analysis | HOB |  | Comparison of CO2 to nutrition conversion efficiency between microbial proteins and conventional crops |
| El Abbadi et al., 202140 | TEA | MOB |  | Estimation of MP production from flared industrial gases in the USA |
| García Martínez et al., 2021, 2022197,198 | TEA | HOB & MOB |  | Implementation costs and economic feasibility of quick MP production scaling in times of disasters |
| Marami et al., 2022 a, b31,199 | cLCA | MOB |  | Integration of MP production within WWTP |
| Van Peteghem et al., 202241 | Efficiency analysis | Diverse |  | Comparison of MP production routes based on CO2 and power (using C1-C2 substrates as intermediates) |
| Li et al., 2023200 | TEA | MOB |  | Comparison of different biogas utilization options for energy and SCP production. |
| Vlaeminck et al., 2023201 | TEA | Diverse |  | Acetate as intermediate, steel industrial off-gases |
| Jean et Brown, 2024202 | TEA | HOB |  | Reusing CO2 from a Corn ethanol facility |

cLCA, consequential LCA; aLCA, attributional LCA; TEA, techno-economic analysis; NG, (fossil) natural gas; PB, purple (non sulphur) bacteria; WWTP, wastewater treatment plant.

One of the key differences between the reviewed HOB and MOB production pathways lies in the origin of the supplied elements, nutrients and energy required for microbial growth. This not only defines the scope of the study but the chain of unit operations required. The scope of possibilities reported in reviewed literature is summarized in Table SI. 56.

Table SI. 56 – Main gas origin for HOB and MOB production, review

| Gas required | Supply | Comment | Used in |
| --- | --- | --- | --- |
| Oxygen | Direct air blowing | Limit ATEX risks but increase pumping costs | 184,186,193 |
|  | Air separation unit | Cryogenic separation, pressure swing absorption (PSA), etc. | 195,197,198,203 |
|  | Electrolysis | Co-product of H2 production | 175,183,187,195–197,199,204–206 |
|  | Other | Electrochemical processes, etc. | 207 |
| Hydrogen | Electrolysis | Either external supply or in-situ MP production (electrolysis within bioreactor) | 175,183,187,191,195–197,199,204–206,208 |
|  | Raw syngas | After cooling, using CO-tolerant HOB | 187,194,197,208,209 |
|  | Processed syngas | After upgrading, reforming, gas shifting, etc. | 194,197 |
|  | Other | Electrochemical processes, etc. | 207 |
| Carbon dioxide | Air separation unit | DAC systems (e.g. temperature-vacuum swing adsorption, amino-resin adsorbent, etc.) | 175,183,191,196,197,204,206 |
|  | Waste, purchased | Process in ecoinvent, need of vaporizing unit if liquid | 183 |
|  | Waste, in-situ recovery | Fossil- or bio-based industries flue gases (e.g. cement, iron, biorefineries, etc.) | 187,191,202,210 |
|  | Syngas | Present in syngas (>10%vol) | 194,197 |
|  | Biogas | Present in biogas (>30%vol) | 22 |
|  | Biomethane | Concentrated stream separated from biogas | 207 |
|  | Other | e.g. flue gas of MOB production | 22 |
| Methane | Natural gas | Purified to remove e.g. ethane | 40,187,197,203 |
|  | Raw biogas | After cleaning (incl. desulfurization) | 22,184,194,195,211,212 |
|  | Biomethane | After biogas upgrading (e.g. water scrubbing, PSA, etc.) | 22,199,213,214 |
|  | Other | e.g. electro microbial cells (CO2-to-CH4), syngas, etc. | 22,207,215 |
| Nitrogen* | Digestate, treated | Liquid fraction filtered, pasteurized and diluted | 211,212,214 |
|  | Digestate, recovery | Using e.g. stripping, electrochemical cells, etc. | 22,194,195,213,216–218 |
|  | Syngas | Presence of NH3 if gasification of N-rich feedstocks | 194 |
|  | Haber-Bosch | Commercially available | 175,187,191,197,198 |
|  | Waste, recovery | e.g. from urea, wastewater, manure, etc. | 204,213,216,219–221 |
|  | Air (N2) | If diazotroph strain | 222 |

*: Rest of elements required are usually assumed supplied through standardized mineral media.

### Set-up selection

#### Gas-to-bioreactor set-up selection

Due to the scope of the study, the MP production model focuses primarily on biogenic gases and nutrients: most of them need to come from the input feedstocks. Yet, many solutions exist or emerge to provide biomass-to-H2 and -to-CH4 pathways, which can be further hybridized or not with synthetic routes (e.g. power-to-X). In an endeavor to identify the best performing biomass-to-H2 pathways, Megret et al., 2015223 suggested that reforming biogas would yield higher H2 yield than upgrading syngas. Similarly assessing the environmentally-best performing biomass-to-CH4 pathways, Lodato et al., 20226 found that reforming syngas would be preferable than upgrading biogas. Considering the interplays between (i) HOB & MOB different yields, (ii) gasification (GA) and anaerobic digestion (AD) different environmental performances as well as (iii) gas upgrading/post-processing possibilities and implications on H2 and CH4 production, it was found hardly possible to discard pathways a-priori. The Table SI. 57 summarizes the six MP production pathways retained in this study. Accordingly, the LCI model (B.MP_module spreadsheet) includes modeling of both HOB and MOB production.

Table SI. 57 – Selected microbial proteins pathways

| Input gate | AD supply chain | GA supply chain |
| --- | --- | --- |
|  | Cleaned biogas | Cleaned syngas |
| Direct use | MP_biogas (A): MOB on cleaned biogas | MP_syngas (F): HOB on cleaned syngas |
| After upgrading | MP_biometh (C): MOB on biomethane | MP_shifts (E): HOB on shifted syngas |
| After shifting | MP_shiftb (B): HOB on shifted biogas | MP_meths (D): MOB on methanized gas |

Each pathway is further detailed in the following sections, PFD is presented in the main manuscript (letters A-F correspondence).

The potential of alternative set-ups (e.g. mentioned in Table SI. 55) and discard reasons are provided afterwards.

First, the co-culture of HOB and MOB (i.e. in the same bioreactor)217 was not retained. This combination is likely promising to convert gas mixes (such as syngas), where CH4, CO2 and H2 are simultaneously present, but also to reuse metabolic CO2 of MOB directly in-situ. Another argument promoted is the increased resilience to shocks (e.g. changes in gas mixture input) of the co-culture22. Yet, not enough data was found to document this pathway and proxy its industrial scale performance. Second, the various synergistic combinations of MOB and HOB production sequences (e.g. as proposed by Khoshnevisan et al., 202222) were not specifically addressed herein neither. These variants mostly aim to make the most of exhaust gases, therefore rather addressing how to best reuse CO2, somehow similarly to the CCU-to-proteins (CCU: carbon capture and use) approaches224. Accordingly, such approaches could plug wherever lies a concentrated CO2 flow within studied pathways, which includes combustion flue gases, CO2 removed from biogas, etc. As there is no clear and generalizable trend on how to best use CO2 streams225, and for diverging too much from the scope of this study, CCU options were not retained. Third, the production of small-chains C compounds (e.g. methanol, formate, acetic acid, etc.) as intermediates to support heterotrophic cell growth are also increasingly promoted in the literature226,227.  On top of reinforcing the safety of MP production (no flammable gas mixes184), gas-to-soluble-to-proteins pathways display overall greater energy conversion efficiencies. For example, Leger et al., 2021175 calculated that H2-to-methanol-to-yeast and H2-to-HOB pathway were the most energy efficient configurations from a panel of gas-to-protein approaches.  Van Peteghem et al., 202241 also showed that the highest carbon to MP ratio was met through the CO2-to-methanol-to-yeast pathway (64% conversion), which also turned out to be one of the less energy-demanding pathways (5-42 kWh.kgCWD-1). Similar conclusions were derived by Claassens et al., 2016226. Such C1-C2 compounds, again, are mainly investigated in the context of CCU systems, and involve electrochemical conversions224,228. Yet, according to Leger et al., 2021175, to date, only H2, CO and formate production are commercially viable through such abiotic pathways. Moreover, some substrates such as diluted formic and acetic acid are reported to limit microbial productivity41. Emerging approaches involving biological generation of C1-C2 compounds through bioelectrosynthesis are also reported, but are of current low TRL (i.e. < 3)215. Other sequences of bioconversion to convert gases into feed are being developed (e.g. dark fermentation)229,230, but the data is too scarce and heterogeneous for robust performances modeling.

To provide future easier comparison between first- and second-generation MP production, and considering both build on the exact same process workflow (from the bioreactor gate), electrolysis-based HOB production and natural gas-based MOB production were also modeled. For HOB, hydrogen requirements were assumed fully met by electrolysis, and CO2 stoichiometrically supplied through direct air capture (DAC) modeled as in Su-ungkavatin et al., (2023). For MOB, natural gas supplies were directly derived from existing ecoinvent processes, and all the CO2 emissions present in the LCI model were modified from biogenic to fossil.

#### MP production set-up selection

First generation HOB and MOB production are scaling, while the industrial development of second generation MP production is rather foreseen to be reached after 203031. Accordingly, the bioreactor stage and downstream operations were modeled according to the latter, of higher TRL and with established set-ups (industrial-scale pilot facilities for MOB by Calysta231 and Unibio232; lab-scale pilot for HOB by Kiverdi Inc.233 and Solar Foods234, among others). MOB production is currently of higher TRL than HOB, and is already officially considered as a safe feed ingredient under specific conditions in the EU235. Process optimization was not part of the scope of this study, albeit operational parameters and optimal set-up designs are still under investigation (e.g. temperature trade-offs between gas solubility and growth rates217,236). Therefore, we assume that current pilot-scale reactors performances remain unchanged at full commercial production in the baseline configuration, and use the parametric analysis to explore the span of potential MP production future scaled performances directions.

### Unit operation design and assumptions – gas post-treatments

In this section, the different gas conversion chains linked with the six selected pathways (Table SI. 57) are described. First, MP_biogas and MP_syngas directly builds on the output of AD (described in the SI of Javourez et al., 20241) and GA model (section SI. 4.8) respectively (after the gas cleaning stage); no additional processes were considered. However, syngas cooling towards 35°C was modeled, to meet the input requirements for the bioreactor. The gas post-treatments requirements to supply MP_meths, MP_biometh, MP_shifts and MP_shiftb are further described. Yet, for ease of reuse, these gas post-treatments were operationally modeled within corresponding AD and GA LCI models. For this purpose, the AD model spreadsheet available in Javourez et al., 20241 was directly reused.

#### Syngas methanation

This gas post-treatment operation supplies the MP_meths variant. The selected set-up aims to maximize CH4 production from syngas. Therefore, water conversion shift (WCS) reactions were first performed to increase the share of H2 and CO2 in syngas. Finally, missing H2 to reach methanation stoichiometry was supplied through electrolysis. Unconverted CO from WCS was also partly converted during methanation237, but other side reactions were neglected. Biological methanation, albeit increasingly investigated238, was not considered here for its current low-TRL. Each unit operations are detailed in the following sections, mostly based on the analysis developed by Megret et al., 2015223.

##### Water conversion shift

First, syngas was cooled towards WCS requirements. WCS is usually developed in two stages (high and low temperature223), but for modeling simplification, a single WCS stage (340°C, ambient pressure) was considered (Equation 8). Albeit exothermic, temperature was assumed constant in the reactor as excess heat was partly recovered as modeled in section SI. 4.1.6. Steam requirements and overall WCS efficiency were parameterized (see Table SI. 70). Mass and energy balances are available in B.GA_module spreadsheet. WCS Catalysis (Cu-Zn-Al) and facilities requirements were based on the fixed-bed reactor set-up by Van der Giesen et al., 2014239 (see Table SI. 58).

Equation 8

Water conversion shift (WCS)

CO+H2O=CO2+H2

Table SI. 58 – WCS building and catalysis modeling parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Steel | mg.Nm-3in | 8.04 | Proxy for WCS reactor and building239 |
| Bulk iron |  | 16.1 |  |
| Chromium steel |  | 8.04 |  |
| Zeolite powder |  | 45.1 |  |
| Copper catalyst | mg.kgCO-1 | 2.15 | Review160 |
| CuO | kg.kgca-1 | 0.57 | Proxy of WCS catalyst composition239 |
| ZnO |  | 0.31 |  |
| Al2O3 |  | 0.11 |  |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants.

##### Methanation

Once the share of H2 and CO2 increased in shifted syngas, methanation is performed to convert the most of present CO2, CO and H2 into CH4. Methanation is also exothermic, and is usually ran at temperatures between 200-350°C and ambient pressure237. To simplify the model, it was assumed (i) a methanation temperature of 340°C, corresponding to WCS gas outlet and (ii) successive methanation of CO2 and CO237 (in this order; Equation 9 and Equation 10). Similarly, to WCS, temperature in the methanation reactor was assumed constant, hence excess heat was partly recovered through heat exchanger as modeled in section SI. 4.1.6. Additional H2 required to meet stoichiometric conditions was supplied through electrolysis, detailed in the next section. CO2 and CO methanation conversion efficiencies were derived from prospective assessments240. Facilities (in particular Ni-catalysis) and utilities were based on current synthetic natural gas (SNG) industrial production data241 and complemented with pilot-scale SNG from syngas (e.g. in Ren et al., 2020237) when available (see Table SI. 59). Mass and energy balances are available in B.GA_module spreadsheet.

Equation 9

Methanation 1

CO2+4H2=CH4+2H2O

Equation 10

Methanation 2

CO+3H2=CH4+H2O

Methanized gas final cooling was modeled as detailed in section SI. 4.1.4, assuming all water is removed and further treated (see Table SI. 62). The optimistic assumption of this modeling is that future improvements will enable N2-rich syngas methanation, while currently this is not technically feasible240.

Table SI. 59 – Methanation building and catalysis modeling parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Steel | g.Nm-3SNG | 1.88 | Proxy for methanation reactor and building48,241 |
| Industrial furnace | unit.Nm-3SNG | 2.1e-8 |  |
| Catalyst | mg.Nm-3SNG | 9.3 | Proxy of methanation catalyst composition48,241 |
| Ni | %catalyst | 19% |  |
| Al alloy |  | 81% |  |
| Zn | g.Nm-3SNG | 1.87 |  |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants.

##### Electrolysis

H2 production from electrolysis was modeled as in Lodato et al., 20226. An alkaline-based set-up was chosen as already commercially available (80°C, 30 bar). The energy conversion efficiency was parameterized (see Table SI. 70) and assumed to include all the other (usually neglectable) electricity requirements (e.g. for compression). Gas and water storages and direct emissions were neglected. Electrolysis installations inventory were derived from an existing pilot plant242. Excess LT heat from the energy balance was assumed partly recovered (as in section SI. 4.1.6). Albeit currently not valorized, the oxygen co-produced was here considered partly recovered (that is, avoiding marginal oxygen supplier production, see Table SI. 70). Finally, the unused water discharge was neglected. Main assumptions for electrolysis modeling are summarized in Table SI. 60. The present H2 production GWP100 performance (baseline values for technology parameters and context) is notably lower than threshold set by the EU commission (3.0 kgCO2-eq.kgH2-1)243 for “sustainable” production.

Table SI. 60 – Electrolysis modeling parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Deionized water | kg.kgH2-1 | 7.8 | Average of 11 (2030 horizon)240,242 and 4.2 160 |
| KOH | g.kgH2-1 | 1.9 | Average of real and projected plant consumption242 |
| N2 | g.kgH2-1 | 0.3 | Used in pilot-plant242 |
| Building | m2.kgH2-1 | 1.7e-3 | Proxy of facility244 |
| Co-produced O2 | kg.kgH2-1 | 7.94 | Stoichiometric proportions |
| Energy efficiency | Technology parameter, see Table SI. 70 |  |  |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants.

##### Summary of technical assumptions and calculation

Calculation assumptions and intermediary results illustrated for one case study (MP_meths gas post-treatment) are presented in Table SI. 61 and Table SI. 62.

Table SI. 61 – Heat recovery modeling involved in MP_meths pathway

| Unit operation | Heat recovery | Temperatures | Modeling |
| --- | --- | --- | --- |
| Grinding | Dust combustion | n.a. | Solid co-products |
| Cooling | Syngas cooling | 900°C to 340°C | HT WHR |
| Water conversion shift | Exothermic reaction | 340°C |  |
| Methanation | Exothermic reaction | 340°C |  |
| Methanation | Methanized gas cooling | 340°C to 35°C |  |
| Electrolysis | Exothermic reaction | 80°C | LT WHR |
|  | Fixed in this study, 35°C to be aligned with MP production |  | See section SI. 4.1 |

Table SI. 62 – MP_meths pathway gas flows and energy performances illustrated for hardwood residues

| Gas composition (%vol) | Hot syngas* | Cooled syngas | Shifted gas | Methanized gas | Cooled methanized gas |
| --- | --- | --- | --- | --- | --- |
| H2 | 12% | 12% | 22.7% | 0.3% | 0.5% |
| CO | 19% | 19% | 2.3% | 0.1% | 0.3% |
| H2O | 5% | 5% | 10.3% | 45.9% | 0% |
| CO2 | 13% | 13% | 23.5% | 0.9% | 1.7% |
| CH4 | 3% | 3% | 2.4% | 21.6% | 40% |
| N2 | 48% | 48% | 38.7% | 31.1% | 57.5% |
| Characteristics |  |  |  |  |  |
| Nm3.tww-1 | 2089 | 2089 | 2562 | 3127 | 1691 |
| GJHHV.tww-1 | 12.7 | 11.7 | 11.1 | 29.0** | 25.3** |
| MJHHV.Nm-3 | 6.1 | 5.6 | 4.3 | 9.3** | 14.9** |
| HGE %HHVin | 78% | 72% | 68% | 63%** | 55%** |
| °C | 900 | 340 | 340 | 340 | 35 |
| Comment | Composition fixed (Table SI. 97) |  |  | Overall HGE of SNG similar to ecoinvent process (50%)48 | Entering MOB bioreactor |

Baseline technology values, rounded values. *: after cleaning losses. **: accounting for electricity input of electrolysis.

#### Biogas methanation

This stage allows to upgrade biogas into biomethane, at grid injection requirements, for further use by MOB culture (MP_biometh pathway). Biomethane production from biogas was already modeled in Javourez et al., 20241 for AD_grid pathway, through CO2 removal. Initially considered, the overall MP pathway building on such upgraded biogas was found to display almost the same environmental performances and pattern than MP produced directly on cleaned biogas (MP_biogas). Indeed, the only difference between these two are the utilities to remove the CO2 and its subsequent valorization, the rest of unit operations and flows are equals. Accordingly, instead, methanation was selected to enrich the panel of results. This option consists in converting biogas CO2 into extra methane through the addition of H2, as performed on shifted syngas in the MP_meths pathway (previous section). However, here, only Equation 9 was considered (no CO present), and H2 requirements were calculated considering CO2 as the limiting compound. The electrolysis was modeled as detailed in previous section, which is also the case for the modeling of methanation, with two differences. First, part of methanation excess heat was used to warm biogas (Table SI. 63). Second, electricity requirements were adapted to fit established data for biogas methanation (Table SI. 70). An illustration of the key flows involved in MP_biometh pathway is presented in Table SI. 64.

Table SI. 63 – Heat recovery modeling involved in MP_biometh pathway gas pretreatment

| Unit operation | Heat recovery | Temperatures | Modeling |
| --- | --- | --- | --- |
| Methanation | Biogas heating | 37°C to 340°C | HT WHR |
| Methanation | Exothermic reaction | 340°C |  |
| Methanation | Methanized gas cooling | 340°C to 35°C |  |
| Electrolysis | Exothermic reaction | 80°C | LT WHR |
|  | Only processes linked with gas post-treatments |  | See section SI. 4.1 |

Table SI. 64 – MP_biometh pathway gas flows and energy performances illustrated for bovine manure

| Gas composition (%vol) | Biogas* | Hot biomethane | Biomethane |
| --- | --- | --- | --- |
| H2 | 0% | 4% | 6% |
| H2O | 0% | 38% | 0% |
| CO2 | 34% | 1% | 2% |
| CH4 | 66% | 57% | 92% |
| Characteristics |  |  |  |
| Nm3.tww-1 | 96 | 163 | 102 |
| GJHHV.tww-1 | 2.4 | 3.6** | 3.5** |
| MJHHV.Nm-3 | 25 | 22** | 34** |
| HGE %HHVin | 46% | 51%** | 49%** |
| °C | 37 | 340 | 35 |
| Comment | Composition fixed |  | Entering MOB bioreactor |

Baseline technology values, rounded values. *: after cleaning losses. **: accounting for electricity input of electrolysis.

#### Syngas hydrogen enrichment

This sequence of stages allows to increase the share of H2 in syngas, and targets HOB production (MP_shifts; as proposed in e.g. Matassa et al., 2020194). The established set-up consists in implementing a steam reforming (SR) operation, followed by water conversion shifts (WCS), as documented by Megret et al., 2015223.

##### Steam reforming

Steam reforming is currently the most used reforming technique223. SR is endothermic, and usually requires a temperature of 800-1000°C and a steam pressure of 30-40 bars. Hot syngas being already at 900°C, only steam production energy and water requirements were accounted (Equation 11). Steam and syngas were both compressed to target pressure (Equation 1). Most of the process parameters were documented from biogas-based SR reactions for the data available, assumed unchanged for syngas (Table SI. 70). SR was modeled with a Ni-based catalysis (Table SI. 65), neglecting current complications (e.g. coke formations). Mass and energy balances are available in B.GA_module spreadsheet.

Equation 11

Steam Reforming (SR)

CH4+H2O=CO+3H2

Table SI. 65 – SR building and catalysis modeling parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| To proxy SR reactor and building, the inventory of Battista et al., 2017245 was reused |  |  |  |
| Nickel oxide | mg.Nm-3in | 1.80 | Proxy of SR catalyst composition246 |
| Aluminum oxide |  | 1.62 |  |
| Heat treatment (J) |  | 9.0 |  |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants.

##### Water conversion shift

Water conversion shift is used to convert CO present in reformed syngas. WCS was modeled as already detailed in the syngas methanation section. Due to the difference between SR and WCS temperatures, reformed syngas was first cooled (Table SI. 66). Shifted syngas (WCS outlet) was further cooled down towards bioreactor’s requirements, assuming all the water condensates at this stage. No CO2 removal from shifted syngas was considered; this compound being required for HOB production.

##### Summary of technical assumptions and calculation

Calculation assumptions and intermediary results illustrated for one case study (MP_meths gas post-treatment) are presented in Table SI. 66 and Table SI. 67.

Table SI. 66 – Heat recovery modeling involved in MP_shifts pathway pretreatment

| Unit operation | Heat recovery | Temperatures | Modeling |
| --- | --- | --- | --- |
| Steam reforming | Reformed gas cooling | 900°C to 340°C | HT WHR |
| Water conversion shift | Exothermic reaction | 340°C |  |
| Water conversion shift | Shifted syngas cooling | 340°C to 35°C |  |
|  | Only processes linked with gas post-treatments |  | See section SI. 4.1 |

Table SI. 67 – MP_shifts pathway gas flows and energy performances illustrated for hardwood residues

| Gas composition (%vol) | Hot syngas* | Reformed gas | Cooled reformed gas | Shifted gas | Cooled shifted gas |
| --- | --- | --- | --- | --- | --- |
| H2 | 12% | 18% | 18% | 28% | 31.5% |
| CO | 19% | 20% | 20% | 2.5% | 2% |
| H2O | 5% | 6% | 6% | 11% | 0% |
| CO2 | 13% | 11.5% | 11.5% | 23% | 26% |
| CH4 | 3% | 0.5% | 0.5% | 0.5% | 0.5% |
| N2 | 48% | 44% | 44% | 35% | 40% |
| Characteristics |  |  |  |  |  |
| Nm3.tww-1 | 2089 | 2256 | 2256 | 2773 | 2477 |
| GJHHV.tww-1 | 12.7 | 13.4** | 12.4** | 11.6** | 10.5** |
| MJHHV.Nm-3 | 6.1 | 5.9** | 5.5** | 4.2** | 4.2** |
| HGE %HHVin | 78% | 69%** | 64%** | 60%** | 54%** |
| °C | 900 | 900 | 340 | 340 | 35 |
| Comment | Composition fixed (Table SI. 97). |  |  |  | Entering HOB bioreactor |

Baseline technology values, rounded values. *: after cleaning losses. **: accounting for SR steam energy input.

#### Hydrogen production from biogas

This sequence of steps allows to produce HOB from biogas (MP_shiftb pathways), and is based on the already established pilot-scale biogas-to-hydrogen set-up247–249. It involves SR to first convert CH4 into CO and H2 (Equation 11), followed by WCS to convert CO (Equation 8). Accordingly, the modeling assumptions were exactly the same than used for MP_shifts pathway. The heat recovery systems considered as well as an illustration of the key flows involved in MP_shiftb pathway are presented in Table SI. 68 and Table SI. 69.

Table SI. 68 – Heat recovery modeling involved in MP_shiftb pathway pretreatment

| Unit operation | Heat recovery | Temperatures | Modeling |
| --- | --- | --- | --- |
| Steam reforming | Biogas heating | 37°C to 900°C | HT WHR |
| Steam reforming | Reformed gas cooling | 900°C to 340°C |  |
| Water conversion shift | Exothermic reaction | 340°C |  |
| Water conversion shift | Shifted syngas cooling | 340°C to 35°C |  |
|  | Only processes linked with gas post-treatments |  | See section SI. 4.1 |

Table SI. 69 – MP_shiftb pathway gas flows and energy performances illustrated for bovine manure

| Gas composition (%vol) | Biogas* | Reformed gas | Cooled reformed gas | Shifted gas | Cooled shifted gas |
| --- | --- | --- | --- | --- | --- |
| H2 | 0% | 41% | 41% | 50% | 68.5% |
| CO | 0% | 13.5% | 13.5% | 2% | 2.5% |
| H2O | 0% | 34.5% | 34.5% | 26.5% | 0% |
| CO2 | 34% | 8.5% | 8.5% | 19% | 26% |
| CH4 | 66% | 2.5% | 2.5% | 2.5% | 3% |
| Characteristics |  |  |  |  |  |
| Nm3.tww-1 | 96 | 391 | 391 | 405 | 298 |
| GJHHV.tww-1 | 2.4 | 3.5** | 3.1** | 3.1** | 2.9** |
| MJHHV.Nm-3 | 25 | 8.9** | 8** | 7.7** | 9.7** |
| HGE %HHVin | 46% | 64%** | 56%** | 56%** | 52%** |
| °C | 37 | 900 | 340 | 340 | 35°C |
| Comment | Composition fixed. |  |  |  | Entering HOB bioreactor |

Baseline technology values, rounded values. *: after cleaning losses. **: accounting for SR steam energy input.

### Technology parameters and background processes – gas post-treatments

The technology parameters retained for MP pathways pretreatments modeling (i.e. AD and GA gases post-treatments) are displayed in Table SI. 70.

Table SI. 70 – Parameters specific to MP pathways pretreatments

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| Gate-to-gas technology parameters for AD and GA are detailed in Javourez et al., 20241 and Table SI. 101 respectively |  |  |  |  |  |
| 167 | Steam turbine efficiency | %MJ | 85% | 80-90% | Lower value for current situation36, higher value forecasted in 205046. Baseline also used by Megret et al., 2015223. |
| 168 | SR electricity | kWh.kgH2-1 | 1.11 | 0.2-4.3 | Here proxy of both SR & WCS (coming together). Linde plant reports 0.2247, while 2.7 223 and 4.3 249 are also reported. |
| 169 | SR pressure | bar | 35 | 30-40 | Range reviewed by Megret et al., 2015223 |
| 170 | SR steam:C ratio | molH2O.molC-1 | 3 | 2.5-4 | 3 used by modelers223,247, but 4 also mentioned249. Stoichiometric is 1. |
| 171 | SR CH4 conversion yield | %CH4 | 85% | 76-95% | Base value achieved at pilot scale223,248, but higher value reported by Linde plant247. Lower value reviewed160. |
| 172 | SR outlet losses | %out | 1% | 1-2% | Assumed, no references found. |
| 173 | WCS steam:gas ratio | vol basis | 0.6 | 0.5-0.7 | Based on Megret et al., 2015223. Gas considered dried and without N2. |
| 174 | WCS CO conversion yield | %CO | 85% | 80-89% | Megret et al., 2015 considered 85% while Pham Minh et al., 2018247 used 89%. Lower value assumed. |
| 175 | WCS outlet losses | %out | 1% | 0-1% | Assumed |
| 176 | Methanation conversion yield | %C | 95% | 92-98% | Forecasted performances of 95-98%, currently lower240 |
| 177 | Methanation outlet losses | %out | 1% | 0-1% | Assumed |
| 178 | Methanation electricity, GA | kWh.kgCH4-1 | 1.24 | 1-10 | Proxy of both syngas WCS and methanation. Higher value used by Lodato et al., 20226. 1-1.24 used in ecoinvent (depend on included processes)48 |
| 179 | Methanation electricity, AD | %HHVin | 1.1% | 1.0-1.4% | Higher value is based on forecasted performances240, but dedicated process in ecoinvent corresponds to ca. 1-1.1%48. |
| 141 | Electrolysis efficiency | %HHV | 80% | 70-82% | Range reviewed by Khoshnevisan et al., 202222. Baseline value used by Megret et al., 2015223. |
| 142 | Electrolysis O2 reuse | %O2 | 95% | 10-95% | Share of co-produced O2 effectively substituting the need of pure O2 supplies. Depend on future integration of electrolysis within symbiotic relations (e.g. can fully meet MP production needs; see section SI. 4.6.4.) |
| Other common parameters in in section SI. 4.1 |  |  |  |  |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to MP pretreatment but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15.

Corresponding background processes used specifically for GA LCI model are detailed in Table SI. 71.

Table SI. 71 – Background processes used specifically for MP pathways pretreatments

| Background process | Comment |
| --- | --- |
| Market for cast iron, GLO | Based on biogas-to-hydrogen SR plant modeled by Battista et al., 2017245 |
| Market for concrete, normal, CH |  |
| Market for steel, chromium steel 18/8, GLO |  |
| Market for steel, unalloyed, GLO |  |
| Market for Nickel, class 1, GLO |  |
| market for iron-nickel-chromium alloy, GLO |  |
| Market for Aluminium alloy, AlMg3, GLO |  |
| Market for aluminium, primary, ingot, IAI Area EU27 |  |
| Market for zinc, GLO |  |
| Market for zinc oxide, GLO |  |
| Market for copper-rich materials, GLO |  |
| Market for steel, unalloyed, GLO | Proxies for the four processes mentioned in Table SI. 58 (WCS facilities) |
| Market or cast iron, LO |  |
| Market for steel, chromium steel 18/8, GLO |  |
| Market for zeolite, powder, GLO |  |
| Market for copper oxides, GLO | Proxies for the three processes mentioned in Table SI. 58 (WCS catalyst) |
| Market for zinc oxides, GLO |  |
| Market for aluminum oxide, non-metallurgical, IAI Area EU27 |  |
| Market for steel, chromium steel 18/8, GLO | Proxies for the two processes mentioned in Table SI. 59 (methanation facility) |
| Market for industrial furnace, natural gas, GLO |  |
| Market for Nickel, class 1, GLO | Proxies for the two processes mentioned in Table SI. 59 (methanation catalyst) |
| Market for Aluminum alloy, AlMg3, GLO |  |
| Market for zinc, GLO |  |
| Air separation, cryogenic, RER | Considered as marginal oxygen supply process. (background electricity harmonized) |
| Market for potassium hydroxide, GLO | Proxies for processes mentioned in Table SI. 60 (electrolysis |
| Market for nitrogen, liquid, RER |  |
| Building construction, hall, CH |  |
| Background inventory of the electrolysis stack composition was directly taken from pilot values from Lee et al., 2021242 |  |
| market for aluminium oxide, non-metallurgical, IAI Area EU27 | Proxies for the two processes mentioned Table SI. 65 (SR catalyst) |
| Market for Nickel, class 1, GLO |  |

All processes are from the ecoinvent 3.8 consequential database. EU is used instead of the full name ecoinvent label “Europe without Switzerland”. Additional pipeline network was neglected. Direct flows to biosphere are displayed in B.GA_module spreadsheet.

The spreadsheets B.AD_module (Javourez et al., 20241) and B.GA_module (this study) provide full AD and GA gate-to-upgraded-gas LCI.

### Unit operation design and assumptions – MP production stage

#### Bioreactor

Among the many bioreactor possibilities to develop C1 gas fermentation250, the U-loop design was selected because corresponding to the scaled design for MOB production232. A scaled volume of 250 m3 was considered (including 25% extra-volume)173,203, which remains lower than the 500 m3 technically-feasible threshold calculated by El Abbadi et al., 202140. Due to the similarity with Quorn’s bioreactor (detailed in section SI. 4.5.5), the same height of 50m was considered. Yearly availability was set to 333 d.y-1 to match with AD and GA plant availabilities. The bioreactor runs continuously, only stopped for planned outage (maintenance, cleaning, etc.). Accordingly, MP seed production was neglected. Additionally, for simplifications, no gas storage facilities were modeled; the production is “just-in-time”. Storage would likely be limited for hydrogen as storage losses as reported to yield up to 0.5 %H2.d-1 223. Finally, MP installations were assumed directly located next to AD and GA facilities, to neglect additional gas grid connections and gas transportation.

#### Microbial strains

Table SI. 72 summarizes the main assumptions and indicators considered regarding microbial biomass production.

Table SI. 72 – Review and calculation of MOB and HOB growth performances

| Performance indicators | Unit | MOB |  | HOB |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Range | Value | Range |  |
| Yield (Yx/s) * | gX.gS-1 | 0.71 | 0.63-1.11 | 2.94 | 2.24-3.4 | Parameter: baseline values as El Abbadi and Criddle, 2019236 (MOB) and Nappa et al., 2020196. Range based on benchmark (Table SI. 55) |
| Biomass concentration | gX.L-1 | 30 | 20-40 | 30 | 20-40 | Parameter: MOB and HOB actors mention same range. Average selected as El Abbadi and Criddle, 2019236 (MOB) and Leger et al., 2021175 (HOB) |
| Dilution rate | h-1 | 0.2 | 0.2-0.25 | 0.08 | 0.03-0.1 | Set value: within benchmarked range (Table SI. 55), set to achieve reported productivity |
| Productivity | kg.m-3.h-1 | 6 | 4-6 | 2.4 | 1-3.3 | Calculated: fit with commonly achieved MOB175 and HOB195 productivities |
| Production | kg.h-1 | 1215 | n.a. | 450 | n.a. | Calculated: using productivity and volume |
| Temperature | °C | 40 | n.a. | 30 | n.a. | Set value: typically reported: 35°C selected |
| Metabolic heat | kJ.molCX-1 | 1277 | 900-1660 | 744 | 700-800 | Parameter: thermodynamic balance236, range based on benchmark (Table SI. 55) |
| Metabolic heat equivalent | persons | 16,000 | n.a. | 9,500 | n.a. | Calculated: heat demand met** |

Calculation developed with baseline values for technology parameters. *: S for “substrate”; CH4 for MOB and H2 for HOB. **: considering an average domestic/tertiary heating and hot water demand of 18000 MJh.per-1.y-1 in France (2020)251. Estimated based on a 9000 t.y-1 MP production plant.

Table SI. 72 also shows that HOB displays a lower volumetric productivity compared to MOB. Therefore, HOB production is more CAPEX-intensive than MOB. It means that in the same bioreactor (assumed here), HOB production is ca. 2.5 times lower than MOB. From the growth parameters, the stoichiometric biomass growth equations were derived. Equation 12 and Equation 13 display their baseline version (using baseline values from Table SI. 72), which are in line with the one used by El Abbadi and Criddle, 2019236 and García Martínez et al., 2021197.

Equation 12

MOB, baseline value

CH4+1.5O2+0.1NH3=0.1X+0.5CO2+1.8H2O

Equation 13

HOB, baseline value

H2+0.24O2+0.05NH3+0.26CO2=0.05X+0.9H2O

With X: C5O2H7N being the fixed microbial biomass composition (ash-free)236.

It is important to note that the variability of used stoichiometric equations to model microbial growth was higher (primary sources more diverse) for HOB than for MOB in reviewed literature (see B.MP_module spreadsheet). Albeit some MOB are reported to also use CO2 as a carbon source along with CH4 40, this option was not contemplated in this model. Here, the proposed MP LCI model builds on the outputs of AD and GA LCI models, which supply a carbon- and energy-containing gas mix (CH4, CO2, H2, CO, N2) which is trace gases- and contaminants-free (removed during cleaning operations). This is because these effects inhibit microbial growth (e.g. if H2S > 1000 ppm in biogas; El Abbadi, 2021; Xu et al., 2020) and are considered incompatibles with nutritional applications 23,184. Yet, selected strains were presumed  CO-tolerant and able to use raw syngas at industrial scale as proposed in e.g. Khoshnevisan et al., (2022) and Pelagalli et al., (2023). This is a rather optimistic assumption, but plausible in front of the wide panel of available strains and possibilities offered by synthetic biology 224.

#### Gases supply

Solubility, regulation and injection of important gas volumes in U-loop bioreactors were considered solved challenges in the near future. It was assumed that bioprocesses can cope with N2-diluted gases and explosive/flammable gas mixes 40,196,253. In addition to AD- and GA-based gases, nitrogen and oxygen supplies are also required to meet MP stoichiometric requirements.

##### Oxygen supply

Usually (see Table SI. 56), pathways following an electrolysis-based HOB production directly reuse the oxygen co-produced (presence in excess)199. Yet, here, not all pathways include electrolysis. To be consistent air was considered as the oxygen source, albeit introducing extra volumes of N2 in the bioreactor. This is not a conservative assumption in terms of energy demand, as supplying pure oxygen to the bioreactor requires ca. ten times more power than compressing the additional volumes of nitrogen present in the air (respectively 3.5 kWh.kgO2-1; ecoinvent, 2022; and 0.32 kWh.kgO2-1 calculated, see data repository).

##### Nitrogen supply

In the literature reviewed, two nitrogen supply approaches were reported (Table SI. 56). The first one directly supplies conventional ammonia to the bioreactor, with the argument that microbes will use it more efficiently than plant, and that N-related losses and emissions will be limited187. The second one considers that a bio-based N supply is necessary for MP to perform environmentally better than conventional plant proteins199. For the latter, nutrients recovery techniques (not only N, but also P and others) from residual streams are increasingly investigated to supply bioprocesses22,254. These techniques are particularly targeting digestate (the liquid fraction) and involve different technical solutions (e.g. struvite, stripping, electrochemical extraction)255. Here, N supplies was modeled as conventional N fertilizer services, as for all fermentative processes (including SSF and mycoproteins). The option to recover nitrogen from AD’s digestate was not assessed for three reasons: digestate is a constrained resource (thus not available on demand), unravelling the best N source to supply bioprocesses remains an open research question (Matassa et al., 2023; Spiller et al., 2022), and finally, only advanced and multiple risk-clearing barriers N recovery pathways would unlock the safe reuse of waste-based N for food and feed services (see risk mitigation criteria in section 2.1.3.). These recovery technologies are currently immature 22, preventing robust modeling. Additionally, syngas was considered N-gases-less; all N embedded in the biomass returns to the air, soil or water bodies.

#### Gas injection

Gases compression is a significant contributor to the infrastructure and energy expenses of bioreactors186. Here, the selected U-loop design is based on a head pressure of 6 bars to favor gas-liquid transfers, based on Unibio design199 (but set as a parameter, see Table SI. 81). The hydrostatic pressure due to the 50m height was accounted, because of the bottom gas injection design to ensure proper mixing of the bioreactor. Related energy requirements were calculated as described in section SI. 4.1.2. Gas-liquid transfers are also increased through the implementation of efficient gas recirculation system208, but associated energy was neglected (no drop in pressure). These systems were assumed designed to achieve the gas use rates as defined in Table SI. 81. All compressed gases were modeled as air at this stage, because of their relative high share of O2 and N2. Finally, the gases and air were considered sterilized through filter/membranes (<0.2 µm)41,217, but in front of the low utilities requirements, the inventory related to this stage was neglected.

#### Off-gases reuse

Sensible heat recovery of off-gases (e.g. mentioned in Levett et al., 2016186) was neglected given the low temperatures involved. As these still host non-neglectable enthalpy (e.g. unused CH4, CO, H2 and NH3), their energy valorization was modeled as detailed in section SI. 4.1.6. Effects of low-energy density due to N2 dilution were neglected. The share of gases dissolved in the liquid medium was neglected: all fractions were assumed as part of off-gases.

#### Metabolic heat

Cooling is required to maintain mesophilic conditions (30-40°C) in the bioreactor, due to the exothermic oxygen reduction by HOB and MOB (Table SI. 72). Yet, passive cooling systems were deemed insufficient 208,  and bioreactor cooling was already reported as the most important contributor to MOB production’s operational expenses. For example, it uses around 60% of installed power according to El Abbadi et al., (2021) and represents 33% of total production expenses in the calculation of Levett et al., (2016). Yet, this low temperature (LT) heat to evacuate (at around 35°C) can partly be recovered through tailored WHR techniques (see section SI. 4.1.6. for modeling technologies and limitations).

#### Cleaning requirements

Periodic installations outage is not only required for maintenance and cleaning, but also to (i) limit the accumulation of extra-cellular compounds in the production cycle and (ii) prevent apparition of mutants. Cleaning was modeled based on the inventory proposed by Järviö et al., 2021183, which includes a clean in place (CIP) procedure for the bioreactor, media preparation, transfer pipelines and downstream processing based on common food industry-grade CIP (see Table SI. 73). To assess the influence of cleaning frequency, this was set as a technology parameter (see Table SI. 81).

Table SI. 73 – Bioreactor cleaning modeling parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NaOH | kg.CIP-1 | 1758 | From Järviö et al., 2021183 , but adapting the production capacity of (160 kg.h-1) to the ones of this study (see Table SI. 72). Cleaning wastewater considered at 5.3 gCOD.L-1 183. |
| HNO3 | kg.CIP-1 | 1118 |  |
| Water | m3.CIP-1 | 1406 |  |
| Autoclave | MWh.CIP-1 | 155 |  |
| Wastewater | m3.CIP-1 | 1410 |  |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants

#### Medium preparation

Microbial culture requires many trace compounds and micronutrients (e.g. Cu, Fe, Mo, Ni, etc.). These seems hardly recoverable from residual substrates as not targeted by current nutrient recovery technologies and likely difficult to separate from undesirable/hazardous compounds22. Therefore, medium was assumed formulated from raw materials. Requirements were assumed the same for HOB and MOB, and were based on already existing inventories (see Table SI. 74).

Table SI. 74 – Medium formulation modeling parameters

| Compounds | Unit | Value | Comment |
| --- | --- | --- | --- |
| P | mg.gX-1 | 30 | Estimated at 3%DM of HOB and MOB186,187,199 |
| K |  | 5 | 183 |
| MgSO4 |  | 25 | 186 “Market for magnesium sulfate, GLO” |
| CuSO4 |  | 0.79 | 186 “Market for copper sulfate, GLO” |
| MnSO4 |  | 1.39 | 186 “Market or manganese sulfate, GLO” |
| FeSO4 |  | 1.68 | 186 “Market for iron(III) sulfate, without water, RoW” |
| ZnSO4 |  | 0.16 | 183 “Market for zinc monosulfate, RER” |
| CoSO4 |  | 0.06 | 183 Inventory of Marami et al., 202231 |
| Al2(SO4)3 |  | 0.19 | 186 “Market for aluminium sulfate, powder, RER” |
| CaSO4 |  | 7.42 | 183 “Market for calcium carbonate, precipitated, RER” |
| Na2MoO4 |  | 0.04 | 186 Inventory of Marami et al., 202231 |
| H3BO3 |  | 1.29 | 186 “Market for boric acid, anhydrous, powder, GLO” |
| CrCl3 |  | 0.08 | 186 Inventory of Marami et al., 202231 |
| NaVO3 |  | 0.01 | 183 Inventory of Marami et al., 202231 |
| NiSO2 |  | 0.02 | 183 Inventory of Marami et al., 202231 |
| CeO2 |  | 0.01 | Assumed, “Market for cerium oxide, GLO” |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants. X: microbial biomass. Antifoam was neglected. P & K requirements LCI were modeled as fertilizers services.

Minerals and micronutrients were assumed mixed with make-up water (impeller) and further sterilized (120°C) before being fed to the bioreactor, as reported for the Pruteen process41. Membrane-based sterilization is also mentioned, but in front of the lack of data, only conventional steam-based sterilization was modeled. An over-pressure was considered to avoid vaporization of media at the sterilization temperature (see section SI. 4.1.3). Growth media was pumped into the bioreactor (accounting for total required pressure as for gas injection), corresponding energy was calculated as also detailed in section SI. 4.1.3. As sterilized media needs to be cooled towards bioreactor’s working temperature (30-40°C), sensible heat was assumed directly reused to pre-warm water make-up input flow (details in B.MP_module spreadsheet). Accordingly, only the heating and cooling requirements to bridge the remaining temperature gaps were considered (see Table SI. 77). Water make-up estimation is detailed in next sections.

#### Post-treatments

The post-treatment unit operations are water removal, biomass inactivation and MP ingredients formulation. These operations (especially dewatering and drying) are usually contributing to a large share of food industries’ energy expenses, and previous assessments tend to show a similar trend for MP production 186,192. Yet, reviewed literature did not provide a clear and unified design for the post-treatment operations sequence. For example, some scaling initiatives report the implementation of a sterilization stage (e.g. 140°C mentioned by Unibio and 120°C by Solar Foods)173,183,203 while other studies do not consider sterilization but only a RNA/DNA removal stage. The latter can be done either by (i) heat shocks (i.e. broth’s temperature quickly brought to 60-70°C with a 260°C steam injection at 13bars, as reported for the Quorn process by Upcraft et al., 2021131), (ii) bead milling as proposed by Leger et al., 2021175 or (iii) alkaline hydrolysis as reported for the Pekilo process by Van Pethem et al., 202241. RNA/DNA represents ca. 7-11%DM of bacterial biomass, and is mostly responsible of the “human intolerance of bacteria as food”258. Albeit often reported as unnecessary to supply compound feed markets (as farmed animals host the uricase enzyme175), unprocessed bacterial meals are rather targeting animals with short life-spans236. Here, RNA removal was modeled as an option, but not considered for MP pathways are these only target feed uses. Corresponding modeling is detailed in section SI. 4.5.5.

The other post-treatment unit operations were based on the sequence implemented by Unibio (as reported by García Martínez et al., 2021, 2020197,198), for being the current higher TRL example of HOB/MOB production. First, a hollow-fiber ultrafiltration (UF) system was selected to concentrate broth with reduced energy requirements (Table SI. 75). Separated water was directly mixed with centrifuge outlet. Then, high-temperature pasteurization is performed (105°C), complemented by the RNA heat-shock treatment if the option is selected. To simplify modeling, heat shocks were assumed performed at the same temperature than pasteurization, considering no additional energy expenses but only an induced biomass loss (23%, ranging 15-30%41). Pasteurized or heat-treated broth was further centrifuged (Table SI. 75). Abiotic conditions (>90°C131) were maintained until the last drying operation, which also allowed to limit spray drying energy expenses. Evaporated water at the spray-drying stage was assumed lost, with no heat recovered. Yet, embedded 120-160°C sensible heat could be used to preheat air inlet44, but this heat synergy was considered indirectly included within the spray-drying energy requirement parameter (Table SI. 75).

Table SI. 75 – MP post-treatments modeling parameters

| Parameter |  | Ultrafiltration | Centrifuge | Spray drying |
| --- | --- | --- | --- | --- |
| Output (%DM) | Value | 13% | 30% | 91% |
|  | Range | 10-15% | 20-35% | n.a. |
|  | Comment | 183,196 | 20-22% commonly used183,191,204. But 35% achievable199. De Boer et al., 2014173 used 30%. | fixed |
| Electricity required (kWh.m-3) | Value | 0.04 | 2.53 | 0.11 |
|  | Range | 0.04-0.15 | 1.35-8 | 0.03-0.28 |
|  | Comment | 183,196 | Range reviewed by Gayen et al., 2019259. Many studies use 1.35175,191,196. | Review by Spykman et al., 202167 |
| Heat required (MJ.kgwater-1) | Value |  |  | 3 |
|  | Range |  |  | 2.7-4.1 |
|  | Comment |  |  | Higher range used by de Boer et al., 2014173. Best reported by Nappa et al., 2020196. MP assessment usually uses 3.5-3.7175,183,187 Base value lower range for heat synergies. |
| Biomass loss (%in) | Value | 0% | 3% | 0% |
|  | Range | n.a. | 1-6% | n.a. |
|  | Comment | fixed | Higher value when double stage centrifuge41, lower value assumed by Marami et al., 2022199. Base value as single stage41,186. | fixed |

RNA treatment not represented as not retained for MP production.

#### Water recovery and make-up

Warm centrifuged water outlet was cooled by mixing with UF outlet water; no specific inventory was modeled (passive system). Most of benchmarked assessments (Table SI. 55) consider that water should be recycled. Indeed, water consumption was reported as a critical aspect for viable MP production, as these are cultured in highly diluted media (Table SI. 72). Moreover, recycling water enables to recover unused nutrients. Nevertheless, a share of water needs to be purged to prevent accumulation of nutrients and neoformed compounds41, this share was defined as a parameter (Table SI. 81). The recirculated broth was considered only UF-treated, albeit Järviö et al., 2021183 considered reverse osmosis might be required. Recirculation energy expenses also includes liquid pumping towards required pressure to enter the bioreactor. The rejected water stream was sent to a WWTP. To select the adapted WWTP background process, the method explained in section SI. 4.1.8 was applied. Fractions present in the rejected stream were calculated by mass balance (see B.MP_module spreadsheet). Selected WWTP background process is displayed in Table SI. 82. Make-up water volumes were calculated with mass balance, and depend on many technology parameters. An overview of the water flows calculated with the baseline values is illustrated in Figure SI. 2. Considering a baseline water recycling of 92%, make-up water requirements only sum to ca. 2.2 L.kgDM-1 MP produced, which corresponds to 6% of total bioreactor’s requirements. This means the direct water consumed by MP production would rise by ca. 16 times if no water recycling systems were implemented (36.3 L.kgDM-1).

Figure SI. 2 – MP production stage water balance to produce 1kgww, illustrated for baseline parameter values

Metabolic water comes from Equation 12 and Equation 13.

Evaporated water was assumed lost to the atmosphere (albeit some studies argue that spray-dried-based water can be recovered to feed e.g. an electrolyser41). The origin of water in reviewed benchmark mostly comes from filtered tap water (selected option, see Table SI. 82), sometimes considering osmosis as pre-treatment183. Humidity brought by air was neglected, as well as humidity lost through exhaust gases.

#### Final ingredient

Dried HOB and MOB were assumed to supply compound feed markets (therefore, entering the CF model after the drying stage). Accordingly, the unit operations modeled for CF pathways (grinding and pelletizing) were reused. The following compositions for HOB and MOB were considered (Table SI. 76) to derive their nutritional value through the calculation detailed in section SI. 4.1.9.

Table SI. 76 – MP composition

| Composition (%DM) | MOB | HOB |
| --- | --- | --- |
| Proteins | 67.0% | 65.0% |
| Lipids | 9.5% | 6.0% |
| Carbohydrates | 2.5% | 2.0% |
| Sugars | 4.5% | 0% |
| Fibers | 0.5% | 11% |
| Ashes | 8.0% | 8.0% |
| RNA/DNA | 8.0% | 8.0% |
| Digestibility (%OM) | 85% | 85% |
| SFU.kgDM-1 | 1.51 | 1.41 |
| Comments | Average of reported values for Uniproteins and Feedkind 173,203,235 | Adapted from Solar Foods183 |

Fixed compositions. Initially several compositions were modeled but yielding low overall influence on the results. SFU: Scandinavian feed unit.

#### MP production key performance indicators summary

This section summarizes the main outcomes of the MP production model, and compare them with benchmarked literature. The Table SI. 77 displays the heat integration modeling of MP production, while Table SI. 78, Table SI. 79 and Table SI. 80.compare achieved performances (by MP LCI model) with references.

Table SI. 77 – Heat recovery modeling involved in MP production and post-treatments

| Unit operation | Heat recovery | Temperatures | Modeling |
| --- | --- | --- | --- |
| MP production | Metabolic heat | 30-40°C | LT WHR |
| MP production | Off-gases enthalpy | n.a. | Gaseous co-products |
| Sterilization | Medium cooling | 66°C to 30-40°C | LT WHR (after heat exchanger) |
| Recirculation | Centrifuge outlet cooling | 105°C to 30-40°C | Mixing with UF outlet and passive system |
|  | Only processes linked with the MP stage |  | See section SI. 4.1 |

Table SI. 78 – Energy performance of MP production and post-treatments

| Unit operation | Electricity* (kWh.kgDM-1) | Heat* (MJ. kgDM-1) |  |  | Van Peteghem et al., 202241 (kWh.kgDM-1) |
| --- | --- | --- | --- | --- | --- |
|  |  | total | w/ off-gases | w/ off-gases nor LT heat |  |
| MOB | 5-6 | ca. -20 | ca. -15 | ca. 9 | 6-21 reviewed among various metabolic routes |
| HOB | 3.5-4.5 | ca. -13 | ca. -5 | ca. 9 |  |
| Comment | This study, calculated with baseline parameter values |  |  |  | same scope, no RNA removal both heat and electricity |

*: range or approximation because depend on composition of input gas mix. Only accounting direct utilities at the bioreactor gate (not the ones related to gas production and post-treatments).

Table SI. 79 – Productivity of the full second-generation MP production routes

|  | Waste-to-MP production (kgDM.tww-1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case studies | MP biogas | MP biometh | MP shiftb | MP syngas | MP meths | MP shifts | Pikaar et al., 2018187 | Matassa et al., 2020194 |
| Hardwood residues |  |  |  | 59 | 305 | 182 |  | 31-92 |
| Sawmill waste |  |  |  | 58 | 299 | 179 |  |  |
| Wheat straw | 108 | 159 | 180 | 64 | 330 | 197 | 179-233 |  |
| Bovine manure | 29 | 42 | 48 |  |  |  |  | 65-124 |
| Sewage sludge | 8 | 12 | 13 |  |  |  |  | 61-117 |
| Countryside management grass | 48 | 71 | 81 | 30 | 155 | 93 | 181 |  |
| Sugarbeet tops | 21 | 31 | 35 | 12 | 61 | 37 |  |  |
| Household food waste | 36 | 53 | 60 | 22 | 114 | 68 |  | 31-91 |
| PAT | 146 | 215 | 243 | 63 | 327 | 196 |  |  |
| Corn spent grains | 55 | 82 | 92 |  |  |  |  |  |
| Potato screenings | 29 | 42 | 48 |  |  |  |  |  |

Calculated with baseline value of technology parameters. Amounts are directly linked with input moisture content, which can explain differences with reported references. Matassa et al., 2020 contemplates AD reuse towards gasification, here excluded.

Table SI. 80 – Calculated biomethane production compared to experimental benchmark

| Case studies, in Nm3CH4.tVS-1 | Calculated, baseline yield (75 %TBMP) | Trials by Garcia et al., 2019260 | Review by Bareha et al., 2021261 |
| --- | --- | --- | --- |
| Wheat straw | 283 | 196 ± 43 (soybean straw) | 320 ± 35 (rye straw) |
| Bovine manure | 278 | 98 ± 9 | 272 ± 30 |
| Sewage sludge | 390 | n.a. | 372 ± 19 (liquid) |
| Countryside management grass | 288* | 322 ± 42 (grass) | 330 ± 16 (grass clippings) |
| Sugarbeet tops | 311 | 312 (carrot leaves) | n.a. |
| Household food waste | 319 | 300-700 (various food items) | 275 ± 16 (kitchen waste) |
| Corn spent grains | 368 | 289 ± 86 (maize) | 400 ± 13 (waste from yeast production) |
| Potato screenings | 317** | 446 (potato peels) | 400 ± 20 (potato peelings) |
| PAT | 421 | 460 (pig meat) / 318 (fish waste) | 313 ± 19 (refusal slaughterhouse) |
| Comments | Calculated values are linear functions of the feedstock composition. *: Ash and lignin content both at ca. 10%DM explain the lower calculated performance compared to references. **: likely limit of the TBMP proxy for homogeneous streams (here ca. 75% starch, underestimated BMP) |  |  |

### Technology parameters and background processes – MP production stage

The technology parameters retained for MP pathways modeling (excluding gas post-treatment operations, already detailed earlier), are displayed in the Table SI. 81.

Table SI. 81 – Parameters specific to MP production stage and downstream operations

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 126 | MP O2 use efficiency | %O2 | 90% | 80-92% | Unibio reports 90%198, 80% often use in MP assessements187,194. Higher value used for mycoproteins. |
| 127 | MP NH3 use efficiency | %NH3 | 90% | 80-92% |  |
| 128 | MP bioreactor head pressure | bar | 6 | 5-8 | 6 mentioned by Unibio199, 5 reported by Calysta25, can reach 8184. |
| 129 | MP CIP frequence | year-1 | 7 | 4-12 | Unibio mentions 8-12173,185, 4 for Solar Foods183. |
| 130 | MP mineral medium use | %supplied | 95% | 80-95% | Aspen+ model by Levett et al., 2016186 gives 95% maximum. 80% lower reported value183. |
| 131 | Medium stirring energy | kWh.m-3 | 0.3 | 0.2-0.5 | Common range for impellers mixing |
| 132 | MOB yield | gVS.gCH4-1 | 0.71 | 0.63-1.11 | See Table SI. 72 |
| 133 | MOB CH4 use efficiency | %CH4 | 90% | 80-90% | As for O2 and NH3, see line. |
| 134 | MOB culture density | g.L-1 | 30 | 20-30 | See Table SI. 72 |
| 135 | MOB metabolic heat | kJ.molCX-1 | 1277 | 900-1660 |  |
| 136 | HOB yield | gVS.gH2-1 | 2.94 | 2.24-3.4 |  |
| 137 | HOB H2 use efficiency | %H2 | 90% | 80-99% | 90% already achieved262, but can be enhanced towards >95%183,196. Lower range often used187,191,194. |
| 138 | HOB CO2 use efficiency | %CO2 | 90% | 80-99% |  |
| 139 | HOB culture density | g.L-1 | 30 | 20-30 | See Table SI. 72 |
| 140 | MOB metabolic heat | kJ.molCX-1 | 744 | 700-800 |  |
| 146 | Share of water reused | %outlet | 92% | 80-92% | 92% to be in line with medium use as in Levett et al., 2016186. Usually 80-90% are considered183,184. |
| 147 | MP sterilization temperature | °C | 105 | 90-120 | Average of 90°C (current Quorn process) and 120°C (mentioned Solar Foods)183 |
| 148 | MP RNA removal losses | %DMin | 23% | 15%-30% | Review different techniques41 |
| 24 | UF electricity | kWh.m-3 | 0.04 | 0.04-0.15 | See Table SI. 75 |
| 25 | UF DM out | %DM | 13% | 10-15% |  |
| 26 | Centrifuge DM out | %DM | 30% | 20-35% |  |
| 27 | Centrifuge DM losses | %DMin | 3% | 1-6% |  |
| 28 | Centrifuge electricity | kWh.m-3 | 2.53 | 1.35-8 |  |
| 15 | Spray dryer electricity | kWh.kgwater-1 | 0.106 | 0.034-0.282 |  |
| 16 | Spray dryer HT heat | MJ.kgwater-1 | 3 | 2.7-4.1 |  |
| Other common parameters in section SI. 4.1. |  |  |  |  |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to MP production stage but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15

The background processes used specifically for this LCI model are gathered in Table SI. 82.

Table SI. 82 – Background processes used specifically for MP production and downstream operations

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Market for sodium hydroxide (50%), GLO | Cleaning | Proxy for the processes mentioned in Table SI. 73. |
| Market for tap water, CH |  |  |
| Market for nitric acid, without water (50%), RoW |  |  |
| Treatment of wastewater from maize starch production, CH |  | Proxy of cleaning wastewater treatment, closer based on COD content (see Table SI. 73) |
| Background inventory for mineral medium already detailed in Table SI. 74. |  |  |
| Market for ultrafiltration module, GLO | Ultrafiltration | 1.12 x 10-7 .kgMP-1 based on Järviö et al., 2021183 |
| Treatment of wastewater from anaerobic digestion of whey, CH |  | Closest proxy of reject wastewater treatment (ca. 13 gCOD.L-1 based on Table SI. 18). Background electricity was replaced by the one of this study as treatment assumed to take place in France. |
| Chemical factory construction, RER | n.a. | Proxy of the MP production facility. Amount adapted to this plant’s production capacity and lifetime: 3.6 g.kgMP-1. |
| Rest of background processes used for MP pathways are detailed in AD and GA models, and Table SI. 71. |  |  |

All processes are from the ecoinvent 3.8 consequential database. Direct flows to biosphere are displayed in B.MP_module spreadsheet.

Background processes and technology parameters associated with transversal unit operations between many models (e.g. pretreatments, transportation, etc.) are displayed in the common section SI. 4.1. Rest of modeling details (e.g. all biosphere flows) are available directly within corresponding excel model.

The main limit of the present model lies in its limited coverage of the many possibilities offered by microbial bioconversion. Among others, implementing a sequential or mixed-cultured microbial production could reach higher substrate-to-proteins conversion efficiencies 263. The use of lipid-rich organisms 264 could supply lipids feed services, and engineering the amino acids profiles of MP could tailor their nutritional composition to specific end-uses 224. Indeed, here, MP production was assessed to supply generic feed services, while targeting more specific bio-compound markets would likely enable larger unitary environmental savings (but smaller volumes).

## Rubisco proteins extraction from green residues LCI model

This section presents the details (including assumptions, choice rationales, calculations and data sources) of the green biorefinery (GB) life cycle inventory (LCI) model, with the objective to perform the LCA of proteins extraction from leafy streams. The present model, concentrating on the protein’s extraction and valorization stages, is available in spreadsheet B.GB_module spreadsheet. In front of the many possibilities to manage the co-products, three were selected (further described), leading to three individual pathways: GB_AD, GB_rough and GB_mycofood. The LCA results of GB pathways are therefore obtained by pairing the present model with the one corresponding to the selected co-products valorization option (AD for GB_AD, CF for GB_rough and Mycoproteins production for GB_mycofood).

### Set-up selection

As many set-ups exist as different targeted functionalities/markets of the generated protein extracts and corresponding co-products. For example, in France, despite an established alfalfa biorefining supply chain (producing ca. 0.85 Mtww.y-1 dried alfalfa forage265), alfalfa LPC production still remain rather a niche market for both animal feed (e.g. Desialis’ PX product targeting pigments for poultry production266) and human food (“Novel food” approval of alfalfa protein isolates as food supplement in 2009267). Nevertheless, ongoing projects (non-exhaustive list in e.g. Table 7 of Krenz and Pleissner, 2022268 and Table 18 of Hermansen et al., 2017269) still aim to rich the broader objective of making green biomass proteins accessible to monogastric animals270. Moreover, so far, available GB assessments mostly target premium green crops, not residual streams (Table SI. 83). Yet, at the lab-scale reported GB refinement of green residual streams include vegetable tops (that is, potato and sugarbeet tops271–273, beans and peas leaves274, kale and broccoli leaves275, carrots and cabbage leaves271, cassava tops276,277, among others) and unused or underused grass streams (such as countryside management grass268, road verges and watercourse clearings278, peatlands and coastal zones279, unharvested meadows, up to parks, gardens, sport courts, airport and military green areas280). This is because, despite different legal and safety compliance challenges (see main manuscript), biorefining of green residual biomasses faces technical challenges compared to dedicated green crops due to their composition, seasonality and spatial dispersion.

Indeed, unmanaged grasslands host higher plant heterogeneity (in particular higher share of poaceae grass), lower protein and higher fiber content leading to lower nutritional values for ruminants, with significative changes even in a weekly basis268,269. The same applies for vegetable tops, as harvesting period is primary defined by the main product (that is the root, tubercula or vegetable), which does not necessarily coincide with the best harvesting period for biorefining the aerial fraction. For example, sugar beet tops are already decaying when the sugar beet roots are harvested (at 6-7th month after sowing), with already lower protein content and higher ash content than at their optimum period272. Vegetable tops are composed by a two matrices structure (blades and stems) which also lead to complex interactions that can yield both positive (e.g. stems acting as filter bed when pressing281) and negative (e.g. particles within the green juice271) effects on overall process efficiency. These considerations limit the implementation of a single standardized GB set-up, which would rather be tailored for each specific input271. Nevertheless, in an endeavor to explore the potential of green biorefining to supply the food, feed and energy markets when sourcing residual biomass feedstocks, a single protein extraction procedure was modeled herein (but three different co-products management were considered). The main performances indicators were derived and compared with similar studies benchmarked (Table SI. 83).

Table SI. 83 – Published LCA of green biorefinery pathways and assimilated

| Reference | Study | Feedstock | Data | Comment |
| --- | --- | --- | --- | --- |
| Skunca et al., 2021282 | aLCA | Sugarbeet tops | Unknown | Rubisco isolate (results only, Royal Cosun) |
| Parajuli et al., 2018283 | cLCA | Grass-clover | Secondary | Integration of GB in livestock systems |
| Parajuli et al., 2017284 | cLCA | Alfalfa | Secondary | Lactic acid, with protein as co-products |
| Prieler et al., 2019285 | aLCA | Grasslands | Primary (pilot scale) | Amino acids production from grass silage |
| Muica, Msc thesis 2014280 | cLCA | Residual grass | Secondary | Exploratory study, residual grass |
| Carbotech, 2000286 | aLCA | Grass | Primary (pilot scale) | Grass refinery, confidential, used in ecoinvent |
| Bühle et al., 2011, 2012287,288 | aLCA | Grasslands | Secondary | Target energy recovery, not proteins |
| Corona et al., 2018a,b289,290 | aLCA & TEA | Grass-legumes | Primary (pilot scale) | Scenarios assessing different uses of the generated (co-)products |
| Broader assessments of green biorefineries useful for LCI modeling |  |  |  |  |
| Tamayo Tenorio et al., 2017272 | Exergy | Sugarbeet tops | Primary (lab scale) | Mass and energy analysis of different GB set-ups and supply chains |
| Kamp et al., 2019291 | Emergy | Grass-clover | Secondary |  |
| Djomo et al., 2020292 | Energy | Grass-clover | Primary (pilot-scale) |  |
| Muneer et al., 2021293 | TEA | Intermediate crops | Secondary | Based on primary field data for intermediate crop system |
| Höltinger et al., 2013294 | TEA | Grass silage | Secondary | Includes global sensitivity analysis |
| O’Keeffe et al., 2010, 2011295,296 | TEA | Grasslands | Secondary | GB business model assessment |
| Tampio et al., 2019297 | TEA | Grass silage | Secondary | GB (juice stage only) with pig production |
| Chan et al., 2024298 | cLCA | Grass | Secondary | Combination of GB with power-to-X |
| Thiébeau et al., 2023299 | aLCA | Alfalfa | Primary (industry) | Key performance indicators of alfalfa concentrate production in France. |

cLCA: consequential LCA, aLCA: attributional LCA, TEA: Techno-economic analaysis

Maximizing proteins recovery yield for nutritional purposes was selected as the main driver for technology and set-up selections, which is in-line with business development assessments 272,290,296,300. Only the recovery of feed-grade LCP was considered, food-grade ingredients production from residual streams was discarded. This is because this approach primarily targets primary green crops (e.g. alfalfa), as the concentration, purification and isolation of food-grade fractions are more easily implementable on already food-grade biomasses301,302. Moreover, a single example scaled example of food-grade extract from residual biomass was found (Rubisco extraction from sugarbeet tops)282, currently under application for the “Novel Food” approval but without publicly available data. Moreover, albeit green biorefining strategies aiming to reach food markets are documented, these differ in the targeted final service. On one hand, an overview of granted patents indicates that scaling projects rather target high value functionalities (e.g. emulsifying, setting agent, etc.) of purified proteins rather than their sole nutritional value303–305. Authors point the intense purification strategies which are required to deliver a food-grade protein, which damage the overall yield and stress the need to reach a higher value addition of the final products on markets (compared to current plant-based proteins mix) for economic viability306. Indeed, one of the main issues when extracting food-grade proteins from green biomasses lies in the polyphenols-proteins interactions and chlorophyll degradation which lead to undesirable neoformed compounds (bound to protein complexes) when using common protein extraction methods (e.g. heat and acid treatments)293,307. Such anti-nutritional compounds lead to loss of functionality, off-flavors of the extracts, and even its unsuitability as food ingredients (e.g. cyanide in cassava LPC)276. On the other hand, an alternative documented set-up consists in extracting two different protein fractions: an animal-grade one called “green proteins” and a food-grade one called “white proteins”275,290,293. Authors argue to target the plant-based proteins market (e.g. ingredient to formulate milk alternatives)290, but no data is made available to sustain this possibility. Therefore, in the framework of this study, it was assumed that food-grade protein extractions from green biomass will promptly stem from dedicated cultures, not residual biomass, so, this option was not modeled. Instead, residual biomass-based LPC was modeled to target monogastric animal feed as the process is already established at industrial scale, and the data availability is sufficient to enable representative LCI modeling (albeit most assessments are based on secondary data and only display intermediary calculation indicators; Table SI. 83). Importantly, no biomass storage was considered to maximize yield, safeguard proteins and prevent microbial contamination. Indeed, fresh green biomass spoils on an hourly basis: authors mention to start processing no longer than 2-6 hours after cutting, which is only extendable if a cold chain is implemented266,272. For the energy costs of refrigerating water-rich streams272, refrigerating (or freezing) storage was not considered here. Silage-like storages also damage green biorefinery efficiency for inducing proteolysis (leading to lower proteins coagulation potential)294,295, these were not considered neither.

Albeit if extremely decentralized (e.g. container-like) facilities are economically implementables296,308 sometimes necessary (to allow fast processing of fresh biomass)268, and already available270, a regional-scale set-up was modeled to be consistent with the rest of the waste-to-nutrition pathways. Indeed, Nynas, 2022271 argues (in the case of southern Sweden) that a mix of green biomasses can be found to supply a green biorefinery almost all year round. For example, vegetable production residues such as potato tops can supply the summer months along with alfalfa and grass-clover, while sugarbeet tops can supply the autumns months and vegetable production residues (e.g. kale) again supply the winter months. Framing a spatially-explicit distribution and operationality of biorefineries, as well as designing their feedstock sourcing coverage radius is out of the scope of this study, but it was assumed that a solution exists to justify the implementation of year-round regional-scale green biorefineries with reduced storage.

### Unit operation design and assumptions

#### Handling

Green residual biomass streams are mostly left on land after cutting (as part of main product harvesting or due to current management practices), their harvesting was modeled as detailed in Javourez et al., 20241 for crop residues. No losses were specifically considered at this stage, these being rather defined in the biomass resource availability (e.g. share effectively harvestable, etc.) than in the present FU. In front of the need of fast processing and careful handling of green streams, no intermediary collection platforms were considered and the biomass was directly sent to the GB facility. Transportation systems to limit leaves damaging (e.g. keeping density to 70-90 kg.m-3)272,293 were not specifically modeled.

#### Pretreatment

Streams were first discharged to the biorefinery conveyor-belt system and washed to remove undesirable fractions (soil, sand, rocks, etc.) which are detrimental to process operationality. Washing water pumping was accounted, and resulting wastewater was assumed to be reused on field nearby (without specific treatment nor transportation), yet not necessarily replacing irrigation water (depend on location, season, pedoclimatic conditions, etc.). Therefore, this was set as an option (Table SI. 84). The share of water not replacing irrigation was accounted as directly discharged to soil (modeled as a biosphere flow in the inventory). No other pretreatments were considered before the juice separation stage, except maceration (see next section). Heat or weak acid pretreatments, albeit documented309,310 , rather target biomass energy valorization, and likely induce negative effects on protein recovery efficiencies (e.g. protein denaturation)311. Other emerging pretreatments such as electric field pulses could enhance protein extractability300,312, but not enough data exist to document the performances and model them. Similarly, leaves and stems separation as well as visual sorting of damaged or discolored leaves were not considered as these actions are not seen implementable at the industrial scale under current circumstances272,275. Equipment and facilities were estimated using a global proxy for the whole biorefinery (Table SI. 93).

Table SI. 84 – GB pretreatment modeling parameters

| Parameter | Unit | Value | Range | Comment |
| --- | --- | --- | --- | --- |
| Feed-in conveyor energy | kWh.tww-1 | 0.12 |  | 295 |
| Washing water pumping | kWh.kgwater-1 | 0.7 |  |  |
| Washing water | m3.tww-1 | 33 | 12-87 | Proxy from the spinach processing industry313 |
| Irrigation replacement | % | 50% | 0-100% | To account for the potential reuse of washing wastewater. Full variation range as depend on the specific context. |
| Forwarding | PMH. tww-1 | 0.315 |  | Assumed, to move biomass within the GB facility |

#### Juice separation

A literature benchmark was developed to capture range of variations of the main parameters (Table SI. 85). Besides the scale (laboratory, pilot, etc.), reported performances of juice separation differ between installations due to different pressing technologies and set-ups.

Range of performances were selected according to the generic data quality guideline (Table SI. 7), acknowledging that juice extraction efficiencies for residual streams are generally lower than for premium green crops. Table SI. 86 further summarizes the modeling details selected for this unit operation. A necessary yet strong assumption lies on the relative independence in-between transfer coefficients of different fractions, and also with energy consumption. Yet, in turns, this modeling flexibility allows to screen wide sets of performances intrinsically linked with the inherent variability in the compositions of input feedstocks. Therefore, no specific technology choices were made: even if the baseline was designed to be representative of twin screw-press scaled performances, range of parameters consider the performances of single screw-press or two stages twin screw-press configurations (Table SI. 86). Overall, no losses were considered, matter either ends up in the press pulp or the green juice. Washings to increase yields283 or prevent over-warming when pressing295 were assumed developed internally with the same green juice, hence not requiring any external water addition (only pumping was accounted as in O’keeffe et al., 2011295). The filtration of green juice is assumed performed285, but was not explicitly modeled. Filtrates are mixed with the press pulp and selected transfer coefficients were assumed to already account for these (Table SI. 86).

Table SI. 85 – Benchmark of green juice extraction performances

| Reference | Data | Maceration |  | Juice separation (%juice/in) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Type | kWh.tww-1 | fresh matter | dry matter | proteins* | fat | Fibers** | solubles | ashes | C | P | K | S | kWh.tww-1 |
| 281 | Lab experiment | No |  | 77% | 58% | 69% | 59% | 19% |  | 89% |  |  |  |  |  |
| 314 |  |  |  | 57-71% | 29-36% | 35-43% |  |  |  |  |  |  |  |  |  |
| 315 |  |  |  | 60% | 25-42% |  |  |  |  |  |  | 44-47% | 57-60% | 45-48% |  |
| 316 |  |  |  | 60-70% |  | 50-57% |  |  |  |  |  |  |  |  |  |
| 317 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.3 |
| 295 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.5 |
| 318 |  |  |  |  | 43-46% | 33-61% |  |  |  |  |  |  |  |  |  |
| 275 |  |  |  |  | 10-20% | 39-44% |  | 4-9% |  |  |  |  |  |  |  |
| 279 |  |  |  |  | 18-38% | 19-48% |  |  |  |  |  |  |  |  |  |
| 276 |  |  |  |  | 30% | 31% |  |  |  |  |  |  |  |  |  |
| 319 |  |  |  |  | 31-44% | 43-63% |  |  |  | 27-100% |  |  |  |  |  |
| 271 |  |  |  |  |  | 15-53% |  |  |  |  |  |  |  |  |  |
| 272 | Pilot experiment | No |  | 65-77% |  |  |  |  |  |  |  |  |  |  | 11.7 |
| 310 |  |  |  |  | 18-28% | 25-39% | 8-36% | 6-26% |  | 22-38% |  | 50-72% | 56-72% | 51-63% |  |
| 288 |  |  |  |  | 18% | 35% |  |  |  | 43% |  | 67% | 78% | 47% |  |
| 289 |  | No |  | 67% | 33% | 45% |  |  |  | 53% |  |  |  |  |  |
| 286 |  | Pulping |  |  | 60% |  |  |  |  |  | 50% |  |  |  |  |
| 290 (one stage) |  | Shredder | 4.1 | 62% | 30% | 45% |  | 1, 3, 15% | 70% | 59% |  |  |  |  | 2.3 |
| 290 (two stages) |  |  |  | 63% | 34% | 65% |  |  |  |  |  |  |  |  | 4.6 |
| 320 |  | Cutter |  | 50% | 21% | 31% |  |  |  |  |  |  |  |  |  |
| 285 |  | Mixer |  | 60% |  | 66% |  |  | 73% |  |  |  |  |  |  |
| 311 (single press) | Industrial*** | No |  | 21-32% | 8-13% | 12-21% |  |  |  | 16-28% |  |  |  |  |  |
| 311 (twin press) |  |  |  | 51-59% | 20-27% | 28-43% |  |  |  | 42-51% |  |  |  |  |  |
|  | Lab *** |  |  | 55-67% | 22-35% | 21-52% |  |  |  | 47-86% |  |  |  |  |  |
| 321 | Industrial |  |  | 58% | 19% | 36% |  |  |  | 54% |  |  |  |  |  |
| 266 |  | Defibering |  | 64% | 31% | 42% |  |  |  |  |  |  |  |  |  |

Studies using secondary data were not included here. No differentiation per biomass type. *: proteins and nitrogen transfer coefficients are mostly reported equivalent. **: fibers are sometimes considered as a whole, sometimes separated in main components in this order: lignin / cellulose / hemicellulose. ***: meta-analysis

Table SI. 86 – Selected parameters and variation ranges for green juice extraction

| Parameters |  | Baseline | Min | Max | Comment |
| --- | --- | --- | --- | --- | --- |
| Juice separation (%juice/in) | fresh matter | Equation 14 65.6% - 50% x %DM |  |  | Empirical equation from meta-analysis311 of large scale twin screw-press*. Equation fixed. |
|  | dry matter | Modeled as the sum of individual fractions recovery (so output) |  |  | Should range between 10% (single press311) and 30-35% (twin press290,314,315). Defined by mass balance |
|  | proteins | 36% | 16% | 69% | Baseline representative of large scale twin screw-press performances310,311,314, and min as currently achieved by large scale single screw-press311. Max selected as higher reported value in reviewed literature281 (Table SI. 85). |
|  | fat | 22% | 18% | 59% | Baseline and min representative of current achievements310 while max selected as higher reported value in reviewed literature281 (Table SI. 85). |
|  | soluble carbs | 40% | 15% | 70% | Few data (Table SI. 85), large range considered |
|  | hemicellulose | 15% | 10% | 19% | Few data, not individually reported except in Corona et al., 2018290 (selected as baseline). Range adapted with values for total fibers (Table SI. 85). |
|  | cellulose | 3% | 1% | 19% |  |
|  | lignin |  |  |  |  |
|  | ashes | 46% | 22% | 59% | Baseline representative of large scale twin screw-press performances311,322, and min considered as currently achieved by large scale single screw-press310,311. Max selected as currently achieved on quality feedstocks290,321. |
|  | rest | Modeled as average of rest |  |  | Calculated. |
|  | N | Modeled as proteins (linked) |  |  | Neglecting potential differences for non-protein N. |
|  | P | 61% | 44% | 67% | Baseline as average of currently achieved yield310, ranges from the benchmark (Table SI. 85). |
|  | K | 64% | 57% | 78% |  |
|  | S | 48% | n.a. |  | Baseline as average of currently achieved yield310. Not a parameter for lack of data (Table SI. 85). |
|  | C | Proportionally adjusted to fit mass balance (so output) |  |  | No data. Calculated by mass balance |
|  | O |  |  |  |  |
|  | H |  |  |  |  |
| kWh.tww-1 | maceration | 4.1 | 2.8 | 8.3 | Range between current pilots290 and scaled estimations323. |
|  | pressing | 3.4 | 2.3 | 4.5 | Range between twin and single screw-press (Table SI. 85). |

*: seen valid for grass silage under 30%DM311.

#### Protein recovery

The goal of this stage is to recover as much feed-grade proteins as possible from the green juice. In this case, the preferred method is protein coagulation which can be thermal, acid or biological, followed by a decanter/centrifuge to recover the “green protein’ fraction. The co-product of this stage is the supernatant also referred to as “brown juice”. In this model, a heat treatment at 80°C was selected. Indeed, this treatment is recognized to maximize proteins recovery272,276,290 and is proven at industrial scale266 (which is not the case for e.g. lactic acid fermentation still under development320). As developed for the juice separation stage, a literature benchmark was established to capture the variation ranges of the key parameters of this unit operation (Table SI. 87).

Table SI. 87 – Benchmark of green protein recovery performances

| Reference | Data | Heat (MJ.tww-1) | Separation (kWh.tww-1) | Green protein recovery (%pellet/in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | fresh matter | dry matter | proteins* | fat | Fibers** | solubles | ashes |
| 272,281 | Lab scale | 210 | 1.2 |  | 28% | 45% | 100% | 63% |  | 15% |
| 314 |  |  |  | 18% | 42-46% | 58-61% |  |  |  |  |
| 295 |  | 126 | 0.8 |  | 18% | 51% |  | 28% |  | 13% |
| 276 |  |  |  |  | 50-58% | 68-84% |  |  |  |  |
| 290 | Pilot scale | 294 | 11 |  | 14% | 90% |  | 90% |  | 50% |

Only considering heat treatments and primary data. *: proteins and nitrogen transfer coefficients are mostly reported equivalent. **: fibers are sometimes considered as a whole, sometimes separated in main components in this order: lignin / cellulose / hemicellulose.

Fewer primary data is available for protein recovery compared to juice extraction (Table SI. 87), leading practitioners to develop their own calculation and estimation. We used a similar approach to estimate the parameters of this unit operation, the main assumptions are summarized in Table SI. 88. No losses were considered: green juice fractions either end in the green pellet or in the brown juice. Passive cooling was considered after heat treatment, as active cooling was only mentioned once in the reviewed literature272. Pumping of the brown juice towards its valorization pathway was accounted using (0.83 kWh.tww-1)290. The required 80°C heat was modeled as LT heat. Finally, green pellet recovery operations (skimming, centrifugation, etc. towards 20-30%DM)272 were generically accounted through an electricity consumption parameter (see Table SI. 88).

Table SI. 88 – Selected parameters and variation ranges for green proteins recovery

| Parameters |  | Baseline | Min | Max | Comment |
| --- | --- | --- | --- | --- | --- |
| Green proteins recovery (%pellet/in) | fresh matter | 10% | 5% | 15% | In-between data from acid fermentation (ca. 5%)320,324 and achieved during lab experiments291,314. |
|  | dry matter | Modeled as the sum of individual fractions recovery (so output) |  |  | Should range between 14%290 and 45%314. |
|  | proteins | 50% | 30% | 90% | Max value for quality grass289 but residual streams report lower yields (45-51%)281,295 chosen as baseline here. Min as currently achieved with fermentation315. |
|  | fat | Modeled as proteins (linked) |  |  | No data. |
|  | soluble carbs | 21% | 5% | 36% | defined as the ratios used in similar assessments290,295. |
|  | hemicellulose | Modeled as soluble carbs (linked) |  |  | Assuming hemicellulose present in juice is soluble. |
|  | cellulose | 28% | 5% | 90% | High variability reported (e.g. 90% lignin recovery290, 28% dietary fibers295): broad variation range. |
|  | lignin |  |  |  |  |
|  | ashes | 15% | 13% | 50% | In-between data from lab experiences281,295 and pilot290 |
|  | rest | Modeled as average of other coef. |  |  | Calculated. |
|  | N | Modeled as proteins (linked) |  |  | Neglecting potential differences for non-protein N. |
|  | P | 38% | n.a. |  | Estimated from literature280. Lack of data so not defined as parameters. |
|  | K | 21% |  |  |  |
|  | S | Modeled as rest (linked) |  |  | No data. |
|  | C | Proportionally adjusted to fit mass balance (so output) |  |  | No data. Calculated by mass balance |
|  | O |  |  |  |  |
|  | H |  |  |  |  |
| kWh.tww-1 | Separation | 5 | 1.4 | 11.1 | Reviewed range (Table SI. 87), base as pilot290 |
| MJ.tww-1 | Heat | 270 | 252 | 294 | Range between two pilot scales (80°C)290,317. Baseline calculated with Equation 3 (input at 20°C) *. |

*: input juice is assumed to be at higher temperature than average of France (13°C) because presence of heat exchanger for prewarming/cooling.

#### Post-treatments

Resulting green pellet was assumed dried towards 91%DM to supply feed markets266. Albeit direct fresh feeding is also contemplated325,326, this assumption was taken for consistency between all modeled waste-to-nutrition pathways. Drying was modeled as spray drying and evaporated water was assumed lost to the air, as for BSF and MP. This is because the benchmark of green pellet’ spray drying efficiency provided similar results (ranging 3-7 MJ.kgwater-1)272,275,290 than spray drying efficiency for the other ingredients. The use of the dried pellet in compound feed was then modeled as for the other ingredients (using the CF model after the drying stage). A notable difference with the other novel feed ingredients is that the final pellet composition is here an output of the model (not fixed, depend on the parameters). The dried pellet’s model outputs for the two case studies (main manuscript), calculated with baseline values, are compared against benchmarked values in Table SI. 89. The crude fibers content was conservatively estimated to be the sum of lignin, hemicellulose and cellulose, and not a parameter itself (to only keep independent parameters). Many feeding experiments have been performed on monogastric animals both terrestrial and aquatic274,327,328 displaying a wide range of performances. Therefore, pellet’s digestibility was also kept as an independent parameter (Table SI. 92).

Table SI. 89 – Calculated green pellet composition and GB efficiency against benchmark

| Reference | Data | Input | Green pellet composition (%DM) |  |  |  |  |  |  |  |  |  |  | %input |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dry matter | proteins | fat | lignin | cellulose | hemicellulose | solubles | ashes | rest | P | K | Protein recovery |
| Calculated |  | SBT | 92% | 38% | 4% | 1% | 1% | 6% | 29% | 17% | 4% | 0.6% | 9% | 18% |
|  |  | CMG | 92% | 30% | 2% | 2% | 4% | 13% | 22% | 16% | 12% | 1.5% | 2% | 18% |
| 272,281 | Lab | SBT |  | 29% | 8-12% | 23-29% |  |  | 14-19% | 17-20% |  |  |  | 31% |
| 327 | Lab | SBT |  | 44% | 5% | 44% |  |  |  | 6% |  |  |  |  |
| 329 | Lab | Diverse |  |  |  |  |  |  |  |  |  |  |  | 34-65% |
| 275 | Lab | Broccoli leaves |  |  |  |  |  |  |  |  |  |  |  | 32-35% |
| 275 | Lab | Kale leaves |  |  |  |  |  |  |  |  |  |  |  | 19-23% |
| 279 | Lab | Grass |  |  |  |  |  |  |  |  |  |  |  | 16-19% |
| 276 | Lab | Cassava tops | 94% | 42% |  |  |  |  |  |  |  |  |  | 21-26% |
| 330 | Pilot | Grass-clover | 97% | 46% | 10% |  | 3% |  | 2% | 12% |  |  |  |  |
| 312 | Pilot | Grass-clover |  | 37% | 5% | 10% |  |  | 13% | 29% |  |  |  |  |
| 289,290 | Pilot | Legumes |  | 46-55% |  | 0-1% | 3-4% | 7-9% | 13-17% | 12-16% |  | 0-0.4% | 1% |  |
| 266 | Indus | Alfalfa | 92% | 50% | 10% | 1% | 2% | 11% |  | 11% |  |  |  |  |

Indicative only, as calculated with the baseline values of (relatively high ranging) technology parameters. SBT: Sugarbeet tops, CMG: countryside management grass

Albeit green pellet’s final composition is tightly linked with input’s composition, the model estimates an output in accordance with values reported in the literature (Table SI. 89). In the baseline configuration, the overall protein recovery (i.e. share of input proteins recovered in the green pellet) lies in the lower range of reported values. Rests are not necessarily lost, as co-products can also be used for nutritional purposes (see next section). The full range of achievable performances are assessed in the sensitivity analysis.

#### Co-products management

The two main co-products of the modeled green biorefinery pathways are (i) the press pulp and (ii) the brown juice. Consisting of ca. 90-95% (mass basis) of the input feedstock (see Table SI. 91), their further valorization is key to the economic and environmental viability of the biorefinery272,290,296,300, and many options have been considered (Table SI. 90).

Table SI. 90 – Benchmarked valorization options for green biorefinery co-products*

| Green biorefinery co-product | Valorization pathway | Reference |
| --- | --- | --- |
| Press-pulp | Returned to field as fertilizer, mulch, etc. | 290 |
|  | Use as biomaterial (e.g. insulation fibers) | 290 |
|  | Anaerobic digestion for biogas and digestate production | 331 |
|  | Formulation of energy pellets for combustion | 288 |
|  | Hydrolysis and fermentation for bioethanol production | 290 |
|  | Biocrude production | 332 |
|  | Hydrolysis and fermentation for protein production | 130 |
|  | Formulation of ruminant feed dried pellets | 290 |
|  | Direct use as ruminant roughage (after e.g. ensiling) | 275,278,295,314 |
| Brown juice | Anaerobic digestion for biogas and digestate production | 290,295,331 |
|  | Direct use as fertilizer (restricted to pedoclimatic conditions) |  |
|  | Hydrothermal liquefaction | 332 |
|  | Use as fermentation medium for amino acids production | 333 |
|  | Use as fermentation medium for microbial oil or proteins | 334 |
|  | Extraction of fructioligosaccharides | 331 |
| Both press-pulp & brown juice | Mixed substrate for anaerobic digestion (only brown juice too liquid and unbalanced) | 285,315,331 |

*: non-exhaustive benchmark

First, in front of the high variability of brown juice and lack of published data on value-added extraction or fermentation performed on this substrate, only its energy valorization was considered (AD_grid pathway). Therefore, selected GB pathways only differ on how press-pulp is used. Three pathways were selected considering the ones which (i) were already highlighted as potentially environmentally beneficial302 and (ii) whose parametric LCI models were already available. The first one consists in reusing the co-products (both press-pulp and brown juice) to supply an in-situ AD platform (therefore feeding the AD_grid model described in Javourez et al., 20241). The second one rather reuses the press-pulp as ruminant forage, following the compound feed (CF) pathway described in section SI. 4.2. Finally, the last one contemplates food-grade mycoprotein productions from press-pulp through enzymatic hydrolysis (Mycofood) as modeled in section SI. 4.5.

#### Green biorefinery mass balances summary

To facilitate the analysis of the LCI results and future comparisons, the Table SI. 91 displays the mass balances of the main fractions when going through green biorefining as calculated with this model and under the baseline technology parameters values. In this model, the amount of proteins in final pellet is linearly linked with the protein content of the input streams through two transfer coefficients: from the input to the green juice, and from the green juice to the green pellet. Most of the input dry matter ends up in the presspulp, while most of the water ends in brown juice. Values in Table SI. 91 are indicative only, as these are direct outputs of the model, calculated with baseline values.

Table SI. 91 – GB mass balances and distribution in final co-products

| Case study | Sugarbeet tops |  |  |  |  | Countryside grass |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage | Input | Green juice | Green pellet | Presspulp | Brown juice | Input | Green juice | Green pellet | Presspulp | Brown juice |
| fresh matter | 1000 | 560 | 56 | 440 | 504 | 1000 | 442 | 44 | 558 | 398 |
| water | 808 | 505 | 42 | 303 | 463 | 572 | 354 | 22.5 | 217 | 332.5 |
| dry matter | 192 | 55 | 14 | 137 | 41 | 428 | 88 | 22 | 341 | 65 |
| proteins | 30 | 11 | 5.5 | 19 | 5.5 | 36 | 13 | 6.5 | 23 | 6.5 |
| fat | 5 | 1 | 0.5 | 4 | 0.5 | 4 | 1 | 0.4 | 3 | 0.4 |
| soluble carbs | 49 | 20 | 4 | 30 | 15.6 | 56 | 22 | 1.7 | 33 | 17.6 |
| hemicellulose | 28 | 4 | 1 | 23 | 3.3 | 87 | 13 | 2.7 | 74 | 10.3 |
| cellulose | 25 | 1 | 0.2 | 24 | 0.5 | 116 | 3.5 | 1.0 | 112 | 2.5 |
| lignin | 12 | 0.5 | 0.1 | 12 | 0.3 | 40 | 1.2 | 0.3 | 39 | 0.9 |
| ashes | 34 | 16 | 2.4 | 18 | 13.3 | 50 | 23 | 3.5 | 27 | 19.8 |
| rest | 9 | 2.5 | 0.6 | 7 | 1.9 | 39 | 10.5 | 2.6 | 28 | 7.9 |
| N | 4.5 | 1.5 | 0.8 | 2.9 | 0.8 | 8.7 | 3.1 | 1.5 | 5.6 | 1.5 |
| P | 0.4 | 0.2 | 0.1 | 0.1 | 0.1 | 1.4 | 0.8 | 0.3 | 0.5 | 0.5 |
| K | 9 | 6 | 1.2 | 3.3 | 4.6 | 3 | 2 | 0.4 | 1.1 | 1.5 |
| C | 78 | 21 | 5.3 | 57.5 | 15.4 | 220 | 43 | 10.2 | 177 | 32.8 |

Potential unmatching balances due to rounding. In grey, the composition of the three co-products. Green pellet is further dried, brown juice sent to AD, and presspulp valorization depend on selected GB pathway.

### Technology parameters and background processes

The technology parameters retained for green biorefinery pathways modeling are displayed in the Table SI. 92. The ones linked with presspulp and brown-juice valorization are displayed in respective model’s description.

Table SI. 92 – Parameters specific to green biorefinery pathways

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 273 | Washing water | m3.tww-1 | 33 | 12-87 | See Table SI. 84 |
| 274 | Irrigation replacement | % | 50% | 0-100% |  |
| 275 | Maceration electricity | kWh.tww-1 | 4.1 | 2.8-8.3 | See Table SI. 86 |
| 276 | Screw-press electricity | kWh.tww-1 | 3.4 | 2.3-4.5 |  |
| 277 | Protein recovery in green juice | %proteins | 36% | 16-69% |  |
| 278 | Lipids recovery in green juice | %lipids | 22% | 18-59% |  |
| 279 | Soluble carbs recovery in green juice | %solubles | 40% | 15-70% |  |
| 280 | Hemicellulose recovery in green juice | %hemicel. | 15% | 10-19% |  |
| 281 | Ashes recovery in green juice | %ashes | 46% | 22-59% |  |
| 282 | Fibers recovery in green juice | %fibers | 3% | 1-19% |  |
| 283 | P recovery in green juice | %P | 61% | 44-67% |  |
| 284 | K recovery in green juice | %K | 64% | 57-78% |  |
| 285 | Heat for protein coaggulation | MJ.tww-1 | 270 | 252-294 | See Table SI. 88 |
| 286 | Protein separation electricity | kWh.tww-1 | 5 | 1.4-11.1 |  |
| 287 | Protein recovery in green pellet | %proteins | 50% | 30-90% |  |
| 288 | Ashes recovery in green pellet | %ashes | 15% | 13-50% |  |
| 289 | Soluble carbs recovery in green pellet | %solubles | 21% | 5-36% |  |
| 290 | Fibers recovery in green pellet | %fibers | 28% | 5-90% |  |
| 291 | Water recovery in green pellet | %water | 10% | 5-15% |  |
| 292 | Green pellet digestibility | %OM | 75% | 50-85% | Reviewed range274 |
| 293 | Presspulp digestibility | %OM | 65% | 60-70% | Average reported (ruminants)269 |
| Spray drying as in Table SI. 81. |  |  |  |  |  |
| Other common parameters in section SI. 4.1. |  |  |  |  |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to MP production stage but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15.

The background processes used specifically for this LCI model are gathered in Table SI. 93.

Table SI. 93 – Background processes used specifically for GB pathways

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Forwarding, forwarder, RER | Handling / Washing | GB input feeding |
| Market for tap water, EU |  | Proxy of washing water, electricity harmonized |
| Market for irrigation, FR |  | Proxy of avoided process by reusing washing water |
| Chemical factory construction, RER | n.a. | Proxy of the GB facility, considering chemical output as the dried protein pellet (3.72 g.kgpellet-1)48. |
| Background processes for brown juice handling (AD) in Javourez et al., 20241. |  |  |
| Background processes for presspulp handling (GB_rough) in Table SI. 25 |  |  |
| Background processes for presspulp handling (GB_mycofood) in Table SI. 47 and Table SI. 54 |  |  |

All processes are from the ecoinvent 3.8 consequential database. EU is used to refer to “Europe without Switzerland”. Direct flows to biosphere are displayed in B.GB_module spreadsheet.

Background processes and technology parameters associated with transversal unit operations between many models (e.g. pretreatments, transportation, etc.) are displayed in the common section SI. 4.1. Rest of modeling details (e.g. all biosphere flows) are available directly within corresponding excel model.

The refinement leverages of the model include, but are not limited to: increasing the modeling granularity of inputs’ composition (e.g. including amino acids, phenols, etc.), integrating the influence of modifying agricultural management practices (e.g. harvesting periods) as well as breeds (e.g. through genotype selection) on the composition of residual streams and their subsequent proteins extractability and quality 278. Importantly, only three out of the many options to valorize press-pulp were analyzed (and only one for the brown juice fraction), meaning the whole range of achievable performances by GB set-ups was not captured herein. Finally, as designed, the GB LCI model is only suitable to assess GB as an alternative management practice for green residues. Indeed, GB is also part of the basket of strategies aiming to repurpose available land in a context of developing grasslands-based bioeconomies 335, and the present model needs to be complemented (e.g. refined technological parameter ranges, spatio-temporal feedstock availability, land use change and agronomic practices accounting, etc.) to address such land allocation problematic.

## Gasification LCI model

This section presents the details (including assumptions, choice rationales, calculations and data sources) of the gasification (GA) life cycle inventory (LCI) model, with the objective to perform the LCA of GA not only as part of a standalone bioresource management option, but also integrated in more complex supply chains as a pretreatment (here for MP production; see section SI. 4.6). Corresponding model is available in B.GA_module.

### Set-up selection

The objective of this model is to provide a feedstock- and process-dependent estimation of gasification’s life cycle inventory. Being of lower TRL than anaerobic digestion, scaled primary data are (i) scarce and (ii) not representative of continuous industrial production. Therefore, the model was rather based on lab- and pilot-based experiments which were used in previous assessment studies (mainly prospective LCAs and reviews; see background data in B.GA_module for the full list of references).

Gasification is a scaling technology which displays many set-up variants (process, temperature, oxidizing agent, etc.). An exhaustive review of gasification techniques is out of the scope of this study. Yet, to provide realistic estimations of gasification performances, one of the most commonly studied set-ups was modeled: air as the oxidizing agent fed in a fluidized bed reactor. Albeit yielding higher hydrogen production, the use of steam or pure oxygen as oxidizing agents were not considered as associated with higher energy costs223, as well as lower data availability. Therefore, the following steps were considered: in-situ collection and transport towards the gasification plant, storage and pretreatments, gasification, producer gas cleaning, gas post-treatment steps, and final energy valorization. GA platform was assumed as a centralized facility (regional-scale hub). The complexity and energetic intensity of the processes involved (e.g. thermochemical unit operations of syngas upgrading) likely require an economy of scale to be viable240,247. A high plant availability was assumed (8000 h.y-1) neglecting start-up effects (ca. 60-80 h.y-1 336). Lifetime of the plant was considered 30 years, with a 20 tww.h-1 treatment capacity. Process integration was proxied by estimating the potential heat synergies and sensible heat recoveries for each unit operation. The Figure SI. 3 illustrates the system boundary as well as the main process flow and unit operations contemplated for the selected GA set-up (harmonized with the figures of the main manuscript).

Figure SI. 3 – Scope, system boundaries and process flow diagram of GA

“Suitable” feedstock categories and corresponding representative case studies are based on the (pathways; feedstocks) compatibility matrix (see main manuscript). Here, gasification (GA) is modeled as a pretreatment of microbial proteins (MP) production, corresponding gas post-treatments are detailed in the section SI. 4.6.2.

### Unit operation design and assumptions

At gate, diffuse streams were considered managed and transported to a collection platform before their transportation to the centralized GA facility, while the other streams (already collected) are directly sent to the centralized GA facility (see Table SI. 94).

Table SI. 94 – GA pretreatment requirements per residual biomass category

| Category | Handling from gate | Storage | Grinding/Drying |
| --- | --- | --- | --- |
| Primary forestry residues | Pre-chipped and collected towards collection platform | Piling up | All |
| Pruning residues | None if already collected |  |  |
| Wood-processing waste | None |  |  |
| Primary crop residues | Collected towards collection platform | Ensiling |  |
| Garden & park waste | None if already collected |  |  |
| Vegetable tops | Collected towards collection platform |  |  |
| Food waste | None | Covered pit |  |
| Slaughterhouse by-products | None |  |  |
| Comments | See Javourez et al., 20241 and Table SI. 102 for modeling details. |  |  |

#### Storage

Three storage strategies were considered depending on the input’s composition-structure (Table SI. 94). All were designed to handle 1 month of operations. First, wood-related residual biomasses were assumed piled up, and modeled as proposed by Brassard et al., 2020167. Emissions were calculated by mass balance, but neglecting water and N emissions because these are low for this residual biomass category. The parameters used are displayed in Table SI. 95. Second, primary crop residues and green residual biomasses were assumed ensiled. LCI modeling was calculated based on Kreuger et al., 2011337 as already detailed for AD storage in Javourez et al., 20241. The other streams were directed towards a covered pit, which was modeled as digestate storage with adapted emissions factors (see Table SI. 95).

Table SI. 95 – GA storage modeling

| Parameter | Unit | Value | Storage | Comment |
| --- | --- | --- | --- | --- |
| Biomass density | kg.m-3 | 222 | Piling up | Assumed as wood chips for all wood-related streams. |
| Surface required | m3.m-2 | 3 |  | 45°C angle piles167 |
| Mass loss | %OM | 2% |  | As in Brassard et al., 2020167 (CO2 emissions calculated by difference) |
| CH4 production capacity | Nm3CH4.tVS-1 | 350 |  |  |
| CH4 conversion factor | %capacity | 2% |  |  |
| CH4 production capacity | Nm3CH4.tVS-1 | 320 | Covered pit | From Javourez et al., 20241, proxy for food-waste-like streams |
| NOx EF | %TAN | 1% |  | Value for solid manure10 |
| Volatilized (FracGASM) | kg (NH3-N+NOx-N) . kgN-1 | 22% |  | Value for solid manure338 |
| Rest of parameters |  |  |  | All corresponding to digestate storage in Javourez et al., 20241 |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants.

#### Standardization

All feedstocks were considered standardized in size (ca. 30mm223) and moisture (at least 90%DM339) as usually required for fluidized bed reactors. Grinding generates losses (dust) which were assumed recovered in-situ in an industrial furnace (as Heat_ind pathway in Javourez et al., 20241). Moisture standardization was developed through a large rotary dryer340, the feedstock was further fed to gasification’s lock hopper system. Parameters used are displayed in Table SI. 96.

Table SI. 96 – GA standardization modeling

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Grinding losses | %ww | 1% | Assumed as proxy of dust generation167 |
| Target moisture | %ww | 10% | To calculate amount of water to remove |
| Rotary dryer electricity | kWh.tDM-1 | 50 | Calculated considering 105°C target340 |
| Rotary dryer heat | MJ.kgwater-1 | 2.9 |  |
| GA feeding | kWh.tDM-1 | 5.9 | Review of lock hopper systems160 |
| Nitrogen requirements | Nm3.tDM-1 | 76 |  |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants. Drying is only applied in ex-storage feedstock displays a moisture content higher than the target.

#### Gasification stage

This study focuses on thermochemical processes targeting electricity and heat energy services as well as hydrogen production for microbial proteins production. Accordingly, the gasification stage was modeled to allow as much hydrogen production as possible, favored with high reactions temperatures (>800°C) and reduced air excess223.

Initiatives involving kinetic and thermochemical modeling to estimate syngas composition from a given input feedstock are available in the literature (e.g. Gagliano et al., 2016341; Lodato et al., 20226). Yet, forecasted GA performances are reported as highly sensitive to processing parameters (particularly the equivalence ratio; ER) and does not necessarily always fit experimental data. In an endeavor to apply the same modeling granularity to all the models, the GA performance estimation approach developed in this study mimicked the one applied to AD. GA performances were derived by selecting, for each feedstock, an ER enabling to fit mass and energy balances as well as GA performance indicators reported in the literature (Table SI. 98). This was done by fixing the composition of producer gas to the average reviewed value for air-based gasifiers (Table SI. 97).

The following simplifications were adopted: (i) biomass-bounded nitrogen only yields N2, (ii) air supplied is dry and only includes O2 and N2, (iii) O2 is fully consumed, (iv) trace gases, contaminants and other volatile compounds are considered within the gaschar fraction or fully removed during gas cleaning operation (but not accounted in balances), (v) gaschar hosts 100% of ashes and unconverted organic compounds, (vi) 900°C gasification is considered autothermal (i.e. no external heat added to the system223). The assumption regarding nitrogen (N) conversion differs significantly from Matassa et al., 2020194. In their study, they considered that 60-90% of N was converted into NH3, providing a direct bio-based N resource for MP production. Yet, as reported by Hongrapipat et al., 2012342, NH3 production during gasification is (i) undesirable for its effects on catalysts and post-treatment operations, (ii) low compared to N2 production at high temperature (900°C here), (iii) bounding hydrogen to ammonia, therefore lowering overall H2 yield. As feedstocks usually considered for gasification display low N content (e.g. wood-related biomass, crop residues), NH3 remains at trace concentrations (ca. mg/L) and is almost not reported in the literature benchmark (B.GA_module spreadsheet), being removed at the cleaning stage. For those reasons, and to ensure feed-grade (risk-free) nutrients supply to the MP bioreactor (section SI. 4.6), a set-up minimizing and removing NH3 from syngas was assumed.

Table SI. 97 – Reviewed performances of air-based GA performances

| Fractions in syngas (%vol) | Selected composition (average reviewed) | Performance indicator | Unit | Reviewed range |
| --- | --- | --- | --- | --- |
| H2 | 12% | Equivalence ratio (ER) | molO2.molO2-1 | 0.25-0.4 |
| CO | 19% | Producer gas yield | Nm3.kgww-1 | 1.7-2.7 |
| CO2 | 13% | Converted carbon | %C | 80-98% |
| CH4 | 3% | Cold gas efficiency (CGE) | %HHV | 60-85% |
| H2O | 5%* | Hot gas efficiency (HGE) | %HHV | 83-95% |
| N2 | 48% | Gaschar yield | kg.kgDM-1 | 0.05-0.2 |
| CnHm, others | Neglected** | Background data in B.GA_module spreadsheet |  |  |

*: deduced by different to yield 100%
**: producer gas contains trace gases (e.g. H2S, HCl, light hydrocarbons, etc.), tar and dust. Set-up and catalyst were assumed selected to minimize tar and CnHm generation, usually corresponding to 1-5%vol of wet gas. Other contaminants were assumed removed during the producer gas cleaning stage, hence not displayed here.

Table SI. 98 shows that GA performances were modeled to represent higher range of reviewed experimental performances, yet in the limit achievable by the initial elemental composition of input streams. For example, unfavorable composition of countryside management grass and PAT (C:O ratio of ca. 2) would normally yield a producer gas of a lower hydrogen content. As this model considers a fixed syngas composition, this lower hydrogen content is achieved by an overall lower GA performance, respecting mass and energy balances.  In practice, input feedstocks would be monitored and mixed to provide homogeneous and suitable characteristics for GA, the present model only estimates the effect of a target feedstock of the whole substrate on GA outputs. Moreover, a direct consequence of choosing air as oxidizing agent is the dilution of producer gas with inert nitrogen (hence lowering the volumetric energy value and hydrogen share of producer gas).

Table SI. 98 – GA performances calculated on selected case studies

| Case studies | Fixed ER | Producer gas yield | Converted carbon | CGE | HGE | Gaschar yield |
| --- | --- | --- | --- | --- | --- | --- |
|  | molO2.molO2-1 | Nm3.kgww-1 | %C | %HHV | %HHV | kg.kgDM-1 |
| Hardwood residues | 0.32 | 2.39 | 93% | 70% | 81% | 0.16 |
| Sawmill waste | 0.32 | 2.28 | 93% | 69% | 81% | 0.21 |
| Wheat straw | 0.32 | 2.36 | 95% | 73% | 86% | 0.17 |
| Countryside management grass | 0.26 | 2.37 | 88% | 80% | 94% | 0.15 |
| Sugarbeet tops | 0.31 | 2.12 | 98% | 74% | 87% | 0.24 |
| Household food waste | 0.29 | 2.48 | 93% | 75% | 87% | 0.09 |
| PAT | 0.22 | 2.35 | 80% | 68% | 80% | 0.07 |
| Comments | ER was fixed in order to achieve GA performances within the higher range of reviewed performances (Table SI. 97). |  |  |  |  |  |

To ensure mass and energy balance of GA through simulations, ER was not considered as a technology parameter for the sensitivity analysis. Hot gas efficiency (HGE) represents the overall energy conversion efficiency of the GA process. Distance to 100% represents the energy lost due to autothermal conditions and gaschar enthalpy. Cold gas efficiency (CGE) represents the energy conversion efficiency not considering the sensible heat present in the 900°C producer gas at reactor’s gate.

Utilities, facilities and emissions were mostly estimated by adapting existing ecoinvent background processes and studies by Ardolino and Arena, 2019339 and Thunman et al., 2018336 (see Table SI. 102). Only an extra electricity need linked to air pumping to the reactor was added, calculated with Equation 1 by considering 1.5 bar overpressure.

#### Gaschar management

The literature reports many gaschar valorization pathways, including (i) return to agricultural soils, (ii) return to forest soils, (iii) reuse in the construction sector and (iv) reuse in industrial processes343. Moreover, for input streams labelled as waste, gaschar might also be disposed in hazardous landfill340. Energetic valorization of gaschar is also mentioned10, but was discarded as the present study already considers high gasification performances (Table SI. 98). are considered, providing a gaschar with neglectable energy value. To simplify and model a “best case” management practice, only two gaschar management practices were retained (Table SI. 99). For inputs under the EU Waste regulation, gaschar was assumed reused as a construction material (e.g. road aggregate). Else, gaschar was assumed to comply with standards to be returned to agricultural soils. Gaschar storage was not accounted (high stability, assumed no losses). Transportation towards selected management was modeled as in Table SI. 15.

Table SI. 99 – Gaschar management modeling

| Management per stream category |  | Modeling parameters for “return to land” gaschar management |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Residual biomass sub-categories | Management | Parameter | Unit | Value | Comment |
| Primary forestry residues | Return to land | Water addition | kg. kggaschar-1 | 1.82 | Easier handling and land application344 |
| Pruning residues | Return to land | Gaschar application | Technology parameter, see Table SI. 101 and Table SI. 102 |  |  |
| Wood-processing waste | As aggregate* | French maize yield | tww.ha-1 | 8.92 | Average54 |
| Primary crop residues | Return to land | French wheat yield | tww.ha-1 | 7.00 |  |
| Garden & park waste | As aggregate | French N-fertilizer application | kg.ha-1 | 120 |  |
| Vegetable tops | Return to land | Gaschar decrease in N2O emissions | Technology parameters, see Table SI. 101 |  |  |
| Food waste | As aggregate | Gaschar increase in maize yield |  |  |  |
| Slaughterhouse by-products | As aggregate | Gaschar increase in wheat yield |  |  |  |
| Here only indicative for the sub-category, management is defined by the status of each specific stream. |  | Gaschar C loss, 100 years |  |  |  |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants (else, mentioned). Calculation details in B.GA_module spreadsheet. *: here sawmill waste was considered returned to land.

Literature reports cumulative effects of gaschar when applied on agricultural lands. It includes, but is not limited to: (i) carbon sequestration, (ii) fertilizer use reduction344, (iii) soil N2O emissions reduction, (iv) increased crop yields, (v) soil organic carbon (SOC) change, (vi) soil CH4 emissions change, (vii) increased water retention, (viii) reduced limestone use, (ix) increased phosphorous availability, (x) modification of soil texture and pH (i-viii in review by Azzi et al., 2021343; i,v,vii,ix,x in meta-analysis by Joseph et al., 2021345). Yet, some effects are highly uncertain (e.g. SOC change ranging ± 20%345) or not significant nor generalized (e.g. reduction in CH4 emissions346). Therefore, we assumed to only account for carbon sequestration, yield increase and N2O emissions reduction, as these display more robust and systematic data. Gaschar was assumed mixed with water (easier handling and land application344 and spread with hydraulic loader167 (see Table SI. 102). The meta-review of Andrade et al., 2022347 was used to derive 100 years horizon carbon sequestration of gaschar when applied to agricultural land (Table SI. 101). N2O emissions reduction (from current situation) was based on He et al., 2017346. Current N2O agricultural emissions were estimated with IPCC tier 1 method348. Finally, a rather optimistic assumption was taken regarding gaschar’s induced increase in crop yields for France, as indicators were derived from the meta-analysis by Farhangi Abriz et al., 2021349 with a scope including countries with a higher yield gap. Wheat and maize represent the higher share of agricultural surface in France350. Therefore, the additional crop production per unit of applied gaschar was estimated based on (i) average gaschar application per hectare, (ii) average France’s wheat and maize yields and crop-specific yield increase factors345,349 (see Table SI. 101). Additional production was considered replacing energy feed services as modeled in section SI. 4.1.9. All the other effects were neglected (e.g. heavy metals leaching, because streams returned to land considered as risk-free).

Regarding its use as a construction aggregate, gaschar was considered to replace gravel in a 1:1 proportion by weight, and to induce the same carbon sequestration than for land application. End-of-life processes were neglected, but heavy metals leakage to soil were accounted as in Dong et al., 2018340, assuming for all waste streams the same metals content.

#### Syngas cleaning

Producer gas flows out at a 900°C temperature, but warm gas cleaning systems are still under development223. Common detoxification systems mainly requires temperatures of 500°C or lower to be efficient (e.g. activated carbon, sedimentation of ashes336,340. Yet, syngas post-treatments operations such as steam reforming also require high temperatures, and it is unlikely that sequences of cooling and heating of the producer gas yields energetically viable pathways. To avoid modeling complications of cleaning/cooling sequences depending on the chosen post-treatment (e.g. for MP production), and acknowledging that producer gas must be cleaned prior any post-treatment (e.g. risk of catalysis inactivation, among others), it was assumed that all gas cleaning operations were developed directly on hot gas. These consist in cyclone or bag filtration, tar removal and detoxification through activated carbon and rapeseed methyl ester (RME; see Table SI. 100). Finally, similarly accounted in AD pathways, a share of producer gas was assumed lost at this stage as fugitive direct emissions to the atmosphere (see Table SI. 101).

Table SI. 100 – Syngas cleaning modeling parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RME use | kg.tDMin-1 | 11 | Gas condenser and scrubber agent336 |
| Limestone use | kg.twwin-1 | 70.2 | precoat material for the filters and biomass ash336,340 |
| Activated carbon | kg.tDMin-1 | 0.435 | Removal of impurities336,339 |
| Cleaning waste | g.Nm-3 | 228 | Proxy of all solid waste generated at cleaning48 |

The parameters present in this table were not retained in the sensitivity analysis, these were considered as constants.

#### Syngas post-treatments

The syngas post-treatments to meet the specificiations of MP production were detailed in the MP LCI model documentation (section SI. 4.6). Cooling of syngas is required to meet temperature entry conditions of subsequent post-treatments. The influence of gas compression was neglected (assumed as ideal gases), and final cooled gas was assumed dry. Condensed water was sent to a wastewater treatment plant (WWTP; here considered unpolluted). N2 present in the producer gas was assumed inert for all the reactions involved in syngas post-treatments, and not influencing their performances (but accounting in the overall mass and volume to process).

### Technology parameters and background processes

The technology parameters retained for GA LCI modeling are displayed in the Table SI. 101.

Table SI. 101 – Parameters specific to GA pathways

| n° | Parameter | Unit | Baseline value | Variation range | Comment |
| --- | --- | --- | --- | --- | --- |
| 180 | Dryer thermal efficiency | %MJ | 90% | 85-95% | Value for large rotary dryers340 |
| 181 | Syngas CHP, power efficiency | %LHV | 35% | 30-40% | Not used in this study |
| 182 | Syngas CHP, heat efficiency | %LHV | 45% | 40-50% |  |
| 166 | Syngas CHP, unburned fraction | %syngas | 1% | 0.5-2% | Assumed as for biogas, not used here. |
| 183 | Gasification stage electricity | kWh.tDM-1 | 180 | 100-650 | 100-350 reviewed240,340, 320 reported by pilot-scale336, EU REF2046 proposes 9%autoconso. Current French project mentions 650 (discussion with experts) |
| 184 | Producer gas cleaning losses | %out | 1% | 0-1% | Assumed |
| 185 | Gaschar increase in maize yield | %yield | 28% | 0-28% | Worlwide average349, so here considered as higher value as France already displays high yields. Other crop-wise worldwide review found 10-42%345. |
| 186 | Gaschar increase in wheat yield | %yield | 14% | 0-14% |  |
| 187 | Gaschar application | tww.ha-1 | 10 | 1-20 | Meta-review, no notable effects after 10349 |
| 188 | Gaschar decrease in N2O emissions | %N2O | 30% | 0-30% | Meta-review (all pedoclimatic conditions)346 |
| 189 | Gaschar C loss, 100 years | %C | 50% | 3-96% | Meta review (average, min, max)347 |
| Other common parameters in section SI. 4.1. |  |  |  |  |  |

Numbering of parameters corresponds to the one of Table SI. 103. Parameters specific to GA model but not mentioned in this table were assumed as constants. Transportation requirements in Table SI. 15.

The background processes used specifically for this LCI model are gathered in Table SI. 102.

Table SI. 102 – Background processes used specifically for GA pathways

| Background process | Unit operation | Comment |
| --- | --- | --- |
| Building construction, hall, CH | Storage | Proxy for all feedstock storage, see Table SI. 95. |
| Wood chipping, industrial residual wood, stationary electric chipper, RER | Grinding | Proxy for lignocellulosic streams grinding (background electricity harmonized) |
| Market for nitrogen, liquid, RER | Grinding | Assumed European market |
| Synthetic gas factory production, CH | n.a. | Used to derive the facilities requirements, direct emissions and utilities (at raw syngas gate, and removing electricity), expressed per Nm3 |
| Market for calcium carbonate, precipitated, RER | Gasification | Proxy for calcine required in pilot operations (17.7 kg.tDM-1)336,339 |
| Market for potassium carbonate, GLO |  | Proxy for potassium carbonate (0.8 L.tDM-1), required in pilot operations336,339 |
| Market for basalt, GLO |  | Proxy for olivine required in pilot operations (10.5 kg.tDM-1)336,339 |
| Market for fatty acid methyl ester, RoW | Syngas cleaning | Proxies for the four processes mentioned in Table SI. 100. |
| Market for limestone, crushed, washed, CH |  |  |
| Market for activated carbon, granular, GLO |  |  |
| Market for inert waste, EU |  |  |
| Market for tap water, CH | Gaschar management | Easier handling for land application. |
| Solid manure loading and spreading, by hydraulic loader, CH |  | Proxy of moisturized gaschar land application |
| Gravel production, crushed, CH |  | Used to model avoided construction aggregates |
| Treatment of wastewater, unpolluted, CH | When required | To treat cooling condensed water (after cleaning operations). Quantity from mass balance. |
| Market for water, deionized, CH |  | To produce steam |

All processes are from the ecoinvent 3.8 consequential database. EU is used instead of the full name ecoinvent label “Europe without Switzerland”. Additional pipeline network was neglected. Direct flows to biosphere are displayed in B.GA_module spreadsheet.

Background processes and technology parameters associated with transversal unit operations between many models (e.g. pretreatments, transportation, etc.) are displayed in the common section SI. 4.1. Rest of modeling details (e.g. all biosphere flows) are available directly within corresponding excel model.

Shortcomings and improvements potential of the LCI model lie in different areas. First, the gasification performances estimation is still based on too few primary data, mostly aggregated, meaning that the proposed LCI is not generic but sensitive to technological choices (for example, the specific catalyst selected for gasification and water shifting reactions). The aggregation of GA documentation also induced many simplifications and inhomogeneous modeling (details in section SI.4.8), which lead to a lower modeling granularity for their technological parameters compared to the ones of other pathways. For instance, the generic parameter “post-treatment GA power requirements” englobes several unit operations. A next version of the GA LCI model should build on process-based analysis351. Yet, the main GA performance indicators estimated were found in line with reviewed literature, and the identified inconsistencies limited to streams with specific elemental composition. Among others improvement leverages, estimates of gasification’s fugitive emissions should be refined. These were found hardly ever accounted for in reviewed LCAs, due to the lack of data. Yet, this omission induces unfair comparison with anaerobic digestion, whose leakage are already documented. Moreover, the effects and potential of gaschar should be better quantified, as gaschar was found as the least studied char from the thermochemical processing families347.

### Process flow diagram of the adapted anaerobic digestion model

Anaerobic digestion modeling was fully detailed in the companion paper Javourez et al., 20241, and is therefore not repeated here. Figure SI. 4 displays how the system boundaries were adapted in this study to use AD as a pretreatment for MP production (option B), or as a co-product valorization (e.g. for GB pathways; option A).

Figure SI. 4 – Scope, system boundaries and process flow diagram of AD

“Suitable” feedstock categories and corresponding representative case studies are based on the (pathways; feedstocks) compatibility matrix (see main manuscript). Here, anaerobic digestion (AD) is modeled as a pretreatment of microbial proteins (MP) production (option B; corresponding gas post-treatments are detailed in the section SI. 4.6.2.) or as a co-product’s valorization strategy (e.g. for GB pathways; option A). Details in Javourez et al., 20241.

## Summary of the technology parameters and pairing with the LCI models

The Table SI. 103 summarizes the total set of technology parameters defined across the LCI models, while the Table SI. 104 lays down which pathways are affected by which technology parameters. As the present study builds on the modeling framework elaborated by Javourez et al., 20241, energy and fertilizing-oriented conversion pathways elaborated in the aforementioned companion paper were also represented (and their corresponding technology parameters aggregated to the picture).

Table SI. 103 – Technology parameters retained for the LCA model

|  | n° | Parameter | Unit | Baseline value | Variation range |
| --- | --- | --- | --- | --- | --- |
| Transportation | 1 | Transport to CF facility | km | 50 | 10-100 |
|  | 2 | Transport to final client | km | 50 | 10-100 |
|  | 3 | Diffuse streams towards collection platforms | km | 40 | 20-60 |
|  | 4 | Diffuse streams towards AD/GB facilities | km | 40 | 3-75 |
|  | 5 | Collection platform to direct CHP/GA facilities | km | 40 | 15-100 |
|  | 6 | Collection platform to composting facilities | km | 33 | 12-54 |
|  | 7 | Transport of compost to land | km | 40 | 20-100 |
|  | 8 | Diffuse streams towards domestic furnace | km | 10 | 2-20 |
|  | 9 | Generation site towards industrial furnace | km | 50 | 30-150 |
|  | 10 | Digestate/slurry spreading distance | km | 10 | 3-50 |
|  | 11 | Collection platforms to waste-to-nutrition facilities | km | 50 | 10-100 |
| Generic process parameters | 12 | Humidity for grinding | %ww | 9% | 9-13% |
|  | 13 | Drying cabinet energy requirements | MJ.kgwater-1 | 5.6 | 5.2-6 |
|  | 14 | Drying cabinet capacity | tww.h-1 | 12 | 5-12 |
|  | 15 | Spray dryer electricity requirements | kWh.kgwater-1 | 0.106 | 0.034-0.282 |
|  | 16 | Spray dryer heat requirements | MJ.kgwater-1 | 3 | 2.7-4.1 |
|  | 17 | Grinding energy requirements | kWh.tww-1 | 1 | 0.2-2 |
|  | 18 | Water loss at pelletizing | %water | 16% | 7-16% |
|  | 19 | Feed pelletizing energy requirements | kWh.tww-1 | 20 | 15-28.2 |
|  | 20 | Humidity for pelletizing | %ww | 10% | 8-10% |
|  | 21 | Humidity for crushing | %ww | 9% | 9-13% |
|  | 22 | Crushing dust generation | %ww | 0.5% | 0-0.5% |
|  | 23 | Coefficient of performance (COP) fridge | kW.kWe-1 | 3.5 | 3-4 |
|  | 24 | Ultrafiltration electricity requirements | kWh.m-3 | 0.04 | 0.04-0.15 |
|  | 25 | Ultrafiltration DM out | %DM | 13% | 10-15% |
|  | 26 | Centrifuge DM out | %DM | 30% | 20-35% |
|  | 27 | Centrifuge DM losses | %DMin | 3% | 1-6% |
|  | 28 | Centrifuge electricity requirements | kWh.m-3 | 2.53 | 1.35-8 |
|  | 29 | Gas compression yield | %kJ | 80% | 70-90% |
|  | 30 | Cooling COP | kW.kWe-1 | 5 | 2-7 |
|  | 31 | Low-temperature (LT) heat reuse | %MJ | 50% | 20-65% |
|  | 32 | Sensible high temperature (HT) heat reuse | %MJ | 80% | 50-80% |
|  | 33 | Meat-equivalent ratio | %DM | 80% | 50-100% |
|  | 34 | Crushing energy requirements | kWh.tDM-1 | 0.096 | 0.02-0.156 |
|  | 35 | Gas boiler efficiency | %LHV | 98% | 92-102% |
|  | 36 | Domestic furnace efficiency | %HHV | 73% | 31-81% |
|  | 37 | Industrial furnace efficiency | %HHV | 82% | 70-90% |
| Generic land emission factors | 38 | EF1 N2O-N direct biosolids | kgN2O-N.kgN-1 | 0.006 | 0.001-0.011 |
|  | 39 | Volatilized (FracGASN) mineral* | kg (NH3-N+NOx-N) . kgN-1 | 0.11 | 0.02-0.33 |
|  | 40 | Volatilized (FracGASM) biosolids | kg (NH3-N+NOx-N) . kgN-1 | 0.21 | 0-0.31 |
|  | 41 | EF4 N2O-N indirect biosolids volatilization | kgN2O-N . kg (NH3-N+NOx-N)-1 | 0.01 | 0.002-0.018 |
|  | 42 | EF5 N2O-N indirect biosolids leaching | kgN2O-N . kg NO3-1 | 0.011 | 0-0.02 |
|  | 43 | EF NOx biosolids | kgNOx-N . kgN -1 | 0.0133 | 0.0066-0.026 |
|  | 44 | EF NO3 biosolids | %N_in | 11% | 1-20% |
|  | 45 | EF NH4+ biosolids | %TAN | 1.6% | 1.3%-1.9% |
|  | 46 | EF NH3-N biosolids* | %TAN | 70% | 60%-80% |
|  | 47 | EF CH4-C biosolids* | %C_in | 0.2% | 0-0.5% |
| Specific to solid fermentation | 48 | SSF moisture | %ww | 65% | 50-80% |
|  | 49 | SSF sterilization energy requirements | MJ.kgww-1 | 0.84 | 0.4-1.3 |
|  | 50 | Fungal energy coefficient | %theoretical | 60% | 50-70% |
|  | 51 | Fungal protein content | %DM | 40% | 24.4-53.3% |
|  | 52 | SSF cellulose degradation | %cellulose | 40% | 2.4%-49.9% |
|  | 53 | SSF hemicellulose degradation | %hemicellulose | 40% | 16%-44.9% |
|  | 54 | SSF lignin degradation | %lignin | 40% | 11.9%-50.9% |
|  | 55 | SSF humidity degradation | %moisture | 10% | 3.2%-15.9% |
|  | 56 | SSF crude fiber degradation | %fibers | 20% | (-2.3)-28.7% |
|  | 57 | SSF free sugars degradation | %sugars | 70% | 16.1%-92.8% |
|  | 58 | Roller mill energy requirements | kWh.tww-1 | 40 | 35-45 |
|  | 59 | Humidity for milling | %ww | 14% | 12-16% |
|  | 60 | Water loss at crushing | %water | 0.15% | 0.1-0.2% |
|  | 61 | Flour-equivalent ratio | %DM | 80% | 50-100% |
|  | 62 | SSF digestibility enhancement: agrifood co-products | %OMdigestibility | 15% | (-10)-20% |
|  | 63 | SSF digestibility enhancement: crop residues | %OMdigestibility | 20% | 0-35% |
| Specific to insects farming | 64 | Frass kmin (Nitrogen mineralization rate) | %N | 55% | 35-75% |
|  | 65 | BSF substrate CH4-C storage emissions | gCH4.kgVS-1 | 1 | 0.85-1.15 |
|  | 66 | Decontamination energy requirements | kWh.tww-1 | 26.5 | 3-50 |
|  | 67 | Target BSF substrate DM | %DM | 30% | 20-38% |
|  | 68 | Starter feed mix | mgww.5-DOL-1 | 12 | 10-15 |
|  | 69 | 5-DOL weight (5 days of life larvae) | mgww.5-DOL-1 | 6 | 2-10 |
|  | 70 | Moisture content BSF larvae | %ww | 64% | 60-70% |
|  | 71 | Fat content BSF larvae | %DM | 34% | 25-43% |
|  | 72 | Protein conversion ratio (PCR) by BSF | %proteins | 38% | 25-60% |
|  | 73 | BSF larvae survival rate | %5-DOL | 90% | 78-98% |
|  | 74 | Finishing BSF larvae weight | mg.larvae-1 | 150 | 100-200 |
|  | 75 | BSF growth period length | days | 20 | 11-29 |
|  | 76 | BSF larvae growth CO2 emissions | kgCO2.kgDM-1 | 2.75 | 1.5-4 |
|  | 77 | BSF larvae growth CH4 emissions | mgCH4.kgDM-1 | 28 | 9-95 |
|  | 78 | BSF larvae growth total N emissions | %N | 4.4% | 0.7-8.0% |
|  | 79 | BSF larvae growth N2O emissions | mgN2O.kgDM-1 | 106 | 7-215 |
|  | 80 | BSF larvae growth NH3 emissions | gNH3.kgDM-1 | 3.8 | 0.1-8.0 |
|  | 81 | BSF frass moisture content | %ww | 65% | 50-75% |
|  | 82 | BSF facilities use efficiency enhancement | %current | 28% | 0-50% |
|  | 83 | BSF killing energy requirements | kWh.kgww-1 | 0.14 | 0.09-0.25 |
|  | 84 | Frass composting electricity | kWh.tww-1 | 26 | 0.9-50 |
|  | 85 | Frass composting total C-emissions | %C | 20% | 10-35% |
|  | 86 | Frass composting CH4 emissions | %C_emit | 1% | 0.2-1.7% |
|  | 87 | Frass composting N2O emissions | %N_emit | 1.2% | 0.5-1.9% |
|  | 88 | Composted frass TAN | %N | 19% | 11-37% |
|  | 89 | Frass composting diesel use | L.tww-1 | 0.65 | 0.3-2.8 |
|  | 90 | Frass MFE-P | %P | 90% | 80-100% |
|  | 91 | BSF larvae digestibility (as feed) | %OM | 88% | 85-90% |
|  | 92 | BSF larvae crude fibers | %DM | 7% | 6-8% |
| Specific to mycoproteins production | 93 | Ionic liquid (IL) make-up moisture content | %ww | 20% | 10-30% |
|  | 94 | Relooped IL | %recovered | 99.5% | 99-99.9% |
|  | 95 | IL requirements | kgIL.kgDM-1 | 5.6 | 4-7 |
|  | 96 | Cellulose-to-pulp | %cell_in | 90% | 70-95% |
|  | 97 | Hemicellulose-to-pulp | %hemi_in | 40% | 20-50% |
|  | 98 | Solubles-to-pulp | %sol_in | 10% | 0-20% |
|  | 99 | Lignin-to-pulp | %lignin_in | 20% | 10-60% |
|  | 100 | Evaporator efficiency | %MJ_th | 75% | 70-80% |
|  | 101 | Evaporator electricity requirements | %MJ_heat | 10% | 5-15% |
|  | 102 | Cellulose-rich pulp loading for hydrolysis | %DM | 7.6% | 5-10% |
|  | 103 | Enzymes loading for hydrolysis | mg.gcell-1 | 15 | 10-20 |
|  | 104 | Glucose yield | %cell | 80% | 50-96% |
|  | 105 | C5 sugars yield | %hemi | 80% | 50-99% |
|  | 106 | Hydrolysate final concentration | %DM | 10% | 5-16% |
|  | 107 | Enzyme production | kg.kgsugar-1 | 0.24 | 0.2-0.28 |
|  | 108 | Starch product loading for hydrolysis | %DM | 10% | 5-20% |
|  | 109 | Amylases loading for hydrolysis | mg.gstarch-1 | 0.44 | 0.4-1 |
|  | 110 | Starch-based glucose yield | %sugars | 95% | 90-99% |
|  | 111 | Sugars use efficiency | %in | 95% | 90-99% |
|  | 112 | Myco C6 yield | gX.gS-1 | 0.29 | 0.26-0.36 |
|  | 113 | Myco C5 yield | gX.gS-1 | 0.29 | 0-0.30 |
|  | 114 | Myco biomass concentration | gX.Lbroth-1 | 30 | 20-40 |
|  | 115 | Myco dilution rate | hour-1 | 0.2 | 0.17-0.25 |
|  | 116 | Myco gas use efficiency | %in | 90% | 80-92% |
|  | 117 | Metabolic heat | kJ.molCX-1 | 250 | 160-332 |
|  | 118 | Myco cleaning in place (CIP) frequency | year-1 | 8 | 7-9 |
|  | 119 | Myco mineral medium use | %supplied | 95% | 80-95% |
|  | 120 | Myco RNA removal losses (compared to input) | %DMin | 28% | 23-30% |
|  | 121 | Myco RNA content (share of own weight) | %DM | 9% | 7-11% |
|  | 122 | Myco digestibility | %OM | 88% | 85-93% |
|  | 123 | Myco crude fibers | %DM | 5% | 1-10% |
| Microbial proteins production pathways | 124 | Stripping N recovery* | %N_in | 90% | 80-90% |
|  | 125 | Stripping heat requirements* | MJ.m-3 | 57.6 | 14.8-219.6 |
|  | 126 | MP O2 use efficiency | %O2 | 90% | 80-92% |
|  | 127 | MP NH3 use efficiency | %NH3 | 90% | 80-92% |
|  | 128 | MP bioreactor head pressure | bar | 6 | 5-8 |
|  | 129 | MP CIP frequency | year-1 | 7 | 4-12 |
|  | 130 | MP mineral medium use | %supplied | 95% | 80-95% |
|  | 131 | Medium stirring energy requirements | kWh.mbroth-3 | 0.3 | 0.2-0.5 |
|  | 132 | MOB yield | gVS.gCH4-1 | 0.71 | 0.63-1.11 |
|  | 133 | MOB CH4 use efficiency | %CH4 | 90% | 80-90% |
|  | 134 | MOB biomass concentration | gX.Lbroth-1 | 30 | 20-30 |
|  | 135 | MOB metabolic heat | kJ.molCX-1 | 1277 | 900-1660 |
|  | 136 | HOB yield | gVS.gH2-1 | 2.94 | 2.24-3.4 |
|  | 137 | HOB H2 use efficiency | %H2 | 90% | 80-99% |
|  | 138 | HOB CO2 use efficiency | %CO2 | 90% | 80-99% |
|  | 139 | HOB biomass concentration | gX.Lbroth-1 | 30 | 20-30 |
|  | 140 | MOB metabolic heat | kJ.molCX-1 | 744 | 700-800 |
|  | 141 | Electrolysis efficiency | %MJ | 80% | 70-82% |
|  | 142 | Electrolysis O2 reuse | %O2 | 95% | 10-95% |
|  | 143 | Direct air capture (DAC) energy efficiency* | %MJ | 80% | 75-85% |
|  | 144 | DAC heat requirements* | MJ.kgCO2-1 | 5.5 | 3.8-6.3 |
|  | 145 | DAC electricity requirements* | kWh.kgCO2-1 | 0.3 | 0.23-0.37 |
|  | 146 | Share of water reused | %outlet | 92% | 80-92% |
|  | 147 | MP sterilization temperature | °C | 105 | 90-120 |
|  | 148 | MP RNA removal losses (compared to input) | %DMin | 23% | 15%-30% |
| Specific to anaerobic digestion | 149 | Digestate CH4 emissions, storage | %BMP | 12% | 8-15% |
|  | 150 | Digestate N2O direct (EF1), storage | kgN2O-N . kgN-1 | 0.005 | 0.0006-0.01 |
|  | 151 | Digestate N volatilization emissions (FracGASM) | kg (NH3-N+NOx-N) . kgN-1 | 0.05 | 0.05-0.22 |
|  | 152 | Digestate N2O indirect (EF4), storage | kgN2O-N . kg (NH3-N+NOx-N)-1 | 0.01 | 0.002-0.018 |
|  | 153 | Digestate N2 emissions, storage | kgN2-N . kg N2O-N-1 | 3 | 1-10 |
|  | 154 | AD methane production | %TBMP | 75% | 50-90% |
|  | 155 | AD electricity required, bioreactor | %CHPpower | 8% | 5-11% |
|  | 156 | AD heat required, bioreactor | MJ.tww-1 | 95.9 | 12-249 |
|  | 157 | AD storage emission reduction potential | %CH4&NH3 | 80% | 40-80% |
|  | 158 | Digestate TAN | %N_digestate | 75% | 55-90% |
|  | 159 | Digestate BMP | Nm3CH4.tVS-1 | 177 | 177-458 |
|  | 160 | Digestate NH3 reduction factor | %NH3 | 50% | 20-70% |
|  | 161 | MFE-P digestate | %P | 85% | 45-85% |
|  | 162 | MFE-K digestate | %K | 85% | 45-85% |
|  | 163 | CO2 recovery use | %CO2 | 95% | 10-95% |
|  | 164 | Biogas CHP, power efficiency | %LHV | 40% | 35-45% |
|  | 165 | Biogas CHP, heat efficiency | %LHV | 45% | 40-50% |
|  | 166 | Biogas/Syngas CHP, unburned fraction | %biogas/syngas | 1% | 0.5-2% |
| Specific to gas post-treatment operations | 167 | Steam turbine efficiency | %MJ | 85% | 80-90% |
|  | 168 | Steam reforming (SR) electricity requirements | kWh.kgH2-1 | 1.11 | 0.2-4.3 |
|  | 169 | SR pressure | bar | 35 | 30-40 |
|  | 170 | SR steam:C ratio | molH2O.molC-1 | 3 | 2.5-4 |
|  | 171 | SR CH4 conversion yield | %CH4 | 85% | 76-95% |
|  | 172 | SR outlet losses | %out | 1% | 1-2% |
|  | 173 | Water conversion shift (WCS) steam:gas ratio | vol basis | 0.6 | 0.5-0.7 |
|  | 174 | WCS CO conversion yield | %CO | 85% | 80-89% |
|  | 175 | WCS outlet losses | %out | 1% | 0-1% |
|  | 176 | Methanation conversion yield | %C | 95% | 92-98% |
|  | 177 | Methanation outlet losses | %out | 1% | 0-1% |
|  | 178 | Methanation electricity, GA | kWh.kgCH4-1 | 1.24 | 1-10 |
|  | 179 | Methanation electricity, AD | %HHVin | 1.1% | 1.0-1.4% |
| Specific to gasification | 180 | Dryer thermal efficiency | %MJ | 90% | 85-95% |
|  | 181 | Syngas CHP, power efficiency | %LHV | 35% | 30-40% |
|  | 182 | Syngas CHP, heat efficiency | %LHV | 45% | 40-50% |
|  | 183 | Gasification stage electricity | kWh.tDM-1 | 180 | 100-650 |
|  | 184 | Syngas cleaning losses | %out | 1% | 0-1% |
|  | 185 | Gaschar increase in maize yield | %yield | 28% | 0-28% |
|  | 186 | Gaschar increase in wheat yield | %yield | 14% | 0-14% |
|  | 187 | Gaschar application density | tww.ha-1 | 10 | 1-20 |
|  | 188 | Gaschar decrease in N2O emissions | %N2O | 30% | 0-30% |
|  | 189 | Gaschar C loss, 100 years | %C | 50% | 3-96% |
| Specific to direct combustion | 190 | Primary crop residues (PCR) Combustion factor | % | 0.85 | 0.8-0.9 |
|  | 191 | PCR CO2 | kgCO2 . kgDM-1 | 1.515 | 1.338-1.692 |
|  | 192 | PCR NOx | kgNOx . kgDM-1 | 0.0023 | 0.0018-0.0027 |
|  | 193 | PCRCO | kgCO . kgDM-1 | 0.06596 | 0.0388-0.0987 |
|  | 194 | PCR SOx | kgSOx . kgDM-1 | 0.00032 | 0.0001-0.0005 |
|  | 195 | PCR Total suspended particles (TSP) | kgTSP . kgDM-1 | 0.0063 | 0.0058-0.0078 |
|  | 196 | PCR PM10 | kgPM10 . kgDM-1 | 0.00622 | 0.0057-0.0077 |
|  | 197 | PCR PM2.5 | kgPM2.5 . kgDM-1 | 0.00594 | 0.0054-0.0074 |
|  | 198 | Primary forestry residues (PFR) Combustion factor | % | 0.62 | 0.5-0.74 |
|  | 199 | PFR CO2 | kgCO2 . kgDM-1 | 1.569 | 1.438-1.7 |
|  | 200 | PFR CH4 | kgCH4 . kgDM-1 | 0.0047 | 0.0028-0.0066 |
|  | 201 | PFR CO | kgCO . kgDM-1 | 0.107 | 0.07-0.144 |
|  | 202 | PFR NOx | kgNOx . kgDM-1 | 0.003 | 0.0016-0.0044 |
|  | 203 | PFR N2O | kgN2O . kgDM-1 | 0.00026 | 0.00019-0.00033 |
|  | 204 | PFR Carbon sequestration (unburned fraction) | kgC.kgCtot-1 | 5% | 0-10% |
|  | 205 | Combustion pellet. electricity | kWh.kgDM-1 | 0.25 | 0.225-0.275 |
|  | 206 | Combustion pellet. heat | MJ.kgwater-1 | 3.3 | 2.97-3.63 |
|  | 207 | Solid fuel CHP power efficiency | %HHV | 22% | 18-26% |
|  | 208 | Solid fuel CHP heat efficiency | %HHV | 70% | 60-78% |
|  | 209 | Solid fuel CHP electricity requirements | kWh.tww-1 | 100 | 65-162 |
|  | 210 | Incineration activated carbon consumption | kg.tww-1 | 1 | 0.63-1.48 |
|  | 211 | Incineration activated ammonia consumption | kg.tww-1 | 1.5 | 1-2.9 |
|  | 212 | Incineration CaCO3 consumption | kg.tww-1 | 5 | 4-11.9 |
|  | 213 | Incineration Ca(OH)2 consumption | kg.tww-1 | 3 | 0.3-10 |
|  | 214 | Incineration NaOH consumption | kg.tww-1 | 1.2 | 0.2-2.6 |
| Specific to composting | 215 | Electricity requirements sorting/depackaging | kWh.tww-1 | 36.5 | 29.2-43.8 |
|  | 216 | Composting unit per input | unit.tww-1 | 3.3E-8 | 7.4E-9-3.3E-8 |
|  | 217 | Compost VS degradation | %VS | 70% | 62-83.5% |
|  | 218 | Compost N degradation | %N | 51% | 38.5-71% |
|  | 219 | Compost CH4-C emissions | %C_deg | 1% | 0.2-3% |
|  | 220 | Compost N2O-N emissions | %N_deg | 1.4% | 0-1.8% |
|  | 221 | Compost N2-N emissions | %N_deg | 1% | 0.1-3.2% |
|  | 222 | Compost water content | %ww | 70.5% | 65-77% |
|  | 223 | Compost CO-C emissions | %C_deg | 0.34% | 0-0.34% |
|  | 224 | Composting leaching | Lleaching.tww-1 | 238 | 0-238 |
|  | 225 | Biofilters N2O emissions reduction | %N20 | 10% | 0-90% |
|  | 226 | Biofilters CH4 emissions reduction | %CH4 | 95% | 33-99.9% |
|  | 227 | Composting diesel | L.tww-1 | 2 | 0.7-3 |
|  | 228 | Composting electricity | kWh.tww-1 | 47 | 40-60 |
|  | 229 | Compost terpens emissions | kg.kgww-1 | 1.2E-6 | 9.8E-7-1.5E-6 |
|  | 230 | Compost H2S emissions | kg.kgww-1 | 1.9E-7 | 1.5E-7-2.3E-7 |
|  | 231 | Compost NH3-N volatization | %N | 1% | 0-1% |
|  | 232 | B0 mature compost | m3CH4.kgVS-1 | 0.3 | 0.2-0.4 |
|  | 233 | Compost storage CO2-C emissions | kgCO2 . kgCH4-1 | 2 | 1.8-2.2 |
|  | 234 | Compost carbon sequestration | kgC.kgCtot-1 | 2% | 0-10% |
|  | 235 | Compost kmin (Nitrogen mineralization rate) | %Norg | 35% | 10-35% |
|  | 236 | Compost MFE-P | %P | 90% | 80-100% |
|  | 237 | Compost MFE-K | %K | 90% | 80-100% |
| Specific to decay on land | 238 | In-situ decay, non-arable land, EF CH4 decay* | kgCH4-C.kgC-1 | 0.00002 | 0.00002-0.01 |
|  | 239 | Carbon sequestration In-situ, non-arable land* | kgC.kgCtot-1 | 2% | 0%-10% |
|  | 240 | In-situ decay, arable land EF CH4 decay* | kgCH4-C.kgC-1 | 0.00002 | 0-0.005 |
|  | 241 | EF NH3-N biosolids | %TAN | 50% | 35%-65% |
|  | 242 | Carbon sequestration, arable land* | kgC.kgCtot-1 | 0.5% | -(1%)-2% |
|  | 243 | MFE-P decay crop residues* | %P | 80% | 80%-100% |
|  | 244 | MFE-K decay crop residues* | %K | 80% | 80%-100% |
|  | 245 | Ploughing diesel | kg.ha-1 | 5.292 | 5-7.5 |
|  | 246 | Manure NH3-N (EF indoor storage) | %TAN | 10% | 8%-19% |
|  | 247 | Manure B0 (TMP) | m3CH4.kgVS-1 | 0.21 | 0.18-0.24 |
|  | 248 | Manure methane conversion factor (MCF), storage | %TMP | 4% | 2%-5% |
|  | 249 | Manure N2O-N direct (EF1), storage | kgN2O-N.kgN-1 | 0.0075 | 0.005-0.01 |
|  | 250 | Manure NH3-N (EF outdoor storage) | %TAN | 37% | 27%-53% |
|  | 251 | Manure NO3-N (EF land) | %Ntot | 12% | 10%-15% |
|  | 252 | Manure NH3-N (EF land) | %TAN | 71% | 68%-79% |
|  | 253 | Carbon sequestration, manure* | kgC.kgCtot-1 | 0.5% | (-1%)-2% |
|  | 254 | MFE-P manure* | %P | 85% | 65%-100% |
|  | 255 | Slurry NH3-N (EF indoor storage) | %TAN | 18% | 9%-24% |
|  | 256 | Slurry B0 (TMP) | m3CH4.kgVS-1 | 0.21 | 0.18-0.24 |
|  | 257 | Slurry MCF | %TMP | 39% | 37%-41% |
|  | 258 | Slurry CH4 land emissions reduction (technology) | %CH4 | 0.4 | 0.25-0.5 |
|  | 259 | Slurry N2O-N direct (EF1) | kgN2O-N.kgN-1 | 0.005 | 0.005-0.01 |
|  | 260 | Slurry NH3-N (EF outdoor storage) | %TAN | 34% | 20%-53% |
|  | 261 | Slurry NO3-N | %Ntot | 12% | 10%-15% |
|  | 262 | Slurry NH3 reduction factor | %NH3 | 50% | 30%-70% |
|  | 263 | Sludge spreading distance | km | 21.5 | 5-40 |
|  | 264 | N2O emissions reject water | %Ntot | 0.3% | 0.2%-1.0% |
|  | 265 | Sludge N2O-N direct (EF1) | kgN2O-N.kgN-1 | 0.01 | 0.001-0.035 |
|  | 266 | Sludge NO3-N, groundwater | %Ntot | 18% | 11%-25% |
|  | 267 | Sludge NO3-N, surface water | %Ntot | 10% | 0%-19% |
|  | 268 | Sludge CH4 emissions | kgCH4-C.kgC-1 | 0.0001 | 0-0.002 |
|  | 269 | Carbon sequestration, sludge | kgC.kgCtot-1 | 1.0% | 0-4.9% |
|  | 270 | Sludge P emissions | %P | 2.2% | 1.0-5.0% |
|  | 271 | MFE-K sludge | %K | 80% | 67-93% |
|  | 272 | MFE-P sludge | %P | 80% | 67-93% |
| Specific to green biorefinery | 273 | Washing water | m3.tww-1 | 33 | 12-87 |
|  | 274 | Irrigation replacement | % | 50% | 0-100% |
|  | 275 | Maceration electricity requirements | kWh.tgreen_juice-1 | 4.1 | 2.8-8.3 |
|  | 276 | Screw-press electricity requirements | kWh.tgreen_juice-1 | 3.4 | 2.3-4.5 |
|  | 277 | Protein recovery in green juice | %proteins | 36% | 16-69% |
|  | 278 | Lipids recovery in green juice | %lipids | 22% | 18-59% |
|  | 279 | Soluble carbs recovery in green juice | %solubles | 40% | 15-70% |
|  | 280 | Hemicellulose recovery in green juice | %hemicel. | 15% | 10-19% |
|  | 281 | Ashes recovery in green juice | %ashes | 46% | 22-59% |
|  | 282 | Fibers recovery in green juice | %fibers | 3% | 1-19% |
|  | 283 | P recovery in green juice | %P | 61% | 44-67% |
|  | 284 | K recovery in green juice | %K | 64% | 57-78% |
|  | 285 | Heat for protein coaggulation | MJ.tww-1 | 270 | 252-294 |
|  | 286 | Protein separation electricity | kWh.tww-1 | 5 | 1.4-11.1 |
|  | 287 | Protein recovery in green pellet | %proteins | 50% | 30-90% |
|  | 288 | Ashes recovery in green pellet | %ashes | 15% | 13-50% |
|  | 289 | Soluble carbs recovery in green pellet | %solubles | 21% | 5-36% |
|  | 290 | Fibers recovery in green pellet | %fibers | 28% | 5-90% |
|  | 291 | Water recovery in green pellet | %water | 10% | 5-15% |
|  | 292 | Green pellet digestibility | %OM | 75% | 50-85% |
|  | 293 | Press-pulp digestibility | %OM | 65% | 60-70% |

*: values displayed as in Javourez et al., (2024)1 (France-scale average values), but values used for scaling implications (section SI. 4.6 of the main manuscript) were taken as displayed in the PhD thesis manuscript352 (larger range to capture potential local conditions pedoclimatic outliers, as the functional unit is per unit ton of input stream).

Table SI. 104 – Influence of parameters on the different pathways

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transportation | 1 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | x | x | x | x | x | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 3 | x |  |  |  | x | x | x |  | x |  | x | x |  |  | x |  |  |
|  | 4 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  | x |  |
|  | 5 |  |  |  |  |  |  |  |  | x |  | x | x |  |  | x |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 7 |  |  |  |  | x |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |
|  | 9 |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |
|  | 10 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  | x |  |
|  | 11 |  |  |  | x | x | x | x |  |  |  |  |  |  |  |  |  |  |
| Generic process parameters | 12 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 |  | x | x | x | x |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 17 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 19 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 | x |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 21 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 22 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 23 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 24 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 25 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 26 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 27 |  |  |  | x |  |  | x | x | x |  |  |  |  |  |  |  |  |
|  | 28 |  |  |  | x |  |  | x | x | x |  |  |  |  |  |  |  |  |
|  | 29 |  |  |  | x |  |  | x | x | x | x | x |  |  |  |  |  |  |
|  | 30 |  |  |  | x |  |  | x | x | x |  | x |  |  |  |  |  |  |
|  | 31 |  |  |  | x |  |  | x | x | x | x | x | x | x |  |  |  |  |
|  | 32 |  |  |  |  |  |  |  | x | x | x | x |  |  |  |  |  |  |
|  | 33 |  |  |  | x | x |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 34 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 35 |  | x | x | x |  |  |  | x | x | x | x |  |  |  |  |  |  |
|  | 36 |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |
|  | 37 |  |  |  | x |  |  | x |  |  |  |  |  | x |  |  |  |  |

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Generic land emission factors | 38 |  | x | x | x | x |  |  | x |  | x |  |  |  | x | x | x | x |
|  | 39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | x | x | x |  |  |  | x |  | x |  |  |  | x | x | x | x |
|  | 41 |  | x | x | x | x |  |  | x |  | x |  |  |  | x | x | x | x |
|  | 42 |  | x | x | x | x |  |  | x |  | x |  |  |  |  | x | x | x |
|  | 43 |  | x | x | x | x |  |  | x |  | x |  |  |  |  | x | x | x |
|  | 44 |  |  |  |  | x |  |  |  |  |  |  |  |  |  | x | x | x |
|  | 45 |  |  |  |  | x |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 46 |  |  |  |  | x |  |  |  |  |  |  |  |  |  | x | x |  |
|  | 47 |  | x | x | x | x |  |  | x |  | x |  |  |  |  | x |  |  |
| Specific to solid fermentation | 48 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 49 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 53 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 54 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 55 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 57 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 58 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 59 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 61 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 62 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
| Specific to insects farming | 64 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 67 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 68 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 71 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 77 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specific to insects farming | 78 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 80 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 81 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 82 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 83 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 85 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 87 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 89 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 90 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 91 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92 |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
| Specific to mycoproteins production | 93 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 94 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 95 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 96 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 98 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 99 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 101 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 102 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 103 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 104 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 105 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 106 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 107 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 109 |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 110 |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 112 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 113 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 114 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 116 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 117 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 118 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 119 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 120 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 121 |  |  |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 122 |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |
|  | 123 |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |
| Microbial proteins production pathways | 124 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 125 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 126 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 127 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 128 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 129 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 130 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 131 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 132 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 133 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 134 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 135 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 136 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 137 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 138 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 139 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 140 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 141 |  |  |  |  |  |  |  | x | x |  | x |  |  |  |  |  |  |
|  | 142 |  |  |  |  |  |  |  | x | x |  | x |  |  |  |  |  |  |
|  | 143 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 144 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 145 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 146 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 147 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
|  | 148 |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |
| Specific to anaerobic digestion | 149 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 150 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 151 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 152 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 153 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 154 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 155 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 156 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 157 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specific to anaerobic digestion | 158 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 159 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 160 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 161 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 162 |  | x | x | x |  |  |  | x |  | x |  |  |  |  |  |  |  |
|  | 163 |  | x | x | x |  |  |  |  |  | x |  |  |  |  |  |  |  |
|  | 164 |  | x | x | x |  |  |  |  |  | x |  |  |  |  |  |  |  |
|  | 165 |  | x | x | x |  |  |  |  |  | x |  |  |  |  |  |  |  |
|  | 166 |  | x | x | x |  |  |  |  |  | x | x |  |  |  |  |  |  |
| Specific to gas post-treatment operations | 167 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 168 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 169 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 170 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 171 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 172 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 173 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 174 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 175 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 176 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 177 |  |  |  |  |  |  |  | x |  |  | x |  |  |  |  |  |  |
|  | 178 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 179 |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |
| Specific to gasification | 180 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 181 |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |
|  | 182 |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |
|  | 183 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 184 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 185 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 186 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 187 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 188 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
|  | 189 |  |  |  |  |  |  |  |  | x |  | x |  |  |  |  |  |  |
| Specific to direct combustion | 190 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 191 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 192 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 193 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 194 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 195 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 196 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 197 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specific to direct combustion | 198 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 199 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 202 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 203 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 204 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
|  | 205 |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |
|  | 206 |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |
|  | 207 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
|  | 208 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
|  | 209 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
|  | 210 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
|  | 211 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
|  | 212 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
|  | 213 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
|  | 214 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |
| Specific to composting | 215 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 216 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 217 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 218 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 219 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 220 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 221 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 222 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 223 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 224 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 225 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 226 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 227 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 228 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 229 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 230 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 231 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 232 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 233 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 234 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 235 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 236 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
|  | 237 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specific to decay on land | 238 |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  | x |
|  | 239 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |
|  | 240 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |
|  | 241 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |
|  | 242 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |
|  | 243 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |
|  | 244 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |
|  | 245 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |
|  | 246 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 247 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 248 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 249 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 252 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 253 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 254 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 255 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 256 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 258 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 259 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 262 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 263 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 264 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 265 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 266 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 267 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 268 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 269 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 270 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 271 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
|  | 272 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |

|  | n° | CF | GB_AD | GB_rough | GB_myco | BSF | SSF | MYCO | MP_AD | MP_GA | AD | GA | DIR_CHP | Heat | Burn | Compost | Spread | Decay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specific to green biorefinery | 273 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 274 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 275 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 276 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 277 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 278 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 279 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 280 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 281 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 282 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 283 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 284 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 285 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 286 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 287 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 288 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 289 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 290 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 291 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 292 |  | x | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 293 |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Parameters defined in the sections SI. 4.2-4.8 and in Javourez et al., 20241 .

# References

1.	Javourez, U., Karan, S. K. & Hamelin, L. Setting the environmental performance threshold for diverting residual streams towards bioeconomy strategies. Preprint at https://doi.org/10.21203/rs.3.rs-3626167/v2 (2023).

2.	Javourez, U., O’Donohue, M. & Hamelin, L. Waste-to-nutrition: a review of current and emerging conversion pathways. Biotechnol. Adv. 53, 107857 (2021).

3.	EC Regulation. Regulation (EC) No 1069/2009 of the European Parliament and of the Council of 21 October 2009 Laying down Health Rules as Regards Animal by-Products and Derived Products Not Intended for Human Consumption and Repealing Regulation (EC) No 1774/2002 (Animal by-Products Regulation). OJ L vol. 300 1–33 (2009).

4.	Javourez, U., Rosero Delgado, E. A. & Hamelin, L. Upgrading agrifood co-products via solid fermentation yields environmental benefits under specific conditions only. Nat Food 3, 911–920 (2022).

5.	FranceAgriMer. L’Observatoire National Des Ressources En Biomasse, Évaluation Des Ressources Agricoles et Agroalimentaires Disponibles En France – Édition 2020. 93 https://www.franceagrimer.fr/content/download/66147/document/DON-ONRB-2020_VF3.pdf (2021).

6.	Lodato, C., Hamelin, L., Tonini, D. & Astrup, T. F. Towards sustainable methane supply from local bioresources: Anaerobic digestion, gasification, and gas upgrading. Appl. Energy 323, 119568 (2022).

7.	Blok, M. C. & Spek, J. W. CVB Feed Table 2018 - Chemical composition and nutritional values of feedstuffs. (2018).

8.	Feedipedia. List of feeds. https://feedipedia.org/content/feeds?category=All (2020).

9.	INRAE, CIRAD, & AFZ. Tables of composition and nutritional values of feed materials. INRAE-CIRAD-AFZ Feed tables https://feedtables.com/ (2021).

10.	Hamelin, L., Naroznova, I. & Wenzel, H. Environmental consequences of different carbon alternatives for increased manure-based biogas. Appl. Energy 114, 774–782 (2014).

11.	Karan, S. Cambioscop RO1: Dataset on characterization, quantity and current use of French residual biomasses. Mendeley Data (2022) doi:10.17632/b9sx3h3584.1.

12.	Le Ravalec, V. Panorama Des Coproduits et Résidus Biomasse à Usage Des Filières Chimie et Matériaux Biosourcés En France. 70 https://www.ademe.fr/sites/default/files/assets/documents/rapport_final_tech2market.pdf (2015).

13.	ADEME. Un Mix de Gaz 100% Renouvelable En 2050 ? Etude de Faisabilité Technico-Économique. 283 (2018).

14.	ADEME. Transition(s) 2050 - Choisir Maintenant, Agir Pour Le Climat - Rapport. (ADEME Editions, Angers, 2021).

15.	Association Solagro. Les scénarios Afterres2050, version 2016. (2016).

16.	Cucurachi, S. et al. Prospective LCA Methodology for Novel and Emerging Technologies for Bio-Based Products : The Planet Bio Project. (Publications Office of the European Union, Luxembourg, 2022).

17.	Delsignore, M. & Siddiqui, S. A. Chapter 8. From waste to food: legislative insights. in Waste to Food - Returning nutrients to the food chain 197–208 (Wageningen Academic Publishers, 2022).

18.	James, K., Millington, A. & Randall, N. Food and feed safety vulnerabilities in the circular economy. EFS3 19, (2022).

19.	Bosch, G. et al. Standardisation of quantitative resource conversion studies with black soldier fly larvae. Journal of Insects as Food and Feed 6, 95–109 (2020).

20.	Verstraete, W., Clauwaert, P. & Vlaeminck, S. E. Used water and nutrients: Recovery perspectives in a ‘panta rhei’ context. Bioresour. Technol. 215, 199–208 (2016).

21.	Hadi, J. & Brightwell, G. Safety of Alternative Proteins: Technological, Environmental and Regulatory Aspects of Cultured Meat, Plant-Based Meat, Insect Protein and Single-Cell Protein. Foods 10, 1226 (2021).

22.	Khoshnevisan, B. et al. From renewable energy to sustainable protein sources: Advancement, challenges, and future roadmaps. Renew. Sust. Energ. Rev. 157, 112041 (2022).

23.	Verstraete, W., Yanuka‐Golub, K., Driesen, N. & De Vrieze, J. Engineering microbial technologies for environmental sustainability: choices to make. Microb. Biotechnol. 1751-7915.13986 (2021).

24.	Capozzi, F. Food Innovation in the Frame of Circular Economy by Designing Ultra-Processed Foods Optimized for Sustainable Nutrition. Front. Nutr. 9, 886220 (2022).

25.	Banks, M., Johnson, R., Giver, L., Bryant, G. & Guo, M. Industrial production of microbial protein products. Curr. Opin. Biotechnol. 75, 102707 (2022).

26.	Colonna, P. Bioraffineries. Techniques de l’ingénieur 27 (2020).

27.	Kreyenschulte, D., Emde, F., Regestein, L. & Büchs, J. Computational minimization of the specific energy demand of large-scale aerobic fermentation processes based on small-scale data. Chemical Engineering Science 153, 270–283 (2016).

28.	Jouhara, H. et al. Waste heat recovery technologies and applications. Thermal Science and Engineering Progress 6, 268–289 (2018).

29.	Vance, D. et al. Estimation of and barriers to waste heat recovery from harsh environments in industrial processes. J Clean Prod 222, 539–549 (2019).

30.	Ammar, Y., Joyce, S., Norman, R., Wang, Y. & Roskilly, A. P. Low grade thermal energy sources and uses from the process industry in the UK. Applied Energy 89, 3–20 (2012).

31.	Marami, H. et al. Going beyond conventional wastewater treatment plants within circular bioeconomy concept – a sustainability assessment study. Water Science and Technology wst2022096 (2022).

32.	ADEME, Millet, B., Monnet, A. & Siragusano, T. Première analyse du potentiel technique d’électrification des procédés industriels thermiques par des technologies matures. 35 https://www.ceren.fr/download/4824/ (2020).

33.	Brückner, S. et al. Industrial waste heat recovery technologies: An economic analysis of heat transformation technologies. Applied Energy 151, 157–167 (2015).

34.	Campana, F. et al. ORC waste heat recovery in European energy intensive industries: Energy and GHG savings. Energy Conversion and Management 76, 244–252 (2013).

35.	Seck, G. S., Guerassimoff, G. & Maïzi, N. Heat recovery with heat pumps in non-energy intensive industry: A detailed bottom-up model analysis in the French food & drink industry. Applied Energy 111, 489–504 (2013).

36.	McKenna, R. C. & Norman, J. B. Spatial modelling of industrial heat loads and recovery potentials in the UK. Energy Policy 38, 5878–5891 (2010).

37.	Zuberi, M. J. S. et al. Excess heat recovery: An invisible energy resource for the Swiss industry sector. Applied Energy 228, 390–408 (2018).

38.	Delpech, B. et al. Energy efficiency enhancement and waste heat recovery in industrial processes by means of the heat pipe technology: Case of the ceramic industry. Energy 158, 656–665 (2018).

39.	Danish Energy Agency. Technology Data - Generation of Electricity and District Heating. 418 https://ens.dk/sites/ens.dk/files/Analyser/technology_data_catalogue_for_el_and_dh.pdf (2022).

40.	El Abbadi, S. H., Sherwin, E. D., Brandt, A. R., Luby, S. P. & Criddle, C. S. Displacing fishmeal with protein derived from stranded methane. Nat. Sustain. 1–10 (2021) doi:10.1038/s41893-021-00796-2.

41.	Van Peteghem, L. et al. Towards new carbon–neutral food systems: Combining carbon capture and utilization with microbial protein production. Bioresour. Technol. 349, 126853 (2022).

42.	Madeddu, S. et al. The CO2 reduction potential for the European industry via direct electrification of heat supply (power-to-heat). Environ. Res. Lett. 15 (2020).

43.	Keil, C., Plura, S., Radspieler, M. & Schweigler, C. Application of customized absorption heat pumps for utilization of low-grade heat sources. Applied Thermal Engineering 28, 2070–2076 (2008).

44.	Law, R., Harvey, A. & Reay, D. Opportunities for low-grade heat recovery in the UK food processing industry. Applied Thermal Engineering 53, 188–196 (2013).

45.	Walsh, C. & Thornley, P. Barriers to improving energy efficiency within the process industries with a focus on low grade heat utilisation. Journal of Cleaner Production 23, 138–146 (2012).

46.	European Commission. EU Reference Scenario 2020: Energy, Transport and GHG Emissions : Trends to 2050. (Publications Office of the European Union, LU, 2021).

47.	Institute National de l’Economie Circulaire. La Valorisation Des Mâchefers - Une Technique d’économie Circulaire Inscrite Dans Les Territoires. 88 https://institut-economie-circulaire.fr/wp-content/uploads/2020/03/etude-inec-la-valorisation-des-machefers.pdf (2020).

48.	ecoinvent. ecoinvent databases. https://ecoinvent.org/ (2022).

49.	USDA. Macronutrients, National Agricultural Library. https://www.nal.usda.gov/human-nutrition-and-food-safety/food-composition/macronutrients (2022).

50.	Davis, T. L. et al. Chemical Oxygen Demand Can Be Converted to Gross Energy for Food Items Using a Linear Regression Model. The Journal of Nutrition 151, 445–453 (2021).

51.	Bailey, R. & Wellesley, L. Chokepoints and Vulnerabilities in Global Food Trade. (Chatham House, London, 2017).

52.	Billen, G. et al. Reshaping the European agro-food system and closing its nitrogen cycle: The potential of combining dietary change, agroecology, and circularity. One Earth 4, 839–850 (2021).

53.	Leip, A. et al. Halving nitrogen waste in the European Union food systems requires both dietary shifts and farm level actions. Global Food Security 35, 100648 (2022).

54.	FAO. FAOSTAT. http://www.fao.org/faostat/en/ (2019).

55.	OECD/FAO. OECD-FAO Agricultural Outlook 2022-2031. (OECD Publishing, Paris, 2022). doi:10.1787/f1b0b29c-en.

56.	Agreste. Chapitre 12 - Produits agroalimentaires. Alimentation animale. in Graph’Agri 2020 (2019).

57.	Smetana, S. et al. Meat substitutes: Resource demands and environmental footprints. Resources, Conservation and Recycling 190, 106831 (2023).

58.	Profeta, A. et al. Discrete Choice Analysis of Consumer Preferences for Meathybrids—Findings from Germany and Belgium. Foods 10, 71 (2021).

59.	Searchinger, T., Waite, R., Hanson, C., Ranganathan, J. & Matthews, E. Creating a Sustainable Food Future: A Menu of Solutions to Feed Nearly 10 Billion People by 2050. Final Report. (World Resource Institute, Washington (USA), 2018).

60.	Smetana, Profeta, A., Voigt, R., Kircher, C. & Heinz, V. Meat substitution in burgers: nutritional scoring, sensorial testing, and Life Cycle Assessment. Future Foods 100042 (2021).

61.	Bianchi, F. et al. Replacing meat with alternative plant-based products (RE-MAP): a randomized controlled trial of a multicomponent behavioral intervention to reduce meat consumption. The American Journal of Clinical Nutrition 115, 1357–1366 (2022).

62.	Parodi, A. et al. The potential of future foods for sustainable and healthy diets. Nat. Sustain. 1, 782–789 (2018).

63.	Howard, P. H. Cellular agriculture will reinforce power asymmetries in food systems. Nat Food 3, 798–800 (2022).

64.	York, R. Poultry and fish and aquatic invertebrates have not displaced other meat sources. Nat Sustain 4, 766–768 (2021).

65.	Albizzati, P. F., Tonini, D. & Astrup, T. F. A Quantitative Sustainability Assessment of Food Waste Management in the European Union. Environ. Sci. Technol. (2021).

66.	Salemdeeb, R., zu Ermgassen, E. K. H. J., Kim, M. H., Balmford, A. & Al-Tabbaa, A. Environmental and health impacts of using food waste as animal feed: a comparative analysis of food waste management options. J Clean Prod 140, 871–880 (2017).

67.	Spykman, R. et al. A modular environmental and economic assessment applied to the production of Hermetia illucens larvae as a protein source for food and feed. Int J Life Cycle Assess (2021) doi:10.1007/s11367-021-01986-y.

68.	Tabassum-Abbasi, Abbasi, T. & Abbasi, S. A. Reducing the global environmental impact of livestock production: the minilivestock option. Journal of Cleaner Production 112, 1754–1766 (2016).

69.	van Huis, A. & Oonincx, D. G. A. B. The environmental sustainability of insects as food and feed. A review. Agron. Sustain. Dev. 37, 43 (2017).

70.	IPIFF. Promoting Insects for Human Consumption & Animal Feed – IPIFF. International Platform of Insects for Food and Feed, Brussels https://ipiff.org/ (2022).

71.	Gravel, A. & Doyen, A. The use of edible insect proteins in food: Challenges and issues related to their functional properties. Innov. Food Sci. Emerg. Technol. 59, 102272 (2020).

72.	Ee, K. Y., Lam, M. Q., Mah, J. K. & Merican, A. Black soldier fly (Hermetia illucens L.) larvae in degrading agricultural waste as a sustainable protein production: feedstock modification and challenges. Int J Trop Insect Sci (2022).

73.	Quan, J., Wang, Y., Cheng, X., Li, C. & Yuan, Z. Revealing the effects of fermented food waste on the growth and intestinal microorganisms of black soldier fly (Hermetia illucens) larvae. Waste Management 171, 580–589 (2023).

74.	Brai, A. et al. Efficient use of agricultural waste to naturally fortify Tenebrio molitor mealworms and evaluation of their nutraceutical properties. Journal of Insects as Food and Feed 1–12 (2022) doi:10.3920/JIFF2022.0090.

75.	Cohn, Z., Latty, T. & Abbas, A. Understanding dietary carbohydrates in black soldier fly larvae treatment of organic waste in the circular economy. Waste Management 137, 9–19 (2022).

76.	Dortmans, B. M. A., Diener, S., Verstappen, B. M. & Zurbrügg, C. Black Soldier Fly Biowaste Processing - A Step-by-Step Guide. (2017).

77.	Guo, H., Jiang, C., Zhang, Z., Lu, W. & Wang, H. Material flow analysis and life cycle assessment of food waste bioconversion by black soldier fly larvae (Hermetia illucens L.). Sci. Total Environ. 750, 141656 (2021).

78.	Smetana, S., Spykman, R. & Heinz, V. Environmental aspects of insect mass production. J. Insects Food Feed 7, 553–571 (2021).

79.	Parodi, A. et al. Bioconversion efficiencies, greenhouse gas and ammonia emissions during black soldier fly rearing – A mass balance approach. J. Clean. Prod. 271, 122488 (2020).

80.	Gligorescu, A., Fischer, C. H., Larsen, P. F., Nørgaard, J. V. & Heckman, L.-H. L. Production and Optimization of Hermetia illucens (L.) Larvae Reared on Food Waste and Utilized as Feed Ingredient. Sustainability 12, 9864 (2020).

81.	Ewald, N. et al. Fatty acid composition of black soldier fly larvae (Hermetia illucens) – Possibilities and limitations for modification through diet. Waste Management 102, 40–47 (2020).

82.	Fuso, A. et al. Effect of the Rearing Substrate on Total Protein and Amino Acid Composition in Black Soldier Fly. Foods 10, 1773 (2021).

83.	Hopkins, I., Newman, L. P., Gill, H. & Danaher, J. The Influence of Food Waste Rearing Substrates on Black Soldier Fly Larvae Protein Composition: A Systematic Review. Insects 12, 608 (2021).

84.	Siddiqui, S. a. et al. Manipulation of the black soldier fly larvae (Hermetia illucens; Diptera: Stratiomyidae) fatty acid profile through the substrate. J. Insects Food Feed 1–20 (2022).

85.	de Quelen, F., Brossard, L., Wilfart, A., Dourmad, J.-Y. & Garcia-Launay, F. Eco-Friendly Feed Formulation and On-Farm Feed Production as Ways to Reduce the Environmental Impacts of Pig Production Without Consequences on Animal Performance. Front. Vet. Sci. 8, 703 (2021).

86.	Parodi, A. et al. Upgrading ammonia-nitrogen from manure into body proteins in black soldier fly larvae. Resources, Conservation and Recycling 182, 106343 (2022).

87.	Bar‐Shmuel, N., Behar, A. & Segoli, M. What do we know about biological nitrogen fixation in insects? Evidence and implications for the insect and the ecosystem. Insect Science 27, 392–403 (2020).

88.	Bava, L. et al. Rearing of Hermetia Illucens on Different Organic By-Products: Influence on Growth, Waste Reduction, and Environmental Impact. Animals 9, 289 (2019).

89.	Ravi, H. K. et al. Larvae Mediated Valorization of Industrial, Agriculture and Food Wastes: Biorefinery Concept through Bioconversion, Processes, Procedures, and Products. Processes 8, 857 (2020).

90.	Mertenat, A., Diener, S. & Zurbrügg, C. Black Soldier Fly biowaste treatment – Assessment of global warming potential. Waste Management 84, 173–181 (2019).

91.	Pang, W. et al. Reducing greenhouse gas emissions and enhancing carbon and nitrogen conversion in food wastes by the black soldier fly. Journal of Environmental Management 260, 110066 (2020).

92.	Parodi, A. et al. Black soldier fly reared on pig manure: Bioconversion efficiencies, nutrients in the residual material, greenhouse gas and ammonia emissions. Waste Management 126, 674–683 (2021).

93.	Rehman, K. ur et al. Conversion of mixtures of dairy manure and soybean curd residue by black soldier fly larvae (Hermetia illucens L.). Journal of Cleaner Production 154, 366–373 (2017).

94.	Bekker, N. S. et al. Impact of substrate moisture content on growth and metabolic performance of black soldier fly larvae. Waste Management 127, 73–79 (2021).

95.	Heckmann, L.-H. L. & Gligorescu, A. Bioconversion of Guldborgsund Municipality’s Residual Sub- Strates into Insect Meal. 26 (2019).

96.	Roffeis, M. et al. Ex-ante life cycle impact assessment of insect-based feed production in West Africa. Agricultural Systems 178, 102710 (2020).

97.	Parodi, A. et al. Principles for the responsible use of farmed insects as livestock feed. Nat Food 1–3 (2022).

98.	Fowles, T. M. Using insects in conversion of agricultural by-products and applications of their value-added products. (University of California, Davis, 2022).

99.	Gold, M. et al. Biowaste treatment with black soldier fly larvae: Increasing performance through the formulation of biowastes based on protein and carbohydrates. Waste Manag. 102, 319–329 (2020).

100.	Amrul, N. F. et al. A Review of Organic Waste Treatment Using Black Soldier Fly (Hermetia illucens). Sustainability 14, 4565 (2022).

101.	Lindberg, L., Ermolaev, E., Vinnerås, B. & Lalander, C. Process efficiency and greenhouse gas emissions in black soldier fly larvae composting of fruit and vegetable waste with and without pre-treatment. Journal of Cleaner Production 130552 (2022).

102.	Shumo, M. et al. The nutritive value of black soldier fly larvae reared on common organic waste streams in Kenya. Sci Rep 9, 10110 (2019).

103.	Boakye-Yiadom, K. A., Ilari, A. & Duca, D. Greenhouse Gas Emissions and Life Cycle Assessment on the Black Soldier Fly (Hermetia illucens L.). Sustainability 14, 10456 (2022).

104.	El Hajj, R. et al. Edible Insects’ Transformation for Feed and Food Uses: An Overview of Current Insights and Future Developments in the Field. Processes 10, 970 (2022).

105.	Wehry, G.-J. J. E., Little, D. C., Newton, R. W. & Bostock, J. C. The feasibility of underutilised biomass streams for the production of insect-based feed ingredients: The case for whisky by-products and Scottish farmed salmon. Clean. Eng. Technol. 100520 (2022).

106.	Cottrell, R. S., Blanchard, J. L., Halpern, B. S., Metian, M. & Froehlich, H. E. Global adoption of novel aquaculture feeds could substantially reduce forage fish demand by 2030. Nat. Food 1, 301–308 (2020).

107.	FEDIAF. Animal proteins used in EU pet food. (2017).

108.	Thompson, A. Ingredients: Where Pet Food Starts. Topics in Companion Animal Medicine 23, 127–132 (2008).

109.	Bosch, G. & Swanson, K. s. Effect of using insects as feed on animals: pet dogs and cats. Journal of Insects as Food and Feed 7, 795–805 (2021).

110.	Arabzadeh, G. et al. Diet Composition Influences Growth Performance, Bioconversion of Black Soldier Fly Larvae: Agronomic Value and In Vitro Biofungicidal Activity of Derived Frass. Agronomy 12, 1765 (2022).

111.	ADEME, EVEA Evaluation et Accompagnement, S3D Ingénierie, Cornelus, M. & Rousseau El Habti, A. Impacts Environnementaux De Filieres De Traitements Biologiques Des Dechets De Cuisine Et De Table: Compostages Et Methanisations. 126 (2019).

112.	Kawasaki, K., Kawasaki, T., Hirayasu, H., Matsumoto, Y. & Fujitani, Y. Evaluation of Fertilizer Value of Residues Obtained after Processing Household Organic Waste with Black Soldier Fly Larvae (Hermetia illucens). Sustainability 12, 4920 (2020).

113.	Houben, D., Daoulas, G., Faucon, M.-P. & Dulaurent, A.-M. Potential use of mealworm frass as a fertilizer: Impact on crop growth and soil properties. Sci Rep 10, 4659 (2020).

114.	Lalander, C. H., Fidjeland, J., Diener, S., Eriksson, S. & Vinnerås, B. High waste-to-biomass conversion and efficient Salmonella spp. reduction using black soldier fly for waste recycling. Agron. Sustain. Dev. 35, 261–271 (2015).

115.	Zurbrugg, C., Dortmans, B., Fadhila, A., Vertsappen, B. & Diener, S. From Pilot to Full Scale Operation of a Waste-to-Protein Treatment Facility. Detritus vol. 1 18–22 (2018).

116.	Kröncke, N. et al. Automation of Insect Mass Rearing and Processing Technologies of Mealworms (Tenebrio molitor). in African Edible Insects As Alternative Source of Food, Oil, Protein and Bioactive Components (ed. Adam Mariod, A.) 123–139 (Springer International Publishing, Cham, 2020).

117.	Roffeis, M. et al. Life Cycle Inventory Analysis of Prospective Insect Based Feed Production in West Africa. Sustainability 9, 1697 (2017).

118.	Brockmann, D., Pradel, M. & Hélias, A. Agricultural use of organic residues in life cycle assessment: Current practices and proposal for the computation of field emissions and of the nitrogen mineral fertilizer equivalent. Resourc. Conserv. Recy. 133, 50–62 (2018).

119.	Van Zanten, H. H. E. et al. Defining a land boundary for sustainable livestock consumption. Glob. Change Biol. 24, 4185–4194 (2018).

120.	Weththasinghe, P., Hansen, J. Ø., Mydland, L. T. & Øverland, M. A systematic meta-analysis based review on black soldier fly (Hermetia illucens) as a novel protein source for salmonids. Reviews in Aquaculture 14, 938–956 (2022).

121.	Harris, E. E. Molasses and Yeast From Wood. US Dept. Agr. Yearbook 887–890 (1951).

122.	Chen, L., Upcraft, T., Piercy, E. & Guo, M. Spatially-explicit projection of future microbial protein from lignocellulosic waste. Curr. Res. Biotechnol. 4, 544–563 (2022).

123.	Derbyshire, E. J. & Delange, J. Fungal Protein – What Is It and What Is the Health Evidence? A Systematic Review Focusing on Mycoprotein. Frontiers in Sustainable Food Systems 5, (2021).

124.	Souza Filho, P. F., Andersson, D., Ferreira, J. A. & Taherzadeh, M. J. Mycoprotein: environmental impact and health aspects. World J Microbiol Biotechnol 35, 147 (2019).

125.	Durkin, A., Guo, M., Wuertz, S. & Stuckey, D. C. Resource recovery from food-processing wastewaters in a circular economy: a methodology for the future. Curr. Opin. Biotechnol. 76, 102735 (2022).

126.	Landeta Salgado, C., Muñoz, R., Blanco, A. & Lienqueo, M. E. Valorization and upgrading of the nutritional value of seaweed and seaweed waste using the marine fungi Paradendryphiella salina to produce mycoprotein. Algal Res. 53, 102135 (2021).

127.	Souza Filho, P. F., Brancoli, P., Bolton, K., Zamani, A. & Taherzadeh, M. J. Techno-Economic and Life Cycle Assessment of Wastewater Management from Potato Starch Production: Present Status and Alternative Biotreatments. Fermentation 3, 56 (2017).

128.	Atatoprak, T., Amorim, M. M., Ribeiro, T., Pintado, M. & Madureira, A. R. Grape stalk valorization for fermentation purposes. Food Chem (Oxf) 4, 100067 (2021).

129.	Bakar, N. A. A. et al. Structural modification of empty and partially filled paddy grain through hydrothermal pre-treatment as a novel substrate for fermentable glucose production. Starch - Stärke 2100299 (2022) doi:10.1002/star.202100299.

130.	Pihlajaniemi, V. et al. Comparison of pretreatments and cost-optimization of enzymatic hydrolysis for production of single cell protein from grass silage fibre. Bioresour. Technol. Rep. 9, 100357 (2020).

131.	Upcraft, T. et al. Protein from renewable resources: mycoprotein production from agricultural residues. Green Chem. 23, 5150–5165 (2021).

132.	Voutilainen, E., Pihlajaniemi, V. & Parviainen, T. Economic comparison of food protein production with single-cell organisms from lignocellulose side-streams. Bioresour. Technol. Rep. 14, 100683 (2021).

133.	Møller, H., Samsonstuen, S., Øverland, M., Modahl, I. S. & Olsen, H. F. Local non-food yeast protein in pig production - environmental impacts and land use efficiency. Livest. Sci. 104925 (2022).

134.	Throup, J. et al. Rapid repurposing of pulp and paper mills, biorefineries, and breweries for lignocellulosic sugar production in global food catastrophes. Food and Bioproducts Processing 131, 22–39 (2022).

135.	Davis, R. E. et al. Process Design and Economics for the Conversion of Lignocellulosic Biomass to Hydrocarbon Fuels and Coproducts: 2018 Biochemical Design Case Update; Biochemical Deconstruction and Conversion of Biomass to Fuels and Products via Integrated Biorefinery Pathways. NREL/TP--5100-71949, 1483234 http://www.osti.gov/servlets/purl/1483234/ (2018) doi:10.2172/1483234.

136.	Blanco, J., Iglesias, J., Morales, G., Melero, J. A. & Moreno, J. Comparative Life Cycle Assessment of Glucose Production from Maize Starch and Woody Biomass Residues as a Feedstock. Applied Sciences 10, 2946 (2020).

137.	Bello, S., Salim, I., Feijoo, G. & Moreira, M. T. Inventory review and environmental evaluation of first- and second-generation sugars through life cycle assessment. Environ. Sci. Pollut. Res. 28, 27345–27361 (2021).

138.	Fabbri, S., Owsianiak, M. & Hauschild, M. Z. Evaluation of sugar feedstocks for bio-based chemicals: A consequential, regionalized life cycle assessment. GCB Bioenergy 15, 72–87 (2023).

139.	Smetana, S., Mathys, A., Knoch, A. & Heinz, V. Sustainability of meat substitutes: a path to future foods? 7 (2015).

140.	Nonhebel, S. & Raats, J. Environmental impacts of meat substitutes: comparison between Quorn and pork. in 73–75 (2007).

141.	Potter, H. K., Lundmark, L. & Röös, E. Environmental Impact of Plant-Based Foods. 170 (2020).

142.	Risner, D., McDonald, K. A., Jones, C. & Spang, E. S. A techno-economic model of mycoprotein production: achieving price parity with beef protein. Front. Sustain. Food Syst. 7, (2023).

143.	Nair, R., Kabir, M., Lennartsson, P., Taherzadeh, M. & Horváth, I. Integrated Process for Ethanol, Biogas, and Edible Filamentous Fungi-Based Animal Feed Production from Dilute Phosphoric Acid-Pretreated Wheat Straw. Applied Biochemistry and Biotechnology 184, 48–62 (2018).

144.	Ivarson, K. C. & Morita, H. Single-Cell Protein Production by the Acid-Tolerant Fungus Scytalidium acidophilum from Acid Hydrolysates of Waste Paper. Appl Environ Microbiol 43, 643–647 (1982).

145.	Nature’s Fynd. A food company for optimists. https://www.naturesfynd.com/origin-story (2021).

146.	Ochsenreither, K., Glück, C., Stressler, T., Fischer, L. & Syldatk, C. Production Strategies and Applications of Microbial Single Cell Oils. Front. Microbiol. 7, 1539 (2016).

147.	Arbiom. Wood to Food. https://arbiom.com/ (2020).

148.	Fazenda, M., Johnston, C. & Mcneil, B. Bioprocess for coproduction of ethanol and mycoproteins. (2017).

149.	Omar, S. S., Merrifield, D. L., Kühlwein, H., Williams, P. E. V. & Davies, S. J. Biofuel derived yeast protein concentrate (YPC) as a novel feed ingredient in carp diets. Aquaculture 330–333, 54–62 (2012).

150.	Kim, S. R., Eckert, C. A. & Mazzoli, R. Editorial: Microorganisms for Consolidated 2nd Generation Biorefining. Front. Microbiol. 13, 940610 (2022).

151.	Nickel, D. B. Process development for platform chemical production from agricultural and forestry residues. (Chalmers University of Technology, Gothenburg, 2021).

152.	Asim, A. M., Uroos, M., Muhammad, N. & Hallett, J. P. Production of Food-Grade Glucose from Rice and Wheat Residues Using a Biocompatible Ionic Liquid. ACS Sustainable Chem. Eng. (2021).

153.	Lin, X., Jiang, K., Liu, X., Han, D. & Zhang, Q. Review on development of ionic liquids in lignocellulosic biomass refining. Journal of Molecular Liquids 359, 119326 (2022).

154.	Halder, P. et al. Progress on the pre-treatment of lignocellulosic biomass employing ionic liquids. Renewable and Sustainable Energy Reviews 105, 268–292 (2019).

155.	Abouelela, A. R. et al. Evaluating the Role of Water as a Cosolvent and an Antisolvent in [HSO 4 ]-Based Protic Ionic Liquid Pretreatment. ACS Sustainable Chem. Eng. 9, 10524–10536 (2021).

156.	Neupane, B., Konda, N. V. S. N. M., Singh, S., Simmons, B. A. & Scown, C. D. Life-Cycle Greenhouse Gas and Water Intensity of Cellulosic Biofuel Production Using Cholinium Lysinate Ionic Liquid Pretreatment. ACS Sustainable Chem. Eng. 5, 10176–10185 (2017).

157.	Zhang, J., Zou, D., Singh, S. & Cheng, G. Recent developments in ionic liquid pretreatment of lignocellulosic biomass for enhanced bioconversion. Sustainable Energy Fuels 5, 1655–1667 (2021).

158.	Zhang, J., Zhang, X., Yang, M., Singh, S. & Cheng, G. Transforming lignocellulosic biomass into biofuels enabled by ionic liquid pretreatment. Bioresource Technology 322, 124522 (2021).

159.	Ovejero-Pérez, A. et al. Technoeconomic Assessment of a Biomass Pretreatment + Ionic Liquid Recovery Process with Aprotic and Choline Derived Ionic Liquids. ACS Sustainable Chem. Eng. 9, 8467–8476 (2021).

160.	Su-ungkavatin, P., Tiruta-Barna, L. & Hamelin, L. Biofuels, electrofuels, electric or hydrogen?: A review of current and emerging sustainable aviation systems. Prog. Energy Combust. Sci. 96, 101073 (2023).

161.	Lopes, T. F., Duarte, L. C., Carvalheiro, F., Cardona, C. A. & Gírio, F. Production of Hemicellulosic Sugars from Residual Lignocellulosic Biomass in an Integrated Small-Scale Biorefinery: Techno-Economic and Life Cycle Assessments. in Hemicellulose Biorefinery: A Sustainable Solution for Value Addition to Bio-Based Products and Bioenergy (ed. Brienzo, M.) 71–109 (Springer, Singapore, 2022). doi:10.1007/978-981-16-3682-0_3.

162.	Kuo, P.-C. & Yu, J. Process simulation and techno-economic analysis for production of industrial sugars from lignocellulosic biomass. Industrial Crops and Products 155, 112783 (2020).

163.	Kemppainen, K., Rommi, K., Holopainen, U. & Kruus, K. Steam explosion of Brewer’s spent grain improves enzymatic digestibility of carbohydrates and affects solubility and stability of proteins. Appl Biochem Biotechnol 180, 94–108 (2016).

164.	Balkan, F., Colak, N. & Hepbasli, A. Performance evaluation of a triple-effect evaporator with forward feed using exergy analysis. International Journal of Energy Research 29, 455–470 (2005).

165.	Parajó, J. J., Villanueva, M., Troncoso, J. & Salgado, J. Thermophysical properties of choline and pyridinium based ionic liquids as advanced materials for energy applications. The Journal of Chemical Thermodynamics 141, 105947 (2020).

166.	Hosseini, S. M., Khosravi-Darani, K., Mohammadifar, M. A. & Nikoopour, H. Production of Mycoprotein by Fusarium venenatum Growth on Modified Vogel Medium. Asian J. Chem. 21, 4017–4022 (2009).

167.	Brassard, P., Godbout, S. & Hamelin, L. Framework for consequential life cycle assessment of pyrolysis biorefineries: A case study for the conversion of primary forestry residues. Renewable and Sustainable Energy Reviews 110549 (2020).

168.	Salim, I., González-García, S., Feijoo, G. & Moreira, M. T. Assessing the environmental sustainability of glucose from wheat as a fermentation feedstock. Journal of Environmental Management 247, 323–332 (2019).

169.	Permanasari, A. R., Yulistiani, F., Purnama, R. W., Widjaja, T. & Gunawan, S. The effect of substrate and enzyme concentration on the glucose syrup production from red sorghum starch by enzymatic hydrolysis. IOP Conf. Ser.: Earth Environ. Sci. 160, 012002 (2018).

170.	Sirohi, R., Prakash Pandey, J., Kumar Gaur, V., Gnansounou, E. & Sindhu, R. Critical overview of biomass feedstocks as sustainable substrates for the production of polyhydroxybutyrate (PHB). Bioresource Technology 311, 123536 (2020).

171.	Dunn, J. B., Mueller, S., Wang, M. & Han, J. Energy consumption and greenhouse gas emissions from enzyme and yeast manufacture for corn and cellulosic ethanol production. Biotechnol Lett 34, 2259–2263 (2012).

172.	Bello-Pérez, L. A., Roger, P., Baud, B. & Colonna, P. Macromolecular Features of Starches Determined by Aqueous High-performance Size Exclusion Chromatography. Journal of Cereal Science 27, 267–278 (1998).

173.	de Boer, H. C., van Krimpen, M. M., Blonk, H. & Tyszler, M. Replacement of Soybean Meal in Compound Feed by European Protein Sources. 47 (2014).

174.	von Stockar, U. & Liu, J.-S. Does microbial life always feed on negative entropy? Thermodynamic analysis of microbial growth. Biochimica et Biophysica Acta (BBA) - Bioenergetics 1412, 191–211 (1999).

175.	Leger, D. et al. Photovoltaic-driven microbial protein production can use land and sunlight more efficiently than conventional crops. Proc Natl Acad Sci USA 118, e2015025118 (2021).

176.	Whittaker, J. A., Johnson, R. I., Finnigan, T. J. A., Avery, S. V. & Dyer, P. S. The Biotechnology of Quorn Mycoprotein: Past, Present and Future Challenges. in Grand Challenges in Fungal Biotechnology (ed. Nevalainen, H.) 59–79 (Springer International Publishing, Cham, 2020). doi:10.1007/978-3-030-29541-7_3.

177.	Lonchamp, J., Akintoye, M., Clegg, P. S. & Euston, S. R. Functional fungal extracts from the Quorn fermentation co-product as novel partial egg white replacers. Eur Food Res Technol 246, 69–80 (2020).

178.	Ping, C. Y., Xi, F., Blank, I. & Yuan, L. Strategies to improve meat-like properties of meat analogs meeting consumers’ expectations. Biomaterials 121648 (2022).

179.	Øverland, M. & Skrede, A. Yeast derived from lignocellulosic biomass as a sustainable feed resource for use in aquaculture. J. Sci. Food Agric. 97, 733–742 (2017).

180.	Finnigan, T., Needham, L. & Abbott, C. Chapter 19. Mycoprotein: A Healthy New Protein With a Low Environmental Impact. 21 (2017).

181.	Ahmad, M. I., Farooq, S., Alhamoud, Y., Li, C. & Zhang, H. A review on mycoprotein: History, nutritional composition, production methods, and health benefits. Trends in Food Science & Technology 121, 14–29 (2022).

182.	Woern, C. & Grossmann, L. Microbial gas fermentation technology for sustainable food protein production. Biotechnol. Adv. 108240 (2023).

183.	Järviö, N., Maljanen, N.-L., Kobayashi, Y., Ryynänen, T. & Tuomisto, H. L. An attributional life cycle assessment of microbial protein production: a case study on using hydrogen-oxidizing bacteria. Sci. Total Environ. 145764 (2021).

184.	El Abbadi, S. H. Engineering the Dark Food Chain:the Role of Methanotrophic Bacteria Infood Production and Climate Change Mitigation. (Standford University, 2021).

185.	Cumberlege, T., Blenkinsopp, T. & Clark, J. Assessment of Environmental Footprint of FeedKind Protein. 26 (2016).

186.	Levett, I. et al. Techno-economic assessment of poly-3-hydroxybutyrate (PHB) production from methane—The case for thermophilic bioprocessing. J. Environ. Chem. Eng. 4, 3724–3733 (2016).

187.	Pikaar, I. et al. Decoupling Livestock from Land Use through Industrial Feed Production Pathways. Environ. Sci. Technol. 52, 7351–7359 (2018).

188.	Tallentire, C. W., Mackenzie, S. G. & Kyriazakis, I. Can novel ingredients replace soybeans and reduce the environmental burdens of European livestock systems in the future? J. Clean. Prod. 187, 338–347 (2018).

189.	Couture, J. L. et al. Environmental Benefits of Novel Nonhuman Food Inputs to Salmon Feeds. Environ. Sci. Technol. 53, 1967–1975 (2019).

190.	Sillman, J. et al. Bacterial protein for food and feed generated via renewable energy and direct air capture of CO2: Can it reduce land and water use? Glob. Food Sec. 22, 25–32 (2019).

191.	Sillman, J. et al. A life cycle environmental sustainability analysis of microbial protein production via power-to-food approaches. Int. J. Life Cycle Assess. 25, 2190–2203 (2020).

192.	Spiller, M. et al. Environmental impact of microbial protein from potato wastewater as feed ingredient: Comparative consequential life cycle assessment of three production systems and soybean meal. Water Res. 171, 115406 (2020).

193.	Khoshnevisan, B. et al. Environmental life cycle assessment of different biorefinery platforms valorizing municipal solid waste to bioenergy, microbial protein, lactic and succinic acid. Renewable and Sustainable Energy Reviews 117, 109493 (2020).

194.	Matassa, S. et al. Upcycling of biowaste carbon and nutrients in line with consumer confidence: the “full gas” route to single cell protein. Green Chem. 22, 4912–4929 (2020).

195.	Verbeeck, K., Vrieze, J. D., Pikaar, I., Verstraete, W. & Rabaey, K. Assessing the potential for up-cycling recovered resources from anaerobic digestion through microbial protein production. Microb. Biotechnol. 14, 897–910 (2020).

196.	Nappa, M. et al. Solar-Powered Carbon Fixation for Food and Feed Production Using Microorganisms—A Comparative Techno-Economic Analysis. ACS Omega 5, 33242–33252 (2020).

197.	García Martínez, J. B. et al. Potential of microbial protein from hydrogen for preventing mass starvation in catastrophic scenarios. Sustain. Prod. Consum. 25, 234–247 (2021).

198.	García Martínez, J. B. et al. Methane Single Cell Protein: Potential to Secure a Global Protein Supply Against Catastrophic Food Shocks. Front. Bioeng. Biotechnol. 10, (2022).

199.	Marami, H. et al. Bridging to circular bioeconomy through a novel biorefinery platform on a wastewater treatment plant. Renew. Sust. Energ. Rev. 154, 111895 (2022).

200.	Li, R. et al. From anaerobic digestion to single cell protein synthesis: A promising route beyond biogas utilization. Water Research 243, 120417 (2023).

201.	Vlaeminck, E. et al. Single-Cell Protein Production from Industrial Off-Gas through Acetate: Techno-Economic Analysis for a Coupled Fermentation Approach. Fermentation 9, 771 (2023).

202.	Jean, A. B. & Brown, R. C. Techno-Economic Analysis of Gas Fermentation for the Production of Single Cell Protein. Environ. Sci. Technol. 58, 3823–3829 (2024).

203.	Huizing, H. J. Single Cell Protein (SCP) als alternatief voor soja: een haalbaarheidsstudie. http://edepot.wur.nl/116725 (2005).

204.	Alvarado, K. A., García Martínez, J. B., Matassa, S., Egbejimba, J. & Denkenberger, D. Food in space from hydrogen-oxidizing bacteria. Acta Astronaut. 180, 260–265 (2021).

205.	Nyyssölä, A. et al. Production of Endotoxin-Free Microbial Biomass for Food Applications by Gas Fermentation of Gram-Positive H2-Oxidizing Bacteria. ACS Food Sci. Technol. (2021) doi:10.1021/acsfoodscitech.0c00129.

206.	Ruuskanen, V. et al. Neo-Carbon Food concept: A pilot-scale hybrid biological–inorganic system with direct air capture of carbon dioxide. Journal of Cleaner Production 278, 123423 (2021).

207.	Acosta, N. et al. Microbial protein production from methane via electrochemical biogas upgrading. Chem. Eng. J. 391, 123625 (2020).

208.	Reed, J., Geller, J. & McDaniel, R. CO2 Conversion by Knallgas Microorganism - Evaluation of Products and Processes. 133 https://pdfslide.net/documents/co2-conversion-by-knallgas-microorganism-pilot-plant-description-66.html (2017).

209.	Pelagalli, V. et al. Syngas-driven sewage sludge conversion to microbial protein through H2S- and CO-tolerant hydrogen-oxidizing bacteria. Water Research 120698 (2023).

210.	Reed, J. S. Biological and Chemical Process Utilizing Chemoautotrophic Microorganisms for the Chemosynthetic Fixation of Carbon Dioxide and/or Other Inorganic Carbon Sources into Organic Compounds and the Generation of Additional Useful Products. (2019).

211.	Salehi, R. & Chaiprapat, S. Conversion of biogas from anaerobic digestion to single cell protein and bio-methanol: mechanism, microorganisms and key factors - A review. Environmental Engineering Research 27, 210109–0 (2021).

212.	Wang, D.-H. et al. Conversion of Renewable Biogas into Single-Cell Protein Using a Combined Microalga- and Methane-Oxidizing Bacterial System. ACS EST Eng. (2022) doi:10.1021/acsestengg.2c00237.

213.	Steinberg, L. M., Kronyak, R. E. & House, C. H. Coupling of anaerobic waste treatment to produce protein- and lipid-rich bacterial biomass. Life Sci. Space Res. 15, 32–42 (2017).

214.	Zha, X. et al. Bioconversion of wastewater to single cell protein by methanotrophic bacteria. Bioresource Technology 320, 124351 (2021).

215.	Xu, M. Beyond the farm: Bioinorganic electro- synthesis of single cell protein from CO2 and green electricity. (Technical University of Denmark, 2021).

216.	Christiaens, M. E. R. et al. Electrochemical Ammonia Recovery from Source-Separated Urine for Microbial Protein Production. Environ. Sci. Technol. 51, 13143–13150 (2017).

217.	Kerckhof, F.-M. et al. From Biogas and Hydrogen to Microbial Protein Through Co-Cultivation of Methane and Hydrogen Oxidizing Bacteria. Front. Bioeng. Biotechnol. 9, 733753 (2021).

218.	Zeng, D. et al. Recycling of acetate and ammonium from digestate for single cell protein production by a hybrid electrochemical-membrane fermentation process. Resources, Conservation and Recycling 188, 106705 (2023).

219.	Patthawaro, S. & Saejung, C. Production of single cell protein from manure as animal feed by using photosynthetic bacteria. MicrobiologyOpen 8, e913 (2019).

220.	Tsapekos, P., Khoshnevisan, B., Zhu, X., Zha, X. & Angelidaki, I. Methane oxidising bacteria to upcycle effluent streams from anaerobic digestion of municipal biowaste. J. Environ. Manage. 251, 109590 (2019).

221.	Yang, X., Xu, M., Zou, R., Angelidaki, I. & Zhang, Y. Microbial protein production from CO2, H2, and recycled nitrogen: focusing on ammonia toxicity and nitrogen sources. J. Clean. Prod. 125921 (2021).

222.	Hu, X. et al. Microbial Protein out of Thin Air: Fixation of Nitrogen Gas by an Autotrophic Hydrogen-Oxidizing Bacterial Enrichment. Environ. Sci. Technol. 54, 3609–3617 (2020).

223.	Megret, O. et al. Production d’hydrogène à partir de déchets: Etat de l’art et potentiel d’émergence. 228 https://hal.archives-ouvertes.fr/hal-01602165 (2015).

224.	Sakarika, M., Ganigué, R. & Rabaey, K. Methylotrophs: from C1 compounds to food. Curr. Opin. Biotechnol. 75, 102685 (2022).

225.	Peplow, M. The race to upcycle CO2 into fuels, concrete and more. Nature 603, 780–783 (2022).

226.	Claassens, N. J., Sousa, D. Z., dos Santos, V. A. P. M., de Vos, W. M. & van der Oost, J. Harnessing the power of microbial autotrophy. Nat. Rev. Microbiol. 14, 692–706 (2016).

227.	Mishra, A., Ntihuga, J. N., Molitor, B. & Angenent, L. T. Power-to-Protein: Carbon Fixation with Renewable Electric Power to Feed the World. Joule 4, 1142–1147 (2020).

228.	Hann, E. C. et al. A hybrid inorganic–biological artificial photosynthesis system for energy-efficient food production. Nat Food 3, 461–471 (2022).

229.	Wada, O. Z., Vincent, A. S. & Mackey, H. R. Single-cell protein production from purple non-sulphur bacteria-based wastewater treatment. Rev Environ Sci Biotechnol 21, 931–956 (2022).

230.	Alloul, A. et al. Capture–Ferment–Upgrade: A Three-Step Approach for the Valorization of Sewage Organics as Commodities. Environ. Sci. Technol. 52, 6729–6742 (2018).

231.	Calysta. The Future of Nutrition. https://www.calysta.com/ (2020).

232.	Unibio. Unibio. https://www.unibio.dk/ (2020).

233.	Kiverdi, Inc. We Are Remaking How Things Are Made With Carbon Transformation. https://www.kiverdi.com (2020).

234.	Solar Foods. Food Out of Thin Air. https://solarfoods.fi/ (2020).

235.	Øverland, M., Tauson, A.-H., Shearer, K. & Skrede, A. Evaluation of methane-utilising bacteria products as feed ingredients for monogastric animals. Arch. Anim. Nutr. 64, 171–89 (2010).

236.	El Abbadi, S. H. & Criddle, C. S. Engineering the Dark Food Chain. Environ. Sci. Technol. 53, 2273–2287 (2019).

237.	Ren, J., Liu, Y.-L., Zhao, X.-Y. & Cao, J.-P. Methanation of syngas from biomass gasification: An overview. International Journal of Hydrogen Energy 45, 4223–4243 (2020).

238.	Rafrafi, Y., Laguillaumie, L. & Dumas, C. Biological Methanation of H2 and CO2 with Mixed Cultures: Current Advances, Hurdles and Challenges. Waste Biomass Valor 12, 5259–5282 (2021).

239.	van der Giesen, C., Kleijn, R. & Kramer, G. J. Energy and Climate Impacts of Producing Synthetic Hydrocarbon Fuels from CO 2. Environ. Sci. Technol. 48, 7111–7121 (2014).

240.	Danish Energy Agency. Technology Data - Renewable Fuels. 313 http://www.ens.dk/teknologikatalog (2022).

241.	Zhang, X., Bauer, C., Mutel, C. L. & Volkart, K. Life Cycle Assessment of Power-to-Gas: Approaches, system variations and their environmental implications. Applied Energy 190, 326–338 (2017).

242.	Lee, B. et al. Integrative techno-economic and environmental assessment for green H2 production by alkaline water electrolysis based on experimental data. Journal of Environmental Chemical Engineering 9, 106349 (2021).

243.	European Commission. Commission Delegated Regulation (EU) 2021/2139 of 4 June 2021 Supplementing Regulation (EU) 2020/852 of the European Parliament and of the Council by Establishing the Technical Screening Criteria for Determining the Conditions under Which an Economic Activity Qualifies as Contributing Substantially to Climate Change Mitigation or Climate Change Adaptation and for Determining Whether That Economic Activity Causes No Significant Harm to Any of the Other Environmental Objectives (Text with EEA Relevance). OJ L vol. 442 (2021).

244.	Palmer, G., Roberts, A., Hoadley, A., Dargaville, R. & Honnery, D. Life-cycle greenhouse gas emissions and net energy assessment of large-scale hydrogen production via electrolysis and solar PV. Energy Environ. Sci. 14, 5113–5131 (2021).

245.	Battista, F. et al. LCA evaluation for the hydrogen production from biogas through the innovative BioRobur project concept. International Journal of Hydrogen Energy 42, 14030–14043 (2017).

246.	Schreiber, A., Peschel, A., Hentschel, B. & Zapp, P. Life Cycle Assessment of Power-to-Syngas: Comparing High Temperature Co-Electrolysis and Steam Methane Reforming. Front. Energy Res. 8, 533850 (2020).

247.	Pham Minh, D. et al. Hydrogen Production From Biogas Reforming: An Overview of Steam Reforming, Dry Reforming, Dual Reforming, and Tri-Reforming of Methane. in Hydrogen Supply Chains 111–166 (Elsevier, 2018).

248.	Cvetković, S. M., Radoičić, T. K., Kijevčanin, M. & Novaković, J. G. Life Cycle Energy Assessment of biohydrogen production via biogas steam reforming: Case study of biogas plant on a farm in Serbia. International Journal of Hydrogen Energy 46, 14130–14137 (2021).

249.	Di Marcoberardino, G., Liao, X., Dauriat, A., Binotti, M. & Manzolini, G. Life Cycle Assessment and Economic Analysis of an Innovative Biogas Membrane Reformer for Hydrogen Production. Processes 7, 86 (2019).

250.	Xu, J. et al. Embracing a low-carbon future by the production and marketing of C1 gas protein. Biotechnol. Adv. 63, 108096 (2023).

251.	Ministère de la Transition écologique et solidaire. Consommation d’énergie par usage du résidentiel. Données et études statistiques pour le changement climatique, l’énergie, l’environnement, le logement et les transports https://www.statistiques.developpement-durable.gouv.fr/consommation-denergie-par-usage-du-residentiel (2022).

252.	Xu, M., Zhou, H., Yang, X., Angelidaki, I. & Zhang, Y. Sulfide restrains the growth of Methylocapsa acidiphila converting renewable biogas to single cell protein. Water Res. 184, 116138 (2020).

253.	Meraz, J. L., Dubrawski, K. L., El Abbadi, S. H., Choo, K.-H. & Criddle, C. S. Membrane and Fluid Contactors for Safe and Efficient Methane Delivery in Methanotrophic Bioreactors. J. Environ. Eng. 146, 03120006 (2020).

254.	Lorick, D., Macura, B., Ahlström, M., Grimvall, A. & Harder, R. Effectiveness of struvite precipitation and ammonia stripping for recovery of phosphorus and nitrogen from anaerobic digestate: a systematic review. Environmental Evidence 9, 27 (2020).

255.	Guilayn, F., Jimenez, J., Monlau, F. & Vaneeckhaute, C. Valorisation of Anaerobic Digestate: Towards Value-Added Products. in Renewable Energy Technologies for Energy Efficient Sustainable Development (eds. Sinharoy, A. & Lens, P. N. L.) 227–262 (Springer International Publishing, Cham, 2022).

256.	Spiller, M., Moretti, M., De Paepe, J. & Vlaeminck, S. E. Environmental and economic sustainability of the nitrogen recovery paradigm: Evidence from a structured literature review. Resour. Conserv. Recycl. 184, 106406 (2022).

257.	Matassa, S. et al. How can we possibly resolve the planet’s nitrogen dilemma? Microb. Biotechnol. 16, 15–27 (2023).

258.	Waslien, C. I., Calloway, D. H. & Margen, S. Human Intolerance to Bacteria as Food. Nature 221, 84–85 (1969).

259.	Sustainable Downstream Processing of Microalgae for Industrial Application. (CRC Press, Boca Raton, 2019). doi:10.1201/9780429027970.

260.	Garcia, N. H. et al. Evaluation of the methane potential of different agricultural and food processing substrates for improved biogas production in rural areas. Renewable and Sustainable Energy Reviews 112, 1–10 (2019).

261.	Bareha, Y., Affes, R., Moinard, V., Buffet, J. & Girault, R. A simple mass balance tool to predict carbon and nitrogen fluxes in anaerobic digestion systems. Waste Manag. 135, 47–59 (2021).

262.	Matassa, S., Verstraete, W., Pikaar, I. & Boon, N. Autotrophic nitrogen assimilation and carbon capture for microbial protein production by a novel enrichment of hydrogen-oxidizing bacteria. Water Res. 101, 137–146 (2016).

263.	Areniello, M., Matassa, S., Esposito, G. & Lens, P. N. L. Biowaste upcycling into second-generation microbial protein through mixed-culture fermentation. Trends Biotechnol. 0, (2022).

264.	Pander, B. et al. Hydrogen oxidising bacteria for production of single-cell protein and other food and feed ingredients. Eng. Biol. 4, 21–24 (2020).

265.	Le Chatelier, D. Luzerne Références 2019-2021. (COOP de France Déshydratation, Paris, France, 2019).

266.	Andurand, J., Coulmier, D., Despres, J. L. & Rambourg, J. C. Extraction industrielle de protéines et de pigments chez la luzerne: état des lieux et perspectives. 147–156 https://hal.inrae.fr/hal-02658324/document (2010).

267.	EFSA. Opinion on the safety of ‘Alfalfa protein concentrate’ as food. EFS2 7, (2009).

268.	Krenz, L. M. M. & Pleissner, D. Valorization of landscape management grass. Biomass Conv. Bioref. (2022) doi:10.1007/s13399-022-02568-0.

269.	Hermansen, J. E. et al. Green Biomass - Protein Production through Biorefining. 72 https://dcapub.au.dk/djfpublikation/djfpdf/dcarapport093.pdf (2017).

270.	Sanders, J. P. M. & Koopmans, S. Method and system for the production of non-ruminant animal feed. (2020).

271.	Nynäs, A.-L. Harnessing the potential of green leaves : agricultural biomass as a source of sustainable. (Swedish University of Agricultural Science, Alnarp, 2022).

272.	Tamayo Tenorio, A., Schreuders, F. K. G., Zisopoulos, F. K., Boom, R. M. & van der Goot, A. J. Processing concepts for the use of green leaves as raw materials for the food industry. J. Clean. Prod. 164, 736–748 (2017).

273.	Kiskini, A., Vissers, A., Vincken, J.-P., Gruppen, H. & Wierenga, P. A. Effect of Plant Age on the Quantity and Quality of Proteins Extracted from Sugar Beet (Beta vulgaris L.) Leaves. J. Agric. Food Chem. 64, 8305–8314 (2016).

274.	Carlsson, R. & Hanczakowski, P. Waste green parts of plants as raw material for leaf protein concentrate production. Biological Wastes 28, 83–90 (1989).

275.	Prade, T. et al. Protein fractionation of broccoli (Brassica oleracea, var. Italica) and kale (Brassica oleracea, var. Sabellica) residual leaves — A pre-feasibility assessment and evaluation of fraction phenol and fibre content. FBP 130, 229–243 (2021).

276.	Gundersen, E., Christiansen, A. H. C., Jørgensen, K. & Lübeck, M. Production of leaf protein concentrates from cassava: Protein distribution and anti-nutritional factors in biorefining fractions. Journal of Cleaner Production 379, 134730 (2022).

277.	Rosas Romero, A. J. & Acosta, X. The biological quality of a cassava leave protein concentrate. Acta cient. venez 441–4 (1986).

278.	Mortensen, E. ø & Jørgensen, U. Danish agricultural biomass production and utilization in 2030. in Advisory memorandum from DCA - Danish Center for Food and Agriculture 24 (Aarhus University, 2022).

279.	Nielsen, C. K., Stødkilde, L., Jørgensen, U. & Lærke, P. E. Effects of Harvest and Fertilization Frequency on Protein Yield and Extractability From Flood-Tolerant Perennial Grasses Cultivated on a fen Peatland. Front. Environ. Sci. 9, (2021).

280.	Muica, V. Enhanced Management of Residual Grass in Denmark. (Syddansk Universitet, 2014).

281.	Tamayo Tenorio, A., Gieteling, J., de Jong, G. A. H., Boom, R. M. & van der Goot, A. J. Recovery of protein from green leaves: Overview of crucial steps for utilisation. Food Chemistry 203, 402–408 (2016).

282.	Skunca, D., Romdhana, H. & Brouwers, R. Rubisco protein production - LCA approach. MEST 9, 175–183 (2021).

283.	Parajuli, R., Dalgaard, T. & Birkved, M. Can farmers mitigate environmental impacts through combined production of food, fuel and feed? A consequential life cycle assessment of integrated mixed crop-livestock system with a green biorefinery. Sci. Total Environ. 619–620, 127–143 (2018).

284.	Parajuli, R. et al. Environmental impacts of producing bioethanol and biobased lactic acid from standalone and integrated biorefineries using a consequential and an attributional life cycle assessment approach. Science of The Total Environment 598, 497–512 (2017).

285.	Prieler, M., Lindorfer, J. & Steinmüller, H. Life-cycle assessment of green biorefinery process options. Biofuels, Bioproducts and Biorefining 13, 1391–1401 (2019).

286.	Jungbluth, N. et al. Life Cycle Inventories of Bioenergy. Ecoinvent Report No. 17. (Swiss Centre for Life Cycle Inventories, Dübendorf, CH, 2007).

287.	Bühle, L., Stülpnagel, R. & Wachendorf, M. Comparative life cycle assessment of the integrated generation of solid fuel and biogas from biomass (IFBB) and whole crop digestion (WCD) in Germany. Biomass and Bioenergy 35, 363–373 (2011).

288.	Bühle, L., Hensgen, F., Donnison, I., Heinsoo, K. & Wachendorf, M. Life cycle assessment of the integrated generation of solid fuel and biogas from biomass (IFBB) in comparison to different energy recovery, animal-based and non-refining management systems. Bioresource Technology 111, 230–239 (2012).

289.	Corona, A., Parajuli, R., Ambye-Jensen, M., Hauschild, M. Z. & Birkved, M. Environmental screening of potential biomass for green biorefinery conversion. J. Clean. Prod. 189, 344–357 (2018).

290.	Corona, A., Ambye-Jensen, M., Vega, G. C., Hauschild, M. Z. & Birkved, M. Techno-environmental assessment of the green biorefinery concept: Combining process simulation and life cycle assessment at an early design stage. Sci. Total Environ. 635, 100–111 (2018).

291.	Kamp, A., Ambye-Jensen, M. & Østergård, H. Modelling matter and energy flows of local, refined grass-clover protein feed as alternative to imported soy meal. Ecological Modelling 410, 108738 (2019).

292.	Djomo, S. N. et al. Green proteins: An energy-efficient solution for increased self-sufficiency in protein in Europe. Biofuels, Bioproducts and Biorefining 14, 605–619 (2020).

293.	Muneer, F. et al. Economic viability of protein concentrate production from green biomass of intermediate crops: A pre-feasibility study. J Clean Prod 126304 (2021).

294.	Höltinger, S., Schmidt, J., Schönhart, M. & Schmid, E. A spatially explicit techno-economic assessment of green biorefinery concepts. Biofuels, Bioproducts and Biorefining 8, 325–341 (2014).

295.	O’Keeffe, S., Schulte, R. P. O., Sanders, J. P. M. & Struik, P. C. I. Technical assessment for first generation green biorefinery (GBR) using mass and energy balances: Scenarios for an Irish GBR blueprint. Biomass Bioenergy. 35, 4712–4723 (2011).

296.	O’Keeffe, S., Schulte, R. P. O., Sanders, J. P. M. & Struik, P. C. II. Economic assessment for first generation green biorefinery (GBR): Scenarios for an Irish GBR blueprint. Biomass Bioenergy. 41, 1–13 (2012).

297.	Tampio, E., Winquist, E., Luostarinen, S. & Rinne, M. A farm-scale grass biorefinery concept for combined pig feed and biogas production. Water Science and Technology 80, 1042–1052 (2019).

298.	Chan, S. W. S. et al. Environmental impacts of a novel biorefinery platform integrated with power-to-protein technology to decrease dependencies on soybean imports. Science of The Total Environment 907, 167943 (2024).

299.	Thiébeau, P., Auberger, J., Clivot, H., Wilfart, A. & Recous, S. Environmental footprint of dehydrated alfalfa production (Medicago sativa L.) in France. Journal of Cleaner Production 423, 138704 (2023).

300.	Kiskini, A., Zondervan, E., Wierenga, P. A., Poiesz, E. & Gruppen, H. Using product driven process synthesis in the biorefinery. Comput Chem Eng . 91, 257–268 (2016).

301.	Nissen, S. H., Lübeck, M., Møller, A. H. & Dalsgaard, T. K. Protein recovery and quality of alfalfa extracts obtained by acid precipitation and fermentation. Bioresource Technology Reports 19, 101190 (2022).

302.	Pérez-Vila, S., Fenelon, M. A., O’Mahony, J. A. & Gómez-Mascaraque, L. G. Extraction of plant protein from green leaves: Biomass composition and processing considerations. Food Hydrocolloids 107902 (2022).

303.	Kerfai, S. et al. Procédé de production de la rubisco à partir d’une biomasse végétale. (2020).

304.	Martens, T., Flores, C., MORENO, E. R. & MOLEN, R. V. D. Process for isolating a high purity protein preparation from plant material and products thereof. (2021).

305.	De Jong, G. A. H., Geerdink, P., Bussmann, P. J. T. & HYLKEMA, N. Economical process for the isolation of functional protein from plants. (2019).

306.	Tamayo Tenorio, A., Kyriakopoulou, K. E., Suarez-Garcia, E., van den Berg, C. & van der Goot, A. J. Understanding differences in protein fractionation from conventional crops, and herbaceous and aquatic biomass - Consequences for industrial use. Trends Food Sci. Technol. 71, 235–245 (2018).

307.	Møller, A. H. et al. Biorefinery of Green Biomass─How to Extract and Evaluate High Quality Leaf Protein for Food? J. Agric. Food Chem. (2021).

308.	Ait Sair, A., Kansou, K., Michaud, F. & Cathala, B. Multicriteria Definition of Small-Scale Biorefineries Based on a Statistical Classification. Sustainability 13, 7310 (2021).

309.	McEniry, J., Finnan, J., King, C. & O’Kiely, P. The effect of ensiling and fractionation on the suitability for combustion of three common grassland species at sequential harvest dates. Grass and Forage Science 67, 559–568 (2012).

310.	Wachendorf, M., Richter, F., Fricke, T., Graß, R. & Neff, R. Utilization of semi-natural grassland through integrated generation of solid fuel and biogas from biomass. I. Effects of hydrothermal conditioning and mechanical dehydration on mass flows of organic and mineral plant compounds, and nutrient balances. Grass and Forage Science 64, 132–143 (2009).

311.	Franco, M., Hurme, T., Winquist, E. & Rinne, M. Grass silage for biorefinery—A meta-analysis of silage factors affecting liquid–solid separation. Grass and Forage Science 74, 218–230 (2019).

312.	Guo, X. et al. Extraction of protein from juice blend of grass and clover pressed by a pilot pressing facility combined with a pulsed electric field treatment. Future Foods 100173 (2022).

313.	FoodNorthWest. Water and Wastewater Use in the Food Processing Industry. https://www.foodnorthwest.org/index.php?option=com_content&view=article&id=83%3Awater-and-wastewater-use-in-the-food-processing-industry&catid=20%3Asite-content&showall=1&id=83:water-and-wastewater-use-in-the-food-processing-industry (2020).

314.	Damborg, V. K., Jensen, S. K., Weisbjerg, M. R., Adamsen, A. P. & Stødkilde, L. Screw-pressed fractions from green forages as animal feed: Chemical composition and mass balances. Anim. Feed Sci. Technol. 261, 114401 (2020).

315.	Santamaría-Fernández, M., Ytting, N. K., Lübeck, M. & Uellendahl, H. Potential Nutrient Recovery in a Green Biorefinery for Production of Feed, Fuel and Fertilizer for Organic Farming. Waste Biomass Valor. 11, 5901–5911 (2020).

316.	Santamaría-Fernández, M., Karkov Ytting, N. & Lübeck, M. Influence of the development stage of perennial forage crops for the recovery yields of extractable proteins using lactic acid fermentation. Journal of Cleaner Production 218, 1055–1064 (2019).

317.	Kamm, B., Schönicke, P. & Kamm, M. Biorefining of Green Biomass - Technical and Energetic Considerations. Clean Soil Air Water 37, 27–30 (2009).

318.	Meeting the Future Demands for Grassland Production. (Natural Resources Institute Finland (Luke), 2020).

319.	Rinne, M. et al. Fibrolytic enzyme treatment prior to ensiling increased press-juice and crude protein yield from grass silage. Bioresource Technology 299, 122572 (2020).

320.	Santamaría-Fernández, M., Ambye‐Jensen, M., Damborg, V. K. & Lübeck, M. Demonstration-scale protein recovery by lactic acid fermentation from grass clover – a single case of the production of protein concentrate and press cake silage for animal feeding trials. Biofuels, Bioproducts and Biorefining 13, 502–513 (2019).

321.	Stefański, T. et al. Grass silage for biorefinery – separation efficiency and aerobic stability of silage and solid fraction. in Proceedings of the 9th Nordic Feed Science Conference (2018).

322.	Bühle, L., Hensgen, F., Donnison, I., Heinsoo, K. & Wachendorf, M. Evaluation of fuel and biogas production on typical semi-natural grasslands in Germany, Wales and Estonia. Grassland farming and land management systems in mountainous regions. Proceedings of the 16th Symposium of the European Grassland Federation, Gumpenstein, Austria, 29th-31st August, 2011 398–400 (2011).

323.	Bitra, V. S. P., Womac, A. R., Igathinathane, C. & Sokhansanj, S. Knife Mill Comminution Energy Analysis of Switchgrass, Wheat Straw, and Corn Stover and Characterization of Particle Size Distributions. Transactions of the ASABE 53, 1639–1651 (2010).

324.	Kamm, B., Hille, C., Schönicke, P. & Dautzenberg, G. Green biorefinery demonstration plant in Havelland (Germany). Biofuels, Bioproducts and Biorefining 4, 253–262 (2010).

325.	Santamaría-Fernández, M. et al. Lactic acid fermentation for refining proteins from green crops and obtaining a high-quality feed product for monogastric animals. J Clean Prod 162, 875–881 (2017).

326.	Presto Åkerfeldt, M., Friman, J., Dahlström, F., Larsen, A. & Wallenbeck, A. Juice from silage in green bio refineries – a potential feed ingredient in liquid diets to weaned pigs. Acta Agriculturae Scandinavica, Section A — Animal Science 0, 1–7 (2022).

327.	Ayyat, M. S., Abdel‐Rahman, G., Ayyat, A. M. N., Abdel‐Rahman, M. S. & Al‐Sagheer, A. A. Evaluation of leaf protein concentrate from Beta vulgaris and Daucus carota as a substitute for soybean meal in Oreochromis niloticus fingerlings diets. Aquaculture Research are.15171 (2021) doi:10.1111/are.15171.

328.	Stødkilde, L., Damborg, V. K., Jørgensen, H., Lærke, H. N. & Jensen, S. K. Digestibility of fractionated green biomass as protein source for monogastric animals. Animal 13, 1817–1825 (2019).

329.	Stødkilde, L., Lashkari, S., Eriksen, J. & Jensen, S. K. Enhancing protein recovery in green biorefineries through selection of plant species and time of harvest. Animal Feed Science and Technology 278, 115016 (2021).

330.	Stødkilde, L., Ambye-Jensen, M. & Jensen, S. K. Biorefined organic grass-clover protein concentrate for growing pigs: effect on growth performance and meat fatty acid profile. Animal Feed Science and Technology 114943 (2021).

331.	Ravindran, R. et al. Biogas, Biomethane and Digestate Potential of By-Products from Green Biorefinery Systems. Clean Technol. 4, 35–50 (2022).

332.	Zoppi, G., Andrade, T. A., Ward, A. J., Ambye-Jensen, M. & Biller, P. Biorefinery integration of a green protein platform for maximum resource utilization. Cleaner and Circular Bioeconomy 6, 100064 (2023).

333.	Thomsen, M. H., Bech, D. & Kiel, P. Manufacturing of Stabilised Brown Juice for L-lysine production – from University Lab Scale over Pilot Scale to Industrial Production. Chem. Biochem. Eng. Q. 10 (2004).

334.	Schoeters, F. et al. Microalgal cultivation on grass juice as a novel process for a green biorefinery. Algal Research 69, 102941 (2023).

335.	Jørgensen, U., Jensen, S. K. & Ambye-Jensen, M. Coupling the benefits of grassland crops and green biorefining to produce protein, materials and services for the green transition. Grass Forage Sci. 77, 295–306 (2022).

336.	Thunman, H. et al. Advanced biofuel production via gasification – lessons learned from 200 man-years of research activity with Chalmers’ research gasifier and the GoBiGas demonstration plant. Energy Sci. Eng. 6, 6–34 (2018).

337.	Kreuger, E., Nges, I. A. & Björnsson, L. Ensiling of crops for biogas production: effects on methane yield and total solids determination. Biotechnology for Biofuels 4, 44 (2011).

338.	Gavrilova, O. et al. Chapter 10 - Emissions from livestock and manure management. in Agriculture, Forestry and Other Land Use vol. 4 209 (IPCC, 2019).

339.	Ardolino, F. & Arena, U. Biowaste-to-Biomethane: An LCA study on biogas and syngas roads. Waste Management 87, 441–453 (2019).

340.	Dong, J. et al. Life cycle assessment of pyrolysis, gasification and incineration waste-to-energy technologies: Theoretical analysis and case study of commercial plants. Science of The Total Environment 626, 744–753 (2018).

341.	Gagliano, A., Nocera, F., Patania, F., Bruno, M. & Castaldo, D. G. A robust numerical model for characterizing the syngas composition in a downdraft gasification process. Comptes Rendus Chimie 19, 441–449 (2016).

342.	Hongrapipat, J., Saw, W.-L. & Pang, S. Removal of ammonia from producer gas in biomass gasification: integration of gasification optimisation and hot catalytic gas cleaning. Biomass Conv. Bioref. 2, 327–348 (2012).

343.	Azzi, E. S., Karltun, E. & Sundberg, C. Assessing the diverse environmental effects of biochar systems: An evaluation framework. Journal of Environmental Management 286, 112154 (2021).

344.	Peters, J. F., Iribarren, D. & Dufour, J. Biomass Pyrolysis for Biochar or Energy Applications? A Life Cycle Assessment. Environ. Sci. Technol. 49, 5195–5202 (2015).

345.	Joseph, S. et al. How biochar works, and when it doesn’t: A review of mechanisms controlling soil and plant responses to biochar. GCB Bioenergy 13, 1731–1764 (2021).

346.	He, Y. et al. Effects of biochar application on soil greenhouse gas fluxes: a meta-analysis. GCB Bioenergy 9, 743–755 (2017).

347.	Andrade, C., Albers, A., Zamora-Ledezma, E. & Hamelin, L. The interplay between bioeconomy and the maintenance of long-term soil organic carbon stock in agricultural soils: A systematic review. Renew. Sust. Energ. Rev. 189, 113890 (2023).

348.	Hergoualc’h, K. et al. Chapter 11 - N2O emissions from managed soils, and CO2 emissions from lime and urea application. in Agriculture, Forestry and Other Land Use vol. 4 48 (IPCC, 2019).

349.	Farhangi-Abriz, S. et al. Biochar effects on yield of cereal and legume crops using meta-analysis. Science of The Total Environment 775, 145869 (2021).

350.	Agreste. Infographics - Farming France. Ministère de l’Agriculture et de l’Alimentation https://agriculture.gouv.fr/infographics-farming-france (2021).

351.	Abdul Azeez, K. T., P, S., C, M. & P, A. Aspen Plus Simulation of Biomass Gasification: a Comprehensive Model Incorporating Reaction Kinetics, Hydrodynamics and Tar Production. Process Integr Optim Sustain (2022).

352.	Javourez, U. Transforming residual biomass into food and feed, towards an LCA optimization platform. (INSA Toulouse, Toulouse, 2023).