Supplementary Information for

NeuroWorm: a Long-term Implantable Soft Microfibre for Multimodal Biosensing

Ruijie Xie1,2†, Qianhengyuan Yu1†, Dong Li3†, Xu Han2, Xiaolong Xu2, Jianping Huang1, Xiaomeng Zhou1, Hang Zhao1, Xinping Deng1, Qiong Tian1, Qingsong Li1, Hanfei Li1, Hairong Zheng1, Guanglin Li1, Meifang Zhu4, Wei Yan4*, Fei Han1*, Tiantian Xu3*, Zhiyuan Liu1*

Corresponding author: Zhiyuan Liu, zy.liu1@siat.ac.cn; Tiantian Xu, tt.xu@siat.ac.cn; Fei Han, fei.han@siat.ac.cn; Wei Yan, weiyan@dhu.edu.cn

The PDF file includes:

The code for processing electromyography (EMG) signals

Other Supplementary information for this manuscript include the following:

Supplementary Video 1.

Fabrication Process of NeuroWorm

Supplementary Video 2.

Implantation of NeuroWorm in an artificial brain under magnetic field

Supplementary Video 3.

Minimally-invasive implantation process of the NeuroWorm

Supplementary Video 4.

Long-term implantable monitoring of intramuscular EMG during movement

The code for processing intramuscular electromyography (EMG) signals is as follows, including filtering the collected EMG signal, plotting the EMG graph, taking a screenshot of a portion of the EMG graph, and processing the central frequency value of the segmented EMG spectrogram:

fs = 2000;

filterRanges = [1 10; 49.5 50.5; 149.5 150.5; 249.5 250.5; 349.5 350.5; 449.5 450.5; 500 1000];

signalIndex = 27;

startpoint = fix(40.83*2e3);

endpoint = fix(42.83*2e3-1);

signal = tableData{:, signalIndex};

subsignal = tableData{startpoint:endpoint, signalIndex};

sosMatrix = [];

gainVector = [];

for i = 1:size(filterRanges, 1)

filt_temp = designfilt('bandstopiir', 'FilterOrder', 4, 'HalfPowerFrequency1', filterRanges(i, 1), 'HalfPowerFrequency2', filterRanges(i, 2), 'SampleRate', fs);

[num, den] = tf(filt_temp);

[sos_temp, gain_temp] = tf2sos(num, den);

sosMatrix = [sosMatrix; sos_temp];

gainVector = [gainVector; gain_temp];

end

filt = dfilt.df2sos(sosMatrix, gainVector);

filteredsubSignal = filter(filt, subsignal);

filteredSignal = filter(filt, signal);

figure;

sublt = 1/fs:1/fs:size(subsignal, 1)/fs;

plot(sublt, filteredsubSignal);

axis([0 max(sublt) -2 2])

set(gca, 'XTick', 0:1:max(sublt))

set(gca, 'YTick', -2:1:2)

title(['Signal ', num2str(signalIndex)]);

ylabel('EMG signal(mv)');

xlabel('t(s)');

figure;

N = length(filteredsubSignal);

freq = (0:N/2-1) * fs / N;

spectrum = abs(fft(filteredsubSignal)/N);

plot(freq, spectrum(1:N/2));

title([' Spectrum plot ', num2str(signalIndex)]);

xlabel('Frequency (Hz)');

ylabel('Amplitude (mV)');

figure;

lt = 1/fs:1/fs:size(signal, 1)/fs;

plot(lt, filteredSignal);

axis([0 max(lt) -2 2])

set(gca, 'XTick', 0:5:max(lt))

set(gca, 'YTick', -2:1:2)

title(['Signal ', num2str(signalIndex)]);

ylabel('EMG signal(mv)');

xlabel('t(s)');

Y = fft(filteredsubSignal);

P2 = abs(Y/N);

P1 = P2(1:N/2+1);

P1(2:end-1) = 2*P1(2:end-1);

f = fs*(0:(N/2))/N;

AF_CF = sum(f'.*P1)/sum(P1);