Supplementary Material for:

Environmental implications of the production of non-centrifugal sugar in the Colombian Andean region

Nestor Sanchez1*, Martha Cobo1, David Rodríguez-Fontalvo1, Ruth Y. Ruiz-Pardo2, Anne Roedl3,

1Energy, Materials, and Environmental Laboratory, Faculty of Engineering, Campus Universitario Puente del Común, Universidad de La Sabana, Km. 7 Autopista Norte, Bogotá 140013, Colombia; davidrodfo@unisabana.edu.co  (D.R.F), martha.cobo@unisabana.edu.co (M.C)

2Grupo de Investigación en Procesos Agroindustriales, Faculty of Engineering, Campus Universitario Puente del Común, Universidad de La Sabana, Km. 7 Autopista Norte, Bogotá 140013, Colombia; Ruth.ruiz@unisabana.edu.co

3Hamburg University of Technology (TUHH), Institute of Environmental Technology and Energy Economics (IUE), Eissendorfer straße 40, D-2107, Hamburg, Germany; anne.roedl@gmail.com

*Correspondence: nestor.sanchez1@unisabana.edu.co

1. Life cycle inventory for the harvesting stage

Information associated with soil quality, fertilizing requirements, and irrigation was retrieved directly from farmers by using interviews. The sugarcane variety employed in the facility is ZC. This variety yields 216 t/ha with a crop period of 16 months (i.e., about 163 t/ha*year) (Ramírez-Durán et al., 2014). According to interviews, the availability of land is 23 ha corresponding to inceptisol and andisol soils as stated by the United States Department of Agriculture (USDA) soil taxonomy. Therefore, the clay content of this soil is about 33.8 ± 17.04%, the average content of phosphorous is 32.4 ± 54.88 ppm, calcium 2.89 ± 3.00 g/kg, magnesium 0.51 ± 0.52 g/kg, aluminum 0.37 ± 0.63 g/kg, and sodium 0.18 ± 0.43 g/kg (Araujo-Carrillo et al., 2021). According to Gonzáles Chavarro et al., (2018), the absorption of nitrogen for sugarcane is 279.2 ± 26.5 kg N/ha, for phosphorous is 42.6 ± 2.56 kg/ha, for potassium is 296.4 ± 44.4 kg/ha, for calcium is 73.7 ± 11.6 kg/ha, and for magnesium is 31.2 ± 4.15 kg/ha. Organic fertilizers such as leaves, buds, manure, and sugarcane press-mud are employed to supply the macronutrients and micronutrients. Based on the interviews, 768 t of sugarcane is yielded annually in the facility. Hence, annually 4.72 ha are harvested.  Moreover, the environmental impacts associated with air, soil, and water emissions due to manure management and fertilizers applications were calculated according to the Intergovernmental Panel on Climate Change (IPCC) guidelines (Dong et al., 2006; Hergoualch et al., 2019; Lasco et al., 2006; Metz et al., 2005) as shown below.

Manure is the main fertilizer in the production facility. According to the interview, the facility has five swine, eighty poultries, and six mules. The latter are employed for transporting the sugarcane from the harvest area to the facility. Emissions associated with animals are divided into enteric fermentation and manure management. In the former case, enteric fermentation produces methane as the sole emission and it was calculated according to Eq (1) where mCH4EF is the amount of methane produced through enteric fermentation (kg CH4/a), GET is the gross energy intake per animal T (MJ/head/day), Ym,T is the methane conversion factor that depends on the animal T, and NT is the total animal T available in the facility (Nemecek et al., 2014). For manure management, Tier 1 of IPCC method was employed as an approach to calculate methane emissions, as shown in Eq. (2) where EFT is the emission factor for manure management that depends on the animal and the region temperature. The emission factors for swine, poultry, and mules were 1, 0.02, and 0.9 kg CH4/head/a, respectively (Dong et al., 2006).

mCH4EF=TGET*Ym,T*36555.45*NT	 						Eq. (1)

CH4,manure=TEFT*NT								Eq. (2)

Manure obtained from the animals is directly deposited in the grass as fertilizer after its management. Eq. (3) was employed to calculate the amount of indirect emissions of N2O where N2O(ATD)-N is the annual amount of N2O-N produced from atmospheric deposition of N volatilized from managed soils (kg N2O-N/year), FON is the annual amount of managed animal manure, compost, and other organic nitrogen source applied to soil (kg N/year), FRACGASM is the fraction of applied organic N fertilizer materials that volatilizes as NH3 and NOx (kg N volatilized/kg N deposited), FPRP is the annual amount of urine and dung deposited by grazing animals (kg N/animal), and EF1 is the emission factor for N2O emissions from atmospheric deposition of N on soils (Hergoualch et al., 2019). In this case, three organic nitrogen sources are employed (i.e., leaves, sugarcane press mud, and manure). FON was calculated based on the amount of residue and the nitrogen content, whose values are shown in Table 2.

N2O(ATD)-N=(FON+FPRP)*FRACGASM*EF1 					Eq. (3)

Table A.1. Production of residues in the local non-centrifugal sugar industry

| Residue | Amount of the residue (t/year) | Manure production (kg/head*year) | Nitrogen (%) | Reference |
| --- | --- | --- | --- | --- |
| Leaves and buds | 211.6 | NA | 0.84 | (Angel Riveros, 2012) |
| Sugarcane press-mud | 29.9 | NA | 1.12 | This study |
| Swine manure | 3.99 | 799 | 0.70 | (Dadrasnia et al., 2021; Sagastume Gutiérrez et al., 2022) |
| Poultry manure | 3.04 | 38 | 3.0 | (Dadrasnia et al., 2021; Sagastume Gutiérrez et al., 2022) |
| Mule manure | 21.9 | 3650 | 0.70 | (Dadrasnia et al., 2021; Mitiku Teferra and Wubu, 2019) |

Direct emissions of N2O were calculated according to Eq. (4) where N2ODirect-N is the annual direct N2O-N emissions produced from managed soils, FON is the annual amount of animal manure and other organic additions applied to soils (kg N/year), EF2 is the emissions factor whose value is 0.010 kg N2O-N/kg N, FPRP,i is the annual amount of urine and dung deposited by grazing animals (kg N/year), and EFi is the emission factor for N2O emissions from urine and dung deposited on pasture (kg N2O-N/kg N). This emission factor is 0.004 for swine and poultry and 0.003 for mules (Hergoualch et al., 2019). FPRP is calculated according to Eq. (5) where N(T) is the number of head of livestock species T in the facility, Nrate,(T) is the annual average N excretion per head of species in the facility (kg N/animal/a), MST,PRP is the fraction of total amount of nitrogen excretion for each specie T that is managed in manure management system S in the country that is deposited on pasture. Said value is 0% for swine and poultry, and 100% for mules. TAM is the average weight of animals whose values for swine are 218 kg, while for poultry and mules are 1.9 and 250 kg, respectively (Moeletsi and Tongwane, 2015).

N2O(D)-N=FON*EF2+iFPRP,i*EFi						 Eq. (4)

FPRP=T365*NT*TAM*Nrate,T1000*MST,PRP 					 Eq. (5)

The N2O emissions from leaching and run-off were calculated according to Eq. (6) where FRACLEACH-H is the fraction of the leached nitrogen (0.24 kg losses by leaching/kg N applied) and EF3 is the emission factor for N2O emissions from N leaching and runoff (0.011 kg N2O-N/kg leaching) (Hergoualch et al., 2019).

N2O(L)-N=FON+FPRP*FRACLeach-H*EF3					  Eq. (6)

The application of nitrogen to soil results in the leaching of nitrates (NO3) which is calculated according to Eq. (7) where P is the precipitation (mm/a) (see Table A1), c is the clay content (%), L is the rooting depth (1.6 m), S is the nitrogen supply through fertilizer, Norg is the nitrogen in organic matter in the soil, and U is the nitrogen uptake by crop in kg N/ha. Eq. (8) calculates the organic matter in the soil that depends on the soil volume (V=5,000 m3/ha), bulk density (db = 900 kg/m3), C/N ratio (17.0), the nitrogen organic ratio (rorg = 0.825), and the content of organic carbon (Corg = 5.00%) (Nemecek et al., 2019). Data were retrieved for Colombian conditions (Araujo-Carrillo et al., 2021; Velasquez et al., 2005).

NO3kgha=21.37+Pc*L*0.0037*S+0.0000601*Norg-0.00362*U 	             Eq. (7)

Norg=(Corg100)*V*dBCN*rorg									Eq. (8)

Ammonia is also emitted due to livestock in the facility. Ammonia emissions could be calculated according to Eq. (9) where FON is the total amount of nitrogen from organic sources (kg N/a), FPRP is the amount of nitrogen in the urine, TAN is the proportion of ammonia in the manure, and EF4 is the emission factor that depends on the animal. For instance, EF4 for swine, poultry, and mules is 0.25, 0.09, and 0.35, respectively. Besides the TAN is 70% for swine and poultry, and 60% for mules (Nemecek et al., 2014). Furthermore, nitrogen emissions such as N2 are also considered for nitrogen balance. N2 emissions (kg N2/year) are calculated according to Eq. (10) where EF5 is the emission factor (0.3 kg N2/TAN).

NH3kgyear=FON+FPRP*TAN*EF4*1714					Eq. (9)

N2kgyear=[FON+FPRP*TAN*EF5 						Eq. (10)

Ammonia from crop residues was also considered and it was calculated according to Eq. (11) where A is the area of the crop (Ha), FN is the amount of nitrogen in the crop residue (kg N/ha), EF6 is the emission factor that is calculated as shown in Eq. (12). In this case N content refers to the nitrogen content in the crop residue (g/kg dry matter) (Vonk et al., 2016). Nitrogen oxides (NOx) were calculated according to Eq. (13).

NH3kgyear=A*FN*EF6*1714							Eq. (11)

EF6=0.40*NContent-5.08 								Eq. (12)

NOxkgyear=0.012*(FON+Fprp)							Eq. (13)

Particulate matter (PM) is formed through the atmospheric oxidation of NOx and SOx causing a lot of human health problems (Franceschi et al., 2018; Hopke et al., 2020). According to Vonk et al., (2016), PM is formed during soil cultivation, harvesting, cleaning, and drying in wet climates. The humidity in the municipality of Villeta is about 85%. Hence, we consider PM formation, and it was calculated as shown in Eq. (14) where A is the crop area (Ha), n is the number of times that operations are carried out, and EF7 is the emission factor whose values are 0.25 kg/ha for PM10 and 0.015 for PM2.5.

PMxkg PMyear=A*n*EF7								Eq. (14)

Phosphorous emissions also occurred in the facility due to the application of manure and crop residues that are commonly employed for fertilizing. In this case, three kinds of phosphorous were considered: 1) leaching of soluble phosphate to groundwater (Eq. (15)); 2) run-off of soluble phosphate to surface water (Eq. (16)); and water erosion of soil particles containing phosphorous (Eq. (17)) (Nemecek et al., 2014).

Pgw=Pgwl*Fgw		 							Eq. (15)

Pro=Prol*Fro									Eq. (16)

Per=Ser*Pcs*Fr*Ferw								Eq. (17)

In Eq. (15), Pgw is the amount of phosphorous leached to groundwater (kg/ha*a), and Pgwl is the average quantity of P leached to groundwater in a land category (kg P/ha*a). In this case, we considered 0.06, whose value applied to permanent crops. Fgw is the correction factor for fertilization by slurry or sewage sludge. Herein, sugarcane press-mud was considered as slurry and the correction factor is 1.01. In Eq. (16), Pro is the quantity of phosphorous lost through run-off to rivers (kg/ha*a), and Prol is the average quantity of P lost through run-off for a specific land. In this case, we consider extensive permanent whose value is 0.15 kg/ha*year, Fro is the correction factor for fertilization which is calculated according to Eq. (18) where P2O5min, P2O5sl, and P2O5man is the quantity of P2O5 in mineral fertilizer, slurry, and solid manure (kg/Ha), respectively (Nemecek et al., 2014). In this case, the amount of phosphorus from manure was retrieved from the literature, and its values were 8 kg/ton for swine, 25 kg/ton for poultry, and 5 kg/ton for mules (Dadrasnia et al., 2021).

Fro=1+0.280*P2O5min+0.780*P2O5sl+0.480*P2O5man				Eq. (18)

In Eq. (17), Per is the quantity of phosphorous emitted through erosion to rivers (kg P/ha*year), Ser is the quantity of soil eroded (kg/ha*year), Pcs is the content of phosphorous in the top soil (kg P/kg soil) whose value is 682.94 mg/kg (Araujo-Carrillo et al., 2021), Fr is the enrichment factor for phosphorous whose value is 1.86, and Ferw is the fraction of eroded soil that reaches the river whose value is 0.2. The quantity of soil eroded (Ser) is calculated according to Eq. (19) where R is the erosivity factor (MJ*mm/ha*h*a), k is the erodibility factor (t*h/MJ*mm), LS is the slope factor, C1 is the crop factor, C2 is the tillage factor, and P is the practice factor (Nemecek et al., 2014). The erosivity factor depends on the precipitation and was calculated according to Eq. (19). In this case P is the precipitation factor whose value was not corrected since this region does not snow. The erodibility factor was 0.04, which is allocated to inceptisol soils. The crop factor for sugarcane is 0.3, the tillage factor is 0.25 since tillage is not employed in the facility, and the practice factor is 1 due to anti-erosion practices that are not employed in the facility (Emmenegger et al., 2009). The slope factor (LS) was calculated according to Eq. (20). This factor depends on the slope length (L = 60 m) and inclination (S = 3%).

R=587.8-1.219*P+0.00415*P2						Eq. (19)

LS=L*3.2803072.60.3*65.41*sinS100 2+4.56*sinS100+0.065 		Eq. (20)

Irrigation demand was estimated through the TURC method, as shown in Eq. (21), where ET0 is the reference evapotranspiration (mm/month), Rs is the solar radiation (MJ/m2), and Tmean is the average air temperature. This method calculates the water balance based on the amount of water available in the soil for crop use, accounts for the water removal through evapotranspiration, and the water added to the soil through rainfall and irrigation (Fisher and Iii, 2013; Kumar Rai et al., 2017). Irrigation was calculated according to Eq. (22), where ETc is the water demand by the crop and Pe is the effective rainfall. Finally, ETc was estimated according to Eq. (23), where Kc is the crop factor of sugarcane.

ET0=0.40*TmeanTmean+15Rs+50 							Eq. (21)

Irrigation= ETc-Pe									Eq. (22)

ETC=ET0*Kc										Eq. (23)

Carbon sequestration by the crop was calculated according to Eq. (24) where wi is the fraction of carbohydrates, proteins, lipids, fibers, and ash in the sugarcane. C is the content of carbon of these fractions whose values are 440, 530, 750, 440, and 0 g C/kg dry matter, respectively (Cañon et al., 2022; Nemecek and Kägi, 2007). We considered both carbon sequestration by sugarcane and for leaves and buds.

CO2,uptake=wi*C*4412								Eq. (24)

2. Land Use Change (LUC)

Permanent croplands can emit or remove carbon depending on crop type, management practices, soil, and climate variations. However, when cropland was present for 20 years or longer, LUC emissions are not significant (Lam et al., 2021). LUC emissions, in this case, were solely attributed to soil drainage, and it is calculated according to Eq. 25 where A is the land area, and EFc is the emission factor that depends on the climate. In this case, this value is 20 t C/ha*year (Lasco et al., 2006).

Lorg=A*EFc*4412 									Eq. (25)

3. Emission factors for fuel combustion

Table A.2. Emission factors for different fuels employed in the facility based on Fedepanela and Aspen Plus simulations

| Emission | Unit | Diesel1 | Bagasse1 | Bagasse2 | Bagasse+Guazuma2 |
| --- | --- | --- | --- | --- | --- |
| CO2 | kg/t fuel | 75 | 732 | 1,340.2 | 1,341.8 |
| NOx | g/ t fuel | 206 | 2446 | 0.0105 | 0.01028 |
| CO | g/ t fuel | 15 | 24.4 | 0.00005 | 0.00005 |
| CH4 | g/ t fuel | 3.1 | 733.8 | 0 | 0 |
| NMVOC | g/ t fuel | 5.15 | 1223 | NA | NA |
| PM | g/ t fuel | 19.2 | 2583 | NA | NA |
| SOx | g/ t fuel | NR | NR | 0.00107 | 0.00106 |

NMVOC: non-methane volatile compounds; PM: Particulate matter; NA

1Data obtained from (Fedepanela, 2015).

2Data obtained from Aspen Plus simulation.

References

Angel Riveros, M., 2012. Aprovechamiento de subproductos de la caña panelera en la alimentación animal [WWW Document]. Fedepanela. URL http://www.fedepanela.org.co/publicaciones/cartillas/Subproductos.pdf (accessed 2.11.19).

Araujo-Carrillo, G.A., Varón-Ramírez, V.M., Jaramillo-Barrios, C.I., Estupiñan-Casallas, J.M., Silva-Arero, E.A., Gómez-Latorre, D.A., Martínez-Maldonado, F.E., 2021. IRAKA: The first Colombian soil information system with digital soil mapping products. Catena 196. https://doi.org/10.1016/j.catena.2020.104940

Cañon, C., Sanchez, N., Cobo, M., 2022. Sustainable production of ethyl levulinate by levulinic acid esterification obtained from Colombian rice straw. J. Clean. Prod. 377. https://doi.org/10.1016/j.jclepro.2022.134276

Dadrasnia, A., de Bona Muñoz, I., Yáñez, E.H., Lamkaddam, I.U., Mora, M., Ponsá, S., Ahmed, M., Argelaguet, L.L., Williams, P.M., Oatley-Radcliffe, D.L., 2021. Sustainable nutrient recovery from animal manure: A review of current best practice technology and the potential for freeze concentration. J. Clean. Prod. 315, 128106. https://doi.org/10.1016/j.jclepro.2021.128106

Dong, H., Mangino, J., McAllister, T.A., Hatfield, J.L., Johnson, D.E., Bartram, D., Gibb, D., Martin, J.H., 2006. Emissions from livestock and manure management, in: Eggleston, S., Buendia, L., Miwa, K., Ngara, T., Tanabe, K. (Eds.), IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4. Institute of Global Environmental Strategies, Japan, p. 87.

Emmenegger, M.F., Reinhard, J., Zah, R., 2009. Sustainability Quick Check for Biofuels Background Report. Dübendorf. https://doi.org/10.13140/RG.2.1.5000.8167

Fedepanela, 2015. Nota De Información De La Nama (Nino) Reconversión Productiva Y Tecnológica Del Subsector Panelero.

Fisher, D.K., Iii, H.C.P., 2013. Evaluation of alternative methods for estimating reference evapotranspiration. Agric. Sci. 4, 51–60.

Franceschi, F., Cobo, M., Figueredo, M., 2018. Discovering relationships and forecasting PM10 and PM2.5 concentrations in Bogotá Colombia, using Artificial Neural Networks, Principal Component Analysis, and k-means clustering. Atmos. Pollut. Res. 9, 912–922. https://doi.org/10.1016/j.apr.2018.02.006

Gonzáles Chavarro, F., Cabezas Gutiérrez, M., Ramírez-Gómez, M., Ramírez Durán, J., 2018. Macronutrient absorption curves in three varieties of sugarcane (Saccharum officinarum L.) for Panela in the Hoya del Río Suarez. Rev. U.D.C.A Actual. Divulg. Científica 21, 395–404.

Hergoualch, K., Akiyama, H., Bernoux, M., Chirinda, N., Prado, A. del, Kasimir, A., MacDonald, J.D., Ogle, S.M., Regina, K., van der Weerden, T.J., 2019. N2O Emissions From Managed Soils, and CO2 Emissions From Lime and Urea Application, in: 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. p. 48.

Hopke, P.K., Dai, Q., Li, L., Feng, Y., 2020. Global review of recent source apportionments for airborne particulate matter. Sci. Total Environ. 740, 140091. https://doi.org/10.1016/j.scitotenv.2020.140091

Kumar Rai, R., Singh, V.P., Upadhyay, A., 2017. Irrigation scheduling, in: Planning and Evaluation of Irrigation Projects. pp. 385–412. https://doi.org/10.1016/B978-0-12-811748-4.00012-1

Lam, W.Y., Chatterton, J., Sim, S., Kulak, M., Mendoza Beltran, A., Huijbregts, M.A.J., 2021. Estimating greenhouse gas emissions from direct land use change due to crop production in multiple countries. Sci. Total Environ. 755, 143338. https://doi.org/10.1016/j.scitotenv.2020.143338

Lasco, R.D., Ogle, S., Raison, J., Verchot, L., Wassmann, R., Yagi, K., 2006. Cropland, in: 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4: Agriculture, Forestry and Other Land Use. p. 66.

Metz, B., Davidson, O., Coninck, H. de, Loos, M., Meyer, L., 2005. IPCC Special Report on Carbon Dioxide Capture and Storage, Intergovernmental Panel on Climate Change. https://doi.org/10.1016/bs.ache.2021.10.005

Mitiku Teferra, D., Wubu, W., 2019. Biogas for Clean Energy, in: Anaerobic Digestion. IntechOpen, pp. 1–20. https://doi.org/10.5772/intechopen.79534

Moeletsi, M.E., Tongwane, M.I., 2015. 2004 methane and nitrous oxide emissions from manure management in South Africa. Animals 5, 193–205. https://doi.org/10.3390/ani5020193

Nemecek, T., Bengoa, X., Lansche, J., Mouron, P., Rossi, V., Humbert, S., 2014. Methodological Guidelines for the Life Cycle Inventory of Agricultural Products. Lausanne and Zurich, Switzerland.

Nemecek, T., Kägi, T., 2007. Life Cycle Inventories of Agricultural Production Systems.

Nemecek, T., Lansche, J., Mouron, P., Rossi, V., 2019. World Food LCA Database Methodological World Food LCA Database Methodological Guidelines for the Life Cycle. Lausanne and Zurich, Switzerland.

Ramírez-Durán, J., Insuasty-Burbano, O., Murcia-Pardo, M., 2014. Variedad de caña de azúcar empleadas para la agroindustria panelera de Colombia. Barbosa, Santander.

Sagastume Gutiérrez, A., Mendoza Fandiño, J.M., Cabello Eras, J.J., Sofan German, S.J., 2022. Potential of livestock manure and agricultural wastes to mitigate the use of firewood for cooking in rural areas. The case of the department of Cordoba (Colombia). Dev. Eng. 7. https://doi.org/10.1016/j.deveng.2022.100093

Velasquez, E., Lavelle, P., Barrios, E., Joffre, R., Reversat, F., 2005. Evaluating soil quality in tropical agroecosystems of Colombia using NIRS. Soil Biol. Biochem. 37, 889–898. https://doi.org/10.1016/j.soilbio.2004.09.009

Vonk, J., Bannink, A., van Bruggen, C., Groenestein, C.M., Huijsmans, J.F.M., van der Kolk, J.W.H., Luesink, H.H., Oude Voshaar, S.V., Sluis, S.M., Velthof, G.L., 2016. Methodology for estimating emissions from agriculture in the Netherlands.