Supplementary Materials

Estimating the carbon footprint of post-flood urban road network restoration efforts: A case study in Carlisle

Table S1. Variables in the integrated model

| Classification | Variable | Item |  |  | Value (Range) | Unit | Data source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Structural | Thickness | Main road | Surface course |  | 40 | mm | (Lu 2020) |
|  |  |  | Binder course |  | 120 | mm |  |
|  |  |  | Base course |  | 150 | mm |  |
|  |  |  | Sub-base course |  | 600 | mm |  |
|  |  | Low-volume road | Surface course |  | 40 | mm |  |
|  |  |  | Binder course |  | 80 | mm |  |
|  |  |  | Base course |  | 150 | mm |  |
|  |  |  | Sub-base course |  | 400 | mm |  |
|  | Diesel consumption rate | Clearing and grubbing | Bulldozer |  | 0.083 | L / m2 | (Chong and Wang 2017) |
|  |  | Cut | Excavator |  | 0.113 | L / m3 |  |
|  |  | Subgrade preparation | Motor grader |  | 0.002 | L / m2 |  |
|  |  |  | Water sprayer |  | 0.0007 | L / m2 |  |
|  |  |  | Soil compactor |  | 0.03 | L / m2 |  |
|  |  | Subgrade treatment | Water sprayer |  | 0.0014 | L / m2 |  |
|  |  | Granular base | Motor grader |  | 0.0089 | L / m3 |  |
|  |  | Binder course | Asphalt paver |  | 0.34 | L / m3 |  |
|  |  |  | Asphalt compactor |  | 0.3 | L / m3 |  |
|  |  | Tack coat | Bitumen spreader |  | 0.03 | L / m3 |  |
| Parameterized | Width | Main road |  |  | 12 (10~15) | m | (Darren 2017) |
|  |  | Low-volume road |  |  | 8 (6~10) | m |  |
|  | Density | Asphalt concrete |  |  | 2460 (2200~2500) | kg/m3 | (Lu 2020) |
|  |  | Unbound material |  |  | 2000 (1800~2200) | kg/m3 |  |
|  | Carbon emission coefficient | Asphalt concrete |  |  | 10.3 (9.27~11.33)* | kgCO2e/t | (OHMPA 2010) *The range is ±10% of the typical value |
|  |  | Unbound material type A |  |  | 8 (7.2~8.8) | kgCO2e/t |  |
|  |  | Unbound material type B |  |  | 5.3 (4.77~5.83) | kgCO2e/t |  |
| Parameterized | Loading capacity | Rigid short |  |  | 14 | t | (TARMAC 2017) |
|  |  | Rigid long |  |  | 28 | t |  |
|  |  | Articulated |  |  | 28 | t |  |
|  | Delivery distance | Asphalt concrete |  |  | 53.31 (39.05~69.4) | km | Survey of suppliers |
|  |  | Unbound material |  |  | 53.34 (41.23~71.18) | km |  |
|  | Carbon emission coefficient | Rigid (7.5~17t) |  | 100% laden | 0.71483 | kgCO2e/km | (BEIS 2022) **The increase in the carbon emission coefficient of HGVs is directly proportional to the load weight, and the ratio is the growth rate. |
|  |  |  |  | 0% laden | 0.55763 | kgCO2e/km |  |
|  |  |  |  | Growth rate** | 0.1572 | - |  |
|  |  | Rigid (>17t) |  | 100% laden | 1.13291 | kgCO2e/km |  |
|  |  |  |  | 0% laden | 0.79097 | kgCO2e/km |  |
|  |  |  |  | Growth rate | 0.3419 | - |  |
|  |  | Articulated (3.5~33t) |  | 100% laden | 0.96112 | kgCO2e/km |  |
|  |  |  |  | 0% laden | 0.64531 | kgCO2e/km |  |
|  |  |  |  | Growth rate | 0.3158 | - |  |
|  | Carbon emission coefficient | Average biofuel blend |  |  | 2.56 | kgCO2e/L |  |
|  |  | 100% mineral diesel |  |  | 2.7 | kgCO2e/L |  |

Figure S1. Cross-sectional structures of (a) main roads and (b) low-volume roads

Reference

Chong D, Wang Y (2017) Impacts of flexible pavement design and management decisions on life cycle energy consumption and carbon footprint. Int J Life Cycle Assess 22(6):952–971. https://doi.org/10.1007/s11367-016-1202-x

Darren C (2017) How wide are roads? In: Highw. Code Resour. https://mocktheorytest.com/resources/how-wide-are-roads/. Accessed 26 Feb 2024

Department for Business, Energy & Industrial Strategy (2022) Greenhouse gas reporting: conversion factors 2022

Lu D (2020) Pavement Flooding Risk Assessment and Management in the Changing Climate. Doctoral Thesis, University of Waterloo

Ontario Hot Mix Producers Association (OHMPA)) (2010) Carbon Footprint: How does asphalt stack up? Ontario Hot Mix Producers Association

TARMAC (2017) Delivery Vehicle Options