Table A.1 Soil and sediment sample results within the Bennett Brook watershed. All

concentrations are in μg/kg dry weight. Bolded samples were collected at the point

source (barn site).

| Sample ID | p,p'-DDT | p,p'-DDE | ΣDDT | Location (decimal degrees) | Sediment (SD) or Soil (S) Sample | Collection Date | Sampling depth (cm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B10 | BRL | 4.1 | 4.57 | 43.781511, -71.552551 | SD | 4/1/2019 | 0-4 |  |
| B11 | ND | 7 | 7.8 | 43.778167, -71.551104 | SD | 4/1/2019 | 0-4 |  |
| B12 | ND | 4.8 | 5.35 | 43.780648, -71.552477 | SD | 4/1/2019 | 0-4 |  |
| B13 | 2.2 | 12.2 | 15.8 | 43.781629, -71.552892 | SD | 4/1/2019 | 0-4 |  |
| B16 | ND | ND | 0 | 43.782473, -71.55463 | SD | 5/29/2019 | 0-4 |  |
| B17 | ND | BRL | BRL | 43.78241, -71.554205 | SD | 5/29/2019 | 0-4 |  |
| B18 | ND | 11.3 | 12.6 | 43.782574, -71.554125 | SD | 5/29/2019 | 0-4 |  |
| B19 | ND | BRL | BRL | 43.782385, -71.554117 | SD | 5/29/2019 | 0-4 |  |
| B20 | ND | BRL | BRL | 43.782147, -71.553883 | SD | 5/29/2019 | 0-4 |  |
| B21 | ND | 16.4 | 18.28 | 43.781905, -71.553462 | SD | 5/29/2019 | 0-4 |  |
| B22 | ND | 24.4 | 27.2 | 43.781574, -71.552939 | SD | 5/29/2019 | 0-4 |  |
| B23 | ND | 15.1 | 16.83 | 43.781941, -71.553427 | SD | 5/29/2019 | 0-4 |  |
| B25 | ND | 18.6 | 20.73 | 43.777912, -71.550434 | SD | 5/29/2019 | 0-4 |  |
| B26 | ND | BRL | 0.11 | 43.782686, -71.554825 | S | 6/27/2019 | 0-4 |  |
| B27 | 3.9 | 14.8 | 20.4 | 43.782387, -71.554452 | S | 6/27/2019 | 0-4 |  |
| B28 | 6.8 | 37.9 | 49.05 | 43.782536, -71.554235 | SD | 6/27/2019 | 0-4 |  |
| B29 | BRL | 13 | 14.59 | 43.782296, -71.554088 | SD | 6/27/2019 | 0-4 |  |
| B30 | 4 | 24.1 | 30.86 | 43.78171, -71.55292 | SD | 6/27/2019 | 0-4 |  |
| B32 | 4.4 | 27.3 | 34.83 | 43.781638, -71.552904 | SD | 6/27/2019 | 0-4 |  |
| B33 | 3.1 | 11.6 | 16.03 | 43.781842, -71.553037 | SD | 6/27/2019 | 0-4 |  |
| B34 | 19.4 | 6 | 26.09 | 43.781806, -71.554229 | S | 8/26/2019 | 0-6 |  |
| B35 | 23.1 | 12.1 | 36.59 | 43.781958, -71.554548 | S | 8/26/2019 | 0-6 |  |
| B36 | 7.4 | 17.9 | 27.35 | 43.780969, -71.554243 | S | 8/26/2019 | 0-6 |  |
| B37 | ND | ND | 0 | 43.780988, -71.554464 | S | 8/26/2019 | 0-6 |  |
| B38 | 9 | 3.9 | 13.35 | 43.778832, -71.553383 | S | 8/26/2019 | 0-6 |  |
| B39 | ND | ND | 0 | 43.778828, -71.553587 | S | 8/26/2019 | 0-6 |  |
| B40 | ND | ND | 0 | 43.776574, -71.553808 | S | 8/26/2019 | 0-6 |  |
| B41 | ND | ND | 0 | 43.776598, -71.553960 | S | 8/26/2019 | 0-6 |  |
| B42 | 723.2 | 721.1 | 1526.99 | 43.781790, -71.554075 | S | 8/26/2019 | 0-6 |  |
| B43 | 8.2 | BRL | 8.31 | 43.781772, -71.55414 | S | 8/26/2019 | 0-6 |  |
| B45 | BRL | BRL | 0.21 | 43.782766, -71.554064 | S | 8/26/2019 | 0-6 |  |
| B46 | ND | ND | 0 | 43.783500, -71.553420 | S | 8/26/2019 | 0-6 |  |
| B47 | ND | ND | 0 | 43.783609, -71.553575 | S | 8/26/2019 | 0-6 |  |
| B48 | ND | ND | 0 | 43.785598, -71.552129 | S | 8/26/2019 | 0-6 |  |
| B49 | ND | ND | 0 | 43.785605, -71.552241 | S | 8/26/2019 | 0-6 |  |
| B50 | ND | ND | 0 | 43.787388, -71.550607 | S | 8/26/2019 | 0-6 |  |
| B51 | ND | ND | 0 | 43.787523, -71.550644 | S | 8/26/2019 | 0-6 |  |
| B52 | 28.6 | 9.8 | 39.52 | 43.782717, -71.553923 | S | 10/14/2019 | 0-4 |  |
| B53 | 165.1 | 110.2 | 287.94 | 43.781776, -71.554075 | S | 10/14/2019 | 0-4 |  |
| B53* | 290 | 191 | 502.92 | 43.781776, -71.554075 | S | 10/14/2019 | 0-4 |  |
| B54 | 167.7 | 57.8 | 232.13 | 43.78181, -71.554067 | S | 10/14/2019 | 0-4 |  |
| B55 | BRL | BRL | BRL | 43.781791, -71.554126 | S | 10/14/2019 | 0-4 |  |
| B56 | BRL | BRL | BRL | 43.781791, -71.554126 | S | 10/14/2019 | 0-4 |  |

*Analyzed by Alpha Analytical; ND No Detection; BRL Below Reporting Limit; ΣDDT = p,p’ DDT + p,p’ DDE, expressed in terms of the DDT after molecular weight adjustment (FAO 2016).