Appendix

Financial Values and Equations

Financial values included below are sourced from NREL’ATB (National Renewable Energy Laboratory 2023).

Table A. Financial Values for Technologies

|  | CAPEX ($/kW) | Fixed Operation Cost ($/kW/yr) | Variable Operational Cost ($/MWh) | Fuel cost ($/MMBtu) | Heat Rate (MMBtu/MWh) | Lifetime (yr) | Interest Rate (%) | Rate of Return on Equity (%) | Debt Fraction (%) | Investment Tax Credit (%) | Production Tax Credit ($/MWh) | Construction Finance Factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | OCC | FOM | VOM |  |  | L | IR | RROE | DF | ITC | PTC | CFF |
| Solar | 1,002 | 18 | 0 | 0 | 0 | 30 | 7 | 8.8 | 45.5 | 0 | 25.46 | 1.04 |
| Wind | 1,083 | 27 | 0 | 0 | 0 | 30 | 7 | 10 | 33 | 0 | 25.46 | 1.06 |
| Offshore Wind | 2,581 | 90 | 0 | 0 | 0 | 30 | 7 | 11 | 50.7 | 30 | 0 | 1.11 |
| Conventional Hydro | 7,328 | 36 | 0 | 0 | 0 | 100 | 7 | 11 | 48.7 | 30 | 0 | 1.07 |
| RoR Hydro | 7,082 | 47 | 0 | 0 | 0 | 100 | 7 | 11 | 48.7 | 30 | 0 | 1.07 |
| Nuclear | 6,115 | 152 | 2.5 | 0.658 | 10.45 | 60 | 8 | 11 | 48.9 | 30 | 0 | 1.26 |
| Natural Gas (CT) | 939 | 23 | 6.4 | 3.99 | 9.72 | 55 | 8 | 11 | 55 | 0 | 0 | 1.12 |
| Natural Gas (CC) | 1,039 | 29 | 1.8 | 3.99 | 6.24 | 55 | 8 | 11 | 55 | 0 | 0 | 1.12 |
| Natural Gas (CCS) | 2,018 | 54 | 4.1 | 3.99 | 6.99 | 55 | 8 | 11 | 55 | 0 | 0 | 1.12 |
| PHS | 3,166 | 18.7 | 0.54 | 0 | 0 | 100 | 7 | 11 | 69.5 | 30 | 0 | 1.07 |
| LIB | 1,204 | 30 | 0 | 0 | 0 | 15 | 7 | 8.8 | 45.5 | 30 | 0 | 1.04 |

Depreciation Values:

ft,y = depreciation factor for technology, t, in year, y

FDt,y = fraction of capital depreciated for technology, t, in year, y

fsolar,y = [0.9421, 0.8876, 0.8363, 0.7879, 0.7423, 0.6993]

FDsolar,y = [0.2, 0.32, 0.192, 0.1152, 0.1152, 0.0576]

fwind,y = [0.9224, 0.8508, 0.7848, 0.7239, 0.6677, 0.6158]

FDwind,y = [0.2, 0.32, 0.192, 0.1152, 0.1152, 0.0576]

foffw,y = [0.9363, 0.8766, 0.8208, 0.7685, 0.7195, 0.6736]

FDoffw,y = [0.2, 0.32, 0.192, 0.1152, 0.1152, 0.0576]

fconventional,y = [0.9244, 0.8546, 0.79, 0.7303, 0.6751, 0.6241]

FDconventional,y = [0.2, 0.32, 0.192, 0.1152, 0.1152, 0.0576]

fRoR,y = [0.9244, 0.8546, 0.79, 0.7303, 0.6751, 0.6241]

FDRoR,y = [0.2, 0.32, 0.192, 0.1152, 0.1152, 0.0576]

fNGCT,y = [0.9499, 0.9023, 0.8571, 0.8142, 0.7734, 0.7346, 0.6978, 0.6629, 0.6297, 0.5981, 0.5682, 0.5397, 0.5127, 0.487, 0.4626, 0.4394]

FDNGCT,y = [0.05, 0.095, 0.0855, 0.077, 0.0693, 0.0623, 0.059, 0.059, 0.0591, 0.059, 0.0591, 0.059, 0.0591, 0.059, 0.0591, 0.0295]

fNGCC,y = [0.9499, 0.9023, 0.8571, 0.8142, 0.7734, 0.7346, 0.6978, 0.6629, 0.6297, 0.5981, 0.5682, 0.5397, 0.5127, 0.487, 0.4626, 0.4394]

FDNGCC,y = [0.05, 0.095, 0.0855, 0.077, 0.0693, 0.0623, 0.059, 0.059, 0.0591, 0.059, 0.0591, 0.059, 0.0591, 0.059, 0.0591, 0.0295]

fNGCCS,y = [0.9499, 0.9023, 0.8571, 0.8142, 0.7734, 0.7346, 0.6978, 0.6629, 0.6297, 0.5981, 0.5682, 0.5397, 0.5127, 0.487, 0.4626, 0.4394]

FDNGCCS,y = [0.05, 0.095, 0.0855, 0.077, 0.0693, 0.0623, 0.059, 0.059, 0.0591, 0.059, 0.0591, 0.059, 0.0591, 0.059, 0.0591, 0.0295]

fPHS,y = [0.9244, 0.8546, 0.79, 0.7303, 0.6751, 0.6241]

FDPHS,y = [0.2, 0.32, 0.192, 0.1152, 0.1152, 0.0576]

fLIB,y = [0.9421, 0.8876, 0.8363, 0.7879, 0.7423, 0.6993]

FDLIB,y = [0.2, 0.32, 0.192, 0.1152, 0.1152, 0.0576]

Converting OCC to capital:

Tax rate = 25.7% (TR)

Inflation = 2.5% (i)

Capacity Factor is dependent on region and technology (CF)

WACCnominalt = DFt * IRt * (1 - TRt) + (1 – DFt) * RROEt

WACCt = (1 + WACCnominalt) / (1 + it) – 1

CRFt = WACCt / (1 - (1 / (1 + WACCt) ** Lt))

PVDt = sum(FDt,i * ft,i) for all i

PFFt = (1 - TRt * PVDt * (1 - ITCt / 2) - ITCt) / (1 - TRt)

FCRt = CRFt * PFFt

CRF_10yrt = WACCt / (1-(1/(1 + WACCt)10))

PTC_discountt = PTCt / (1 - TRt) * CRFt / CRF_10yrt

capital = OCCt * FCRt * CFFt - PTC_discountt * CFt * 8760 / 1000 [$/kW/yr]

LCOE calculation:

LCOE is not used anywhere in this study, except in Figure 2, to orient the reader of the general emissions-cost space.

LCOEt = (capitalt + FOMt) * 1000 / 8760 / CFt + VOMt + fuelcostt *heatratet [$/MWh]

Emissions Values

Emissions values below are sourced from MIT’s pathway analysis tool (“SESAME” 2022).

Table B. Emissions Values for all Technologies

|  | Embodied Emissions (gCO2/kW) |  |  |  |  |  |  |  |  | Operational Emissions (gCO2/kWh) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid Emissions Intensity (gCO2/kWh) | 100 | 90 | 80 | 70 | 60 | 50 | 40 | 30 | 20 | all |
| Abbreviation | eGC |  |  |  |  |  |  |  |  | eOP |
| Solar | 45,096 | 44,413 | 43,730 | 43,047 | 42,193 | 41,509 | 40,826 | 40,143 | 39,459 | 0 |
| Wind | 13,374 | 13,238 | 13,102 | 12,966 | 12,830 | 12,695 | 12,559 | 12,423 | 12,287 | 0 |
| Offshore Wind | 22,269 | 22,016 | 21,763 | 21,509 | 21,256 | 21,003 | 20,749 | 20,496 | 20,243 | 0 |
| Conventional Hydro | 110,376 |  |  |  |  |  |  |  |  | 0 |
| RoR Hydro | 28,330 |  |  |  |  |  |  |  |  | 0 |
| Nuclear | 3,735 |  |  |  |  |  |  |  |  | 8 |
| Natural Gas (CT) | 32,755 |  |  |  |  |  |  |  |  | 781 |
| Natural Gas (CC) | 39,901 |  |  |  |  |  |  |  |  | 481 |
| Natural Gas (CCS) | 44,334 |  |  |  |  |  |  |  |  | 162 |
| PHS | 3,150 |  |  |  |  |  |  |  |  | 0 |
| LIB | 19,440 |  |  |  |  |  |  |  |  | 0 |

LCA calculation:

This LCA calculation is not used anywhere in this study, except in Figure 2, to orient the reader of the general emissions-cost space.

LCAt = Embodied_emissionst / 8760 / CFt + operational_emissionst