# Supplement

This supplementary document provides details about the mathematical models used to analyze FCI images, as these differ from MRI, as well as the statistical models used to quantify FCI images.

## Analysis of the magnitude images

The T1 images were obtained using a mono-exponential model:

Equation 1

S(B0E, tE) = A [(M0P – M0E) exp(- tE/T1) + M0E]

where A is a scaling factor that accounts for proton density, magnetic susceptibility, and RF chain efficiency, M0P is the longitudinal magnetization obtained after the polarization stage, M0E is the equilibrium magnetization level during the evolution stage and tE is the evolution time. In conventional MRI, this equation is simpler as there is a direct relationship between M0P and M0E, but for FCI the model has to take into account the change in equilibrium magnetization M0E, which depends on the evolution field.

## Analysis of the T1 NMRD profiles

The NMRD profiles of tumours were analyzed using a power law model:

Equation 2

1/T1(B) =  B

where B0E is the evolution field, and  and  are the amplitude and dispersion of the power law model respectively. In a log-log plot,  corresponds to the value of the NMRD profile at 1 mT and  to the slope of the curve. This model choice was motivated by its simplicity, which allows extracting structural information from a limited number of points in the T1 NMRD profiles.

Previous experiments (data not published) from excised breast tissues by our group showed that their T1 NMRD profile consists of two regions, above and below 22 mT, following power law models with different dispersions.

Similar analyses from breast tumours showed only one single power law behavior, but with added relaxation enhancements known as quadrupolar peaks (QP). These appear around 60 mT and are known to be due to cross-relaxation between water protons and 14N from slowly moving proteins.

In this exploratory study, we limited the total scan time to one hour maximum for patient comfort. This only allowed us to scan four evolution fields with a sufficient number of evolution times for T1 estimation. Therefore, to explore all relevant water dynamic regimes, we decided to use three evolution fields, at 200, 22 and 2.3 mT, since these measurements are sufficient to estimate the T1 dispersion in the low- and high-field regions. We allocated the last evolution field at 65.8 mT to assess changes of quadrupolar relaxation.

## Extraction of the image quality metrics

The Signal-to-Noise Ratio (SNR) of FCI images was measured using the Rayleigh correction factor:

Equation 3

SNR = 0.66 x S(ROIS) / S(ROIN)

where S(ROIS) is the mean intensity from the region of interest (ROI) on the contralateral healthy tissue and S(ROIN) is the mean intensity from the ROI on background noise.

The contrast-to-noise ratio (CNR) was measured from the two images, one at the highest field strength (200 mT) and one at the lowest field strength (0.2 mT) from the first evolution time. The CNR between the tumour and adjacent healthy tissue was calculated as:

Equation 4

CNR =S(ROIT) - S(ROIH)S(ROIN) × 0.66

where S(ROIT) is the mean signal intensities from the ROIs in tumour and S(ROIH) is the mean signal intensities from the ROIs in adjacent healthy tissue which generally includes a mix of adipose and glandular tissue.

The tumour-to-background contrast ratio in T1 maps was calculated as the percentage difference in T1 from:

Equation 5

ΔT1% =T1(ROIT) -T1(ROIH)[T1ROIT+T1(ROIH)]/2×100

where T1(ROIT) and T1(ROIH) are the mean T1 values from the ROIs in tumour and adjacent healthy tissue respectively from T1 maps.