Biohydrogen Production from Lignocellulosic Hydrolysate: Unveiling Synergistic Impact of Substrate Concentration and Furfural Inhibition

Basem Harouna,b*, Mohamed El-Qelisha,b , Chinaza Akobia, Hisham Hafezc,d, Fayza Nasrb, Mingu Kima, George Nakhlaa,c

aChemical and Biochemical Engineering, University of Western Ontario, London, Ontario, N6A 5B9, Canada

bWater Pollution Research Department, National Research Center, El Buhouth St., Dokki, Cairo12622, Egypt

cCivil and Environmental Engineering, University of Western Ontario, London, Ontario, N6A 5B9, Canada

dGreenField Ethanol Inc., Chatham, Ontario, N7M 5J4, Canada

Corresponding author email: bharoun@uwo.ca

List of Tables

| Table 1 Batch set up of hydrogen production from different lignocellulosic hydrolysate concentrations at initial furfural concentrations of 0, 1 and 2 g/L at initial pH 5.9 and incubated at 37 °C |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Batch name | Substrate conc. (g sugar/L) | Furfural conc. (g/L) | mL of stock solution | mL of sludge | mg VSS | Initial VSS (mg/L) | F/B (g/g) | F/S (g/g) |
| B2-0 | 2 | 0 | 1.6 | 2.9 | 35.6 | 356 | 0 | 0 |
| B2-1 |  | 1 |  |  |  |  | 2.81 | 0.56 |
| B2-2 |  | 2 |  |  |  |  | 5.62 | 1.0 |
| B8-0 | 8 | 0 | 6.3 | 11.4 | 142 | 1423 | 0 | 0 |
| B8-1 |  | 1 |  |  |  |  | 0.7 | 0.125 |
| B8-2 |  | 2 |  |  |  |  | 1.41 | 0.25 |
| B16-0 | 16 | 0 | 12.5 | 22.8 | 285 | 2845 | 0 | 0 |
| B16-1 |  | 1 |  |  |  |  | 0.35 | 0.0625 |
| B16-2 |  | 2 |  |  |  |  | 0.7 | 0.125 |
| B24-0 | 24 | 0 | 18.8 | 34.1 | 427 | 4268 | 0 | 0 |
| B24-1 |  | 1 |  |  |  |  | 0.23 | 0.042 |
| B24-2 |  | 2 |  |  |  |  | 0.47 | 0.083 |
| B32-0 | 32 | 0 | 25 | 45.5 | 569 | 5688 | 0 | 0 |
| B32-1 |  | 1 |  |  |  |  | 0.18 | 0.031 |
| B32-2 |  | 2 |  |  |  |  | 0.35 | 0.062 |

| Table 2 Hydrogen yield and MSHPR at furfural concentrations between 1.0 and 2.0 g/L at initial pH 5.9 and incubated at 37 °C |  |  |  |  |
| --- | --- | --- | --- | --- |
| Batch No. | HY mol H2/mol sugar) |  | (MSHPR)mLH2/gVSS/h |  |
|  | Average | SD | Average | SD |
| B16-0-2 | 1.17 | 0.06 | 38 | 6.0 |
| B16-1.2 | 1.24 | 0.06 | 45 | 3.5 |
| B16-1.4 | 1.22 | 0.07 | 41 | 2.5 |
| B16-1.6 | 1.00 | 0.03 | 34 | 3.0 |
| B16-1.8 | 0.16 | 0.03 | 9 | 0.3 |

| Table 3 Hydrogen production yields and rates from different lignocellulosic hydrolysate concentrations at initial furfural concentrations of 0, 1 and 2 g/L at initial pH 5.9 and incubated at 37 °C |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Batch name | Substrate conc. | Furfural conc. | Cumulative H2 production | H2 production yield | H2 production yield | MSHPR |
| Unit | (g/L) | (g/L) | mL | mLH2/g sugar | mol H2/mol sugar | mLH2/gVSS.h |
| B2-0 | 2 g/L | 0 | 58 ± 6.2 (3) a | 292 ± 31 | 1.77 ± 0.19 | 53 ± 5 (2) |
| B2-1 |  | 1 | 38 ± 1.8 (2) | 191 ± 8.9 | 1.16 ± 0.05 | 38 ± 1 (2) |
| B2-2 |  | 2 | 0 | 0 | 0.00 | 0 |
| B8-0 | 8 g/L | 0 | 217 ± 3 (3) | 271 ± 3.7 | 1.65 ± 0.02 | 58 ± 3 (3) |
| B8-1 |  | 1 | 214 ± 5.6 (3) | 268 ± 6.9 | 1.62 ± 0.04 | 58 ± 10 (2) |
| B8-2 |  | 2 | 0 | 0 | 0.00 | 0 |
| B16-0 | 16 g/L | 0 | 408 ± 11.5 (3) | 255 ± 7.2 | 1.55 ± 0.04 | 107 ± 5 (3) |
| B16-1 |  | 1 | 400 ± 20 (3) | 250 ± 12.2 | 1.52 ± 0.07 | 128 ± 9 (3) |
| B16-2 |  | 2 | 0 | 0 | 0.00 | 0 |
| B24-0 | 24 g/L | 0 | 419 ± 20 (3) | 175 ± 8.2 | 1.06 ± 0.05 | 76 ± 4 (3) |
| B24-1 |  | 1 | 361 ± 21 (3) | 151 ± 8.7 | 0.91 ± 0.05 | 56 ± 4 (2) |
| B24-2 |  | 2 | 0 | 0 | 0.00 | 0 |
| B32-0 | 32 g/L | 0 | 391 ± 27 (3) | 122 ± 8.6 | 0.74 ± 0.05 | 31 ± 3 (2) |
| B32-1 |  | 1 | 391 ± 31 (2) | 122 ± 9.8 | 0.74 ± 0.06 | 48 (1) |
| B32-2 |  | 2 | 0 | 0 | 0.00 | 0 |
| a Number of replicates |  |  |  |  |  |  |

| Table 4 Kinetic parameters from substrate inhibition model |  |  |  |
| --- | --- | --- | --- |
| Furfural conc. (g/L) | µmax (h-1)a | Ks (g/L) | Ki (g/L) |
| 0 | 0.11 | 6 | 28.3 |
| 1 | 0.13 | 8.8 | 23 |
| a Expressed as maximum specific hydrogen production rate |  |  |  |

| Table 5 Kinetic parameters for glucose, molasses and cheese whey grown on suspended and immobilized cells of Thermotoga neapolitana adopted from (Frascari et al., 2013) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Glucose |  | Molasses |  | Cheese whey |  |
|  | Suspended cells | Attached cells | Suspended cells | Attached cells | Suspended cells | Attached cells |
| µmax (h-1) | 0.024 ± 0.005 | 0.09 ± 0.01 | 0.055 ± 0.005 | 0.19 ± 0.02 | 0.033 ± 0.006 | 0.042 ± 0.007 |
| Ks (g sugars/L) | 1.1 ± 0.3 | 0.09 ± 0.05 | 0.20 ± 0.05 | 0.6 ± 0.2 | 1.5 ± 0.5 | 1.2 ± 0.3 |
| Ki (g sugars/L) | 8 ± 2 | 90 ± 30 | 25 ± 7 | 12 ± 3 | 28 ± 10 | 11 ± 3 |
| Sx* (g/L) | 2.9 ± 0.5 | 2.8 ± 0.9 | 2.5 ± 0.4 | 2.8 ± 0.5 | 6 ± 2 | 3.7 ± 0.6 |
| * Substrate concentration above which specific hydrogen production rate decreases |  |  |  |  |  |  |

| Table 6 Pearson coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Substrate Conc. | MSHPR | F/B | F/S | S⁰/X⁰ | Furfural conc. | Initial VSS |
| H2 yield | -0.30 | 0.82 | -0.38 | -0.38 | N/A | -0.82 | -0.30 |
| P value | >0.05 | <0.05 | >0.05 | >0.05 | N/A | <0.05 | >0.05 |
| MSHPR | -0.02 | N/A | -0.40 | -0.40 | N/A | -0.68 | -0.02 |
| P value | >0.05 | N/A | >0.05 | >0.05 | N/A | <0.05 | >0.05 |

| Table 7 Summary of volatile fatty acids and sugar conversion of hydrogen production from different lignocellulosic hydrolysate concentrations at initial furfural concentrations of 0, 1 and 2 g/L at initial pH 5.9 and incubated at 37 °C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Batch name | gCOD Lactate/g CODTVFA | Residual sugars (g/L) | sugar conversion (%) | TVFA (g COD) | mol HAc/ mol HBu |
| B2-0 | 0.00 | 0.00 | 100 | 1.8 | 3.2 |
| B2-1 | 0.39 | 0.00 | 100 | 2.7 | 7.7 |
| B2-2 | 0.39 | 0.01 | 100 | 4.1 | 8.9 |
| B8-0 | 0.00 | 0.00 | 100 | 7.2 | 0.9 |
| B8-1 | 0.13 | 0.00 | 100 | 8.5 | 1.4 |
| B8-2 | 0.34 | 0.37 | 95 | 9.6 | 4.2 |
| B16-0 | 0.01 | 0.68 | 96 | 11.2 | 1.1 |
| B16-1 | 0.12 | 0.31 | 98 | 12.1 | 1.3 |
| B16-2 | 0.48 | 7.15 | 55 | 8.7 | 12.5 |
| B24-0 | 0.24 | 1.48 | 94 | 18.0 | 1.1 |
| B24-1 | 0.32 | 4.20 | 83 | 18.0 | 1.8 |
| B24-2 | 0.48 | 14.90 | 38 | 9.6 | 2.1 |
| B32-0 | 0.42 | 5.56 | 83 | 21.4 | 2.3 |
| B32-1 | 0.48 | 9.40 | 71 | 22.3 | 2.9 |
| B32-2 | 0.44 | 19.18 | 40 | 17.0 | 490 |

| Table 8 Liquid metabolites produced and residual sugars concentrations from hydrogen production from different lignocellulosic hydrolysate concentrations at initial furfural concentrations of 0, 1 and 2 g/L at initial pH 5.9 and incubated at 37 °C |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Batch | Glucose | Xylose | Arabinose | Lactic | Formic | Acetic | Propionic | Butyric | Furfural | Ethanol | Isobutyric | Valeric | Isovaleric |
| name | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) | (g/L) |
| B2-0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.64 | 0.09 | 0.29 | 0.00 | 0.00 | 0.112 | 0.100 | 0.014 |
| B2-1 | 0.00 | 0.00 | 0.00 | 1.00 | 0.00 | 1.10 | 0.02 | 0.21 | 0.00 | 0.30 | 0.014 | 0.020 | 0.009 |
| B2-2 | 0.00 | 0.01 | 0.00 | 1.50 | 0.11 | 1.33 | 0.41 | 0.22 | 0.22 | 0.16 | 0.007 | 0.023 | 0.004 |
| B8-0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.55 | 0.35 | 2.57 | 0.00 | 0.01 | 0.149 | 0.021 | 0.028 |
| B8-1 | 0.00 | 0.00 | 0.00 | 1.01 | 0.00 | 1.92 | 0.43 | 2.00 | 0.00 | 0.11 | 0.076 | 0.442 | 0.023 |
| B8-2 | 0.00 | 0.31 | 0.06 | 3.04 | 0.01 | 2.85 | 0.52 | 1.00 | 0.26 | 0.02 | 0.011 | 0.345 | 0.002 |
| B16-0 | 0.28 | 0.40 | 0.00 | 0.14 | 0.00 | 2.87 | 0.42 | 3.90 | 0.00 | 0.00 | 0.113 | 0.007 | 0.011 |
| B16-1 | 0.30 | 0.00 | 0.01 | 1.32 | 0.00 | 3.20 | 0.49 | 3.49 | 0.00 | 0.00 | 0.008 | 0.090 | 0.009 |
| B16-2 | 0.16 | 5.84 | 1.15 | 3.89 | 0.07 | 3.42 | 0.00 | 0.40 | 0.62 | 0.00 | 0.009 | 0.043 | 0.001 |
| B24-0 | 0.25 | 1.23 | 0.00 | 4.10 | 0.04 | 3.18 | 1.25 | 4.22 | 0.00 | 0.00 | 0.104 | 0.108 | 0.106 |
| B24-1 | 0.28 | 3.74 | 0.18 | 5.43 | 0.05 | 3.94 | 1.33 | 3.29 | 0.00 | 0.00 | 0.000 | 0.000 | 0.000 |
| B24-2 | 0.28 | 13.57 | 1.05 | 4.32 | 0.10 | 1.68 | 0.60 | 1.20 | 0.83 | 0.00 | 0.000 | 0.033 | 0.010 |
| B32-0 | 0.30 | 5.03 | 0.23 | 8.32 | 0.12 | 4.49 | 1.58 | 2.90 | 0.00 | 0.00 | 0.001 | 0.001 | 0.000 |
| B32-1 | 0.36 | 8.78 | 0.26 | 9.96 | 0.16 | 5.53 | 0.03 | 2.84 | 0.00 | 0.00 | 0.00 | 0.22 | 0.00 |
| B32-2 | 0.25 | 17.44 | 1.49 | 6.96 | 0.14 | 8.69 | 0.08 | 0.03 | 1.03 | 0.00 | 0.00 | 0.02 | 0.01 |

| Table 9 Theoretical hydrogen production based on the acetate and butyrate produced |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Batch name | Substrate conc. | Furfural conc. | Acetic acid | Butyric acid | Propionic acid | From Acetic acid | From Butyric acid | From Propionic acid | Theoretical H2 | Measured H2 | % difference |
| Unit | g/L | g/L | g/L | g/L | g/L | mL H2 | mL H2 | mL H2 | mL H2 | mLH2 | % |
| B2-0 | 2 | 0 | 0.64 | 0.29 | 0.09 | 54 | 17 | 3 | 68 | 58.4 | 14 |
| B2-1 |  | 1 | 1.10 | 0.21 | 0.02 | 93 | 12 | 1 | 104 | 34.3 | 67 |
| B8-0 | 8 | 0 | 1.55 | 2.57 | 0.35 | 132 | 148 | 12 | 268 | 217 | 19 |
| B8-1 |  | 1 | 1.92 | 2.00 | 0.43 | 163 | 116 | 15 | 264 | 214 | 19 |
| B16-0 | 16 | 0 | 2.87 | 3.90 | 0.42 | 243 | 225 | 14 | 454 | 408 | 10 |
| B16-1 |  | 1 | 3.20 | 3.49 | 0.49 | 271 | 202 | 17 | 456 | 400 | 12 |
| B24-0 | 24 | 0 | 3.18 | 4.22 | 1.25 | 269 | 244 | 43 | 470 | 419 | 11 |
| B24-1 |  | 1 | 3.94 | 3.29 | 1.33 | 334 | 190 | 45 | 479 | 361 | 25 |
| B32-0 | 32 | 0 | 4.49 | 2.90 | 1.58 | 381 | 167 | 54 | 494 | 391 | 21 |
| B32-1 |  | 1 | 5.53 | 2.84 | 0.03 | 469 | 164 | 1 | 632 | 391 | 38 |