Table 1. Results from CFD simulation

| Description | Value | Unit |
| --- | --- | --- |
| Surface Area | 110.06 | m2 |
| Surface Heat Transfer Coef. | 22.01 | W/m2K |
| Total Surface Heat Flux | 352.02 | W/m2 |
| Total Heat Transfer Rate | 38.74 | kW |
| LCO2 Liquid Fraction | 1 | - |
| LCO2 Vapor Fraction | 0 | - |

Table 2. Liquefied CO2 flow rate and composition.

| Parameter |  | Liquefied CO2 |
| --- | --- | --- |
| Composition (wt%) | CO2 | 99.924 |
|  | CO | 0.013 |
|  | H2 | 0.028 |
|  | CH4 | 0.033 |
|  | H2O | 0.002 |
| CO2 Flow Rate (kg/h) |  | 50,000 |
| Total Flow Rate (kg/h) |  | 50,038 |

Table 3. Dynamic simulation scenarios.

| Scenario | Details |
| --- | --- |
| Cold Start | Optimized step up injection method during initial operation (how many steps should be taken at intervals of how many hours to reach the target flow rate from 0, how the CO2 flow in the pipeline will change, the time it takes to stabilize, etc., be careful when injecting) What to do, etc.) - ex) Deriving a stable time and appropriate increase amount, such as when increasing the flow rate from 0 to 500, it must be increased by 100 every hour. |
| Shutdown | Step down response method in case of emergency shutdown due to injection system failure (time, appropriate reduction amount, etc.) When shutting down for a long period of time for maintenance, step down response method (time, appropriate reduction amount, etc.) - ex) When lowering the flow rate from 500 to 0, it must be lowered by 100 every 2 hours, etc. to derive a stable time and appropriate reduction amount. |
| Abnormal Operation | How to safely change the flow rate at a specific flow rate |
|  | The safest operating conditions to avoid vapor and freezing of LCO2 (Flow rate, temperature, pressure) derived |

Table 4. Feed flow rate ramp-up for start-up scenario.

| Time (h) | Feed Flow Rate Ramp Up (kg/h) | Flow Increment (kg/h) [22] |
| --- | --- | --- |
| 1 | 0 | - |
|  |  | 3000 |
| 2 | 3000 |  |
|  |  | 7300 |
| 3 | 10300 |  |
|  |  | 4100 |
| 4 | 14400 |  |
|  |  | 3600 |
| 5 | 18000 |  |
|  |  | 4000 |
| 6 | 22000 |  |
|  |  | 4500 |
| 7 | 26500 |  |
|  |  | 5000 |
| 8 | 31500 |  |
|  |  | 6000 |
| 9 | 37500 |  |
|  |  | 6500 |
| 10 | 4400 |  |
|  |  | 6038 |
| 11 | 50038 |  |
|  |  | - |

[22,23]Flow difference between 2 consecutive times.

Table 5. Feed flow rate ramp-down for shutdown scenario.

| Time (h) | Feed Flow Rate Ramp Down (kg/h) | Flow Decrement (kg/h) [22] |
| --- | --- | --- |
| 1 | 50038 | - |
|  |  | 6838 |
| 2 | 43200 |  |
|  |  | 5700 |
| 3 | 37500 |  |
|  |  | 5300 |
| 4 | 32200 |  |
|  |  | 5300 |
| 5 | 26900 |  |
|  |  | 5700 |
| 6 | 21200 |  |
|  |  | 5400 |
| 7 | 15800 |  |
|  |  | 2600 |
| 8 | 13200 |  |
|  |  | 2200 |
| 9 | 11000 |  |
|  |  | 1800 |
| 10 | 9200 |  |
|  |  | 1500 |
| 11 | 7700 |  |
|  |  | 1300 |
| 12 | 6400 |  |
|  |  | 1000 |
| 13 | 5400 |  |
|  |  | 900 |
| 14 | 4500 |  |
|  |  | 700 |
| 15 | 3800 |  |
|  |  | 600 |
| 16 | 3200 |  |
|  |  | 500 |
| 17 | 2700 |  |
|  |  | 500 |
| 18 | 2200 |  |
|  |  | 500 |
| 19 | 1700 |  |
|  |  | 500 |
| 20 | 1200 |  |
|  |  | 500 |
| 21 | 700 |  |
|  |  | 700 |
| 22 | 0 |  |
|  |  | - |

[22,23]Flow difference between 2 consecutive times.

Table 6. Stream summary results.

|  |  | Feed (S1) | Node 1 (S2) | Node 2 (S3) | Node 3 (S4) | Node 4 (S5) | Node 5 (S6) | Node 6 (S7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase |  | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase |
| Temperature | C | 20 | 19.99 | 19.47 | 18.75 | 14.21 | 13.91 | 10.64 |
| Pressure | barg | 65 | 64.96 | 66.79 | 66.85 | 70.8 | 70.79 | 73.44 |
| Mass Flows | kg/h | 50,038 | 50,038 | 50,038 | 50,038 | 50,038 | 50,038 | 50,038 |
|  |  | Node 7 (S8) | Node 8 (S9) | Node 9 (S10) | Node 10 (S11) | Node 11 (S12) | Node 12 (S13) | Node 13 (S14) |
| Phase |  | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase | Liquid Phase |
| Temperature | C | 9.65 | 7.54 | 6.33 | 4.46 | 3.99 | 3.33 | 3.33 |
| Pressure | barg | 73.72 | 75.05 | 75.4 | 76.61 | 76.56 | 63.06 | 59.72 |
| Mass Flows | kg/h | 50,038 | 50,038 | 50,038 | 50,038 | 50,038 | 50,038 | 50,038 |