# Appendix for additional text, data sets and data sources for the LEAP modelling

# (A1). Structure of Bhutan-LEAP model

# (A2). General Assumptions

An exchange rate of Nu. 61.5 per US dollar was used [1]. Costs information available in other calendar years were deflated or inflated to the base year value using the Consumer Price Index. A discount rate of 10% was considered, which was used in some of the earlier studies for Bhutan [2, 3] and the EFFECT model study for India conducted by the Energy Sector Management Assistance Program of the World Bank [4] mentions a discount rate of 10% as the global average opportunity cost of capital.

As forest cover of Bhutan is considered as a carbon sink, for accounting purposes emissions from wood-based energy consumption was accounted as positive to avoid the double benefit of carbon sequestration as well as the usual treatment of bioenergy as a carbon neutral energy source. The emission factors and the global warming potential (GWP) were taken from the technology and emission database (TED) of LEAP. The emissions from agriculture activities (farming and livestock rearing) and waste disposal were separately accounted for under ‘non-energy’. The amount of waste generation and disposal are assumed to increase along with the growth in urban population, whereas emissions from agriculture are assumed to remain constant as per the past trend [5].

Technology learning rates are based on available literature [6, 7]. For some of the household appliances market prices were obtained from a popular shop in Thimphu, Bhutan. The energy load shape is generated as per the monthly electricity consumption obtained from Bhutan Power Corporation Limited [8], a public electric utility company in Bhutan. The distinct feature about the electricity system in Bhutan is that the planning reserve margin is not specified and perhaps is not a requirement given that Bhutan’s electricity network is interconnected with the Indian grid. The Indian Grid acts like an infinite bus system that consumes excess electricity generated by Bhutan and supplies the unmet electricity demand of Bhutan during the lean water flow season.

# (A3). Data sets for Bhutan-LEAP model

Table A.1 Population and GDP projection based on [9] and [10, 11] respectively

| Year | Million US$ | Population |
| --- | --- | --- |
| 2014 | 902 | 745,153 |
| 2020 | 1,324 | 809,397 |
| 2025 | 1,822 | 850,976 |
| 2030 | 2,508 | 886,523 |
| 2035 | 3,453 | 931,745 |
| 2040 | 4,753 | 979,273 |
| 2045 | 6,542 | 1,029,226 |
| 2050 | 9,005 | 1,081,727 |

US$ - United States Dollar

Table A.2 Electricity sector key feature, based on [12]

| Particulars | GWh | US cents/kWh* |
| --- | --- | --- |
| Export | 5179.3 | 3.36 |
| Import | 187.6 | 3.77 |
| Generation | 7166.3 |  |
| T&D loss | 3.87% |  |

*authors’ calculation based on [12]

T&D – transmission and distribution

Table A.3 Power plant techno-economic data

| Plant | Capacity, MW | Capital cost (US$/kW)* | Data source |
| --- | --- | --- | --- |
| Existing hydropower plant |  |  |  |
| CHP | 360 | 646.57 | [13] www.drukgreen.bt |
| Kurichu | 60 | 1517.62 |  |
| Basochu | 64 | 828.51 |  |
| Tala | 1020 | 657.72 |  |
| Dagachu | 126 | 1577.75 |  |
| Hydro_Micro | 8.20 | 6952.00 | [14] |
| Candidate hydropower plant |  |  |  |
| PHPA I | 1200 | 1270.40 | [15] |
| PHPA II | 1020 | 1162.22 | [15] |
| MHPA | 720 | 1088.16 | [16] |
| Tangsibji | 118 | 1648.06 | [13] www.drukgreen.bt |
| KHP | 600 | 1048.51 | http://www.khepbhutan.com |
| Mega_hydro | 4000 | 1173.60 | this study |
| Other power plant |  |  |  |
| Solar | 0.12 | 1626 | [14]; [17] |
| Wind | 0.60 | 4848.85 | [18] |
| Diesel Generator Set | 10.7 | 2500 | [14]; [19] |
| WTE | 3.4 | 2746 | [14] |

*authors’ calculations based on the available data sources

Table A.4 Monthly electricity generation and consumption in 2014 based on utilities [8, 20]

| Month | Generation (GWh) | % peak generation | Consumption (GWh) |
| --- | --- | --- | --- |
| Jan | 243.7 | 20.3% | 169.8 |
| Feb | 178.9 | 14.9% | 154.6 |
| Mar | 211.4 | 17.6% | 161.7 |
| Apr | 232.6 | 19.4% | 166 |
| May | 458.9 | 38.2% | 172.5 |
| Jun | 785.1 | 65.4% | 167.6 |
| Jul | 1187.5 | 98.9% | 155.9 |
| Aug | 1200.8 | 100.0% | 164.3 |
| Sep | 1162.6 | 96.8% | 157 |
| Oct | 767.9 | 63.9% | 168.7 |
| Nov | 411.5 | 34.3% | 175.1 |
| Dec | 306.1 | 25.5% | 191.7 |

*authors’ calculations to construct hydropower availability curve

Table A.5 Cooking and heating end-use technology in the residential

and service sectors [3, 14, 21, 22]

| Device | US$/device | Life (years) | Efficiency |
| --- | --- | --- | --- |
| Wood cook stove | 5.69 | 5 | 10 |
| Efficient wood cook stove | 135.45 | 5 | 25 |
| LPG/biogas stove | 46.34 | 5 | 85 |
| Electric stove | 43.9 | 5 | 90 |
| Efficient wood heating stove | 189.66 | 5 | 75 |
| Electric heater | 57.98 | 5 | 90 |
| Kerosene heater | 203.25 | 5 | 45 |
| Wood heating stove | 56.90 | 5 | 12 |

Table A.6 Household electric appliances, based on [14, 23]

| Device | US$/device* | Life |
| --- | --- | --- |
| Fridge 2-star | 268.29 | 5 years |
| Fridge 3-star | 284.55 | 5 years |
| Washing m/c semi-auto | 240.65 | 5 years |
| Washing m/c auto | 256.91 | 5 years |
| 60Watt incandescent lamp | 0.16 | 1200 hours |
| 14Watt CFL | 1.95 | 10000 hours |
| 42Watt Fluorescent lamp | 4.55 | 10000 hours |
| 7Watt LED | 7.31 | 50000 hours |

*authors’ calculation derived from the given data sources;

CFL – compact fluorescent lamp; LED – light emitting diode

Table A.7 Cost of passenger vehicle [24-26]

| Vehicle | Cost (US$) |
| --- | --- |
| Electric-cars | 20,488 |
| Light vehicle_gasoline | 7,073.17 |
| Light vehicle_diesel | 29,268.29 |
| Electric-bike | 945.59 |
| Diesel bus | 39,204.75 |
| Electric-bus | 300,000.00 |
| Light rail* | 1.62 million US$ |
| (30 years life; 0.3MJ/pkm; 242 persons/coach) |  |

Table A.8 Transport sector technology characteristics, based on [2, 3, 26, 27]

| Passenger transport |  |  |  |  |
| --- | --- | --- | --- | --- |
| Vehicle type | Fleet | km/litre | Occupancy (person/vehicle) | mp-km/year |
| 2-wheeler | 9988 | 53.8 | 1.6 | 36.1 |
| Taxi | 4109 | 15 | 2.93 | 297.6 |
| Light | 41924 | 15 | 2.55 | 524.8 |
| Bus | 354 | 3.27 | 18.85 | 696.0 |
| Freight transport |  |  |  |  |
| Vehicle type | Fleet | km/litre | Average capacity (tonnes/vehicle) | mt-km/year |
| Heavy | 8120 | 3.7 | 6 | 1933.78 |
| Medium | 1392 |  | 3 | 392.73 |

mt-km: million tonnes kilometre; mp-km: million passenger kilometre

Table A.9 Freight transport technology, based on [2, 28]

| Freight vehicle | Diesel Heavy truck | Heavy_electric-truck* | Diesel Light truck | Light_electric-truck* |
| --- | --- | --- | --- | --- |
| MJ/vehicle-km | 7.46 | -76% | 3.92 | -77% |
| Capital cost | 32,000 | 406% | 20,292 | 297% |
| kWh/vehicle-km |  | 0.49 |  | 0.49 |
| O&M cost | 0.13 | -82% | 0.0561 | -76% |

*authors’ calculation based on [28]

Table A.10 Transformation processes other than power generation,

data imputed based on [23, 29, 30]

| Process | Capital cost | O&M cost | Life (years) | Production in 2014 (tonnes) | Resource potential |
| --- | --- | --- | --- | --- | --- |
| Coal Mining | 49.59 (US$/ton) | 82.62 (US$/ton) | 50 | 121,891.00 | 1.9 Mt |
| Biogas production | 18.96 (US$/GJ) | NA | 30 | 898.45 | 633,756.19 GJ |
| Briquette making | 4.05 (US$/GJ) | 5% of capital cost | 20 | 367.40 | 6,832.80 GJ |

Table A.11 Industry sector techno-economic data

| Industry | SEC (kWh/tonnes) | Production (tonnes) | US$/tonnes* |  | Life (years) | Data source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capital cost | O&M cost |  |  |
| Cement | 132.71 | 525,240.00 | 113.04 | 60.39 | 20 | [14, 29, 31]; http://lhakibhutan.com; http://peldenenterprise.com/bhutan-steel-industries-ltd.html; http://www.pendencement.com.bt/ |
| BCCL | 5,340.58 | 32,340.48 | 812.38 | 842.76 | 20 |  |
| Steel | 825.50 | 196,172.22 | 46.07 | 335.40 | 20 |  |
| Ferro Alloys | 9,000.00 | 105,050.00 | 660.98 | 895.05 | 20 |  |

*authors’ calculation based on the listed data sources

# (A4). Demand projection and CO2e emissions

Following expression syntax in LEAP expression builder was used to link energy demand branch to the key assumption branch.

GrowthAs(Branch:Variable, Elasticity)

The elasticity value is assumed and specified by the modeller. In the Bhutan-LEAP model, the above expression syntax translates to the following equation when invoked for demand projection in the demand branches.

Demand(t) = Demand(t-1) * driver(t)	Eq. (A.1)
                                     driver(t-1)

The drivers are the chosen macroeconomic parameter such as GDP, per capita GDP, population, number of households, etc., provided under the ‘key assumption’ branch. The energy demand ultimately drives the CO2e emissions.

End-use demands are expressed in terms of physical output such as the passenger-kilometre and tonne-kilometre for the transport demand and kilo-tonnes in the industry sector. Energy demand due to various end-uses are connected through the energy intensity values. Further, the end-use demands are linked in the following ways: the residential sector is linked to the number of households; the service sector is linked to its own value added (VA); the transport sector is linked to per capita GDP; and, the industry sector is linked to the national GDP. These linkages are modelled under the activity level of each of the demand sectors with an expression syntax in the Bhutan-LEAP model. According to the Bhutan Living Standard Survey [32], around 50% of dwellings do not have heating facilities and this is explicitly modeled under the activity level in heating end-use in the residential sector branch. Average number of rooms in a dwelling is taken as 3.03 as provided in [33], which was then used to calculate the number of lighting devices in the residential sector and then ultimately equated to the lighting energy demand provided in DRE [14].

Table A.12 Projected demand

| Sector | Activity unit | 2014 | 2020 | 2025 | 2030 | 2035 | 2040 | 2045 | 2050 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transport sector |  |  |  |  |  |  |  |  |  |
| Passenger travel | bp-km | 1.93 | 2.84 | 3.91 | 5.39 | 7.43 | 10.24 | 14.11 | 19.45 |
| Freight travel | bt-km | 2.34 | 3.67 | 5.34 | 7.77 | 11.31 | 16.46 | 23.97 | 34.89 |
| Industry sector |  |  |  |  |  |  |  |  |  |
| BCCL | Kt | 32.3 | 40 | 57.6 | 81.7 | 113.7 | 155.7 | 209.4 | 276.9 |
| Ferro Alloys | Kt | 106.8 | 132.1 | 190.4 | 269.7 | 375.6 | 514.1 | 691.6 | 914.5 |
| Iron and Steel | Kt | 196.2 | 242.7 | 349.7 | 495.4 | 689.8 | 944.2 | 1,270.30 | 1,679.80 |
| Cement | Kt | 525.2 | 649.8 | 936.3 | 1,326.30 | 1,847.00 | 2,528.10 | 3,401.30 | 4,497.60 |
| Residential and Service sector |  |  |  |  |  |  |  |  |  |
| Residential | Thousand HH | 164.13 | 175.27 | 185.12 | 195.53 | 206.52 | 218.13 | 230.4 | 243.35 |
| Service | Million US$ | 320.56 | 502.57 | 731.63 | 1,065.08 | 1,550.52 | 2,257.20 | 3,285.98 | 4,783.64 |

Table A.13 Fuel price projection, based on [14, 34]

| Fuel | Unit | 2014 | 2020 | 2025 | 2030 | 2035 | 2040 | 2045 | 2050 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diesel | US$/lt | 0.82 | 1.29 | 1.49 | 1.74 | 2.00 | 2.31 | 2.67 | 3.09 |
| Gasoline | US$/lt | 0.95 | 1.49 | 1.72 | 2.01 | 2.31 | 2.67 | 3.08 | 3.56 |
| Kerosene (Domestic) | US$/lt | 0.21 | 0.34 | 0.39 | 0.46 | 0.52 | 0.61 | 0.70 | 0.81 |
| Kerosene (Industry) | US$/lt | 0.77 | 1.22 | 1.41 | 1.64 | 1.89 | 2.18 | 2.52 | 2.92 |
| Aviation Turbine Fuel | US$/lt | 1.00 | 1.58 | 1.82 | 2.12 | 2.44 | 2.82 | 3.26 | 3.77 |
| LPG | US$/kg | 0.42 | 0.67 | 0.77 | 0.90 | 1.03 | 1.19 | 1.38 | 1.59 |

(A5). Additional text on Hydropower, industry and carbon neutral measures

A5.1 Additional text on hydropower in Bhutan

With regard to the impact of climate change on precipitation levels in Bhutan, NEC [5] reports that the mean annual precipitation will decline by 2% in the dry season and increase by 4-8% in the wet season – a net increase in the precipitation level. It was assumed to have a corresponding direct impact on the hydrological condition thus affecting hydropower generation as run-off characteristics are not known with any detail. Such issues were modelled by varying the hydropower availability curve, which in turn affects the hydropower generation profile. No significant changes were observed in the power generation level. This result also aligns with the belief of the key people in the energy sector of Bhutan, who are hopeful that climate change will have an insignificant impact on Bhutan’s hydropower generation profile for a long time [35]. The impact of varying precipitation levels on the river flow and further to the hydropower generation are complex and much is dependent on the climate models used, the timeframe considered and a host of other factors [36], which could be exacerbated by consideration of sediment build up, thus warranting separate research specific to Bhutan. Nonetheless, the design and construction of hydropower dams in Bhutan are said to have incorporated features which are resilient to glacier lake outburst floods (GLOF) that are attributed to climate change [5].

While the role of forest for carbon neutral Bhutan is clear, the role of hydropower for carbon neutrality can be assessed in terms of avoided emissions. This can be done by converting the electricity demand shown in table 2 (in main text) to an equivalent emission unit by using an emission factor of 0.82 tons CO2/MWh, which is being used for evaluating hydropower based CDM projects in Bhutan. For instance under the BAU scenario  the avoided CO2e emission is calculated at 1.5 MT’s, that is 65% of the total emissions of 2.4 MT’s in 2014, which further increases to 13 MT’s CO2e, that is 156% of the total emissions of 8.28 MT’s by 2050. This indicates that without hydroelectricity, the total CO2e emissions from the total energy consumption in the demand sectors would exceed the sink capacity much earlier than that shown in Table 3 (in main text). This directly underscores the contribution of the hydropower sector for the present as well as the future carbon neutral status of Bhutan.

A5.2 Additional text on manufacturing industries in Bhutan

Industry in Bhutan reflects the same issues found around the globe about fossil fuels used in process heat and fossil carbon used in chemical processes. In Bhutan, electricity (which is predominantly hydropower based) meets only 57% of the final energy demand in the industry sector [14, 22] as process heat and chemical use of carbon are based on carbonaceous raw materials, which would require material substitution under carbon neutral considerations for their products. There was much discussion about green industry development in Bhutan, however, the existing Economic Development Policy of Bhutan [37] has remained silent on such critical policy decisions and has instead used the carbon neutral status of the whole system to remain the major policy. As set out below this will not be enough going into the future based on scenario analysis.

A5.3 Additional text on carbon neutral (CN) measures

The four aggregate carbon neutral measures are outlined below.

- Industrial Symbiosis (IS) measure looks at expanding the use of steel slag as clinker substitutes and waste heat recoveries for the remaining energy intensive industries. Aspects of and ideas on IS were taken from literature [38-41]. Using Industrial Symbiosis, Yu, Han [42] found a reduction in carbon emissions of 10.8% in an Industrial enterprise in China.

- ATECH represents ‘Advanced TECHnology’ in the transport and industry sectors only. Transport and industry sectors are the main consumers of coal and oil and they are considered to be hard to decarbonize [43], although not impossible. For carbon neutral transport, Shafiei, Davidsdottir [7] highlighted the need for radical changes both on the supply side and demand side of the integrated energy-transport system. Using a dynamic simulation model, Shafiei, Davidsdottir [7] found that a carbon neutral transport system in Iceland is possible by 2050.  Newman, Kenworthy [44] and Newman and Kenworthy [45] highlighted the opportunity for rail and urban design changes to reduce emissions by 50% through reduced automobile dependence. Kołoś and Taczanowski [46] found light rail transport (LRT) to be a feasible option in medium sized towns in central Europe with a population between 100,000 to 300,000. Furthermore, there are cities with populations just over 100,000 having rail transport as a successful mode of transportation [47]. In the industry sector, ATECH is represented by oxy-combustion technology in the cement industry [28] and Maerz PFR kiln system in the calcium carbide industry [48].

- MF30 represents ‘Modern Fuel by 2030’, which intends to capture the provisions on universal access to modern renewable energy for residential and commercial sectors reflected under Sustainable Development Goal 7 [49].

- The F8050 represents reducing Fossil fuel usage by 80% by 2050 in accordance with IPCC [50] and it is applied to all of the demand sectors.

Exclusive references for the appendix

1.	NSB. Statistical Year Book of Bhutan 2015. Thimphu, Bhutan: National Statistics Bureau; 2015.

2.	DoE. Integrated Energy Master Plan for Bhutan. Thimphu, Bhutan: Department of Energy; 2010.

3.	Yangka D, Diesendorf M. Modeling the benefits of electric cooking in Bhutan: A long term perspective. Renewable and Sustainable Energy Reviews. 2016;59:494-503.

4.	Gaba KM, Cormier CJ, Rogers JA. Energy Intensive Sectors of the Indian economy: path to low carbon development. Washington D.C,: World Bank, Bank W; 2011.  Contract No.: 54607-IN.

5.	NEC. Second National Communication to the UNFCCC. Thimphu, Bhutan: National Environment Commission; 2011.

6.	Rubin ES, Azevedo IML, Jaramillo P, Yeh S. A review of learning rates for electricity supply technologies. Energy Policy. 2015;86:198-218.

7.	Shafiei E, Davidsdottir B, Leaver J, Stefansson H, Asgeirsson EI. Energy, economic, and mitigation cost implications of transition toward a carbon-neutral transport sector: A simulation-based comparison between hydrogen and electricity. Journal of Cleaner Production. 2017;141:237-47.

8.	BPC. Electricity consumption_monthly_EDCD_BPC. Thimphu: Bhutan Power Corporation Limited; 2017.

9.	NSB. Population projections of Bhutan 2005-2030. Thimphu, Bhutan: National Statistical Bureau; 2005.

10.	NSB. National accounts statistics. Thimphu: National Statistical Bureau; 2015.

11.	NSB. Bhutan national accounts statistics. THimphu: National Statistical Bureau; 2004.

12.	BPC. BPC Power Data Book 2015. Thimphu, Bhutan: Bhutan Power Corporation Limited; 2015.

13.	Druk Green Power Corporation. Hydropower Plants_project finance Thimphu, Bhutan: Druk Green webpage;  [Available from: https://www.drukgreen.bt.

14.	DRE. Bhutan Energy Data Directory. Thimphu, Bhutan: Department of Renewable Energy; 2015.

15.	Indian Embassey. Ongoing hydropower projects Thimphu, Bhutan2016 [Available from: https://www.indianembassythimphu.bt/pages.php?id=36.

16.	MHPA. Status report: September 2016. Thimphu, Bhutan: Mangdechu Hydroelectric Project Authority, Authority MHP; 2016.

17.	TERI. Reaching the Sun with Rooftop Solar. Delhi: TERI and Shakti Sustainable Energy Foundation; 2015.

18.	BPC. Data on wind power. 2016.

19.	Oladokun VO, Asemota OC. Unit cost of electricity in Nigeria: A cost model for captive diesel powered generating system. Renewable and Sustainable Energy Reviews. 2015;52:35-40.

20.	Druk Green Power Corporation. Annual Report 2014. Thimphu, Bhutan: Druk Green Power Corporation Limited, Limited DGPC; 2014.

21.	UNDP. Bhutan Sustainable Rural Biomass Energy Project. Thimphu, Bhutan: United Nations Development Programme; 2012.

22.	DoE. Bhutan Energy Data Directory 2005. Thimphu, Bhutan: Department of Energy; 2007.

23.	DRE. Bhutan Building energy efficiency study-Part 1 (Main Report). Thimphu, Bhutan: Department of Renewable Energy; 2015.

24.	MoIC. Intelligent Transport Systems (ITS) Feasibility Study and Preparation of the Comprehensive ITS action plan for Thimphu City. Thimphu, Bhutan: Ministry of Information and Communications; 2015.

25.	IUT India. Life Cycle Cost Analysis of Five Urban Transport Systems. New Delhi, India: Institute of Urban Transport (India); 2012.

26.	Zhu D, Patella DP, Steinmetz R, Peamsilpakulchorn P. Bhutan electric vehicle initiative: Scenarios, Implications and economic impact. Washington D.C: World Bank Group; 2016.

27.	MoIC. Annual Info-Comm and Transport statistical Bulletin. Thimphu, Bhutan: Ministry of Information and Communications; 2015.

28.	ETSAP. Technology brief 103: Cement Production. Energy Technology System Analysis Programme: International Energy Agency; 2010.  Contract No.: Technology Brief 103.

29.	DoI. Industry Report. Thimphu, Bhutan; 2015.

30.	DRE, UNDP. Feasibility study on sawdust briquetting for sawmills in Bhutan. Department of Renewable Energy; 2014.

31.	DRE. Bhutan energy efficiency baseline study. Thimphu, Bhutan: Department of Renewable Energy; 2012.

32.	NSB. Bhutan Living standard survey report. Thimphu, Bhutan; 2012.

33.	NSB and ADB. Bhutan Living Standards Survey 2012 Report. Bhutan: National Statistics Bureau and Asian Development Bank; 2013. Available from: http://www.nsb.gov.bt/publication/files/pub1tm2120wp.pdf.

34.	US EIA. International Energy Outlook 2016. US Energy Information Administration, Administration EI; 2016.  Contract No.: DOE/EIA-0484(2016).

35.	Yangka D, Rauland V, Newman P. Carbon neutral policy in action: the case of Bhutan. Climate Policy. 2018:1-16.

36.	Shrestha S, Bajracharya AR, Babel MS. Assessment of risks due to climate change for the Upper Tamakoshi Hydropower Project in Nepal. Climate Risk Management. 2016;14:27-41.

37.	RGoB. Economic Development Policy. Thimphu: Royal Government of Bhutan; 2016.

38.	Benhelal E, Zahedi G, Shamsaei E, Bahadori A. Global strategies and potentials to curb CO2 emissions in cement industry. Journal of Cleaner Production. 2013;51:142-61.

39.	Huisingh D, Zhang Z, Moore JC, Qiao Q, Li Q. Recent advances in carbon emissions reduction: policies, technologies, monitoring, assessment and modeling. Journal of Cleaner Production. 2015;103:1-12.

40.	Kajaste R, Hurme M. Cement industry greenhouse gas emissions – management options and abatement cost. Journal of Cleaner Production. 2016;112:4041-52.

41.	Morrow WR, Hasanbeigi A, Sathaye J, Xu T. Assessment of energy efficiency improvement and CO2 emission reduction potentials in India's cement and iron & steel industries. Journal of Cleaner Production. 2014;65:131-41.

42.	Yu F, Han F, Cui Z. Reducing carbon emissions through industrial symbiosis: a case study of a large enterprise group in China. Journal of Cleaner Production. 2015;103:811-8.

43.	Rogelj J, Luderer G, Pietzcker RC, Kriegler E, Schaeffer M, Krey V, et al. Energy system transformations for limiting end-of-century warming to below 1.5 °C. Nature Climate change. 2015.

44.	Newman P, Kenworthy J, Glazebrook G. Peak Car Use and the Rise of Global Rail: Why This Is Happening and What It Means for Large and Small Cities. Journal of Transportation Technologies. 2013;03(04):272-87.

45.	Newman P, Kenworthy J. The End of Automobile Dependence - How Citites are Moving Beyond Car-based Planning. Washington DC: Island Press; 2015.

46.	Kołoś A, Taczanowski J. The feasibility of introducing light rail systems in medium-sized towns in Central Europe. Journal of Transport Geography. 2016;54:400-13.

47.	Newman P, Kenworthy J, Glazebrook G. Peak Car Use and the Rise of Global Rail: Why this is happening and what it means for large and small cities. Journal of Transportation Technologies. 2013;3(4).

48.	Liu X, Zhu B, Zhou W, Hu S, Chen D, Griffy-Brown C. CO2 emissions in calcium carbide industry: An analysis of China's mitigation potential. International Journal of Greenhouse Gas Control. 2011;5(5):1240-9.

49.	United Nations. Transforming our world: The 2030 agenda for sustainable development. The United Nations; 2015.

50.	IPCC. Climate Change 2014: Synthesis Report. Contribution of Working Groups I, II and III to the Fifth Assessment Report of the Intergovernmental Panel on Climate Change [Core Writing Team, R.K. Pachauri and L.A. Meyer (eds.)]. Geneva, Switzerland: Intergovernmental Panel on Climate Change,; 2014.