# APPENDIX

List of Main Equations Used in the System Dynamics Model

CO2 Emissions section

1.	Emission factor from construction (t CO2)

EFC=(EFR* NCR) + (EFT * NCT) + (EFB * NCB) 1000000

where

EFC is emission factor for construction,

EFR is emission factor for renewables,

EFT is averaged emission factor for traditional power plants (natural gas and coal),

EFB is emission factor for batteries,

NCR is new construction (renewables),

NCT is new construction (coal and natural gas),

NCB are new batteries.

2.	Emission factor for operations (t CO2)

EFO  = (((GenT * (((ShC * 100) / (ShNG + ShC)) /100)) * EFC) + ((GenT * (((ShNG * 100) / (ShNG + ShC)) /100)) * EFNG) + (GenN * EFN) + ((GenR * (((ShS * 100) / (ShW + ShS)) /100)) * EFS) + ((GenR * (((ShW * 100) / (ShW + ShS)) /100)) * EFW)) / 1000000

where

EFO is emission factor for operations,

GenT is coal/natural gas electricity generated,

GenN is nuclear electricity generated,

GenR is renewable electricity generated,

ShC is share of coal plants,

ShNG is share of natural gas plants,

ShS is share of solar plants,

ShW is share of wind plants,

EFC is emission factor for coal,

EFNG is emission factor for natural gas,

EFN is emission factor for nuclear,

EFS is emission factor for solar,

EFW is emission factor for wind.

3.	Accumulated CO2 emissions (t CO2)

Accumulated CO2 emissions = EFO + EFC

Economics section

1.	Sales (MWh/year)

Sales are calculated differently depending on the value of surplus.

If surplus is positive (i.e., Surplus>0),

Sales=G*(1=0.01*Surplus)

In the opposite case (i.e., Surplus≤0),

Sales=G

where G is central electricity generation.

2.	Profit from sales ($)

PS=Sales*Pe

where

PS is profit from sales,

Pe is wholesale electricity price.

3. 	Wholesale electricity price ($/MWh)

Wholesale electricity price Pe is introduced as a random variable with Gaussian distribution G(x,5),

where mean x takes one of the following values: (20, 40, 50, 60, 80, 120) depending on the supply-demand equilibrium.

4. 	Capital expenditures ($)

CAPEX=NCT*PT+NCR*PR+NCB*PB+De*PR

where

NCT is new construction (coal and natural gas),

NCR is new construction (renewables),

NCB is new batteries,

PT is price of construction (coal and natural gas),

PR is price of construction (renewables),

PB is price of batteries,

De is switch to decentralized solutions.

Community decentralized energy section

- Interest in distributed solutions

Interest= Pe*AccCO2*Potential

where

Interest is interest in distributed solutions,

Pe is wholesale electricity price,

AccCO2 is accumulated CO2 emissions,

Potential is prosumers potential.

Electricity demand section

- Surplus

Surplus=100*(G-D)D

where

G  is central electricity generation,

D is electricity demand.

- Demand growth

Dgrowth=Cave*N*(1+0.01*Loss)

where

Dgrowth  is demand growth,

Cave is a household consumption,

N is population,

Loss is transmission losses.

Electricity supply section

- Existing production capacity (coal and natural gas) (MWh)

CapT=0.01*Dgrowth* ShC+ShNG+Shother*1-1CapLifeT

where

CapT  is existing production capacity (coal and natural gas),

Dgrowth  is demand growth,

ShC is share of coal plants,

ShNG is share of natural gas plants,

Shother is share of other plants,

CapLifeT is coal and natural gas capacity lifespan.

- Existing production capacity (nuclear) (MWh)

Capn=0.01*Dgrowth* Shn*1-1CapLifen

where

Capn is existing production capacity (nuclear),

Dgrowth  is demand growth,

Shn is share of nuclear plants,

CapLifen is nuclear capacity lifespan.

- Existing production capacity (renewables) (MWh)

CapR=0.01*Dgrowth* ShS+ShW*1-1CapLifeR

where

CapR is existing production capacity (renewables),

Dgrowth  is demand growth,

ShS is share of solar plants,

ShW is share of wind plants,

CapLifeR is renewables capacity lifespan.

- New construction (coal and natural gas) (MWh)

New construction (coal and natural gas) is calculated as

NCT=0.01*Dgrowth*ShC-32+ShNG-27.37

where

NCT is new construction (coal and natural gas),

Dgrowth  is demand growth,

ShC is share of coal plants,

ShNG is share of natural gas plants,

if the following condition is satisfied in the model:

0.01*Dgrowth*ShC-32+ShNG-27.37>0

Otherwise, the construction of new coal and natural gas plants is not being implemented, and the variable NCT takes zero value.

- New construction (nuclear) (MWh)

New construction (nuclear) is calculated as

NCn=0.01*Dgrowth*Shn-35.68

where

NCn is new construction (nuclear),

Dgrowth  is demand growth,

Shn is share of nuclear plants,

if the following condition is satisfied in the model:

0.01*Dgrowth*Shn-35.68>0

Otherwise, the construction of new nuclear plants is not being implemented, and the variable NCntakes zero value.

- New construction (renewables) (MWh)

NCR=0.01*Dgrowth*ShS-0.1837+ShW-2.899

where

NCR is new construction (renewables),

Dgrowth  is demand growth,

ShS is share of solar plants,

ShW is share of wind plants.

Share of plants for base scenario is based on the ComEd real data form 5-year period and further projections:

- Share of Natural gas plants (%)

ShNG=8.9807*lnt+27.365

where

ShNG is share of natural gas plants,

t represents time in years and takes integer values from 1 (corresponds to the year of 2017) to 50 (corresponds to the year of 2067).

- Share of Coal plants (%)

ShC=31.995*t-0.316

where

ShC is share of coal plants,

t represents time in years and takes integer values from 1 (corresponds to the year of 2017) to 50 (corresponds to the year of 2067).

- Share of Nuclear plants (%)

Shn=35.684*t-0.034

where

Shn is share of nuclear plants,

t represents time in years and takes integer values from 1 (corresponds to the year of 2017) to 50 (corresponds to the year of 2067).

- Share of Solar plants (%)

ShS=0.5458*lnt+0.1837

where

ShS is share of solar plants,

t represents time in years and takes integer values from 1 (corresponds to the year of 2017) to 50 (corresponds to the year of 2067).

- Share of Wind plants (%)

ShW=2.899*t0.0427

where

ShW is share of wind plants,

t represents time in years and takes integer values from 1 (corresponds to the year of 2017) to 50 (corresponds to the year of 2067).

Control variable (not included in the model and used as a hidden variable)

- Supply/demand equilibrium

SDeq=D-S

where

SDeq is supply/demand equilibrium,

D is demand,

S  is supply.