1. Method of ecological risk and health risk assessment

1.1 Ecological risk assessment

The ecological risk assessment of heavy metal(loids) in Jiangmen city was calculated according to the pollution index (Pi) of each element and the Nemerow synthetical pollution index (Pn). Equation (1) shows the formulation of Pn.

(1)

(2)

where Pi is the pollution index of each element, and i represents different trace elements. Ci is the measured content, and Si is the reference value. For each metal, the Pi was calculated and the pollution classification were defined as safety (Pn≤0.7), precaution (0.7 < Pn ≤ 1.0), slightly polluted (1.0 < Pn ≤ 2.0), moderately polluted (2.0 < Pn ≤ 3.0), and severely polluted (Pn > 3.0).

The potencial ecological risk index (RI) reflects the susceptibility of biomes to pollutants, which indicating the potential ecological risks of pollutants (Wu et al., 2020). It can be calculated as following equation:

(3)

where n is the number of trace metals, represents potential ecological index for a single element. is the toxic response factor, Ci and Si have the same meaning as in Eq. (2). According to RI, low risk (RI ≤ 150), moderate risk (150＜RI ≤ 300), considerable risk (300＜RI ≤ 600), and high risk (RI > 600) were identified.

1.1 Source oriented daily dose calculation

Generally, ingestion, inhalation and dermal contact are the main exposure pathways for children and adults. The source oriented health risk was calculated according to Liu et al. (2022). The equations are as follows:

................................................(1)

................................................(2)

...............................(3)

..................................................................(4)

........................................................................(5)

................................................................(6)

.......................................................................(7)

Where Cik is the concentration of ith from the kth source of pollutant i. Ring is the inhalation rate, Ring is the soil ingestion rate. EF, ED, BW, AT, PEF, SA, AF, and ABF are the exposure frequency, exposure duration, body weight, average life span, soil particle emission factor, dermal exposure area, dermal adherence factor, and Dermal adsorption factor, respectively. CRi represents the lifetime carcinogenic risk level of carcinogenic species i; CR is the sum of the Ri values; ADDi is the average daily dose, and SFi represents the slope factor of the ith pollutant. HQi is the non-cancer risk level of pollutant i; HQ is the total non-cancer risks; and RfDi means the reference dose.The related reference values are shown in Table S1.

Table S1 Parameters and values used in the human health risk models

| Factor | Definition | Unit | Value |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Children | Adult |  |
| Ring | Ingestion rate of soil | mg/day | 200 | 100 | MEP, 2014 |
| EF | Exposure frequency | days/year | 252 | 252 | Yang et al., 2014 |
| ED | Exposure duration | years | 6 | 25 | MEP, 2014 |
| BW | Average body weight | kg | 15.9 | 56.8 | Chen et al., 2012 |
| AT | Average time | days | 9125 (non-carcinogen) |  | MEP, 2014 |
|  |  |  | 26,280 (carcinogen) |  |  |
| Rinh | Inhalation rate | m3/day | 7.5 | 14.5 | MEP, 2014 |
| PEF | Particle emission factor | m3/kg | 1.36 × 109 | 1.36 × 109 | US DOE, 2011 |
| SA | Exposed skin area | cm2 | 2800 | 5700 | US DOE 2011 |
| AF | Dermal adherence factor | mg/cm2 | 0.49 | 0.65 | Wu et al., 2020 |
| ABF | Dermal adsorption factor | unitless | 0.001 (non-carcinogen) |  | Shi et al.,2011 |
|  |  |  | 0.01 (carcinogen) |  |  |
| SF | Carcinogeicity slope factor | Per （mg/kg）day | Inh SF: 6.3(Cd), 15.1(As), 42(Cr), 0.84(Ni) |  | Wu et al., 2020 |
| RfD | Chronic reference dose | mg/kg/day | Oral RfD: 0.001(Cd), 0.0003(Hg), 0.0003(As), 0.003(Pb), 0.003(Cr), 0.02(Ni), 0.04(Cu), 0.3(Zn) Dermal RfD: 1.00E-05(Cd), 2.10E-05(Hg), 1.23E-04(As), 5.25E-04(Pb),6.00E-05(Cr), 5.40E-03(Ni), 1.20E-02(Cu), 6.00E-02(Zn) Inhal RfD: 8.57E-05(Hg), 2.86E-05(Cr) |  | Wu et al., 2020 |

Table S2 Statistics of heavy metal(loids) concentrations in soil of different area in Jiangmen city

| sample area | sample number | statistics | Items |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | As | Cd | Cr | Cu | Hg | Ni | Pb | Zn |
| Jianghai | 5 | average/mg·kg-1 | 11.9 | 0.79 | 485 | 54.6 | 0.13 | 39.6 | 60.4 | 134 |
|  |  | maximun/mg·kg-1 | 14.9 | 2.50 | 2154 | 87.9 | 0.19 | 84.9 | 85.5 | 221 |
|  |  | minimun/mg·kg-1 | 6.60 | 0.11 | 30.5 | 24.5 | 0.03 | 13.3 | 42.7 | 45.7 |
|  |  | standard deviation | 3.13 | 0.98 | 934 | 27.7 | 0.06 | 30.3 | 17.6 | 69.4 |
|  |  | variable coefficient (%) | 26 | 125 | 193 | 51 | 49 | 77 | 29 | 52 |
| Pengjiang | 9 | average/mg·kg-1 | 16.2 | 0.38 | 44.8 | 41.4 | 0.26 | 19.1 | 60.5 | 226 |
|  |  | maximun/mg·kg-1 | 29.1 | 1.55 | 91.3 | 157 | 1.00 | 50.8 | 173 | 1188 |
|  |  | minimun/mg·kg-1 | 7.29 | 0.02 | 17.8 | 10.9 | 0.00 | 7.34 | 22.3 | 24.1 |
|  |  | standard deviation | 6.97 | 0.47 | 20.8 | 44.8 | 0.35 | 13.2 | 44.3 | 370 |
|  |  | variable coefficient (%) | 43 | 122 | 47 | 108 | 133 | 69 | 73 | 164 |
| Xinhui | 16 | average/mg·kg-1 | 11.5 | 0.28 | 58.3 | 42.2 | 0.18 | 21.5 | 62.7 | 115 |
|  |  | maximun/mg·kg-1 | 22.7 | 0.57 | 146 | 140 | 1.42 | 45.9 | 146 | 247 |
|  |  | minimun/mg·kg-1 | 3.55 | 0.05 | 12.3 | 9.46 | 0.02 | 4.52 | 12.1 | 17.0 |
|  |  | standard deviation | 7.24 | 0.17 | 38.6 | 37.8 | 0.34 | 14.7 | 32.2 | 64.7 |
|  |  | variable coefficient (%) | 63 | 61 | 66 | 90 | 182 | 68 | 51 | 56 |
| Taishan | 33 | average/mg·kg-1 | 12.4 | 0.15 | 32.4 | 18.0 | 0.11 | 10.4 | 41.6 | 67.8 |
|  |  | maximun/mg·kg-1 | 54.2 | 1.53 | 86.9 | 66.6 | 0.69 | 29.0 | 125 | 206 |
|  |  | minimun/mg·kg-1 | 2.34 | 0.03 | 9.54 | 6.72 | 0.01 | 3.39 | 13.2 | 29.3 |
|  |  | standard deviation | 13.9 | 0.25 | 21.0 | 14.3 | 0.12 | 6.07 | 22.8 | 35.2 |
|  |  | variable coefficient (%) | 112 | 164 | 65 | 80 | 107 | 58 | 55 | 52 |
| Kaiping | 19 | average/mg·kg-1 | 8.77 | 0.12 | 42.2 | 33.3 | 0.08 | 13.0 | 36.3 | 81.9 |
|  |  | maximun/mg·kg-1 | 53.2 | 0.44 | 101 | 277 | 0.23 | 43.0 | 92.4 | 298 |
|  |  | minimun/mg·kg-1 | 2.09 | 0.02 | 18.1 | 7.08 | 0.02 | 4.84 | 11.7 | 17.8 |
|  |  | standard deviation | 11.5 | 0.09 | 24.5 | 60.5 | 0.06 | 9.40 | 24.3 | 77.5 |
|  |  | variable coefficient (%) | 131 | 78 | 58 | 181 | 72 | 72 | 67 | 95 |
| Enping | 17 | average/mg·kg-1 | 7.44 | 0.16 | 41.4 | 16.0 | 0.06 | 10.0 | 42.0 | 56.1 |
|  |  | maximun/mg·kg-1 | 18.2 | 0.74 | 89.3 | 33.5 | 0.11 | 23.8 | 117 | 105 |
|  |  | minimun/mg·kg-1 | 1.35 | 0.03 | 14.2 | 7.72 | 0.01 | 5.50 | 8.74 | 19.3 |
|  |  | standard deviation | 5.06 | 0.16 | 22.3 | 5.93 | 0.03 | 4.42 | 25.0 | 23.9 |
|  |  | variable coefficient (%) | 68 | 105 | 54 | 37 | 55 | 44 | 59 | 43 |
| Heshan | 14 | average/mg·kg-1 | 10.9 | 0.13 | 40.7 | 26.4 | 0.04 | 13.6 | 65.2 | 79.5 |
|  |  | maximun/mg·kg-1 | 19.7 | 0.34 | 80.7 | 136 | 0.07 | 24.9 | 160 | 243 |
|  |  | minimun/mg·kg-1 | 4.75 | 0.03 | 19.1 | 11.0 | 0.01 | 7.43 | 32.3 | 34.3 |
|  |  | standard deviation | 4.45 | 0.10 | 19.0 | 32.0 | 0.02 | 5.72 | 30.4 | 51.3 |
|  |  | variable coefficient (%) | 41 | 72 | 47 | 121 | 49 | 42 | 47 | 65 |
| Jiangmen city | 113 | average/mg·kg-1 | 11.00 | 0.21 | 61.09 | 28.22 | 0.11 | 14.72 | 49.03 | 92.08 |
|  |  | maximun/mg·kg-1 | 54.22 | 1.55 | 2154.24 | 276.67 | 1.42 | 84.91 | 172.85 | 1188.22 |
|  |  | minimun/mg·kg-1 | 1.35 | 0.02 | 9.54 | 6.72 | 0.004 | 3.39 | 8.74 | 17.03 |
|  |  | standard deviation | 9.94 | 0.23 | 200.41 | 35.56 | 0.18 | 12.28 | 29.27 | 119.31 |
|  |  | variable coefficient (%) | 90 | 121 | 328 | 126 | 20 | 83.4 | 59.7 | 129 |
| Background value* |  |  | 9.7 | 0.074 | 68.1 | 23.6 | 0.07 | 29 | 25.40 | 102.6 |
| Percentage exceed background values |  |  | 42.5% | 78.8% | 18.6% | 30.1% | 46.9% | 12.4% | 80.5% | 23% |

Table S3 Classification criterions for Nemerow synthetical pollution index (Pn) and Potential ecological risk index (RI).

| Nemerow synthetical pollution index (Pn) |  |  | Potential ecological risk index (RI) |  |  |
| --- | --- | --- | --- | --- | --- |
| Grade |  | Appraisal result | Grade |  | Appraisal result |
| 1 | Pn ≤ 0.7 | Safety | 1 | RI ≤ 150 | Low risk |
| 2 | 0.7＜Pn ≤ 1.0 | Precaution | 2 | 150＜RI ≤ 300 | Moderate risk |
| 3 | 1.0＜Pn ≤ 2.0 | Slightly polluted | 3 | 300＜RI ≤ 600 | Considerable risk |
| 4 | 2.0＜Pn ≤ 3.0 | Moderately polluted | 4 | RI > 600 | High risk |
| 5 | Pn > 3.0 | Severely polluted |  |  |  |

Table S4 Hazard quotients and lifelong carcinogenic risk for the metal(loid)s analyzed in this study

| Pollution source | HQ |  | CR |  |
| --- | --- | --- | --- | --- |
|  | children | adult | children | adult |
| Traffic and ship emissions | 5.44E-03 | 4.59E-02 | 1.69E-08 | 3.81E-08 |
| Industrial emission | 8.78E-03 | 7.40E-02 | 2.73E-08 | 6.15E-08 |
| Mixed source | 3.34E-03 | 2.81E-02 | 1.04E-08 | 2.34E-08 |
| Total | 1.76E-02 | 1.48E-01 | 5.45E-08 | 1.23E-07 |

Fig. S1 Factor profiles and source contributions of toxic elements from the PMF model.

Fig. S2 Spatial distribution of roads, factories and sampling sites in Jiangmen city.

Reference

Chen J W, Wang S L, Hsieh D, et al., 2012. Carcinogenic potencies of polycyclic aromatic hydrocarbons for back-door neighbors of restaurants with cooking emissions. Sci. Total Environ. 417: 68-75.

China National Environmental Monitoring Centre. Background Value of Soil Elements in China. Beijing: China Environmental Science Press, 1990.

Hothorn, T., Hornik, K., Zeileis, A., 2006. Unbiased recursive partitioning: A conditional inference framework. J. Comput. J. Comput. Craph. Stat. 15: 651-674.

Liu Q Y, Zhao W H, Ma J, 2022. Spatial clustering and source-specific risk of combined pollutants in soils from an industrial area in Shanxi Province, China. Environ. Pollut. 299: 118925.

MEP (Ministry of Environmental Protection of China), 2014. Technical Guidelines for Risk Assessment of Contaminated Sites. HJ 25.3-2014. Ministry of Environ- mental Protection of China (in Chinese).

Shi, G.T., Chen, Z.L., Bi, C.J., Wang, L., et al., 2011. A comparative study of health risk of potentially toxic metals in urban and suburban road dust in the most populated city of China. Atmos. Environ. 45, 764-771.

Strobl, C., et al., 2007. Bias in random forest variable importance measures: illustrations, sources and a solution. BMC Bioinformatics. 8.

US DOE (United States Department of Energy), 2011. In: The Risk Assessment Information System (RAIS). U. U.S Department of Energy’s Oak Ridge Operations Office (ORO).

Wang, Q., et al., 2016. Identification and apportionment of the drivers of land use change on a regional scale: Unbiased recursive partitioning-based stochastic model application. Agr. Ecosyst. Environ. 217: 99-110.

Wu H W, Liu Q Y, Ma J, et al., 2020. Heavy Metal(loids) in typical Chinese tobacco-growing soils: Concentrations, influence factors and potential health risks. Chemosphere. 245: 125591.

Yang W, Lang Y, Li G, 2014. Cancer risk of polycyclic aromatic hydrocarbons (PAHs) in the soils from Jiaozhou Bay wetland. Chemosphere. 112: 289-295.