Supplementary Material

Atomistic Molecular Dynamics Simulation and COSMO-SAC Approach for Enhanced 1,3-Propanediol Extraction with Imidazolium-Based Ionic Liquids.

Raj Akshata, Anand Bhartib* and Padmini Padmanabhanc*

aDepartment of Bioengineering and Biotechnology, Birla Institute of Technology, Ranchi, India; bDepartment of Chemical Engineering, Birla Institute of Technology, Ranchi, India;                 cDepartment of Bioengineering and Biotechnology, Birla Institute of Technology, Ranchi, India

* Corresponding author: E-mail: abharti@bitmesra.ac.in, padmini@bitmesra.ac.in

Tel: +91-8011097281, +91 7550008592

| Sl. No. | Title of the table/figure | Page no. |
| --- | --- | --- |
| Figure S1 | Molecular Structures of System Compounds: Optimized Configurations. | S5 |
| Figure S2 | Validation of COSMO-Generated Sigma Profile against Reference Data. | S6 |
| Figure S3 | Atom types for the 1-alkyl-3-methylimidazolium. | S7 |
| Table S1 | Charges unscaled and scaled by 0.8. | S7 |
| Table S2 | Non-Bonded Parameters σ(Å) and є (Kcal/mol), Bond force constants r0 (Å) Kr (Kcal mol-1Å-2) and Angle force constants θ0 (degrees) Kθ (Kcal mol-1rad-2), for cation 1-Butyl-3-methylimidazolium (Bmim). | S8 |
| Table S3 | Torsional Fourier Coefficients (Kcal/mol) for 1-Butyl-3-methylimidazolium [Bmim]. | S9 |
| Figure S4 | Atom Type Representation of Anions. (a) Bis(pentafluoroethanesulfonyl)amide [NPF2] (b) Bis[(trifluoromethyl)sulfonyl]imide [NTF2] (c) Thiocyanate [SCN] and (d) Trifluoromethanesulfonate [TFO]. | S10 |
| Table S4 | Non-Bonded parameters σ(Å) and є (Kcal/mol), for selected anions. | S11 |
| Table S5 | Bond force constants r0 (Å) and Kr (Kcal mol-1Å-2) , for anions. | S12 |
| Table S6 | Angle force constants θ0 (degrees) and Kθ (Kcal mol-1rad-2), for selected Anions. | S13 |
| Table S7 | Torsional Fourier Coefficients (Kcal/mol) for selected Anions. | S14 |
| Figure S5 | Atom Type Representation of 1,3-Propanediol (1,3-PDO). | S15 |
| Table S8 | Charges of 1,3-Propanediol: Unscaled and Scaled by 0.8. | S15 |
| Table S9 | Non-Bonded Parameters σ(Å) and є (Kcal/mol), Bond force constants r0 (Å) Kr (Kcal mol-1Å-2) of 1,3-PDO. | S16 |
| Table S10 | Angle force constants θ0 (degrees) Kθ (Kcal mol-1rad-2), for 1,3-PDO. | S17 |
| Table S11 | Torsional Fourier Coefficients (Kcal/mol) for 1,3-PDO. | S18 |
| Table S12 | Distribution of components in extract and raffinate phases from MD simulation at 298 K and 1 atm. | S19 |
| Figure S6 | Density Profile Validation Using OPLS-AA Force Fields over 12ns Simulation at 298.15 K. | S20 |
| Figure S7 | Validation of RDF Plot for [Bmim][NPF2] | S21 |
| Figure S8 | Validation of RDF Plot for [Bmim][NTF2] | S22 |
| Figure S9 | Validation of RDF Plot for [Bmim][SCN] | S23 |
| Figure S10 | Validation of RDF Plot for [Bmim][TFO] | S24 |
| Figure S11 | Spatial Distribution Function (SDF) Analysis of [Bmim][NPF2]: Molecular Density Distribution. (a) and (b) show the vertical and horizontal views of [NPF2] around [Bmim] cation. Similarly, (c) and (d) display [Bmim] cation around [Bmim] cation. Cyan color represents cation [Bmim] density, while purple depicts [NPF2] anion density. | S25 |
| Figure S12 | Spatial Distribution Function (SDF) Analysis of [Bmim][SCN]: Molecular Density Distribution. (a) and (b) display vertical and horizontal views of [SCN] around [Bmim] cation, while (c) and (d) depict [Bmim] cation around [Bmim] cation. Cyan color indicates [Bmim] cation density, and red signifies [SCN] anion density. | S26 |
| Figure S13 | Spatial Distribution Function (SDF) Analysis of [Bmim][TFO]: Molecular Density Distribution. (a) and (b) show vertical and horizontal views of [TFO] around [Bmim] cation, while (c) and (d) depict [Bmim] cation around [Bmim] cation. Cyan color represents [Bmim] cation density, and lime signifies [TFO] anion density. | S27 |
| Figure S14 | Spatial Distribution Function (SDF) visualisations of ILs around 1,3-PDO, Color code: Cyan for [Bmim]+ cation, Purple for [NPF2]-, Blue for [NTF2]-, Red for [SCN]-, and Lime for [TFO]-. (a) SDF of [NPF2]- and [Bmim]+ around 1,3-PDO in [Bmim][NPF2], (b) SDF of [NTF2]- and [Bmim]+ around 1,3-PDO in [Bmim][NTF2], (c) SDF of [SCN]- and [Bmim]+ around 1,3-PDO in [Bmim][SCN] and (d) SDF of [TFO]- and [Bmim]+ around 1,3-PDO in [Bmim][TFO]. | S28 |
| Figure S15 | Molecular Distribution Analysis and Interatomic RDF of [Bmim][NTF2] with 1,3-PDO. (a) SDF of 1,3-PDO around [NTF2]-, (b) SDF of 1,3-PDO around [Bmim]+, (c) SDF of [NTF2]- around 1,3-PDO, (d) SDF of [Bmim]+ around 1,3-PDO, and (e) Interatomic RDF Plot of ILs and 1,3-PDO at 298 K. | S29 |
| Figure S14 | This figure displays the average number of hydrogen bonds observed after equilibration at 298 K. (a) H-Bonding between ILs and 1,3-PDO and (b) H-Bonding involving ILs anions and 1,3-PDO. | S30 |
| Figure S15 | Comparative Analysis of Water Entry in Various Ionic Liquids (ILs) at 298 K. Distinguished by colors (a) [Bmim][NPF2] in purple, (b) [Bmim][NTF2] in blue, (c) [Bmim][SCN] in red, and (d) [Bmim][TFO] in lime—the figure unveils the unique behavior of each IL in accommodating water molecules under specified conditions. | S31 |

Figure S1. Molecular Structures of System Compounds: Optimized Configurations.

Figure  S2. Validation of COSMO-Generated Sigma Profile against Reference Data (1).

OPLS-AA Force field of 1-butyl-3-methylimidazolium cation (2,3)

TABLE S1. Charges unscaled and scaled by 0.8.

| Atom type | q (e) 0.8* q (e) |  |
| --- | --- | --- |
| Cm | -0.35 | -0.280 |
| Na | 0.22 | 0.176 |
| Cr | -0.09 | -0.072 |
| Cw | -0.24 | -0.192 |
| Ca | -0.17 | Figure S3. Atom types for the 1-alkyl-3-methylimidazolium. -0.136 |
| Cs | -0.12 | -0.096 |
| Ct | -0.24 | -0.192 |
| Hm | 0.18 | 0.144 |
| Hr | 0.21 | 0.168 |
| Hw | 0.27 | 0.216 |
| Ha | 0.18 | 0.144 |
| Hs | 0.06 | 0.048 |
| Ht | 0.08 | 0.064 |

TABLE S2. Non-Bonded Parameters σ(Å) and є (Kcal/mol), Bond force constants r0 (Å) Kr (Kcal mol-1Å-2)  and Angle force constants θ0 (degrees) Kθ (Kcal mol-1rad-2), for cation 1-Butyl-3-methylimidazolium [Bmim]+.

| Atom type; σ(Å) є (Kcal/mol). Cm; 3.55; 0.066. Na; 3.25; 0.170. Cr; 3.55; 0.070. Cw; 3.55; 0.070. Ca; 3.50; 0.066. Cs; 3.50; 0.066. Ct; 3.50; 0.066. Hm; 2.50; 0.030. Hr; 2.42; 0.030. Hw; 2.42; 0.030. Ha; 2.50; 0.030. Hs; 2.50; 0.030. Ht; 2.50; 0.030. | Bonds; r0 (Å) Kr (Kcal mol-1Å-2). Cr-Na; 1.315; 477. Hm-Cm; 1.080; 340. Na-Cm; 1.465; 337. Cr-Hr; 1.069; 367. Cw-Na; 1.378; 427. Cw-Cw; 1.336; 520. Cw-Hw; 1.068; 367. Na-Ca; 1.476; 427. Ca-Ha; 1.080; 520. Ca-Cs; 1.526; 367. Cs-Hs; 1.087; 337. Cs-Cs; 1.531; 340. Cs-Ct; 1.528; 268. Ct-Ht; 1.084; 340. Ca-Ct; 1.521; 268. |
| --- | --- |

| Angles; θ0 (degrees); Kθ (Kcal mol-1rad-2). Na-Ca-Ct; 112.6; 58.35. Ca-Cs-Ct; 113.1; 58.35. Ca-Ct-Ht; 110.7; 37.50. Ht-Ct-Ht; 107.9; 33.00. Cs-Ct-Ht; 111.1; 37.50. Cs-Cs-Cs; 112.3; 58.35. Cs-Cs-Ct; 112.3; 58.35. Hs-Cs-Hs; 106.7; 33.00. Hs-Cs-Cs; 109.6; 37.50. Hs-Cs-Ct; 109.7; 37.50. Ca-Cs-Cs; 113.3; 58.35. Cs-Cs-Hs; 108.6; 37.50. Ha-Ca-Ha; 108.9; 58.35. Ha-Ca-Cs; 111.1; 37.50. | Angles; θ0 (degrees); Kθ (Kcal mol-1rad-2). Na-Ca-Cs; 113.0; 70.00. Na-Ca-Ha; 107.5; 70.00. Cr-Na-Ca; 126.8; 35.00. Cw-Na-Ca; 125.3; 35.00. Hw-Cw-Cw; 130.9; 70.00. Na-Cw-Hw; 122.0; 70.00. Na-Cw-Cw; 107.1; 70.00. Cr-Na-Cw; 107.9; 35.00. Na-Cr-Na; 109.8; 70.00. Na-Cr-Hr; 125.1; 35.00. Cm-Na-Cw; 125.6; 70.00. Cm-Na-Cr; 126.4; 70.00. Hm-Cm-Na; 109.2; 37.50. Hm-Cm-Hm; 109.8; 33.00. |
| --- | --- |

TABLE S3. Torsional Fourier Coefficients (Kcal/mol) for 1-Butyl-3-methylimidazolium [Bmim].

| Torsion; V1; V2; V3. Hm-Cm-Na-Cr; 0.000; 0.000; 0.000. Hm-Cm-Na-Cw; 0.000; 0.000; 0.124. Cm-Na-Cr-Hr; 0.000; 4.651; 0.000. Cm-Na-Cr-Na; 0.000; 4.651; 0.000. Cm-Na-Cw-Cw; 0.000; 3.000; 0.000. Cm-Na-Cw-Hw; 0.000; 3.000; 0.000. Na-Cr-Na-Ca; 0.000; 4.651; 0.000. Na-Cw-Cw-Hw; 0.000; 10.750; 0.000. Na-Cw-Cw-Na; 0.000; 10.750; 0.000. Hr-Cr-Na-Cw; 0.000; 4.651; 0.000. Cr-Na-Cw-Hw; 0.000; 3.000; 0.000. Hr-Cr-Na-Ca; 0.000; 4.651; 0.000. Hw-Cw-Na-Ca; 0.000; 3.000; 0.000. Cw-Cw-Na-Ca; 0.000; 3.000; 0.000. Cr-Na-Ca-Ha; 0.000; 0.000; 0.000. Ca-Cs-Cs-Hs; 0.000; 0.000; 0.366. Ca-Cs-Cs-Ct; 1.300; -0.050; 0.200. Cw-Na-Ca-Ct; -4.355; -4.575; -1.375. Cw-Na-Ca-Ha; -2.700; -5.650; 0.355. Cr-Na-Ca-Cs; -0.159; 0.095; -0.010. Ha-Ca-Cs-Hs; 0.000; -0.150; 0.518. Na-Ca-Cs-Cs; -0.788; -0.800; 0.400. Cr-Na-Cw-Cw; 0.000; 3.000; 0.000. | Torsion; V1; V2; V3. Hs-Cs-Cs-Hs; 0.000; 0.000; 0.318. Hs-Cs-Ct-Ht; 0.000; 0.000; 0.318. Hs-Cs-Cs-Ct; 0.000; 0.000; 0.366. Ca-Cs-Cs-Cs; 1.300; -0.050; 0.200. Cs-Cs-Cs-Cs; 1.300; -0.050; 0.200. Cs-Cs-Cs-Ct; 1.300; -0.050; 0.200. Cs-Cs-Cs-Hs; 0.000; 0.000; 0.366. Cs-Cs-Ct-Ht; 0.000; 0.000; 0.366. Hw-Cw-Cw-Hw; 0.000; 10.750; 0.000. Na-Ca-Cs-Hs; 0.000; 0.000; 0.000. Ha-Ca-Cs-Cs; 0.000; 0.000; 0.366. Cw-Na-Cr-Na; 0.000; 4.651; 0.000. Ha-Ca-Ct-Ht; 0.000; 0.000; 0.318. Ca-Cs-Ct-Ht; 0.000; 0.000; 0.366. Na-Ca-Cs-Ct; -0.688; 0.650; 0.900. Ha-Ca-Cs-Ct; 0.000; 0.000; 0.366. Na-Cs-Ct-Ht; 0.000; 0.000; 0.000. Cr-Na-Ca-Ct; -2.000; -0.275; -1.650. Cr-Na-Ca-Cs; -2.110; -5.000; 0.345. X-Cw-X-Xa; 0.000; 2.200; 0.000. X-Cr-X-Xa; 0.000; 2.200; 0.000. X-Na-X-Xa; 0.000; 2.200; 0.000. |
| --- | --- |

OPLS-AA Force field of anions (3).

Figure S4. Atom Type Representation of Anions. (a) Bis(pentafluoroethanesulfonyl)amide [NPF2]- (b) Bis[(trifluoromethyl)sulfonyl]imide [NTF2]- (c) Thiocyanate [SCN]- and (d) Trifluoromethanesulfonate [TFO]-.

TABLE S4. Non-Bonded parameters σ(Å) and є (Kcal/mol), for selected anions.

| Anion | Atom type | q (e) | 0.8 * q (e) | σ (Å) | є (Kcal/mol) |
| --- | --- | --- | --- | --- | --- |
| [TFO]- | S | 1.1887 | 0.95096 | 3.550 | 0.250 |
|  | O | -0.6556 | -0.52448 | 2.960 | 0.210 |
|  | C | 0.2692 | 0.21536 | 3.500 | 0.066 |
|  | F | -0.1637 | -0.13096 | 2.950 | 0.053 |
| [SCN]- | N | -0.6452 | -0.51616 | 3.2 | 0.170 |
|  | C | 0.3263 | 0.26104 | 3.3 | 0.066 |
|  | S | -0.6811 | -0.54488 | 3.55 | 0.250 |
| [NTF2]- | F | -0.16 | -0.128 | 2.95 | 0.053 |
|  | C | 0.35 | 0.280 | 3.5 | 0.066 |
|  | S | 1.02 | 0.816 | 3.55 | 0.250 |
|  | O | -0.53 | -0.424 | 2.96 | 0.210 |
|  | N | -0.66 | -0.528 | 3.25 | 0.170 |
| [NPF2]- | N | -0.66 | -0.528 | 3.25 | 0.170 |
|  | S | 1.09 | 0.872 | 3.55 | 0.250 |
|  | O | -0.56 | -0.448 | 2.96 | 0.21 |
|  | C1 | 0.12 | 0.096 | 3.5 | 0.066 |
|  | C2 | 0.51 | 0.408 | 3.5 | 0.066 |
|  | F1 | -0.13 | -0.104 | 2.95 | 0.053 |
|  | F2 | -0.17 | -0.136 | 2.95 | 0.053 |

Table S5. Bond force constants r0 (Å) and Kr (Kcal mol-1Å-2) , for anions.

| Anion | Bonds | r0 (Å) | Kr (Kcal mol-1Å-2) | Anion | Bonds | r0 (Å) | Kr (Kcal mol-1Å-2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [TFO]- | F-C | 1.323 | 441.92 | [NTF2] | N-S | 1.570 | 374.88 |
|  | C-S | 1.818 | 233.03 |  | S-O | 1.437 | 637.07 |
|  | S-O | 1.437 | 637.07 |  | S-C | 1.818 | 233.03 |
| [SCN]- | C-N | 1.157 | 650.0 |  | C-F | 1.323 | 441.92 |
|  | S-C | 1.655 | 236.29 | ### | ### | ### | ### |
| [NPF2]- | N-S | 1.570 | 374.88 |  | C1-F1 C2-F2 C1-C2 | 1.323 | 441.92 |
|  | S-O | 1.437 | 637.07 |  |  | 1.323 | 441.92 |
|  | S-C1 | 1.818 | 233.03 |  |  | 1.529 | 268.00 |

Table S6. Angle force constants θ0 (degrees) and Kθ (Kcal mol-1rad-2), for selected Anions.

| Anion | Angles | θ0 (degrees) | Kθ (Kcal mol-1 rad-2) | Anion | Angles | θ0 (degrees) | Kθ (Kcal mol-1 rad-2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [TFO] | F-C-F | 107.1 | 93.33 | [NTF2] | N-S-O | 113.6 | 94.29 |
|  | F-C-S | 111.7 | 82.93 |  | N-C-S | 103.5 | 91.30 |
|  | O-S-O | 118.5 | 115.80 |  | S-C-F | 111.7 | 82.93 |
|  | C-S-O | 102.6 | 103.97 |  | S-N-S | 125.6 | 80.19 |
| [SCN] | S-C-N | 180.0 | 150.0 |  | F-C-F | 107.1 | 93.33 |
| [NPF2] | N-S-O | 113.6 | 94.29 |  | ### | ### | ### |
|  | N-S-C1 | 103.5 | 91.30 |  | C1-S-O | 102.6 | 103.97 |
|  | S-N-S | 125.6 | 80.19 |  | C1-C2-F2 | 109.5 | 50.00 |
|  | S-C1-F1 | 111.7 | 82.93 |  | C2-C1-F1 | 109.5 | 50.00 |
|  | S-C1-C2 | 115.9 | 50.00 |  | F1-C1-F1 | 107.1 | 93.33 |
|  | O-S-O | 118.5 | 115.80 |  | F2-C2-F2 | 107.1 | 93.33 |

TABLE S7. Torsional Fourier Coefficients (Kcal/mol) for selected Anions.

| Anion | Torsion | V1 | V2 | V3 |
| --- | --- | --- | --- | --- |
| [NTF2] | N-S-C-F | 0.000 | 0.000 | 0.316 |
|  | S-N-C-O | 0.000 | 0.000 | -0.004 |
|  | S-N-C-S | 7.833 | -2.490 | -0.764 |
|  | O-S-C-F | 0.000 | 0.000 | 0.347 |
| [TFO] | O-S-C-F | 0.000 | 0.000 | 1.451 |
| [NPF2] | N-S-C1-F1 | 0.000 | 0.000 | 0.316 |
|  | N-S-C1-C2 | -0.739 | 0.000 | 0.000 |
|  | S-N-S-O | 0.000 | 0.000 | -0.004 |
|  | S-N-S-C1 | 7.833 | -2.490 | -0.764 |
|  | S-C1-C2-F2 | 0.000 | 0.000 | 0.347 |
|  | F1-C1-C2-F2 | -2.500 | 0.000 | 0.250 |
|  | O2-S-C1-F1 | 0.000 | 0.000 | 0.347 |
|  | C2-C1-S-O | 0.000 | 0.000 | -0.177 |
|  | N-S-C1-F1 | 0.000 | 0.000 | 0.316 |

OPLS-AA Force field of 1,3-Propanediol (1,3-PDO) (4).

Table S8. Charges of 1,3-Propanediol: Unscaled and Scaled by 0.8.

| Atom Type | q (e) | 0.8*q(e) |
| --- | --- | --- |
| Ca | 0.0083 | 0.00664 |
| Cb | 0.0085 | 0.0068 |
| Cc | -0.1981 | -0.15848 |
| Ha | 0.1216 | 0.09728 |
| Hb | 0.1216 | Figure S5. Atom Type Representation of 1,3-Propanediol (1,3-PDO). 0.09728 |
| Hc | 0.0668 | 0.05344 |
| Hd | 0.0668 | 0.05344 |
| He | 0.0669 | 0.05352 |
| Hf | 0.0669 | 0.05352 |
| Hg | 0.4122 | 0.32976 |
| Hh | 0.412 | 0.3296 |
| Oa | -0.5766 | -0.46128 |
| Ob | -0.5769 | -0.46152 |

TABLE S9. Non-Bonded Parameters σ(Å) and є (Kcal/mol), Bond force constants r0 (Å) Kr (Kcal mol-1Å-2) of 1,3-PDO.

| Atom Type; σ (Å); є (Kcal/mol). Ca; 0.066; 3.50. Cb; 0.066; 3.50. Cc; 0.066; 3.50. Ha; 0.030; 2.50. Hb; 0.030; 2.50. Hc; 0.030; 2.50. Hd; 0.030; 2.50. He; 0.030; 2.50. Hf; 0.030; 2.50. Hg; 0.003; 2.50. Hh; 0.00; 0.00. Oa; 0.170; 3.120. Ob; 0.170; 3.120. | Bonds; r0 (Å); Kr (Kcal mol-1Å-2). Ca-Cc; 268.00; 1.5290. Ca-Hc; 340.00; 1.0900. Ca-Hd; 340.00; 1.0900. Ca-Oa; 320.00; 1.4100. Cc-Cb; 268.00; 1.5290. Cc-Hb; 340.00; 1.0900. Cc-Hg; 340.00; 1.0900. Cb-He; 340.00; 1.0900. Cb-Hf; 340.00; 1.0900. Cb-Ob; 320.00; 1.4100. Cb-Oa; 553.00; 0.9450. Hh-Ob; 553.00; 0.9450. |
| --- | --- |

TABLE S10. Angle force constants θ0 (degrees) Kθ (Kcal mol-1rad-2), for 1,3-PDO.

| Angles; θ0; Kθ. Ca-Cc-Cb; 112.70; 58.35. Ca-Cc-Hb; 110.70; 37.50. Ca-Cc-Ha; 110.70; 37.50. Ca-Oe-Hg; 108.50; 55.00. Cc-Ca-Hc; 110.70; 37.50. Cc-Ca-Hd; 110.70; 37.50. Cc-Ca-Oa; 109.50; 50.00. Ca-Cb-He; 110.70; 37.50. Cc-Cb-Hf; 110.70; 37.50. Cc-Cb-Ob; 109.50; 50.00. | Angles; θ0; Kθ. Cb-Cc-Hb; 110.70; 37.50. Cb-Cc-Ha; 110.70; 37.50. Cb-Cc-Hd; 108.50; 55.00. Hf-Ca-Hd; 107.80; 33.00. Hf-Ca-Oa; 109.50; 35.00. Hd-Ca-Oa; 109.50; 35.00. Hb-Cc-Ha; 107.80; 33.00. He-Cb-Hf; 107.80; 33.00. He-Cb-Ob; 109.50; 35.00. Hf-Cb-Ob; 109.50; 35.00. |
| --- | --- |

TABLE S11. Torsional Fourier Coefficients (Kcal/mol) for 1,3-PDO.

| Torsion; V1; V2; V3. Ca-Cc-Cb-He; 0.000; 0.000; 0.300. Ca-Cc-Cb-Hf; 0.000; 0.000; 0.300. Ca-Cc-Cb-Ob; -1.552; 0.000; 0.000. Cc-Ca-Ob-Hg; -0.356; -0.174; 0.492. Cc-Cb-Ob-Hh; -0.356; -0.174; 0.492. Hf-Ca-Cc-Cb; 0.000; 0.000; 0.300. Hf-Ca-Cc-Hb; 0.000; 0.000; 0.300. Hf-Ca-Cc-Ha; 0.000; 0.000; 0.300. Hf-Ca-Oa-Hg; 0.000; 0.000; 0.352. Hd-Ca-Cc-Cb; 0.000; 0.000; 0.300. Hd-Ca-Cc-Hb; 0.000; 0.000; 0.300. Hd-Ca-Cc-Ha; 0.000; 0.000; 0.300. | Torsion; V1; V2; V3. Hd-Ca-Oa-Hn; 0.000; 0.000; 0.352. Hb-Cc-Cb-He; 0.000; 0.000; 0.300. Hb-Cc-Cb-Hf; 0.000; 0.000; 0.300. Hb-Cc-Cb-Ob; 0.000; 0.000; 0.468. Ha-Cc-Cb-He; 0.000; 0.000; 0.300. Ha-Cc-Cb-Hf; 0.000; 0.000; 0.300. Ha-Cc-Cb-Ob; 0.000; 0.000; 0.468. He-Cb-Ob-Hh; 0.000; 0.000; 0.352. Hf-Cb-Ob-Hh; 0.000; 0.000; 0.352. Oe-Ca-Cc-Cb; -1.552; 0.000; 0.000. Oe-Ca-Cc-Hb; 0.000; 0.000; 0.468. Oe-Ca-Cc-Ha; 0.000; 0.000; 0.468. |
| --- | --- |

Table S12. Distribution of components in extract and raffinate phases from MD simulation at 298 K and 1 atm.

| Systems | Extract Phase (IL phase) |  |  | Raffinate Phase (Aqueous phase) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | N1,3-PDOE | NWaterE | NILE | N1,3-PDOR | NWaterR | NILR |
| [Bmim][NPF2] | 82 | 1252 | 198 | 192 | 3348 | 2 |
| [Bmim][NTF2] | 101 | 979 | 196 | 114 | 2521 | 4 |
| [Bmim][SCN] | 145 | 375 | 237 | 35 | 2625 | 108 |
| [Bmim][TFO] | 122 | 205 | 163 | 58 | 2795 | 72 |

Where, E represents Extract phase and R represents Raffinate phase.

Figure S6. Density Profile Validation Using OPLS-AA Force Fields over 12ns Simulation at 298 K   (5,6).

Figure S7. RDF Plot for [Bmim][NPF2].

Figure S8. RDF Plot for [Bmim][NTF2] (7).

Figure S9. RDF Plot for [Bmim][SCN] (8).

Figure S10. RDF Plot for [Bmim][TFO] (9).

Figure S11. Spatial Distribution Function (SDF) Analysis of [Bmim][NTF2]: Molecular Density Distribution. (a) and (b) show the vertical and horizontal views of [NTF2] around [Bmim] cation. Similarly, (c) and (d) display [Bmim] cation around [Bmim] cation. Cyan color represents cation [Bmim] density, while purple depicts [NTF2] anion density (10).

Figure S12. Spatial Distribution Function (SDF) Analysis of [Bmim][SCN]: Molecular Density Distribution. (a) and (b) display vertical and horizontal views of [SCN] around [Bmim] cation, while (c) and (d) depict [Bmim] cation around [Bmim] cation. Cyan color indicates [Bmim] cation density, and red signifies [SCN] anion density (11).

Figure S13. Spatial Distribution Function (SDF) Analysis of [Bmim][TFO]: Molecular Density Distribution. (a) and (b) show vertical and horizontal views of [TFO] around [Bmim] cation, while (c) and (d) depict [Bmim] cation around [Bmim] cation. Cyan color represents [Bmim] cation density, and lime signifies [TFO] anion density (12).

Figure S14. Spatial Distribution Function (SDF) visualisations of ILs around 1,3-PDO, Color code: Cyan for [Bmim]+ cation, Purple for [NPF2]-, Blue for [NTF2]-, Red for [SCN]-, and Lime for [TFO]-. (a) SDF of [NPF2]- and [Bmim]+  around 1,3-PDO in [Bmim][NPF2], (b) SDF of [NTF2]- and [Bmim]+ around 1,3-PDO in [Bmim][NTF2], (c) SDF of [SCN]- and [Bmim]+ around 1,3-PDO in [Bmim][SCN] and (d) SDF of [TFO]- and [Bmim]+ around 1,3-PDO in [Bmim][TFO].

Figure S15.  Molecular Distribution Analysis and Interatomic RDF of [Bmim][NTF2] with 1,3-PDO. (a) SDF of 1,3-PDO around [NTF2]-, (b) SDF of 1,3-PDO around [Bmim]+, (c) SDF of  [NTF2]- around 1,3-PDO, (d) SDF of [Bmim]+ around 1,3-PDO, and (e) Interatomic RDF Plot of ILs and 1,3-PDO at 298 K.

Figure  S16. This figure displays the average number of hydrogen bonds observed after equilibration at 298 K. (a) H-Bonding between ILs and 1,3-PDO  and (b) H-Bonding involving ILs anions and 1,3-PDO.

Figure S17. Comparative Analysis of Water Entry in Various Ionic Liquids (ILs) at 298 K. Distinguished by colors (a) [Bmim][NPF2] in purple, (b) [Bmim][NTF2] in blue, (c) [Bmim][SCN] in red, and (d) [Bmim][TFO] in lime—the figure unveils the unique behavior of each IL in accommodating water molecules under specified conditions.

Reference:

1.	Banerjee T, Singh MK, Khanna A. Prediction of binary VLE for imidazolium based ionic liquid systems using COSMO-RS. Ind Eng Chem Res. 2006 Apr 26;45(9):3207–19. Available from: https://pubs.acs.org/doi/full/10.1021/ie051116c

2.	Doherty B, Zhong X, Gathiaka S, Li B, Acevedo O. Revisiting OPLS Force Field Parameters for Ionic Liquid Simulations. J Chem Theory Comput. 2017 Dec 12;13(12):6131–5.

3.	Sambasivarao S V., Acevedo O. Development of OPLS-AA force field parameters for 68 unique ionic liquids. J Chem Theory Comput. 2009 Apr 14;5(4):1038–50.

4.	Dodda LS, De Vaca IC, Tirado-Rives J, Jorgensen WL. LigParGen web server: An automatic OPLS-AA parameter generator for organic ligands. Nucleic Acids Res. 2017 Jul 3;45(W1):W331–6.

5.	Zhang R, Yue X, Li B, Yang J, Wu Z, Zhang J. Dynamic viscosity, density and surface tension of 1,3-propanediol (1) + 1,2-propanediamine (2) binary system at T = (293.15 to 318.15) K and atmosphere pressure. J Mol Liq. 2020 Feb 1;299.

6.	Thomas M, Sanz IS, Kirchner B, Holloczki O. Ab Initio Molecular Dynamics Simulations of Ionic Liquids. NIC Symposium 2016. Forschungszentrum Jülich GmbH, Zentralbibliothek; 2016.

7.	Jiang K, Liu L, Liu X, Zhang X, Zhang S. Insight into the Relationship between Viscosity and Hydrogen Bond of a Series of Imidazolium Ionic Liquids: A Molecular Dynamics and Density Functional Theory Study. Ind Eng Chem Res. 2019 Oct 9;58(40):18848–54.

8.	Thar J, Brehmy M, Seitsonen AP, Kirchner B. Unexpected hydrogen bond dynamics in imidazolium-based ionic liquids. Journal of Physical Chemistry B. 2009 Nov 19;113(46):15129–32.

9.	Fatima U, Riyazuddeen, Montes-Campos H, Varela LM. Experimental and MD simulation investigation on thermophysical properties of binary/ternary mixtures of 1-butyl-3-methylimidazolium trifluoromethanesulfonate with molecular solvents. J Mol Liq. 2020 Mar 15;302.

10.	Dhakal P, Shah JK. Recent advances in molecular simulations of ionic liquid–ionic liquid mixtures. Vol. 18, Current Opinion in Green and Sustainable Chemistry. Elsevier B.V.; 2019. p. 90–7.

11.	Wang N, Zhang Y, Al-Barghouti KS, Kore R, Scurto AM, Maginn EJ. Structure and Dynamics of Hydrofluorocarbon/Ionic Liquid Mixtures: An Experimental and Molecular Dynamics Study. Journal of Physical Chemistry B. 2022 Oct 20;126(41):8309–21. Available from: https://pubs.acs.org/doi/full/10.1021/acs.jpcb.2c05787

12.	Marekha BA, Koverga VA, Chesneau E, Kalugin ON, Takamuku T, Jedlovszky P, et al. Local Structure in Terms of Nearest-Neighbor Approach in 1-Butyl-3-methylimidazolium-Based Ionic Liquids: MD Simulations. Journal of Physical Chemistry B. 2016 Jun 9;120(22):5029–41. Available from: https://pubs.acs.org/doi/full/10.1021/acs.jpcb.6b04066