Supplementary Information for

In-target Production of [11C]CH4 from a Nitrogen/Hydrogen Gas Target as a Function of Beam Current, Irradiation Time, and Target Temperature

Semi Helin1, Johan Rajander2, Jussi Aromaa2, Eveliina Arponen1, Jatta S. Helin3,4 and Olof Solin1,2,5,6

1Turku PET Centre, Radiopharmaceutical chemistry laboratory, University of Turku, Kiinamyllynkatu 4-8, FI-20520 Turku, Finland

2Turku PET Centre, Accelerator laboratory, Åbo Akademi University, Kiinamyllynkatu 4-8, FI-20520 Turku, Finland

3MediCity Research Laboratory, University of Turku, Tykistökatu 6 A, FI-20520 Turku, Finland

4Turku PET Centre, Preclinical Imaging Laboratory, University of Turku, Tykistökatu 6 A, FI-20520 Turku, Finland

5Department of Chemistry, University of Turku, Henrikinkatu 2, FI-20500 Turku, Finland

6Turku PET Centre, Turku University Hospital, Kiinamyllynkatu 4-8, FI-20520 Turku, Finland

Contents

Table S1. Target pressures during irradiations at various nominal beam currents.	2

Detailed statistical results on the Table S1 data;  effect of nominal current and temperature on target pressures during irradiations for [11C]CO2 and [11C]CH4 production	2

Table S2. Measured data from the first category; repeatability	3

Table S3. Measured data from the second category.	4

Additional statistical results on Table S3 data; second category, [11C]CO2 and [11C]CH4 yields	5

Table S4. Measured data from the third category.	6

Table S5. Means and standard deviations (SD) of saturation activities YSAT [GBq/µA] at temperatures 10 °C, 40 °C and 70 °C.	7

Table S1. Target pressures during irradiations at various nominal beam currents.

Mean values with standard deviations (N=8) before irradiations (zero current). Target was loaded to 35 bar pressure while the heat exchanger setting was 20 °C. After loading, the setting was changed to the desired value. Zero current values were recorded when the chamber body reached the temperature.

| Target pressure [bar] |  | [11C]CO2 production |  |  | [11C]CH4 production |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal current [μA] |  | 10 °C | 40 °C | 70 °C | 10 °C | 40 °C | 70 °C |
| 0 | Mean | 34.6 | 38.2 | 41.1 | 35.0 | 38.5 | 41.7 |
|  | SD | 0.17 | 0.43 | 0.53 | 0.15 | 0.12 | 0.10 |
| 10 |  | 51.16 | 53.74 | 56.75 | 50.76 | 54.99 | 57.99 |
|  |  | 50.20 | 53.86 | 57.49 | 50.12 | 53.91 | 57.33 |
| 20 |  | 58.43 | 62.16 | 65.20 | 59.22 | 61.95 | 64.71 |
|  |  | 58.86 | 62.83 | 65.58 | 58.28 | 60.28 | 63.45 |
| 30 |  | 64.74 | 68.89 | 72.11 | 64.51 | 68.79 | 68.94 |
|  |  | 66.52 | 70.09 | 71.95 | 62.66 | 66.40 | 70.37 |
| 40 |  | 73.43 | 77.29 | 79.07 | 68.62 | 71.02 | 74.95 |
|  |  | 73.35 | 78.21 | 80.91 | 71.39 | 74.51 | 76.97 |

Detailed statistical results on the Table S1 data; 
effect of nominal current and temperature on target pressures during irradiations for [11C]CO2 and [11C]CH4 production

[11C]CO2

ANOVA; there was a statistically significant single effect of temperature [F (2, 12) = 186.3, P < 0.0001] as well as nominal current [F (3, 12) = 1259, P < 0.0001] on the target pressure, but no interaction effect was observed (P = 0.8903).

Detailed differences across the simple main effect of temperature was examined after multiple comparison, which demonstrated significantly higher target pressures at 70 °C compared to 10 °C or 40 °C regardless the nominal current (ranging from P < 0.0001 to P = 0.0165).

Tukey’s post hoc test; the simple main effect of nominal current was examined, which showed that significantly higher target pressures were obtained at 40 µA compared to lower nominal currents regardless of the temperature (P < 0.0001) in the [11C]CO2 production.

[11C]CH4

ANOVA; there was a statistically significant single effect of temperature [F (2, 12) = 42.84, P < 0.0001] as well as nominal current [F (3, 12) = 216.9, P < 0.0001] on the target pressure, but no interaction effect was observed (P = 0.9201).

Detailed differences across the simple main effect of nominal current was examined after multiple comparison, which demonstrated significantly higher target pressures at 40 μA compared to lower nominal currents at all temperatures (ranging from P < 0.0001 to P = 0.0156).

Tukey’s post hoc test; the simple main effect of temperature on the target pressure was examined, which showed that at 10 μA nominal current, significantly lower target pressures were obtained at 10 °C compared to 40 °C (P = 0.0268) or 70 °C (P = 0.0004). When nominal current was adjusted to 20 μA, significantly higher target pressure was obtained at 70 °C compared to 10 °C (P = 0.0046). At nominal current of 30 μA, significantly lower target pressures were obtained at 10 °C compared to 40 °C (P = 0.0269) or 70 °C (P = 0.0017). Finally at 40 μA, significantly higher target pressure was obtained at 70 °C compared to 10 °C (P = 0.0020).

Table S2. Measured data from the first category; repeatability

Collected radioactivities, AEOB(11CH4) and AEOB(11CO2), are decay corrected to the EOB. I is the irradiation current, measured total from the target grid and body and compensated by the grid factor. T is the measured target chamber temperature, and t is the recorded irradiation time. Theoretical AEOB(11C) is calculated based on the recorded irradiation values. Mean, Standard deviation (SD) and Relative standard deviation (RSD) are calculated for the repeated runs.

| 1st cat. | [11C]CO2 |  |  |  |  |  | [11C]CH4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Meas. | Theor. |  |  |  |  | Meas. | Theor. |  |
|  | I | T | t | AEOB(11CO2) | AEOB(11C) | Meas./theor. | I | T | t | AEOB(11CH4) | AEOB(11C) | Meas./theor. |
|  | [μA] | [°C] | [min] | [GBq] | [GBq] |  | [μA] | [°C] | [min] | [GBq] | [GBq] |  |
|  | 19.9 | 21.4 | 9.98 | 22.7 | 35.2 | 0.65 | 19.0 | 21.7 | 10.07 | 18.5 | 33.8 | 0.55 |
|  | 20.1 | 21.4 | 10.00 | 23.3 | 35.7 | 0.65 | 19.9 | 21.7 | 10.00 | 18.6 | 35.2 | 0.53 |
|  | 20.0 | 21.3 | 9.98 | 23.5 | 35.4 | 0.66 | 19.6 | 21.7 | 9.98 | 18.2 | 34.8 | 0.52 |
|  | 19.7 | 21.4 | 10.00 | 23.4 | 35.0 | 0.67 | 19.6 | 21.6 | 10.00 | 17.8 | 34.9 | 0.51 |
|  | 19.8 | 21.3 | 9.98 | 24.3 | 35.1 | 0.69 | 19.6 | 21.6 | 10.02 | 17.8 | 34.8 | 0.51 |
|  | 19.8 | 21.3 | 9.98 | 23.6 | 35.1 | 0.67 | 19.6 | 21.4 | 10.00 | 18.2 | 34.8 | 0.52 |
|  | 19.6 | 21.4 | 9.97 | 25.1 | 34.7 | 0.72 | 19.6 | 21.4 | 10.00 | 18.2 | 34.8 | 0.52 |
|  | 19.6 | 21.4 | 10.00 | 25.3 | 34.8 | 0.73 | 19.9 | 20.1 | 9.95 | 19.4 | 35.1 | 0.55 |
|  | 19.9 | 21.4 | 10.00 | 24.9 | 35.2 | 0.71 | 19.9 | 21.6 | 9.98 | 18.5 | 35.3 | 0.52 |
|  | 19.7 | 21.3 | 10.00 | 24.3 | 35.0 | 0.69 | 19.3 | 21.4 | 10.00 | 17.7 | 34.3 | 0.51 |
| Mean | 19.8 | 21.4 | 9.99 | 24.0 | 35.1 | 0.68 | 19.6 | 21.4 | 10.00 | 18.3 | 34.8 | 0.53 |
| SD | 0.17 | 0.05 | 0.01 | 0.87 | 0.29 | 0.03 | 0.28 | 0.48 | 0.03 | 0.52 | 0.44 | 0.01 |
| RSD (%) | 0.8 | 0.2 | 0.1 | 3.6 | 0.8 | 4.2 | 1.4 | 2.2 | 0.3 | 2.8 | 1.3 | 2.7 |

Table S3. Measured data from the second category.

Collected radioactivities, AEOB(11CH4) and AEOB(11CO2), are decay corrected to the EOB. I is the irradiation current, measured total from the target grid and body and compensated by the grid factor. T is the measured target chamber temperature, and t is the recorded irradiation time. Theoretical AEOB(11C) is calculated based on the recorded irradiation values.

| 2nd cat. |  | [11C]CO2 |  |  |  |  |  | [11C]CH4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal |  |  |  |  | Meas. | Theor. |  |  |  |  | Meas. | Theor. |  |
| current | temperature | I | T | t | AEOB(11CO2) | AEOB(11C) | Meas./theor. | I | T | t | AEOB(11CH4) | AEOB(11C) | Meas./theor. |
| [μA] | [°C] | [μA] | [°C] | [min] | [GBq] | [GBq] |  | [μA] | [°C] | [min] | [GBq] | [GBq] |  |
| 10 | 10 | 10.3 | 11.2 | 20.00 | 22.2 | 31.3 | 0.71 | 10.2 | 11.7 | 20.02 | 17.9 | 31.1 | 0.57 |
|  |  | 10.1 | 11.3 | 20.00 | 21.1 | 30.6 | 0.69 | 9.9 | 11.3 | 20.00 | 18.3 | 30.2 | 0.61 |
|  | 40 | 10.1 | 39.2 | 19.98 | 21.4 | 30.5 | 0.70 | 10.2 | 40.2 | 20.00 | 21.1 | 30.9 | 0.68 |
|  |  | 10.0 | 39.4 | 20.03 | 22.6 | 30.4 | 0.74 | 9.9 | 39.3 | 20.00 | 21.4 | 29.9 | 0.72 |
|  | 70 | 10.2 | 68.5 | 19.93 | 23.3 | 30.9 | 0.75 | 10.3 | 67.1 | 19.95 | 23.2 | 31.1 | 0.75 |
|  |  | 10.1 | 67.3 | 20.03 | 23.1 | 30.6 | 0.75 | 10.0 | 67.1 | 19.98 | 23.0 | 30.3 | 0.76 |
| 20 | 10 | 20.0 | 12.1 | 20.05 | 41.3 | 60.8 | 0.68 | 19.9 | 12.1 | 20.02 | 26.4 | 60.6 | 0.44 |
|  |  | 19.8 | 11.2 | 19.93 | 40.8 | 60.1 | 0.68 | 19.7 | 12.0 | 19.98 | 27.6 | 59.9 | 0.46 |
|  | 40 | 19.9 | 39.7 | 20.00 | 42.5 | 60.3 | 0.70 | 20.0 | 39.8 | 19.98 | 30.6 | 60.6 | 0.51 |
|  |  | 19.3 | 40.7 | 19.98 | 41.6 | 58.5 | 0.71 | 19.7 | 39.8 | 20.00 | 30.3 | 59.9 | 0.51 |
|  | 70 | 19.8 | 67.7 | 19.98 | 38.4 | 60.1 | 0.64 | 19.8 | 68.5 | 20.00 | 35.2 | 60.1 | 0.59 |
|  |  | 19.8 | 68.9 | 20.00 | 37.9 | 60.2 | 0.63 | 19.9 | 68.5 | 19.97 | 35.0 | 60.5 | 0.58 |
| 30 | 10 | 29.5 | 12.3 | 20.00 | 59.6 | 89.5 | 0.67 | 29.8 | 12.9 | 19.98 | 17.1 | 90.5 | 0.19 |
|  |  | 29.4 | 12.0 | 20.02 | 58.8 | 89.4 | 0.66 | 29.7 | 13.1 | 20.02 | 22.0 | 90.4 | 0.24 |
|  | 40 | 29.5 | 40.3 | 19.98 | 58.5 | 89.4 | 0.65 | 29.6 | 41.5 | 20.02 | 26.7 | 90.0 | 0.30 |
|  |  | 29.2 | 40.4 | 20.03 | 60.9 | 88.6 | 0.69 | 28.6 | 40.4 | 19.98 | 27.8 | 86.8 | 0.32 |
|  | 70 | 29.8 | 69.5 | 19.98 | 58.4 | 90.4 | 0.65 | 29.3 | 69.6 | 20.00 | 38.4 | 88.9 | 0.43 |
|  |  | 29.7 | 70.1 | 20.00 | 55.9 | 90.1 | 0.62 | 29.6 | 69.2 | 20.00 | 35.5 | 89.8 | 0.40 |
| 40 | 10 | 39.0 | 13.0 | 19.98 | 75.0 | 118.3 | 0.63 | 39.0 | 13.4 | 20.02 | 9.3 | 118.7 | 0.08 |
|  |  | 38.9 | 13.0 | 19.95 | 78.5 | 117.9 | 0.67 | 38.9 | 13.4 | 20.00 | 12.1 | 118.1 | 0.10 |
|  | 40 | 39.2 | 41.0 | 19.98 | 78.5 | 119.0 | 0.66 | 39.4 | 41.1 | 20.00 | 16.1 | 119.7 | 0.13 |
|  |  | 38.6 | 40.9 | 20.02 | 78.8 | 117.5 | 0.67 | 39.1 | 40.9 | 19.98 | 17.4 | 118.6 | 0.15 |
|  | 70 | 38.9 | 70.9 | 20.03 | 72.9 | 118.1 | 0.62 | 39.3 | 69.6 | 19.98 | 23.7 | 119.2 | 0.20 |
|  |  | 38.8 | 71.0 | 20.00 | 71.8 | 117.9 | 0.61 | 38.5 | 70.4 | 20.02 | 25.8 | 116.9 | 0.22 |

Additional statistical results on Table S3 data; second category, [11C]CO2 and [11C]CH4 yields

Effect of nominal current and temperature on the second category measured radioactivities of [11C]CO2 and [11C]CH4

[11C]CO2

Tukey’s post hoc test. Detailed differences across the simple main effect of temperature was examined after multiple comparison, which demonstrated significantly higher radioactivity at 40 °C compared to 70 °C, when the nominal current was 20 µA (P = 0.0120). In addition at 40 µA, significantly higher AEOB(11CO2) was observed at 10 °C (P = 0.0055) and 40 °C (P = 0.0003) compared to 70 °C.

Furthermore, the simple main effect of nominal current was examined, which showed significantly higher AEOB(11CO2) at 40 µA compared to lower nominal current values regardless of the temperature (P < 0.0001).

[11C]CH4

Tukey’s post hoc test. Detailed differences across the simple main effect of T was examined after multiple comparison, which demonstrated significantly higher radioactivity at 70 °C compared to 10 °C, when the nominal current was 10 µA (P = 0.0115). At 20 µA, 30 µA, and 40 µA, significantly higher AEOB(11CH4) was observed at 70 °C compared to 10 °C (ranging from P < 0.0001 to P = 0.0003) and 40 °C (ranging from P < 0.0001 to P = 0.0188), which confirmed that the highest measured radioactivities were gained at higher temperature. Interestingly, the highest radioactivity was measured at 30 µA, while the measured AEOB(11CH4) at 40 µA could not exceed any AEOB(11CH4) at the corresponding T setting in other I groups.

Furthermore, the simple main effect of nominal current was examined, which showed significantly higher AEOB(11CH4) at 20 µA compared to 10 µA (P = 0.0002) or 30 µA (P = 0.0012), or 40 µA (P < 0.0001) when the temperature was 10 °C. At 40 °C, significantly higher AEOB(11CH4) was also observed at 20 µA compared to 10 µA (P = 0.0002) or 40 µA (P < 0.0001), which was similar with the temperature of 70 °C (P < 0.0001).

Table S4. Measured data from the third category.

Collected radioactivities, AEOB(11CH4), are decay corrected to EOB. I is the irradiation current, measured total from the target grid and body and compensated by the grid factor. Theoretical AEOB(11C) is calculated based on the recorded irradiation values. T is the measured target chamber temperature, and t is the recorded irradiation time.

| 3rd cat. | [11C]CH4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Fixed | Varied | Nominal | Delivered | Meas. | Theor. |  |
| Nominal current | I | T | t | Charge | Charge | AEOB(11CH4) | AEOB(11C) | Meas./theor. |
| [μA] | [μA] | [°C] | [min] | [µA∙min] | [µA∙min] | [GBq] | [GBq] |  |
| 10 | 10.0 | 39.2 | 11.67 | 120 | 117 | 14.0 | 20.1 | 0.70 |
|  | 10.2 | 39.1 | 22.72 | 240 | 231 | 21.9 | 33.6 | 0.65 |
|  | 10.1 | 39.1 | 34.92 | 360 | 353 | 29.2 | 43.2 | 0.68 |
|  | 10.1 | 39.2 | 45.92 | 480 | 461 | 31.8 | 48.9 | 0.65 |
|  | 10.3 | 39.1 | 56.03 | 600 | 576 | 34.1 | 53.9 | 0.63 |
|  | 10.1 | 39.1 | 75.62 | 800 | 766 | 37.5 | 57.6 | 0.65 |
| 20 | 20.1 | 40.3 | 5.92 | 120 | 119 | 13.1 | 22.6 | 0.58 |
|  | 19.9 | 39.8 | 11.90 | 240 | 236 | 22.6 | 40.6 | 0.56 |
|  | 20.0 | 39.7 | 17.67 | 360 | 354 | 32.1 | 55.6 | 0.58 |
|  | 19.9 | 39.6 | 23.75 | 480 | 473 | 34.7 | 67.8 | 0.51 |
|  | 20.0 | 39.6 | 29.50 | 600 | 591 | 39.8 | 78.0 | 0.51 |
|  | 20.0 | 39.6 | 39.33 | 800 | 785 | 42.0 | 90.5 | 0.46 |
| 40 | 39.6 | 40.3 | 3.05 | 120 | 121 | 5.7 | 24.0 | 0.24 |
|  | 39.7 | 40.7 | 6.08 | 240 | 241 | 8.6 | 45.6 | 0.19 |
|  | 39.9 | 40.8 | 9.02 | 360 | 360 | 11.0 | 64.9 | 0.17 |
|  | 40.0 | 40.9 | 12.00 | 480 | 480 | 12.1 | 82.5 | 0.15 |
|  | 39.7 | 40.9 | 15.10 | 600 | 600 | 16.0 | 98.1 | 0.16 |
|  | 39.9 | 40.8 | 20.03 | 800 | 799 | 18.0 | 121.3 | 0.15 |

Table S5. Means and standard deviations (SD) of saturation activities YSAT [GBq/µA] at temperatures 10 °C, 40 °C and 70 °C.

[11C]CO2 and [11C]CH4 productions at various nominal irradiation currents (compare to data groups at Fig. 2).

| Nominal | [11C]CO2 production |  | [11C]CH4 production |  |
| --- | --- | --- | --- | --- |
| current | YSAT [GBq/µA] |  | YSAT [GBq/µA] |  |
| µA | Mean | SD | Mean | SD |
| 10 | 5.88 | 0.24 | 5.52 | 0.61 |
| 20 | 5.46 | 0.27 | 4.15 | 0.49 |
| 30 | 5.31 | 0.18 | 2.54 | 0.74 |
| 40 | 5.21 | 0.21 | 1.19 | 0.45 |
| Pooled mean ± SD: | 5.47 ± 0.30 |  |  |  |