Table S1. Multiple reaction monitoring (MRM) transitions and optimized mass spectrometry parameters for lovastatin and lovastatin hydroxy acid

| Compound | MRM transition (m/z) | Declustering potential (V) | Collision energy (eV) | Entrance potential (V) | Collision cell exit potential (V) |
| --- | --- | --- | --- | --- | --- |
| Lovastatin | 405.3 >243.2 | 49.85 | 16.74 | 12 | 25 |
|  | 405.3 > 303.3 |  | 13.44 |  |  |
| Lovastatin hydroxy acid | 421.5 > 101.5 | 51.54 | 36.75 | 15 | 20 |
|  | 421.5 > 319.9 |  | 24.56 |  |  |

* Transitions shown in bold were used for quantification

Table S2. Multiple reaction monitoring (MRM) transitions and optimized mass spectrometry parameters for bile acids

| Compound | MRM transition (m/z) | Declustering potential (V) | Collision energy (eV) | Entrance potential (V) | Collision cell exit potential (V) |
| --- | --- | --- | --- | --- | --- |
| Cholic acid (CA) | 407.1 >407.1 | -160 | -18 | -15 | -20 |
|  | 407.1 > 343.1 |  | -44 |  |  |
| Deoxycholic acid (DCA) | 391.3 > 391.3 | -160 | -9 | -15 | -20 |
|  | 391.3 > 345.3 |  | -46 |  |  |
| Chenodeoxycholic acid (CDCA) | 391.3 > 391.3 | -170 | -8 | -15 | -20 |
| Lithocholic acid (LCA) | 375.3 > 375.3 | -160 | -8 | -15 | -20 |
| Dehydrocholic acid (DDCA) | 401.2 > 401.2 | -160 | -9 | -15 | -20 |
|  | 401.2 > 331.2 |  | -35 |  |  |
| Ursodeoxycholic acid (UDCA) | 391.3 > 391.3 | -180 | -8 | -15 | -20 |
| Glycocholic acid (GCA) | 464.3 > 464.3 | -120 | -8 | -15 | -20 |
|  | 464.3 > 74.1 |  | -67 |  |  |
| Glycochenodeoxycholic acid (GCDCA) | 448.4 > 448.4 | -160 | -7 | -15 | -20 |
|  | 448.4 > 74.2 |  | -71 |  |  |
| Glycodeoxycholic acid (GDCA) | 448.3 > 448.3 | -130 | -9 | -15 | -20 |
|  | 448.3 > 386.2 |  | -48 |  |  |
| Glycoursodeoxycholic acid (GUDCA) | 448.3 > 386.1 | -160 | -47 | -15 | -20 |
|  | 448.3 > 74.0 |  | -72 |  |  |
| Taurocholic acid (TCA) | 514.3 > 80.0 | -180 | -84 | -15 | -20 |
|  | 514.3 > 124.1 |  | -70 |  |  |
| Taurochenodeoxycholic acid (TCDCA) | 498.4 > 498.4 | -190 | -7 | -15 | -20 |
|  | 498.4 > 80.0 |  | -120 |  |  |
| Taurodeoxycholic acid (TCDA) | 497.9 > 497.9 | -180 | -8 | -15 | -20 |
|  | 497.9 > 355.5 |  | -60 |  |  |
| Taurolithocholic acid (TLCA) | 482.1 > 482.1 | -190 | -8 | -15 | -20 |
|  | 482.1 > 79.8 |  | -90 |  |  |
| Tauroursodeoxycholic acid (TUDCA) | 498.3 > 498.3 | -180 | -9 | -15 | -20 |
|  | 498.3 > 80.0 |  | -92 |  |  |

* Transitions shown in bold were used for quantification

Table S3. The survival rate of Lacticaseibacillus paracasei Zhang in the near-real human digestion system

| Time_ replicate no. | Total viable counts (CFU/mL) | Survival rate (%) | Average survival rate at the specific time point (%) | Standard deviation |
| --- | --- | --- | --- | --- |
| 0h_1 | 2.00e7 | 100.00 | 100.00 | 0.00 |
| 0h_2 | 2.00e7 | 100.00 |  |  |
| 0h_3 | 2.00e7 | 100.00 |  |  |
| 6h_1 | 2.00e6 | 55.00 | 63.25 | 9.72 |
| 6h_2 | 2.20e6 | 60.78 |  |  |
| 6h_3 | 2.70e6 | 73.98 |  |  |
| 12h_1 | 1.70e6 | 45.63 | 39.60 | 6.84 |
| 12h_2 | 1.50e6 | 41.04 |  |  |
| 12h_3 | 1.20e6 | 32.13 |  |  |
| 24h_1 | 9.60e5 | 25.44 | 24.82 | 4.32 |
| 24h_2 | 8.60e5 | 22.79 |  |  |
| 24h_3 | 9.90e5 | 26.24 |  |  |