How good can Damped Shifted Force Monte Carlo predict vapor-liquid equilibria for natural gas systems?

Alexandre S. Zimmerman1,*, Raphael Lírio1, Beatriz Montes Castro1, Lucienne Romanielo2, Silvana Mattedi1.

1Chemical Engineering Graduate Program, Federal University of Bahia, Brazil

2Dep. of Chem. Engineering, Federal University of Uberlândia

*Corresponding author: alexandresz.eq@hotmail.com

SUPPLEMENTARY MATERIAL

# Appendix A

## Appendix A.1: Mole fraction of the vapor-liquid equilibria for binary CH4, CO2 and H2O systems as predicted by Damped Shifted Force Monte Carlo simulations.

Table A.1. Mole fraction of the vapor-liquid equilibrium for the CH4+CO2 binary at 250 K.

| P /MPa | xCH4 | yCH4 |
| --- | --- | --- |
| 1.80 ± 0.01 | 0.0000 ± 0.0000 | 0.000 ± 0.000 |
| 2.10 ± 0.04 | 0.0136 ± 0.0002 | 0.117 ± 0.002 |
| 2.40 ± 0.01 | 0.0273 ± 0.0002 | 0.205 ± 0.004 |
| 2.94 ± 0.07 | 0.0555 ± 0.0003 | 0.332 ± 0.003 |
| 3.92 ± 0.05 | 0.1168 ± 0.0002 | 0.477 ± 0.005 |
| 4.94 ± 0.05 | 0.181 ± 0.001 | 0.552 ± 0.002 |
| 5.89 ± 0.04 | 0.249 ± 0.003 | 0.596 ± 0.003 |
| 6.33 ± 0.03 | 0.289 ± 0.003 | 0.612 ± 0.002 |

Table A.2. Mole fraction of the vapor-liquid equilibrium for the CH4+CO2 binary at 270 K.

| P /MPa | xCH4 | yCH4 |
| --- | --- | --- |
| 3.25 ± 0.02 | 0.0000 ± 0.0000 | 0.0000 ± 0.0000 |
| 3.57 ± 0.05 | 0.0140 ± 0.0003 | 0.071 ± 0.001 |
| 3.86 ± 0.07 | 0.0288 ± 0.0004 | 0.128 ± 0.001 |
| 4.49 ± 0.03 | 0.0579 ± 0.0009 | 0.218 ± 0.002 |
| 5.73 ± 0.05 | 0.1180 ± 0.0005 | 0.330 ± 0.002 |
| 6.77 ± 0.01 | 0.190 ± 0.003 | 0.4053 ± 0.0006 |

Table A.3. Mole fraction of the vapor-liquid equilibrium for the CO2+H2O binary at 348.15 K.

| P /MPa | xCO2 | yCO2 |
| --- | --- | --- |
| 0.2369 ± 0.0002 | 0.0003 ± 0.0002 | 0.911 ± 0.004 |
| 2.09 ± 0.01 | 0.0034 ± 0.0005 | 0.9902 ± 0.0003 |
| 3.58 ± 0.02 | 0.006 ± 0.001 | 0.994 ± 0.002 |
| 5.04 ± 0.05 | 0.0072 ± 0.0007 | 0.995 ± 0.001 |
| 7.55 ± 0.06 | 0.010 ± 0.001 | 0.9963 ± 0.0002 |
| 10.1 ± 0.1 | 0.012 ± 0.002 | 0.9969 ± 0.0008 |
| 15.1 ± 0.2 | 0.014 ± 0.001 | 0.9964 ± 0.0004 |

Table A.4. Mole fraction of the vapor-liquid equilibrium for the CH4+H2O binary at 323.56 K.

| P /MPa | xCH4 | yCH4 |
| --- | --- | --- |
| 4.981 ± 0.001 | 0.00070 ± 0.00007 | 0.9984 ± 0.0009 |
| 6.491 ± 0.003 | 0.00097 ± 0.00009 | 0.9983 ± 0.0009 |
| 8.481 ± 0.002 | 0.0011 ± 0.0001 | 0.9994 ± 0.0002 |
| 11.487 ± 0.004 | 0.0012 ± 0.0002 | 0.999 ± 0.001 |
| 18.491 ± 0.003 | 0.0021 ± 0.0004 | 0.9997 ± 0.0001 |

## Appendix A.2: Densities of the vapor-liquid equilibria for binary CH4, CO2 and H2O systems as predicted by Damped Shifted Force Monte Carlo simulations.

Table A.5. Densities of the vapor-liquid equilibrium for the CH4+CO2 binary at 250 K.

| P /MPa | ρliq /kg.m-3 | ρvap /kg.m-3 |
| --- | --- | --- |
| 1.80 ± 0.01 | 1039 ± 2 | 46.35 ± 0.08 |
| 2.10 ± 0.04 | 1023.38 ± 0.02 | 51.02 ± 0.02 |
| 2.40 ± 0.01 | 1008 ± 1 | 55 ± 1 |
| 2.94 ± 0.07 | 975 ± 2 | 63 ± 1 |
| 3.92 ± 0.05 | 904 ± 1 | 77 ± 1 |
| 4.94 ± 0.05 | 833 ± 2 | 97 ± 1 |
| 5.89 ± 0.04 | 756 ± 5 | 121 ± 2 |
| 6.33 ± 0.03 | 709 ± 4 | 133 ± 2 |

Table A.6. Densities of the vapor-liquid equilibrium for the CH4+CO2 binary at 270 K.

| P /MPa | ρliq /kg.m-3 | ρvap /kg.m-3 |
| --- | --- | --- |
| 3.25 ± 0.02 | 941 ± 2 | 87.6 ± 0.9 |
| 3.57 ± 0.05 | 925 ± 3 | 93 ± 1 |
| 3.86 ± 0.07 | 907 ± 2 | 98 ± 2 |
| 4.49 ± 0.03 | 873 ± 4 | 110 ± 2 |
| 5.73 ± 0.05 | 804 ± 2 | 142 ± 1 |
| 6.77 ± 0.01 | 709 ± 5 | 168 ± 2 |

Table A.7. Densities of the vapor-liquid equilibrium for the CO2+H2O binary at 348.15 K.

| P /MPa | ρliq /kg.m-3 | ρvap /kg.m-3 |
| --- | --- | --- |
| 0.2369 ± 0.0002 | 964 ± 2 | 3.44 ± 0.01 |
| 2.09 ± 0.01 | 964 ± 1 | 33.64 ± 0.06 |
| 3.58 ± 0.02 | 966 ± 2 | 60.49 ± 0.09 |
| 5.04 ± 0.05 | 968 ± 2 | 89.9 ± 0.05 |
| 7.55 ± 0.06 | 971 ± 1 | 149.6 ± 0.04 |
| 10.1 ± 0.1 | 974 ± 1 | 226 ± 2 |
| 15.1 ± 0.2 | 977 ± 2 | 442 ± 9 |

Table A.8. Densities of the vapor-liquid equilibrium for the CH4+H2O binary at 323.56 K.

| P /MPa | ρliq /kg.m-3 | ρvap /kg.m-3 |
| --- | --- | --- |
| 4.981 ± 0.001 | 983 ± 3 | 31.46 ± 0.02 |
| 6.491 ± 0.003 | 981.6 ± 0.6 | 41.67 ± 0.02 |
| 8.481 ± 0.002 | 982 ± 2 | 55.50 ± 0.01 |
| 11.487 ± 0.004 | 987 ± 3 | 77.0 ± 0.1 |
| 18.491 ± 0.003 | 987.3 ± 0.9 | 125.94 ± 0.07 |