SUPPLEMENTARY MATERIAL

Detecting phytoestrogens and mammalian estrogens in blackcurrants (Ribes nigrum L.)

Naoki Nanashima 1, 2, Kayo Horie 2, Hayato Maeda 3, Yoji Kato 4

1Department of Nutrition, Faculty of Health Science, Aomori University of Health and Welfare, Aomori, Japan

2Department of Bioscience and Laboratory Medicine, Hirosaki University Graduate School of Health Sciences, Hirosaki, Japan

3Faculty of Agriculture and Life Science, Hirosaki University, Hirosaki, Japan

4YoKa Food Science Laboratory Inc., Hirosaki, Japan

Corresponding Author: Dr Naoki Nanashima, Department of Nutrition, Faculty of Health Science, Aomori University of Health and Welfare, 58-1 Mase, Hamadate Aomori 030-8505, Japan

E-mail: n_nanashima@auhw.ac.jp

CONTENTS

Table S1……………………………………………………………………………………………3-4

Table S2……………………………………………………………………………………………5-7

Fig S1……………………………………………………………………………………..………….8

Table S1 CE-TOFMS measurement conditions

Cationic metabolites (cation mode)

Equipment

Agilent CE-TOFMS system (Agilent Technologies)

Capillary: fused silica capillary i.d.: 50 μm × 80 cm

Condition

Run buffer: cation buffer solution (p/n: H3301-1001)

Rinse buffer: cation buffer solution (p/n: H3301-1001)

Sample injection: pressure injection 50 mbar, 10 s

CE voltage: positive, 30 kV

MS ionization: ESI positive

MS capillary voltage: 4000 V

MS scan range: m/z 50–1000

Sheath liquid: HMT sheath liquid (p/n: H3301-1020)

Anionic metabolites (anion mode)

Equipment

Agilent CE-TOFMS system (Agilent Technologies)

Capillary: fused silica capillary i.d.: 50 μm × 80 cm

Condition

Run buffer: anion buffer solution (p/n: H3302-1023)

Rinse buffer: anion buffer solution (p/n: H3302-1023)

Sample injection: pressure injection 50 mbar, 22 s

CE voltage: positive, 30 kV

MS ionization: ESI negative

MS capillary voltage: 3500 V

MS scan range: m/z: 50–1000

Sheath liquid: HMT sheath liquid (p/n: H3301-1020)

Table S2 LC-TOFMS measurement conditions

Cationic metabolites (cation mode)

Equipment

LC system: Agilent 1200 series RRLC system SL (Agilent Technologies)

Column: ODS column, 2 × 50 mm, 2 μm

MS system: Agilent LC/MSD TOF (Agilent Technologies)

Condition

Column temp.: 40 ℃

Mobile phase A: H2O/0.1% HCOOH

Mobile phase B: isopropanol:acetonitrile:H2O (65:30:5)/0.1% HCOOH, 2 mM HCOONH4

Flow rate: 0.3 mL/min

Run time: 20 min

Post time: 7.5 min

Gradient condition: 0–0.5 min: B 1%, 0.5–13.5 min: B 1–100%, 13.5–20 min: B 100%

MS ionization mode: ESI positive

MS nebulizer pressure: 40 psi

MS dry gas flow: 10 L/min

MS dry gas temp: 350 ℃

MS capillary voltage: 4000 V

MS scan range: m/z: 100–1700

Sample injection: 1 μL

Anionic metabolites (anion mode)

Equipment

LC system: Agilent 1200 series RRLC system SL (Agilent Technologies)

Column: ODS column, 2 × 50 mm, 2 μm

MS system: Agilent LC/MSD TOF (Agilent Technologies)

Condition

Column temp.: 40 ℃

Mobile phase A: H2O/0.1% HCOOH

Mobile phase B: isopropanol:acetonitrile:H2O (65:30:5)/0.1% HCOOH, 2 mM HCOONH4

Flow rate: 0.3 mL/min

Run time: 20 min

Post time: 7.5 min

Gradient condition: 0–0.5 min: B 1%, 0.5–13.5 min: B 1–100%, 13.5–20 min: B 100%

MS ionization mode: ESI negative

MS nebulizer pressure: 40 psi

MS dry gas flow: 10 L/min

MS dry gas temperature: 350 ℃

MS capillary voltage: 3500 V

MS scan range: m/z: 100–1700

Sample injection: 1 μL

Result and Discussion

Fig. S1 Metabolome analysis of blackcurrants and blueberries. The bars indicate the relative areas of each molecule in Aomori and blueberries.