- A proposed technique utilizes Otsu threshold segmentation and two-dimensional discrete wavelet transform to extract edge artifact features from Si3N4 bearing roller microcracks.

- To enhance the contrast and eliminate noise in images with edge artifact features, the two-dimensional discrete wavelet transform function equation is developed.

- To eliminate edge artifacts, the best threshold is chosen after constructing an equation for the threshold segmentation function that maximizes the inter class variance.

- The technique allows silicon nitride bearing roller micro crack edge artifacts to be extracted.

- The method makes the average PSNR of enhanced feature images close to 62.69dB, and the average SSIM is about 0.77.