Optimized Approach for Near-Real-Time Three-Dimensional Water Vapor in the GNSS Based on the Informer Model

Zhu Yixin1, Xia Pengfei1,3, Ye Shirong1,2, Sha Zhimin1, Jiang Junfei4, E Shenglong4

1GNSS Research Center, Wuhan University, Wuhan 430079, China

2School of Electronic Information, Wuhan Qingchuan University, Wuhan, China

3Key Laboratory of Smart Earth, Beijing, China.

4Electric Power Research Institute of Guangdong Power Grid Corporation, Guangzhou, 510080, China.

Supplement 1 Tomography model

Supplement 1 is an additional detailed description about the tomography model mentioned in the paper section 2.1, describing the method we utilized in building the observation equations and the costraint equations. The purpose of GNSS water vapor 3D tomography is to obtain 3D water vapor information in the study area, and the tomography model normally consists of observation equations and constraint equations. Among them, the observation equation is constructed by using GNSS observation signal data, and the constraint equation is often based on empirical formulas.

  - Tomographic observation equation

Slant water vapor (SWV) is an informative volume used to acquire information on the three-dimensional distribution of atmospheric water vapor located on the GNSS signal path as an observation in the water vapor tomography observation equation. This study uses a tomography model based on the chunking method. Assuming that the study area is divided into h, m, and n grids in elevation, longitude, and latitude, respectively, the q-th signal ray can be constructed as the following observational equation:

i=1nj=1mk=1hSi,j,kq∙ρi,j,k=SWVq#(1)

where  i, j, k are the corresponding 3D coordinates of a voxel block traversed by the signal ray, ρi,j,k refers to the water vapor density value of the voxel (unit: g/cm3), Si,j,kq is the intercept length of the signal line within the voxel (unit: km), and SWVq represents the total water vapor content over the path of the q-th GNSS signal (unit: mm).

The observation equation consisting of all signal rays is expressed in matrix form as follows:

vsn×1=Asn×gn∙xgn×1-ysn×1 #(2)

where A refers to the coefficient matrix of the observation equation, consists of the corresponding voxel internal intercepts, y is a column vector of observations that consists of all GNSS SWV values that traverse the tomography region, x is a vector of columns of unknown parameters, consisting of water vapor density values within each voxel, v is the residual vector, sn is the total number of valid GNSS observation signal rays (signal number); gn is the overall number of prime blocks in the tomography grid delineation (grid number), with the value of h×m×n.

When the top height of the h2 (h2<h) grid in the vertical direction from bottom to top of the study area is taken as the top layer of the grid to be tomography, then the parameters to be solved in the observation equation at this time are the values of water vapor density within the h2×m×n voxel blocks, and the rest of the voxel blocks are not inverted for tomography. Assuming that the height of the top of the original tomography layer is H1 and the height of the top of the new layer is H2, then the water vapor content of the oblique paths in the new tomography observation equations that make up the observation vector needs to be excluded from the original SWV based on the water vapor content of each signal path between H1 and H2.

At this point, the total water vapor content of that part of the signal path that makes up the observation vector is:

SWVH0-H2=SWV-SWVH1-H2 #(3)

where SWVH0-H2 presents the water vapor content on the observed signal path between the station height and  H2,  SWV is obtained from GNSS inversion, SWVH1-H2 is the water vapor content of the observed signal path between H1 and H2 that requires rejection, which is calculated using the initial value of the water vapor density of the corresponding voxel block and the signal intercept:

SWVH1-H2=i=1nj=1mk=h2+1hSi,j,kq∙ρi,j,k #(4)

where ρi,j,k is a known value expressed by the initial value of water vapor density. Therefore, the initial value of water vapor density in the part from H1 to H2 demands a high precision value, and the higher the precision, the better the accuracy of SWVH1-H2 calculated for the constituent observation vector.

Theoretically, the lower the height of the top layer of the tomography grid, the higher the number of effective GNSS signal rays, and the higher the accuracy of the to-be-demanded water vapor density value can be obtained. However, from the above analysis, it is clear that the height of the top layer of tomography should not be blindly reduced, and an appropriate H2 value needs to be determined according to the accuracy of the predicted value of water vapor density in the vertical direction.

  - Constraint equations

Owing to the geographic distribution characteristics of the terrestrial GNSS stations and the ray-dispersion-like geometric distribution characteristics of the satellite signals, part of the voxel blocks are not traversed by the observed signals within the voxel blocks (mostly the low-level grids and the lateral interface grids), resulting in the pathological coefficient matrices. Therefore, it is necessary to add a priori constraints to obtain the values of the voxels with no signal traversal corresponding to the parameters to be calculated, based on mathematical-physical relationships or empirical values. Conventional constraints are categorized as horizontal, vertical, and boundary constraints, and only horizontal and vertical constraints are considered in this paper.

Considering that atmospheric water vapor tends to vary steadily with a continuous uniformity over a sub-scale in the horizontal direction, the water vapor density of the grid to be solved is constrained by the water vapor content of the surrounding grids. It is assumed that a total of k voxels in a certain plane, in which the water vapor density xi of an arbitrary voxel i can be expressed by the sum of the weighted water vapor densities of the remaining voxels:

xi=ω1x1+⋯+ωi-1xi-1+ωi+1xi+1+⋯+ωjxj #(5)

where xi is the water vapor density value of the i-th voxel block, ωi represents the horizontal constraint coefficient of the i-th voxel block, which is determined by the distance of the weighted voxel from the target (to-be-estimated) voxel, and the closer the distance, the larger the value, and vice versa. In this paper, we use the Gaussian distance weighting function to determine the value of ωi, as follows:

ωi=e-di,j22σ2iie-di,jj22σ2 #(6)

where di,j refers to the distance between voxel i and voxel j, di,jj2 stands for the sum of distances between voxel i and other voxels in the same stratum, and σ denotes the smoothing factor. Furthermore, when i=j, keep ωi=-1. When the center of gravity of neighboring voxels in the same layer is 5 km apart, σ should be around 15.8 mm/km2 to ensure that the above equation is valid.

Considering the exponential decrease of atmospheric water vapor in the vertical direction with increasing height, an exponential function can express the relationship between the water vapor parameters of two adjacent layers as a vertical constraint, as follows:

ρk+1ρk=exp-hk+1-hkH #(7)

where k is the number of horizontal layers of the grid, ρk+1 and ρk denote the water vapor density of the layer k+1 and k, respectively, hk+1 and hk denote the heights of the layer k+1 and k, separately, and H is the water vapor elevation, with values usually taken as 1-2 km, which can also be obtained by fitting the data of the meteorological parameters in the field.

Finally, the overall tomography equations can be obtained by combining the observation and constraint equations:

vvHvVvP=Am×nHc1×nVc2×nVc3×n∙xn×1-ym×10c1×10c2×10c3×1 #(8)

where A stands for the coefficient matrix of the observation equation, H, V, P represent the coefficient matrices of the horizontal, vertical, and initial value constraints, respectively, suffix m, c1, c2, c3 are the total number of the observation equations, horizontal, vertical and a priori constraints, while v, vH, vV, vP are the residuals of the four, respectively, and n is the number of parameters to be solved.

Supplement 2 ART algorithm

In this study, we use the Algebraic Reconstruction Technique (ART) to solve the tomography equation, so the supplement 2 is the detailed algorithmic process of ART, mentioned in section 2.2 of the paper.

The steps are as follows: Iterate through each satellite-to-station GNSS observation signal, and calculate the difference between the tomography reconstruction value  SWViART  and the observation value  SWVi  by using the water vapor density value in each voxel block. By using this deviation and the corresponding crossing grid distance, the water vapor density values in the voxel block are corrected until the iteration termination condition is satisfied. For the grid which is not traversed by the observation signal, the additional constraints are mainly used to correct the parameters to be solved. The advantage of this method is that it avoids the inverse operation of the coefficient matrices of the observation equations of tomography, which makes the computational efficiency become higher and also avoids the emergence of ill-posed results. Therefore, its iteration speed is fast, the solution efficiency is high, and the stability is high. The iterative formula is as follows:

ρjk+1=ρjk+λai,jj=1nai,j2SWVi-j=1nai,jρjk #(9)

where ρjk is the water vapor density value of the j-th independent voxel block in the k-th iteration, λ refers to the relaxation factor, which is a constant used to regulate the degree of convergence during the iteration process, ai,j stands for the element of the i-th row and j-th column in the coefficient matrix A of the observation equation for tomography,  SWVi is the element of the i-th row of the observation vector, and  j=1nai,jρjk denotes the tomography reconstruction value  SWViART .

Supplement 3 ERA5 dataset and relevant parameters

The supplement 3 is an additional file for section 3.1 in the paper, containing the detailed information of the ERA5 dataset we used in the experiment, and the formulas for obtaining the parameters we utilized in the model.

ERA5 is the fifth generation of a global high-resolution atmospheric reanalysis dataset of ECMWF. It is obtained from data assimilation systems and model forecasts, providing hourly estimates of an extensive range of meteorological parameters. The dataset has a temporal resolution of 1 hour, a horizontal grid resolution of 0.25°×0.25° (interpolated 0.125°×0.125° grid data are also available), and is vertically organized into 37 atmospheric pressure strata, with a delay of 5 days, and is downloadable free of charge from the website (https://www.ecmwf.int/en/forecasts/dataset/ecmwf-reanalysis-v5).

In this study, ERA5 reanalysis data at 0.125° × 0.125° grid resolution over the Hong Kong region from January 1, 2018, to December 31, 2022, are used, with the first four years of data utilized to train the Informer-WV model and the latter year's data utilized to predict the real-time water vapor density involved in tomography. Horizontally, 20 grid points adjacent to the study area are selected, as shown in Fig. 1 left; vertically, meteorological parameter data from a total of 21 baroclinic layers in the range of 1000 hPa (closest to the surface) - 250 hPa (corresponding to a potential height of about 11km) are used, as shown in Fig. 1 right.

Fig. 1 The study area ERA5 dataset latitude/longitude division with grid point layout (left) and 3D distribution schematic (right).

Hence, based on the known meteorological parameters of the ERA5 reanalysis data, the water vapor density values of the corresponding grid points can be calculated according to the water vapor equation of state:

ρ=eRvT #(10)

where ρ is the water vapor density (unit: g/m3), e is the water vapor pressure (unit: hPa), T is the temperature (unit: K), and Rv is the specific gas constant for water vapor with a value of 461.5 J∙kg-1∙K-1.

The wet refractive indices of the corresponding grid points were calculated from the atmospheric refractive index model:

Nw=k2-k1RdRv∙eT+k3eT2 #(11)

where Nw refers to the refractive index of the wet component of the atmosphere (unit: dimensionless), kii=1,2,3 are the refractive constants, which are usually taken as k1=77.6 K∙hPa-1, k2=70.4 K∙hPa-1 and  k3=3.739×105 K∙hPa-1, Rd and Rv are the molar masses of the dry and wet atmospheres, respectively, and are usually taken as constants Rd=28.96 kg∙kmol-1andRv=18.02 kg∙kmol-1, respectively.

Therefore, the following nine parameters can be acquired at each grid point: Longitude (Lon, unit: °), Latitude (Lat, unit: °), Pressure (P, unit: hPa), Geopotential height (H, unit: m), Temperature (T, unit: K),  Specific Humidity (q, unit: g/kg), Water Vapor Pressure (e, unit: hPa), Wet Refractivity (Nw, unit: dimensionless), and Water Vapor Density(ρ, unit: g/m3). These nine parameters are utilized in Section 2.2 in the paper to perform the training and prediction of the Informer-WV model.