# APPENDIX A

Table A.1. Detailed description of the support activities that the collaborative system can perform categorised by the different elementary type of task.

| TASK TYPE | COLLABORATIVE SYSTEM SUPPORTIVE ACTIVITY | DESCRIPTION |
| --- | --- | --- |
| Identification & Handling | Target component | The collaborative system shows the operator which component is to be assembled from the set of components present in the servicing area. |
|  | Real-time identification feedback | The collaborative system provides real-time feedback on the correctness of the component identified by the operator |
|  | Sorting Assistance | The collaborative system provides a systematic and continuous arrangement of parts based on specific attributes like size, type, or function. By reducing alternatives diminishes the likelihood of selection inaccuracies, thereby promoting process efficiency. |
|  | List of activity sharing | The collaborative system maintains a real-time, dynamic list outlining the sequence of components for identification and assembly. This list adapts to assembly progress and component availability, enhancing the operator's contextual knowledge and decision-making. |
|  | Component defects detection | The collaborative system scans and evaluates components to determine if they meet specified requirements, indicating any discrepancies to assist the human operator in decision-making. |
|  | Lifting Assistance | The collaborative system provides mechanical assistance, such as a counterbalance or leverage aid, to help the operator lift heavy components. |
|  | Component pre-positioning | The collaborative system places the component in an optimal position and orientation for the operator to easily pick it up. |
|  | Posture monitoring | The collaborative system monitors the operator's posture during the handling task and provides feedback or adjustments. This aims to reduce long-term physical strain and the risk of injury by encouraging ergonomic practices. |
|  | Real-time Handling feedback | The collaborative system continuously monitors the operator's actions during the handling process, focusing on whether the component has been gripped in the correct manner and whether it is following the correct trajectory. |
|  | Grip Assistance | The collaborative system displays the most ergonomic and efficient point to grip the component. This minimizes physical strain and maximizes timeliness of the handling process. |
|  | Safety Alerts | The collaborative system detect potentially unsafe handling practices by the operator, such as gripping at points that could lead to slippage or imbalance. Upon detecting such actions, the collaborative system issues immediate safety alerts. |
|  | Stress-driven Handling scheduling | The collaborative system suggests an ideal work pace to the operator and dynamically adjusts it according to the operator's current workload. |
|  | Process-driven Handling Scheduling | The collaborative system assesses the current state of the process and the importance of pending tasks to suggest an optimal pace for task execution to the human operator. |
|  | Tool Handover | The collaborative system provides the necessary tool to the human operator for component handling, facilitating tool identification and reducing the likelihood of errors. |
|  | Data Logging | The collaborative system records handling-related data for future analysis and continuous improvement. This enables more informed decision-making and facilitates the fine-tuning of the process over time. |
| Alignment | Target component positioning | The collaborative system places the component onto its mounting position, requiring the human operator only to precisely align it for the subsequent joining phase. |
|  | Active Alignment Stabilization | The collaborative system holds the subassembly in a stable position, reducing operator effort and increasing the speed of alignment. |
|  | Subassembly pre-orientation | The collaborative system orients the sub-assembly in a manner that facilitates the identification of the mounting position and the correct alignment of the component by the operator. |
|  | Operator-Optimized Subassembly Orientation | Upon receiving specific requests from the operator, the collaborative system dynamically adjusts the position and orientation of the subassembly to better suit the operator's needs for the alignment task.. |
|  | Real-Time Alignment Feedback (Profile Matching) | The collaborative system monitors the alignment process in real time and reports any errors in the procedure. This allows for immediate corrective action to be taken by the operator, enhancing the efficiency and quality of the process. |
|  | Alignment sequence sharing | The collaborative system provides the operator with a sequence of steps for optimal alignment. This action enhances the operator's understanding of the task at hand, thereby aiding in decision-making and improving the overall speed of the alignment process. |
|  | Stress-driven Alignment scheduling | The collaborative system suggests an ideal work pace to the operator and dynamically adjusts it according to the operator's current workload. |
|  | Process-driven Alignment Scheduling | The collaborative system assesses the current state of the production line, including bottlenecks, delays, and inventory levels. Based on this information, it suggests an optimal work pace to the operator to meet process-driven needs. |
|  | Safety Alerts | The collaborative system issues alerts if it detects that the operator is about to align the component in a manner that could be unsafe, such as aligning near sharp edges, hot surfaces, or electrical components. |
|  | Precision Positioning | Involves the collaborative system fine-tuning the alignment after the human operator has completed the initial alignment. |
|  | Alignment instructions | The collaborative system provides step-by-step instructions to the operator for correctly executing the alignment. |
|  | Visual Field Optimization | The collaborative system provides accurate and targeted illumination within the work area, facilitating accurate identification and manipulation of components during assembly processes. |
| Joining | Synchronized Component Joining | The collaborative system aids in stabilization and alignment and also takes part in the joining, while the operator focuses on complementary joining tasks. |
|  | Secure Holding | The collaborative system maintains the sub-assembly stable during the fastening phase, reducing the effort required from the operator and favoring the timeliness of the task. |
|  | Subassembly Pre-orientation | The collaborative system sets the sub-assembly's position and orientation to facilitate a more comfortable and efficient fastening process for the operator. |
|  | Operator-Optimized Subassembly Orientation | Upon receiving specific requests from the operator, the collaborative system dynamically adjusts the position and orientation of the subassembly to better suit the operator's needs for the joining task.. |
|  | Joining Instructions | The collaborative system provides step-by-step instructions to the operator for correctly executing the fastening. |
|  | Real Time Joining Feedback | The collaborative system monitors the progress of the fixing in real time, providing feedback to the operator on its accuracy. This allows for immediate corrective action to be taken by the operator, enhancing the efficiency and quality of the process. |
|  | Ergonomic Subassembly Positioning | The collaborative system positions the subassembly to avoid awkward or uncomfortable assembly angles, bringing it to a level that is ergonomic for the operator. |
|  | Adaptive Ergonomic Subassembly Positioning | The collaborative system dynamically adjusts the position of the sub-assembly based on the operator's physical stress levels, i.e. bringing it closer to the operator or lowering it. |
|  | Stress-driven Joining scheduling | The collaborative system suggests an ideal work pace to the operator and dynamically adjusts it according to the operator's current workload.. |
|  | Process-driven Joining Scheduling | The collaborative system assesses the current state of the production line, including bottlenecks, delays, and inventory levels. Based on this information, it suggests an optimal work pace to the operator to meet process-driven needs |
|  | Safety Alerts | The collaborative system continuously monitors the joining process and identifies if the operator is engaging in potentially hazardous actions or touching risky areas on the sub-assembly. |
|  | Tool Handover | The collaborative system identifies and supplies the necessary joining tool to the human operator. By automating tool retrieval, it allows the operator to focus on critical aspects of the joining process, thus enhancing overall workflow efficiency |
|  | Visual Field Optimization | The collaborative system adjusts the lighting conditions to provide the human operator with an optimal visual field for the joining task. |
|  | Data Logging | The collaborative system records relevant data during the joining process, such as torque values, component positions, and time stamps. This data is made accessible to the human operator for future analysis and continuous improvement of the joining process. |
| Checking | Human-Error Mitigation | The collaborative system continuously monitors the subassembly for quality compliance and provides immediate feedback to the operator if any non-conformities persist post-inspection. |
|  | Operator-Optimized product Orientation | Upon receiving specific requests from the operator, the collaborative system dynamically adjusts the position and orientation of the product to better suit the operator's needs for the inspection. |
|  | Inspection Instructions | The collaborative system provides a step-by-step procedure for the operator to follow during the inspection process. |
|  | Safety Alerts | The collaborative system actively monitors the assembly and the operator's actions during checking. This allows the operator to be alerted if he is approaching hazardous areas or if a defective assembly is detected that could potentially result in unsafe conditions |
|  | Ergonomic Product positioning | Based on the areas requiring inspection, the collaborative system autonomously adjusts the position of the product to ensure ergonomics. |
|  | Adaptive Ergonomic Product Positioning | The collaborative system dynamically repositions the sub-assembly during the inspection process, taking into account the operator's physiological stress indicators. This may involve bringing the sub-assembly closer or adjusting its height to facilitate more comfortable and efficient inspection. |
|  | Stress-driven Checking scheduling | The collaborative system suggests an ideal work pace to the operator and dynamically adjusts it according to the operator's current workload. |
|  | Process-driven Checking Scheduling | The collaborative system analyzes the current state of the assembly process to suggest an optimal schedule for checking tasks. |
|  | Defect Tracking Assistance | The collaborative system assists the human operator by systematically tracking defects detected during the inspection or assembly process and logging them in a dedicated system. This action enables informed adjustment decisions and defect management. |
|  | Visual Field Optimization | The collaborative system furnishes targeted lighting to assist the operator in inspecting areas that suffer from low visibility or inadequate illumination. |
|  | Tool Handover | The collaborative system supplies the operator with the necessary inspection tools and positions them conveniently. This action eliminates the need for the operator to search for or retrieve tools, thereby streamlining the inspection process and reducing effort. |
| Adjustment | In-Process Validation | The collaborative system performs real-time validations during adjustments to confirm that all changes fall within specified tolerances. |
|  | Real-Time Adjustment Guidance | The collaborative system supplies operators with real-time data concerning the state of the assembled component during the adjustment process. This targeted feedback improves decision-making, leading to a more accurate and efficient inspection process. |
|  | Operator-Optimized Product Orientation | In response to specific directives from the operator, the collaborative system dynamically modifies the product position and orientation to streamline the adjustment process. |
|  | Adjustment Instructions | The collaborative system delivers step-by-step guidance to the operator, outlining the appropriate torque and tolerances to be maintained during the adjustment process. |
|  | Safety Alerts | The collaborative system monitors the operator's activities and the condition of the assembly during the adjustment. It provides immediate warnings if the operator is about to interact with defective, dangerously shaped or fragile components, reducing potential safety risks. |
|  | Ergonomic Product positioning | The collaborative system autonomously reorients the component to enhance ergonomic accessibility for the operator during the adjustment process. |
|  | Adaptive Ergonomic Positioning | The collaborative system responsively changes the sub-assembly's orientation in real-time, based on the operator's physiological stress indicators. This optimization may involve bringing the sub-assembly nearer or altering its elevation to create a more ergonomic setting for adjustment activities. |
|  | Stress-driven Adjustment Scheduling | The collaborative system proposes a suitable work rhythm to the operator, making dynamic adjustments in response to the operator's ongoing workload. |
|  | Process-driven Adjustment Scheduling | The collaborative system evaluates the prevailing conditions of the assembly process to offer a well-timed schedule for adjustment activities. |
|  | Visual Field Optimization | The collaborative system adjusts the lighting conditions to provide the human operator with an optimal visual field during the adjustment. |
|  | Tool Handover | The collaborative system provides the operator with essential tools for adjustment tasks and places them strategically for easy access. By eliminating the operator's need to locate and retrieve tools, the adjustment process becomes more efficient and less labor onerous. |

Table A.2. Detailed description of the support activities that the human can perform categorised by the different elementary type of task.

| TASK TYPE | HUMAN SUPPORTIVE ACTIVITY | DESCRIPTION |
| --- | --- | --- |
| Identification & Handling | Target Component | The operator indicates to the collaborative system the specific component for assembly, thereby enhancing the collaborative system’s accuracy in the identification stage and boosting the overall efficiency of the task. |
|  | Component Feeding | The operator places the chosen component for assembly in a designated, consistent location. This action serves as a cue for the collaborative system, which is programmed to exclusively retrieve components from this designated area. |
|  | Component Defects Detection | The human operator scans and assesses components to verify if they conform to specified tolerances, alerting the collaborative system to any deviations that require further action or decision-making. |
|  | Component pre-positioning | The operator places the component in a specific orientation that aligns with the collaborative system's programmed picking mechanism. |
|  | Handling prompting | The act of signaling to the collaborative system on when to execute a handling task. |
|  | Balance Assistance | During the component's movement, the operator assists the collaborative system by manually stabilizing the component to maintain its balance. This action minimizes the risk of component slippage or misalignment during transit, thereby enhancing the accuracy and safety of the handling process. |
|  | Gripping Optimization | After the collaborative system secures an initial grip on the component, the operator conducts a rapid verification to assess its adequacy, minimizing the risk of component slippage or breakage. |
|  | Handling Instructions | The operator provides cues or signals to the collaborative system to guide it in picking up and moving the component. By offering real-time guidance, the operator enhances the collaborative system's ability to handle components more efficiently and accurately. |
|  | Tool Handover | The human equips the collaborative system with the most suitable tool for handling the component involved in the assembly. By doing so, efficiency and flexibility of handling are maximised. |
|  | Data Logging | Human operator records inefficiencies or non-conformities detected during the handling process. This data collection enables the future refinement of gripping procedures or the redefinition of the gripping tools used by the robot. |
| Alignment | Target Component Positioning | The human operator positions the component onto its mounting location, enabling the collaborative system to focus on precise alignment for the subsequent joining phase. |
|  | Subassembly Pre-Orientation | The operator ensures that the subassembly is positioned in a manner that facilitates the collaborative system's alignment task. This involves placing the subassembly in a stable and accessible location, so the collaborative system can easily align the component. |
|  | Active Alignment Stabilization | The operator holds the subassembly in a stable position, increasing the speed and precision of alignment. |
|  | Precision Positioning | Following the collaborative system's initial alignment, the operator conducts a rapid review, adjusting enhance alignment precision. This step minimizes the requirement for subsequent corrections, thereby optimizing assembly process efficiency |
|  | Alignment prompting | The act of signaling to the collaborative system on when to execute an alignment task. |
|  | Alignment Instructions | The operator communicates to the collaborative system the precise position on the subassembly where the component should be positioned. |
|  | Manual Alignment Assistance | As the collaborative system proceeds with aligning the component onto the subassembly, the operator collaboratively ensures the alignment's accuracy and correctness. The human's role is to provide real-time adjustments and confirmations during the collaborative system's alignment process. |
| Joining | Synchronized Component Joining | The human operator and collaborative system work in tandem to join components. The human operator aids in stabilization and alignment while also participating in the actual joining tasks, allowing the collaborative system to focus on complementary aspects of the joining process. |
|  | Secure Holding | The operator holds the subassembly or component in a stable position to facilitate the collaborative system's joining task. |
|  | Subassembly Pre-Orientation | The operator positions the subassembly in a predetermined orientation that aligns with the collaborative system's programmed task sequence. This ensures that the collaborative system can easily access and work on the subassembly. |
|  | Joining Prompting | The act of signaling to the collaborative system on when to execute a joining task. |
|  | Joining Instructions | The operator guides the collaborative system in the exact positioning of connectors and the specific procedures for completing the joining process. This dynamic interaction enables the collaborative system to adapt to various product configurations, with the operator's decision-making determining the appropriate methods and steps to follow. |
|  | Data Logging | The human operator logs any inefficiencies or non-conformities encountered during the joining process. This data collection informs future refinements in the methods of component assembly or the tools used for secure and accurate joining. |
|  | Tool Handover | The operator replaces the collaborative system's current tool for one that is better suited to the shapes and sizes of the components to be joined. This ensures that the collaborative system is equipped with the most appropriate tool for the specific joining task at hand, enhancing collaborative system's adaptability to different component shapes. |
| Checking | Collaborative system Error Mitigation | The human operator continuously monitors the product for quality compliance and provides immediate feedback to the collaborative system if any non-conformities are detected post-inspection. |
|  | Product Orientation for Inspection | The operator adjusts the orientation of the product to provide optimal access to the inspection areas, aligning it according to the collaborative system's needs. |
|  | Inspection Instructions | The operator provides step-by-step guidance to the collaborative system during the inspection process, indicating which areas to examine in a specific order. This approach leverages the operator's expertise and problem-solving skills with the collaborative system's accuracy. |
|  | Checking prompting | The act of signaling to the collaborative system on when to start a checking task. |
|  | Defect Tracking Assistance | Human operator aids the collaborative system by systematically identifying and logging defects detected during the inspection into a dedicated system. |
|  | Tool Handover | The operator mounts the necessary inspection tool on the collaborative system, enabling inspection. |
| Adjustments | Product Orientation for Adjustment | The operator reorients the product to facilitate targeted adjustments, aligning it in accordance with the collaborative system needs. |
|  | In-Process Validation | The human operator conducts real-time validations during the collaborative system's adjustments to ensure all changes are within specified tolerances. |
|  | Adjustment Instruction | The operator provides step-by-step guidance to the collaborative system during the adjustment process, indicating the appropriate torque or tolerances to be maintained. |
|  | Real-Time Adjustment Guidance | The human operator provides the collaborative system with real-time data about the state of the assembled component during the adjustment process. This targeted input aids the collaborative system's decision-making, resulting in a more accurate and efficient adjustment process. |
|  | Adjustment Prompting | The act of signaling to the collaborative system on when to start an adjustment task. |
|  | Tool Handover | The operator affixes the requisite tool to the collaborative system, thereby enabling precise adjustment. |