Table 1. Monte Carlo Algorithm Results vs. SANS 241:2015 Standard

| Value | Frequency | Probability of Impact |
| --- | --- | --- |
| >0 | 8524 | 85.24% |
| Value | Frequency | Probability that Impact exceeds SANS 241:2015 Aesthetic Limit |
| >250 | 6631 | 66.31% |
| Value | Frequency | Probability that Impact exceeds SANS 241:2015 Acute/Chronic Health Limit |
| >500 | 4082 | 40.82% |

Table 2. Rating of Probability Percentage

| Probability Percentage | 0-20 | 20-40 | 40-60 | 60-80 | 80-100 |
| --- | --- | --- | --- | --- | --- |
| Rating | 1 | 2 | 3 | 4 | 5 |

Table 3: Risk Rating Categories

| Risk Rating (Probability × Severity) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Very Low 1-2 | Low 3-9 | Medium 10-15 | High 16-20 | Very High >20 |

Table 4. Input data for the Shallow Weathered Zone Aquifer

| Water Quality Parameter | Data Source | Shallow Weathered Zone Aquifer |  |  |  | Deep Fractured Zone Aquifer |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Count (n) | Average | Standard Deviation | Log-Transformed (Yes/No) | Count (n) | Average | Standard Deviation | Log-Transformed (Yes/No) |
| EC | Waste Facility | 585 | 309.558 | 248.624 | No | 1647 | 262.094 | 220.484 | No |
|  | Background Data | 32 | 65.816 | 26.466 | No | 232 | 94.448 | 36.745 | No |
| Ca | Waste Facility | 585 | 190.563 | 189.068 | No | 1656 | 214.649 | 200.917 | No |
|  | Background Data | 32 | 27.351 | 23.143 | No | 232 | 67.535 | 42.856 | No |
| Mg | Waste Facility | 584 | 151.047 | 143.624 | No | 1654 | 117.847 | 121.683 | No |
|  | Background Data | 32 | 33.440 | 28.491 | No | 232 | 32.687 | 21.465 | No |
| Na | Waste Facility | 585 | 2.336 | 0.463 | Yes | 1654 | 269.726 | 360.226 | No |
|  | Background Data | 32 | 73.966 | 36.611 | No | 232 | 108.133 | 90.074 | No |
| K | Waste Facility | 584 | 0.562 | 0.612 | Yes | 1654 | 8.689 | 19.403 | No |
|  | Background Data | 32 | 2.186 | 0.898 | No | 232 | 0.590 | 0.342 | Yes |
| Cl | Waste Facility | 584 | 2.380 | 0.608 | Yes | 1644 | 377.769 | 442.337 | No |
|  | Background Data | 32 | 18.303 | 11.019 | No | 232 | 55.932 | 66.025 | No |
| SO4 | Waste Facility | 584 | 2.551 | 0.816 | Yes | 1645 | 760.505 | 957.367 | No |
|  | Background Data | 32 | 1.011 | 0.841 | Yes | 230 | 50.914 | 66.152 | No |
| NO3 as N | Waste Facility | 560 | -0.574 | 0.820 | Yes | 1604 | 0.323 | 0.813 | Yes |
|  | Background Data | 32 | -1.101 | 0.692 | Yes | 229 | -0.380 | 0.809 | Yes |
| NH4 as N | Waste Facility | 574 | -0.324 | 0.670 | Yes | 1467 | -0.582 | 0.507 | Yes |
|  | Background Data | 31 | -0.530 | 0.649 | Yes | 190 | 0.391 | 0.711 | Yes |
| F | Waste Facility | 548 | -0.740 | 0.587 | Yes | 1611 | 0.556 | 0.590 | Yes |
|  | Background Data | 32 | 0.716 | 0.237 | No | 232 | 0.479 | 0.460 | No |
| Zn | Waste Facility | 104 | -1.931 | 0.627 | Yes | 538 | -1.989 | 0.451 | Yes |
|  | Background Data | 32 | -2.096 | 5.874 | Yes | 32 | -2.096 | 5.874 | Yes |
| B | Waste Facility | 540 | -0.270 | 0.679 | Yes | 1269 | 0.442 | 0.758 | Yes |
|  | Background Data | 31 | 0.144 | 0.064 | No | 193 | -0.961 | 0.578 | Yes |
| Al | Waste Facility | 534 | -1.643 | 0.786 | Yes | 1137 | 0.121 | 0.647 | No |
|  | Background Data | 30 | -1.858 | 0.671 | Yes | 166 | -1.577 | 0.676 | Yes |
| Fe (Total) | Waste Facility | 551 | -0.590 | 0.897 | Yes | 1369 | 5.170 | 20.054 | No |
|  | Background Data | 31 | -1.043 | 0.436 | Yes | 219 | 0.903 | 2.730 | No |
| Mn | Waste Facility | 545 | -0.847 | 0.693 | Yes | 1219 | 0.509 | 1.481 | No |
|  | Background Data | 31 | -1464 | 0.857 | Yes | 186 | -1.685 | 0.606 | Yes |
| As | Waste Facility | 140 | -2.299 | 0.177 | Yes | 516 | 0.248 | 2.856 | No |
|  | Background Data | 32 | -2.286 | 0.264 | Yes | 32 | 0.008 | 0.017 | No |
| Cd | Waste Facility | 141 | 0.003 | 0.003 | No | 527 | -2.971 | 0.480 | Yes |
|  | Background Data | 37 | -2.674 | 6.393 | Yes | 37 | -2.674 | 6.393 | Yes |
| Cr (Total) | Waste Facility | 128 | -2.403 | 0.462 | Yes | 530 | -2.448 | 0.514 | Yes |
|  | Background Data | 33 | -2.254 | 5.981 | Yes | 33 | -2.254 | 5.981 | Yes |
| Co | Waste Facility | 135 | -2.521 | 0.439 | Yes | 494 | -2.707 | 0.419 | Yes |
|  | Background Data | 27 | -2.689 | 5.524 | Yes | 27 | -2.689 | 6.744 | Yes |
| Cu | Waste Facility | 119 | -2.144 | 0.363 | Yes | 560 | -2.179 | 0.420 | Yes |
|  | Background Data | 43 | -2.148 | 6.744 | Yes | 43 | -2.148 | 6.744 | Yes |
| Cn (Total) | Waste Facility | 155 | -2.601 | 0.527 | Yes | 829 | -2.503 | 0.621 | Yes |
|  | Background Data | 25 | -2.652 | 5.339 | Yes | 25 | -2.652 | 5.339 | Yes |
| Pb | Waste Facility | 143 | -2.161 | 0.356 | Yes | 572 | -2.166 | 0.398 | Yes |
|  | Background Data | 32 | -2.108 | 5.880 | Yes | 32 | -2.108 | 5.880 | Yes |
| Hg | Waste Facility | 63 | 0.005 | 0.001 | No | 128 | -2.234 | 0.770 | Yes |
|  | Background Data | 14 | -2.401 | 4.134 | Yes | 14 | -2.401 | 4.134 | Yes |
| Li | Waste Facility | 365 | -1.048 | 1.064 | Yes | 742 | -1.121 | 0.839 | Yes |
|  | Background Data | 68 | -1.801 | 8.357 | Yes | 68 | -1.801 | 8.357 | Yes |
| Ni | Waste Facility | 137 | -2.233 | 0.438 | Yes | 527 | -2.256 | 0.462 | Yes |
|  | Background Data | 27 | -2.475 | 5.477 | Yes | 27 | -2.475 | 5.477 | Yes |
| Se | Waste Facility | 100 | -2.270 | 0.280 | Yes | 505 | -2.363 | 0.338 | Yes |
|  | Background Data | 20 | -2.169 | 4.751 | Yes | 20 | -2.169 | 4.751 | Yes |
| U | Waste Facility | 85 | 0.009 | 0.008 | No | 326 | -1.860 | 0.404 | Yes |
|  | Background Data | 12 | -2.051 | 3.778 | Yes | 12 | -2.051 | 3.778 | Yes |
| V | Waste Facility | 92 | -1.916 | 0.690 | Yes | 535 | -2.220 | 0.500 | Yes |
|  | Background Data | 31 | -1.962 | 5.771 | Yes | 31 | -1.962 | 5.771 | Yes |

Table 5: Average Water Quality Concentrations for the Shallow Weathered Zone Aquifer versus the SANS 241:2015 Standard

| Water Quality Parameter | pH | EC | Ca |  | Mg | Na | K | Si | Cl | SO4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Units | - | mS/m | mg/L |  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
| Average Concentration | 7.7 | 310 | 191* |  | 151* | 372 | 12 | 6 | 473 | 967 |
| Water Quality Parameter | NO3(N) | NH4(N) | F |  | Zn | B | Al | Fe (Total) | Mn | As |
| Units | mg/L | mg/L | mg/L |  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
| Average Concentration | 2.30 | 2.16 | 0.402 |  | 0.054 | 1.728 | 0.286 | 4.071 | 0.609 | 0.007 |
| Water Quality Parameter | Cd | Cr (Total) | Co |  | Cu | CN (Total) | Pb | Hg | Li | Ni |
| Units | mg/L | mg/L | mg/L |  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
| Average Concentration | 0.003 | 0.017 | 0.005 |  | 0.013 | 0.007 | 0.012 | 0.005 | 1.013** | 0.012 |
| Water Quality Parameter | Se | U | V |  |  |  |  |  |  |  |
| Units | mg/L | mg/L | mg/L |  |  |  |  |  |  |  |
| Average Concentration | 0.008 | 0.009 | 0.257* |  |  |  |  |  |  |  |
| SANS 241: 2015 Limits | Aesthetic |  |  | *Limits from SANS 241:2006 Drinking Water Standards ** Limits from the U.S. Environmental Protection Agency (2008) |  |  |  |  |  |  |
|  | Acute/Chronic Health |  |  |  |  |  |  |  |  |  |
|  | Operational |  |  |  |  |  |  |  |  |  |

Table 6: Average Water Quality Concentrations for the Deep Fractured Zone Aquifer versus the SANS 241:2015 Standard

| Water Quality Parameter | pH | EC | Ca |  | Mg | Na | K | Si | Cl | SO4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Units | - | mS/m | mg/L |  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
| Average Concentration | 7.6 | 262 | 215* |  | 118* | 270 | 9 | 14 | 378 | 761 |
| Water Quality Parameter | NO3(N) | NH4(N) | F |  | Zn | B | Al | Fe (Total) | Mn | As |
| Units | mg/L | mg/L | mg/L |  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
| Average Concentration | 2.65 | 2.57 | 0.760 |  | 0.021 | 1.185 | 0.121 | 5.170 | 0.509 | 0.248 |
| Water Quality Parameter | Cd | Cr (Total) | Co |  | Cu | CN (Total) | Pb | Hg | Li | Ni |
| Units | mg/L | mg/L | mg/L |  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
| Average Concentration | 0.003 | 0.022 | 0.004 |  | 0.011 | 0.012 | 0.019 | 0.326 | 0.594** | 0.011 |
| Water Quality Parameter | Se | U | V |  |  |  |  |  |  |  |
| Units | mg/L | mg/L | mg/L |  |  |  |  |  |  |  |
| Average Concentration | 0.01 | 0.02 | 0.014* |  |  |  |  |  |  |  |
| SANS 241: 2015 Limits | Aesthetic |  |  | *Limits from SANS 241:2006 Drinking Water Standards ** Limits from the U.S. Environmental Protection Agency (2008) |  |  |  |  |  |  |
|  | Acute/Chronic Health |  |  |  |  |  |  |  |  |  |
|  | Operational |  |  |  |  |  |  |  |  |  |

Table 7. GWQP Analysis and Frequencies of Measurements

| Quarterly GWQP |  | Biannual GWQP |  | Annual GWQP |  |
| --- | --- | --- | --- | --- | --- |
| Water Quality Parameter | Measurement Unit of Concentration | Water Quality Parameter | Measurement Unit of Concentration | Water Quality Parameter | Measurement Unit of Concentration |
| pH | pH Units | Ammonium as N | mg/L | Aluminium | mg/L |
| Electrical Conductivity | mS/m | Boron | mg/L | Arsenic | mg/L |
| Total Dissolved Solids | mg/L | Phosphate | mg/L | Cadmium | mg/L |
| Total Alkalinity | mg/L | Silica | mg/L | Chromium (Total) | mg/L |
| Calcium | mg/L |  |  | Chromium (Hexavalent)* | mg/L |
| Magnesium | mg/L |  |  | Cobalt | mg/L |
| Sodium | mg/L |  |  | Copper | mg/L |
| Potassium | mg/L |  |  | Cyanide (Total) | mg/L |
| Chloride | mg/L |  |  | Lead | mg/L |
| Sulphate | mg/L |  |  | Lithium | mg/L |
| Nitrate as N | mg/L |  |  | Nickel | mg/L |
| Fluoride | mg/L |  |  | Selenium | mg/L |
| Iron (Total) | mg/L |  |  | Uranium | mg/L |
| Manganese | mg/L |  |  | Vanadium | mg/L |
|  |  |  |  | Zinc | mg/L |
| * Only when chromium total is detected |  |  |  |  |  |