Supplementary Table 1 Comparison of the practical applications of various types of electrodes for electrophysiology.

| Sample | ECG | EMG | EEG | Wearable disease detection | Human-machine-interaction | Ref |
| --- | --- | --- | --- | --- | --- | --- |
| AgPH | Higher SNR ECG recording both in static and exercise state | No | No | No | No | 10.1007/s42114-022-00596-y |
| IG-50 | Higher SNR ECG recording, with comparison to Ag/AgCl in the static state | No | No | No | No | 10.1039/C5TC01888A |
| PTG | Higher SNR and lower noise voltage vs. Ag/AgCl both in static and dynamic status | Higher SNR and lower noise voltage vs. Ag/AgCl both in static and dynamic status | EEG monitoring for ~12 hours | No | Multichannel, four gestures repetition by robotic hands | 10.1038/s41467-021-25152-y |
| PWS | Lower noise voltage vs. Ag/AgCl both in static and dynamic status | No comparison with other electrodes | EEG record during sound stimuli | Atrial fibrillation detection with the absence of P wave | Open hands and close hands repetition of artificial limbs | 10.1038/s41467-020-18503-8 |
| CEAB | Higher SNR vs Ag/AgCl gel both in the static and dynamic state | Higher SNR and lower RMS noise vs Ag/AgCl for ~3.5 h | ~12 h EEG monitoring during exercise, rest, sleep | Spinal pathway health detection with thigh EMG signals during knee jerk reflex. Depression detection with explainable EEG digital biomarkers | Single channel, finger movement record, six types of hand gesture recognition, and repetition of artificial limbs | This work |

Supplementary Table 2 Young’s modulus of CEAB film with different thickness

| Thickness (μm) | Young’s modulus (kPa) |
| --- | --- |
| 3.55 | 59.74 ± 0.23 |
| 50 | 3.99 ± 0.01 |
| 150 | 3.97 ± 0.01 |
| 250 | 3.90 ± 0.01 |
| 500 | 3.23 ± 0.01 |

Supplementary Table 3 Comparison of our electrode with representative ionic gels

| Sample | Self-healing | Skin-like | Thickness (μm) | Visible light transmittance (%) | σ (mS/cm) | Adhesion (J/m2) | λ (%) |  |  |  | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| gelatin-supported DES gel | No | No | 500 | 80-90 | 2.5 | - |  | ~300 |  | https://doi.org/10.1039/C8TC05918G |  |
| PEA and the solvent [BMMIm][TFSI] | No | No | 1000 | >90 | 0.1 | - |  | >800 |  | 10.34133/2020/2505619 |  |
| PAAc–DES gel | No | No | 1000 | ~92 | 1.26 | 100 (too much) |  | >1000 |  | 10.1038/s41528-021-00118-8 |  |
| SAP | No | No | 500 | 93 | > 1 | < 0.0127 (too less) |  | <304 | 10.1021/acsami.8b18947 |  |  |
| CEAB | Yes | Yes | 3.55 | ~100 | 1.33 | ~20 (mild strong) |  | ~800 | This work |  |  |