SUPPORTING INFORMATION

Chemistry of some physicochemical methods

Dissolved oxygen (DO) by Winkler’s method

MnSO4 + 2KOH   →   Mn(OH)2  + K2SO4	     (white ppt)

2Mn(OH)2 + O2	→  2MnO(OH)2        (brown ppt)

MnO(OH)2 + 2H2SO4 + 3H2O	   (acidification) (acidification)

Mn(SO4)2  +  2KI   →    MnSO4  + K2SO4  + I2   ↑

MnSO4 + 2KOH   →   Mn(OH)2  + K2SO4	     (white ppt)

2Mn(OH)2 + O2	→  2MnO(OH)2        (brown ppt)

MnO(OH)2 + 2H2SO4 + 3H2O	   (acidification) (acidification)

Mn(SO4)2  +  2KI   →    MnSO4  + K2SO4  + I2   ↑

Chemical oxygen demand (COD)

2Cr2O72-(aq)  + 8H2SO4(aq)       →          SO42-(aq)  +  2Cr2(SO4)3(aq) + 8H2O(l) + 3O2(g)  ↑

C6H12O6  + 6O2(g)   →  6CO2(g) + 6H2O(l)

2Cr2O72-(aq)  + 8H2SO4(aq)       →          SO42-(aq)  +  2Cr2(SO4)3(aq) + 8H2O(l) + 3O2(g)  ↑

C6H12O6  + 6O2(g)   →  6CO2(g) + 6H2O(l)

Ag2SO4

Ag2SO4

Cr2O72-(aq) + 6Fe2+(aq) + 14H+(aq)   →   6Fe3+(aq) + 2Cr3+(aq) + 7H2O(l)

Cr2O72-(aq) + 6Fe2+(aq) + 14H+(aq)   →   6Fe3+(aq) + 2Cr3+(aq) + 7H2O(l)

Ca2+   + EDTA   →  Ca.EDTA complex

↑

Ca2+   + EDTA   →  Ca.EDTA complex

↑

Total hardness by EDTA Titrimetric method

Ag+(aq)  +  Cl-(aq)   →   AgCl(s)

↑

Ag+(aq)  +  Cl-(aq)   →   AgCl(s)

↑

Chloride determination by Mohr Titration

Ag+  +  CrO42-   →  Ag2CrO4   (reddish-brown)

Ag+  +  CrO42-   →  Ag2CrO4   (reddish-brown)

H+

Ba2+  +  SO42-    →    BaSO4      (white suspension)

H+

Ba2+  +  SO42-    →    BaSO4      (white suspension)

Sulphate (SO42-) determination by Turbidimetric method

2H+

PO43-(aq)   +  (NH4)6Mo7O24.4H2O    →    Molybdo-phosphoric acid

+  SnCl2.2H2O

Mo6+(aq)    (blue colour)

2H+

PO43-(aq)   +  (NH4)6Mo7O24.4H2O    →    Molybdo-phosphoric acid

+  SnCl2.2H2O

Mo6+(aq)    (blue colour)

Determination of total phosphate by stannous chloride colourimetric method

H+

Ca(C17H35COO)2    →   2C17H35COOH  +  Ca2+

H+

Ca(C17H35COO)2    →   2C17H35COOH  +  Ca2+

Determination of oil and grease by partition – gravimetric method

Sediment Quality Guidelines (SQGs) for Metals (μg/g)

| Description | Cd | Fe | Pb | Cu |
| --- | --- | --- | --- | --- |
| Average metal concentration in Apapa Bay | 2.070 | 32.847 | 36.309 | 11.714 |
| Swedish Environmental Sediment Quality Guideline (SESQG, 1996) |  |  |  |  |
| Effect range low | 0.3 | - | 34 | 47 |
| Effect range medium | 3. 5 | - | 410 | 270 |
| Portuguese Legislation Classification of Sediments in Coastal Zone |  |  |  |  |
| Class 1: Clean dredge materials | - | - | <35 | <50 |
| Class 2: Trace contaminated sediment | - | - | 35 - 150 | 50 - 150 |
| Class 3: Lightly contaminated sediment | - | - | 150 - 300 | 150 - 500 |
| Class 4: Contaminated sediment | - | - | 300 - 500 | 500 - 1000 |
| Class 5: Highly contaminated sediment | - | - | >500 | >1000 |
| Washington Department of Environment (WDOE, 1995) |  |  |  |  |
| Non polluted | ND | <25 | - | <40 |
| Moderately polluted | >0.5 | - | 25 - 75 | 40 - 70 |
| Heavily polluted | 1.0 | - | >75 | >70 |
| Ontario Ministry of Environment and Energy (1993) |  |  |  |  |
| No effect level | - | - | - | - |
| Lowest effect level | 0.6 | 2 | 31 | 16 |
| Severe effect level | 10.0 | 4 | 250 | 110 |

ND – Not detected

Analysis of variance between Apapa Bay Terminals and Takwa Bay

| Toxic metal | Terminal A | Terminal C | Terminal D | Takwa Bay (control) |
| --- | --- | --- | --- | --- |
| Cd | 3.024 + 0.057 a | 1.210+0.001 a | 1.976+0.057 a | 0.600+0.001 a |
| Fe | 35.625+2.946 a | 30.000+0.884 b | 32.917+0.880 c | 6.876+0.010 abc |
| Pb | 65.902+6.730 a | 1.756+0.170 b | 41.268+6.380 c | 8.585+0.510 abc |
| Cu | 13.423+0.907 a | 10.859+0.907 a | 10.859+0.904 a | 10.859+0.910 a |

Means with the same superscript column-wise are not significantly different

Terminal  A   =   APM Terminal             Terminal  C   =   Ship Repair and Building Yard

Terminal  D   = Petrochemical /Tank farm