Table 1 Quality classification standard for entropy weight water quality index of groundwater

| EWQI | Rank | Water quality |
| --- | --- | --- |
| <50 | 1 | Excellent |
| 50-100 | 2 | Good |
| 100-150 | 3 | Medium |
| 150-200 | 4 | Poor |
| >200 | 5 | Extremely poor |

Table 2 Quality classification standard for IWQI of groundwater

| IWQI | Rank | Water quality |
| --- | --- | --- |
| 0-25 | 1 | Excellent |
| 26-50 | 2 | Good |
| 51-75 | 3 | Poor |
| 76-100 | 4 | Very poor |
| >100 | 5 | Unsuitable |

Table 3 Calculation parameters for chemical non-carcinogens

| Parameters | Unit | F- | NO-3 |
| --- | --- | --- | --- |
| Skin permeability coefficient (Kp) | cm/hr | 0.001 | 0.001 |
| Reference dose (RfD) | mg/kg·d | 0.06 | 1.6 |

Table 4 Exposure dose calculation parameters for computing the health risks through oral and dermal pathways

| Parameters | Unit | Children | Females | Males |
| --- | --- | --- | --- | --- |
| IR | L/d | 1.5 | 2.66 | 3.62 |
| EForal | d/a | 365 | 365 | 365 |
| ED | Year | 6 | 30 | 30 |
| BW | kg | 25.9 | 64.0 | 73.0 |
| AT | d | 2190 | 10950 | 10950 |
| tevent | h | 0.33 | 0.25 | 0.25 |
| EV | d-1 | 1 | 1 | 1 |
| EFdermal | d/a | 200 | 200 | 200 |
| SA | cm2 | 8650 | 15350 | 17000 |

Table 5 Definition and unit of acronym in the calculation of HI and IWQI

| Acronym | Definition | Unit |
| --- | --- | --- |
| CDI | Chronic daily intake | mg/(kg·day) |
| CW | Chemical concentration in water | mg/L |
| IR | Ingestion rate | L/day |
| EForal | Exposure frequency through drinking | days/year |
| ED | Exposure duration | years |
| BW | Body weight | kg |
| AT | Averaging time | days |
| DAD | Dermal absorbed dose | mg/(kg·day) |
| Kp | Dermal permeability coefficient of compound in water | cm/hr |
| tevent | Event duration | hr/event |
| EV | Event frequency | events/day |
| EFdermal | Exposure frequency through skin contact | days/year |
| SA | Skin surface area available for contact | cm2 |
| EC | Electrical conductivity | μs/cm |
| SAR | Sodium adsorption ratio | - |
| Na% | Sodium percent | - |
| PI | Permeability index | - |
| MH | Magnesium hazard | - |
| RSC | Residual sodium carbonate | meq/L |

Table 6 The dissolution formula of mineral and the formation of chemical components in groundwater

| Mineral | formula |
| --- | --- |
| Halite | NaCl → Na+ + Cl- |
| Albite | 2NaAlSi3O8+2CO2+11H2O→Al2Si2O5(OH)4+4H4SiO4+2Na++2HCO3- |
| Gypsum | CaSO4→Ca2++SO42- |
| Calcite | CaCO3 + CO2 + H2O→Ca2+ + 2HCO3- |
| Dolomite | CaMg(CO3)2 + 2CO2 + 2H2O→Mg2+ + Ca2+ + 4HCO3- |
| Cation exchange reaction | Ca2++2NaX→CaX2+2Na+ Mg2++2NaX→MgX2+2Na+ Ca2++2KX→CaX2+2K+ Mg2++2KX→MgX2+2K+ |

Table 7 Groundwater in the study area children, adult females, and men F- and NO-3 no carcinogenic risk assessment

| Group | Values | F |  |  | NO- 3 |  |  | HI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HIoral | HIdermal | HIF | HIoral | HIdermal | HIF | HIoral | HIdermal | HIF |
| Children | Min | 9.7E-3 | 1.0E-5 | 9.7E-3 | 0 | 0 | 0 | 0.106 | 1.1E-4 | 0.106 |
|  | Max | 0.728 | 7.6E-4 | 0.729 | 6.690 | 7.0E-3 | 6.700 | 6.914 | 7.2E-3 | 6.921 |
|  | Mean | 0.292 | 3.1E-4 | 0.292 | 1.005 | 1.1E-3 | 1.006 | 1.336 | 1.4E-3 | 1.337 |
| Females | Min | 6.9E-3 | 5.5E-6 | 6.9E-3 | 0 | 0 | 0 | 7.6E-2 | 5.9E-5 | 0.076 |
|  | Max | 0.522 | 4.1E-4 | 0.523 | 4.801 | 3.8E-3 | 4.805 | 4.962 | 3.9E-3 | 4.970 |
|  | Mean | 0.210 | 1.7E-4 | 0.210 | 0.721 | 5.7E-4 | 0.722 | 0.959 | 7.4E-4 | 9.6E-1 |
| Males | Min | 8.3E-3 | 5.3E-6 | 8.3E-3 | 0 | 0 | 0 | 0.091 | 5.7E-5 | 9.1E-2 |
|  | Max | 0.623 | 4.0E-4 | 0.624 | 5.728 | 3.7E-3 | 5.732 | 5.920 | 3.8E-3 | 5.924 |
|  | Mean | 0.250 | 1.6E-4 | 0.250 | 0.861 | 5.5E-4 | 0.861 | 1.140 | 7.2E-4 | 1.145 |

Table 8 Pollutant concentration distributions and exposure parameter distributions used in Monte Carlo simulation

|  | Parameters | Distribution type | Distributed parameter |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Children | Females | Males |
| Contaminant | F- | Lognormal | LN(0.30, 0.02) |  |  |
|  | NO- 3 | Lognormal | LN(27.77, 1036.62) |  |  |
| Exposure parameters | IR | Normal | N(1.5, 0.15) | N(2.66, 0.27) | N(3.62, 0.36) |
|  | BW | Lognormal | LN(25.9, 2.59) | LN(64.0, 6.40) | LN(73.0, 7.30) |
|  | EF | Triangular | T(180.0, 365.0, 345.0) |  |  |
|  | SA | Lognormal | LN(8650, 865) | LN(15350, 1535) | LN(17000, 1700) |