SUPPLEMENTARY APPENDIX

This appendix has been provided by the authors to give readers additional information about their work.

CONTENTS:

SUPPLEMENTAL METHODS

Detailed description of the GEP recording: Water and sucrose solutions were driven through the taste stimulation system by compressed air. Two parallel silicone tubes were used: one for water and the other for a taste solution. Water and sucrose solutions were switched back and forth by two electromagnetic valves controlled by an electronic device. The electronic box sent a real-time signal to the computer software for collecting taste evoked potentials (GEPs) when the taste solution was administered (with an accuracy of one millisecond), to obtain reliable GEP latencies. The semi-rigid silicone tubes were placed side by side on the middle of the tongue, approximately 1.5 ± 0.5 cm from the anterior arch. The solutions were delivered on the tongue through a small hole at the end of each tube. The sucrose solution was delivered intermittently at a flow rate of 200 mL/hour by one tube and water alternately by the other tube, at a flow rate of 100 mL/hour. A flow rate of 200 mL/hour was chosen for the sucrose solutions because it allowed uniform stimulation of a large part of the tongue in a quasi-instantaneous manner (21-24 cm², a methylene blue test having been previously performed). The water flow rate was set at 100 mL/hour to minimize swallowing in order to eliminate artifacts in the GEP recordings. Preliminary tests were performed to ensure that the subjects did not perceive the difference in flow rates and that the change in flow rate did not create artifacts in GEP recordings (control recordings with water in both tubes did not induce cortical responses). The recording sessions lasted 20 minutes. Concentration of the sucrose solution was set at 10 g.100 mL-1 water (Evian® water) as in previous studies (24,32). During each session, the sucrose solution was delivered 20 times on the tongue for one second. The inter tasting stimulus interval was 1 minute to avoid adaptation and to stimulate the taste receptors during the refractory period. During the GEP recordings, subjects listened to soft music to mask the rattling noise of the solenoid valves (preliminary results did not detect any evoked potentials in response to the noise from these ones). Subjects also had to keep their eyes closed to avoid light stimulation and eye opening/closing artifacts. Subjects also had a nose clip during taste stimulation to avoid that retro-nasal olfaction skews taste results. GEPs were recorded using external scalp electrodes, according to the international 10/20 nomenclature of the EEG. Three electrodes were placed on Pz, Cz, and Fz. The electrodes were referenced against linked earlobes (ear clip electrodes enfolded by Ag, 10 mm diameter - SystemPLUS EVOLUTION), and the ground electrode was placed on the forehead. Disposable cup electrodes enfolded by Ag-AgCl (6 mm diameter), with a long polyurethane cable (SystemPLUS EVOLUTION) were used. The EEG recordings were amplified, filtered and digitized with Micromed software (SystmPLUS EVOLUTION, 2007, Micromed S.p.A, Italy) with the following data: time constant 1 second, sampling frequency 2048 Hz, 50 Hz filter (main voltage), 200 Hz low-pass filter and 0.4 Hz high-pass filter during the GEPs recording sessions. The GEP results obtained after each of the 20 stimulations were averaged for the same subject and stimulus to improve the signal-to-noise ratio. The GEP records were analyzed with the abovementioned software. The software averaged the responses and then detected the different peaks. All GEP recordings were reviewed and re-analyzed by the same well-trained neurophysiologist who was blinded to the type of stimulation, in order to adjust the different response sliders, if necessary. The GEPs were defined according to three peaks: the first positive peak (P1), the second negative peak (N1), and the third positive peak (P2). For each GEP average (that is to say, for the same subject and stimulus at the same time), the initial P1 latency, peak N1 latency and P1N1 amplitude were collected. The P1 latency corresponded to the time between the initial stimulation and the P1 peak. The N1 latency was the time between the initial stimulation and the N1 peak. The P1N1 amplitude was calculated as an absolute value from P1 peak to N1 peak. Then, two analyses were performed: on the one hand, a statistical average of the results in each condition was done; on the other hand, a GEP curve called “grand average” was done by pooling GEP curves of all similar subjects recording in a similar condition. The presence of two separate analyses to obtain the results explains why the statistical average of N1 peaks did not correspond exactly to the arrows in the grand average figures.

SUPPLEMENTARY TABLE:

Table S1: Comparison of plasma levels of hormones involved in food eating behavior (acylated ghrelin, insulin, leptin and serotonin) between obese and normal-weight subjects, before and after a session without (fasting session) or with ingestion of a standardized meal (feeding session).

Mean ± standard deviations of plasma levels of hormones. F-value with degree of freedom and p-values for the global model (multiple-way ANOVA); ns: statistically not-significantly different.

SUPPLEMENTARY FIGURES:

Figure S1: Design of the study, describing the course of the fasting session and the feeding session. Indirect calorimetry and bio-electric impedancemetry were performed only one time, either at the feeding session, or at the fasting session, in a random order; GEP: gustatory Evoked Potentials

Figure S2: Recordings of gustatory evoked potentials (GEPs) on the Fz electrode in the morning and afternoon, during a feeding session (A) and a fasting session (B) in patients living with obesity (OB) and normal-weight (NW) subjects.

The curves show the average of the 28 OB and the 22 NW subjects after stimulation with a sucrose solution (for each subject, the curve represents an average of 20 taste stimuli recorded over 20 minutes). Statistical analyses were performed using ANOVAs with several factors (session effect, obesity effect, hourly effect). A p-value of less than 0.01 was considered statistically significant (Bonferroni correction). Results are expressed as mean ± SD. Significant differences obtained with the obesity effect are indicated by * with p <0.001, and differences with the time effect are indicated by # with p <0.001.

Figure S3: Recordings of gustatory evoked potentials (GEPs) on the Pz electrode in the morning and afternoon, during a feeding session (A) and a fasting session (B) in patients living with obesity (OB) and normal-weight (NW) subjects.

The curves show the average of the 28 OB and the 22 NW after stimulation with a sucrose solution (for each subject, the curve represents an average of 20 taste stimuli recorded over 20 minutes). Statistical analyses were performed using ANOVAs with several factors (session effect, obesity effect, hourly effect). A p-value of less than 0.01 was considered statistically significant (Bonferroni correction). Results are expressed as mean ± SD. Significant differences obtained with the obesity effect are indicated by * with p <0.001, and differences with the time effect are indicated by # with p <0.001.

Figure S4: Comparisons of hunger sensation between fasting and feeding sessions in patients living with obesity and normal-weight subjects.

Hunger sensation at the end of the morning and at the beginning of the afternoon are represented in the 2 separated sessions (fasting and feeding sessions). Results are expressed as means ± SD. Analyses were performed using ANOVAs with several factors (session effect, obesity effect, time effect). A p-value of less than 0.01 was considered statistically significant (Bonferroni correction). * indicates significant difference (p <0.01).

Figure S5: Correlation between plasma levels of hormones that may be involved in food eating behavior [acylated ghrelin (A), leptin (B), insulin (C) and serotonin (D)] and GEP latencies in P1Cz after sucrose taste stimulation in patients living with obesity (grey points) and normal-weight (black points) subjects.

Similar significant correlations are observed for P1Pz and P1Fz latencies. The analyses were performed using Spearman correlations.