Impacts of climate change on thermal stratification and water quality of Reservoirs

Sina Najari a, Nima Heidarzadeh a*

a Department of Civil Engineering, Faculty of Engineering, Kharazmi University, Tehran, Iran,

* E-mail: N.heidarzadeh@khu.ac.ir (Corresponding Author)

Journal of Water Resources Management

Appendix

Table A Calibrated parameters for thermal (a), dissolved oxygen and phosphate (b) stratifications

|  | Parameters | Default values | Calibrated values |
| --- | --- | --- | --- |
| (a) | Bottom friction solution | Chezy | Chezy |
|  | Vertical Eddy Viscosity | W2 | W2 |
|  | Time-weighting for vertical advection scheme | 0.5 | 0.5 |
|  | Longitudinal eddy viscosity (m2/s) | 1 | 1 |
|  | Longitudinal eddy diffusivity (m2/s) | 1 | 1 |
|  | Wind sheltering Coefficient sheltering | 0.9 | 0.8 |
|  | Solar radiation absorbed in surface layer | 0.45 | 0.45 |
|  | Water surface roughness height | 0.001 | 0.001 |
|  | Extinction coefficient for pure water | 0.45 | 0.45 |
|  | Solar radiation extinction coefficient(1/m) | 0.07 | 0.07 |
|  | Fraction radiation absorbed by ice | 0.6 | 0.6 |
|  | Coefficient of water-ice heat exchange (W/m2 °C) | 10 | 10 |
|  | Minimum ice thickness before ice formation is allowed (m) | 0.03 | 0.03 |
|  | Ratio of reflection to incident radiation (albedo of ice) | 0.15 | 0.15 |
| (b) | Maximum algal growth rate (1/day) | 2 | 1.2 |
|  | Algae extinction coefficient | 0.2 | 0.2 |
|  | Maximum algal respiration rate (1/day) | 0.04 | 0.04 |
|  | Maximum algal excretion rate (1/day) | 0.04 | 0.04 |
|  | Maximum algal mortality rate (1/day) | 0.1 | 0.1 |
|  | Lower temperature for algal growth (°C) | 5 | 5 |
|  | Lower temperature for maximum algal growth(°C) | 30 | 32 |
|  | Upper temperature for maximum algal growth(°C) | 35 | 35 |
|  | Upper temperature for algal growth (°C) | 40 | 40 |
|  | Fraction of Org-P | 0.005 | 0.005 |
|  | Fraction of Org-N | 0.05 | 0.08 |
|  | Equation number for algal ammonium preference (either 1 or 2) | 1 | 1 |
|  | Sediment release rate of phosphorus | 0.015 | 0.015 |
|  | Partitioning Coefficients of Phosphorous | 1.2 | 1.2 |
|  | Lower temperature for ammonia decay (°C) | 5 | 5 |
|  | Lower temperature for maximum ammonia decay(°C) | 25 | 25 |
|  | Nitrate decay rate(1/day) | 0.03 | 0.05 |
|  | Oxygen stoichiometry for algal respiration | 1.1 | 1.1 |
|  | Light saturation intensity at maximum photosynthetic rate (W/m2) | 100 | 100 |
|  | Specifies either implicit, IMP, or explicit, EXP, treatment of the vertical eddy viscosity in the longitudinal momentum equation. | IMP | IMP |
|  | Sediment release rate of phosphorus, fraction of SOD | 0.001 | 0.001 |
|  | Sediment release rate of ammonia, fraction of SOD | 0.001 | 0.001 |
|  | Ammonia decay | 0.12 | 0.12 |

| a | b |
| --- | --- |
| c | d |
| e | f |
| g | h |

Fig. A Daily meteorological variables (a) air temperature, (b) dew point, (c) wind direction, (d) wind speed, (e) cloudiness and (f) daily Qin, Qout, and storage volume from 2012 to 2022 and the observed vs. simulated inflow in (g) Armand and (h) Bazoft.

| a | b |
| --- | --- |
| c | d |

Fig. B The observed vs. simulated (a) geometry calibration, (b) Water surface level of baseline (2019-2020) and, Water surface level of (c) SSP1-1.9 and (d) SSP8-8.5

| a |  |  |
| --- | --- | --- |
| b |  |  |
| c |  |  |

Fig. C Observed and simulated (a) thermal, (b) and (c) PO4 profile

a

b

c

Fig. D Daily temperature for the future period 2081-2100 under two scenarios in (a) surface layer, (b) middle layer and (c) bottom layer.

a

b

c

Fig. E Daily PO4 concentration for the future period 2081-2100 under two scenarios in (a) surface layer, (b) middle layer and (c) bottom layer.