# Supplemental Materials

This study’s Supplemental Materials have been included a separate attachment. These materials contain the following files:

- A note describing the sample collection method and a table listing the analytical methods used to quantify each analyte. (These are also included below for convenience.)

- Correlograms displaying the autocorrelation and partial autocorrelation for each analyte at each sampling location. These values were calculated using timeseries of the daily concentrations.

- Plots displaying the %Error for each interpolated load calculation. These are the individual series contained in Figure 7. A rolling mean (of size 5) has also been plotted.

- Descriptive statistics for each analyte at each sampling location. These values were calculated using timeseries of the daily concentrations.

- Timeseries containing the daily analyte concentrations on the primary axis and coincident mean daily streamflow values on the secondary axis. A red “x” is used to denote values below the detection limit.

Summary of Collection and Analysis Methods

- All samples were collected using the standard grab sampling procedure of the IDNR (UHL001). In this procedure, grab samples are collected by dipping an HPDE bottle from a bridge. When there is ice cover, a hole in the ice is chopped and the HPDE bottle is dipped for sample collection at the ice surface. Dissolved Oxygen samples were collected in stainless steel containers.

- The following table contains the analysis method used to quantify each analyte.

| Short Name | Analyte | Analysis Method |
| --- | --- | --- |
| 2-Chloro-4 | 2-Chloro-4-isopropylamino-6-amino-s-triazine | EPA Method 507 |
| 2-Chloro-6 | 2-Chloro-6-ethylamino-4-amino-s-triazine | EPA Method 507 |
| Acetochlor | Acetochlor | EPA Method 507 |
| Alachlor | Alachlor | EPA Method 507 |
| Ammonia | Ammonia-nitrogen (as N) | EPA Method 350.1* |
| Atrazine | Atrazine | EPA Method 507 |
| Butylate | Butylate | EPA Method 507 |
| Cyanazine | Cyanazine | EPA Method 507 |
| DO | Dissolved oxygen | Standard Methods 4500-O-G |
| Fecal Coliform | Fecal Coliform | Standard Methods 9222-D |
| Metolachlor | Metolachlor | EPA Method 507 |
| Metribuzin | Metribuzin | EPA Method 507 |
| Nitrate | Inorganic nitrogen (nitrate and nitrite) (as N) | EPA Method 353.2* |
| OP | Orthophosphate (as P) | EPA Method 365.1* |
| TP | Phosphate-phosphorus (as P) | EPA Method 365.4* |
| Temp | Temperature, water | Standard Methods 2550 |
| Trifluralin | Trifluralin | EPA Method 507 |
| Turbidity | Turbidity | EPA Method 180.1 |

*Analytical methods were not officially documented for these parameters; however, based on similar testing conducted by the IDNR around the same time, it is highly likely the methods listed here were used.