Table 2. Groundwater observation water-level data for well 1

| Well No. 1 |  |  |  | Depth: 70 m |  |  |  |  | Discharge: 1296 m3/day |  |  | Diameter: 30 cm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lon. 44°18'02.3" |  |  | Lat. 31°45'04.828" |  |  |  | Test Date: 10/4/2021 |  |  |  | 9:00 A.M |  | Alt.: 30 m.a.s.l |  |
| Static Water-level: 8.3 m |  |  |  |  |  | Dynamic Water-level: 38.16 m |  |  |  |  | SC: 33.96 m2/day |  |  |  |
| No. | t(min) | D.D (M) |  |  | "S (m) |  |  | "t. min |  | Residual D.D "S (m) |  |  |  | "t/t min |
| 1 | 0 | 8.3 |  |  | ….. |  |  | 0 |  | 29.86 |  |  |  | ….. |
| 2 | 1 | 11.21 |  |  | 2.91 |  |  | 1 |  | 25.31 |  |  |  | 361 |
| 3 | 2 | 14.12 |  |  | 5.82 |  |  | 2 |  | 20.96 |  |  |  | 181 |
| 4 | 3 | 17.11 |  |  | 8.81 |  |  | 3 |  | 16.71 |  |  |  | 120 |
| 5 | 4 | 20.09 |  |  | 11.79 |  |  | 4 |  | 12.07 |  |  |  | 90 |
| 6 | 5 | 23.51 |  |  | 15.21 |  |  | 5 |  | 8.85 |  |  |  | 72 |
| 7 | 7 | 26.05 |  |  | 17.75 |  |  | 7 |  | 7.18 |  |  |  | 52 |
| 8 | 10 | 29.68 |  |  | 21.38 |  |  | 10 |  | 6.49 |  |  |  | 36 |
| 9 | 15 | 31.13 |  |  | 22.83 |  |  | 15 |  | 5.73 |  |  |  | 24 |
| 10 | 20 | 32.74 |  |  | 24.44 |  |  | 20 |  | 5.13 |  |  |  | 18 |
| 11 | 25 | 33.19 |  |  | 24.89 |  |  | 25 |  | 4.87 |  |  |  | 14 |
| 12 | 30 | 34.86 |  |  | 26.56 |  |  | 30 |  | 4.05 |  |  |  | 12 |
| 13 | 45 | 35.08 |  |  | 26.78 |  |  | 45 |  | 3.02 |  |  |  | 8 |
| 14 | 60 | 36.91 |  |  | 28.61 |  |  | 60 |  | 2.61 |  |  |  | 6 |
| 15 | 90 | 37.69 |  |  | 29.39 |  |  | 90 |  | 2.23 |  |  |  | 4 |
| 16 | 120 | 38.03 |  |  | 29.73 |  |  | 120 |  | 1.16 |  |  |  | 3 |
| 17 | 180 | 38.08 |  |  | 29.78 |  |  | 180 |  | 0.14 |  |  |  | 2 |
| 18 | 240 | 38.13 |  |  | 29.83 |  |  | 240 |  | 0.09 |  |  |  | 2 |
| 19 | 300 | 38.16 |  |  | 29.86 |  |  | 300 |  | 0.05 |  |  |  | 1 |
| 20 | 360 | 38.16 |  |  | 29.86 |  |  | 360 |  | 0 |  |  |  | 1 |

t. min: Time, D.D (M): Drawdown of groundwater by minute, "S (m): Drawdown ratio to the static water level by meter, "t. min: Time of groundwater return per minute. Residual D.D "S (m): Return of groundwater by meters when the pump has shut down.

Table 3. Groundwater observation water level data for well 3

| Well No. 3 |  |  | Depth: 60 m |  |  |  |  | Discharge: 345.6 m3/day |  |  |  | Diameter: 20 cm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long. 44°21'41.295" |  |  | lat. 31°53'40.308" |  |  |  | Test Date: 13/4/2021 |  |  | 7.00 A.M |  |  | Alt.: 9 m.a.s.l |  |
| Static Water-level: 3.25 m |  |  |  |  | Dynamic Water-level: 15.54 m |  |  |  |  |  | SC: 22.23 m2/day |  |  |  |
| No. | t. min | D.D (M) |  | "S (m) |  | "t. min |  |  | Residual D.D "S (m) |  |  |  |  | "t/t min |
| 1 | 0 | 3.52 |  | ……. |  | 0 |  |  | 12.02 |  |  |  |  | ………… |
| 2 | 1 | 9.05 |  | 5.53 |  | 1 |  |  | 9.13 |  |  |  |  | 361 |
| 3 | 2 | 10.37 |  | 6.85 |  | 2 |  |  | 7.19 |  |  |  |  | 180.5 |
| 4 | 3 | 11.72 |  | 8.2 |  | 3 |  |  | 5.05 |  |  |  |  | 120.3 |
| 5 | 4 | 11.98 |  | 8.46 |  | 4 |  |  | 3.94 |  |  |  |  | 90.3 |
| 6 | 5 | 12.15 |  | 8.63 |  | 5 |  |  | 3.21 |  |  |  |  | 72.2 |
| 7 | 7 | 12.81 |  | 9.29 |  | 7 |  |  | 2.68 |  |  |  |  | 51.6 |
| 8 | 10 | 13.26 |  | 9.74 |  | 10 |  |  | 2.25 |  |  |  |  | 36.1 |
| 9 | 15 | 13.78 |  | 10.26 |  | 15 |  |  | 2.09 |  |  |  |  | 24.1 |
| 10 | 20 | 14.06 |  | 10.54 |  | 20 |  |  | 2.01 |  |  |  |  | 18.1 |
| 11 | 25 | 14.51 |  | 10.99 |  | 25 |  |  | 1.92 |  |  |  |  | 14.4 |
| 12 | 30 | 14.96 |  | 11.44 |  | 30 |  |  | 1.78 |  |  |  |  | 12.0 |
| 13 | 45 | 15.03 |  | 11.51 |  | 45 |  |  | 1.53 |  |  |  |  | 8.0 |
| 14 | 60 | 15.21 |  | 11.69 |  | 60 |  |  | 1.28 |  |  |  |  | 6.0 |
| 15 | 90 | 15.32 |  | 11.8 |  | 90 |  |  | 1.03 |  |  |  |  | 4.0 |
| 16 | 120 | 15.43 |  | 11.91 |  | 120 |  |  | 0.84 |  |  |  |  | 3.0 |
| 17 | 180 | 15.49 |  | 11.97 |  | 180 |  |  | 0.61 |  |  |  |  | 2.0 |
| 18 | 240 | 15.53 |  | 12.01 |  | 240 |  |  | 0.31 |  |  |  |  | 1.5 |
| 19 | 300 | 15.54 |  | 12.02 |  | 300 |  |  | 0.09 |  |  |  |  | 1.2 |
| 20 | 360 | 15.54 |  | 12.02 |  | 360 |  |  | 0.02 |  |  |  |  | 1.0 |

Table 4. Data results from solution methods for studied wells

| Well | Solution method | T (m2/day) | K (m/day) |
| --- | --- | --- | --- |
| 1 | Moench’s (1984) | 43.99 | 0.261 |
|  | Theis (Recovery) | 38.53 | 0.229 |
| 2 | Moench’s (1984) | 72.02 | 0.377 |
|  | Theis (Recovery) | 44.81 | 0.234 |
| 3 | Moench’s (1984) | 22.21 | 0.097 |
|  | Theis (Recovery) | 14.29 | 0.062 |
| 4 | Moench’s (1984) | 24.04 | 0.139 |
|  | Theis (Recovery) | 10.92 | 0.063 |
| 5 | Moench’s (1984) | 124.26 | 0.634 |
|  | Theis (Recovery) | 82.04 | 0.418 |
| 6 | Moench’s (1984) | 222.85 | 1.027 |
|  | Theis (Recovery) | 207.8 | 0.957 |
| 7 | Moench’s (1984) | 98.81 | 0.482 |
|  | Theis (Recovery) | 92.99 | 0.453 |

The SC was calculated to be 25.05 m2/day. Table (3) shows the hydraulic properties of this well and Fig. (4) shows the procedure for analyzing hydraulic properties.

Table 5. Groundwater observation water level data for well 2

| Well No. 2 |  |  |  | Depth: 60 m |  |  |  |  | Discharge: 604.8 m3/day |  |  | Diameter: 25 cm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long. 44°16'49.89" |  |  | Lat. 31°48'05.2" |  |  |  | Test Date: 12/4/2021 |  |  |  | 7.30 A.M |  | Alt.: 9 m.a.s.l |  |
| Static Water-level: 1.74 m |  |  |  |  |  | Dynamic Water-level: 24.14 m |  |  |  |  | SC: 25.05 m2/day |  |  |  |
| No. | t. min | D.D (M) |  |  | "S (m) |  |  | "t. min |  | Residual D.D "S (m) |  |  |  | "t/t min |
| 1 | 0 | 1.74 |  |  | …… |  |  | 0 |  | 22.4 |  |  |  | ……. |
| 2 | 1 | 5.06 |  |  | 3.32 |  |  | 1 |  | 17.18 |  |  |  | 361 |
| 3 | 2 | 9.31 |  |  | 7.57 |  |  | 2 |  | 13.52 |  |  |  | 181 |
| 4 | 3 | 14.32 |  |  | 12.58 |  |  | 3 |  | 10.94 |  |  |  | 120 |
| 5 | 4 | 16.77 |  |  | 15.03 |  |  | 4 |  | 8.19 |  |  |  | 90 |
| 6 | 5 | 18.92 |  |  | 17.18 |  |  | 5 |  | 7.26 |  |  |  | 72 |
| 7 | 7 | 19.74 |  |  | 18 |  |  | 7 |  | 6.72 |  |  |  | 52 |
| 8 | 10 | 20.84 |  |  | 19.1 |  |  | 10 |  | 5.93 |  |  |  | 36 |
| 9 | 15 | 21.63 |  |  | 19.89 |  |  | 15 |  | 5.12 |  |  |  | 24 |
| 10 | 20 | 21.96 |  |  | 20.22 |  |  | 20 |  | 4.68 |  |  |  | 18 |
| 11 | 25 | 22.39 |  |  | 20.65 |  |  | 25 |  | 4.08 |  |  |  | 14 |
| 12 | 30 | 22.86 |  |  | 21.12 |  |  | 30 |  | 3.51 |  |  |  | 12 |
| 13 | 45 | 23.15 |  |  | 21.41 |  |  | 45 |  | 2.95 |  |  |  | 8 |
| 14 | 60 | 23.48 |  |  | 21.74 |  |  | 60 |  | 2.04 |  |  |  | 6 |
| 15 | 90 | 23.71 |  |  | 21.97 |  |  | 90 |  | 1.37 |  |  |  | 4 |
| 16 | 120 | 23.98 |  |  | 22.24 |  |  | 120 |  | 0.88 |  |  |  | 3 |
| 17 | 180 | 24.08 |  |  | 22.34 |  |  | 180 |  | 0.31 |  |  |  | 2 |
| 18 | 240 | 24.12 |  |  | 22.38 |  |  | 240 |  | 0.11 |  |  |  | 2 |
| 19 | 300 | 24.13 |  |  | 22.39 |  |  | 300 |  | 0.03 |  |  |  | 1 |
| 20 | 360 | 24.14 |  |  | 22.4 |  |  | 360 |  | 0.02 |  |  |  | 1 |

The SC was calculated to be 22.23 m2/day. Table (3) shows the hydraulic properties of this well and Fig. (5) shows the procedure for analyzing hydraulic properties.

Table 6. Groundwater observation water level data for well 4

| Well No. 4 |  |  |  | Depth: 110 m |  |  |  | Discharge: 864 m3/day |  |  |  | Diameter: 30 cm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long. 44°15'14.725" |  |  | Lat. 31°47'19.995" |  |  |  | Test Date: 16/4/2021 |  |  |  | 8.00 A.M |  | Alti.: 38 m.a.s.l |  |
| Static Water-level: 5.03 m |  |  |  |  |  | Dynamic Water-level: 35.25 m |  |  |  | S.C: 24.51 m2/day |  |  |  |  |
| No. | t. min | D.D (M) |  |  | "S (m) |  | "t. min |  | Residual D.D "S (m) |  |  |  |  | "t/t min |
| 1 | 0 | 5.03 |  |  | ....... |  | 0 |  | 30.22 |  |  |  |  | ……. |
| 2 | 1 | 8.25 |  |  | 3.22 |  | 1 |  | 26.31 |  |  |  |  | 361.0 |
| 3 | 2 | 11.56 |  |  | 6.53 |  | 2 |  | 23.11 |  |  |  |  | 180.5 |
| 4 | 3 | 14.06 |  |  | 9.03 |  | 3 |  | 20.65 |  |  |  |  | 120.3 |
| 5 | 4 | 16.35 |  |  | 11.32 |  | 4 |  | 17.83 |  |  |  |  | 90.3 |
| 6 | 5 | 18.16 |  |  | 13.13 |  | 5 |  | 14.17 |  |  |  |  | 72.2 |
| 7 | 7 | 22.61 |  |  | 17.58 |  | 7 |  | 11.79 |  |  |  |  | 51.6 |
| 8 | 10 | 26.84 |  |  | 21.81 |  | 10 |  | 8.08 |  |  |  |  | 36.1 |
| 9 | 15 | 29.92 |  |  | 24.89 |  | 15 |  | 6.42 |  |  |  |  | 24.1 |
| 10 | 20 | 31.09 |  |  | 26.06 |  | 20 |  | 5.23 |  |  |  |  | 18.1 |
| 11 | 25 | 32.27 |  |  | 27.24 |  | 25 |  | 4.9 |  |  |  |  | 14.4 |
| 12 | 30 | 33.47 |  |  | 28.44 |  | 30 |  | 4.09 |  |  |  |  | 12.0 |
| 13 | 45 | 34.15 |  |  | 29.12 |  | 45 |  | 3.51 |  |  |  |  | 8.0 |
| 14 | 60 | 35.02 |  |  | 29.99 |  | 60 |  | 2.97 |  |  |  |  | 6.0 |
| 15 | 90 | 35.17 |  |  | 30.14 |  | 90 |  | 1.85 |  |  |  |  | 4.0 |
| 16 | 120 | 35.21 |  |  | 30.18 |  | 120 |  | 1.16 |  |  |  |  | 3.0 |
| 17 | 180 | 35.23 |  |  | 30.2 |  | 180 |  | 0.67 |  |  |  |  | 2.0 |
| 18 | 240 | 35.24 |  |  | 30.21 |  | 240 |  | 0.17 |  |  |  |  | 1.5 |
| 19 | 300 | 35.25 |  |  | 30.22 |  | 300 |  | 0.04 |  |  |  |  | 1.2 |
| 20 | 360 | 35.25 |  |  | 30.22 |  | 360 |  | 0.03 |  |  |  |  | 1.0 |

The SC was calculated to be 24.51m2/day. Table (3) shows the hydraulic properties of this well and Fig. (6) shows the procedure for analyzing hydraulic properties.

Table 7. Groundwater observation water level data for well 5

| Well No. 5 |  |  | Depth: 210 m |  |  |  |  | Discharge: 864 m3/day |  |  | Diameter: 30 cm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long. 44°12'13.641" |  |  | Lat. 31°52'13.636" |  |  | Test Date: 15/4/2021 |  |  |  | 9.00: A.M |  | Alti.: 35 m.a.s.l |  |
| Static Water-level: 0.27 m |  |  |  |  | Dynamic Water-level: 6.05 m |  |  |  |  | S.C: 142.8 m2/day |  |  |  |
| No. | t. min | D.D (M) |  | "S (m) |  |  | "t. min |  | Residual D.D "S (m) |  |  |  | "t/t min |
| 1 | 0 | 0.27 |  | ....... |  |  | 0 |  | 5.78 |  |  |  | ……… |
| 2 | 1 | 1.96 |  | 1.69 |  |  | 1 |  | 3.96 |  |  |  | 241 |
| 3 | 2 | 2.84 |  | 2.57 |  |  | 2 |  | 2.84 |  |  |  | 121 |
| 4 | 3 | 3.49 |  | 3.22 |  |  | 3 |  | 1.75 |  |  |  | 80 |
| 5 | 4 | 4.07 |  | 3.8 |  |  | 4 |  | 1.37 |  |  |  | 60 |
| 6 | 5 | 4.68 |  | 4.41 |  |  | 5 |  | 1.09 |  |  |  | 48 |
| 7 | 7 | 5.11 |  | 4.84 |  |  | 7 |  | 0.85 |  |  |  | 34 |
| 8 | 10 | 5.42 |  | 5.15 |  |  | 10 |  | 0.71 |  |  |  | 24 |
| 9 | 15 | 5.53 |  | 5.26 |  |  | 15 |  | 0.63 |  |  |  | 16 |
| 10 | 20 | 5.67 |  | 5.4 |  |  | 20 |  | 0.55 |  |  |  | 12 |
| 11 | 25 | 5.72 |  | 5.45 |  |  | 25 |  | 0.47 |  |  |  | 10 |
| 12 | 30 | 5.76 |  | 5.49 |  |  | 30 |  | 0.39 |  |  |  | 8 |
| 13 | 45 | 5.78 |  | 5.51 |  |  | 45 |  | 0.28 |  |  |  | 5 |
| 14 | 60 | 5.98 |  | 5.71 |  |  | 60 |  | 0.17 |  |  |  | 4 |
| 15 | 90 | 6.01 |  | 5.74 |  |  | 90 |  | 0.09 |  |  |  | 3 |
| 16 | 120 | 6.03 |  | 5.76 |  |  | 120 |  | 0.04 |  |  |  | 2 |
| 17 | 180 | 6.05 |  | 5.78 |  |  | 180 |  | 0.02 |  |  |  | 1 |
| 18 | 240 | 6.05 |  | 5.78 |  |  | 240 |  | 0 |  |  |  | 1 |

The SC was calculated to be 142.8 m2/day. Table (3) shows the hydraulic properties of this well and Fig. (7) shows the procedure for analyzing hydraulic properties.

Table 8. Groundwater observation water-level data for well 6

| Well No. 6 |  |  |  | Depth: 112 m |  |  |  |  | Discharge: 604.8 m3/day |  |  |  | Diameter: 30 cm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long. 44°04'19.035" |  |  | Lat. 31°58'27.541" |  |  |  | Test Date: 11/4/2021 |  |  |  |  | 7.00 A.M |  | Alti.: 34 m.a.s.l |  |
| Static Water-level: 4.86 m |  |  |  |  |  | Dynamic Water-level: 6.67 m |  |  |  |  | S.C: 90.67 m2/day |  |  |  |  |
| No. | t. min | D.D (M) |  |  | "S (m) |  |  | "t. min |  | Residual D.D "S (m) |  |  |  |  | "t/t min |
| 1 | 0 | 4.86 |  |  | …. |  |  | 0 |  | 1.81 |  |  |  |  | ……. |
| 2 | 1 | 6.1 |  |  | 1.24 |  |  | 1 |  | 1.28 |  |  |  |  | 361 |
| 3 | 2 | 6.13 |  |  | 1.27 |  |  | 2 |  | 1.18 |  |  |  |  | 180.50 |
| 4 | 3 | 6.18 |  |  | 1.32 |  |  | 3 |  | 1.14 |  |  |  |  | 120.33 |
| 5 | 4 | 6.21 |  |  | 1.35 |  |  | 4 |  | 1.11 |  |  |  |  | 90.25 |
| 6 | 5 | 6.23 |  |  | 1.37 |  |  | 5 |  | 1.08 |  |  |  |  | 72.20 |
| 7 | 7 | 6.24 |  |  | 1.38 |  |  | 7 |  | 1.04 |  |  |  |  | 51.57 |
| 8 | 10 | 6.25 |  |  | 1.39 |  |  | 10 |  | 0.99 |  |  |  |  | 36.10 |
| 9 | 15 | 6.29 |  |  | 1.43 |  |  | 15 |  | 0.8 |  |  |  |  | 24.07 |
| 10 | 20 | 6.31 |  |  | 1.45 |  |  | 20 |  | 0.72 |  |  |  |  | 18.05 |
| 11 | 25 | 6.33 |  |  | 1.47 |  |  | 25 |  | 0.65 |  |  |  |  | 14.44 |
| 12 | 30 | 6.35 |  |  | 1.49 |  |  | 30 |  | 0.59 |  |  |  |  | 12.03 |
| 13 | 45 | 6.37 |  |  | 1.51 |  |  | 45 |  | 0.52 |  |  |  |  | 8.02 |
| 14 | 60 | 6.42 |  |  | 1.56 |  |  | 60 |  | 0.46 |  |  |  |  | 6.02 |
| 15 | 90 | 6.49 |  |  | 1.63 |  |  | 90 |  | 0.39 |  |  |  |  | 4.01 |
| 16 | 120 | 6.55 |  |  | 1.69 |  |  | 120 |  | 0.26 |  |  |  |  | 3.01 |
| 17 | 180 | 6.62 |  |  | 1.76 |  |  | 180 |  | 0.16 |  |  |  |  | 2.01 |
| 18 | 240 | 6.65 |  |  | 1.79 |  |  | 240 |  | 0.07 |  |  |  |  | 1.50 |
| 19 | 300 | 6.67 |  |  | 1.81 |  |  | 300 |  | 0.02 |  |  |  |  | 1.20 |
| 20 | 360 | 6.67 |  |  | 1.81 |  |  | 360 |  | 0.01 |  |  |  |  | 1.00 |

The SC was calculated to be 90.67 m2/day. Table (3) shows the hydraulic properties of this well and Fig. (8) shows the procedure for analyzing hydraulic properties.

Table 9. Groundwater observation water-level data for well 7

| Well No. 7 |  |  |  | Depth: 106 m |  |  |  |  | Discharge: 604.8 m3/day |  |  |  | Diameter: 30 cm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long. 44°06'01.896" |  |  | Lat. 31°57'33.92" |  |  |  | Test Date: 11/4/2021 |  |  |  | 1:00: P.M |  |  | Alti. 26 m.a.s.l |  |
| Static Water-level: 5.86 m |  |  |  |  |  | Dynamic Water-level: 12.39 m |  |  |  |  |  | S.C: 48.81m2/day |  |  |  |
| No. | t. min | D.D (M) |  |  | "S (m) |  |  | "t. min |  | Residual D.D "S (m) |  |  |  |  | "t/t min |
| 1 | 0 | 5.86 |  |  | ........ |  |  | 0 |  | 6.53 |  |  |  |  | …….. |
| 2 | 1 | 11.29 |  |  | 5.43 |  |  | 1 |  | 4.13 |  |  |  |  | 361.0 |
| 3 | 2 | 11.87 |  |  | 6.01 |  |  | 2 |  | 3.18 |  |  |  |  | 180.5 |
| 4 | 3 | 11.88 |  |  | 6.02 |  |  | 3 |  | 2.25 |  |  |  |  | 120.3 |
| 5 | 4 | 11.89 |  |  | 6.03 |  |  | 4 |  | 2.03 |  |  |  |  | 90.3 |
| 6 | 5 | 11.94 |  |  | 6.08 |  |  | 5 |  | 1.87 |  |  |  |  | 72.2 |
| 7 | 7 | 11.99 |  |  | 6.13 |  |  | 7 |  | 1.51 |  |  |  |  | 51.6 |
| 8 | 10 | 12.06 |  |  | 6.2 |  |  | 10 |  | 1.15 |  |  |  |  | 36.1 |
| 9 | 15 | 12.11 |  |  | 6.25 |  |  | 15 |  | 1.07 |  |  |  |  | 24.1 |
| 10 | 20 | 12.17 |  |  | 6.31 |  |  | 20 |  | 0.83 |  |  |  |  | 18.1 |
| 11 | 25 | 12.22 |  |  | 6.36 |  |  | 25 |  | 0.62 |  |  |  |  | 14.4 |
| 12 | 30 | 12.26 |  |  | 6.4 |  |  | 30 |  | 0.51 |  |  |  |  | 12.0 |
| 13 | 45 | 12.3 |  |  | 6.44 |  |  | 45 |  | 0.43 |  |  |  |  | 8.0 |
| 14 | 60 | 12.33 |  |  | 6.47 |  |  | 60 |  | 0.31 |  |  |  |  | 6.0 |
| 15 | 90 | 12.35 |  |  | 6.49 |  |  | 90 |  | 0.26 |  |  |  |  | 4.0 |
| 16 | 120 | 12.38 |  |  | 6.52 |  |  | 120 |  | 0.17 |  |  |  |  | 3.0 |
| 17 | 180 | 12.39 |  |  | 6.53 |  |  | 180 |  | 0.07 |  |  |  |  | 2.0 |
| 18 | 240 | 12.4 |  |  | 6.54 |  |  | 240 |  | 0.03 |  |  |  |  | 1.5 |
| 19 | 300 | 12.39 |  |  | 6.53 |  |  | 300 |  | 0.02 |  |  |  |  | 1.2 |
| 20 | 360 | 12.39 |  |  | 6.53 |  |  | 360 |  | 0.01 |  |  |  |  | 1.0 |

The SC was calculated to be 48.81 m2/day. Table (3) shows the hydraulic properties of this well and Fig. (9) shows the procedure for analyzing hydraulic properties.