Supporting Information:

Phthalate intake in Chinese homes and its contribution to total intake among children

# Yeganeh Ataei1, Yuexia Sun1*, Changqi Zhu1, Yuxuan Zhao1

1Tianjin Key Laboratory of Indoor Air Environmental Quality Control, School of Environmental Science and Engineering, Tianjin University, Tianjin 300350, China

* yuexiasun@tju.edu.cn

Contents:

- Supplementary methods “Calculation of phthalates concentration in both gas and particle phase in child’s bedroom”

- Supplementary Tables

Table S1Parameters of target phthalates and corresponding metabolites.

Table S2 Exposure parameters for children

Table S3 Descriptive statistics on mass-fractions of phthalates in homes dust (μg/g) and metabolite concentration of urinary phthalates (µg/L) in children investigated in Tianjin, China

Table S4 Comparison of median daily total phthalate intake (µg/d/kg-bw) with prior studies

Table S5 Comparison of median mass-fractions of phthalates in dust (μg/g) and the corresponding indoor intake (µg/d/kg-bw) with prior studies

- Supplementary methods “Calculation of phthalates concentration in both gas and particle phase in child’s bedroom”

Equations used to estimate the concentration of both gas and particle phases of indoor environment phthalates are shown as follows.

Linear relationship of Diethyl phthalate (DEP) in gas and dust phase concentrations was provide by Beko et al.(2013) 1:

CgDEP=(CdDEP+0.2318)/0.0092                                                     (1)

Where Cg is phthalate gas-phase (in air) concentration (ng/m3) and Cd denotes concentration of phthalates in dust (µg/g). DEP exhibits the highest volatility and is predominantly present in gas phase. As a result, DEP concentration in small particles in the air may be considered insignificant.

We used the equilibrium theory and models to calculate the phthalate’s concentration of gas phase (Equation 2) 2,3and particle phase (Equation 3), for diisobutyl phthalate (DiBP), di-2-ethylhexyl phthalate (DEHP) and di-n-butyl phthalate (DnBP):

Kd=CdCg                                                                                                           (2)

Kp=FTSPCg=CpCg                                                                                                (3)

Where Kd  is the dust-gas phase distribution coefficient (m3/mg), Kp denotes particle-gas phase distribution coefficient (m3/µg), F represents the indoor phthalate concentrations in particulate phase (ng/m3). The total suspended particles concentration in the air is denoted by  TSP (50 µg/m3)4, Cp=F/TSP represents the concentration of phthalate in unit mass of particle (ng/µg).The particle-gas (Kp) and dust-gas (Kd) partition coefficient are the two important parameters that govern the distribution of indoor environment phthalates in the gas phase, suspended particulate matter in air, and settled dust. In this study, we used two empirical equations 3 to calculate Kp and Kd  values as follows:

log10Kp=log10Koa+ log10fom-pρp                                                              (4)

log10Kd=log10Koa+ log10fom-dρd                                                              (5)

The coefficient of octanol air partition is donoted by Koa, fom-p denotes the fraction of the volume of organic matter in suspended particulate matter in air, assumed 0.4 and ρp is density of suspended particulate matter in air, taken to be 1 ×1012 µg/m3. fom-d denotes the settled dust organic matter as volume fraction, assumed to be 0.2 then ρd is dust density, assumed to be 2×109 mg/m3.

The coefficient of octanol-air partition, Koa, can be explained as the ratio of the concentration of a pollutant in the octanol phase to that in the air phase at equilibrium distribution.  Koa exhibits a strong dependence on temperature, with higher temperature resulting in lower Koa values and an increased likelihood of contaminants in the gas phase 5. The indoor temperature at the time of sampling might be different from 298 k (25°C), at which Koa was calculated 3. Therefore, the Kp and Kd values should be adjusted based on the actual air temperature during sampling. The correction procedure 6 is based on the Van’t Hoff equation using the negative value of the heat of vaporization (∆Hv) as a proxy for the enthalpy change (∆Hθ) at standard state (Equation 6). ∆Hv, the heat of vaporization of phthalates, can be calculated using the Thiesen equation (Equation 7)7  with n set as 0.41. The heat of vaporization at the boiling point (∆Hvb) can be calculated using the Riedel equation (Equation 8)7:

ln(K2K1)=-∆HθR(1T2-1T1)                                                                                     (6)

∆Hv=∆Hvb1-TTc1-TbTcn                                                                                          (7)

∆Hvb=Tb5logPc -2.170.930-TbTc                                                                                       (8)

Where, T2 represents the actual indoor temperature obtained from on-site measurement in each home, K; T1 represents the ideal temperature, which is T1=298k; K2 represents the distribution coefficient (Kp/Kd) at the actual indoor temperature T2; K1 represents the distribution coefficient at T1=298 k (25°c); ΔHθ represents the enthalpy change at standard state, in KJ/mol; ΔHv represents heat of vaporization at T=298 (25°c) cal/mol; ΔHvb represents the vaporization heat at boiling point, in cal/mol; R denotes the constant of gas, which is 8.314 J/(mol.K); Tb denotes the boiling point of phthalate in K; Tc denotes the critical phthalate temperature in K; Pc denotes the critical phthalate pressure in atm. The following table provides the variable values used in the equations in this section.

Parameters for calculating phthalate partition coefficient at 25 °C.

| Phthalate | Tb (K) | θ= Tb / Tc | Tc (K) | Pc (atm) | ΔHvb (kJ/mol) | ΔHv (kJ/mol) | Log Koa | log Kp (m3/µg) | log Kd (m3/mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DnBP | 611 | 0.78 | 778 | 17.20 | 70.66 | 108.87 | 8.63 | -3.77 | -1.37 |
| DiBP | 596 | 0.77 | 765 | 17.50 | 66.80 | 101.33 | 8.41 | -3.99 | -1.59 |
| DEHP | 690 | 0.81 | 846 | 11.60 | 79.83 | 133.73 | 12.56 | 0.16 | 2.56 |

Note 1: The boiling point (Tb) and log Koa  were calculated by the Estimation Program of Interface (EPI Suite TM) which was developed by the U.S. Environmental Protection Agency’s Office of Pollution Prevention and toxics and Syracuse Research Corporation (SRC);

Note 2: The boiling ratio point to critical temperature (θ= Tb / Tc) and critical pressure (Pc) were calculated by the Lydersen method;

Note 3: The units of ΔHvb and ΔHv were calculated by equations 7 and 8 in cal/mol, which are then converted to kJ/mol in the table

- Supplementary Tables

Table S1 Parameters of target phthalates and corresponding metabolites

| Diester | Formulaa | MWp (g/mol)b | kp_g (m/day)c | fd d (unitless) | Metabolite | Fue (unitless)e | MWm (g/mol)b |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DEP | C12H14O4 | 222 | 81.6 | 0.002051 | MEP | 0.69 | 194 |
| DnBP | C16H22O4 | 278 | 115.2 | 0.001556 | MnBP | 0.69 | 222 |
| DiBP | C16H22O4 | 278 | 110.4 | 0.001202 | MiBP | 0.71f | 222 |
| DEHP | C24H38O4 | 391 | 139.2 | 0.000106 | MEHP | 0.059 | 278g |
|  |  |  |  |  | MEHHP | 0.233 | 294 |
|  |  |  |  |  | MEOHP | 0.15 | 292 |
|  |  |  |  |  | MECPP | 0.185 | 308 |

a: Cao X L 8.

b: Data was taken from Beko et al.(2013) 1

C:  Weschler, and Nazaroff (2014) 9

d: Wormuth et al. (2006) 10

e:  Wittassek et al. (2011) 11

f: Koch et al. (2012) 12

g: Woff et al. (2010) 13

Table S2 Exposure parameters for children

|  | < 1 year | 1-<2 years | 2- <3 years | 3- <6 year | 6 -<8 years |
| --- | --- | --- | --- | --- | --- |
| Inhalation rate, Vinhalation (m3/day)a | 6.03 | 8.01 | 9.48 | 10.85 | 12.39 |
| Dust ingestion rate, Mingestion (dust) (g/day)a | 0.03 | 0.06 | 0.06 | 0.06 | 0.06 |
|  | <3 years |  | 3 -< 8 years |  |  |
| Exposure frequency, Fe (h/day) | 20 |  | 18/15 |  |  |
| Exposure days, ED (day) | 365 |  | 158/207 |  |  |
| Exposure factor, EF | 0.833 |  | 0.679 |  |  |

a: Child-Specific Exposure Factors Handbook 14

Table S3 Descriptive statistics on mass-fractions of phthalates in homes dust (μg/g) and metabolite concentration of urinary phthalates (µg/L) in children investigated in Tianjin, China

| Mass-fractions of phthalates in dust (N=398 (351 valid and 47 missing)) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean (SD) | Minimum | 25th percentile | Median | 75th percentile | 95th percentile | Maximum |
| DEP | 0.56 (0.83) | 0.01 | 0.17 | 0.30 | 0.62 | 2.02 | 10.01 |
| DiBP | 43.10 (104.34) | 0.12 | 6.52 | 16.27 | 35.79 | 174.77 | 1202.86 |
| DnBP | 230.10 (730.14) | 0.09 | 14.10 | 41.47 | 131.13 | 878.25 | 7184.74 |
| BBzP | 0.90 (9.86) | 0.01 | 0.03 | 0.09 | 0.25 | 1.18 | 182.90 |
| DEHP | 4 57.38 (1172.03) | 0.90 | 38.96 | 126.94 | 351.29 | 2016.17 | 10927.76 |
| DiNP | 0.70 (1.46) | 0.01 | 0.17 | 0.28 | 0.70 | 2.2860 | 18.40 |
| Urinary phthalate metabolites concentrations (N=398 (373 valid and 25 missing)) |  |  |  |  |  |  |  |
|  | Mean (SD) | Minimum | 25th percentile | Median | 75th percentile | 95th percentile | Maximum |
| MEP | 34.54(68.61) | 0.61 | 5.99 | 18.36 | 49.45 | 91.53 | 1095.78 |
| MiBP | 27.84 (13.05) | 0.05 | 24.18 | 31.69 | 33.55 | 39.48 | 96.85 |
| MnBP | 30.18(35.93) | 0.00 | 4.35 | 25.83 | 43.48 | 78.21 | 512.85 |
| MBzP | 0.12(0.14) | 0.09 | 0.09 | 0.09 | 0.09 | 0.30 | 1.65 |
| MEHP | 7.55(3.64) | 0.15 | 5.23 | 7.50 | 9.77 | 13.48 | 31.54 |
| MECPP | 37.41(23.35) | 0.05 | 14.60 | 43.92 | 55.68 | 64.46 | 164.97 |
| MEHHP | 13.70(9.52) | 0.04 | 4.52 | 16.57 | 18.70 | 26.69 | 68.82 |
| MEOHP | 8.71(6.03) | 0.06 | 3.19 | 9.51 | 13.47 | 16.42 | 41.07 |
| MiNP | 0.03(0.04) | 0.00 | 0.03 | 0.03 | 0.03 | 0.12 | 0.38 |

Table S4 Comparison of median daily total phthalate intake (µg/d/kg-bw) with prior studies

| References | Country | Subjects age (years) | Sample size (n) | DEP | DiBP | DnBP | DEHP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Beko et al. (2013) 1 | Denmark | 3-6 | 441 | 0.62 | 2.93 | 3.26 | 4.42 |
| Hartmann et al. (2015) 15 | Australia | 6-15 | 250 | 0.69 | 1.40 | 0.44 | 1.60 |
| Dewalque et al. (2014) 16 | Belgian | 1-12 | 52 | 1.47 | 2.29 | 2.38 | 3.37 |
| Ait Bamai et al. (2015) 17 | Japan | 3-12 (girls) | 92 | - | 0.56 | 0.04 | 4.23 |
|  |  | 3-12 (boys) | 115 | - | 0.70 | 0.05 | 4.86 |
| Larsson et al. (2017) 18 | Sweden | 4 | 113 | - | - | 2.3a | 2.4a |
| Wang et al. (2015) 19 | China | 8-11 | 782 | 0.70 | 1.50 | 1.90 | 3.70 |
| This study | China | 0-8 | 398 | 0.68 | 1.25 | 1.05 | 3.26 |

a: Geometric mean

Table S5 Comparison of median mass-fractions of phthalates in dust (μg/g) and the corresponding indoor intake (µg/d/kg-bw) with prior studies

| References | Country | Subjects age (years) | Sample size | phthalate mass-fractions in dust (μg/g) |  |  |  |  | Intakes from indoor environment (Calculated via ingestion, inhalation and dermal absorption) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DEP | DiBP | DnBP | DEHP | DEP |  | DiBP | DnBP | DEHP |
| Beko et al. (2013) 1 | Denmark | 3-6 | 479 homes | 1.7 | 27 | 15 | 210 | 0.53 |  | 1.52 | 0.60 | 0.37 |
|  |  |  | 151 kindergartens | 2.2 | 23 | 38 | 500 |  |  |  |  |  |
| Fromme et al. (2004)20 | Germany | 0-4 | 59 homes | 6.10 | - | 47.00 | 703.40 | 0.61a |  | - | 0.82a | 6.03a |
|  |  |  | 74 kindergartens | - | - | - | - |  |  |  |  |  |
| Wang et al. (2014)21 | China | 0-10 | 14 homes | - | 233.80 | 134.77 | 581.51 | - |  | 2.20 | 1.29 | 2.23 |
| This study | China | 0_8 | 390 homes | 0.3 | 16.27 | 41.47 | 126.94 | 0.04 |  | 0.39 | 0.57 | 0.14 |

a: Sum exposure of airborne particles and dust phases

References:

1	Bekö, G. et al. Children's phthalate intakes and resultant cumulative exposures estimated from urine compared with estimates from dust ingestion, inhalation and dermal absorption in their homes and daycare centers. PLoS. One. 8, e62442; 10.1371/journal.pone.0062442 (2013).

2	Weschler, C. & Nazaroff, W. Semivolatile organic compounds in indoor environments. Atmos. Environ. 42, 9018-9040; 10.1016/j.atmosenv.2008.09.052 (2008).

3	Weschler, C. J. & Nazaroff, W. W. SVOC partitioning between the gas phase and settled dust indoors. Atmos. Environ. 44, 3609-3620; 10.1016/j.atmosenv.2010.06.029 (2010).

4	Salthammer, T. & Schripp, T. Application of the Junge- and Pankow-equation for estimating indoor gas/particle distribution and exposure to SVOCs. Atmos. Environ. 106, 467-476; 10.1016/j.atmosenv.2014.09.050 (2015).

5	Zhang, L. et al. Process on the Study of the octanol-air partition coefficient. Res. Environ. Sci. 17, 77-80 (In Chinese) (2004).

6	Huang, L. et al. Phthalates in house dust in Chinese urban residences: concentrations, partition, origin and determinants. Chemospher. 286, 131703; 10.1016/j.chemosphere.2021.131703 (2022).

7	Lyman, W. J., Reehl, W. F. & Rosenblatt, D. H. Handbook of chemical property estimation methods - Environmental behavior of organic compounds.  (American Chemical Society: Washington, DC, 1990).

8	Cao, X. L. Phthalate esters in foods: sources, occurrence, and analytical methods. Compr. Rev. Food Sci. Food Saf. 9, 21-43; 10.1111/j.1541-4337.2009.00093.x (2010).

9	Weschler, C. J. & Nazaroff, W. W. Dermal Uptake of Organic Vapors Commonly Found in Indoor Air. Environ. Sci. Technol. 48, 1230-1237; 10.1021/es405490a (2014).

10	Wormuth, M., Scheringer, M., Vollenweider, M. & Hungerbühler, K. What are the sources of exposure to eight frequently used phthalic acid esters in Europeans? Risk Anal. 26, 803-824; 10.1111/j.1539-6924.2006.00770.x (2006).

11	Wittassek, M., Koch, H. M., Angerer, J. & Brüning, T. Assessing exposure to phthalates - The human biomonitoring approach. Mol. Nutr. Food. Res. 55, 7-31; 10.1002/mnfr.201000121 (2011).

12	Koch, H. M., Christensen, K. L., Harth, V., Lorber, M. & Brüning, T. Di-n-butyl phthalate (DnBP) and diisobutyl phthalate (DiBP) metabolism in a human volunteer after single oral doses. Arch. Toxicol. 86, 1829-1839; 10.1007/s00204-012-0908-1 (2012).

13	Wolff, M. S. et al. Investigation of relationships between urinary biomarkers of phytoestrogens, phthalates, and phenols and pubertal stages in girls. Environ. Health Perspect. 118, 1039-1046; 10.1289/ehp.0901690 (2010).

14	US environmental protection agency (EPA). Child-specific exposure factors handbook (Final Report).  (Environmental protection agency, Washington, DC, EPA/600/R-06/096F, 2008).

15	Hartmann, C. et al. Human biomonitoring of phthalate exposure in Austrian children and adults and cumulative risk assessment. Int. J. Hyg. Environ. Health. 218, 489-499; 10.1016/j.ijheh.2015.04.002 (2015).

16	Dewalque, L., Charlier, C. & Pirard, C. Estimated daily intake and cumulative risk assessment of phthalate diesters in a Belgian general population. Toxicol. Lett. 231, 161-168; 10.1016/j.toxlet.2014.06.028 (2014).

17	Ait Bamai, Y. et al. Comparisons of urinary phthalate metabolites and daily phthalate intakes among Japanese families. Int. J. Hyg. Environ. Health. 218, 461-470; 10.1016/j.ijheh.2015.03.013 (2015).

18	Larsson, K. et al. Phthalates, non-phthalate plasticizers and bisphenols in Swedish preschool dust in relation to children's exposure. Environ. Int. 102, 114-124; 10.1016/j.envint.2017.02.006 (2017).

19	Wang, B. et al. Urinary excretion of phthalate metabolites in school children of China: implication for cumulative risk assessment of phthalate exposure. Environ. Sci. Technol. 49, 1120-1129; 10.1021/es504455a (2015).

20	Fromme, H. et al. Occurrence of phthalates and musk fragrances in indoor air and dust from apartments and kindergartens in Berlin (Germany). Indoor Air. 14, 188-195; 10.1111/j.1600-0668.2004.00223.x (2004).

21	Wang, X., Tao, W., Xu, Y., Feng, J. & Wang, F. Indoor phthalate concentration and exposure in residential and office buildings in Xi'an, China. Atmos. Environ. 87, 146-152; 10.1016/j.atmosenv.2014.01.018 (2014).