Free water surface constructed wetlands: Review of pollutant removal performance and modeling approaches

Mohamed S. Gaballahab1*, Hooshyar Yousefyania, Mohammadjavad Karamia, Roderick W. Lammersa1

aSchool of Engineering and Technology & Institute for Great Lakes Research, Central Michigan University, Mt. Pleasant, MI 48859, USA

b National Institute of Oceanography and Fisheries, Cairo, Egypt

* Corresponding author email: awad1m@cmich.edu/saadga22@gmail.com (Mohamed S. Gaballah)

1Mohamed S. Gaballah and Roderick W. Lammers contributed equally to this work.

Table SI 1. FWSCWs performances for removing nutrients from different wastewater sources

| System Scale | Area (m²) | Capacity (m³/day) | HRT (days) | Depth (m) | Wastewater type | Vegetation | Temp ( ºC) | Pollutants removal (%) |  |  | Country |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Parameter | Initial Concentration (mg/L) | Removal% |  |  |  |
| Full-scale | 125000 | 21500 | 8 | 0.1-0.6 | Agricultural and municipal wastewater | Phragmites australis, Typha latifolia | 17-34.5 | BOD | 28-100 | 52 | Egypt |  | (El-Sheikh et al., 2010) |
|  |  |  |  |  |  |  |  | COD | 72-122 | 50 |  |  |  |
|  |  |  |  |  |  |  |  | TSS | 50-130 | 87 |  |  |  |
|  |  |  |  |  |  |  |  | NH4-N | 1.8-4.5 | 66 |  |  |  |
|  |  |  |  |  |  |  |  | PO4 | 0.8-2.3 | 52 |  |  |  |
| Pilot-scale | 9.99 | 0.1-0.35 | 14-18 | 0.4 | Graywater | Typha latifolia | 5.2-18.9 | BOD5 | 3.1-45.6 | 78-92 | USA |  | (Jokerst et al., 2011) |
|  |  |  |  |  |  |  |  | TN | 0.04-0.664 | 64-85 |  |  |  |
|  |  |  |  |  |  |  |  | TSS | 4.0-15.8 | 73 |  |  |  |
|  |  |  |  |  |  |  |  | TP | 0.8-2.1 | 65-78 |  |  |  |
| Full-scale | 43200 | 49.5-122.4 m3/h | 0.9 | 1.0 | steel industry wastewater | P. australis Cav., T. angustifolia L, C. tegetiformis |  | BOD5 | 7.5 | 30 | Vietnam |  | (Nguyen et al., 2019) |
|  |  |  |  |  |  |  |  | COD | 23.1 | 30 |  |  |  |
|  |  |  |  |  |  |  |  | NH4-N | 0.72 | >90 |  |  |  |
|  |  |  |  |  |  |  |  | TN | 10.5 | 14 |  |  |  |
| Full-scale | 2000 | 100 | 7-12 | 0.4-0.5 | Wastewater | Typha domingensis | / | BOD | 31.3 | 73 | Argentina |  | (Hadad et al., 2018) |
|  |  |  |  |  |  |  |  | COD | 85 | 75 |  |  |  |
|  |  |  |  |  |  |  |  | TP | 0.396 | 22 |  |  |  |
|  |  |  |  |  |  |  |  | NH4+ | 0.88 | 12 |  |  |  |
|  |  |  |  |  |  |  |  | NO2- | 2.22 | 84 |  |  |  |
|  |  |  |  |  |  |  |  | NO3- | 50.6 | 80 |  |  |  |
| Pilot-scale | 2000 | 100 | 7 | 0.5-0.8 | Industrial wastewater and sewage | T. domingensis | 20.5 | NO2- | 2.61 | 98.4 | Argentina |  | (Maine et al., 2013) |
|  |  |  |  |  |  |  |  | NO3- | 37.3 | 81.0 |  |  |  |
|  |  |  |  |  |  |  |  | TP | 0.43 | 23.4 |  |  |  |
|  |  |  |  |  |  |  |  | NH4+ | 3.04 | 12.8 |  |  |  |
|  |  |  |  |  |  |  |  | SO42- | 1711.3 | 65.2 |  |  |  |
| Full-scale | 48000 | 0.17m/d | 7.1 | 0.5 | eutrophic waters | Typha angustifolia |  | NH4+ | 0.21 | 12.5 | Spain |  | (Hernández-Crespo et al., 2017) |
|  |  |  |  |  |  |  |  | TN | 2.1 | 52.5 |  |  |  |
|  |  |  |  |  |  |  |  | TP | 0.088 | 48.2 |  |  |  |
| Full-scale | 3674 | 182m/d | 3 | 0.9 | urban watershed | Mixed plants | 17 | TN | 4.72 | 46.9 | China |  | (Han et al., 2017) |
|  |  |  |  |  |  |  |  | TP | 0.05 | 52.3 |  |  |  |
| Pilot-scale | 0.4 | 0.1 | 7 | 0.15-0.25-0.35 | Polluted lake water | Eichhornia Crassipes | 32.0 | BOD5 | 95 | 75 | Egypt |  | (Gaballah et al., 2021) |
|  |  |  |  |  |  |  |  | TN | 4.7 | 82 |  |  |  |
|  |  |  |  |  |  |  |  | TP | 1.12 | 84.2 |  |  |  |
|  |  |  |  |  |  |  |  | NH4+ | 3 | 97.4 |  |  |  |
| Pilot-scale | 0.34 | 0.1 | 7 | 0.15-0.25-0.35 | Polluted lake water | Pistia stratiotes | 32.0 | BOD | 95 | 83.5 | Egypt |  | (Gaballah et al., 2019) |
|  |  |  |  |  |  |  |  | TN | 4.71 | 90.3 |  |  |  |
|  |  |  |  |  |  |  |  | TP | 1.12 | 87 |  |  |  |
|  |  |  |  |  |  |  |  | NH4+ | 3 | 97.53 |  |  |  |
| Pilot-scale | 0.16 | 2L | Continuous flow | 0.24 | greywater | Eichhornia crassipes | Cold season | Ammonia | 1.9 | 60 | Nigeria |  | (Chikogu Ameso et al., 2023) |
|  |  |  |  |  |  |  |  | COD | 600 | 85 |  |  |  |
|  |  |  |  |  |  |  |  | TP | 2.7 | 79.58 |  |  |  |
| Pilot-scale | 24.15 | 41mm/d | 3 | 0.6 | wastewater | Typha latifolia/ Phragmites australis/ Spargamium erectum | / | BOD5 | 181.49 | 54 | Spain |  | ( Acero-Oliete et al., 2022) |
|  |  |  |  |  |  |  |  | COD | 394.88 | 59 |  |  |  |
|  |  |  |  |  |  |  |  | TSS | 179.72 | 54 |  |  |  |
|  |  |  |  |  |  |  |  | P | 2.0 | -52 |  |  |  |
|  |  |  |  |  |  |  |  | NH4+ | 28.76 | -33 |  |  |  |
|  |  |  |  |  |  |  |  | TN | 21.26 | 9 |  |  |  |
| Full-scale | 4500 | 970 m3/h | 6.5 | 0.4 | Combined sewer overflows | Mixed plants | / | COD | 186 | 83.2 | Italy |  | (Masi et al., 2023) |
|  |  |  |  |  |  |  |  | TP | 3.2 | 64.6 |  |  |  |
|  |  |  |  |  |  |  |  | N- NH4 | 11 | 63.8 |  |  |  |
| Pilot-scale | 0.96 | 0.096 | 20 | 1.45 | domestic wastewater | Cannas generalis | 20 to 45 | BOD5 | 85.0 | 89 | Vietnam |  | (Tran et al., 2019) |
|  |  |  |  |  |  |  |  | TSS | 67.5 | 97 |  |  |  |
|  |  |  |  |  |  |  |  | NH4-N | 23.4 | 97 |  |  |  |
|  |  |  |  |  |  |  |  | PO4-P | 0.64 | 68 |  |  |  |
| Pilot-scale | 0.18 | 0.06 | 7 | 0.35 | Domestic Wastewaters | Azolla pinnata | dry and wet seasons | TP | 12.5/7.68 | 59.3/16.3 | Malaysia |  | (Akinbile et al., 2016) |
|  |  |  |  |  |  |  |  | TN | 900/504.43 | 16.7/19.6 |  |  |  |
|  |  |  |  |  |  |  |  | NH3-N | 10.56/12.56 | 29.5/25.7 |  |  |  |
|  |  |  |  |  |  |  |  | BOD5 | 30.45/16.34 | 15.5/17.1 |  |  |  |
|  |  |  |  |  |  |  |  | COD | 50.65/25.65 | 15.3/21.1 |  |  |  |
|  |  |  |  |  |  |  |  | TSS | 57.6/34.26 | 40.3/26.7 |  |  |  |
| Pilot-scale | 2 | 0.01 | 4 | 0.45 | wastewater | T. domingensis | 28.4 ±2.3 | TSS | 122 | 88.7 | Egypt |  | (Hamad, 2023) |
|  |  |  |  |  |  |  |  | NH4+ | 13.1 | 94.8 |  |  |  |
|  |  |  |  |  |  |  |  | COD | 129 | 65.8 |  |  |  |
|  |  |  |  |  |  |  |  | BOD | 34-40 | 84.6 |  |  |  |
|  |  |  |  |  |  | C. papyrus |  | TSS | 122 | 74.1 |  |  |  |
|  |  |  |  |  |  |  |  | NH4+ | 13.1 | 84.4 |  |  |  |
|  |  |  |  |  |  |  |  | COD | 129 | 79.0 |  |  |  |
|  |  |  |  |  |  |  |  | BOD | 34-40 | 70.2 |  |  |  |

Table SI 2. FWSCWs performances for removing heavy metals from different wastewater sources

| System Scale | Area (m²) | Capacity (m³/day) | HRT | Depth (m) | Wastewater type | Vegetation | Temp ( ºC) | Pollutants removal (%) |  |  | Country | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Parameter | Initial Concentration (mg/L) | Removal% |  |  |
| Pilot-scale | 0.4 | 0.1 | 7 | 0.15-0.25-0.35 | Polluted lake water | Pistia stratiotes | 32.0 | Fe | 0.0193 | 90.6 | Egypt | (Gaballah et al., 2019) |
|  |  |  |  |  |  |  |  | Pb | 0.0023 | 97.3 |  |  |
|  |  |  |  |  |  |  |  | Cu | 0.0046 | 90.4 |  |  |
|  |  |  |  |  |  |  |  | Ni | 0.0053 | 70.2 |  |  |
| Pilot-scale | 2000 | / | 2 | 0.3 | storm runoff water | Schoenoplectus californicu | / | TCu | 0.021 | 63.8 | USA | (Xu et al., 2019) |
|  |  |  |  |  |  |  |  | TZn | 0.0297 | 70.5 |  |  |
| Pilot-scale | 2000 | 100 | 7 | 0.5-0.8 | Industrial wastewater and sewage | T. domingensis | 20.5 | Fe | 8.833 | 97.2 | Argentina | (Maine et al., 2013) |
|  |  |  |  |  |  |  |  | Cr | 0.053 | 84.9 |  |  |
|  |  |  |  |  |  |  |  | Ni | 0.054 | 66.7 |  |  |
|  |  |  |  |  |  |  |  | Zn | 0.04 | 50.0 |  |  |
| Pilot-scale | 2000 | / | 2 | 0.3 | storm runoff water | Schoenoplectus californicu | / | Cu | 0.021 | 65.9 | USA | (Xu and Mills, 2018) |
|  |  |  |  |  |  |  |  | Zn | 0.0297 | 71.1 |  |  |
| Pilot-scale | 2 | 0.01 | 4 | 0.45 | wastewater | T. domingensis | 28.4 ±2.3 | Cr | 0.61 | 88.5 | Egypt | (Hamad, 2023) |
|  |  |  |  |  |  |  |  | Cu | 0.73 | 88.2 |  |  |
|  |  |  |  |  |  |  |  | Zn | 0.58 | 91.4 |  |  |
|  |  |  |  |  |  |  |  | Cd | 0.046 | 91.3 |  |  |
|  |  |  |  |  |  |  |  | Ni | 0.278 | 83.1 |  |  |
|  |  |  |  |  |  |  |  | Fe | 3.23 | 75.2 |  |  |
|  |  |  |  |  |  |  |  | Pb | 0.3 | 80.0 |  |  |
|  |  |  |  |  |  | Phragmites australis |  | Cr | 0.61 | 18.8 |  |  |
|  |  |  |  |  |  |  |  | Cu | 0.73 | 84.9 |  |  |
|  |  |  |  |  |  |  |  | Zn | 0.58 | 87.9 |  |  |
|  |  |  |  |  |  |  |  | Cd | 0.046 | 91.3 |  |  |
|  |  |  |  |  |  |  |  | Ni | 0.278 | 94.6 |  |  |
|  |  |  |  |  |  |  |  | Fe | 3.23 | 67.8 |  |  |
|  |  |  |  |  |  |  |  | Pb | 0.3 | 83.3 |  |  |
| Pilot-scale | 0.4 | 0.1 | 7 | 0.15-0.25-0.35 | Polluted lake water | Eichhornia Crassipes | 32.0 | Fe | 0.0193 | 62.5 | Egypt | (Gaballah et al., 2021) |
|  |  |  |  |  |  |  |  | Pb | 0.0023 | 88.9 |  |  |
|  |  |  |  |  |  |  |  | Cu | 0.0046 | 81.7 |  |  |
|  |  |  |  |  |  |  |  | Ni | 0.0053 | 80.4 |  |  |
| Full-scale | 2000 | 100 | 7-12 | 0.4-0.5 | Wastewater | Typha domingensis | / | Cr | 0.092 | 85 | Argentina | (Hadad et al., 2018) |
|  |  |  |  |  |  |  |  | Zn | 0.041 | 51 |  |  |
|  |  |  |  |  |  |  |  | Ni | 0.068 | 70 |  |  |
|  |  |  |  |  |  |  |  | Fe | 0.824 | 89 |  |  |
| Pilot-scale | 20, 13, 10, 3 | 0.39/0.57 m/h | 193/163 min | 0.3 | Aquaculture and upflow anaerobic sludge blanket reactors | Cyperus giganteus, Typha domingensis, Eichhornia crassipes, and Pontederia cordata | Winter/summer | Al | 0.441/0.588 | 75.7/68.3 | Brazil | (Travaini-Lima et al., 2015) |
|  |  |  |  |  |  |  |  | Cd | 0.0002/0.0002 | 33.0/85.0 |  |  |
|  |  |  |  |  |  |  |  | Cr | 0.0032/0.0275 | 58.3/57.6 |  |  |
|  |  |  |  |  |  |  |  | Cu | 0.00157/0.007 | 85.9/0.0 |  |  |
|  |  |  |  |  |  |  |  | Fe | 2.208/3.439 | 96.9/68.1 |  |  |
|  |  |  |  |  |  |  |  | Ni | 0.197/0.277 | 77.1/72.3 |  |  |
|  |  |  |  |  |  |  |  | Mn | 0.0063/0.062 | 74.5/64.2 |  |  |
|  |  |  |  |  |  |  |  | Pb | 0.0061/0.059 | 55.1/82.1 |  |  |
|  |  |  |  |  |  |  |  | Zn | 0.053/0.07 | 53.4/70.9 |  |  |
| Full-scale | 43200 | 49.5-122.4 m3/h | 0.9 | 1.0 | steel industry wastewater | Phragmites australis Cav., Typha angustifolia L, Cyperus tegetiformis |  | Mn | 0.43 | >60 | Vietnam | (Nguyen et al., 2019) |
|  |  |  |  |  |  |  |  | Fe | 0.33 | >45 |  |  |
| Pilot-scale | 0.18 | 0.06 | 7 | 0.35 | Domestic Wastewaters | Azolla pinnata | dry and wet seasons | Zn | 11.276.34 | 17.5/20.8 | Malaysia | (Akinbile et al., 2016) |
|  |  |  |  |  |  |  |  | Fe | 2.2/4.3 | 54.1/20.0 |  |  |
|  |  |  |  |  |  |  |  | Mg | 26.5/6.15 | 11.6/7.05 |  |  |

Note: TCu: means total Cu; Tzn means Total Zn

Table SI 3. FWSCWs performances for removing antibiotics from different wastewater sources

| System Scale | Area (m²) | Capacity (m³/day) | HRT | Depth (m) | Wastewater type | Vegetation | Temp ( ºC) | Pollutants removal (%) |  |  | Country | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Parameter | Initial Concentration (µg/L) | Removal% |  |  |
| Pilot-scale | 6 | 2cm/d | 14 | 0.3 | Swine Wastewater | Phragmites australis | 6 | Oxytetracycline | 30 | 87.6 | China | (Liu et al., 2019) |
|  |  |  |  |  |  |  |  | ciprofloxacin |  | 83.5 |  |  |
|  |  |  |  |  |  |  | 20 | oxytetracycline |  | 90.3 |  |  |
|  |  |  |  |  |  |  |  | ciprofloxacin |  | 86.8 |  |  |
| Pilot-scale | 2 | 0.01 | 4 | 0.45 | wastewater | T. domingensis | Summer | Roxithromycin | 100000 | 91.4 | Egypt | (Hamad, 2023) |
|  |  |  |  |  |  |  |  | Levofloxacin |  | 89.3 |  |  |
|  |  |  |  |  |  |  | Winter | Roxithromycin |  | 78.7 |  |  |
|  |  |  |  |  |  |  |  | Levofloxacin |  | 76.25 |  |  |
|  |  |  |  |  |  | C. papyrus | Summer | Roxithromycin |  | 93.8 |  |  |
|  |  |  |  |  |  |  |  | Levofloxacin |  | 91.5 |  |  |
|  |  |  |  |  |  |  | Winter | Roxithromycin |  | 83.2 |  |  |
|  |  |  |  |  |  |  |  | Levofloxacin |  | 80.3 |  |  |
| Pilot-scale | 1.04 | 50L/d | 3.3 | 0.3 | WWTP primary effluent | Typha angustifolia |  | Doxycycline | 0.18 | 65 | Spain | (Hijosa-Valsero et al., 2011) |
|  |  |  |  |  |  |  |  | Amoxicillin | 46 | 39 |  |  |
|  |  |  |  |  |  |  |  | Clarithromycin | 0.25 | 32 |  |  |
|  |  |  |  |  |  |  |  | Sulfamethoxazole | 0.26 | 65 |  |  |
|  |  |  | 2.9 |  |  | Phragmites australis |  | Doxycycline | 0.18 | 62 |  |  |
|  |  |  |  |  |  |  |  | Amoxicillin | 46 | 27 |  |  |
|  |  |  |  |  |  |  |  | Clarithromycin | 0.25 | 11 |  |  |
|  |  |  |  |  |  |  |  | Sulfamethoxazole | 0.26 | 59 |  |  |
| Pilot-scale | 30 | 6.5 | 1.5 | / | rural domestic wastewaters | Myriophyllum verticillatum |  | Ofloxacin | 0.193 | 73.3 | China | (Chen et al., 2015) |
|  |  |  |  |  |  |  |  | Salinomycin | 0.00404 | 11.1 |  |  |
|  |  |  |  |  |  |  |  | Sulfacetamide | 0.00304 | 9.9 |  |  |
|  |  |  |  |  |  |  |  | Sulfamethazine | 0.00240 | 16.9 |  |  |
|  |  |  |  |  |  |  |  | Sulfamonomethoxine | 0.0508 | 36.2 |  |  |
|  |  |  |  |  |  |  |  | Trimethoprim | 0.00193 | 61.5 |  |  |
| Large-scale | 20000 | 8640 | 4 | / | 43% domestic, 57% industriala & hospital | Phragmites australis |  | Naproxen | 355.0 | 30-60 | Netherlands | (He et al., 2018) |
|  |  |  |  |  |  |  |  | Ibuprofen | 5319 | 30-60 |  |  |
|  |  |  |  |  |  |  |  | Carbamazepine | 230 | 0 |  |  |
|  |  |  |  |  |  |  |  | Bisphenol A | 1232.0 | 0 |  |  |
|  |  |  |  |  |  |  |  | Sulfamethoxazole | 740 | 75 |  |  |
|  | 7009 | 7200 | 0.82 |  | 78% domestic, 22% industrial | Phragmites australis (reed bed); trees (swamp) |  | Naproxen | 180 | 30-60 |  |  |
|  |  |  |  |  |  |  |  | Ibuprofen | 6492 | 30-60 |  |  |
|  |  |  |  |  |  |  |  | Carbamazepine | 204 | 0 |  |  |
|  |  |  |  |  |  |  |  | Bisphenol A | 3682 | 0 |  |  |
|  |  |  |  |  |  |  |  | Sulfamethoxazole | 362 | 75 |  |  |
| mesocosm-scale | 1.44 |  | 70 | 0.2 | wastewater | Cyperus alternifolius | 30-35 | diclofenac | 120 | 69.3 | China | (Zhai et al., 2016) |
| Pilot-scale | 0.23 | 2 cm/d | 15.5 | 0.5-0.8 | wastewater | P. australis | 30 | sulfamethazine | 23.7 | 57 | China | (Liu et al., 2014) |
|  |  |  |  |  |  |  |  | tetracycline | 4.5 | 93 |  |  |
|  |  |  |  |  |  |  | 13 | sulfamethazine | 29.3 | 11 |  |  |
|  |  |  |  |  |  |  |  | tetracycline | 3.52 | 85 |  |  |
| Pilot-scale | 9.29 | 1 L/min | 2.2 | 0.15 | wastewater | Phalaris arundinaceae L. | 21.51 | monensin | 100 | 26.77 | Canada | (Hussain et al., 2012) |
|  |  |  |  |  |  |  |  | salinomycin | 100 | 28.86 |  |  |
|  |  |  |  |  |  |  |  | narasin | 100 | 34.21 |  |  |

Table SI 4. FWSCWs performances for removing pesticides from different wastewater sources

| System Scale | Area (m²) | Capacity (m³/day) | HRT | Depth (m) | Wastewater type | Vegetation | Temp ( ºC) | Pollutants removal (%) |  |  | Country |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Parameter | Initial Concentration (µg/L) | Removal% |  |  |  |
| Pilot-scale | 227 | 103 L/min | 2 | 0.8 | agricultural run-off | Juncus effuses, Phragmites australis | 22.5 | Atrazine | 197 | 19-49 | USA | (Tournebize et al., 2017) |  |
|  |  |  |  |  |  |  |  | S-Metolachlor | 287 | 56-58 |  |  |  |
|  |  |  | 28 |  |  |  |  | Atrazine | 197 | 50-97 |  |  |  |
|  |  |  |  |  |  |  |  | S-Metolachlor | 287 | 67-99 |  |  |  |
| mesocosm-scale | 0.18 | 0.34 L/d | 14 | 0.18 | Agricultural Runoff | Nymphaea amazonum | 27.5 | imidacloprid | 1.0, 30, 60, 180, 1000 | 100 | Suriname | (Mahabali and Spanoghe, 2014) |  |
|  |  |  |  |  |  |  |  | lambda-cyhalothrin | 1.0, 30, 60, 180, 1000 | 100 |  |  |  |
|  |  |  | 9 |  |  | Eleocharis mutata |  | imidacloprid | 1.0, 30, 60, 180, 1000 | 100 |  |  |  |
|  |  |  |  |  |  |  |  | lambda-cyhalothrin | 1.0, 30, 60, 180, 1000 | 100 |  |  |  |
| mesocosm-scale | 120 | 0.505 | 280 min | 0.4 | Agricultural Runoff | Phalaris arundinacea L. | 17.6 | dicamba | 50 | 89 | Norway | (Elsaesser et al., 2011) |  |
|  |  |  |  |  |  |  |  | dimethoate | 10 |  |  |  |  |
|  |  |  |  |  |  |  |  | trifloxystrobin | 5 |  |  |  |  |
|  |  |  |  |  |  |  |  | Metamitron | 10 |  |  |  |  |
|  |  |  |  |  |  |  |  | tebuconazole | 10 |  |  |  |  |
|  | 0.568 |  | 330 min |  |  | Typha latifolia L. |  | dicamba | 50 | 91 |  |  |  |
|  |  |  |  |  |  |  |  | dimethoate | 10 |  |  |  |  |
|  |  |  |  |  |  |  |  | trifloxystrobin | 5 |  |  |  |  |
|  |  |  |  |  |  |  |  | Metamitron | 10 |  |  |  |  |
|  |  |  |  |  |  |  |  | tebuconazole | 10 |  |  |  |  |
| mesocosm-scale | 0.094 | 18 ml/min | 14 | 0.1 | wastewater | P. australis | / | diuron | 10000 | 63.0 | France | (Bois et al., 2013) |  |
|  |  |  |  |  |  |  |  | 3,4-dichloroanilin | 10000 | 84.2 |  |  |  |
|  |  |  |  |  |  |  |  | glyphosate | 50000 | 99.8 |  |  |  |
| Large-scale | 110000 | 3000 | 7 | 0.2 | Industrial and residential catchment | Phragmites australis, Eleocharis sphacelata, Schoenoplectus validus, Baumea articulate and Typha orientali | / | Diuron | 0.338 | 42 | Australia | (Page et al., 2010) |  |
|  |  |  |  |  |  |  |  | Simazine | 0.195 | 44 |  |  |  |
|  |  |  |  |  |  |  |  | Atrazine | 0.059 | 49 |  |  |  |