Table 1: Background radiation dose rates (BRDR) in Calabar

Location		BRD (µSv/hr)		AEDR (mSv/yr)

Airport field		0.89 ± 0.03		1.71

UNICALa field	0.92 ± 0.06		1.77

CSCb field		0.90 ± 0.01		1.73

HWTIc field		0.98 ± 0.04		1.92

aUniversity of Calabar; bCalabar South Council; cHope Waddle Training Institute

Table 2: Radiation doses for waste dumps in Section A

Location		Coordinates			E (µSv/hr)	AEDR (mSv/yr)

SA01			N4o55.24ꞌ E8o18.84ꞌ		0.12 ± 0.02	0.23

SA02			N4055.59ꞌ E8019.41ꞌ		0.11 ± 0.02	0.21

SA03			N4055.94ꞌ E8019.14ꞌ		0.15 ± 0.03	0.29

SA04			N4056.25ꞌ E8019.21ꞌ		0.12 ± 0.03	0.23

SA05			N4056.38ꞌ E8019.73ꞌ		0.10 ± 0.01	0.19

SA06			N4056.23ꞌ E8020.26ꞌ		0.13 ± 0.02	0.25

SA07			N4057.99ꞌ E8018.65ꞌ		0.10 ± 0.01	0.19

SA08			N4056.47ꞌ E8020.27ꞌ		0.10 ± 0.02	0.19

SA09			N4056.98ꞌ E8019.91ꞌ		0.11 ± 0.01	0.21

SA10			N4056.96ꞌ E8020.16ꞌ		0.13 ± 0.02	0.25

SA11			N4057.39ꞌ E8018.98ꞌ		0.13 ± 0.02	0.25

SA12			N4057.49ꞌ E8019.21ꞌ		0.12 ± 0.01	0.23

SA13			N4057.21ꞌ E8021.46ꞌ		0.14 ± 0.01	0.27

Table 3: Radiation dose rates for waste dumps in Section B

Location		Coordinates			E (µSv/hr)	AEDR (mSv/yr)

SB01			N4o57.66ꞌ E8o20.68ꞌ		0.09 ± 0.02	0.17

SB02			N4o57.76ꞌ E8o19.74ꞌ		0.09 ± 0.01	0.17

SB03			N4o58.10ꞌ E8o21.59ꞌ		0.10 ± 0.02	0.19

SB04			N4o58.12ꞌ E8o20.75ꞌ		0.11 ± 0.02	0.21

SB05			N4o58.20ꞌ E8o21.45ꞌ		0.14 ± 0.03	0.27

SB06			N4o55.58ꞌ E8o20.48ꞌ		0.13 ± 0.01	0.25

SB07			N4o58.94ꞌ E8o19.95ꞌ		0.09 ± 0.01	0.17

SB08			N4o58.79ꞌ E8o20.13ꞌ		0.09 ± 0.01	0.17

SB09			N4o58.82ꞌ E8o20.74ꞌ		0.12 ± 0.02	0.23

SB10			N4o57.72ꞌ E8o19.42ꞌ		0.09 ± 0.01	0.17

SB11			N4o59.43ꞌ E8o21.29ꞌ		0.13 ± 0.02	0.25

SB12			N4o59.85ꞌ E8o20.34ꞌ		0.10 ± 0.01	0.19

SB13			N4o59.46ꞌ E8o19.63ꞌ		0.11 ± 0.02	0.21

SB14			N4o59.99ꞌ E8o19.82ꞌ		0.12 ± 0.02	0.23

Table 4: Radiation dose rates for waste dumps in Section C

Location		Coordinates			E (µSv/hr)	AEDR (mSv/yr)

SC01			N5o06.21ꞌ E8o19.91ꞌ		0.08 ± 0.02	0.15

SC02			N5o06.21ꞌ E8o20.41ꞌ		0.11 ± 0.01	0.21

SC03			N5o05.93ꞌ E8o20.56ꞌ		0.10 ± 0.02	0.19

SC04			N5o05.87ꞌ E8o20.49ꞌ		0.10 ± 0.01	0.19

SC05			N5o01.12ꞌ E8o19.35ꞌ		0.14 ± 0.02	0.27

SC06			N5o01.13ꞌ E8o20.21ꞌ		0.09 ± 0.02	0.17

SC07			N5o01.97ꞌ E8o20.34ꞌ		0.09 ± 0.01	0.17

SC08			N5o01.52ꞌ E8o20.51ꞌ		0.10 ± 0.01	0.19

SC09			N5o01.53ꞌ E8o20.68ꞌ		0.09 ± 0.01	0.17

SC10			N5o02.18ꞌ E8o21.92ꞌ		0.19 ± 0.01	0.36

SC11			N5o02.84ꞌ E8o21.42ꞌ		0.10 ± 0.01	0.19

SC12			N5o03.27ꞌ E8o21.33ꞌ		0.11 ± 0.02	0.21

SC13			N5o01.18ꞌ E8o19.13ꞌ		0.10 ± 0.02	0.19

Table 5: Lifetime attributed cancer incident and mortality for waste dumps in Section A

Location	AEDR (mSv/yr)	LACIm		LACMm	LACIf		LACMf

SA01		0.23			70		39		99		55

SA02		0.21			64		36		90		50

SA03		0.29			89		49		125		69

SA04		0.23			70		39		99		55

SA05		0.19			58		32		82		45

SA06		0.25			76		43		107		60

SA07		0.19			58		32		82		45

SA07		0.19			58		32		82		45

SA07		0.21			64		36		90		50

SA08		0.25			76		43		107		60

SA09		0.25			76		43		107		60

SA10		0.23			70		39		99		55

SA11		0.27			83		46		116		65

Table 6: Lifetime attributed cancer incidence and mortality for waste dumps in Section B

Location	AEDR (mSv/yr)	LACIm		LACMm	LACIf		LACMf

SB01		0.17			52		29		73		41

SB02		0.17			52		29		73		41

SB03		0.19			58		32		82		45

SB04		0.21			64		36		90		50

SB05		0.27			83		46		116		65

SB06		0.25			76		43		107		60

SB07		0.17			52		29		73		41

SB08		0.17			52		29		73		41

SB09		0.23			70		39		99		55

SB10		0.17			52		29		73		41

SB11		0.25			76		43		107		60

SB12		0.19			58		32		82		45

SB13		0.21			64		36		90		50

SB14		0.23			70		39		99		55

Table 7: Lifetime attributed cancer incidence and mortality for waste dumps in Section C

Location	AEDR (mSv/yr)	LACIm		LACMm	LACIf		LACMf

SC01		0.15			46		26		64		36

SC02		0.21			64		36		90		50

SC03		0.19			58		32		82		45

SC04		0.19			58		32		82		45

SC05		0.27			83		46		116		65

SC06		0.17			52		29		73		41

SC07		0.17			52		29		73		41

SC08		0.19			58		32		82		45

SC09		0.17			52		29		73		41

SC10		0.36			110		61		154		86

SC11		0.19			58		32		82		45

SC12		0.21			64		36		90		50

SC13		0.19			58		32		82		45

Table 8: Lifetime attributed cancer mortality calculated using equations [8]

Location	LACMm	LACMm(E)		LACMf	LACMf(E)

SA01		39		39.03			55		55.49

SA02		36		35.71			50		50.47

SA03		49		49.36			69		70.00

SA04		39		39.03			55		55.49

SA05		32		32.38			45		46.00

SA06		43		42.37			60		59.95

SA07		32		32.38			45		46.00

SA07		32		32.38			45		46.00

SA07		36		35.71			50		50.47

SA08		43		42.37			60		59.95

SA09		43		42.37			60		59.95

SA10		39		39.03			55		55.49

SA11		46		46.24			65		64.97