Experimental and Modeling Solid-Fluid and Liquid-Vapor Equilibrium Data for the CO2 + Ethanol + Ketoprofen and CO2 + Ketoprofen System

J. V. Mattos a, M. J. Molina c,d, S. B. Rodriguez-Reartes c,d, L. Ferreira-Pinto b, M. S. Zabaloy c,d, L. Cardozo-Filho a,*

a Departamento de Engenharia Química, Universidade Estadual de Maringá (UEM), Av. Colombo 5790, Maringá, Brasil

b Departamento de Engenharia de Energia, Universidade Estadual Paulista (UNESP), Av. dos Barrageiros 1881, Primavera, Rosana, Brasil

c Planta Piloto de Ingeniería Química – PLAPIQUI (UNS-CONICET), Camino “La Carrindanga” Km 7, Bahía Blanca, (8000), Argentina

d Departamento de Ingeniería Química, Universidad Nacional del Sur (UNS), Avda. Alem 1253, Bahía Blanca, Argentina

∗Corresponding author

e-mail addresses, lcfilho@uem.br

Supplementary material

According to Hukkerikar (2012) the property prediction model to estimate the properties of pure components employing MG method [2] has the form.

fX=iNiCi+wjMjDj+zkEkOk					(1)

The function f(X) (Eq. 1) is a function of property X and it may contain additional adjustable model parameters (universal constants) depending on the property involved. In Eq. (1), Ci is the contribution of the first-order group of type-i that occurs Ni times. Dj is the contribution of the second-order group of type-j that occurs Mj times. Ek is the contribution of the third-order group of type-k that has Ok occurrences in a component. Note that, Eq. (1) is a general model for all the properties and the definition of f(X) is specific for each property X.

The specific functions for each of the properties calculated for ketoprofen are listed below.

Critical Temperature=fX=exp⁡(TcTc0)				(2)

Critical Pressure=fX=Pc-Pc1-0.5-Pc2 			(3)

Acentric Factor=fX=expωωaωb-ωc				(4)

In Eq. (2), the symbol Tc0 corresponds to the universal constant for critical temperature and its unit is kelvin (K), in Eq. (3), the symbol Pc1 and Pc2 corresponds to the universal constant for critical pressure and its unit is (bar) and in Eq. (4) the symbols ωa, ωb, and ωc corresponds to the universal constant for acentric factor. All universal constants values used is for step-wise method and are present in Table 1.

Table 1. Universal constants value used for the calculus of critical properties and acentric factor

| Universal constant | Value |
| --- | --- |
| Tc0 | 181.6716 |
| Pc1 | 0.0519 |
| Pc2 | 0.1347 |
| ωa | 0.908 |
| ωb | 0.1055 |
| ωc | 1.0012 |

The molecular structure of ketoprofen (Figure 1) allows all the groups to be separated in order to predict its critical properties and acentric factor. Table 2 shows the groups used and the number of occurrences of each group and Table 3 shows the value of the desire properties of ketoprofen calculated by the contribution of the Ref group [1].

In its support material of Ref [1] contains all the tables with each group and their respective values for each of their properties, as well as classifying each group as first order, second order and third order. The groups used to calculate the critical properties and acentric factor of ketoprofen are classified as first-order groups.

Figure 1. Ketoprofen chemical structure Ref. [3]

Table 2. Groups used and their occurrence numbers

| Group | Number of components |
| --- | --- |
| CH3 | 1 |
| CH | 1 |
| aCH | 9 |
| COOH | 1 |
| aC-CO | 2 |

Table 3 – Estimate properties using group contribution

| Compound | Tcrit / K | Pcrit / MPa | ω |
| --- | --- | --- | --- |
| Ketoprofen | 882.59c | 2.5052c | 1.0091c |

References

- A.S. Hukkerikar, B. Sarup, A. Ten Kate, J. Abildskov, G. Sin, R. Gani, Group-contribution+ (GC+) based estimation of properties of pure components: Improved property estimation and uncertainty analysis, Fluid Phase Equilibria 321 (2012) 25–43.

- J. Marrero, R. Gani, Fluid Phase Equilibr. 183-184 (2001) 183–208.

- NIST Chemistry WebBook. NIST Standard Reference Database Number 69 (2022). Access in: Sep. 28, 2023. Available in: < https://webbook.nist.gov/cgi/cbook.cgi?ID=22071154&Units=SI>