# Appendix A

|  | Analyser | Measuring principle | Manufacturer | Measuring location/measuring medium |
| --- | --- | --- | --- | --- |
| Gas Composition | IRD CO2/COh | Non-dispersive infrared analyser CO2/CO high | AVL | Raw exhaust gas and after catalyst |
|  | PMD O2 | Paramagnetic O2 analyser | AVL | Raw exhaust gas and after catalyst |
|  | IRD 4000 COl | Non-dispersive infrared analyser CO low | AVL | Raw exhaust gas and after catalyst |
|  | CLD 4000 hhd | Chemiluminescence detector undiluted, hot measurement | AVL | Raw exhaust gas and after catalyst |
|  | Cutter FID I60 LHD | Flame ionisation detector diluted, hot measurement | AVL | Raw exhaust gas and after catalyst |
|  | FTIR | Fourier transform infrared spectrometer | IAG | Raw exhaust gas and after catalyst |
|  | Lambda-Meter | Lambda | ETAS | Raw exhaust gas |
|  | Lambda-Sensor LSU4.9 | Lambda | Bosch | Raw exhaust gas |
|  | URAS | Non-dispersive infrared analyser | ABB | CO2 composition at inlet and exhaust |
|  | TOTALFLOW | Gas chromatograph | ABB | Natural gas |
|  | AFK-G | Humidity | Kobold | Charge air |
| Pressure Indication | 6056AU20 | High pressure relative | Kistler | Cylinder 1 |
|  | 6056AU20 | High pressure relative | Kistler | Cylinder 2 |
|  | 6056AU20 | High pressure relative | Kistler | cylinder 3 |
|  | 4075A10V39 | Low pressure absolute | Kistler | Intake manifold |
|  | 4049A5S | Low pressure absolute | Kistler | Exhaust gas after engine |
|  | 4075A10V200S | Low pressure absolute | Kistler | Exhaust gas before catalyst |
|  | 4045A5 | Low pressure absolute | Kistler | Exhaust gas after catalyst |
| Flow Measurement | EcoRPM | Volume flow via rotary piston gas meter (Rotary Piston Meter) | RMA | Charge air |
|  | PROMASS 83 | Mass flow via Coriolis | Endress + Hauser | Natural gas |
|  | DF8 | Mass flow via differential pressure transmitter | Systec | Exhaust gas in bypass |

# Appendix B

Fig. 9 CH4 concentrations over measurement time with the higher level of emissions representing the pre catalyst values and the lower level showing the one measured post catalyst