Supporting Information

Sedimentation of large, soluble proteins up to 140 kDa for 1H-detected MAS NMR and 13C DNP NMR – practical aspects

Dallas Bell1,2, Florian Lindemann2, Lisa Gerland2, Hanna Aucharova3, Alexander Klein3, Daniel Friedrich4, Matthias Hiller2, Kristof Grohe5, Barth van Rossum2, Anne Diehl2, Jon Hughes6,7, Leonard J. Mueller8, Rasmus Linser3, Anne-Frances Miller1,9 and Hartmut Oschkinat2

Affiliations

1Faculty II-Mathematics and Natural Sciences, Technische Universität Berlin, Straße des 17. Juni 135, 10623 Berlin, Germany

2Leibniz-Forschungsinstitut für Molekulare Pharmakologie, Robert-Rössle-Str. 10, 13125 Berlin, Germany

3Department of Chemistry and Chemical Biology, TU Dortmund University, Otto-Hahn-Str. 4a, 44227 Dortmund, Germany.

4Department of Chemistry and Biochemistry, University of Cologne, Greinstr. 4, 50939 Cologne, Germany

5Bruker BioSpin GmbH & Co. KG, Rudolf-Plank-Str. 23, 76275 Ettlingen, Germany

6Justus Liebig University, Institute for Plant Physiology, Senckenbergstr. 3, 35360 Gießen, Germany

7Free University of Berlin, Department of Physics, Arnimallee 14, 14195 Berlin, Germany

8Department of Chemistry, University of California - Riverside, CA 92521 Riverside, USA

9Department of Chemistry, University of Kentucky, KY 40506 Lexington, USA

Fig. S1 Technical drawing of the 0.7mm filling tool, overview.

Fig. S2 Technical drawings of the 0.7mm filling tool, detailed view.

Fig. S3 Effect of magic angle spinning on the sample inside the NMR rotor. (a) Top view of a 0.7 mm diameter rotor directly after filling StTS by ultracentrifugation. Due to the filling procedure, the bottom cap is attached. (b) Top view of the identical 0.7 mm rotor after multiple sessions of MAS NMR measurements at 100 kHz spinning after removing both caps. Enlarged images are shown on the right

Fig. S4 1D slices along a 1H chemical shift of 8.95 ppm from 1H-15N correlations of deuterated SulfETF by solid-state NMR (CP-based, yellow) and solution NMR (INEPT-based ,blue). Full spectra are shown in main text Fig. 3.

Fig. S5 1D 1H spectra used to determine the change of water content over time. A 0.7 mm diameter rotor containing sedimented SulfETF was measured at 900 MHz 1H Larmor frequency and at 100 kHz MAS. Spectra and integration regions of freshly filled sample (a and b) and after 1 year (c and d) are shown. The determined integrals are: (a and b) |1|: 1.00; |2|: 12.35; |3|: 2.24, (c and d) |1|: 1.00; |2|: 13.77; |3|: 2.86.

Fig. S6 Comparison of CP-based 1H-15N SulfETF spectra to examine the change of the sample over time. Measurements were conducted with a 0.7 mm rotor spinning at 100 kHz MAS on a spectrometer with 900 MHz 1H Larmor frequency directly after filling (orange, from the main text) and after one year of storage (blue).

Fig. S7 3D hCANH and hCONH spectra acquired on 1H,13C,15N-labelled StTS, filled into a 0.7 mm rotor and at 900 MHz 1H Larmor frequency and 100 kHz MAS. (a) Cube representation of the hCANH. (b) Cube representation of the hCONH. (c) 1H-13C projection of the hCANH. (d) 1H-13C projection of the hCONH. (e) 1H-15N projection of the hCANH. (f) 1H-15N projection of the hCONH.

Fig. S8 CP-based 1H-13C correlation of StTS, recorded with a 0.7 mm rotor at 100 kHz MAS and 900 MHz 1H Larmor frequency.

Fig. S9 Comparisons of transfer efficiencies on PfTrpB at 700 MHz 1H Larmor frequency. Individual bulk intensities were determined on the first 1D free induction decay with 256 scans and compared to the bulk intensity of the first 1D experiment of a 2D hNH experiment (also measured with 256 scans). The relative intensities are given as percentages. (a) Measurements using 13C, 15N-labelled protein in a 0.7 mm rotor at 100 kHz. The 1H bulk T1 time was determined to be 750 ms, a recycle delay of 1 s was used. (b) Measurements using 2H, 13C, 15N-labelled and back-exchanged sedimented into a 1.3 mm rotor at 55 kHz MAS. The 1H bulk T1 time was determined to be 800 ms, a recycle delay of 1 s was used.

Fig. S10 DNP enhancement obtained by the radical BcTol-M on Cph1Δ2 from Synechocystis at 800 MHz 1H Larmor frequency spinning at 20 kHz MAS. The spectra shown were acquired with (blue) and without (red) microwave irradiation.

Table S1 Acquisition parameters for the 2D 1H-15N spectra of SulfETF.

| Acquisition parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CP-based hNH (Fig. 3) |  |  | Solution 1H-15N HSQC (Fig. 3) |  |
|  |  | F2 | F1 | F2 | F1 |
| Quad. Mode |  |  | States-TPPI |  | States-TPPI |
| TD points |  | 1024 | 640 | 1024 | 512 |
| Nucleus |  | 1H | 15N | 1H | 15N |
| SW / ppm |  | 39.7 | 99.7 | 16.3 | 79.5 |
| Aq.time / ms |  | 14.3 | 35.2 | 34.8 | 35.3 |
| Scans |  | 32 |  | 64 |  |
| Acquisition parameters |  |  |  |  |  |
| CP-based hNH @ 289 K (Fig. 4) |  |  |  | CP-based hNH @ 313 K (Fig. 4) |  |
|  |  | F2 | F1 | F2 | F1 |
| Quad. Mode |  |  | States-TPPI |  | States-TPPI |
| TD points |  | 2048 | 512 | 2048 | 512 |
| Nucleus |  | 1H | 15N | 1H | 15N |
| SW / ppm |  | 39.7 | 80.0 | 39.7 | 85.7 |
| Aq.time / ms |  | 28.7 | 35.1 | 34.8 | 32.8 |
| Scans |  | 32 |  | 32 |  |
| Acquisition parameters |  |  |  |  |
| INEPT-based hNH @ 289 K (Fig. 4) |  |  |  | INEPT-based hNH @ 313 K (Fig. 4) |  |
|  |  | F2 | F1 | F2 | F1 |
| Quad. Mode |  |  | States-TPPI |  | States-TPPI |
| TD points |  | 2048 | 384 | 2048 | 256 |
| Nucleus |  | 1H | 15N | 1H | 15N |
| SW / ppm |  | 39.7 | 80.0 | 39.7 | 85.7 |
| Aq.time / ms |  | 28.7 | 26.3 | 28.7 | 16.4 |
| Scans |  | 32 |  | 48 |  |  |

| Acquisition parameters |  |  |
| --- | --- | --- |
| CP-based hNH after 1 year (Fig. S6) |  |  |
|  | F2 | F1 |
| Quad. Mode |  | States-TPPI |
| TD points | 2048 | 2048 |
| Nucleus | 1H | 15N |
| SW / ppm | 39.7 | 142.0 |
| Aq.time / ms | 28.7 | 79.1 |
| Scans | 32 |  |

Table S2 RF fields used to obtain the 2D 1H-15N spectra of SulfETF.

| CP-based hNH (Fig. 3) |  |  | Solution 1H-15N HSQC (Fig. 3) |  |  |
| --- | --- | --- | --- | --- | --- |
| Hard pulses | µs | RF / kHz |  | µs | RF / kHz |
| 1H | 0.85 | 294.12 |  | 10.29 | 24.30 |
| 15N | 5 | 50 |  | 40 | 6.25 |
| 13C | 5 | 50 |  |  |  |
| CP-based hNH @ 289 K (Fig. 4) |  |  | CP-based hNH @ 313 K (Fig. 4) |  |  |
| Hard pulses | µs | RF / kHz |  | µs | RF / kHz |
| 1H | 0.9 | 277.78 |  | 1 | 250.0 |
| 15N | 4.6 | 54.35 |  | 5 | 50 |
| 13C | 2.5 | 100 |  | 5 | 50 |
| INEPT-based hNH @ 289 K (Fig. 4) |  |  | INEPT-based hNH @ 313 K (Fig. 4) |  |  |
| Hard pulses | µs | RF / kHz |  | µs | RF / kHz |
| 1H | 0.9 | 277.78 |  | 1 | 250.0 |
| 15N | 4.6 | 54.35 |  | 5 | 50 |
| 13C | 5 | 50 |  | 5 | 50 |
| CP-based hNH after 1 year (Fig. S6) |  |  |  |  |  |
| Hard pulses | µs | RF / kHz |  |  |  |
| 1H | 0.9 | 277.78 |  |  |  |
| 15N | 5.7 | 44.17 |  |  |  |
| 13C | 2.5 | 100 |  |  |  |

Table S3 CP conditions used to obtain the hNH spectra of SulfETF.

| hNH (Fig. 3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | N-H CP |  | RF 1H / kHz | RF 15N / kHz |
| Contact time/µs | 800 | 68.6 | 34.13 | Contact time/µs | 600 | 68.6 | 29.22 |
| Shape |  | rectangular | 100-60 tang. | Shape |  | rectangular | 60-100 tang. |
| hNH @ 289 K (Fig. 4) |  |  |  |  |  |  |  |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | N-H CP |  | RF 1H / kHz | RF 15N / kHz |
| Contact time/µs | 1000 | 78.16 | 37.23 | Contact time/µs | 700 | 78.16 | 33.30 |
| Shape |  | rectangular | 100-60 tang. | Shape |  | rectangular | 60-100 tang. |
| hNH @ 313 K (Fig. 4) |  |  |  |  |  |  |  |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | N-H CP |  | RF 1H / kHz | RF 15N / kHz |
| Contact time/µs | 800 | 70.84 | 36.40 | Contact time/µs | 400 | 70.84 | 30.06 |
| Shape |  | rectangular | 100-60 tang. | Shape |  | rectangular | 60-100 tang. |
| hNH after 1 year (Fig. S6) |  |  |  |  |  |  |  |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | N-H CP |  | RF 1H / kHz | RF 15N / kHz |
| Contact time/µs | 1100 | 78.16 | 31.97 | Contact time/µs | 700 | 78.16 | 28.92 |
| Shape |  | rectangular | 100-60 tang. | Shape |  | rectangular | 60-100 tang. |

Table S4 Acquisition parameters for the spectra of StTS.

| Acquisition parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | hNH (Fig. 5a) |  |  |  | hNH (Fig. 5b) |  |  |  |  |  |  |  |
|  | F2 | F1 |  |  |  |  | F2 |  | F1 |  |  |  |  |
| Quad. Mode |  | States-TPPI |  |  |  |  |  |  | States-TPPI |  |  |  |  |
| TD points | 2048 | 400 |  |  |  |  | 2048 |  | 400 |  |  |  |  |
| Nucleus | 1H | 15N |  |  |  |  | 1H |  | 15N |  |  |  |  |
| SW / ppm | 39.7 | 137 |  |  |  |  | 39.7 |  | 78.34 |  |  |  |  |
| Aq.time / ms | 28.7 | 16 |  |  |  |  | 28.7 |  | 28 |  |  |  |  |
| Scans | 128 |  |  |  |  |  | 128 |  |  |  |  |  |  |
| Acquisition parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| hCANH (Fig. S7a, c and e) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | F3 |  |  | F2 |  |  | F1 |  |  |  |  |  |  |
| Quad. Mode |  |  |  | States-TPPI |  |  | States-TPPI |  |  |  |  |  |  |
| TD points | 1024 |  |  | 64 |  |  | 126 |  |  |  |  |  |  |
| Nucleus | 1H |  |  | 15N |  |  | 13C |  |  |  |  |  |  |
| SW / ppm | 39.7 |  |  | 34.3 |  |  | 44.2 |  |  |  |  |  |  |
| Aq.time / ms | 14.3 |  |  | 10.2 |  |  | 6.4 |  |  |  |  |  |  |
| Scans | 16 |  |  |  |  |  |  |  |  |  |  |  |  |
| Acquisition parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| hCONH (Fig. S7b, d and f) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | F3 |  |  | F2 |  |  | F1 |  |  |  |  |  |  |
| Quad. Mode |  |  |  | States-TPPI |  |  | States-TPPI |  |  |  |  |  |  |
| TD points | 1024 |  |  | 80 |  |  | 64 |  |  |  |  |  |  |
| Nucleus | 1H |  |  | 15N |  |  | 13C |  |  |  |  |  |  |
| SW / ppm | 39.7 |  |  | 34.3 |  |  | 17.7 |  |  |  |  |  |  |
| Aq.time / ms | 14.3 |  |  | 12.8 |  |  | 8.0 |  |  |  |  |  |  |
| Scans | 24 |  |  |  |  |  |  |  |  |  |  |  |  |

Table S5 RF fields and CP conditions used for the spectra of StTS.

| hNH (Fig. 5a and b) |  |  | hCANH and hCONH (Fig. S7) |  |  |
| --- | --- | --- | --- | --- | --- |
| Hard pulses | µs | RF / kHz |  | µs | RF / kHz |
| 1H | 0.85 | 294.11 |  | 2.5 | 100 |
| 15N | 5 | 50 |  | 5 | 50 |
| 13C | 5 | 50 |  | 5 | 50 |

| hNH (Fig. 5a and b) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H-N CP |  | RF 1H / kHz |  | RF 15N / kHz |  | N-H CP |  |  | RF 1H / kHz | RF 15N / kHz |
| Contact time/µs | 1200 | 70.15 |  | 34.61 |  | Contact time/µs |  | 800 | 70.15 | 28.87 |
| Shape |  | rectangular |  | 100-60 tang. |  | Shape |  |  | rectangular | 60-100 tang. |
| hCANH (Fig. S7a, c and e) |  |  |  |  |  |  |  |
| H-C CP |  | RF 1H / kHz |  |  | RF 15N / kHz |  | RF 13C / kHz |
| Contact time/µs | 1400 | 71.98 |  |  |  |  | 23.61 |
| Shape |  | 80-100 ramp |  |  |  |  | rectangular |
| CA-N |  |  |  |  |  |  |  |
| Contact time/µs | 10000 |  |  |  | 23.71 |  | 71.39 |
| Shape |  |  |  |  | rectangular |  | 70-100 tang. |
| H-N CP |  | RF 1H / kHz |  |  | RF 15N / kHz |  | RF 13C / kHz |
| Contact time/µs | 800 | 70.89 |  |  | 28.2 |  |  |
| Shape |  | 80-100 ramp |  |  | rectangular |  |  |
| hCONH (Fig. S7b, d and f) |  |  |  |  |  |  |  |
| H-C CP |  | RF 1H / kHz |  |  | RF 15N / kHz |  | RF 13C / kHz |
| Contact time/µs | 1400 | 71.98 |  |  |  |  | 23.0 |
| Shape |  | 80-100 ramp |  |  |  |  | rectangular |
| CO-N CP |  | RF 1H / kHz |  |  | RF 15N / kHz |  | RF 13C / kHz |
| Contact time/µs | 10000 |  |  |  | 25.52 |  | 67.94 |
| Shape |  |  |  |  | rectangular |  | 70-100 tang |
| H-N CP |  | RF 1H / kHz |  |  | RF 15N / kHz |  | RF 13C / kHz |
| Contact time/µs | 800 | 70.89 |  |  | 21.65 |  |  |
| Shape |  | 80-100 ramp |  |  | rectangular |  |  |

Table S6 Acquisition parameters for the 2D hNH spectrum of the u-[13C,15N] labeled sample of the 2B9 mutant of PfTrpB spun at 100 kHz MAS in a 0.7 mm rotor as shown in main text Fig. 6.

| Acquisition parameters |  |  |
| --- | --- | --- |
|  | F2 | F1 |
| Quad. Mode |  | States-TPPI |
| TD points | 800 | 138 |
| Nucleus | 1H | 15N |
| SW / ppm | 39.7 | 39.1 |
| Aq.time / ms | 14.4 | 24.8 |
| Scans | 128 |  |

Table S7 RF fields and CP conditions used to obtain the 1HN bulk intensities in main text Fig. 6 and Fig. S9 for the u-[13C,15N] labeled sample of the 2B9 mutant of PfTrpB spun at 100 kHz MAS in a 0.7 mm rotor.

| Hard pulses | µs | RF / kHz |
| --- | --- | --- |
| 1H | 1.4 | 178.57 |
| 15N | 2.8 | 89.29 |
| 13C | 2.6 | 96.15 |

| hNH |  |  |  |  |
| --- | --- | --- | --- | --- |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1150 | 80.4 | 28.2 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| hCANH |  |  |  |  |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 300 | 80.4 |  | 31.3 |
| Shape |  | 100-50 tang. |  | rectangular |
| Ca-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 66.6 | 31.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 500 | 80.4 | 28.2 |  |
| Shape |  | 100-50 tang. | rectangular |  |

| hCONH |  |  |  |  |
| --- | --- | --- | --- | --- |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1500 | 80.4 |  | 29.5 |
| Shape |  | 100-50 tang. |  | rectangular |
| CO-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 66.6 | 31.3 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 500 | 80.4 | 28.2 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| hCACONH |  |  |  |  |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 650 | 80.4 |  | 31.3 |
| Shape |  | 100-50 tang. |  | rectangular |
| CO-CA BSH-CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Trim pulse/µs | 1.04 |  |  | 96.2 |
| Contact time/µs | 5500 |  |  | 42.2 |
| Shape |  |  |  | 90-100 ramp |
| CO-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 66.6 | 31.3 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 500 | 80.4 | 28.2 |  |
| Shape |  | 100-50 tang. | rectangular |  |

| hCOCANH |  |  |  |  |
| --- | --- | --- | --- | --- |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1500 | 80.4 |  | 29.5 |
| Shape |  | 100-50 tang. |  | rectangular |
| CO-CA BSH-CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Trim pulse/µs | 1.04 |  |  | 96.2 |
| Contact time/µs | 5500 |  |  | 44.7 |
| Shape |  |  |  | 90-100 ramp |
| Ca-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 66.6 | 31.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 500 | 80.4 | 28.2 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| HNcocANH |  |  |  |  |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1500 | 80.4 | 28.2 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| CO-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 66.6 | 31.3 |
| Shape |  |  | 90-100 ramp | rectangular |
| CO-CA BSH-CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Trim pulse/µs | 1.04 |  |  | 96.2 |
| Contact time/µs | 5500 |  |  | 44.7 |
| Shape |  |  |  | 90-100 ramp |
| Ca-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 66.6 | 31.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 500 | 80.4 | 28.2 |  |
| Shape |  | 100-50 tang. | rectangular |  |

Table S8 Acquisition parameters for the 2D hNH spectrum of the u-[2H,13C,15N] labeled sample of the 2B9 mutant of PfTrpB spun at 55 kHz MAS in a 1.3 mm rotor as shown in main text Fig. 6.

| Acquisition parameters |  |  |
| --- | --- | --- |
|  | F2 | F1 |
| Quad. Mode |  | States-TPPI |
| TD points | 800 | 138 |
| Nucleus | 1H | 15N |
| SW / ppm | 39.7 | 39.1 |
| Aq.time / ms | 19.2 | 24.8 |
| Scans | 128 |  |

Table S9 RF fields and CP conditions used to obtain the 1HN bulk intensities in main text Fig. 6 and Fig. S9 for the u-[2H,13C,15N] labeled sample of the 2B9 mutant of PfTrpB spun at 55 kHz MAS in a 1.3 mm rotor.

| Hard pulses | µs | RF / kHz |
| --- | --- | --- |
| 1H | 1.6 | 156.25 |
| 15N | 5 | 50.00 |
| 13C | 3.2 | 78.13 |

| hNH |  |  |  |  |
| --- | --- | --- | --- | --- |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 150 | 13.4 | 42.5 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| hCANH |  |  |  |  |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1200 | 15.9 |  | 45.4 |
| Shape |  | 100-80 ramp |  | rectangular |
| Ca-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 38.3 | 18.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 150 | 13.4 | 42.5 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| hCONH |  |  |  |  |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1300 | 15.9 |  | 47.0 |
| Shape |  | 100-80 ramp |  | rectangular |
| CO-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 38.3 | 18.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 150 | 13.4 | 42.5 |  |
| Shape |  | 100-50 tang. | rectangular |  |

| hCACONH |  |  |  |  |
| --- | --- | --- | --- | --- |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1300 | 15.9 |  | 46.0 |
| Shape |  | 100-80 ramp |  | rectangular |
| CO-CA BSHCP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Trim pulse/µs | 1.45 |  |  | 78.13 |
| Contact time/µs | 6500 |  |  | 28.5 |
| Shape |  |  |  | 90-100 ramp |
| Ca-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 38.3 | 18.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 150 | 13.4 | 42.5 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| hCOCANH |  |  |  |  |
| H-C CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 1300 | 15.9 |  | 47.0 |
| Shape |  | 100-80 ramp |  | rectangular |
| CO-CA BSH-CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Trim pulse/µs | 1.45 |  |  | 78.13 |
| Contact time/µs | 6500 |  |  | 28.5 |
| Shape |  |  |  | 90-100 ramp |
| Ca-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 38.3 | 18.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 150 | 13.4 | 42.5 |  |
| Shape |  | 100-50 tang. | rectangular |  |

| HNcoCANH |  |  |  |  |
| --- | --- | --- | --- | --- |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 150 | 13.4 | 42.5 |  |
| Shape |  | 100-50 tang. | rectangular |  |
| CO-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 38.3 | 18.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| CO-CA BSHCP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Trim pulse/µs | 1.45 |  |  | 78.13 |
| Contact time/µs | 6500 |  |  | 28.5 |
| Shape |  |  |  | 90-100 ramp |
| Ca-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 11000 |  | 38.3 | 18.7 |
| Shape |  |  | 90-100 ramp | rectangular |
| H-N CP |  | RF 1H / kHz | RF 15N / kHz | RF 13C / kHz |
| Contact time/µs | 500 | 13.4 | 42.5 |  |
| Shape |  | 100-50 tang. | rectangular |  |