Supplementary Tables

Supplementary Table 1 Input parameters for the calculation of the technologies involved in the DACCS and DACCU pathways. To calculate the CAPEX of water electrolysers in €/kgH2, we assume an electrolyser lifetime of 20 years, a stack lifetime of 7 years, and 8760 hours of operation per year. Starred technologies indicate values on which a sensitivity analysis was performed.

| Technology | CAPEX |  | Installed capacity |  |
| --- | --- | --- | --- | --- |
|  | 2020 | Ref. | 2020 | Ref. |
| DAC* | 870 €/tCO2 | c | 10 ktCO2 | 88 |
| Polymer membrane eletrolysis | 920 €/kW | 89 | 7 MtH2 | 22 |
| Alkaline electrolysis | 758 €/kW | 89,90 | 7 MtH2 | 22 |
| Reverse-Water-Gas-Shift | 4.98 €/tCO | 28,91 | 0.002 MtCO | 22 |
| Electrochemical CO2 reduction | 421.53 €/tCO | 92,93 | 0.002 MtCO | 22 |
| Capital and operational costs assumed constant |  |  |  |  |
| Fischer-Tropsch synthesis CAPEX | 0.108 €/kgfuel assumed constant | 22 |  |  |
| Low-temperature heat | 10 €/kWhth | 94 |  |  |
| High-temperature heat | 40 €/kWhth | 94 |  |  |
| CO2 transport and storage cost | 20 €/tCO2 |  |  |  |
| Cost fossil fuels* | 0.6 €/L |  |  |  |
| Electricity cost* | 0.03 €/kWh | 61 |  |  |

Supplementary Table 2 Input parameters for the yearly energy consumption of the technologies involved in the DACCS and DACCU pathways.

| Technology | Electricity consumption |  |  |  | Heat consumption |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2020 | Ref. | 2050 | Ref. |  | Ref. |
| DAC | 0.5 MWhe/tCO2 | 32 | 0.25 | 22 | 1.75 MWhth/tCO2 | 95 |
| Polymer membrane eletrolysis | 57 kWhe/kgH2 | 89,96–100 | 47.5 kWhe/kgH2 |  |  |  |
| Alkaline electrolysis | 53 kWhe/kgH2 | 22,89,103–106 | 45.5 kWhe/kgH2 |  |  |  |
| Reverse-Water-Gas-Shift | 0.11 kWhe/kgCO | 107 | 0.11 kWhe/kgCO | 107 | 0.49 kWhth/kgCO | 107 |
| Electrochemical CO2 reduction | 6.34 kWhe/kgCO | 22,92,93 | 6.34 kWhe/kgCO | 22,92,93 |  |  |
| Fischer-Tropsch synthesis | 0.26 kWh/kgfuel | 28 | 0.26 kWh/kgfuel | 28 |  |  |

Supplementary Table 3 Other input parameters and assumptions. Starred parameters indicate values on which a sensitivity analysis was performed.

| Parameter | Value | Reference |
| --- | --- | --- |
| Growth aviation demand* | 2% | 10,70–75 |
| Fuel efficiency increase* | 2% | 76 |
| Learning rate DAC* | 12% | 39 |
| Learning rate electrolysers* | 8% | Based on future CAPEX estimates from 89,108–110 |
| Learning rate CO reduction* | 7.5% | 111 |

| Parameter | Description |  |  |
| --- | --- | --- | --- |
| Occupancy aircraft | 80% |  |  |
|  | London - Berlin | London – New York | London - Perth |
| Number passengers | 178 | 318.5 | 296 |
| Distance flown (km) | 964 | 5570 | 14500 |
| Average 2023 cost60 | 65 | 300 | 1500 |
| Percentage of cost due to fuel | 20% | 25% | 13% |

Supplementary Table 4 Input parameters and basic assumptions to calculate increase in price per passenger per flight.

Supplementary Table 5 Input parameters and basic assumptions for the policy scenarios.

| Parameter | Description |
| --- | --- |
| Cost on CO2 emissions | €100/tCO2 in 2020 and then increase by 1% yearly |
| Cost on all aviation climate impacts | €100/tCO2e* in 2020 and then increase by 1% yearly |
| Subsidy on DACCS | €100/tCO2 in 2020 and then decrease by 1% yearly |
| Subsidy on DACCU | €100/tCO2 (€33/tfuel) in 2020 and then decrease by 1% yearly |
| Excess electricity | Electricty cost is 10% of basic assumption (€0.003/kWh) |

Supplementary Figures

Supplementary Figure 1 Absolute emissions of DACCS and DACCU pathway (without including the offsets via DACCS). Grey bars show uncertainties due to the non-CO2 effects of aviation.

Supplementary Figure 2 Installed DAC rates under different climate targets and technology scenarios.

Supplementary Figure 3 Energy needed under different climate targets and technology scenarios.

Supplementary Figure 4 Total cost of the two pathways until 2060.

Supplementary Figure 5 Final cost by 2050 in € for different configurations of DACCU, resulting from the different combination of polymer membrane electrolysis (PEM), alkaline electrolysis (AEC), electrochemical CO2 reduction (El.CO2), reverse-water-gas-shift (RWGS), and Fischer-Tropsch synthesis (FT).

Supplementary Figure 6 Impact of electricity cost and learning rate on the difference in cost by 2060 of DACCS and DACCU pathways to reach CO2 neutrality. Each cell shows the difference in cost between DACCU and DACCS pathways under varying assumptions on the learning rate (y-axis) and electricity cost (x-axis).

Supplementary Figure 7 Impact of electricity cost and fossil fuel cost on the difference in cost by 2060 of DACCS and DACCU pathways to reach CO2 neutrality. Each cell shows the difference in cost between DACCU and DACCS pathways under varying assumptions on the learning rate (y-axis) and fossil fuel cost (x-axis).

Supplementary Figure 8 Impact of electricity cost and fossil fuel cost on the difference in cost by 2060 of DACCU and fossil kerosene. Each cell shows the difference in cost between DACCU and fossil kerosene under varying assumptions on the learning rate (y-axis) and fossil fuel cost (x-axis).

Supplementary Figure 9 Impacts of the local variation in important input parameters on the 2050 difference between DACCU and DACCS pathway to achieve climate neutrality. Blue cells indicate when DACCU becomes cheaper than fossil kerosene, while red cells are the opposite.

With Boeing 787-9 Dreamliner

Average of Boeing 787-9 Dreamliner, Boeing 777-200LR, Boeing 777-3000LR, and Boeing 767-300

With Boeing 737-900